Draft D1 IEEE Std 802.15.13™-20xx

IEEE Standard for

Local and metropolitan area networks—

Part 15.13:

Multi Giga bit/sec Optical Wireless

Communications

Sponsor

LAN/MAN Standards Committee

of the

IEEE Computer Society

Approved TBD

IEEE-SA Standards Board

editor note: abstract and keywords need to be revisited

Abstract: A PHY and a MAC layer for short-range optical wireless communications in optically transparent media are defined using the light wavelength from 10,000 nm to 190 nm where transmitting and receiving devices incorporate light emitting sources and photodiodes as well as digital cameras with a lens and an image sensor. The standard is capable of delivering data rates sufficient to support audio and video multimedia services and robustness as well as low latency for communications between machines and also considers mobility of the optical link, compatibility with various light infrastructures, impairments due to noise and interference from sources like ambient light and a MAC layer that accommodates the unique needs of digital communications using the light. The standard adheres to applicable eye safety regulations.

Keywords: IEEE 802.15.13, laser diode, LD, LED, light-emitting diode, short-range optical wireless communications, visible light, visible-light communication, VLC, OWC, LiFi, discrete multitoned, DMT, orthogonal frequency-division multiplex, OFDM, adaptive multiple subcarrier transmission, multiple-input multiple-output, MIMO, coordinated wireless network, optical camera communications, OCC

The Institute of Electrical and Electronics Engineers, Inc.

3 Park Avenue, New York, NY 10016-5997, USA

Copyright © 20xx by the Institute of Electrical and Electronics Engineers, Inc.

All rights reserved. Published 6 September 2011. Printed in the United States of America.

IEEE and 802 are registered trademarks in the U.S. Patent & Trademark Office, owned by The Institute of Electrical and

Electronics Engineers, Incorporated.

POF:	18BN 978-0-7381-6665-0    8T097117

Prin!:	18BN 978-0-7381-6666-7    8TOP097117

/EEE  prohibits  discriminatio.  harassn	nt  and  bu//yilη  For  n 	information visit !:J.ffno//lMIvWie     ora/weblaboufusiINhafis/no/icies/n9-

Z

No part of this pub/ication may be reproduced in any formin an e/ecfronic retrie   /   system  or otherwisewithout the prior written permission fthe pub/isher.

IEEE Standards documents are developed within the IEEE Societies and the Standards Coordinating Committees of the IEEE  Standards  Association  (IEEE-SA)  Standards  Board.  The  IEEE  develops  its  standards  through  a  consensus development   process,   approved   by  the  American   National   Standards   Institute,   which  brings  together   volunteers representing varied viewpoints and interests to achieve the final product. Volunteers are not necessarily members of the Institute and serve without compensation. While the IEEE administers the process and establishes rules to promote fairness in the consensus development process, the IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

Use of an IEEE Standard is wholly voluntary. The IEEE disclaims  liability for any personal injury, property or other damage, of any nature whatsoever, whether special, indirect, consequential, or compensatory, directly or indirectly resulting from the publication, use of, or reliance upon this, or any other IEEE Standard document.

The IEEE does not warrant or represent the accuracy or content of the material contained herein, and expressly disclaims any express or implied warranty, including any implied warranty of merchantability or fitness for a specific purpose, or that the use of the material contained herein is free from patent infringement. IEEE Standards documents are supplied “AS IS.”

The existence of an IEEE Standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE Standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard. Every IEEE Standard is subjected to review at least every five years for revision or reaffirmation, or every ten years for stabilization. When a document is more than five years old and has not been reaffirmed, or more than ten years old and has not been stabilized, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE Standard.

In publishing and making this document available, the IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity. Nor is the IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing this, and any other IEEE Standards document, should rely upon the advice of a competent professional in determining the exercise of reasonable care in any given circumstances.

Interpretations: Occasionally questions may arise regarding the meaning of portions of standards as they relate to specific applications. When the need for interpretations is brought to the attention of IEEE, the Institute will initiate action to prepare appropriate responses. Since IEEE Standards represent a consensus of concerned interests, it is important to ensure that any interpretation  has also received the concurrence of a balance of interests.For this reason, IEEE and the members of its societies and Standards Coordinating  Committees  are not able to provide an instant response to interpretation  requests except in those cases where the matter has previously received formal consideration. A statement, written or oral, that is not processed in accordance with the IEEE-SA Standards Board Operations Manual shall not be considered the official position of IEEE or any of its committees and shall not be considered to be, nor be relied upon as, a formal interpretation of the IEEE.At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position, explanation, or interpretation  of the IEEE. Comments for revision of IEEE Standards are welcome from any interested party, regardless of membership affiliation with IEEE. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Recommendations to change the status of a stabilized standard should include a rationale as to why a revision or withdrawal is required.

Comments and recommendations on standards, and requests for interpretations should be addressed to: Secretary, IEEE-SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854

USA

Authorization to photocopy portions of any individual standard for internal or personal use is granted by the Institute of Electrical and Electronics  Engineers, Inc., provided that the appropriate  fee is paid to Copyright Clearance  Center. To arrange for payment of licensing fee, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Introduction

This introduction is not part of IEEE Std 802.15.13-2011, IEEE Standard for Local and metropolitan area networks— Part 15.13: Multi Giga bit/sec Optical Wireless Communications

Visible-light communication (VLC) transmits data by intensity modulating optical sources, such as light- emitting diodes (LEDs) and laser diodes (LDs), faster than the persistence of the human eye. VLC merges lighting and data communications in applications such as area lighting, signboards, streetlights, vehicles, traffic signals, and status indicators. This standard describes the use of VLC for wireless personal area networks (WPAN) and covers topics such as network topologies, addressing, collision avoidance, acknowledgement, performance quality indication, dimming support, visibility support, colored status indication and color-stabilization.

Notice to users

Laws and regulations

Users of these documents should consult all applicable laws and regulations. Compliance with the provisions of this standard does not imply compliance to any applicable regulatory requirements. Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

This document is copyrighted by the IEEE. It is made available for a wide variety of both public and private uses. These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making this document available for use and adoption by public authorities and private users, the IEEE does not waive any rights in copyright to this document.

Updating of IEEE documents

Users of IEEE standards should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. An official IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect. In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit the IEEE Standards Association website at http:// ieeexplore.ieee.org/xpl/standards.jsp, or contact the IEEE at the address listed previously.

For more information about the IEEE Standards Association or the IEEE standards development process, visit the IEEE-SA website at http://standards.ieee.org.

Errata

Errata, if any, for this and all other standards can be accessed at the following URL: http:// standards.ieee.org/reading/ieee/updates/errata/index.html. Users  are  encouraged to  check  this  URL  for errata periodically.

Interpretations

Current interpretations can be accessed at the following URL:  http://standards.ieee.org/reading/ieee/interp/

index.html.

Patents

Attention is called to the possibility that implementation of this amendment may require use of subject matter covered by patent rights. By publication of this amendment, no position is taken with respect to the existence or validity of any patent rights in connection therewith. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non- discriminatory. Users of this amendment are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this amendment was submitted to the IEEE-SA for approval, the IEEE P802.15.13 Working

Group had the following membership:

Robert F. Heile, Chair

Rick Alfvin, Co-Vice Chair

Patrick W. Kinney, Co-Vice Chair and Secretary

James P. K. Gilb, Working Group Technical Editor

xxxx, IEEE 802.15.13 Chair

xxxx, IEEE 802.15.13 Vice Chairs

xxxx, IEEE 802.15.13 Technical Editors

TBD

Major technical contributions were received from the following individuals:

TBD

The following  members  of the individual  balloting  committee  voted on this standard.  Balloters  may have voted for approvaldisapprovalor abstβnt 	lOn

TBD

When the IEEE-SA Standards Board approved this standard on TBD, it had the following membership:

Richard H. Hulett, Chair John Kulick, Vice Chair Robert M. Grow, Past Chair Judith Gorman, Secretary

TBD

*Member Emeritus

Also included are the following nonvoting IEEE-SA Standards Board liaisons:

Satish Aggarwal, NRC Representative Richard DeBlasio, DOE Representative Michael Janezic, NIST Representative

Catherine Berger

IEEE Project Editor

Patricia Gerdon

IEEE Standards Program Manager, Technical Program Development

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1.Overview....................................................................................................................................................... 1 | 1 |
| --- | --- |
|  | 2 |
| 1.1 Scope............................................................................................................................................ 1 | 3 |
| 1.2 Purpose......................................................................................................................................... 1 | 4 |

5

2.Normative references .................................................................................................................................... 2	6

3.Definitions, acronyms, and abbreviations..................................................................................................... 2	7

8

3.1  Definitions ................................................................................................................................... 2	9

3.2  Acronyms and abbreviations ....................................................................................................... 3	10

11

4.General description ....................................................................................................................................... 6	12

13

4.1  Introduction.................................................................................................................................. 6	14

4.2  Network topologies ...................................................................................................................... 6	15

4.2.1	Peer-to-peer topology ......................................................................................................8	16

4.2.2	Star topology .................................................................................................................... 8	17

4.2.3	Broadcast topology .......................................................................................................... 9	18

4.2.4	Coordinated topology ...................................................................................................... 9	19

4.2.5	Relay functionality........................................................................................................... 9	20

4.2.6	Heterogeneous RF & OWC functionality........................................................................ 9	21

4.3  Modulation-domain spectrum .................................................................................................... 11	22

4.4  Architecture ............................................................................................................................... 11	23

4.4.1	PHY layer ...................................................................................................................... 12	24

4.4.2	MAC sublayer ................................................................................................................ 15	25

4.4.3	Dimming support ........................................................................................................... 15	26

4.4.4	Flicker mitigation support.............................................................................................. 17	27

4.5  Functional overview .................................................................................................................. 17	28

4.5.1	Superframe structure ...................................................................................................... 18	29

4.5.2	Data transfer model........................................................................................................ 18	30

4.5.3	Clock-rate selection ....................................................................................................... 20	31

4.5.4	Frame structure .............................................................................................................. 20	32

4.5.5	Improving probability of successful delivery ................................................................ 20	33

4.6  Security ...................................................................................................................................... 21	34

4.7  Concept of primitives................................................................................................................. 22	35

36

5.MAC protocol specification........................................................................................................................ 23	37

5.1  MAC functional description ...................................................................................................... 23	38

5.1.1	Superframe structure ...................................................................................................... 24	39

5.1.2	Channel access ............................................................................................................... 30	40

5.1.3	Starting a OWPAN ........................................................................................................ 39	41

5.1.4	Neighboring OWPAN status monitoring....................................................................... 43	42

5.1.5	Maintaining OWPANs................................................................................................... 43	43

5.1.6	Association and disassociation ...................................................................................... 47	44

5.1.7	Synchronization ............................................................................................................. 50	45

5.1.8	Transaction handling...................................................................................................... 51	46

5.1.9	Transmission, reception, and acknowledgment ............................................................. 52	47

5.1.10	GTS allocation and management ................................................................................... 59	48

5.1.11	Fast link recovery........................................................................................................... 64	49

5.1.12	OWC cell design and mobility support.......................................................................... 68	50

5.1.13	CSI feedback and link adaptation .................................................................................. 72	51

5.1.14	Relaying, ........................................................................................................................ 73	52

5.1.15	Interference coordination ............................................................................................... 77	53

54

Copyright © 20xx IEEE. All rights reserved.	0

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1	5.1.16	Mobility and handover ................................................................................................... 78

2	5.2  MAC frame formats ................................................................................................................... 81

3	5.2.1	General MAC frame format........................................................................................... 82

4	5.2.2	Format of individual frame types................................................................................... 85

5	5.3  MAC frame formats ................................................................................................................... 96

6	5.3.1	Conventions ................................................................................................................... 97

7	5.3.2	General frame format ..................................................................................................... 97

8	5.3.3	Frame fields ................................................................................................................... 98

9	5.3.4	Specific Frame Formats ............................................................................................... 101

10	5.4  MAC command frames............................................................................................................ 116

11	5.4.1	Association request command ..................................................................................... 117

12	5.4.2	Association response command ................................................................................... 117

13	5.4.3	Re-association request command................................................................................. 119

14	5.4.4	Disassociation notification command .......................................................................... 120

15	5.4.5	Data request command................................................................................................. 120

16	5.4.6	Scan-over-backhaul request command ........................................................................ 122

17	5.4.7	Scan-over-backhaul confirmation command ............................................................... 122

18	5.4.8	OWPAN ID conflict notification command ................................................................ 123

19	5.4.9	Neighboring OWPAN report request command.......................................................... 123

20	5.4.10	Neighboring OWPAN report indication command ..................................................... 123

21	5.4.11	Beacon request command ............................................................................................ 124

22	5.4.12	Additional beacon request command........................................................................... 125

23	5.4.13	Coordinator realignment command ............................................................................. 126

24	5.4.14	GTS request command................................................................................................. 127

25	5.4.15	Blinking notification command ................................................................................... 128

26	5.4.16	Fast link recovery command ........................................................................................ 129

27	5.4.17	Mobility notification command ................................................................................... 130

28	5.4.18	GTS response command .............................................................................................. 130

29	5.4.19	Clock rate change notification command .................................................................... 131

30	5.4.20	Multiple channel assignment command ...................................................................... 131

31	5.4.21	CVD disable command ................................................................................................ 132

32	5.4.22	CSI Control Frame ....................................................................................................... 132

33	5.4.23	Information element command .................................................................................... 134

34

35	6.MAC sublayer service specification ......................................................................................................... 142

36

37	6.1  Overview.................................................................................................................................. 142

38	6.2  MAC data service .................................................................................................................... 142

39	6.2.1	MCPS-DATA.request .................................................................................................. 143

40	6.2.2	MCPS-DATA.confirm................................................................................................. 146

41	6.2.3	MCPS-DATA.indication ............................................................................................. 147

42	6.2.4	MCPS-PURGE.request ................................................................................................ 150

43	6.2.5	MCPS-PURGE.confirm............................................................................................... 150

44	6.2.6	Data service message sequence chart .......................................................................... 151

45	6.2.7	Primitives for heterogeneous network ......................................................................... 151

46	6.3  MAC management service....................................................................................................... 155

47	6.3.1	Association primitives ................................................................................................. 156

48	6.3.2	Disassociation primitives ............................................................................................. 164

49	6.3.3	Beacon notification primitive ...................................................................................... 170

50	6.3.4	Additional beacon primitives ....................................................................................... 173

51	6.3.5	Primitives for reading PIB attributes ........................................................................... 174

52	6.3.6	GTS management primitives ....................................................................................... 176

53	6.3.7	Primitives for resetting the MAC sublayer .................................................................. 181

54	6.3.8	Primitives for specifying the receiver enable time ...................................................... 183

1	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.9	Primitives for channel scanning................................................................................... 186	1

6.3.10	Neighbor report primitives........................................................................................... 190	2

6.3.11	Communication status primitive .................................................................................. 194	3

6.3.12	Primitive for heterogeneous RF & OWC functionality ............................................... 197	4

6.3.13	Primitives for writing PIB attributes............................................................................ 199	5

6.3.14	Primitives for updating the superframe configuration ................................................. 201	6

6.3.15	Primitive for synchronizing with a coordinator ........................................................... 206	7

6.3.16	Primitive for synchronization loss with a coordinator ................................................. 207	8

6.3.17	Primitives for requesting data from a coordinator ....................................................... 211	9

6.4  MAC constants and PIB attributes........................................................................................... 214	10

6.4.1	MAC constants ............................................................................................................ 214	11

6.4.2	MAC PIB attributes ..................................................................................................... 214	12

6.5  Optical-clock-rate selection ..................................................................................................... 225	13

6.5.1	Optical-clock-rate selection for P2P topology ............................................................. 225	14

6.5.2	Optical-clock-rate selection for star topology ............................................................. 228	15

6.5.3	Clock-rate selection for multicast topology ................................................................. 230	16

6.6  Message sequence charts illustrating MAC-PHY interaction ................................................. 232	17

18

7.Security suite specifications...................................................................................................................... 240	19

20

7.1  Overview.................................................................................................................................. 240	21

7.2  Functional description.............................................................................................................. 241	22

7.2.1	Outgoing frame security procedure ............................................................................. 241	23

7.2.2	Outgoing frame key retrieval procedure ...................................................................... 242	24

7.2.3	Incoming frame security procedure ............................................................................. 243	25

7.2.4	Incoming frame security material retrieval procedure................................................. 244	26

7.2.5	Key descriptor lookup procedure................................................................................. 246	27

7.2.6	Blacklist checking procedure ....................................................................................... 246	28

7.2.7	Device descriptor lookup procedure ............................................................................ 246	29

7.2.8	Incoming security level checking procedure ............................................................... 247	30

7.2.9	Incoming key usage policy checking procedure .......................................................... 247	31

7.3  Security operations .................................................................................................................. 247	32

7.3.1	Integer and octet representation ................................................................................... 248	33

7.3.2	CCM* nonce ................................................................................................................ 248	34

7.3.3	CCM* prerequisites ..................................................................................................... 248	35

7.3.4	CCM* transformation data representation................................................................... 248	36

7.3.5	CCM* inverse transformation data representation ...................................................... 250	37

7.4  Auxiliary Security header ........................................................................................................ 251	38

7.4.1	Integer and octet representation ................................................................................... 251	39

7.4.2	Security Control field................................................................................................... 251	40

7.4.3	Frame Counter field ..................................................................................................... 253	41

7.4.4	Key Identifier field....................................................................................................... 253	42

7.5  Security-related MAC PIB attributes....................................................................................... 254	43

7.5.1	PIB security material ................................................................................................... 254	44

7.5.2	Key table ...................................................................................................................... 257	45

7.5.3	Device table ................................................................................................................. 258	46

7.5.4	Minimum security level table ...................................................................................... 258	47

7.5.5	Frame counter .............................................................................................................. 258	48

7.5.6	Automatic request attributes ........................................................................................ 258	49

7.5.7	Default key source ....................................................................................................... 258	50

7.5.8	Coordinator address ..................................................................................................... 259	51

8.PHY layer specification ............................................................................................................................ 259	52

53

54

Copyright © 20xx IEEE. All rights reserved.	2

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 1 | 8.1 | Overview.................................................................................................................................. 259 |  |
| --- | --- | --- | --- |
| 2 | 8.2 | Operating modes ...................................................................................................................... 259 |  |
| 3 | 8.3 | General requirements ............................................................................................................... 263 |  |
| 4 | 8.3.1 |  | Optical mapping........................................................................................................... 263 |
| 5 | 8.3.2 |  | Maximum error tolerance for multiple optical sources ............................................... 263 |
| 6 | 8.3.3 |  | TX-to-RX turnaround time .......................................................................................... 264 |
| 7 | 8.3.4 |  | RX-to-TX turnaround time .......................................................................................... 264 |
| 8 | 8.3.5 |  | Transmit data clock frequency tolerance ..................................................................... 264 |
| 9 | 8.3.6 |  | Clear channel assessment (CCA)................................................................................. 264 |
| 10 | 8.4 | Data modes .............................................................................................................................. 265 |  |
| 11 | 8.5 | Flicker mitigation..................................................................................................................... 266 |  |
| 12 | 8.6 | PPDU format............................................................................................................................ 267 |  |

13	8.6.1	Preamble field .............................................................................................................. 267

14	8.6.2	PHY header .................................................................................................................. 269

15	8.6.3	Header check sequence (HCS) .................................................................................... 271

16	8.6.4	Optional fields.............................................................................................................. 271

17	8.6.5	PSDU field ................................................................................................................... 272

18	8.7  PPDU Format........................................................................................................................... 273

19	8.7.1	PPDU format for low bandwidth PHY ........................................................................ 273

20	8.7.2	PPDU Format for high bandwidth PHY ...................................................................... 283

21

22	9.PHY service specifications ....................................................................................................................... 289

23

24	9.1  Overview.................................................................................................................................. 289

25	9.2  PHY management service........................................................................................................ 289

26	9.2.1	PLME-CCA.request..................................................................................................... 290

27	9.2.2	PLME-CCA.confirm.................................................................................................... 290

28	9.2.3	PLME-GET.request ..................................................................................................... 291

29	9.2.4	PLME-GET.confirm .................................................................................................... 291

30	9.2.5	PLME-SET.request ...................................................................................................... 292

31	9.2.6	PLME-SET.confirm..................................................................................................... 293

32	9.2.7	PLME-SET-TRX-STATE.request............................................................................... 294

33	9.2.8	PLME-SET-TRX-STATE.confirm.............................................................................. 294

34	9.2.9	PLME-SWITCH.request.............................................................................................. 295

35	9.2.10	PLME-SWITCH.confirm ............................................................................................ 296

36	9.3  PHY data service ..................................................................................................................... 296

37	9.3.1	PD-DATA.request ....................................................................................................... 296

38	9.3.2	PD-DATA.confirm ...................................................................................................... 297

39	9.3.3	PD-DATA.indication ................................................................................................... 298

40	9.4  PHY enumeration description.................................................................................................. 302

41	9.5  PHY constants and PIB attributes............................................................................................ 302

42	9.5.1	PHY constants.............................................................................................................. 303

43	9.5.2	PHY PIB attributes ...................................................................................................... 303

44	10.PHY II specifications .............................................................................................................................. 306

45

46	10.1 Reference modulator diagram.................................................................................................. 306

47	10.2 Forward error correction encoder ............................................................................................ 306

48	10.3 Run-length limiting encoder .................................................................................................... 307

49	10.4 Data mapping for VPPM ......................................................................................................... 307

50

51	11.PHY VII Low-bandwidth PHY .............................................................................................................. 307

52

53	11.1 Adaptive OFDM Concept ........................................................................................................ 307

54	11.2 Waveform ................................................................................................................................ 308

3	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

11.2.1	Adaptive OFDM signal generation .............................................................................. 308	1

11.2.2	Carrier mapping ........................................................................................................... 308	2

11.2.3	IDFT............................................................................................................................. 309	3

11.2.4	Cyclic prefix ................................................................................................................ 309	4

11.2.5	Subcarrier modulation mapping................................................................................... 310	5

11.2.6	Single-carrier modulation ............................................................................................ 311	6

11.2.7	Unipolar modulation .................................................................................................... 313	7

11.3 MIMO ...................................................................................................................................... 318	8

11.3.1	Modified frame structure ............................................................................................. 318	9

11.3.2	Reference symbols for MIMO channel estimation ...................................................... 319	10

11.3.3	MIMO transmission modes ......................................................................................... 321	11

11.4 Channel coding ........................................................................................................................ 322	12

11.4.1	Channel coding for the header ..................................................................................... 323	13

11.4.2	Channel coding for the data ......................................................................................... 323	14

11.4.3	Data interleaving .......................................................................................................... 325	15

11.5 Relaying ................................................................................................................................... 326	16

11.5.1	Amplify and forward ................................................................................................... 326	17

11.5.2	Decode and forward ..................................................................................................... 327	18

11.6 PHY Service Specifications ..................................................................................................... 329	19

11.6.1	Overview...................................................................................................................... 329	20

11.6.2	PHY Management Service........................................................................................... 329	21

22

12.PHY VIII High-bandwidth PHY ............................................................................................................ 336	23

24

12.1 Adaptive OFDM ...................................................................................................................... 336	25

12.1.1	Adaptive OFDM Waveform ........................................................................................ 337	26

12.1.2	Single-carrier modulation ............................................................................................ 338	27

12.1.3	Unipolar modulation .................................................................................................... 342	28

12.1.4	Multiple-input multiple-output .................................................................................... 345	29

12.1.5	Coordinated wireless network ..................................................................................... 351	30

12.1.6	Relaying ....................................................................................................................... 352	31

12.1.7	Payload data ................................................................................................................. 352	32

12.1.8	Data scrambling ........................................................................................................... 352	33

34

Annex A ...................................................................................................................................................... 355	35

36

A.1	General ............................................................................................................................. 355	37

A.2	Regulatory documents ..................................................................................................... 356	38

39

Annex B ...................................................................................................................................................... 359	40

41

B.1	IEEE 802.2 convergence sublayer ................................................................................... 359	42

B.1.1   MA-UNITDATA.request ................................................................................................ 359	43

B.1.2   MA-UNITDATA.indication ............................................................................................ 360	44

B.1.3   MA-UNITDATA-STATUS.indication............................................................................ 361	45

46

Annex C ...................................................................................................................................................... 363	47

48

Annex D ...................................................................................................................................................... 364	49

50

D.1	Introduction—Dynamic displays vs. addressed displays ................................................ 364	51

D.2	Dynamic displays............................................................................................................. 364	52

D.2.1   Operation mechanism ...................................................................................................... 364	53

D.2.2   Reduced brightness mitigation on OWC dynamic displays ............................................ 365	54

Copyright © 20xx IEEE. All rights reserved.	4

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1	D.2.3   OWC application using dynamic displays ....................................................................... 366

2	D.3	Addressed displays .......................................................................................................... 367

3	D.3.1   LCD display using LED backlighting modulation .......................................................... 367

4	D.3.2   LED pixel modulation ..................................................................................................... 367

5

6	Annex E ...................................................................................................................................................... 369

7

8	Annex F ....................................................................................................................................................... 373

9

10	Annex G ...................................................................................................................................................... 374

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

5	Copyright © 20xx IEEE. All rights reserved.

1

2

3

4

5

6

7

8

9

10

11

IEEE S tandard fo r	12

13

Lo cal and me tro po litan area ne two rks —	14

15

16

18

Part 15.13: Multi Gig a bit/s e c Optic al	17

20

Wire le s s Co mmunic atio ns	19

21

IMPORTANT NOTICE: This standard is not intended to ensure safety, security, health, or environmental protection.	22

Implementers of the standard are responsible for determining appropriate safety, security, environmental, and health	23

practices or regulatory requirements.	24

25

This IEEE document is made available for use subject to important notices and legal disclaimers. These notices and	26

disclaimers appear in all publications containing this document and may be found under the heading “Important	27

Notice” or “Important Notices and Disclaimers Concerning IEEE Documents.” They can also be obtained on request

from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.	28

29

30

1. Overview	31

32

33

1.1 Scope	34

35

This standard defines a PHY and MAC layer for short-range optical wireless communications in optically              36 transparent media using the light wavelengths using the light wavelength from 10,000 nm to 190 nm in              37 wavelength. The  standard  is  capable  of  delivering  data  rates  sufficient  to  support  audio  and  video              38 multimedia services and also considers mobility of the optical link, compatibility with variable light             39 infrastructures, impairments due to noise and interference from sources like ambient light and a MAC layer              40 that accommodates visible links as well as the other targeted light wavelengths.   It also accommodates              41 optical communications for cameras where transmitting devices incorporate light emitting sourses and              42 receivers are digital cameras with a lens and image sensor. The standard adheres to applicable eye safety              43 regulations.                                                                                                                                                                        44

45

1.2 Purpose	46

47

This standard provides a global standard for short-range optical wireless communication. The standard              48 provides (i) access to several hundred THz of unlicensed spectrum; (ii) immunity to electromagnetic              49 interference and  noninterference with  Radio  Frequency (RF)  systems;  (iii)  for  optical  light  systems,              50 additional security by  allowing the user to  see  the  communication channel; and  (iv)  communication              51 augmenting and complementing existing services (such as illumination, display, indication, decoration, etc.).              52

53

54

Copyright © 2011 IEEE. All rights reserved.	1

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	2. Normative references

2

3	The following referenced documents are indispensable for the application of this document (i.e., they must

4	be understood and used, so each referenced document is cited in text and its relationship to this document is

5	explained). For dated references, only the edition cited applies. For undated references, the latest edition of

6	the referenced document (including any amendments or corrigenda) applies.

7

8	ANSI/INCITS 373: Fiber Channel Framing and Signaling Interface (FC-FS).1

9

10	IEEE  Std  802.15.4TM-2006,  IEEE  Standard  for  Information  technology—Telecommunications  and

11	information exchange between systems—Local and metropolitan area networks—Specific requirements—

12	Part 15.4: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Low-Rate

13	Wireless Personal Area Networks (LR-WPANs).2, 3

14

15	ITU-T  I.432.1,  Series  I:  Integrated  Services  Digital  Network,  ISDN  user-network  interfaces—Layer

16	1RecommendationsB-ISDN user-network interface—Physical layer specification: General characteristics,

17	http://www.itu.int/rec/T-REC-I.432.1-199902-I/en.4

18

19	IEEE  Std  802.15.7TM-2011,  IEEE  Standard  for  Information  technology—Telecommunications  and

20	information exchange between systems—Local and metropolitan area networks—Specific requirements—

21	Part 15.7: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Short-

22	Range Wireless Optical Communication Using Visible Light.5

23

24

25	3. Definitions, acronyms, and abbreviations

26

27

28	3.1 Definitions

29

30	For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictio-

31	nary: Glossary of Terms & Definitions should be consulted for terms not defined in this clause.6

32

33	baseband bandwidth: The bandwidth as measured in the electrical domain at the LED driver.

34

35	color function: A function that provides information, such as device status and channel quality, to the

36	human eye via color.

37

38	color visibility dimming (CVD) frame: A frame used for color, visibility and dimming support. The color

39	visibility dimming frame visually provides information such as communication status and channel quality to

40	the user via various colors. The color visibility dimming frame may also be sent during idle or receive

41	modes of operation for continuous visibility and dimming support. During the color visibility dimming

42	frame transmission, the device is still emitting light while not communicating, and it is thus able to fulfill its

43	lighting function. The payload of the frame consists of visibility patterns of appropriate intensity and color.

44

45

46	1ANSI publications are available from the Sales Department, American National Standards Institute, 25 West 43rd Street, 4th Floor,

47	New York, NY 10036, USA (http://www.ansi.org/).

48	2IEEE publications are available from the Institute of Electrical and Electronics Engineers, Inc., 445 Hoes Lane, Piscataway, NJ 08854,

49	USA (http://standards.ieee.org/).

50	3The IEEE standards or products referred to in this clause are trademarks of the Institute of Electrical and Electronics Engineers, Inc.

51	4ITU-T publications are available from the International Telecommunications Union, Place des Nations, CH-1211, Geneva 20, Switzer- land/Suisse (http://www.itu.int/).

52	5IEEE publications are available from the Institute of Electrical and Electronics Engineers, Inc., 445 Hoes Lane, Piscataway, NJ 08854,

53	USA (http://standards.ieee.org/).

54	6The IEEE Standards Dictionary: Glossary of Terms & Definitions is available at http://shop.ieee.org/.

2	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

compensation time: The idle time inserted in the idle pattern or in the data frame, where the light is turned	1 “ON” or “OFF” with the appropriate ratio to meet dimming requirements.	2

3

dimming: Reducing the radiant power of a transmitter while preserving the color of the transmitted light.	4

5

field of view (FOV): The angular extent of coverage for the optical transmitter or receiver.	6

7 idle pattern: A pattern whose duty cycle variation results in a change of brightness for dimming support and              8 may be transmitted during idle or receive mode.                                                                                                              9

10                                                                                                                                              12

13 modulation-domain spectrum: The spectrum observed at the output of the receiver’s photodetector;              14 typically measured at the output of the trans impedance amplifier                                                                                  15

16 on-off keying (OOK): A simple modulation technique that represents digital data as the presence (‘ON’) or              17 absence (‘OFF’) of a signal. Note that ‘ON’ and ‘OFF’ are simply two logic levels or two distinct amplitude              18 levels for the purposes of communication and does not necessarily require that the light source be turned              19

OFF completely.	20

21

optical clock rate: The frequency at which the data is clocked out to the optical source.	22

23 photodetector: A photodetector captures optical power and translates it into an output signal. Most             24 photodetectors convert optical power into an electrical current or an electrical voltage.                                                 25

26

PHY switch: A switch at the transmission interface between the PHY and the optical SAP, used to send and	27 receive data to and from a single or multiple optical sources and photodetectors in a selective manner.	28

29                                                                                                        31

32 switching level: A distinct amplitude level that defines ‘ON’ and ‘OFF’ of the light source for the purpose              33 of communications and does not necessarily require that the light source be turned off completely.                             34

35 variable pulse-position modulation (VPPM): A modulation scheme for visible-light communication that              36 allows pulse-width control for light dimming support, mitigating intra-frame flicker.                                                    37

38

visibility pattern: An in-band idle pattern used in the payload of a color visibility dimming frame.	39

40

OWPAN cluster: OWPANs managed by the same master coordinator (MC).	41

42

Heterogeneous network: a network functionality where data transmission is done over a combination of	43 optical and RF wireless links.	44

45

3.2 Acronyms and abbreviations	46

47

48

A/D	analog-to-digital converter	49

ACK	acknowledgment	50

AES	advanced encryption standard	51

AR	acknowledgment request	52

BE	backoff exponent	53

BI	beacon interval	54

Copyright © 20xx IEEE. All rights reserved.	3

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	BO	beacon order

2	BP	beacon period

3	BSN	beacon-sequence number

4	CAP	contention access period

5	CC	convolutional code

6	CCA	clear channel assessment

7	CDR	clock and data recovery

8	CFP	contention-free period

9	CIE	Commission Internationale de l'Eclairage (International Commission on Illumination)

10	CRC	cyclic redundancy check

11	CSK	color-shift keying

12	CSMA/CA	carrier sense multiple access with collision avoidance

13	CVD	color visibility dimming

14	D/A	digital-to-analog converter

15	D/L	downlink

16	DC	direct current

17	DME	device management entity

18	DSN	data-sequence number

19	ED	energy detection

20	ENC	encryption mode

21	FCS	frame check sequence

22	FDM	frequency division multiplexing

23	FEC	forward error correction

24	FER	frame-error ratio

25	FLP	fast locking pattern

26	FLR	fast link recovery

27	FOV	field of view

28	GF	Galois field

29	GTS	guaranteed time slot

30	HCS	header-check sequence

31

32	IFS	interframe space

33	ID	identifier

34	IE	information element

35	LD	laser diode

36	LED	light-emitting diode

37	LIFS	long interframe space

38	LLC	logical link control

39	LPDU	logical link control protocol data unit

40	LOS	line of sight

41	MAC	medium access control

42	MC	master coordinator

43	MCPS	medium-access-control common-part sublayer

44	MCS	modulation and coding scheme

45	MD	mobile device

46	MFB	MCS feedback

47	MFR	medium-access-control footer

48	MFTP	maximum flickering-time period

49	MHR	medium-access-control header

50	MIC	message-integrity code

51	MLME	medium-access-control link-management entity

52	MPDU	medium-access-control protocol-data unit

53	MSDU	medium-access-control service-data unit

54	MSI	MCS sequence index

4	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

NB	number of backoffs	1

OOK	on-off keying	2

OWC	Optical Wireless Communications	3

OWPAN	Optical Wireless Personal Area Network	4

PAN	personal-area network	5

PD	physical-layer data	6

PHR	physical-layer header	7

PHY	physical layer	8

PIB	physical-layer personal-area-network information base	9

PID	personal-area-network identifier	10

PLME	physical-layer management entity	11

PPDU	physical-layer data unit	12

PPM	pulse position modulation	13

PSDU	PHY service data unit	14

PWM	pulse-width modulation	15

P2MP	point-to-multipoint	16

P2P	peer-to-peer	17

QoS	quality of service	18

RIFS	reduced interframe space	19

RLL	run-length limited	20

ROI	Region of Interest	21

RS	Reed-Solomon	22

RX	receiver	23

SAP	service access point	24

SC	spreading code	25

SHR	synchronization header	26

SIFS	short interframe space	27

SPDU	session-protocol data unit	28

SO	superframe order	29

SS	Spread Spectrum	30

SSCS	service-specific convergence sublayer	31

TDP	topology dependent pattern	32

TRX	transceiver	33

TX	transmitter	34

U/L	uplink	35

OWPAN	Optical wireless personal area network	36

VLC	visible-light communication	37

VPPM	variable pulse-position modulation	38

WPAN	wireless personal area network	39

WQI	wavelength quality indication	40

FSK	Frequency Shift Keying	41

FL	Frequency Labeling	42

DS	Data Symbol	43

MCU	MicroController Unit	44

FLR	Frequency Labeling Ratio	45

DF	Data Frame	46

DSF	Data Sub-Frame	47

SS	Splitter Symbol	48

PF	Preamble Frequency	49

SF	Synchronization Frame	50

IF	Inactive Frame	51

SEQ	Sequence Number	52

DP	Data Packet	53

FP	FEC Packet	54

Copyright © 20xx IEEE. All rights reserved.	5

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	MFH	MAC Frame Header

2	MFT	MAC Frame Tail

3	RS-FSK	Rolling Shutter - Frequency Shift Keying

4

5

6	4. General description

7

8

9	4.1 Introduction

10

11	Visible-light communication (OWC) transmits data by intensity modulating optical sources, such as light-

12	emitting diodes (LEDs) and laser diodes (LDs), faster than the persistence of the human eye. OWC merges

13	lighting and data communications in applications such as area lighting, signboards, streetlights, vehicles,

14	and traffic signals. This standard describes the use of OWC for wireless personal area networks (WPAN).

15	Some of the characteristics found in this standard are as follows:

16

17	a)	Star, peer-to-peer, or broadcast operation

18	b)	16-bit short or 64-bit extended addresses

19	c)	Scheduled or slotted random access with collision avoidance transmission

20	d)	Fully acknowledged protocol for transfer reliability

21	e)	Wavelength quality indication (WQI)

22	f)	Dimming support (editor note: unresolved comment on whether to keep dimming support here, see

23	https://mentor.ieee.org/802.15/dcn/17/15-17-0479-03-0013-combined-comments-against-d0.xlsx,

24	comment 29)

25	g)	Visibility support

26	h)	Color function support

27

28	4.2 Network topologies

29

30	As shown in Table 1, three classes of devices are considered for OWC: infrastructure, mobile, and vehicle.

31

32

33	Table  1—De vice  c las s ific ation

34

|  | Infrastructure | Mobile | Vehicle |
| --- | --- | --- | --- |
| Fixed coordinator | Yes | No | No |
| Power supply | Ample | Limited | Moderate |
| Form factor | Unconstrained | Constrained | Unconstrained |
| Light source | Intense | Weak | Intense |
| Physical mobility | No | Yes | Yes |
| Range | Short/long | Short | Long |
| Data rates | High/low | High | Low |

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49	The  IEEE  802.15.13  optical  wireless  personal  area  network  (OWPAN)  standard  maps  the  intended

50	applications  to  four  topologies:  peer-to-peer,  star,  broadcast  and  coordinated,  as  shown  in  Figure 1.

51	Moreover, two advanced network functionalities are supported, relaying and heterogeneous networking of

52	OWC and RF, as shown in Figure 2.

53

54

6	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

In the star topology, the communication is established between devices and a single central controller, called              1 the coordinator. In the peer-to-peer topology, one of the two devices in an association takes on the role of the              2 coordinator.  In  the  coordinated  topology,  multiple  devices  communicate  with  multiple  coordinators,              3 supervised by a master coordinator. The master coordinator is connected to each coordinator via backhaul              4 link. Note that the functionality of the master coordinator is not part of this standard. In addition, two              5 advanced network functionalities may be enabled: relaying and heterogeneous RF-OWC. With the relaying              6 functionality, an intermediate relay node is introduced between the coordinator and the device. With the              7 heterogeneous RF-OWC functionality, data transmission over the optical wireless link can be combined with              8 a parallel radio-based wireless link.                                                                                                                                  9

10

Each device, coordinator or relay node has a unique 64-bit address. When a device associates with a              11 coordinator or relay node it is allowed to be allocated a short 16-bit address. Either address is allowed to be              12 used for communication within the OWPAN managed by the coordinator, the relay node or the master              13 coordinator. The coordinator, the relay node and the master coordinator might often be mains powered,              14 while the devices will often be battery powered.                                                                                                              15

16

17

Peer to peer 	Star 	18

19

20

21

22

23

24

Visibility	25

Broadcast

Master

Coordinated

support across

all topologies 	26

27

28

29

30

31

32

33

34

35

36

coordinator 	Coordinator 	Device 	37

Figure 1—Supported MAC topologies	38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	7

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

Relaying 	Heterogeneous RF & OWC

4

5

6

7

8

9	RF Link

10

11

12

13

14

15	Master

16	coordinator 	Coordinator

17

Relay

Device

18	OWC link 	RF link

19	Figure 2—Advanced network functionalities

20

21

22

23	Each independent OWPAN has an identifier, as defined in 5.2.1.3 and 5.2.1.5. This OWPAN identifier

24	allows communication between devices within a network using short addresses. The mechanism by which

25	OWPAN identifiers are chosen is outside the scope of this standard.

26

27	The network formation is performed by the higher layer, which is not part of this standard. Apart from the

28	peer-to-peer and star topologies, IEEE 802.15.13 devices are also allowed to operate in a broadcast-only

29	topology without being part of a network, i.e., without being associated to any device or having any devices

30	associated to them. Moreover, the devices are allowed to operate in the coordinated topology, where a

31	master coordinator is introduced to perform higher layer functions such as handover between adjacent

32	coordinators and interference management. In addition, IEEE 802.15.13 devices are allowed to operate with

33	relay network functionality, where an additional relay is used between the device and the coordinator.

34	Finally, the devices are allowed to operate with heterogeneous RF-OWC network functionality, where an

35	additional  bidirectional  radio  link  is  introduced,  where  the  optical  link  is  either  unidirectional  or

36	bidirectional. A brief overview on how each supported topology may be formed is provided in 4.2.1, 4.2.2,

37	4.2.3, 4.2.4, 4.2.5 and 4.2.6.

38

39	In case illumination function is required, visibility support is also provided in the absence of communication

40	or in the idle or receive modes of operation. The purpose of this mode is to maintain illumination and

41	mitigate flicker.

42

43	4.2.1 Peer-to-peer topology

44

45	The basic structure of a peer-to-peer topology is illustrated in Figure 1. In a peer-to-peer topology, each

46	device is capable of communicating with any other device within its coverage area. In a peer-to-peer

47	topology, one of the peers acts as a coordinator. One peer defaults as the coordinator, for instance, by virtue

48	of being the first device to communicate on the channel.

49

50	4.2.2 Star topology

51

52	The basic structure of a star topology is illustrated in Figure 1. All star networks operate independently from

53	all other star networks currently in operation. This is achieved by choosing a OWPAN identifier that is not

54	currently used by any other network within the coverage area. Once the OWPAN identifier is chosen, the

8	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

coordinator allows other devices to join its network. The higher layer is allowed to use the procedures              1 described in 5.1.3 and 5.1.6 to form a star network.                                                                                                         2

3

4.2.3 Broadcast topology                                                                                                                             4

5

The basic structure of a broadcast topology is illustrated in Figure 1. The device in a broadcast mode can              6 transmit a signal to other devices without forming a network. The communication is uni-directional and the              7 destination address is not required.                                                                                                                                   8

9

4.2.4 Coordinated topology                                                                                                                         10

11

The basic structure of a coordinated topology is illustrated in Figure 1. In the coordinated topology, multiple              12 coordinators are connected to each other and to a master coordinator through backhaul network. The              13 backhaul is out of the scope of this specification. Multiple OWPANs are coordinated by the master              14 coordinator. The master coordinator may be in charge of various kinds of coordination among the multiple              15

OWPANs, e.g. handover, interference management, OWPAN status monitoring, etc.                                                    16

17

Note that there is a need for synchronization between multiple coordinators through the master coordinator              18 that can be achieved over the fronthaul network. How to achieve such synchronization is out of scope for              19 this standard 7.                                                                                                                                                                   20

21

4.2.5 Relay functionality                                                                                                                               22

23

With the relay functionality, an intermediate relay is used to assist a transmission via a direct optical wireless              24 link. With the relay functionality, each relay supports different duplexing and relay modes. For full duplex              25 (FD), the relay receives and transmits data simultaneously, while in half duplex (HD), the relay receives the              26 data in one time slot and retransmits it in another transmission slot. The relay supports two modes; amplify-              27 and-forward (AF), and decode-and-forward (DF).                                                                                                           28

29 a)	In AF mode, the RD receives the data from the coordinator, which are then retransmitted after		30 amplification.	31

b)	In DF mode, the received data is decoded by the relay and then retransmitted to the destination	32 device.	33

34

In case the device is disconnected from the coordinator, a relay search request is conducted, including the              35 relay capabilities. The coordinator broadcasts a relay search request frame. Each relay replies back on the              36 control channel with its own capabilities including duplexing and relaying modes. The coordinator selects              37 the relay that provides the best connectivity. The coordinator sets up a relay link between itself and the              38 device through the selected relay. A connection remains active until the direct link between the coordinator              39 and the device is reinitiated and the coordinator requires a termination of the link between the coordinator              40 and the relay.                                                                                                                                                                     41

42

4.2.6 Heterogeneous RF & OWC functionality                                                                                           43

44

The IEEE 802.15.13 specification supports heterogeneous RF-OWC functionality. The functionality may be              45 used in conjunction with different network topologies as demonstrated in Figure 3. The RF AP may be co-              46 located with a coordinator or a master coordinator. Each coordinator provides optical wireless links to the              47 devices while the RF AP provides a parallel RF link.                                                                                                       48

49

50

51

7There are techniques for both time and frequency synchronization. Time synchronization can be achieved e.g. by the precision time                 52 protocol (PTP) defined in in the IEEE Std. 1588-2008. Frequency synchronization can be achieved e.g. by the Synchronous Ethernet                 53 protocol (SynchE) defined in three ITU-T recommendations G.8261, G.8262 and G.8264.                                                                                          54

Copyright © 20xx IEEE. All rights reserved.	9

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

Broadcast

9

10

11

12

13

14

15

16

17

18

Coordinated

MC with co- located RF node

19		Master coordinator

20

Coordinator

Relay

Device

21	OWC link	RF link

22	Figure 3—Heterogeneous RF-OWC functionality

23

24

25

26	As shown in Table 2, three types of devices according to the capabilities in supporting OWC and RF are

27	considered for IEEE 802.15.13. Type 1 devices support OWC only operations. Type 2 devices support OWC

28	downlink operations as well as RF bidirectional operations. Type 3 devices support OWC bidirectional

29	operations as well as RF bidirectional operations.

30

31	Type 2 and type 3 devices can simultaneously operate via OWC and RF. In the downlink, aggregated

32	transmission through both OWC link and RF link may be used, besides switching between OWC link and

33	RF link. For the coordinated topology, command frames or ACK frames with destination address set to the

34	coordinator may be first transmitted to the master coordinator through RF link and then forwarded to the

35	coordinator through the backhaul link. Specification of these networking functionalities is out of scope in

36	802.15.13.

37

38

39

40

41	Table 2— De vic e c las s ific atio n ac c o rding to s uppo rte d RF c apabilitie s

| Device Type | RF Down Link | RF Uplink | OWC Downlink | OWC Uplink |
| --- | --- | --- | --- | --- |
| Type 1 |  |  | x | x |
| Type 2 | x | x | x |  |
| Type 3 | x | x | x | x |

42

43

44

45

46

47

48

49

50

51

52

53

54

10	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

4.3 Modulation-domain spectrum	1

2

Figure 4 illustrates the concept of the modulation-domain spectrum. In Figure 4, the optical light source is              3 on all the time; hence, the output of the photodetector can be observed for performing clear channel              4 assessment (CCA). Prior to time t = T1, the spectrum is all at DC. After t = T1, the spectrum is split between              5

DC and the modulating signal.	6

7

8

9

m odulation-domain

m odulation-domain 	10

amplitude

spectrum

T1

t <  T1

amplitude

spectrum 	11

T1

t > T1	12

13

14

15

0	frequency

voltage 	voltage

0 	Fm   frequency 	16

17

18

19

+V

0

T1

t = T1

time

+V

0

T1

t = T1

20

21

time 	22

23

visible light source

24

photodetector 	25

26

4.4 Architecture

Figure 4—Illustration of modulation-domain spectrum	27

28

29

30

31

The IEEE 802.15.13 architecture is defined in terms of a number of layers and sublayers in order to simplify	32 the standard. Each layer is responsible for one part of the standard and offers services to the higher layers.	33

The interface between the layers serve to define the logical links that are described in this standard.	34

35

A OWPAN device comprises of a PHY layer, which contains the light transceiver along with its low-level              36 control mechanism, and a medium access control (MAC) sublayer that provides access to the physical              37 channel for all types of transfers. Figure 5 shows these layers in a graphical representation, which are              38 described in more detail in 4.4.1 and 4.4.2.                                                                                                                      39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	11

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7	LLC

8

9	SSCS

10

11

upper layers

device management entity

dimmer

12	MCPS-SAP 		MLME-SAP MAC

13

14

15

PD-SAP 	PLME-SAP

16

17	PHY

18

19	PHY switch

20

21

22	OPTICAL-SAP

23	optical media

(DME)

24

25	cell #1

26

cell #2

...

cell

#n-1

cell #n

27	Figure 5—OWPAN device architecture

28

29

30	The upper layers, shown in Figure 5, consist of a network layer, which provides network configuration,

31	manipulation, and message routing, and an application layer, which provides the intended function of the

32	device. The definition of these upper layers is outside the scope of this standard.

35

36

47

48	4.4.1 PHY layer

49

50	The PHY layer supports multiple PHY types.

51

52	a)	PHY II: This PHY type is intended for indoor usage with moderate data rate applications. This mode

53	uses OOK and VPPM with data rates in the tens of Mb/s, as defined in Table 106.

54

12	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

b)	PHY VII: This PHY is intended for low rate LiFi applications with data rates in the tens of Mb/s	1 using OFDM modulation.	2

c)	PHY VIII: This PHY is intended for high rate LiFi applications with data rates in the hundreds of	3

Mb/s using OFDM modulation.	4

5

4.4.1.1 PHY frame structure	6

7

The MAC protocol data unit (MPDU) at the output of the MAC sublayer passes through the PHY layer and	8

becomes the PHY service data unit (PSDU) at the output of the PHY layer after being processed via the	9

various PHY blocks such as channel coding and line coding. The PSDU is prefixed with a synchronization	10

header (SHR), containing the preamble sequence field; and a PHY header (PHR), which, among other	11

things, contains the length of the PSDU in octets. The preamble sequence enables the receiver to achieve	12

synchronization. The SHR, PHR, and PSDU together form the PHY frame or PHY layer data unit (PPDU).	13

The format of the PHY frame is shown in Figure 137.	14

15

4.4.1.2 Interoperability and coexistence between PHY types	16

17

Editor note: interoperability and coexistence between PHY types is FFS.	18

19

4.4.1.3 Pulsed modulation PHY introduction	20

21

Editor note: TBD	22

23

4.4.1.4 Low bandwidth OFDM PHY introduction	24

25

Editor note: TBD	26

27

4.4.1.5 High bandwidth OFDM PHY introduction	28

29

The high-bandwidth PHY extends the capabilities and improves the transmission performance in order to	30

address the specific requirements of new use cases in scenarios B1-B4 mentioned in the Technical Consider-	31

ations Document (TCD) for 802.15.7r1 [8], such as optical wireless access in indoor/home/office, industrial	32

wireless (with specific requirements for robustness, low latency and secure data transmission), communica-	33

tions between vehicles and vehicle-to-the-roadside-infrastructure communications, and as a wireless back-	34

haul technology.	35

36

The high-bandwidth PHY supports fixed wireless links and multiple mobile user links via an OWC infra-	37

structure, which consists of one or more optical wireless access points. The PHY is wavelength-agnostic and	38

extends the optical wavelength range beyond the scope of 802.15.13 standard for Optical Wireless Commu-	39

nications (OWC) also below and above the wavelengths of 380 nm and 780 nm, respectively hereby includ-	40

ing the invisible light. A wide range of data rates (i.e., 1 Mb/s to 10 Gb/s) are supported, targeting an	41

efficient use of the optical bandwidth also under time-variant channel conditions.	42

43

The high-bandwidth PHY introduces modern wireless transmission technologies, such as orthogonal fre-	44

quency-division multiplexing (OFDM), adaptive transmission, multiple-input multiple-output (MIMO) and	45

coordinated wireless networking of multiple access points (APs) to provide mobility for mobile user devices	46

(UDs) in an OWC network infrastructure. In addition, specific requirements for enhanced robustness and	47

lower latency are addressed to support e.g. industrial wireless, vehicular and backhaul scenarios (B2, B3,	48

B4).	49

50

For coordinated networking, unified interfaces are introduced for the user plane and an open interface to the	51

control plane, which can be used at the network layer to manage the interference between parallel optical	52

wireless links and to support user mobility. These interfaces enable also the coexistence of OWC with radio	53

based wireless links.	54

Copyright © 20xx IEEE. All rights reserved.	13

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	4.4.1.5.1 P HY bas e d o n G.hn in c o ax mo de

2

3	As the above transmission concept is already realized for other media in home environments, such as twisted

4	pair, coax and transmission over plastic optical fibers, the following specifications take references to the

5	ITU-T recommendation for home networking G.hn that has been recently developed for all these media. The

6	requirements of the TCD can be met by

7

8	- Starting from G.hn PHY baseband coax modes for 50, 100 and 200 MHz bandwidth

9

10	- adding further modes for scalability towards lower and higher bandwidths

11

12	- adding new features for the coordinated wireless (COW) topology

13

14	- adding new features for the wireless relaying (WREL) topology

15

16	Starting from the G.hn recommendation is efficient because optical wireless is an emerging technology

17	expecting fast market growth. The home networking community has developed G.hn for several fixed net-

18	working media and is adopting OWC as a next medium to support as an further evolution of G.hn. By align-

19	ing the G.hn evolution with the high-bandwidth mode in 802.15.13, a powerful new specification is obtained

20	that fulfills the requirements in the TCD by evolving an existing technology rather defining a new one.

21

22	In the following, an implementable overview of the necessary features taken over from 2015 release of G.hn

23	is given it is demonstrated how the existing G.hn PHY can be further developed in order to enable the new

24	COW and WREL modes over the optical wireless link.

25

26	4.4.1.5.2 Fe ature s

27

28	Scalable data rates

29

30	The high-bandwidth PHY supports variable data rates from 1 Mbit/s to 10 Gbit/s by means of a scalable

31	design. In all PHY modes, subcarrier spacing and cyclic prefix (CP) are the same. Used bandwidth is scal-

32	able by adapting number of used subcarriers. Interoperability among all PHY modes is enabled, i.e. a trans-

33	ceiver with a smaller bandwidth can synchronize with respect to, and exchange control information and data

34	with, another transceiver having a higher bandwidth and vice versa. Bandwidth adaptation is supported to

35	operate the link at the lowest bandwidth during link setup, and uses subcarriers at low frequencies only for

36	control information transmission, before switching eventually to a higher bandwidth mode.

37

38	Waveform

39

40	An adaptive, real-valued OFDM waveform denoted as DC-biased discrete multi-tone (DMT) is used. It can

41	be extended by an optional preprocessing in order to improve energy efficiency. Moreover, adaptive bit- and

42	power loading is supported using variable modulation formats on each subcarrier or on groups of subcarri-

43	ers, depending on the channel-, interference- and noise-characteristics of the optical wireless link.

44

45	Efficient use of the optical bandwidth

46

47	The high-bandwidth PHY supports the efficient use of the optical bandwidth by means of a highly scalable

48	PHY layer design, together with closed-loop adaptive transmission and the efficient support of MIMO,

49	cooperative transmission and relaying. This combination allows robustness in the multi-path propagation

50	channel, in case of mobility and in interference scenarios. Moreover, PHY is defined so that one-way laten-

51	cies of 1 ms are achievable.

52

53	Dimming support, coexistence

54

14	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The high-bandwidth PHY allows dimming for use cases based on visible light such as indoor office/home              1 applications, data centers, industrial establishments, secure wireless and vehicular communications. Due to              2 adaptive transmission, coexistence is supported with ambient light and other light sources.                                          3

4

Metrics reporting	5

6

The high-bandwidth PHY provides metrics to be reported for efficient operation of higher layer protocols.	7

Depending on the topology, metrics to be reported comprise signal strength of strongest APs and UDs, fre-	8 quency-selective signal-to-interference-and-noise ratio (SINR) and channel state information (CSI) for	9 strongest APs and UDs. Short time intervals between metric reports and control messages are enabled target-	10 ing fast adaptation to the time-varying wireless channel, low latency and minimal overhead.	11

12

Advanced wireless networking, high availability	13

14

The high-bandwidth PHY allows robust wireless transmission and thereby high availability for all channel              15 conditions. Advanced wireless networking is supported in S, COW, HET and WREL topologies. The link is              16 available in both, line-of-sight (LOS) and non-LOS (NLOS) scenarios, at low signal-to-noise-and-interfer-             17 ence ratio (SINR) and in interference-limited conditions.                                                                                                18

19

4.4.2 MAC sublayer	20

21

The MAC sublayer provides two services accessed through two service access points (SAPs). MAC data is              22 accessed through the MAC common part sublayer SAP (MCPS-SAP) while MAC management is accessed              23 through the MAC sublayer management entity SAP (MLME-SAP). The MAC data service enables the              24 transmission and reception of MPDUs across the PHY data service.                                                                               25

26

The features of the MAC sublayer are beacon management, channel access, guaranteed time slot (GTS)              27 management, frame validation, acknowledged frame delivery, association, and disassociation. The MAC              28 sublayer provides hooks for implementing application-appropriate security mechanisms.                                             29

30

Clause 5 contains the specifications for the MAC sublayer.	31

32

4.4.3 Dimming support	33

34

Dimming in general is an optional feature and designed so that it is independent of the communication func-	35 tionality. The receiver is not required to know what dimming level is used while communication is ongoing.	36

37

4.4.3.1 Dimming Support Using Constant Bias Current	38

39

Dimming can be controlled via a bias current which is constant over time and orthogonal to the modulator	40 output signal used for the data transmission (denoted as modulation signal).	41

42

The OWC system is responsible for the modulation while the lighting system provides a dimming level              43 which is processed in a control unit being independent of the OWC system. The control unit can set both, the              44 bias and the modulation index to achieve the required dimming level and to avoid clipping. Finally, the bias              45 and the modulation are added, as shown in Figure 6.                                                                                                       46

47

In this way, the receiver does not even need to know the dimming level for decoding the data from the com-              48 pound signal received after the PD, and if needed, the transmitter is free to set the dimming level indepen-              49 dent of the receiver. Accordingly, there is no need for signalling fields telling the receiver the dimming level              50 used at the transmitter, or an accordingly used parameter setting of the modulation scheme.                                         51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	15

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4	Data

5

6

7

8

9

10

11

Modulator

Modulation

Index

DC Bias

12	Dimming Control Unit

13

14

15	Dimming Level

17

16	Lighting

18	System

19

20

21	Figure 6—Dimming control through modulation index and DC bias

22

23

24

25	4.4.3.2 Dimming support by using the pulse width control

26

27	This method is using the pulse width control in the pulse-shaping filter following the modulation part of the

28	system, as e.g. in combination with the VPPM modulation shown in Figure 8. Instead of changing the bias

29	current, here the pulse width of VPPM symbol is adapted according to the dimming level, yielding shorter

30	pulse width for lower light level, for instance.

31

32

33

35

34	Data	BPPM	Pulse-width control

36	by VPPM modulator

37

38	Dimming Control Unit

39

40

41	Dimming Level

42

43	Lighting System

44

45	Figure 7—Dimming control through the VPPM modulation

46

47

48

49

50

51

52

53

54

16	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

Figure 8—Schematic mechanism for VPPM dimming	21

22

23

4.4.4 Flicker mitigation support	24

25

Flicker is defined as the fluctuation of the brightness of light that can cause noticeable physiological changes              26 in humans. This standard strives for the mitigation of flicker that may be caused due to modulation of the              27 light sources for communication. The maximum flickering time period (MFTP) is defined as the maximum              28 time period over which the light intensity can be changing, but for which the resulting flicker is not              29 perceivable by the human eye (Berman, et al. [B13]). To avoid flicker, the brightness changes over periods              30 longer than MFTP must be avoided.                                                                                                                                 31

32

The flicker in OWC is classified into two categories according to its generation mechanism: intra-frame              33 flicker and inter-frame flicker. Intra-frame flicker is defined as the perceivable brightness fluctuation within              34 a frame. Interframe flicker is defined as the perceivable brightness fluctuation between adjacent frame             35 transmissions.                                                                                                                                                                    36

37

4.4.4.1 Intra-frame flicker mitigation	38

39

For PM-PHY, intra-frame flicker mitigation is accomplished by either the use of the line code or HCM. For	40

LB-PHY and HB-PHY, intra-frame flicker mitigation is inherently supported by OFDM waveform if DC	41 subcarrier is not used.	42

43

4.4.4.2 Inter-frame flicker mitigation	44

45

The scheme used for inter-frame flicker mitigation is the transmission of an idle pattern between data frames	46 whose average brightness is equal to that of the data frames, as defined in 8.5.	47

48

50

4.5 Functional overview	49

51

This clause provides a brief overview of the general functions of a OWPAN MAC sublayer and includes	52

information on the superframe structure, data transfer model, data frame structure, acknowledgments, and	53

security.	54

Copyright © 20xx IEEE. All rights reserved.	17

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	4.5.1 Superframe structure

2

3	This standard allows the optional use of a superframe structure. The format of the superframe is defined by

4	the coordinator and varies with the network topologies. The superframe is bounded by beacons sent by the

5	coordinator, as shown in Figure 9a, and is divided into equally sized slots. Optionally, the superframe can

6	have an active and an inactive portion, as shown in Figure 9b. The beacon frame is transmitted in the first

7	slot of each superframe. If a coordinator does not wish to use a superframe structure, it will turn off the

8	beacon transmissions. Each superframe can have two or three portions: beacon period (BP), contention

9	access period (CAP) and optional contention-free period (CFP), as shown in Figure 21. The beacons aresent

10	in the BP and are used to synchronize the attached devices, to identify the OWPAN, and to describe the

11	structure of the superframes. Any device wishing to communicate during the contention access period

12	(CAP) between two beacons competes with other devices via slotted random access.The standard defines

13	four random access methods: unslotted random access, slotted random access, unslotted CSMA/CA, and

14	slotted CSMA/CA. These methods are described in 5.1.2.1.

15

16	All transactions are completed by the time of the next BP.

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37	Figure 9—Superframe structure without GTSs

38

39

40	For low-latency applications or applications requiring specific data bandwidth, the coordinator is allowed to

41	dedicate portions of the active superframe to that application. These portions are called guaranteed time slots

42	(GTSs). The GTSs form the contention-free period (CFP), which always appears at the end of the active

43	superframe starting at a slot boundary immediately following the CAP. More information on the GTS

44	structure is provided in Figure 11. The coordinator is allowed to allocate a number of these GTSs, and a GTS

45	may occupy more than one slot period. More information on the GTS slots and the maximum number

46	available can be found in 5.2.2.1. All contention-based transactions are completed before the CFP begins.

47	Also each device transmitting in a GTS ensures that its transaction is complete before the time of the next

48	GTS or the end of the CFP. More information on the superframe structure as described in 5.1.1.

49

50	4.5.2 Data transfer model

51

52	Five types of data transfer transactions exist:

53

54	a)	The data transfer to a coordinator in which a device transmits the data.

18	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

b)     The data transfer from a coordinator in which the device receives the data.                                                       1 c)     The data transfer between two peer devices.                                                                                                        2 d)     The data transfer between a device and a relay node.                                                                                          3 e)     The data transfer between a relay node and a coordinator.                                                                                  4

5

In a star or broadcast topology, only the first two transaction types are used, while in a peer-to-peer topology,              6 all three transaction types are allowed.                                                                                                                             7

8

The mechanisms for each transfer type depend on whether the network supports the transmission of	9 beacons. A beacon-enabled OWPAN is used in networks that either require synchronization or support for	10 low-latency devices. If the network does not need synchronization or support for low latency devices, it can	11 elect not to use the beacon for normal transfers. However, the beacon is still required for network discovery.	12

The structure of the frames used for data transfer is specified in 5.2.                                                                              13

14

4.5.2.1 Data transfer to a coordinator                                                                                                         15

16

When a device wishes to transfer data to a coordinator in a beacon-enabled OWPAN, it first listens for the              17 beacon. When the beacon is found, the device synchronizes to the superframe structure. At the appropriate              18 time, the device transmits its data frame, using slotted random access, to the coordinator. The coordinator is              19 allowed to acknowledge the successful reception of the data by transmitting an optional acknowledgment              20 frame.                                                                                                                                                                                21

22

When a device wishes to transfer data in a non beacon-enabled OWPAN, it simply transmits its data frame,              23 using unslotted random access, to the coordinator. The coordinator acknowledges the successful reception of              24 the data by transmitting an optional acknowledgment frame. The transaction is now complete.                                    25

26

4.5.2.2 Data transfer from a coordinator                                                                                                     27

28

When the coordinator wishes to transfer data to a device in a beacon-enabled OWPAN, it indicates in the              29 beacon that the data message is pending. The device periodically listens to the beacon and, if a message is              30 pending, transmits a MAC command requesting the data, using slotted random access. The coordinator              31 acknowledges the successful reception of the data request by transmitting an acknowledgment frame. The              32 pending data frame is then sent using slotted random access, or; if possible, immediately after the             33 acknowledgment as described in 5.1.9.3. The device is allowed to acknowledge the successful reception of              34 the data by transmitting an optional acknowledgment frame. The transaction is now complete. Upon              35 successful completion of the data transaction, the message is removed from the list of pending messages in              36 the beacon.                                                                                                                                                                        37

38

When a coordinator wishes to transfer data to a device in a non beacon-enabled OWPAN, it stores the data              39 and waits for the appropriate device to make contact and request the data. A device is allowed to make              40 contact by transmitting a  MAC command requesting the data, using unslotted random access, to  its              41 coordinator. The coordinator acknowledges the successful reception of the data request by transmitting an              42 acknowledgment frame. If a data frame is pending, the coordinator transmits the data frame, using unslotted              43 random access to the device. If a data frame is not pending, the coordinator indicates this fact either in the              44 acknowledgment frame following the data request or in a data frame with a zero-length payload as described              45 in 5.1.9.3. If requested, the device acknowledges the successful reception of the data frame by transmitting              46 an acknowledgment frame.                                                                                                                                               47

48

4.5.2.3 Peer-to-peer data transfers                                                                                                              49

50

In a peer-to-peer OWPAN, every device is allowed to communicate with every other device in its coverage              51 area. In order to do this effectively, the devices wishing to communicate will need to either receive              52 constantly or synchronize with each other. In the former case, the device can simply transmit its data using              53

54

Copyright © 20xx IEEE. All rights reserved.	19

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	unslotted  random  access.  In  the  latter  case,  other  measures  need  to  be  taken  in  order  to  achieve

2	synchronization. Such measures are beyond the scope of this standard.

3

4	4.5.3 Clock-rate selection

5

6	The standard supports multiple optical clock rates in order to accommodate a wide variety of optical sources

7	and receivers. The standard also supports the use of asymmetric clock rates between two devices since the

8	transmitter and receiver in a device are independent and may support different clock-rate ranges. As an

9	example, the infrastructure transmitter may be unable to switch rapidly but may be able to transmit with high

10	power and require lower error correction while the mobile device transmitter may be able to switch rapidly

11	but may require higher error correction support due to its lower transmit power. The optical clock rate for

12	communication is established using the MAC and can be communicated to the receiver prior to data transfer.

13	The clock-rate selection and negotiation procedure is described in 6.5.

14

15	4.5.4 Frame structure

16

17	The frame structures have been designed to keep the complexity to a minimum while providing for error

18	protection for transmission on a noisy channel. Each successive protocol layer adds to the structure with

19	layer-specific headers and footers.

20

21	a)	A beacon frame, used by a coordinator to transmit beacons.

22	b)	A data frame, used for all transfers of data.

23	c)	An acknowledgment frame, used for confirming successful frame reception.

24	d)	A MAC command frame, used for handling all MAC peer entity control transfer.

25	e)	A control frame, used for RTS/CTS procedure.

26	f)	A CVD frame, used to maintain the proper light intensity between data frames, support dimming

27	and for visually providing information such as communication status and channel quality to the user.

28

29	4.5.5 Improving probability of successful delivery

30

31	The IEEE 802.15.13 OWPAN employs various mechanisms to improve the probability of successful data

32	transmission. These mechanisms are random access, frame acknowledgment, and data verification.

33

34	4.5.5.1 Random access mechanism

35

36	The IEEE 802.15.13 OWPAN uses four types of channel access mechanism, depending on the network

37	configuration. Non-beacon-enabled OWPANs use an unslotted random channel access mechanism, with or

38	without CSMA/CA, as described in 5.1.2.1. Each time a device wishes to transmit data frames or MAC

39	commands, it waits for a random back off period. Following the random back off, the device transmits its

40	frame of data. If the optional carrier sense mechanism is active and the channel is found to be busy following

41	the random back off, the device waits for another random period before trying to access the channel again.

42	Acknowledgment frames are sent without using a random access mechanism (i.e., scheduled).

43

44	Beacon-enabled OWPANs use a slotted random channel access mechanism, with or without CSMA/CA,

45	where the back off slots are aligned with the start of the beacon transmission. Each time a device wishes to

46	transmit data frames during the CAP, it locates the boundary of the next back off slot and then waits for a

47	random number of back off slots. If the optional collision avoidance mechanism is active, and the channel is

48	busy, following this random back off the device waits for another random number of back off slots before

49	trying to access the channel again. If the channel is idle or the optional carrier sense mechanism is not active,

50	the device begins transmitting on the next available back off slot boundary. Acknowledgment and beacon

51	frames are sent without using a random access mechanism (i.e., scheduled).

52

53	RTS/CTS mechanism is used in addition, if configured.

54

20	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

4.5.5.2 Frame acknowledgment                                                                                                                   1

2

A successful reception and validation of a data or MAC command frame can be optionally confirmed with              3 an acknowledgment, as described in 5.1.9.4. If the receiving device is unable to handle the received data              4 frame for any reason, the message is not acknowledged.                                                                                                 5

6

If the originator does not receive an acknowledgment after some period, it assumes that the transmission was              7 unsuccessful. When the acknowledgment is not required, the originator assumes the transmission was              8 successful.                                                                                                                                                                         9

10

4.5.5.3 Data verification                                                                                                                                11

12

A cyclic redundancy check is included in the MAC frame and the PHY header, as defined in Annex C, to              13 verify the validity of the received data.                                                                                                                             14

15

4.6 Security                                                                                                                                    16

17

18

From a security perspective, IEEE 802.15.13 OWPAN is slightly different from other wireless networks, due              19

to directionality and visibility because of the choice of the visible optical spectrum. Because of directionality              20

and visibility, if an unauthorized receiver is in the path of the communication signal, it can be recognized.              21

Also, the signal will not travel across medium such as walls, unlike other radio frequency based wireless              22

networks.  However,  security  algorithms  are  still  provided  in  the  standard  for  features  such  as  data              23

confidentiality, authentication and replay protection.                                                                                                       24

25

Devices can be low-cost and have limited capabilities in terms of computing power, available storage, and              26

power drain; and it cannot always be assumed they have a trusted computing base nor a high-quality random              27

number generator aboard. Communications cannot rely on the online availability of a fixed infrastructure              28

and might involve short-term relationships between devices that may never have previously communicated.              29

These constraints limit the choice of cryptographic algorithms and protocols and influence the design of the              30

security architecture because the establishment and maintenance of trust relationships between devices need              31

to be addressed with care. In addition, battery lifetime and cost constraints can put severe limits on the              32

security overhead these networks can tolerate, something that is of far less concern with higher bandwidth              33

networks. Most of these security architectural elements can be implemented at higher layers and may,              34

therefore, be considered to be outside the scope of this standard.                                                                                    35

36

The cryptographic mechanism in this standard is based on symmetric-key cryptography and uses keys that              37

are provided by higher layer processes. The establishment and maintenance of these keys is outside the              38

scope of this standard. The mechanism assumes a secure implementation of cryptographic operations and              39

secure and authentic storage of keying material.                                                                                                              40

41

The cryptographic mechanism provides particular combinations of the following security services:                             42

43 a)	Data confidentiality: Assurance that transmitted information is only disclosed to parties for whom it		44 is intended.	45

b)	Data  authenticity: Assurance  of  the  source  of  transmitted  information  (and,  thereby,  that	46 information was not modified in transit).	47

c)	Replay protection: Assurance that duplicate information is detected.	48

49

The actual frame protection provided can be adapted on a frame-by-frame basis and allows for varying              50

levels of data authenticity (to minimize security overhead in transmitted frames where required) and for              51

optional data confidentiality. When nontrivial protection is required, replay protection is always provided.                 52

53

54

Copyright © 20xx IEEE. All rights reserved.	21

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Cryptographic frame protection is allowed to use a key shared between two peer devices (link key) or a key

2	shared among a group of devices (group key), thus allowing some flexibility and application-specific trade-

3	off between key storage and key maintenance costs versus the cryptographic protection provided. If a group

4	key is used for peer-to-peer communication, protection is provided only against outsider devices and not

5	against potential malicious devices in the key-sharing group.

6

7	The cryptographic security mechanisms used for protected MAC frames is described in Clause 7.

8

9	4.7 Concept of primitives

10

11

12	This subclause provides a brief overview of the concept of service primitives. The services of a layer are the

13	capabilities it offers to the next higher layer or sublayer by building its functions on the services of the next

14	lower layer. This concept is illustrated in Figure 10, showing the service hierarchy and the relationship of the

15	two correspondent users and their associated layer (or sublayer) peer protocol entities.

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31	Figure 10—Service primitives

32

33

34	The  services are  specified by  describing the  information flow  between the  user  and  the  layer. This

35	information flow is modeled by discrete, instantaneous events, which characterize the provision of a service.

36	Each event consists of passing a service primitive from one layer to the other through a layer service access

37	point (SAP) associated with an user. Service primitives convey the required information by providing a

38	particular service. These service primitives are an abstraction because they specify only the provided service

39	rather  than  the  means  by  which  it  is  provided.  This  definition  is  independent  of  any  interface

40	implementation.

41

42	Services are specified by describing the service primitives and parameters that characterize it. A service may

43	have one or more related primitives that constitute the activity that is related to that particular service. Each

44	service primitive may have zero or more parameters that convey the information required to provide the

45	service.

46

47	A primitive can be one of the following four generic types:

48

49	a)	Request: The request primitive is used to request a service to be initiated.

50	b)	Confirm: The confirm primitive is used to convey the results of one or more associated previous

51	service requests.

52	c)	Indication: The indication primitive is used to indicate the next higher layer of an internal event.

53

54

22	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

d)	Response: The response primitive is used to complete a procedure previously invoked by an	1 indication primitive.	2

3

4

5. MAC protocol specification                                                                                         5

6

This clause specifies the MAC sublayer of this standard. The MAC sublayer handles all access to the              7 physical layer and is responsible for the following tasks:                                                                                                8

9

a)     Generating network beacons if the device is a coordinator                                                                                 10 b)     Synchronizing to network beacons                                                                                                                      11 c)     Supporting OWPAN association and disassociation                                                                                            12 d)     Supporting device security                                                                                                                                   13 e)     Providing a reliable link between two peer MAC entities                                                                                   14 f)      Supporting mobility                                                                                                                                             15

16

Peer-to-peer, star and broadcasting and coordinated capabilities, as shown in Figure 1, are provided with a              17 single MAC frame structure. All of  these diverse modes are supported via  a  single low complexity              18 integrated frame structure.                                                                                                                                                19

20

Constants and attributes that are specified and maintained by the MAC sublayer are written in the text              21 of this clause in italics. Constants have a general prefix of “a”, e.g., aBaseSlotDuration, and are listed in              22

Table 90 (see 6.4.1). Attributes have a general prefix of “mac”, e.g., macAckWaitDuration, and are listed in              23

Table 91 (see 6.4.2), while the security attributes are listed in Table 98 (see 7.5.1).                                                       24

25

26

5.1 MAC functional description                                                                                                    27

28

editor’s note: the following introduction paragraphs should be revisited after clause 5 gets stable.                              29

30

This subclause provides a detailed description of the MAC functionality. Subclause 5.1.2 describes the              31

following two mechanisms for channel access: contention based and contention free. Contention-based              32

access allows devices to access the channel in a distributed fashion using an unslotted random access              33

backoff algorithm. Contention-free access is controlled entirely by the coordinator through the use of GTSs.              34

35

The mechanisms used for starting and maintaining a OWPAN are respectively described in 5.1.3 and 5.1.5.              36

Channel scanning is used by a device to assess the current state of a channel (or channels), locate all beacons              37

within its operating space, or locate a particular beacon with which it has lost synchronization. Before              38

starting a new OWPAN, the results of a channel scan can be used to select an appropriate logical channel, as              39

well as a OWPAN identifier that is not being used by any other OWPAN in the area. Because it is still              40

possible for the operating space of two OWPANs with the same OWPAN identifier to overlap, a procedure              41

exists to detect and resolve this situation. Following a  channel scan and suitable OWPAN identifier              42

selection, operation as a coordinator shall commence. Also described in the subclause is a method to allow              43

coordinator beaconing to discover other such devices during normal operations, i.e., when not scanning.                    44

45

The mechanisms to allow devices to join or leave a OWPAN are defined in 5.1.6. The association procedure              46

describes the conditions under which a device may join a OWPAN and the conditions necessary for a              47

coordinator to permit devices to join. Also described is the disassociation procedure, which can be initiated              48

by the associated device or its coordinator.                                                                                                                      49

50

The mechanisms to allow devices to acquire and maintain synchronization with a coordinator are described              51

in 5.1.7. Synchronization on a beacon-enabled OWPAN is described after first explaining how a coordinator              52

generates beacon frames. Following this explanation, synchronization on a nonbeacon-enabled OWPAN is              53

described. Also described is a procedure to reestablish communication between a device and its coordinator,              54

Copyright © 20xx IEEE. All rights reserved.	23

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	as it is possible that a device may lose synchronization in the case of either a beacon-enabled or a

2	nonbeacon-enabled OWPAN.

3

4	This standard has been designed so that application data transfers can be controlled by the devices on a

5	OWPAN rather than by the coordinator. The procedures the coordinator uses to handle multiple transactions

6	while preserving this requirement are described in 5.1.8.

7

8	The mechanisms for  transmitting, receiving, and  acknowledging frames, including frames sent using

9	indirect transmission, are  described in  5.1.9. In  addition, methods for  retransmitting frames are  also

10	described.

11

12	The mechanisms for allocating and deallocating a GTS are described in 5.1.10. The deallocation process

13	may result in the fragmentation of the GTS space, i.e., an unused slot or slots. The subclause describes a

14	mechanism to resolve fragmentation.

15

16	The MAC sublayer uses the mechanisms described in Clause 7 for all incoming and outgoing frames.

17

18	Throughout this subclause, the receipt of a frame is defined as the successful receipt of the frame by the

19	PHY and the successful verification of the FCS by the MAC sublayer, as described in 5.2.1.9.

20

21	The mechanisms to allow devices to recover quickly in case of temporary interference using a fast link

22	recovery process are defined in 5.1.11. The fast link recovery process also enables devices to save power by

23	letting the infrastructure initiate the link recovery.

24

25

28

29

32

33

37

38

41

42

46

47	5.1.1 Superframe structure

48

49	A coordinator on a  OWPAN can  optionally bound its  channel time using a  superframe structure. A

50	superframe is bounded by the transmission of a beacon frame and can have an active portion and an inactive

51	portion. The coordinator may enter a low-power (sleep) mode during the inactive portion.

52

53	The structure of this superframe is described by the values of macBeaconOrder and macSuperframeOrder.

54	The MAC PIB attribute macBeaconOrder, describes the interval at which the coordinator shall transmit its

24	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

beacon frames. The value of macBeaconOrder, BO, and the beacon interval, BI, are related as follows: for 0	1

 BO  14, BI = aBaseSuperframeDuration  2BO optical clocks. If BO = 15, the coordinator shall not	2 transmit beacon frames except when requested to do so, such as on receipt of a beacon request command.	3

The value of macSuperframeOrder shall be ignored if BO = 15.	4

5

The MAC PIB attribute macSuperframeOrder describes the length of the active portion of the superframe,              6 which includes the beacon frame. The value of macSuperframeOrder, SO, and the superframe duration, SD,              7 are related as follows: for 0  SO  BO, SD = aBaseSuperframeDuration  2SO optical clocks. If SO =              8

15, the superframe shall not remain active after the beacon. If BO = 15, the superframe shall not exist (the	9 value of macSuperframeOrder shall be ignored), and macRxOnWhenIdle shall define whether the receiver is	10 enabled during periods of transceiver inactivity.	11

12

The active portion of each superframe shall be divided into aNumSuperframeSlots equally spaced slots of              13 duration 2SO  aBaseSlotDuration and is composed of three parts: a BP, a CAP and a CFP. The BP exists in              14 coordinated topologies when comb division is not used in PHY VII. Beacon interference avoidance using              15 comb division for PHY VII coordinated topology is described in TBD. The beacon shall be transmitted,              16 without the use of any random access, at the start of slot 0, if BP doesn’t exist. If BP exits, beacon is              17 transmitted without the use of any random access, in one beacon slot of BP. The CAP shall commence              18 immediately following the beacon. The CFP, if present, follows immediately after the CAP and extends to              19 the end of the active portion of the superframe. Any allocated GTSs shall be located within the CFP.                           20

21

The MAC sublayer shall ensure that the integrity of the superframe timing is maintained, e.g., compensating	22 for clock drift error.	23

24

OWPANs that wish to use the superframe structure (referred to as a beacon-enabled OWPAN) shall set	25

macBeaconOrder to a value between 0 and 14, both inclusive, and macSuperframeOrder to a value between	26

0 and the value of macBeaconOrder, both inclusive.	27

28

OWPANs that do not wish to use the superframe structure (referred to as a nonbeacon-enabled OWPAN)              29 shall set both macBeaconOrder and macSuperframeOrder to 15. In this case, a coordinator shall not transmit              30 beacons, except upon receipt of a beacon request command; all transmissions, with the exception of             31 acknowledgment frames and any data frame that quickly follows the acknowledgment of a data request              32 command, see 5.1.9.3, shall use an unslotted random access mechanism to access the channel. In addition,              33

GTSs shall not be permitted.	34

35

An example of a superframe structure is shown in Figure 11. In this case, the beacon interval, BI, is twice as	36 long as the active superframe duration, SD, and the CFP contains two GTSs.	37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

Figure 11—An example of the superframe structure	52

53

54

Copyright © 20xx IEEE. All rights reserved.	25

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3	peer-to-peer mode

4

5

6

7

8	star

9

10

11

12

13

14	broadcast

15

16

17

18	visibility mode

19

20

21

22

23	legend for slot indication

24

25

26

(a ) 	all slots in frame used for same data

(b) beacon, contention,  uplink, downlink slots

(c) broadcast slots (beacon, downlink)

( d ) 	visibility slots

data 	beacon 	contention 	uplink 	downlink 	visibility

27	Figure 12—Example usage of frame structure for multiple topologies

28

29

30	Figure 12 provides an example usage of frame structure configuration for multiple topologies such as peer-

31	to-peer, star, broadcast and visibility modes. The beacon slots are used for the beacons and the contention

32	slots are used in the CAP period. The uplink and downlink GTS slots are used in the CFP periods. Visibility

33	or idle patterns can be sent in the visibility slots during idle or RX modes of the infrastructure to ensure

34	continuous output and mitigate flicker and are also used for point-and-shoot mode to ensure visibility.

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

26	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

beacon	beacon	2

3

4

5

（a）	6

7

8

9

Peer-to-Peer / Broadcast topology	10

11

beacon	beacon	12

CAP	CFP	13

14

（b）	15

16

17

18

Star topology	19

20

| BP CAP CFP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | … |  |  | GTS | GTS | GTS | GTS |  |

beacon	beacon	21

22

23

24

（c）	25

26

27

28

Coordinated topology	29

Figure 13—Superframe structure for difference topologies	30

31

32

5.1.1.1 Beacon period (BP)	33

34

The BP exists in coordinated topology when comb division is not used, in PHY VII. Beacon interference	35

avoidance using comb division for coordinated topology is specified in section 12. (editor note: please	36

Volker provide text for comb division in high bandwidth PHY) The BP shall start at slot 0 of a superframe	37

and complete before the beginning of the CAP on a superframe slot boundary. BP is further divided into	38

multiple beacon slots as shown in Figure 14. The duration of each beacon slot is equal to the sum of the	39

duration of a beacon physical layer data unit (PPDU) and the subsequent beacon-to-beacon inter-frame	40

space. In peer-to-peer mode and star mode, the BP shall contain only one beacon slot. In coordinated	41

network, the BP consists one to a maximal of MaxBeaconSlot beacon slots as shown in Figure 14. The	42

structure of a BP is notified in the beacon frame (see 5.2.2.1.3).	43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	27

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

| Beacon Slot 1 | Beacon Slot 2 | Beacon Slot 3 | Beacon Slot 4 |
| --- | --- | --- | --- |

9

10

11

12

13

14

15

16

17	Beacon PPDU

18

19

20

Beacon Period

B2BIFS

21	Figure 14—An example of the structure of beacon period

22

23	5.1.1.2 Contention access period (CAP)

24

25	The CAP shall start immediately following the beacon and complete before the beginning of the CFP on a

26	superframe slot boundary. If the CFP is zero length, the CAP shall complete at the end of the active portion

27	of the superframe. The CAP shall be at least aMinCAPLength optical clocks, unless additional space is

28	needed to temporarily accommodate the increase in the beacon frame length needed to perform GTS

29	maintenance (see 5.2.2.1.4) and shall shrink or grow dynamically to accommodate the size of the CFP.

30

31	All frames, except acknowledgment frames and any data frame that quickly follows the acknowledgment of

32	a data request command (see 5.1.9.3) transmitted in the CAP shall use a slotted random access mechanism to

33	access the channel. A device transmitting within the CAP shall ensure that its transaction is complete (i.e.,

34	including the reception of any acknowledgment) one interframe space (IFS) period (see 5.1.1.4) before the

35	end of the CAP. If this is not possible, the device shall defer its transmission until the CAP of the following

36	superframe.

37

38	In the case when a polling mechanism is employed, a poll by the coordinator to all devices can be used to

39	indicate the beginning of a transmission slot for slotted random access. The poll synchronizes and enables

40	each transmission of the different devices. In the case when a polling mechanism is not employed, the

41	devices will be configured to synchronize according to the beacon frame and will be responsible for keeping

42	track of the valid transmission slots in case slotted random access is employed.

43

44	5.1.1.3 Contention-free period (CFP)

45

46	The CFP shall start on a slot boundary immediately following the CAP and it shall complete before the end

47	of the active portion of the superframe. If any GTSs have been allocated by the coordinator, they shall be

48	located within the CFP and occupy contiguous slots. The CFP shall therefore grow or shrink depending on

49	the total length of all of the combined GTSs. Communication between devices can take a variable number of

50	slots. A single device or user can have access to more than a single slot for sustained data transfer in the

51	frame, if there are slots available.

52

53

54

28	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

No transmissions within the CFP shall use a unslotted random access mechanism to access the channel. A              1 device transmitting in the CFP shall ensure that its transmissions are complete one IFS period (see 5.1.1.4)              2 before the end of its GTS.                                                                                                                                                 3

4

In the case when a polling mechanism is employed, a poll by the coordinator to a specific device shall be              5 used to indicate the beginning of a transmission slot allocated solely and specifically to the device of choice.              6

The  polling  mechanism  synchronizes  and  enables  each  transmission  of  a  device,  thus,  completely	7 eliminating contention. No restrictions shall be imposed on the order and/or the number of times devices are	8 being polled. The polling mechanism allows contention access to be enabled at random points of the	9 superframe by issuing a poll to all stations as described in 6.2.1.1.1. Furthermore, transmission slots during	10 the CFP can be allocated dynamically as the coordinator chooses at its own discretion which device shall be	11 polled. The algorithm used to determine the polling sequence is outside the scope of this specification.	12

13

5.1.1.4 Interframe spacing (IFS)                                                                                                                   14

15

The MAC sublayer needs a finite amount of time to process data received by the PHY. To allow for this, two              16 successive frames transmitted from a device shall be separated by at least an IFS period; if the first              17 transmission requires an acknowledgment, the separation between the acknowledgment frame and the              18 second transmission shall be at least an IFS period. The length of the IFS period is dependent on the size of              19 the frame that has just been transmitted. Frames (i.e., MPDUs) of up to aMaxSIFSFrameSize octets in length              20 shall be followed by a SIFS period of a duration of at least macMinSIFSPeriod optical clocks. Frames (i.e.,              21

MPDUs) with lengths greater than aMaxSIFSFrameSize octets shall be followed by a long interframe space              22 (LIFS) period of a duration of at least macMinLIFSPeriod optical clocks. Burst frames shall have a reduced              23 interframe space (RIFS) of exactly macMinRIFSPeriod. The IFS for the different modes are defined in  and              24 the concepts are illustrated in Figure 15.                                                                                                                          25

26

The slotted random access algorithm shall take this requirement into account for transmissions in the CAP.                27

28

acknowledged transmission 	29

30

long frame 	ACK 	short frame 	ACK 	31

tack unacknowledged transmission

long frame 	s hort frame

LIFS 	tack

SIFS 	32

33

34

35

36

37

38

burst transmission

LIFS 	SIFS 	39

40

41

42

long frame 	burst frame

LIFS

RIFS

burst frame

RIFS

burst frame 	43

44

45

46

Figure 15—Interframe spacing	47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	29

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.2 Channel access

2

3	This subclause describes the mechanisms for accessing the physical optical channel. The standard provides a

4	single OWC MAC frame structure that can be configured for multiple modes. The frame is composed of a

5	variable number of slots. A slot can be defined as the minimum time needed to communicate to send the

6	smallest data to a device and is fixed.

7

8	5.1.2.1 Random access algorithm

9

10	The slotted random access algorithm shall be used before the transmission of data or MAC command frames

11	transmitted within the CAP, unless the frame can be quickly transmitted following the acknowledgment of a

12	data  request  command  (as  defined  in  5.1.9.3  for  timing  requirements). None  of  the  random  access

13	algorithms  shall  be  used  for  the  transmission  of  beacon  frames  in  a  beacon-enabled  OWPAN,

14	acknowledgment frames, or data frames transmitted in the CFP.

15

16	If periodic beacons are being used in the OWPAN, the MAC sublayer shall employ the slotted version of the

17	random access algorithm for transmissions in the CAP of the superframe. Conversely, if periodic beacons

18	are not being used in the OWPAN or if a beacon could not be located in a beacon-enabled OWPAN, the

19	MAC sublayer shall transmit using the unslotted version of the random access algorithm. In both cases, the

20	algorithm is implemented using units of time called backoff periods, where one backoff period shall be equal

21	to aUnitBackoffPeriod optical clocks.

22

23	In slotted random access, the backoff period boundaries of every device in the OWPAN shall be aligned with

24	the superframe slot boundaries of the coordinator, i.e., the start of the first backoff period of each device is

25	aligned with the start of the beacon transmission. In slotted random access, the MAC sublayer shall ensure

26	that the PHY commences all of its transmissions on the boundary of a backoff period. In unslotted random

27	access, the backoff periods of one device are not related in time to the backoff periods of any other device in

28	the OWPAN. In the case a polling mechanism is used, each poll by the coordinator node simultaneously

29	addressed to all device nodes will indicate the beginning of the next transmission slot.

30

31	Each device shall maintain two variables for each transmission attempt: NB and BE. NB is the number of

32	times the access algorithm was required to backoff while attempting the current transmission; this value

33	shall be initialized to zero before each new transmission attempt. The variable BE is the backoff exponent,

34	which is related to how many backoff periods a device shall wait before attempting to access/assess a

35	channel. BE shall be initialized to the value of macMinBE.

36

37	Figure 16 illustrates the steps of the access algorithm. The MAC sublayer shall first initialize NB and BE for

38	slotted random access then locate the boundary of the next backoff period. The MAC sublayer shall delay

39	for a random number of complete backoff periods in the range 0 to 2BE – 1 [step (2)] and then request that

40	the PHY perform a transmission or optionally a CCA. In a slotted random access system, the transmission,

41	or CCA if active, shall start on a backoff period boundary. In an unslotted system, the transmission, or CCA

42	if active, shall start immediately.

43

44	In a slotted random access system, the MAC sublayer shall ensure that, after the random backoff, the

45	remaining slotted random access operations can be undertaken and the entire transaction can be transmitted

46	before the end of the CAP. Note that any bit padding used by the supported PHY shall be considered in

47	making this determination. If the number of backoff periods is greater than the remaining number of backoff

48	periods in the CAP, the MAC sublayer shall pause the backoff countdown at the end of the CAP and resume

49	it at the start of the CAP in the next superframe. If the number of backoff periods is less than or equal to the

50	remaining number of backoff periods in the CAP, the MAC sublayer shall apply its backoff delay and then

51	evaluate whether it can proceed. The MAC sublayer shall proceed if the remaining unslotted random access

52	algorithm steps, the frame transmission, and any acknowledgment can be completed before the end of the

53	CAP. If the MAC sublayer can proceed and CCA is active, it shall request that the PHY perform the CCA in

54

30	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

the current superframe. If the MAC sublayer cannot proceed, it shall wait until the start of the CAP in the	1 next superframe and apply a further random backoff delay before evaluating whether it can proceed again.	2

3

If CCA is active and the channel is assessed to be busy, the MAC sublayer shall increment both NB and BE              4 by one, ensuring that BE shall be no more than macMaxBE. If the value of NB is less than or equal to              5 macMaxRABackoffs, the access algorithm shall return to perform a random backoff as shown in Figure 16. If              6 the value of NB is greater than macMaxRABackoffs, the access algorithm shall terminate with a channel              7 access failure status.                                                                                                                                                          8

9

10

11

random access 	12

13

NB=0 	14

BE=macMinBE 	15

16 no 		17 slotted? 	18

yes 	  19 locate backoff 	20 period boundary 	  21

22 delay for 	23 random (2BE-1) unit 		24

backoff periods

25

26

no 	carrier sense active?

yes

perform CCA

no channel idle?

yes transmit

no

NB

27

28

29

30

31

32

33

34

35

36

37

38

39

yes 	40

ACK required?

yes

ACK received within

macAckWaitDuration?

yes success

≤macMaxRABackoffs?

NB=NB+1,

BE=min(BE+1,macMaxBE)

41

42

43

44

45

46

47

failure 	48

Figure 16—Random access flowchart	49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	31

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.2.2 RTS / CTS protocol

2

3	RTS/CTS protocol maybe optionally used as indicated in the beacon frame (see 5.2.2.1). A device may

4	compete for the transmission of the RTS frame first. When the device gains the right for transmission after

5	contention it shall transmit an RTS frame.

6

7	After receiving the RTS frame successfully, the recipient shall transmit a CTS frame to the device that has

8	sent the RTS frame. CTS frame carries a Duration field that holds a time value that indicates the duration for

9	which the sending device expects the medium to be busy referenced from the end of the last symbol of the

10	PPDU carrying the MAC frame. Any device other than the addressed device that successfully demodulates

11	the frame should stop competing for the transmission until the time indicated by the Duration field expires.

12

13	5.1.2.3 Multiple modulation bandwidths supporting

14

15	The coordinator and the devices in the OWPAN may support different modulation bandwidths and the

16	coordinator has no prior knowledge of which device will win during the contention in the CAP. To support

17	multiple modulation bandwidths, the RTS/CTS mechanism shall be used (see 5.1.2.2).

18

19	The coordinator records the supported modulation bandwidth of each associated device in its OWPAN

20	during association (see clause 5.1.6).

21

22	The coordinator and a device shall set the operation modulation bandwidth according to the following rules:

23

24	The downlink PPDUs shall be modulated with the modulation bandwidth of BW_SEL1 that is equal to or

25	lower	than	MAX_BW_SEL1.	MAX_BW_SEL1	equals	to	min{max

26	COODINATOR_Tx_modulation_bandwidth,  max  DEVICE_Rx_modulation_bandwidth}  ,  the  PPDU

27	carrying the corresponding ACK frames shall be modulated on the modulation bandwidth of BW_SEL2 that

28	is	equal	to	or	lower	than	MAX_BW_SEL2.	MAX_BW_SEL2	equals	to	min{max

29	COODINATOR_Rx_modulation_bandwidth, max DEVICE_Tx_modulation_bandwidth}.

30

31	The "max COODINATOR_Tx_modulation_bandwidth" represents the maximum modulation bandwidth

32	that the transmitter of the coordinator can support. "max DEVICE_Rx_modulation bandwidth" represents

33	the   maximum   modulation   bandwidth   that   the   receiver   of   a   device   can   support.   The   "max

34	COODINATOR_Rx_modulation_bandwidth" represents  the  maximum  modulation  bandwidth  that  the

35	receiver  of  the  coordinator  can  support.  "max  DEVICE_Tx_modulation_bandwidth" represents  the

36	maximum modulation bandwidth that the transmitter of a device can support.

37

38	The uplink PPDUs shall be modulated on the modulation bandwidth of BW_SEL2 that is equal to or lower

39	than MAX_BW_SEL2. MAX_BW_SEL2 equals to min{max COODINATOR_Rx_modulation_bandwidth,

40	max DEVICE_Tx_modulation_bandwidth}. And the PPDU carrying the corresponding ACK frames shall

41	be  on  the  modulation  bandwidth  of  BW_SEL1  that  is  equal  to  or  lower  than  MAX_BW_SEL1.

42	MAX_BW_SEL1	equals	to	min{max	COODINATOR_Tx_modulation	bandwidth,	max

43	DEVICE_Rx_modulation bandwidth}.

44

45	Regarding the broadcast downlink transmission, the coordinator shall modulate the frames on the minimum

46	bandwidth supported by any device. This minimum bandwidth is configured by the network and not in the

47	scope of this specification.

48

49	The following RTS/CTS protocol may be applied for bandwidth negotiation. The coordinator shall indicate

50	the application of the RTS/CTS protocol in the in the CAP Descriptor field of the beacon, see 5.2.2.1.4. If

51	RTS/CTS protocol is disabled, the minimum bandwidth shall be used for transmissions in the CAP.

52

53	A device may compete for the transmission of the RTS frame first. When the device gains the right for

54	transmission after  contention it  shall  transmit  an  RTS  frame.  RTS  is  transmitted using  the  minimal

32	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

bandwidth. The RTS frame shall include modulation bandwidth that is recommended by the device, which              1 will be used by the coordinator to determine the bandwidth for the device to transmit the subsequent data/              2 command frame.                                                                                                                                                                3

4

originator	recipient	5

6

7

RTS frame (contain recommended bandwidth)	8

9

10

CTS frame (contain assigned bandwidth)	11

12

13

Data / Command frame	14

15

16

ACK	17

18

19

20

21

22

RTS: modulated with minimal bandwidth	23

CTS: modulated with minimal bandwidth	24

Data/Command frame : modulated with assigned bandwidt	25

ACK: modulated with minimal bandwidth	26

27

Figure 17—An example of the usage of RTS/CTS	28

29

30

After receiving the RTS frame successfully, the coordinator shall transmit a CTS frame to the device that has              31 sent the RTS frame. Upon the evaluation of the recommended bandwidth in the received RTS frame, the              32 coordinator shall inform the device about the bandwidth assigned for mutual communications using the CTS              33 frame. The CTS frame shall include the modulation bandwidth that is assigned by the coordinator to the              34 device, and which will be used for the device to transmit the subsequent data/command frame.                                    35

36

The device shall transmit the data/command frame using the modulation bandwidth assigned in the CTS              37 frame, after if it has received the corresponding CTS frame after the RTS frame it has sent. If the device does              38 not receive the CTS frames during SIFS after it sent the RTS frame, it shall not transmit the data/command              39 frame and shall try to compete to resend the RTS frame. The coordinator may send an ACK frame following              40 the data/command frame, if requested. Transmission of a frame sequence including a data/command frame              41 with ACK using RTS/CTS is presented in Figure 17.                                                                                                       42

43

In this RTS/CTS mechanism, the coordinator and a device shall set the operation modulation bandwidth	44 according to the following rules:	45

46

The  PPDU  carrying  a  RTS/CTS/ACK frame  shall  be  modulated on  the  minimum of  the  maximum              47 modulation bandwidths supported by both devices. The PPDUs carrying data/command shall be modulated              48 on the modulation bandwidth that is indicated in the CTS frame.                                                                                    49

50

For the downlink transmissions, the coordinator shall use the same RTS/CTS protocol described above.	51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	33

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.2.4 Polling

2

3	The start of a transmission period is marked by the transmission of a beacon frame, which carriers the infor-

4	mation required for new STAs to initiate an association procedure. Following the beacon frame transmis-

5	sion, a contention-based transmission period is enabled. The duration of the period is measured in terms of

6	transmission slots, which the coordinator node enables via its polling mechanism. Each frame transmission

7	(other than the Beacon frame transmission) by the coordinator can be used to poll one or multiple STAs.

8	During the contention-based period, all STAs are effectively polled by the coordinator and, as soon as they

9	recognize the beginning of the polling frame (polling information is contained in the frame header at the

10	beginning of the frame), they have the possibility to transmit.  Collisions result in failed frame reception,

11	which results in lack of acknowledgments and retransmission of the respective frames. Acknowledgments

12	and retransmissions are described in subsection ….. A STA which does not take the opportunity to transmit

13	when polled during the CP needs to wait for the next valid frame transmitted by the coordinator, which

14	marks the beginning of the next contention slot, in order to have the opportunity to transmit again. The poll-

15	ing mechanism enables very low-complexity synchronization of the transmission slots and also enables the

16	coordinator to dynamically adapt and assign the slots for transmission. The duration of the CP is measured

17	in terms of transmission slots and can be configured at the coordinator node. Following the CP, a CFP is

18	enabled, during which the coordinator node polls each STA individually and allows it to transmit without

19	any interference from other STAs. The specific polling algorithm and order is outside the scope of this spec-

20	ification and is left as a system designer's choice. STA association and disassociation begins in the CP, but

21	can be completed during the subsequent CFP. The concept is presented in Fig. 46.

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40	Figure 18—Superframe structure example with four STAs

41

42

43	5.1.2.4.1 As s o c iatio n e xample us ing po lling

44

45	The AP sends a Beacon frame periodically at the beginning of a polling cycle. In a short contention period

46	after receiving the Beacon frame, the STA can reply with a "connection request". The first valid downlink

47	frame after the beacon marks the beginning of the contention period, i.e., the first valid frame after the bea-

48	con is interpreted as a poll to all stations that have not been connected to transmit their "connection request".

49	All stations that have already been associated with the AP may ignore any polling during the contention

50	period. If no stations have been connected to the AP, the first valid frame would be the subsequent beacon

51	frame (see Fig. 47 for a description of this procedure). The AP starts polling a station in the next polling

52	round after receiving the "connection request". In case several STAs want to connect at the same time, a col-

53	lision occurs. As a solution, a random backoff is introduced, for example, one STA retries to establish a con-

54	nection after 2 beacons, another STA retries after 5 beacons, etc. If it is not polled during the next polling

34	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

round, a STA assumes a collision has occurred and proceeds accordingly with a back-off and an attempt to	1 reconnect. A STA transmits a poll request frame immediately after it decodes a valid MAC frame header.	2

Hence, as depicted in Fig. 47, it will not wait for the entire frame on the downlink from the AP to be trans-              3 mitted. Upon a successful polling request (the AP has successfully received the station's request to be polled              4 and has indicated this by polling the station within the next transmission round), a station exchanges the nec-              5 essary association and authentication information with the AP via association and authentication control              6 frames. Upon successful authentication and association, the STA is assigned a unique 16-bit address which              7 will identify the STA among all other devices connected to the same AP via the VLC link.                                           8

9

10

11

12

13

14

15

16

17

18

19

20 (a) Case1: No STAs are connected to the AP	21

22

23

24

25

26

27

28

29

30

31 (b) Case2: At least one STA is connected to the AP	32

33

Figure 19—A STA sending a connection request to the AP	34

35

36

5.1.2.4.2 Multiple ac c e s s e xample us ing po lling	37

38

The multiple access in the presented MAC protocol is based on the function of a PCF which provides CF	39

frame transfer. The PC shall reside in the AP. It is also a requirement for a STA to be able to respond to a poll	40

request received from a PC. A STA should also be able to request to be polled by an active PC as described	41

in subclause 6.2.1.9.1. When polled by the PC, a STA may transmit only one MPDU, which can be sent to	42

the PC but may have any destination. The acknowledgments for any data packet "piggyback" on the trans-	43

mission of any management or data frame (including Null frames). If a frame is not acknowledged, the STA	44

shall not retransmit the frame unless it is polled again by the PC. If a polled STA does not have any data to	45

transmit, it simply ignores the polling request. The request times out after a specified period of time, which	46

indicates to the PC that the polled station might either be out of range or will not take advantage of the pos-	47

sibility to transmit.	48

49

Any frames transmitted from the AP to the STA are treated as polling frames, except for the Beacon frame.	50

This can be assumed because every packet transmission contains a polled STA number, which enables the	51

stations to keep track of the polling order. A newly connected STA is made aware of its queue number with	52

the first valid packet it receives from the AP, which contains both its queue number and its MAC address.	53

Furthermore, the AP will never transmit a frame before the uplink from a STA due to a previous poll has	54

Copyright © 20xx IEEE. All rights reserved.	35

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	been completed. Hence, no collisions in the uplink are possible and each donwlink packet transmission can

2	function as a polling frame. In case the response to a poll request is not detected by the AP, the AP will

3	attempt to poll the next STA in the queue. Upon detection of a subsequent downlink packet by the STA

4	whose response was not detected, the STA will stop transmission immediately in order to avoid any colli-

5	sions in the uplink as illustrated in Figure 20. This functionality is necessary because the AP does not expect

6	to always receive a response to its poll. If a STA has no acknowledgment or information to transmit, it will

7	simply ignore its possibility to transmit. An AP always sends polling frames in order. In case there is no

8	information to be transmitted to a STA, which is next in the queue, a Data Null frame is used as a polling

9	frame.

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25	Figure 20—Consequences of the AP failing to recognize a poll response

26

27

28	5.1.2.4.3 Ac kno wle dg e me nt and Re trans mis s io n Example Us ing Po lling

29

30	The reception of every packet at the MAC layer level has to be acknowledged by the receiving side (AP or

31	STA). If a packet is not acknowledged by the STA or the AP at the following polling round, then it is retrans-

32	mitted during the subsequent polling cycle. Packets may arrive out of order. Hence, even if a STA or an AP

33	does not receive the expected acknowledgment during a given polling round, it can proceed with the trans-

34	mission of the next packet. If packets are not acknowledged in the course of a predetermined time period

35	equal to four transmission opportunities for the acknowledging device, they are considered lost and are

36	dropped from the transmission. Figure 21 illustrates the acknowledgment procedure.

37

38	The principle of operation is illustrated in Figure 21 for transmission on the downlink. The principle of oper-

39	ation is analogous for the uplink direction. Two STAs are already connected to the AP. The AP sends a data

40	frame to the first STA, which also polls the STA. The STA responds to the poll with a payload frame or a

41	management frame as soon as it manages to decode the MAC header of the received frame. Note that if the

42	high-reliability MAC header encoding at the PHY layer is implemented, then the STA responds as soon as

43	this information is decoded at the PHY layer and the MLME is notified even before the entire DATA portion

44	of the PHY packet is received and decoded. After both the downlink and the uplink packet transmissions are

45	complete, the AP sends a data frame to the second STA, which also replies to the poll with a payload frame

46	as soon as the PHY header is decoded. After completion of the transmission in both directions, the AP

47	attempts to send a second data frame to the first STA. This time a CRC error is detected at the PHY layer, but

48	the packet transmission is correctly detected by the STA, which interprets this as an invitation to transmit

49	and responds with an acknowledgment to the first data packet in the downlink (note that if the high-reliabil-

50	ity MAC header encoding at the PHY layer is not used, the entire packet would be dropped without the STA

51	being polled, so uplink transmission would not be possible). The acknowledgment in this case is part of a

52	Null frame, but can also be transmitted with a data frame if the STA had any data to upload. After transmis-

53	sion is complete in both directions, the AP sends a second data packet to the second STA. The STA detects

54

36	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

this and after decoding the header, transmits an acknowledgment for the reception of the first data packet.	1

Upon completion of both transmissions, the AP sends a third data packet to the first STA. The STA responds              2 with a management frame, but does not provide any acknowledgments as the second data packet was not              3 successfully received during the previous transmission cycle. In this case, the STA could simply ignore the              4 possibility to transmit if no management information or data is available for transmission. In both cases, the              5

AP would recognize the lack of an acknowledgment for the second data packet which failed to successfully              6 reach the first STA. Upon completion of transmission in both directions, the AP sends a third data packet to              7 the second STA. This time an error at the PHY layer causes the packet to not get detected at the PHY layer.              8

As a result, the STA does not recognize the poll and does not transmit any data. After completing transmis-	9 sion, the AP transmits another data frame to the first STA. In this case, the second packet is retransmitted	10 since no acknowledgment was received at the AP during the previous transmission cycle. At this point, the	11

AP will keep transmitting the second data packet at each polling cycle, until an acknowledgment is received	12 from the STA or until enough unsuccessful attempts are made so that the AP decides to drop the packet. The	13

STA correctly identifies the packet header and transmits a response with an acknowledgment for the third              14 data packet, which was received during the previous transmission round. Upon completion of both transmis-              15 sions, the AP sends the fourth data packet to the second STA. The STA recognizes the header and sends a              16 response in a management frame. In this case, again, the STA could simply ignore the possibility to transmit              17 if no management information or data is available for transmission. In both cases, the AP would recognize              18 the lack of an acknowledgment for the third data packet which failed to successfully reach the second STA.              19

Upon completion of the bidirectional transmission, the AP transmits the fourth data packet to the first STA.	20

The STA recognizes the header and transmits an acknowledgment for the second data packet, which was	21 successfully retransmitted during the previous round. After completion of the bidirectional transmission, the	22

AP retransmits the third data packet to the second STA for which it did not receive an acknowledgment in              23 the previous round. The STA recognizes the header and attempts to transmit an acknowledgment for the              24 fourth data packet which was successfully received during the previous round. The transmission, however,              25 contains at least one error, which causes the PHY layer algorithm at the AP to drop the data, and, hence, the              26

MAC layer algorithm does not receive the acknowledgment for the fourth data packet. Upon completion of              27 both data transmissions, the AP continues with transmission of the fifth data packet to the first STA. It              28 should be noted that if for any reason the AP failed to recognize the end of the uplink transmission from the              29 second STA, which could be due to the fact that the packet was not detected at all or due to the fact that              30 because of a PHY layer error the packet length was not correctly interpreted, and began transmitting the next              31 packet to the first STA prematurely, the second STA would have discontinued its transmission immediately              32 upon detecting the next donwlink packet transmission. This example was explained in subclause 6.2.1.9.2              33 and illustrated in Fig. 49. All subsequent packet transmissions in Fig. 50 are successful and analogous to the              34 already explained examples. It is interesting to note that since the acknowledgment for the fourth data packet              35 to the second STA was unsuccessful, the AP transmits this packet a second time (in the last illustrated              36 frame), so the STA receives the packet successfully twice. The MAC layer protocol is able to handle the              37 packet redundancy as well as reordering based on the packet sequence numbers described in subclause              38

6.4.1.2.	39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	37

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9	(a) No packet loss

10

11

12

13

14

15

16

17

18	(b) Various packet loss and retransmission scenarios.

19	Figure 21—Downlink transmission with two STAs

20

21

22

23	5.1.2.5 Visibility support during channel access

24

25

26

27	star topology

28	(infrastructure)

29

beacon

contention

uplink

downlink

send visibility pattern during contention/uplink/unused

30

31

32		full-duplex for visibility

33

34

35

36

37

38

39	all modes

40

41

42

43

44		full-duplex for visibility

45

46

47

visibility for infrastructure  (downlink)

(a)

unused slots 	unused slots

visibility for point-and-shoot (uplink)

and for infrastructure  (downlink)

(b)

slots in star toplogy

send visibility pattern during unused slots in all applications

48	Figure 22—Usage of CVD frames during idle or RX modes of operation

49

50

51	The visibility slots can be used during contention and uplink slots in star topology mode and unused slots in

52	all modes to maintain visibility, reduce flicker and keep the transmitter always “ON” for the infrastructure.

53	This is shown in Figure 22. Visibility support is a very important distinguishing feature for OWC. One may

54	need to transmit idle patterns during receive and idle modes. This can be done by simultaneous reception of

38	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

data and the transmission of visibility or idle patterns. This is possible due to spatial separation of the light              1 source and the receiving circuitry. As shown in Figure 22, idle patterns are sent during contention, uplink              2 slots and unused downlink slots by the infrastructure to maintain visibility. Idle patterns are also sent during              3 unused slots by the mobile device to help with pointing and alignment for optimal data transfer.                                  4

5

If the continuous visibility bit is set in the capabilities field shown in Table 37, then infrastructure devices              6 shall provide continuous visibility.                                                                                                                                   7

8

9

10

5.1.3 Starting a OWPAN                                                                                                                                11

12

5.1.3.1 Scanning through channels                                                                                                            13

14

A scan procedure shall be performed by a device or a prospective coordinator to detect any OWPANs and              15 coordinators that are operating in its vicinity. Two types of scanning are specified, active scan, passive scan,              16 and a third type, scan-over-backhaul, is enabled via the use of specified primitives. All devices shall be              17 capable of performing passive scans across a specified list of channels. In addition, a coordinator shall be              18 able  to  perform  active  scans. A device  is  instructed  to  begin  a  channel  scan  through  the  MLME-              19

SCAN.request primitive. For the duration of the scan, the device shall suspend beacon transmissions, if              20 applicable, and shall only accept frames received over the PHY data service that are relevant to the scan              21 being performed. Upon the conclusion of the scan, the device shall recommence beacon transmissions. The              22 results of the scan shall be returned via the MLME-SCAN.confirm primitive.                                                               23

24

5.1.3.1.1 Ac tive c hanne l s c an                                                                                                                                    25

26

It is anticipated that the active channel scan is used with the peer-to-peer topology and the coordinated              27 topology.                                                                                                                                                                            28

29

An active scan allows a device to locate any coordinator transmitting beacon frames within its coverage              30 area. This could be used by a prospective OWPAN coordinator to select a OWPAN identifier prior to starting              31 a new OWPAN, or it could be used by a device prior to association. This could also be used by a device              32 when it is requested by the coordinator to report a neighboring OWPANs descriptors list.                                            33

34

During an active scan, the MAC sublayer shall discard all frames received over the PHY data service that              35 are not beacon frames. If a beacon frame is received that contains the address of the scanning device in its              36 list of pending addresses, the scanning device shall not attempt to extract the pending data.                                          37

38

Before commencing an active scan, the MAC sublayer shall store the value of macOWPANId and then set it              39 to 0xffff for the duration of the scan. This enables the receive filter to accept all beacons rather than just the              40 beacons from its current OWPAN, see 5.1.9.2. On completion of the scan, the MAC sublayer shall restore              41 the value of macOWPANId to the value stored before the scan began. An active scan is requested using the              42

MLME-SCAN.request primitive with the ScanType parameter set to indicate an active scan. The device              43 shall send a beacon request command, see 5.4.11. Upon successful transmission of the beacon request              44 command, the device shall enable its receiver for at most [aBaseSuperframeDuration × (2n + 1)] optical              45 clocks, where n is the value of the ScanDuration parameter. During this time, the device shall reject all              46 nonbeacon frames and record the information contained in all unique beacons in a OWPAN descriptor              47 structure, see 6.3.3.1. If a beacon frame is received when macAutoRequest is set to TRUE, the list of              48

OWPAN descriptor structures shall be stored by the MAC sublayer until the scan is complete; at this time,              49 the list shall be sent to the next higher layer in the OWPANDescriptorList parameter of the MLME-              50

SCAN.confirm primitive. A device shall be able to store between one and an implementation-specified             51 maximum number of OWPAN descriptors. A beacon frame shall be assumed to be unique if it contains both              52 a OWPAN identifier and a source address that has not been seen before during the scan. If a beacon frame is              53 received when macAutoRequest is set to FALSE, each recorded OWPAN descriptor is sent to the next higher              54

Copyright © 20xx IEEE. All rights reserved.	39

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	layer in a separate MLME-BEACON-NOTIFY. indication primitive. A received beacon frame containing

2	one or more octets of payload shall also cause the OWPAN descriptor to be sent to the next higher layer via

3	the MLME-BEACON-NOTIFY.indication primitive. If a  protected beacon frame is received, i.e., the

4	Security Enabled subfield in the frame control field is set to one, the device shall attempt to unsecure the

5	beacon frame using the unsecuring process described in 7.2.3. The security-related elements of the OWPAN

6	descriptor corresponding to the beacon, see 6.3.3.1, shall be set to the corresponding parameters returned by

7	the unsecuring process. The SecurityFailure element of the OWPAN descriptor shall be set to SUCCESS if

8	the status from the unsecuring process is SUCCESS and set to one of the other status codes indicating an

9	error in the security processing otherwise. The information from the unsecured frame shall be recorded in

10	the OWPAN descriptor even if the status from the unsecuring process indicated an error. If a coordinator of

11	a beacon-enabled OWPAN receives the beacon request command, it shall ignore the command and continue

12	transmitting its periodic beacons as usual. If a coordinator of a nonbeacon-enabled OWPAN receives this

13	command, it shall transmit a single beacon frame using unslotted random access or unslotted CSMA-CA.

14

15	If macAutoRequest is set to TRUE, the active scan shall terminate when the number of beacons found equals

16	the implementation-specified limit or the channel has been scanned for the full time, as specified in

17	5.1.3.1.1. If macAutoRequest is set to FALSE, the active scan shall terminate when the channel has been

18	scanned for the full time. If a channel was not scanned for the full time, it shall be considered to be

19	unscanned.

20

21	5.1.3.1.2 P as s ive c hanne l s c an

22

23	It is anticipated that the passive channel scan is used with the star or broadcast topology.

24

25	A passive scan, like an active scan, allows a device to locate any coordinator transmitting beacon frames

26	within its coverage area. The beacon request command, however, is not transmitted. This type of scan could

27	be used by a device prior to association. During a passive scan, the MAC sublayer shall discard all frames

28	received over the PHY data service that are not beacon frames. If a beacon frame is received that contains

29	the address of the scanning device in its list of pending addresses, the scanning device shall not attempt to

30	extract the pending data.

31

32	Before commencing a passive scan, the MAC sublayer shall store the value of macOWPANId and then set it

33	to 0xffff for the duration of the scan. This enables the receive filter to accept all beacons rather than just the

34	beacons from its current OWPAN, see 5.1.9.2. On completion of the scan, the MAC sublayer shall restore

35	the value of macOWPANId to the value stored before the scan began. A passive scan is requested using the

36	MLME-SCAN.request primitive with the ScanType parameter set to indicate a passive scan. The device

37	shall enable its receiver for at most [aBaseSuperframeDuration × (2n + 1)] optical clocks, where n is the

38	value of the ScanDuration parameter. During this time, the device shall reject all nonbeacon frames and

39	record the information contained in all unique beacons in a OWPAN descriptor structure, see 6.3.3.1. If a

40	beacon frame is received when macAutoRequest is set to TRUE, the list of OWPAN descriptor structures

41	shall be stored by the MAC sublayer until the scan is complete; at this time, the list shall be sent to the next

42	higher layer in the OWPANDescriptorList parameter of the MLME-SCAN.confirm primitive. A device shall

43	be able to store between one and an implementation- specified maximum number of OWPAN descriptors. A

44	beacon frame shall be assumed to be unique if it contains both a OWPAN identifier and a source address that

45	has not been seen before during the scan of the current channel. If a beacon frame is received when

46	macAutoRequest is set to FALSE, each recorded OWPAN descriptor is sent to the next higher layer in a

47	separate  MLME-BEACON-NOTIFY.  indication  primitive.  Once  the  scan  is  complete,  the  MLME-

48	SCAN.confirm shall be issued to the next higher layer with a null OWPANDescriptorList. A received

49	beacon frame containing one or more octets of payload shall also cause the OWPAN descriptor to be sent to

50	the next higher layer via the MLME-BEACON-NOTIFY. indication primitive.

51

52	If a protected beacon frame is received (i.e., the Security Enabled subfield in the frame control field is set to

53	one), the device shall attempt to unsecure the beacon frame using the unsecuring process described in 7.2.3.

54

40	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The security-related elements of the OWPAN descriptor corresponding to the beacon, as shown in 6.3.3.1,              1 shall be set to the corresponding parameters returned by the unsecuring process. The SecurityFailure              2 element of the OWPAN descriptor shall be set to SUCCESS if the status from the unsecuring process is              3

SUCCESS and set to one of the other status codes indicating an error in the security processing otherwise.                 4

5

The information from the unsecured frame shall be recorded in the OWPAN descriptor even if the status              6 from the unsecuring process indicated an error.                                                                                                               7

8

If macAutoRequest is set to TRUE, the passive scan shall terminate when the number of beacons found	9 equals  the  implementation  specified  limit  or  the  channel  has  been  scanned  for  the  full  time.  If	10 macAutoRequest is set to FALSE, the passive scan shall terminate when the channel has been scanned for	11 the full time. If a channel was not scanned for the full time, it shall be considered to be unscanned.	12

13

5.1.3.1.3 Scan-over-backhaul                                                                                                                      14

15

Scan-over-backhaul shall be performed by a prospective coordinator that plans to establish a OWPAN if              16 inter-coordinator communication over backhaul is feasible.                                                                                            17

18

Scan-over-backhaul is requested by the next higher layer of the prospective coordinator using the MLME-              19

SCAN.request primitive with the ScanType parameter set to indicate a scan-over-backhaul, as defined in              20

TBD. On reception of the primitive, the MLME of the prospective coordinator shall generate a scan-over-              21 backhaul request command (see TBD) and send it to neighboring coordinators that are connected to the              22 prospective coordinator through the backhaul. After the prospective coordinator transmitted the scan-over-              23 backhaul request command, it shall enable its receiver (on the backhaul) to receive scan-over-backhaul              24 confirmation commands (as specified in TBD) sent by other coordinators for ScanDuration. Coordinators              25 that have received the scan-over-backhaul request command through the backhaul shall respond with a scan-              26 over-backhaul confirmation command, with its own OWPAN descriptor embedded in it. When the MLME              27 of the prospective coordinator receives a scan-over-backhaul confirmation command, it shall record the              28 information contained in the unique scan-over-backhaul confirmation command in a local neighboring              29

OWPAN descriptor list. A scan-over-backhaul confirmation command frame shall be assumed to be unique              30 if it contains both a OWPAN ID and a source address that has not been seen before. When the ScanDuration              31 time  expires,  the  MLME  of  the  prospective coordinator shall  report  the  local  neighboring OWPAN              32 descriptor list to the next higher layer by issuing a MLME-SCAN.confirm primitive.                                                   33

34

5.1.3.2 OWPAN initiation                                                                                                                              35

36

A prospective coordinator shall perform the following procedures to establish an OWPAN.                                         37

38

On receipt of the primitive MLME-RESET.request, the MLME of the prospective coordinator shall turn off              39 the transceiver by issuing a PLME-SET-TRX-STATE.request to the PHY layer, and then the MAC sublayer              40 is set to its initial conditions, clearing all internal variables to their default values.                                                        41

42

The MLME of the device shall respond with a MLME-RESET.confirm primitive to notify the next higher              43 layer the result of the reset operation.                                                                                                                               44

45

After the reset, the next higher layer of the prospective coordinator shall issue a MLME-SCAN.request              46 primitive to require the MLME to perform a scan to discover other OWPANs in its vicinity. Two types of              47 scan may be performed. If inter-coordinator communication over backhaul is feasible, then a scan-over-              48 backhaul is performed. Otherwise, a passive scan shall be performed. The specific scan type shall be indi-              49 cated by the next higher layer in the MLME-SCAN.request primitive. After the requested scan is done, the              50

MLME shall issue a MLME-SCAN.confirm primitive to report the result of the scan.                                                  51

52

The next higher layer of the prospective coordinator shall select an OWPAN ID and the short address of the              53 coordinator based on the OWPAN IDs and the short addresses recorded in the local neighboring OWPANs              54

Copyright © 20xx IEEE. All rights reserved.	41

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	descriptor list obtained via the scan once it has received the MLME-SCAN.confirm. The MLME of the pro-

2	spective coordinator shall select an OWPAN ID and short address that are different from the OWPAN IDs

3	and short addressed obtained via the scan. Then the next higher layer shall provide the selected OWPAN ID

4	and short address to the MLME by issuing a MLME-START.request primitive.

5

6	On receipt of the MLME-START.request, the MLME of the prospective coordinator shall start a superframe

7	and broadcast beacon frames with the selected parameters (e.g., OWPAN ID, short address) periodically.

8

9	5.1.3.3 Beacon generation

10

11	A device shall be permitted to transmit beacon frames only if macShortAddress is not equal to 0xffff.

12

13	A coordinator shall use the MLME-START.request primitive to begin transmitting beacons only if the

14	BeaconOrder parameter is less than 15. The coordinator may begin beacon transmission as the coordinator

15	of a new OWPAN.

16

17	For the coordinator (i.e., the OWPANCoordinator parameter is set to TRUE), the MAC sublayer shall ignore

18	the StartTime parameter and begin beacon transmissions immediately. Setting the StartTime parameter to

19	zero shall also cause the MAC sublayer to begin beacon transmissions immediately.

20

21	On receipt of the MLME-START.request primitive, the MAC sublayer shall set the OWPAN identifier in

22	macOWPANId and use this value in the Source OWPAN Identifier field of the beacon frame. The address

23	used in the Source Address field of the beacon frame shall contain the value of aExtendedAddress if

24	macShortAddress is equal to 0xfffe or macShortAddress otherwise.

25

26	The time of transmission of the most recent beacon shall be recorded in macBeaconTxTime and shall be

27	computed so that its value is taken at the same position in each beacon frame, the location of which is

28	implementation specific. The position, which is specified by the macTimeStampOffset attribute, is the same

29	as that used in the timestamp of the incoming beacon frame, as described in 5.1.7.1.

30

31	All beacon frames shall be transmitted at the beginning of each superframe at an interval equal to aBase-

32	SuperframeDuration  2n optical clocks, where n is the value of macBeaconOrder (the construction of the

33	beacon frame is specified in 5.2.2.1).

34

35	Beacon transmissions shall be given priority over all other transmit and receive operations.

36

37	5.1.3.4 Device discovery

38

39	The coordinator indicates its presence on a OWPAN to other devices by transmitting beacon frames. This

40	allows the other devices to perform device discovery.

41

42	Device discovery shall be performed at 11.67 Kbps with a 200 KHz optical clock for PHY I and at 1.25

43	Mbps with a 3.75 MHz optical clock for PHY II. PHY III does not provide device discovery support and

44	shall rely on device discovery using PHY II before operating in that mode. The dimmed OOK mode can be

45	used to support dimming in the device discovery process. This mode is indicated using the MAC PIB

46	attribute, macUseDimmedOOKmode, as defined in Table 91. The MAC and PHY capabilities are exchanged

47	in the device discovery process. The clock rate support capabilities are also exchanged. Once the capabilities

48	are exchanged, regular data transmission mode resumes for all three PHY types. Device discovery requires

49	bi-directional communication and is not applicable for broadcasting.

50

51

52

53

54

42	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.1.4 Neighboring OWPAN status monitoring                                                                                            1

2

Neighboring OWPANs status monitoring procedure allows the coordinator and the devices to detect and              3 maintain neighboring OWPANs information. Neighboring OWPAN status monitoring maybe performed for              4 the purposes of OWPAN maintenance, interference coordination and handover.                                                            5

6

Both  coordinators  and  the  devices  shall  participate  in  the  neighboring  OWPANs  status  monitoring              7 procedure. The information of neighboring OWPANs that can be detected shall be recorded in a neighboring              8

OWPANs descriptor list.                                                                                                                                                   9

10

Both the coordinator and the devices shall each maintain a local neighboring OWPANs descriptor list             11 respectively. The local neighboring OWPAN descriptor list is maintained by receiving beacon frames or              12 other frames from the neighboring OWPANs. When a beacon frame or any other frame from neighboring              13

OWPANs is received for the first time, a new record corresponding to the neighboring OWPAN shall be              14 added to the list. The record in the list has an ageing time of macAgeingTime superframes. Whenever a              15 beacon frame or any other frame from a neighboring OWPAN corresponding to a record in the list has been              16 received, the device shall update the LastTimeDetected of the record instead of adding a new record. Once              17 the beacon frames or any other frame from a neighboring OWPAN corresponding to a record in the list has              18 not been received within the ageing time since last time it was detected, the record corresponding to that              19 neighboring OWPAN shall be deleted from the list.                                                                                                         20

21

The devices shall update their local neighboring OWPANs descriptor list and send a neighboring OWPAN              22 report indication command to the coordinator to report its local neighboring OWPANs descriptor list              23 whenever any of the following events occurs:                                                                                                                  24

25 a)	It has received a beacon frame or any other frames sent by the neighboring coordinators or devices		26 for the first time;	27

b)	It has been macLastDetect superframes since the neighboring OWPAN was detected last time;	28 c)	A neighboring OWPAN report request command is received by the device from the coordinator;	29

30

The coordinator shall update its local neighboring OWPAN descriptor list whenever any of the following              31 events occurs:                                                                                                                                                                    32

33 a)	It has received a beacon frame or any other frames sent by the neighboring coordinators or devices		34 for the first time;	35

b)	It has been macLastDetect superframes since the neighboring OWPAN was detected last time;	36

37

The coordinator shall also maintain a global neighboring OWPAN descriptor list. The global neighboring              38

OWPAN descriptor list is maintained and updated by gathering the reported local neighboring OWPAN              39 descriptor lists from devices and its own local neighboring OWPAN descriptor list. The global neighboring              40

OWPANs descriptor list is shown in Table 3.                                                                                                                   41

42

The coordinator shall update its global neighboring OWPAN descriptors list whenever it has received a              43 neighboring OWPAN report indication command from devices in the OWPAN, or when its own local             44 neighboring OWPAN descriptors list has been updated. The coordinator shall also update the global              45 neighboring OWPAN descriptors list by deleting the corresponding information if the device which has              46 reported the corresponding information is disassociated from the OWPAN.                                                                   47

48

5.1.5 Maintaining OWPANs                                                                                                                          49

50

In some instances a situation could occur in which two OWPANs exist in the same operating space with the              51 same OWPAN identifier. If this conflict happens, the coordinator and its devices shall perform the OWPAN              52 identifier conflict resolution procedure.                                                                                                                            53

54

Copyright © 20xx IEEE. All rights reserved.	43

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 3—Glo bal ne ig hbo ring OWPANs de s c riptio n lis t

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| NumberNeighbor- ingOWPANRe- cords | Integer | 0x00 - 0xff | The number of records of neighboring OWPANs that are maintained by the coordinator. |
| OWPANDescrip- tion[0] |  | Refer to Table yy | The record of the first neighboring OWPAN that can be detected by devices. |
| NumberDetectDe- vice | Integer | 000 - oxff | The number of devices that have detected the neigh- boring OWPAN specified by OWPANDescriptor [0]. Assuming the total number is K. |
| DeviceAddr[0] | Integer |  | The short address of the first device that has reported it can detect the neighboring OWPAN specified by OWPANDescriptor [0]. |
| LinkQuality[0] |  |  | The RSS at which the first device receives the bea- con frame or other frames from the OWPAN speci- fied by OWPANDescriptor [0]. |
| DeviceAddr[k-1] | Integer |  | The short address of the Kth device that has reported it can detect the neighboring OWPAN specified by OWPANDescriptor [0]. |
| LinkQuality[K-1] |  |  | The RSS at which the Kth device receives the bea- con frame or other frames from the OWPAN speci- fied by OWPANDescriptor [0]. |
| OWPANDescrip- tor[N-1] |  | Refer to Table yy | The record of the Nth neighboring OWPAN that can be detected by devices. |
| NumberDetectDe- vice | Integer | 0x00 - 0xff | The number of devices that have detected the neigh- boring OWPAN specified by OWPANDescriptor [N-1]. Assuming the total number is L. |
| DeviceAddr[0] | Integer |  | The short address of the first device that has reported it can detect the neighboring OWPAN specified by OWPANDescriptor [N-1]. |
| LinkQuality[0] |  |  | The RSS at which the first device receives the bea- con frame or other frames from the OWPAN speci- fied by OWPANDescriptor [N-1]. |
| DeviceAddr[L-1] | Integer |  | The short address of the Lth device that has reported it can detect the neighboring OWPAN specified by OWPANDescriptor [N-1]. |
| LinkQuality[L-1] |  |  | The RSS at which the Lth device receives the bea- con frame or other frames from the OWPAN speci- fied by OWPANDescriptor [N-1]. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46	This procedure is optional for a device.

47

48	5.1.5.1 Detection

49

50	The OWPAN coordinator shall conclude that a OWPAN identifier conflict is present if either of the

51	following apply:

52

53	—	A beacon frame is received by the OWPAN coordinator with the OWPAN Coordinator subfield, see

54	5.2.2.1.2, set to one and the OWPAN identifier equal to macOWPANId.

44	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

—	A OWPAN ID conflict notification command, see 5.4.8, is received by the OWPAN coordinator	1 from an associated device on its OWPAN.	2

3

A device that is associated through the OWPAN coordinator (i.e., macAssociatedOWPANCoord is set to              4

TRUE) shall conclude that a OWPAN identifier conflict is present if the following applies:                                         5

6

—	A beacon frame is received by the device with the OWPAN Coordinator subfield set to one, the	7

OWPAN   identifier   equal   to   macOWPANId,   and   an   address   that   is   equal   to   neither	8

macCoordShortAddress nor macCoordExtendedAddress.	9

5.1.5.2 Resolution                                                                                                                                         10

11

On the detection of a OWPAN identifier conflict by a device, it shall generate the OWPAN ID conflict noti-              12 fication command, defined in 6.7.5, and send it to its coordinator. Since the OWPAN ID conflict notification              13 command contains an acknowledgment request (see 6.7.3.1), the coordinator shall confirm its receipt by              14 sending an acknowledgment frame. Once the device has received the acknowledgment frame from the coor-              15 dinator, the MLME shall issue an MLME -SYNC -LOSS.indication primitive with the LossReason parame-              16 ter set to OWPAN_ID_CONFLICT. If the device does not receive an acknowledgment frame, the MLME              17 shall not inform the next higher layer of the OWPAN identifier conflict.                                                                        18

19

On the detection of a OWPAN identifier conflict by the coordinator, the MLME shall issue an MLME-              20

SYNC-LOSS.indication to the next higher layer with the LossReason parameter set to OWPAN_ID_CON-              21

FLICT. The next higher layer of the coordinator may then perform an active scan and, using the information              22 from the scan, select a new OWPAN identifier.                                                                                                                23

24

In addition, the next higher layer of the coordinator shall issue a MLME-NeighborReport.request primitive              25 to its MLME to trigger a neighboring OWPAN report. On receipt of the MLME-NeighborReport.request             26 primitive, the MLME of the coordinator shall generate a neighboring OWPAN report request command and              27 send it to the device(s) indicated in the ReportDevicesList of the MLME-NeighborOWPANReport.request             28 primitive. To which device(s) the neighboring OWPAN request command shall be sent is determined by the              29 next higher layer and is out of the scope of the specification. Then the coordinator shall wait for the neigh-              30 boring OWPAN report indication commands sent by the devices for macResponseWaitTime optical clocks.                31

32

Devices that have received the neighboring OWPAN report request command from the coordinator may first              33 perform an active scan. The device updates its local neighboring OWPAN descriptors list according to the              34 result obtained from the active scan. When the active scan requested by the coordinator is done, the device              35 shall generate a neighboring OWPAN report indication command based on its updated local neighboring              36

OWPAN descriptor list and send the neighboring OWPAN report indication command to the coordinator. If              37 the device choose not to perform the active scan after it receives the neighboring OWPAN report request              38 command, it shall generate a neighboring OWPAN report indication command which include its local neigh-              39 boring OWPAN descriptor list and send the command to the coordinator.                                                                      40

41

When the coordinator has received a unique neighboring OWPAN report indication command from the              42 device, it shall update its global neighboring OWPAN descriptor list. When macResponseWaitTime optical              43 clocks elapsed, the MLME of the coordinator shall notify its next higher layer the result of the requested              44 neighboring OWPAN report by issuing a MLME-NeighborReport.confirm primitive. The next higher layer              45 of the coordinator shall select a new OWPAN ID which is different from the OWPAN IDs recorded in its              46 updated global neighboring OWPAN descriptor list. The algorithm for selecting a suitable OWPAN identi-              47 fier is out of the scope of this standard. If the next higher layer does select a new OWPAN identifier, it may              48 then issue an MLME-START.request with the CoordRealignment parameter set to TRUE in order to realign              49 the OWPAN, as described in 6.2.3.3.                                                                                                                                50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	45

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.5.3 Realigning a OWPAN

2

3	If a coordinator receives the MLME-START.request primitive (see 7.3.11.1) with the CoordRealignment

4	parameter set to TRUE, the coordinator shall attempt to transmit a coordinator realignment command

5	containing the new parameters (e.g. OWPANId) and EffectiveTime.

6

7	When the coordinator is already transmitting beacons and the CoordRealignment parameter is set to TRUE,

8	the  next scheduled beacon shall be  transmitted on  the  current channel using the  current superframe

9	configuration, with the frame pending subfield of the frame control field set to one. Immediately following

10	the transmission of the beacon, the coordinator realignment command shall also be transmitted on the

11	current channel using unslotted random access.

12

13	When the coordinator is not already transmitting beacons and the CoordRealignment parameter is set to

14	TRUE, the coordinator realignment command shall be transmitted immediately on the current channel using

15	unslotted random access.

16

17	If the transmission of the coordinator realignment command fails due to a channel access failure, the MLME

18	shall notify the next higher layer by  issuing the  MLME -START.confirm primitive with a  status of

19	CHANNEL_ACCESS_FAILURE.  The  next  higher  layer  may  then  choose  to  issue  the  MLME-

20	START.request primitive again.

21

22	Upon successful transmission of the coordinator realignment command, the new superframe configuration

23	and channel parameters shall be  put  into operation as  described below. The coordinator realignment

24	command shall indicate in which superframe the new parameters will take effect by setting a proper value to

25	the Effective Time field. The coordinator shall also indicate it by setting the CountDown field in the

26	following beacon frames. Both the coordinator and devices shall make sure the new parameters are properly

27	set and start to use the new parameters (e.g., the selected OWPAN ID) when the effective time arrives.

28

29	If the coordinator is not already transmitting beacons, the new superframe configuration and channel

30	parameters shall be put into operation immediately.

31

32	Then the MAC sublayer shall issue the MLME-START.confirm primitive with a status of SUCCESS.

33

34	5.1.5.4 Realignment in a OWPAN

35

36	If a device has received the coordinator realignment command (see 5.4.13) from the coordinator through

37	which it  is  associated, the  MLME shall issue the MLME-SYNC-LOSS.indication primitive with the

38	LossReason parameter set to REALIGNMENT and the OWPANId, LogicalChannel, and the security-

39	related parameters set to the respective fields in the coordinator realignment command. The next higher

40	layer of a coordinator may then issue an MLME-START.request primitive with the CoordRealignment

41	parameter set to TRUE. The next higher layer of a device that is not a coordinator may instead change the

42	superframe configuration or channel parameters through use of the MLME-SET.request primitive.

43

44	5.1.5.5 Updating superframe configuration and channel PIB attributes

45

46	To update the superframe configuration and channel attributes, the MLME shall assign values from the

47	MLME-START.request primitive  parameters  to  the  appropriate  PIB  attributes. The  MLME  shall  set

48	macBeaconOrder to the value of the BeaconOrder parameter. If macBeaconOrder is equal to 15, the MLME

49	will also set macSuperframeOrder to 15. In this case, this primitive configures a nonbeacon-enabled

50	OWPAN. If macBeaconOrder is less than 15, the MAC sublayer will set macSuperframeOrder to the value

51	of the SuperframeOrder parameter. The MAC sublayer shall also update macOWPANId with the value of the

52	OWPANId parameter and update phyCurrentChannel with the values of the LogicalChannel parameters by

53	issuing the PLME-SET.request primitive.

54

46	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.1.6 Association and disassociation                                                                                                         1

2

This subclause specifies the procedures for association and disassociation.                                                                    3

4

5.1.6.1 Association                                                                                                                                       5

6

A device shall attempt to associate only after having first performed a MAC sublayer reset, by issuing the              7

MLME-RESET.request  primitive  with  the  SetDefaultPIB  parameter  set  to  TRUE,  and  then  having	8 completed either an active channel scan, see 5.1.3.1.1, or a passive channel scan as shown in 5.1.3.1.2. The	9 results of the channel scan would have then been used to choose a suitable OWPAN. The algorithm for	10 selecting a suitable OWPAN with which to associate from the list of OWPAN descriptors returned from the	11 channel scan procedure is out of the scope of this standard.	12

13

If no OWPAN is detected, the next higher layer of the device may request the MLME to send an additional              14 beacon request command (see 9.5.8) by issuing a MLME-AdditionalBeacon.request primitive. After sending              15 the additional beacon request command, the device shall start a passive scan to discover beacon frames or              16 additional  beacon  frames  during Tcoordscan (see  5.1.3.1.2).  The  results  of  the  channel  scan  during              17

Tcoordscan would have then been also used to choose a suitable OWPAN. If the device cannot detect any              18 beacon when the Tcoordscan expires, the next higher layer of the device may request to send another              19 additional beacon request command.                                                                                                                                20

21

Following the selection of a OWPAN with which to associate, the next higher layers shall request through              22 the MLME-ASSOCIATE.request primitive that the MLME configures the following PHY and MAC PIB              23 attributes to the values necessary for association:                                                                                                            24

25

—	macOWPANId   shall   be   set   equal   to   the   CoordOWPANId   parameter   of   the   MLME-	26

ASSOCIATE.request primitive.	27

—	macCoordExtendedAddress or macCoordShortAddress, depending on which is known from the	28 beacon frame from the coordinator through which it wishes to associate, shall be set equal to the	29

CoordAddress parameter of the MLME-ASSOCIATE.request primitive.	30

31

A coordinator shall allow association only if macAssociationPermit is set to TRUE. Similarly, a device              32 should attempt to associate only with a OWPAN through a coordinator that is currently allowing association,              33 as indicated in the results of the scanning procedure. If a coordinator with macAssociationPermit set to              34

FALSE receives an association request command from a device, the command shall be ignored.                                  35

36

A coordinator receives the additional beacon request commands from a device may start to send additional              37 beacons in each superframe from the next superframe in addition to the original beacons. The coordinator              38 shall allocate a GTS in the CFP to transmit the additional beacon frames. The content of the additional              39 beacon frame shall be the same as the beacon sent in the BP in the same superframe except the beacon type              40 and shall be modulated on the minimum bandwidth. The BeaconType field in the beacon frame can be used              41 to identify the type of the beacons. If the coordinator receives another additional beacon request command              42 from the same device within macResponseWaitTime optical clocks, it may allocate a different GTS in the              43

CFP to  transmit  additional  beacon  frames  from  the  next  superframe. If  the  coordinator receives  an              44 association request from the device that sent the additional beacon request command within            45 macResponseWaitTime optical clocks, it shall handle the association request. If the coordinator does not              46 receive any association request from the device that sent the additional beacon request command within              47 macResponseWaitTime optical clocks, it may stop transmitting the additional beacons and only transmit the              48 original beacons.                                                                                                                                                               49

50

In order to optimize the association procedure on a beacon-enabled OWPAN, a device may begin tracking              51 the beacon of the coordinator through which it wishes to associate. This is achieved by the next higher layer              52 issuing the MLME-SYNC.request primitive with the TrackBeacon parameter set to TRUE.                                         53

54

Copyright © 20xx IEEE. All rights reserved.	47

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	A device that is instructed to associate with a OWPAN, through the MLME-ASSOCIATE.request primitive,

2	shall try to associate only with an existing OWPAN and shall not attempt to start its own OWPAN.

3

4	The MAC sublayer of an unassociated device shall initiate the association procedure by sending an

5	association request command, see 5.4.1, to the coordinator of an existing OWPAN; if the association request

6	command cannot be sent due to a channel access failure, the MAC sublayer shall notify the next higher

7	layer. Because the association request command contains an acknowledgment request (see 5.4.1.1), the

8	coordinator shall confirm its receipt by sending an acknowledgment frame.

9

10	The acknowledgment to an association request command does not mean that the device has associated. The

11	next higher layer of the coordinator needs time to determine whether the current resources available on the

12	OWPAN are sufficient to allow another device to associate. The next higher layer should make this decision

13	within macResponseWaitTime optical clocks. If the next higher layer of the coordinator finds that the device

14	was previously associated on its OWPAN, all previously obtained device-specific information should be

15	removed. If sufficient resources are available, the next higher layer should allocate a 16-bit short address to

16	the device, and the MAC sublayer shall generate an association response command, see 5.4.2, containing the

17	new address and a status indicating a successful association. If sufficient resources are not available, the next

18	higher layer of the coordinator should inform the MAC sublayer, and the MLME shall generate an

19	association response command containing a status indicating a failure as shown in Table 29. The association

20	response command shall be sent to the device requesting association using indirect transmission, i.e., the

21	association response command frame shall be added to the list of pending transactions stored on the

22	coordinator and extracted at the discretion of the device concerned using the method described in 5.1.9.3.

23

24	If the allocate address subfield of the capability information field (see 5.4.23.1.1) of the association request

25	command is set to one, the next higher layer of the coordinator shall allocate a 16-bit address with a range

26	depending on the addressing mode supported by the coordinator, as described in Table 4. If the Allocate

27	Address subfield of the association request command is set to zero, the 16-bit short address shall be equal to

28	0xfffe. A short address of 0xfffe is a special case that indicates that the device has associated, but has not

29	been allocated a short address by the coordinator. In this case, the device shall use only its 64-bit extended

30	address to operate on the network.

31

32	On receipt of the acknowledgment to the association request command, the device shall wait for at most

33	macResponseWaitTime optical clocks for the coordinator to make its association decision; the PIB attribute

34	macResponseWaitTime is a network-topology-dependent parameter and may be set to match the specific

35	requirements of the network that a device is trying to join. If the device is tracking the beacon, it shall

36	attempt to extract the association response command from the coordinator whenever it is indicated in the

37	beacon frame. If the device is not tracking the beacon, it shall attempt to extract the association response

38	command from the coordinator after macResponseWaitTime optical clocks. If the device does not extract an

39	association response command frame from the coordinator within macResponseWaitTime optical clocks, the

40	MLME  shall  issue  the  MLME-ASSOCIATE.confirm primitive  with  a  status  of  NO_DATA, and  the

41	association attempt shall be deemed a failure. In this case, the next higher layer shall terminate any tracking

42	of the beacon. This is achieved by issuing the MLME-SYNC.request primitive with the TrackBeacon

43	parameter set to FALSE.

44

45	The  MLME-ASSOCIATE.response and  the  subsequent Association  response  (see  5.4.2)  also  contain

46	information about what capabilities the device and the coordinator will and will not use during future

47	communication.

48

49	Because the association response command contains an acknowledgment request (see 5.4.2.1), the device

50	requesting association shall confirm its receipt by sending an acknowledgment frame. If the Association

51	Status field of the command indicates that the association was successful, the device shall store the address

52	contained in the 16-bit Short Address field of the command in macShortAddress; communication on the

53	OWPAN using this short address shall depend on its range, as described in Table 4. If the original beacon

54	selected for association following a scan contained the short address of the coordinator, the extended address

48	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

of the coordinator, contained in the MHR of the association response command frame, shall be stored in	1

macCoordExtendedAddress.	2

3

Table 4—Us ag e of the 16-bit s ho rt addres s	4

| Value of macShortAddress | Description |
| --- | --- |
| 0x0000–0xfffd | If a source address is included, the device shall use short source addressing mode for beacon and data frames and the appropriate source addressing mode specified in 5.4 for MAC command frames. |
| 0xfffe | If a source address is included, the device shall use extended source addressing mode for beacon and data frames and the appropriate source addressing mode specified in 5.4 for MAC command frames. |
| 0xffff | The device is not associated and, therefore, shall not perform any data frame communication. The device shall use the appropriate source addressing mode specified in 5.4 for MAC command frames. |

5

6

7

8

9

10

11

12

13

14

15

16

17

If the Association Status field of the command indicates that the association was unsuccessful, the device	18 shall set macOWPANId to the default value (0xffff).	19

20

5.1.6.2 Disassociation	21

22

The   disassociation   procedure   is   initiated   by   the   next   higher   layer   by   issuing   the   MLME-	23

DISASSOCIATE.request primitive to the MLME.	24

25

When a coordinator wants one of its associated devices to leave the OWPAN, the MLME of the coordinator              26 shall send the disassociation notification command in the manner specified by the TxIndirect parameter of              27 the MLME-DISASSOCIATE.request primitive previously sent by the next higher layer. If TxIndirect is              28

TRUE, the MLME of the coordinator shall send the disassociation notification command to the device using              29 indirect transmission, i.e., the disassociation notification command frame shall be added to the list of              30 pending transactions stored on the coordinator and extracted at the discretion of the device concerned using              31 the method described in 5.1.9.3. If the command frame is not successfully extracted by the device, the              32 coordinator should consider the device disassociated. Otherwise, the MLME shall send the disassociation              33 notification command to the device directly. In this case, if the disassociation notification command cannot              34

be sent due to a channel access failure, the MAC sublayer shall notify the next higher layer.	35

36

Because the disassociation command contains an acknowledgment request (see 5.4.4.1), the receiving              37 device shall confirm its receipt by sending an acknowledgment frame. If the direct or indirect transmission              38 fails, the coordinator should consider the device disassociated.                                                                                       39

40

If an associated device wants to leave the OWPAN, the MLME of the device shall send a disassociation              41 notification command to its coordinator. If the disassociation notification command cannot be sent due to a              42 channel access failure, the MAC sublayer shall notify the next higher layer. Because the disassociation              43 command contains an acknowledgment request (see 5.4.4.1), the coordinator shall confirm its receipt by              44 sending an acknowledgment frame. However, even if the acknowledgment is not received, the device should              45 consider itself disassociated.                                                                                                                                             46

47

If  the  source  address  contained  in  the  disassociation  notification  command  is  equal  to  macCoord-	48

ExtendedAddress, the  device  should  consider  itself  disassociated.  If  the  command  is  received  by  a              49 coordinator and the source is not equal to macCoordExtendedAddress, it shall verify that the source address              50 corresponds to one of its associated devices; if so, the coordinator should consider the device disassociated.              51

If none of the conditions for disassociation are satisfied, the command shall be ignored.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	49

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	An associated device shall disassociate itself by removing all references to the OWPAN; the MLME shall set

2	macOWPANId,	macShortAddress,	macAssociatedOWPANCoord,	macCoordShortAddress	and

3	macCoordExtendedAddress to the default values. The next higher layer of a coordinator should disassociate

4	a device by removing all references to that device.

5

6	The next higher layer of the requesting device shall be notified of the result of the disassociation procedure

7	through the MLME-DISASSOCIATE.confirm primitive.

8

9	5.1.7 Synchronization

10

11	This subclause specifies the procedures for coordinators to generate beacon frames and for devices to

12	synchronize  with  a  coordinator.  For  OWPANs  supporting  beacons,  synchronization is  performed  by

13	receiving and decoding the beacon frames. For OWPANs not supporting beacons, synchronization is

14	performed by polling the coordinator for data.

15

16	5.1.7.1 Synchronization with beacons

17

18	All devices operating on a beacon-enabled OWPAN (i.e., macBeaconOrder < 15) shall be able to acquire

19	beacon synchronization in order to detect any pending messages or to track the beacon. Devices shall be

20	permitted to acquire beacon synchronization only with beacons containing the OWPAN identifier specified

21	in macOWPANId. If macOWPANId specifies the broadcast OWPAN identifier (0xffff), a device shall not

22	attempt to acquire beacon synchronization.

23

24	A device is instructed to attempt to acquire the beacon through the MLME-SYNC.request primitive. If

25	tracking is specified in the MLME-SYNC.request primitive, the device shall attempt to acquire the beacon

26	and keep track of it by regular and timely activation of its receiver. If tracking is not specified, the device

27	shall either attempt to acquire the beacon only once or terminate the tracking after the next beacon if

28	tracking was enabled through a previous request.

29

30	To  acquire  beacon  synchronization,  a  device  shall  enable  its  receiver  and  search  for  at  most

31	[aBaseSuperframeDuration  (2n + 1)] optical clocks, where n is the value of macBeaconOrder. If a beacon

32	frame containing the current OWPAN identifier of the device is not received, the MLME shall repeat this

33	search. Once the number of missed beacons reaches aMaxLostBeacons, the MLME shall notify the next

34	higher   layer   by   issuing   the   MLME-SYNC-LOSS.indication   primitive   with   a   loss   reason   of

35	BEACON_LOSS.

36

37	The MLME shall timestamp each received beacon frame at the same symbol boundary within each frame,

38	the location of which is described by the macTimeStampOffset attribute. The position shall be the same as

39	that used in the timestamp of the outgoing beacon frame, stored in macBeaconTxTime. The timestamp value

40	shall be that of the local clock of the device at this position. The timestamp is intended to be a relative time

41	measurement that may or may not be made absolute, at the discretion of the implementer.

42

43	If a protected beacon frame is received (i.e., the Security Enabled subfield in the frame control field is set to

44	one), the device shall attempt to unsecure the beacon frame using the unsecuring process described in 7.2.3.

45

46	If the status from the unsecuring process is not SUCCESS, the MLME shall issue an MLME-COMM-

47	STATUS.indication primitive with the status parameter set to the status from the unsecuring process,

48	indicating the error.

49

50	The security-related elements of the OWPAN descriptor corresponding to the beacon (see Table 62) shall be

51	set to the corresponding parameters returned by the unsecuring process. The SecurityFailure element of the

52	OWPAN descriptor shall be set to SUCCESS if the status from the unsecuring process is SUCCESS and set

53	to one of the other status codes indicating an error in the security processing otherwise.

54

50	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

If a beacon frame is received, the MLME shall discard the beacon frame if the Source Address and the              1

Source OWPAN Identifier fields of the MHR of the beacon frame do not match the coordinator source              2 address (macCoordShortAddress or macCoordExtendedAddress, depending on the addressing mode) and the              3 identifier of the device (macOWPANId).                                                                                                                          4

5

If a valid beacon frame is received and macAutoRequest is set to FALSE, the MLME shall indicate the              6 beacon parameters to the next higher layer by issuing the MLME-BEACON-NOTIFY.indication primitive.              7

If a beacon frame is received and macAutoRequest is set to TRUE, the MLME shall first issue the MLME-              8

BEACON-NOTIFY.indication primitive if the beacon contains any payload. The MLME shall then compare	9 its address with those addresses in the Address List field of the beacon frame. If the Address List field	10 contains the 16-bit short or 64-bit extended address of the device and the source OWPAN identifier matches	11 macOWPANId, the MLME shall follow the procedure for extracting pending data from the coordinator as	12 shown in 5.1.9.3.	13

14

If beacon tracking is activated, the MLME shall enable its receiver at a time prior to the next expected              15 beacon frame transmission, i.e., just before the known start of the next superframe. If the number of              16 consecutive beacons missed by the MLME reaches aMaxLostBeacons, the MLME shall respond with the              17

MLME-SYNC-LOSS.indication primitive with a loss reason of BEACON_LOST.                                                       18

19

5.1.7.2 Synchronization without beacons                                                                                                  20

21

All devices operating on a nonbeacon-enabled OWPAN (macBeaconOrder = 15) shall be able to poll the              22 coordinator for data at the discretion of the next higher layer.                                                                                         23

24

A device is instructed to poll the coordinator when the MLME receives the MLME-POLL.request primitive.              25

On receipt of this primitive, the MLME shall follow the procedure for extracting pending data from the              26 coordinator as shown in 5.1.9.3.                                                                                                                                       27

28

5.1.8 Transaction handling                                                                                                                           29

30

Transactions can be instigated from the devices themselves rather than from the coordinator. In other words,              31 either the coordinator needs to indicate in its beacon when messages are pending for devices or the devices              32 themselves need to poll the coordinator to determine whether they have any messages pending. Such              33 transfers are called indirect transmissions.                                                                                                                       34

35

The coordinator shall begin handling a transaction on receipt of an indirect transmission request either via              36 the MCPS-DATA.request primitive or via a request from the MLME to send a MAC command instigated by              37 a primitive from the next higher layer, such as the MLME-ASSOCIATE.response primitive as shown in              38

6.3.1.3. On completion of the transaction, the MAC sublayer shall indicate a status value to the next higher              39 layer. If a request primitive instigated the indirect transmission, the corresponding confirm primitive shall be              40 used to convey the appropriate status value. Conversely, if a response primitive instigated the indirect             41 transmission, the MLME-COMM-STATUS.indication primitive shall be used to convey the appropriate              42 status value. The MLME-COMM-STATUS.indication primitive can be related to its corresponding response              43 primitive by examining the Destination Address field.                                                                                                     44

45

The information contained in the indirect transmission request forms a transaction, and the coordinator shall              46 be capable of storing at least one transaction. On receipt of an indirect transmission request, if there is no              47 capacity to store another transaction, the MAC sublayer shall indicate to the next higher layer a status of              48

TRANSACTION_OVERFLOW in the appropriate corresponding primitive.                                                                49

50

If the coordinator is capable of storing more than one transaction, it shall ensure that all the transactions for              51 the same device are sent in the order in which they arrived at the MAC sublayer. For each transaction sent, if              52 another exists for the same device, the MAC sublayer shall set its frame pending subfield to one, indicating              53 the additional pending data.                                                                                                                                              54

Copyright © 20xx IEEE. All rights reserved.	51

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Each  transaction  shall  persist  in  the  coordinator  for  at  most  macTransactionPersistenceTime. If  the

2	transaction  is  not  successfully  extracted  by  the  appropriate  device  within  this  time,  the  transaction

3	information shall be discarded and the MAC sublayer shall indicate to the next higher layer a status of

4	TRANSACTION_EXPIRED in  the  appropriate  corresponding  primitive.  In  order  to  be  successfully

5	extracted, an acknowledgment shall be received if one was requested.

6

7	If the transaction was successful, the transaction information shall be discarded, and the MAC sublayer shall

8	indicate to the next higher layer a status of SUCCESS in the appropriate corresponding primitive.

9

10	If the coordinator transmits beacons, it shall list the addresses of the devices to which each transaction is

11	associated  in  the Address  List  field  and  indicate  the  number  of  addresses  in  the  Pending Address

12	Specification field of the beacon frame. If the coordinator is able to store more than seven pending

13	transactions, it shall indicate them in its beacon on a first-come-first-served basis, ensuring that the beacon

14	frame contains at most seven addresses. For transactions requiring a GTS, the coordinator shall not add the

15	address of the recipient to its list of pending addresses in the beacon frame. Instead it shall transmit the

16	transaction in the GTS allocated for the device as shown in 5.1.10.3.

17

18	On a beacon-enabled OWPAN, if there is a transaction pending for the broadcast address, the frame pending

19	subfield of the frame control field in the beacon frame shall be set to one, and the pending message shall be

20	transmitted immediately following the beacon using the unslotted random access algorithm. If there is a

21	second message pending for the broadcast address, its transmission shall be delayed until the following

22	superframe. Only one broadcast message shall be allowed to be sent indirectly per superframe.

23

24	On a beacon-enabled OWPAN, a device that receives a beacon containing its address in the list of pending

25	addresses shall attempt to extract the data from the coordinator. On a nonbeacon-enabled OWPAN, a device

26	shall attempt to extract the data from the coordinator on receipt of the MLME-POLL.request primitive. The

27	procedure for extracting pending data from the coordinator is described in 5.1.9.3. If a device receives a

28	beacon  with  the  frame  pending  subfield  set  to  one,  it  shall  leave  its  receiver  enabled  for  up  to

29	macMaxFrameTotalWaitTime optical clocks to receive the broadcast data frame from the coordinator.

30

31	5.1.9 Transmission, reception, and acknowledgment

32

33	This subclause describes the fundamental procedures for transmission, reception, and acknowledgment.

34

35	5.1.9.1 Transmission

36

37	Each device shall store its current data-sequence number (DSN) value in the MAC PIB attribute macDSN

38	and initialize it to a random value; the algorithm for choosing a random number is out of the scope of this

39	standard. Each time a data or a MAC command frame is generated, the MAC sublayer shall copy the value

40	of macDSN into the Sequence Number field of the MHR of the outgoing frame and then increment it by one.

41	Each device shall generate exactly one DSN regardless of the number of unique devices with which it

42	wishes to communicate. The value of macDSN shall be permitted to roll over.

43

44	Each coordinator shall store its current beacon-sequence number (BSN) value in the MAC PIB attribute

45	macBSN and initialize it to a random value; the algorithm for choosing a random number is out of the scope

46	of this standard. Each time a beacon frame is generated, the MAC sublayer shall copy the value of macBSN

47	into the Sequence Number field of the MHR of the outgoing frame and then increment it by one. The value

48	of macBSN shall be permitted to roll over.

49

50	It should be noted that both the DSN and BSN are 8-bit values and, therefore, have limited use to the next

51	higher layer (e.g., in the case of the DSN, in detecting retransmitted frames).

52

53	The Source Address field, if present, shall contain the address of the device sending the frame. When a

54	device has associated and has been allocated a 16-bit short address (i.e., macShortAddress is not equal to

52	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

0xfffe  or  0xffff),  it  shall  use  that  address  in   preference  to   its  64-bit  extended  address  (i.e.,              1 aExtendedAddress)  wherever  possible.  When  a  device  has  not  yet  associated  to  a  OWPAN  or              2 macShortAddress is equal to 0xffff, it shall use its 64-bit extended address in all communications requiring              3 the Source Address field. If the Source Address field is not present, the originator of the frame shall be              4 assumed to be the coordinator, and the Destination Address field shall contain the address of the recipient.                 5

6

The Destination Address field, if present, shall contain the address of the intended recipient of the frame,              7 which may be either a 16-bit short address or a 64-bit extended address. If the Destination Address field is              8 not present, the recipient of the frame shall be assumed to be the coordinator, and the Source Address field              9 shall contain the address of the originator.                                                                                                                        10

11

If both destination and source addressing information is present, the MAC sublayer shall compare the              12 destination and source OWPAN identifiers. If the OWPAN identifiers are identical the source OWPAN              13 identifier shall be omitted from the transmitted frame. If the OWPAN identifiers are different, both the              14

Destination OWPAN Identifier and Source OWPAN Identifier fields shall be included in the transmitted              15 frame. If only either the destination or the source addressing information is present the OWPAN identifier              16 field of the single address shall be included in the transmitted frame.                                                                             17

18

If the frame is to be transmitted on a beacon-enabled OWPAN, the transmitting device shall attempt to find              19 the beacon before transmitting. If the beacon is not being tracked, as shown in 5.1.7.1, and hence the device              20 does  not know  where  the  beacon  will  appear,  it  shall  enable  its  receiver  and  search  for  at  most              21 [aBaseSuperframeDuration  (2n + 1)] optical clocks, where n is the value of macBeaconOrder, in order to              22

find the beacon. If the beacon is not found after this time, the device shall transmit the frame following the              23 successful application of the unslotted version of the random access algorithm as shown in 5.1.2.1. Once the              24 beacon has been found, either after a search or due to its being tracked, the frame shall be transmitted in the              25 appropriate portion of the superframe. Transmissions in the CAP shall follow a successful application of the              26 slotted version of the random access algorithm, see 5.1.2.1, and transmissions in a GTS shall not use any              27 random access.                                                                                                                                                                  28

29

If the frame is to be transmitted on a nonbeacon-enabled OWPAN, the frame shall be transmitted following              30 the successful application of the unslotted version of the random access algorithm as shown in 5.1.2.1.                       31

32

For either a beacon-enabled OWPAN or a nonbeacon-enabled OWPAN, if the transmission is direct and              33 originates due to a primitive issued by the next higher layer and the access algorithm fails, the next higher              34 layer shall be notified. If the transmission is indirect and the access algorithm fails, the frame shall remain in              35 the transaction queue until it is requested again and successfully transmitted or until the transaction expires.              36

37

The device shall process the frame using the outgoing frame security procedure described in 7.2.1.                             38

39

If the status from the outgoing frame security procedure is not SUCCESS, the MLME shall issue the             40 corresponding confirm or MLME-COMM-STATUS.indication primitive with the status parameter set to the              41 status from the outgoing frame security procedure, indicating the error.                                                                         42

43

To transmit the frame, the MAC sublayer shall first enable the transmitter by issuing the PLME-SET-              44

TRX-STATE.request primitive with a state of TX_ON to the PHY. On receipt of the PLME-SET-TRX-              45

STATE.confirm primitive with a status of either SUCCESS or TX_ON, the constructed frame shall then be              46 transmitted  by  issuing  the  PD-DATA.request primitive.  Finally,  on  receipt  of  the  PD-DATA.confirm              47 primitive, the MAC sublayer shall disable the transmitter by issuing the PLME-SET-TRX-STATE.request             48 primitive with a state of RX_ON or TRX_OFF to the PHY, depending on whether the receiver is to be              49 enabled following the transmission. In the case where the Acknowledgment Request subfield of the frame              50 control  field  is  set  to  one,  the  MAC  sublayer  shall  enable  the  receiver  immediately  following  the              51 transmission of the frame by issuing the PLME-SET-TRX-STATE.request primitive with a state of RX_ON              52 to the PHY.                                                                                                                                                                        53

54

Copyright © 20xx IEEE. All rights reserved.	53

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.9.2 Reception and rejection

2

3	Each device may choose whether the MAC sublayer is to enable its receiver during idle periods. During

4	these idle periods, the MAC sublayer shall still service transceiver task requests from the next higher layer.

5	A transceiver task shall be defined as a transmission request with acknowledgment reception, if required, or

6	a reception request. On completion of each transceiver task, the MAC sublayer shall request that the PHY

7	enables or disables its receiver, depending on the values of macBeaconOrder and macRxOnWhenIdle. If

8	macBeaconOrder is less than 15, the value of macRxOnWhenIdle shall be considered relevant only during

9	idle periods of the CAP of the incoming superframe. If macBeaconOrder is equal to 15, the value of

10	macRxOnWhenIdle shall be considered relevant at all times.

11

12	A device with its receiver enabled will be able to receive and decode transmissions from all devices

13	complying with this standard that are currently operating on the same channel and are in its operating space,

14	along with interference from other sources. The MAC sublayer shall, therefore, be able to filter incoming

15	frames and present only the frames that are of interest to the upper layers.

16

17	The MAC sublayer shall discard all received frames that do not contain a correct value in their FCS field in

18	the MFR (see 5.2.1.9). The FCS field shall be verified on reception by recalculating the purported FCS over

19	the MHR and MSDU of the received frame and by subsequently comparing this value with the received FCS

20	field. The FCS field of the received frame shall be considered to be correct if these values are the same and

21	incorrect otherwise.

22

23	The MAC sublayer shall accept only frames that satisfy all of the following filtering requirements:

24

25	—	The Frame Type subfield shall not contain a reserved frame type.

26	—	If a destination OWPAN identifier is included in the frame, it shall match macOWPANId or shall be

27	the broadcast OWPAN identifier (0xffff).

28	—	If a short destination address is included in the frame, it shall match either macShortAddress or the

29	broadcast address (0xffff). Otherwise, if an extended destination address is included in the frame, it

30	shall match aExtendedAddress.

31

32	—	If the frame type indicates that the frame is a beacon frame, the source OWPAN identifier shall

33	match macOWPANId unless macOWPANId is equal to 0xffff, in which case the beacon frame shall

34	be accepted regardless of the source OWPAN identifier.

35	—	If only source addressing fields are included in a data or MAC command frame, the frame shall be

36	accepted  only  if  the  device  is  the  coordinator  and  the  source  OWPAN  identifier  matches

37	macOWPANId.

38

39	If any of the third-level filtering requirements are not satisfied, the MAC sublayer shall discard the incoming

40	frame without processing it further. If all of the third-level filtering requirements are satisfied, the frame

41	shall be considered valid and processed further. For valid frames that are not broadcast, if the Frame Type

42	subfield indicates a data or MAC command frame and the Acknowledgment Request subfield of the frame

43	control field is set to one, the MAC sublayer shall send an acknowledgment frame. Prior to the transmission

44	of the acknowledgment frame, the sequence number included in the received data or MAC command frame

45	shall be copied into the Sequence Number field of the acknowledgment frame. This step will allow the

46	transaction originator to know that it has received the appropriate acknowledgment frame.

47

48	If both the destination and source addressing information is included in the frame, the MAC sublayer shall

49	assume that the omitted Source OWPAN Identifier field is identical to the Destination OWPAN Identifier

50	field.

51

52	The device shall process the frame using the incoming frame security procedure described in 7.2.3.

53

54

54	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

If the status from the incoming frame security procedure is not SUCCESS, the MLME shall issue the              1 corresponding confirm or MLME-COMM-STATUS.indication primitive with the status parameter set to the              2 status from the incoming frame security procedure, indicating the error, and with the security-related              3 parameters set to the corresponding parameters returned by the unsecuring process.                                                      4

5

If the valid frame is a data frame, the MAC sublayer shall pass the frame to the next higher layer. This is              6 achieved by issuing the MCPS-DATA.indication primitive containing the frame information. The security-              7 related parameters of the MCPS-DATA.indication primitive shall be set to the corresponding parameters              8 returned by the unsecuring process.                                                                                                                                  9

10

If the valid frame is a MAC command or beacon frame, it shall be processed by the MAC sublayer              11 accordingly, and a corresponding confirm or indication primitive may be sent to the next higher layer. The              12 security-related parameters  of  the  corresponding  confirm  or  indication  primitive  shall  be  set  to  the              13 corresponding parameters returned by the unsecuring process.                                                                                       14

15

5.1.9.3 Extracting pending data from a coordinator                                                                                  16

17

A device on a beacon-enabled OWPAN can determine whether any frames are pending for it by examining              18 the contents of the received beacon frame, as described in 5.1.7.1. If the address of the device is contained in              19 the Address List field of the beacon frame and macAutoRequest is TRUE, the MLME of the device shall              20 send a data request command, see 5.4.5, to the coordinator during the CAP with the Acknowledgment              21

Request subfield of the frame control field set to one; the only exception to this is if the beacon frame is              22 received while performing an active or passive scan as shown in 5.1.5.1. There are two other cases for which              23 the MLME shall send a data request command to the coordinator. The first case is when the MLME receives              24 the MLME-POLL.request primitive. In the second case, a device may send a data request command             25 macResponseWaitTime optical clocks after the acknowledgment to a request command frame, such as during              26 the association procedure. If the data request is intended for the coordinator, the destination address              27 information may be omitted.                                                                                                                                             28

29

If the data request command originated from an MLME-POLL.request primitive, the MLME shall perform              30 the security process on the data request command based on the SecurityLevel, KeyIdMode, KeySource, and              31

KeyIndex parameters of the MLME-POLL.request primitive, according to 7.2.1. Otherwise, the MLME              32 shall perform the security process on the data request command based on the macAutoRequestSecurityLevel,             33 macAutoRequestKeyIdMode, macAutoRequestKeySource, and macAutoRequestKeyIndex PIB attributes,             34 according to 7.2.1.                                                                                                                                                             35

36

On successfully receiving a data request command, the coordinator shall send an acknowledgment frame,              37 thus confirming its receipt. If the coordinator has enough time to determine whether the device has a frame              38 pending before sending the acknowledgment frame (see 5.1.9.4.2), it shall set the frame pending subfield of              39 the frame control field of the acknowledgment frame accordingly to indicate whether a frame is actually              40 pending for the device. If this is not possible, the coordinator shall set the frame pending subfield of the              41 acknowledgment frame to one.                                                                                                                                         42

43

On receipt of the acknowledgment frame with the frame pending subfield set to zero, the device shall              44 conclude that there are no data pending at the coordinator.                                                                                              45

46

On receipt of the acknowledgment frame with the frame pending subfield set to one, a device shall enable its              47 receiver for at most macMaxFrameTotalWaitTime CAP optical clocks in a beacon-enabled OWPAN, or in a              48 nonbeacon-enabled OWPAN, to receive the corresponding data frame from the coordinator. If there is an              49 actual data frame pending within the coordinator for the requesting device, the coordinator shall send the              50 frame to the device using one of the mechanisms described in this subclause. If there is no data frame              51 pending  for  the  requesting  device,  the  coordinator  shall  send  a  data  frame  without  requesting              52 acknowledgment to the device containing a zero length payload, indicating that no data are present, using              53 one of the mechanisms described in this subclause.                                                                                                         54

Copyright © 20xx IEEE. All rights reserved.	55

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The data frame following the acknowledgment of the data request command shall be transmitted using one

2	of the following mechanisms:

3

4	—	Without using slotted random access, if the MAC sublayer can commence transmission of the data

5	frame between aTurnaroundTime-RX-TX and (aTurnaroundTime-RX-TX + aUnitBackoffPeriod) optical

6	clocks, on a backoff slot boundary, and there is time remaining in the CAP for the message,

7	appropriate IFS, and acknowledgment as defined in 9.5.1. If a requested acknowledgment frame is

8	not received following this data frame, the process shall begin anew following the receipt of a new

9	data request command.

10	—	Using slotted random access, otherwise.

11

12	If	the	requesting	device	does	not	receive	a	data	frame	from	the	coordinator	within

13	macMaxFrameTotalWaitTime CAP optical clocks in a beacon-enabled OWPAN, or in a nonbeacon-enabled

14	OWPAN, or if the requesting device receives a data frame from the coordinator with a zero length payload,

15	it shall conclude that there are no data pending at the coordinator. If the requesting device does receive a data

16	frame from the coordinator, it shall send an acknowledgment frame, if requested, thus confirming receipt.

17

18	If the frame pending subfield of the frame control field of the data frame received from the coordinator is set

19	to one, the device still has more data pending with the coordinator. In this case it may extract the data by

20	sending a new data request command to the coordinator.

21

22	5.1.9.4 Use of acknowledgments and retransmissions

23

24	A data or MAC command frame shall be sent with the Acknowledgment Request subfield of its frame

25	control field set appropriately for the frame. A beacon or acknowledgment frame shall always be sent with

26	the Acknowledgment Request subfield set to zero. Similarly, any frame that is broadcast shall be sent with

27	its Acknowledgment Request subfield set to zero.

28

29	5.1.9.4.1 No ac kno wle dg me nt

30

31	A frame transmitted with its Acknowledgment Request subfield set to zero shall not be acknowledged by its

32	intended recipient. The originating device shall assume that the transmission of the frame was successful.

33

34	The message sequence chart in Figure 23 shows the scenario for transmitting a single frame of data from an

35	originator to a recipient without requiring an acknowledgment. In this case, the originator transmits the data

36	frame with the Acknowledgment Request (AR) subfield of the frame control field equal to zero.

37

38	originator next

39	higher layer

40

41

42

originator

MAC

recipient

MAC

recipient next higher layer

43	MCPS-DATA.request (AR=0)

44

45

46	MCPS-DATA.confirm

47

48

49

50

Data (AR=0)

MCPS-DATA.indication

51	Figure 23—Successful data transmission without an acknowledgment

52

53

54

56	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.1.9.4.2 Ac kno wle dg me nt                                                                                                                                         1

2

A frame transmitted with the Acknowledgment Request subfield of its frame control field set to one shall be              3 acknowledged by the recipient. If the intended recipient correctly receives the frame, it shall generate and              4 send an acknowledgment frame containing the same DSN from the data or MAC command frame that is              5 being acknowledged.                                                                                                                                                        6

7

The transmission of an acknowledgment frame in a nonbeacon-enabled OWPAN or in the CFP shall              8 commence aTurnaroundTime-RX-TXoptical clocks after the last optical clock of the data or MAC command              9 frame. The transmission of an acknowledgment frame in the CAP shall commence either aTurnaroundTime-              10

RX-TX optical clocks after the reception of the last optical clock of the data or MAC command frame or at a              11 backoff slot boundary. In the latter case, the transmission of an acknowledgment frame shall commence              12 between aTurnaroundTime-RX-TX and (aTurnaroundTime-RX-TX + aUnitBackoffPeriod) optical clocks              13 after the reception of the last optical clock of the data or MAC command frame. The constants             14 aTurnaroundTime-RX-TX and aTurnaroundTime-TX-RX are defined in Table 139.                                                        15

16

The message sequence chart in Figure 24 shows the scenario for transmitting a single frame of data from an              17 originator to a recipient with an acknowledgment. In this case, the originator indicates to the recipient that it              18 requires an acknowledgment by transmitting the data frame with the Acknowledgment Request (AR)              19 subfield of the frame control field set to one.                                                                                                                   20

21

originator next higher layer

originator

MAC

recipient

MAC

recipient next 	22

higher layer 	23

24

25

MCPS-DATA.request (AR=1)

MCPS-DATA.confirm

data (AR=1)

acknowledgment

26

27

28

29

MCPS-DATA.indication 	30

31

32

33

34

35

Figure 24—Successful data transmission with an acknowledgment	36

37

38

5.1.9.4.3 Re trans mis s io ns	39

40

A device that sends a frame with the Acknowledgment Request subfield of its frame control field set to zero              41 shall assume that the transmission was successfully received and shall hence not perform the retransmission              42 procedure.                                                                                                                                                                          43

44

A device that sends a data or MAC command frame with its Acknowledgment Request subfield set to one              45 shall wait for at most macAckWaitDuration optical clocks for the corresponding acknowledgment frame to              46 be received. If an acknowledgment frame is received within macAckWaitDuration optical clocks and              47 contains the same DSN as the original transmission, the transmission is considered successful, and no              48 further action regarding retransmission shall be taken by the device. If an acknowledgment is not received              49 within macAckWaitDuration optical clocks or an acknowledgment is received containing a DSN that was              50 not the same as the original transmission, the device shall conclude that the single transmission attempt has              51 failed.                                                                                                                                                                                52

53

54

Copyright © 20xx IEEE. All rights reserved.	57

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	If a single transmission attempt has failed and the transmission was indirect, the coordinator shall not

2	retransmit the data or MAC command frame. Instead, the frame shall remain in the transaction queue of the

3	coordinator and can only be extracted following the reception of a new data request command. If a new data

4	request command is received, the originating device shall transmit the frame using the same DSN as was

5	used in the original transmission.

6

7	If a single transmission attempt has failed and the transmission was direct, the device shall repeat the process

8	of transmitting the data or MAC command frame and waiting for the acknowledgment, up to a maximum of

9	macMaxFrameRetries times. The retransmitted frame shall contain the same DSN as was used in the

10	original transmission. Each retransmission shall only be attempted if it can be completed within the same

11	portion of the superframe, i.e., the CAP or a GTS in which the original transmission was attempted. If this

12	timing is not possible, the retransmission shall be deferred until the same portion in the next superframe. If

13	an acknowledgment is still not received after macMaxFrameRetries retransmissions, the MAC sublayer

14	shall assume the transmission has failed and notify the next higher layer of the failure.

15

16	5.1.9.5 Transmission scenarios

17

18	Due to the imperfect nature of the wireless medium, a transmitted frame does not always reach its intended

19	destination. Figure 25 illustrates three different data transmission scenarios:

20

21	—	Successful data transmission. The originator MAC sublayer transmits the data frame to the recipient

22	via the PHY data service. In waiting for an acknowledgment, the originator MAC sublayer starts a

23	timer  that  will  expire  after  macAckWaitDuration optical clocks. The  recipient MAC  sublayer

24	receives the data frame, sends an acknowledgment back to the originator, and passes the data frame

25	to the next higher layer. The originator MAC sublayer receives the acknowledgment from the

26	recipient before its timer expires and then disables and resets the timer. The data transfer is now

27	complete, and the originator MAC sublayer issues a success confirmation to the next higher layer.

28	—	Lost data frame. The originator MAC sublayer transmits the data frame to the recipient via the PHY

29	data service. In waiting for an acknowledgment, the originator MAC sublayer starts a timer that will

30	expire after macAckWaitDuration optical clocks. The recipient MAC sublayer does not receive the

31	data frame and so does not respond with an acknowledgment. The timer of the originator MAC

32	sublayer expires before an acknowledgment is received; therefore, the data transfer has failed. If the

33	transmission was direct, the originator retransmits the data, and this entire sequence may be repeated

34	up to a maximum of macMaxFrameRetries times; if a data transfer attempt fails a total of (1 +

35	macMaxFrameRetries) times, the originator MAC sublayer will issue a failure confirmation to the

36	next higher layer. If the transmission was indirect, the data frame will remain in the transaction

37	queue until either another request for the data is received and correctly acknowledged or until

38	macTransactionPersistenceTime is  reached.  If  macTransactionPersistenceTime is  reached,  the

39	transaction information will be discarded, and the MAC sublayer will issue a failure confirmation to

40	the next higher layer.

41	—	Lost acknowledgment frame. The originator MAC sublayer transmits the data frame to the recipient

42	via the PHY data service. In waiting for an acknowledgment, the originator MAC sublayer starts a

43	timer that  will expire  after macAckWaitDuration optical clocks. The  recipient MAC sublayer

44	receives the data frame, sends an acknowledgment back to the originator, and passes the data frame

45	to the next higher layer. The originator MAC sublayer does not receive the acknowledgment frame,

46	and its timer expires. Therefore, the data transfer has failed. If the transmission was direct, the

47	originator retransmits the data, and this entire sequence may be repeated up to a maximum of

48	macMaxFrameRetries times. If a data transfer attempt fails a total of (1 + macMaxFrameRetries)

49	times, the originator MAC sublayer will issue a failure confirmation to the next higher layer. If the

50	transmission was indirect, the data frame will remain in the transaction queue either until another

51	request for the data is received and correctly acknowledged or until macTransactionPersistenceTime

52	is  reached.  If  macTransactionPersistenceTime is  reached,  the  transaction  information  will  be

53	discarded, and the MAC sublayer will issue a failure confirmation to the next higher layer.

54

58	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

Figure 25—Transmission scenarios, using direct transmission, for frame reliability	26

27

28

5.1.10 GTS allocation and management	29

30

A GTS allows a device to operate on the channel within a portion of the superframe that is dedicated (on the	31

OWPAN) exclusively to that device. A GTS shall be allocated only by the coordinator, and it shall be used	32

only for communications between the coordinator and a device associated with the OWPAN through the	33

coordinator. A single GTS may extend over one or more superframe slots. The coordinator may allocate a	34

number of GTSs at the same time, provided there is sufficient capacity in the superframe.	35

36

A GTS shall be allocated before use, with the coordinator deciding whether to allocate a GTS based on the	37

requirements of the GTS request and the current available capacity in the superframe. GTSs shall be	38

allocated on a first-come-first-served basis, and all GTSs shall be placed contiguously at the end of the	39

superframe and after the CAP. Each GTS shall be deallocated when the GTS is no longer required, and a	40

GTS can be deallocated at any time at the discretion of the coordinator or by the device that originally	41

requested the GTS. A device that has been allocated a GTS may also operate in the CAP.	42

43

A data frame transmitted in an allocated GTS shall use only short addressing.	44

45

The management of GTSs shall be undertaken by the coordinator only. To facilitate GTS management, the	46

coordinator shall be able to store all the information necessary to manage seven GTSs. For each GTS, the	47

coordinator shall be able to store its starting slot, length, direction, and associated device address.	48

49

The GTS direction, which is relative to the data flow from the device that owns the GTS, is specified as	50

either transmit or receive. The device address and direction shall, therefore, uniquely identify each GTS.	51

Each device may request one transmit GTS and/or one receive GTS. For each allocated GTS, the device	52

shall be able to store its starting slot, length, and direction. If a device has been allocated a receive GTS, it	53

shall enable its receiver for the entirety of the GTS. In the same way, the coordinator shall enable its receiver	54

Copyright © 20xx IEEE. All rights reserved.	59

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	for the entirety of the GTS if a device has been allocated a transmit GTS. If a data frame is received during a

2	receive GTS and an acknowledgment is requested, the device shall transmit the acknowledgment frame as

3	usual. Similarly, a device shall be able to receive an acknowledgment frame during a transmit GTS.

4

5	A device shall attempt to allocate and use a GTS only if it is currently tracking the beacons. The MLME is

6	instructed to track beacons by issuing the MLME-SYNC.request primitive with the TrackBeacon parameter

7	set to TRUE. If a device loses synchronization with the coordinator, all its GTS allocations shall be lost.

8

9	The use of GTSs is optional.

10

11	5.1.10.1 CAP maintenance

12

13	The coordinator shall preserve the minimum CAP length of aMinCAPLength and take preventative action if

14	the minimum CAP is not satisfied. However, an exception shall be allowed for the accommodation of the

15	temporary increase in the beacon frame length needed to perform GTS maintenance. If preventative action

16	becomes necessary, the action chosen is left up to the implementation, but may include one or more of the

17	following:

18

19	—	Limiting the number of pending addresses included in the beacon.

20	—	Not including a payload field in the beacon frame.

21	—	Deallocating one or more of the GTSs.

22

23

24	5.1.10.2 GTS allocation

25

26	A device is instructed to request the allocation of a new GTS through the MLME-GTS.request primitive,

27	with GTS characteristics set according to the requirements of the intended application.

28

29	To request the allocation of a new GTS, the MLME shall send the GTS request command, see 5.4.18, to the

30	coordinator. The Characteristics Type subfield of the GTS Characteristics field of the request shall be set to

31	one  (GTS  allocation),  and  the  length  and  direction  subfields  shall  be  set  according  to  the  desired

32	characteristics of the required GTS. Because the GTS request command contains an acknowledgment

33	request (see 5.4.4.1), the coordinator shall confirm its receipt by sending an acknowledgment frame.

34

35	On receipt of a GTS request command indicating a GTS allocation request, the coordinator shall first check

36	if there is available capacity in the current superframe, based on the remaining length of the CAP and the

37	desired length of the requested GTS. The superframe shall have available capacity if the maximum number

38	of GTSs has not been reached and allocating a GTS of the desired length would not reduce the length of the

39	CAP to less than aMinCAPLength. GTSs shall be allocated on a first-come-first-served basis by the

40	coordinator provided there is sufficient bandwidth available. The coordinator shall make this decision within

41	aGTSDescPersistenceTime superframes.

42

43	On receipt of the acknowledgment to the GTS request command, the device shall continue to track beacons

44	and wait for at most aGTSDescPersistenceTime superframes. If no GTS descriptor for the device appears in

45	the beacon within this time, the MLME of the device shall notify the next higher layer of the failure. This

46	notification is achieved when the MLME issues the MLME-GTS.confirm primitive, see 6.3.6.3, with a

47	status of NO_DATA.

48

49	When the coordinator determines whether capacity is available for the requested GTS, it shall generate a

50	GTS descriptor with the requested specifications and the 16-bit short address of the requesting device. If the

51	GTS was allocated successfully, the coordinator shall set the start slot in the GTS descriptor to the

52	superframe slot at which the GTS begins and the length in the GTS descriptor to the length of the GTS. In

53	addition, the coordinator shall notify the next higher layer of the new GTS. This notification is achieved

54	when the MLME of the coordinator issues the MLME-GTS.indication primitive, see 6.3.6.2, with the

60	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

characteristics of the allocated GTS. If there was not sufficient capacity to allocate the requested GTS, the              1 start slot shall be set to zero and the length to the largest GTS length that can currently be supported. The              2 coordinator shall then include this GTS descriptor in its beacon and update the GTS Specification field of              3 the beacon frame accordingly. The coordinator shall also update the Final CAP Slot subfield of the              4

Superframe Specification field of the beacon frame, indicating the final superframe slot utilized by the              5 decreased CAP. The GTS descriptor shall remain in the beacon frame for aGTSDescPersistenceTime             6 superframes, after which it shall be removed automatically. The coordinator shall be allowed to reduce its              7

CAP below aMinCAPLength to accommodate the temporary increase in the beacon frame length due to the              8 inclusion of the GTS descriptor.                                                                                                                                       9

10

On receipt of a beacon frame containing a GTS descriptor corresponding to macShortAddress, the device              11 shall process the descriptor. The MLME of the device shall then notify the next higher layer of whether the              12

GTS allocation request was successful. This notification is achieved when the MLME issues the MLME-              13

GTS.confirm primitive with a status of SUCCESS (if the start slot in the GTS descriptor was greater than              14 zero) or DENIED (if the start slot was equal to zero or if the length did not match the requested length).                     15

16

5.1.10.3 GTS usage                                                                                                                                       17

18

When the MAC sublayer of a device that is not the coordinator receives an MCPS-DATA.request primitive,              19 see 6.2.1, with the TxOptions parameter indicating a GTS transmission, it shall determine whether it has a              20 valid transmit GTS. If a valid GTS is found, the MAC sublayer shall transmit the data during the GTS, i.e.,              21 between its starting slot and its starting slot plus its length. At this time, the MAC sublayer shall transmit the              22

MPDU immediately without using any random access, provided the requested transaction can be completed              23 before the end of the GTS. If the requested transaction cannot be completed before the end of the current              24

GTS, the MAC sublayer shall defer the transmission until the specified GTS in the next superframe. Note              25 that the MAC shall allow for the PHY overhead in making this determination.                                                              26

27

If the device has any receive GTSs, the MAC sublayer of the device shall ensure that the receiver is enabled              28 at a time prior to the start of the GTS and for the duration of the GTS, as indicated by its starting slot and its              29 length.                                                                                                                                                                                30

31

When the MAC sublayer of the coordinator receives an MCPS-DATA.request primitive with the TxOptions              32 parameter  indicating  a  GTS  transmission,  it  shall  determine  whether  it  has  a  valid  receive  GTS              33 corresponding to the device with the requested destination address. If a valid GTS is found, the coordinator              34 shall defer the transmission until the start of the receive GTS. In this case, the address of the device with the              35 message requiring a GTS transmission shall not be added to the list of pending addresses in the beacon              36 frame as shown in 5.1.8. At the start of the receive GTS, the MAC sublayer shall transmit the data without              37 using any random access, provided the requested transaction can be completed before the end of the GTS. If              38 the requested transaction cannot be completed before the end of the current GTS, the MAC sublayer shall              39 defer the transmission until the specified GTS in the next superframe.                                                                           40

41

For all allocated transmit GTSs (relative to the device), the MAC sublayer of the coordinator shall ensure              42 that its receiver is enabled at a time prior to the start and for the duration of each GTS.                                                 43

44

Before commencing transmission in a GTS, each device shall ensure that the data transmission, the             45 acknowledgment, if requested, and the IFS, suitable to the size of the data frame, can be completed before              46 the end of the GTS.                                                                                                                                                           47

48

If a device misses the beacon at the beginning of a superframe, it shall not use its GTSs until it receives a              49 subsequent beacon correctly. If a loss of synchronization occurs due to the loss of the beacon, the device              50 shall consider all of its GTSs deallocated.                                                                                                                        51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	61

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.10.4 GTS deallocation

2

3	A device is instructed to request the deallocation of an existing GTS through the MLME-GTS.request

4	primitive specified in 6.3.6.1, using the characteristics of the GTS it wishes to deallocate. From this point

5	onward, the GTS to be deallocated shall not be used by the device, and its stored characteristics shall be

6	reset.

7

8	To request the deallocation of an existing GTS, the MLME shall send the GTS request command, specified

9	in 5.4.18, to the coordinator. The Characteristics Type subfield of the GTS Characteristics field of the

10	request shall be set to zero (i.e., GTS deallocation), and the length and direction subfields shall be set

11	according to the characteristics of the GTS to deallocate. Because the GTS request command contains an

12	acknowledgment request, specified in 5.4.4.1, the coordinator shall confirm its receipt by sending an

13	acknowledgment frame. On receipt of the acknowledgment to the GTS request command, the MLME shall

14	notify the next higher layer of the deallocation. This notification is achieved when the MLME issues the

15	MLME-GTS.confirm primitive, see 6.3.6.3, with a status of SUCCESS and a GTSCharacteristics parameter

16	with its Characteristics Type subfield set to zero. If the GTS request command is not received correctly by

17	the coordinator, it shall determine that the device has stopped using its GTS by the procedure described in

18	5.1.10.6.

19

20	On receipt of a GTS request command with the Characteristics Type subfield of the GTS Characteristics

21	field set to zero (GTS deallocation), the coordinator shall attempt to deallocate the GTS. If the GTS

22	characteristics contained in the GTS request command do not match the characteristics of a known GTS, the

23	coordinator shall ignore the request. If the GTS characteristics contained in the GTS request command

24	match the characteristics of a known GTS, the MLME of the coordinator shall deallocate the specified GTS

25	and notify the next higher layer of the change. This notification is achieved when the MLME issues the

26	MLME-GTS.indication  primitive,  see  6.3.6.2,  with  a  GTSCharacteristics  parameter  containing  the

27	characteristics of the deallocated GTS and a Characteristics Type subfield set to zero. The coordinator shall

28	also update the Final CAP Slot subfield of  the Superframe Specification field of the beacon frame,

29	indicating the final superframe slot utilized by the increased CAP. It shall not add a descriptor to the beacon

30	frame to describe the deallocation.

31

32	GTS deallocation may be initiated by the coordinator due to a deallocation request from the next higher

33	layer, the expiration of the GTS (see 5.1.10.6), or maintenance required to maintain the minimum CAP

34	length, aMinCAPLength (see 5.1.10.1).

35

36	When a GTS deallocation is initiated by the next higher layer of the coordinator, the MLME shall receive the

37	MLME-GTS.request primitive with the GTS Characteristics field of the request set to zero (i.e. GTS

38	deallocation) and the length and direction subfields set according to the characteristics of the GTS to

39	deallocate.

40

41	When a GTS deallocation is initiated by the coordinator either due to the GTS expiring or due to CAP

42	maintenance, the MLME shall notify the next higher layer of the change. This notification is achieved when

43	the MLME issues the MLME-GTS.indication primitive with a GTSCharacteristics parameter containing the

44	characteristics of the deallocated GTS and a Characteristics Type subfield set to zero.

45

46	In the case of any deallocation initiated by coordinator, the coordinator shall deallocate the GTS and add a

47	GTS descriptor into its beacon frame corresponding to the deallocated GTS, but with its starting slot set to

48	zero. The descriptor shall remain in the beacon frame for aGTSDescPersistenceTime superframes. The

49	coordinator shall be allowed to reduce its CAP below aMinCAPLength to accommodate the temporary

50	increase in the beacon frame length due to the inclusion of the GTS descriptor.

51

52	On receipt of a beacon frame containing a GTS descriptor corresponding to macShortAddress and a start slot

53	equal to zero, the device shall immediately stop using the GTS. The MLME of the device shall then notify

54	the next higher layer of the deallocation. This notification is achieved when the MLME issues the MLME-

62	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

GTS.indication  primitive  with  a  GTSCharacteristics  parameter  containing  the  characteristics  of  the	1 deallocated GTS and a Characteristics Type subfield set to zero.	2

3

5.1.10.5 GTS reallocation	4

5

The deallocation of a GTS may result in the superframe becoming fragmented. For example, Figure 26	6 shows three stages of a superframe with allocated GTSs. In stage 1, three GTSs are allocated starting at slots	7

14, 10, and 8, respectively. If GTS 2 is now deallocated (stage 2), there will be a gap in the superframe              8 during which nothing can happen. To solve this, GTS 3 will have to be shifted to fill the gap, thus increasing              9 the size of the CAP (stage 3).                                                                                                                                            10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

Figure 26—CFP defragmentation on GTS deallocations	42

43

44

The coordinator shall ensure that any gaps occurring in the CFP, appearing due to the deallocation of a GTS,	45

are removed to maximize the length of the CAP.	46

47

When a GTS is deallocated by the coordinator, it shall add a GTS descriptor into its beacon frame indicating	48

that the GTS has been deallocated. If the deallocation is initiated by a device, the coordinator shall not add a	49

GTS descriptor into its beacon frame to indicate the deallocation. For each device with an allocated GTS	50

having a starting slot lower than the GTS being deallocated, the coordinator shall update the GTS with the	51

new starting slot and add a GTS descriptor to its beacon corresponding to this adjusted GTS. The new	52

starting slot is computed so that no space is left between this GTS and either the end of the CFP, if the GTS	53

appears at the end of the CFP, or the start of the next GTS in the CFP.	54

Copyright © 20xx IEEE. All rights reserved.	63

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	In situations where multiple reallocations occur at the same time, the coordinator may choose to perform the

2	reallocation   in   stages.   The   coordinator   shall   keep   each   GTS   descriptor   in   its   beacon   for

3	aGTSDescPersistenceTime superframes.

4

5	On receipt of a beacon frame containing a GTS descriptor corresponding to macShortAddress and a

6	direction and length corresponding to one of its GTSs, the device shall adjust the starting slot of the GTS

7	corresponding to the GTS descriptor and start using it immediately.

8

9	In cases where it is necessary for the coordinator to include a GTS descriptor in its beacon, it shall be

10	allowed to reduce its CAP below aMinCAPLength to accommodate the temporary increase in the beacon

11	frame  length. After  aGTSDescPersistenceTime  superframes,  the  coordinator  shall  remove  the  GTS

12	descriptor from the beacon.

13

14	5.1.10.6 GTS expiration

15

16	The MLME of the coordinator shall attempt to detect when a device has stopped using a GTS using the

17	following rules:

18

19	—	For a transmit GTS, the MLME of the coordinator shall assume that a device is no longer using its

20	GTS if a data frame is not received from the device in the GTS at least every 2n superframes, where

21	n is defined below.

22	—	For receive GTSs, the MLME of the coordinator shall assume that a device is no longer using its

23	GTS if an acknowledgment frame is not received from the device at least every 2n superframes,

24	where n is defined below. If the data frames sent in the GTS do not require acknowledgment frames,

25	the MLME of the coordinator will not be able to detect whether a device is using its receive GTS.

26	However, the coordinator is capable of deallocating the GTS at any time.

27

28	The value of n is defined as follows:

29

30	n = 2(8-macBeaconOrder)	0 macBeaconOrder 8

31

32	n = 1	9  macBeaconOrder 14

33

34	5.1.11 Fast link recovery

35

36	In the star topology, a fast link recovery process may be triggered at the device end during communication.

37	The trigger may be initiated when the device does not receive ACKs for a number of times given by the

38	MAC PIB attribute macNumAcks, as defined in Table 91. In the fast link recovery process, the device may

39	decide on its own to stop sending data. The device may also send the fast link recovery (FLR) signal

40	repeatedly (within the allocated resource) to the coordinator if the device is connected to mains power. Upon

41	receiving the FLR signal, the coordinator shall send a FLR response to the device. The communication

42	resumes after the device receives the response. If there is bi-directional data transfer during communication,

43	the device may wait after stopping sending data. If the device does not receive any FLR response signal

44	within a timer given by the MAC PIB attribute macLinkTimeOut, the device may assume the link is broken

45	and may disassociate.

46

47	The FLR signal and response are defined in 5.4.16. The FLR signal and response shall be sent at the lowest

48	data rate corresponding to the currently negotiated optical rate.

49

50	Figure 27 shows an example of the process of device stopping sending data based on the retransmission

51	count.

52

53

54

64	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

Figure 27—An example of the process of device stopping data	22

transmission based on the retransmission count, and triggering FLR	23

24

25

26

In the star topology, a fast link recovery may also be triggered by the coordinator. The trigger may be	27

initiated when the coordinator does not receive contiguous ACKs for a number of times given by the MAC	28

PIB attribute macNumAcks. In the fast link recovery process, the coordinator may stop sending data to the	29

device. The coordinator then sends fast link recovery (FLR) signal repeatedly to the device. The coordinator	30

may hold the uplink grant allocated to the device. Upon receiving FLR signal, the device shall send a FLR	31

response to the coordinator. The communication resumes after the device receives the response.	32

33

Figure 28  shows  an  example  of  the  process  of  the  coordinator stopping  sending  data  based  on  the	34

retransmission count.	35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	65

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18	Figure 28—An example of the process of the coordinator stopping sending

19	data based on the retransmission count, and triggering FLR

20

21

22

23	In peer-to-peer OWC, the devices may let each other know their battery life. If the conditions to trigger the

24	fast link recovery process are satisfied, the device may further compare its own battery life with the battery

25	life of its peer (the one it is communicating). If the battery life of the device is shorter than its peer’s, then the

26	device stops sending data, and waits. If the battery life of the device is longer than its peer’s, then the device

27	stops sending data and initiates the fast link recovery process.

28

29	When the fast link recovery is triggered, and if the device has spare wavelength bands, some or all of the

30	spare bands may also start sending fast link recovery signals to recover the link. The device then shall

31	choose a band that gets the fast link recovery response to continue the communication.

32

33	The address field of MHR in FLR signal and response may include the address or the identifier of the color

34	bands.

35

36	Figure 29 shows a flowchart of the process for color band assisted fast link recovery.

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

66	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

Figure 29—Flowchart of process for color band assisted fast link recovery	22

23

24

25

When the fast link recovery is triggered, if the device has other communication directions/angles (e.g., a              26 light with multiple LEDs with different angles) some or all of the other angles may also start sending fast              27 link recovery signaling to recover the link. The device then shall choose an angle that gets the fast link              28 recovery response to continue the communication. The process of fast link recovery on other directions/              29 angles is done successively (i.e., one direction after another). The direction is indicated in the link recovery              30 mechanism provided by the command frame structure.                                                                                                   31

32

The address field of MHR in FLR signal and response may include the address or the identifier of the angles	33 or directions.	34

35

Figure 30 shows a flowchart of the process for multiple angle assisted fast link recovery.	36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	67

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26	Figure 30—Flowchart showing process of multiple angles assisted fast link recovery

27

28

29	5.1.12 OWC cell design and mobility support

30

31

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

68	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

2

3

12

13

14

15

time slot n for device 1	16

PHY switch	17

18

19

Cell_ID(i+1,j)	20

Cell_ID(i,j)

Cell_ID(i+2,j)	21

22

23

24

25

26

27

28

29

30

31

D/L	U/L	D/L	U/L	32

33

34

35

36

37

38

39

40

41

42

43

device 1	44

45

46

54

Copyright © 20xx IEEE. All rights reserved.	69

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

9

10

11

12	coordinator

13	Cell_ID  (i,j)

14

Cell_ID  (i+1,j)

Cell_ID (i+2,j)

15	Cell_ID (i,1) 	Cell_ID  (i,2) 	Cell_ID (i,3) 	Cell_ID (i+1,1)

16

18

17

Cell_ID (i,4) 	Cell_ID  (i,5) 	Cell_ID (i,6) 	Cell_ID (i+1,3)

19

Cell_ID  (i+1,2)      Cell_ID (i+ 2,1)       Cell_ID  (i+2,2)

Cell_ID  (i+1,4)      Cell_ID (i+2,5)     Cell_ID  (i+2,6)

Cell_ID  (i+2,3)

Cell_ID (i+2,7)

Device 2

Cell_ID (i+2,4)

Cell_ID  (i+2, 8)

20	Device 1

21	Time slot n

Device 1

Time slot n+ 2

Time slot n+1

22	Cell_ID (i,7) 	Cell_ID  (i,8) 	Cell_ID (i,9) 	Cell_ID (i+1,5)

23

24

Cell_ID  (i+1,6)      Cell_ID (i+ 2,9)    Cell_ID (i+2,10)    Cell_ID  (i+2,11)

Cell_ID (i+2,12)

25

26

27

28

29

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

70	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Cell_ID (1,j) 	Cell_ID (2,j) 	Cell_ID (1,j)

1

2

3

4

5

6

7

8

9

10

device 2

11

12

13

14

15

Cell_ID (2,j)

16

device 1

|  |  |  | CFP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | beacon | CAP |  |  |  |  |  |  |
| macro cell |  |  |  |  |  |  |  |  |
| device 2 device 1 |  |  |  |  |  |  |  |  |

Cell_ID (i,j)

Cell_ID (3,j)

Cell_ID (4,j) 	Cell_ID (3,j) 	Cell_ID (4,j) 	17

18

19

20

21

22

23

24

25

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	71

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

|  |  | CFP |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | cell search |  |  |  |  |
| beacon | CAP | CS | CS | CS | CS |  |

2

3

4

5

6

7

8

9

10

11	macro cell

12

13

1 	2 	3 	4

14	device 2

15

16

Cel l_ID (1, j) 	Cel l_ID (2,j)

device 1

Cell_ID (4,j )

Cell_ID ( 1, j) 	Cell_ID (2,j)

devi ce 1

Cel l_ID (4,j)

Cell _ID (1, j) 	Cell_ID (2,j)

device 1

Cell_ID (4,j)

Cell_ID (1,j ) 	Cel l_ID (2,j)

device 1

Cell _ID (4, j)

17	device 1

18

19

20

21

Cell_ID (i,j)

Cell_ID (3,j )

Cell_ID (3,j ) 	Cel l_ID (3,j) 	Cell_ID (3,j )

22

23

24

25

33

34	5.1.13 CSI feedback and link adaptation

35

36	5.1.13.1 CSI feedback for MCS selection

37

38	In MAC header, a link adaptation control field is defined.

39

40

41	Table 5—Link adaptatio n c o ntro l fie ld

42

| MCS request | MCS sequence | Feedback sequence index | MCS feedback |
| --- | --- | --- | --- |
| 1 bit | 3 bits | 3 bits | 3 bits |
| 0 no MCS request 1 MCS request | The index of the MCS request | To which MCS request the MCS feedback applies | Suggest MCS level |

43

44

45

46

47

48

49

50

51

52	Two types of CSI feedbacks are supported.

53

54

72	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1)	Solicited feedback: a CSI requester request a CSI responder to feedback CSI.	1

2)	Unsolicited feedback: a coordinator/device report CSI to another device/coordinator without a	2 request.	3

4

For solicited feedback, the CSI requester may set the MCS request to 1 to request a responder to provide              5

MCS recommendation. In each MCS request, the requester shall set the MSI (MCS sequence index) subfield              6 in the link adaptation control field to a value in the range 0 to 6. How the requester chooses the MSI value is              7 implementation dependent.                                                                                                                                               8

9

On receipt of a frame with the MCS request subfield equal to 1, an CSI responder initiates computation of              10 the MCS estimate and labels the result of this computation with the MSI value. The MFB responder includes              11 the received MSI value in the feedback sequence index field of the corresponding response frame. The sug-              12 gested MCS level is transmitted in MCS feedback field.                                                                                                 13

14

For unsolicited feedback, a device/coordinator feedbacks a suggest MCS level without being requested. In              15 this case, feedback sequence index is set to 7, and suggested MCS level is transmitted in MCS feedback              16 field.                                                                                                                                                                                  17

18

After CSI feedback is obtained by the transmitter, it selects a MCS level for future transmissions. The              19 selected MCS level may or may not be the one suggested by the receiver.                                                                      20

21

The selected MCS level for data transmission is indicated in PHY header.                                                                     22

23

5.1.13.2 CSI feedback for bit-loading                                                                                                          24

25

Editor note: Volker will provide text in the next round of comment resolution, see comment 80 of https://men-              26 tor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx The   procedures  described              27 here may also influence specifications for primitives, command frames, frame structures.                                            28

29

5.1.13.3 CSI feedback for MIMO operation                                                                                                 30

31

Editor note: Volker will provide text in the next round of comment resolution, see comment 83 of https://men-              32 tor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx The   procedures  described              33 here may also influence specifications for primitives, command frames, frame structures.                                            34

35

5.1.13.4 CSI feedback for coordinated topology                                                                                        36

37

Editor note: Volker will provide text in the next round of comment resolution, see comment 81 of https://men-              38 tor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx The   procedures  described              39 here may also influence specifications for primitives, command frames, frame structures.                                            40

41

5.1.14 Relaying,                                                                                                                                             42

43

5.1.14.1 Relay Discovery Procedure                                                                                                           44

45

The MAC protocol supports optional relaying mechanism for the cases when dedicated relay terminals are              46 available. The relay terminals are named as "relay STAs". The AP keeps track of the relay STAs in the              47 environment. At any given point in time, the AP can send an Advanced Modulation Control frame to a STA,              48 which contains the APs capabilities, but alos serves as a signal to the STA to transmit its own capabilities              49 using an Advanced Modulation Control frame. All the STAs in the environment, that have received an              50

Advanced Modulation Control frame including the relay STAs, reply with their capabilities. The AP collects              51 the relaying capabilities of the STAs along with the full set of capabilities included in the Advanced              52

Modulation Control frame. Subclause 5.8.4.3.1.5 describes the signalling mechanism for the relaying              53

54

Copyright © 20xx IEEE. All rights reserved.	73

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	capabilities using the Advanced Modulation control frame. Note that this procedure can occur at different

2	points in time for the different STAs.

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30	Figure 36—Protocol for establishing STA relay capabilities

31

32

33	The PHY layer protocol at each STA measures the signal to interference and noise (SINR) on each

34	subcarrier for each link and signals it to the AP through a CSI control frame. The AP obtains CQI reports,

35	which can also be used for the additional advanced modulation capabilities. Subclause 5.8.4.3.2 describes

36	the structure of the CSI control frame. Note that this procedure can occur at different points in time for the

37	different STAs and also is not obligatory before every relaying instruction, given that the CSI available to

38	the AP is still applicable. The relay discovery procedure is illustrated in Figure 36.

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

74	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

Figure 37—Relay selection procedure	30

31

5.1.14.2 Relay Selection Procedure	32

33

The relay selection procedure defines selection of the best relay STA and relaying mode among multiple              34 relay STAs. The relay selection procedure is based on the capabilities for each relay STA and the CSI              35 available at the AP. Figure 37 shows the best relay selection algorithm based on the predefined constraints              36 and performance metrics. The AP calculates the constraint metric for each relay with HD/FD and AF/DF              37 relaying modes. Then, the transmitter decides the best relay together with a relaying mode that satisfies the              38 constraint.                                                                                                                                                                          39

40

5.1.14.3 Relay Link Setup (RLS) Procedure	41

42

Following the selection of the relay to be used between the AP and STA, a RLS request frame is sent to the              43 selected relay. The RLS Request frame includes the capabilities and the IDs of the AP, the STA and the relay              44 transfer parameter set. Upon receiving the RLS Request frame, the relay STA shall transmit a RLS Request              45 frame to the destination containing the same information as received within the frame body of the source              46

RLS Request frame. RLS response frame indicates whether the STA can participate in the RLS. Finally, the	47

AP transmits the RLS announcement that indicates the RLS procedure was successfully completed.	48

49

5.1.14.4 Frame Exchange Rules	50

51

Figure 39 pictures the packet loss and retransmission scenarios for HD relaying. In Figure 39, there is a	52 single user in the network. The AP transmits the first data packet to the STA. The packet is received by both	53

54

Copyright © 20xx IEEE. All rights reserved.	75

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	the STA and the relay STA. The STA responds as soon as it decodes the header. The response is received by

2	both the AP and the relay STA. The AP is aware of the relay STA and waits for its response. The response

3	from the relay STA is also received by the AP. The relay STA stores the first data packet until it receives its

4	acknowledgment.

5

6	Then in the second transmission cycle, the AP sends the second data packet which is received by both the

7	STA and the relay STA. The relay STA starts to store the second data packet. First, the STA responds with

8	the ACK for the first packet. Then, the relay STA sends its response with the ACK of the first packet and

9	deletes the first package from its memory. In the next transmission cycle, the AP transmits the third data

10	packet which is this time received by the relay STA only. The STA does not respond to this packet. The relay

11	STA waits for the response from the STA over a predetermined amount of time and then sends its response

12	to the AP. The response from the relay STA includes the information that it did not receive a response from

13	the STA and will retransmit the third packet to the STA. The AP starts to wait for the next response from the

14	relay STA. The relay STA retransmits the third packet. The STA responds with the ACK for the second data

15	packet. The relay STA also sends its response with ACK from STA. Then, the relay STA deletes the second

16	data packet.

17

18	5.1.14.5 RLS Teardown Procedure

19

20	The RLS Teardown procedure is to terminate the relay operation. The RLS teardown frame is sent by the AP

21	and received by the relay STA. Then, the relay STA terminates the relaying operation for any subsequent

22	packets.

23

24

25

26

27

28

29

30

31

32

33

34

35	Figure 38—RLS procedure

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53	Figure 39—Packet loss and retransmission scenario in HD relaying

54

76	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.1.15 Interference coordination                                                                                                                 1

2

5.1.15.1 General description                                                                                                                        3

4

The OWPANs managed by the same master coordinator (MC) forms a OWPAN cluster. In this clause, only              5 intra-cluster interference coordination are specified.                                                                                                        6

7

At the beginning of the OWPAN establishment, the boundary of the superframe of all the OWPANs in the              8 same cluster should be aligned, and different OWPANs may use the same beacon slot for beacon frame              9 transmission.                                                                                                                                                                     10

11

A device and the coordinator should be capable of detecting the presence of other neighboring OWPANs              12 that have the overlapped coverage with the OWPAN it associated with. The coordinator is responsible for              13 collect ing the interference information from all  devices associated with it  and report to  the master              14 coordinator. The coordinator should be capable of receive the resource coordination information from the              15 master coordinator, which will be used for scheduling the allocations in the superframe.                                              16

17

The interference coordination is executed based on the coordination period. The interference information              18 that is reported in current coordination period should be used for the resource coordination during next coor-              19 dination period. The duration of a coordination period Tcoordination shall equal to the length of             20 macCoordinationTime superframe.                                                                                                                                   21

22

The interference coordination procedure includes the following mechanisms.                                                               23

24 a)     Interference measurement and report                                                                                                                   25 b)     Resource coordination                                                                                                                                          26 c)     Interference parameters/resource coordination update                                                                                        27

5.1.15.2 Interference measurement and report                                                                                          28

29

The  interference  measurement  and  report  shall  follow  the  neighboring  OWPANs  status  monitoring              30 procedure, see 5.1.4.                                                                                                                                                          31

32

The  coordinator  shall  report  the  interference  information  to  the  master  coordinator  every            33 macCoordinationTime superframes (i.e., a coordination period). The value of the coordination period can be              34 determined by the master coordinator.                                                                                                                              35

36

5.1.15.3 Resource Coordination                                                                                                                  37

38

The resource coordination shall be executed for both BP and non-BP (i.e, CAP and CFP) by the global con-              39 troller according to the interference information received from the coordinators. For the BP, the global con-              40 troller shall coordinate the transmissions of beacons in a TDM manner and allocate different beacon slots for              41 the OWPANs that interfere with each other. The structure of the beacon period of a OWPAN refers to section              42

7. For the non-BP part, the master coordinator may coordinate the resources in TDM/WDM/FDM manners,              43 which is vendor discretionary. Other rules and criteria for resource coordination is out of the scope of this              44 standard.                                                                                                                                                                            45

46

The master coordinator allocates resources for the OWPANs that it manages and notify the coordinators              47 about the allocations. The coordinator that does not receive the allocations information from the master              48 coordinator can makes the scheduling by itself; the coordinator that receive the allocations information from              49 the network controller shall take the allocation information into account for scheduling.                                               50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	77

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.15.4 Interference coordination update

2

3	Both the device and the coordinator shall be capable of updating the interference information. The coordina-

4	tor shall update the global record of neighboring OWPANs according to the reports from the associated

5	devices and its detection of neighboring OWPANs, see 5.1.4. The coordinator shall report the updated inter-

6	ference infor-mation to the master coordinator for the coordination in the next coordination period. The mas-

7	ter coordinator shall then update the resource coordination using the latest interference information.

8

9

10

11

12

13

14	GC

GC distributes the allocation inform ation

GC distributes the allocation inform ation

GC distributes the allocation information

15

16

17

18	Coordinator

19

20

21

22	Device1

Coordinator reports the interference info to GC

Device1 reports the interference info to coordinator

Coordinator reports the interference info to GC

Device1 updates the interference info to coordinator

T Coordinator reports the

interference info to GC

T

23

24

25

26	Device2

27

28

29

Tcoordination

Device2 reports the	T

interference info to coordinator

T

30	Figure 40—An example of the interference coordination

31

32	5.1.16 Mobility and handover

33

34	Handover is used when a device moves from the coverage of one OWPAN to other. Two types of handover

35	procedures are specified,

36

37	a)	handover initiated by device

38	b)	handover initiated by master coordinator

39

40

41

42	5.1.16.1 Handover initiated by device

43

44	After association to a OWPAN, a device may search the area for available neighboring coordinators and per-

45	form received signal strength (RSS) measurement. The measurement is based on beacons or reference sig-

46	nals.

47

48	A device may perform alpha-filtering on the measurements based on

49

50

51	RSSn =

52

53

54

 1 –   RS Sn – 1 + Mn

(1)

78	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Where Mn is the latest received measurement result from the physical layer; RSSn is the updated filtered              1 measurement result, that is used for evaluation of reporting criteria or for measurement reporting; RSSn-1 is              2 the old filtered measurement result; α is a filtering-coefficient that can be configured.                                                  3

4

If the RSS of neighbor cells satisfy	5

6

RSSn, tg – RSSn, asso ci ate =

th1

(2)	7

8

11

Then the device should initiate the handover to the target coordinator. Here RSStg is the RSS of the target              9 coordinator and RSSassociate is the RSS of the associated coordinator and Δth1 is a predefined threshold.                      10

Once the handover is initiated by the device, it sends a re-association request command (see 9.5.3) to the tar-              12 get coordinator. The device uses the re-association request to request association as well as to send its pre-              13 ferred QoS requirements to the target coordinator.                                                                                                           14

15

In the reassociation response command (see 9.5.4), the target coordinator indicates whether the request is              16 permitted. Besides, the target coordinator also inform the QoS resources allocated to the device, or suggests              17 alternate level of QoS the target coordinator can support.                                                                                                18

19

The previous coordinator may continue to send the packets that have been store in the buffer to the device.	20

The device may receive these packets to its best effort. If the previous coordinator does not received             21 acknowledgement from the device for N consecutive frames, then the previous coordinator consider the              22 device has left the OWPAN and the transmission is ceased.                                                                                            23

24

25

26

27

28

Device 		Old coordinator

Association and data exchange

Search for Neighbor VPAN and RSS measurement

Re-association request (with QoS requirement)

New 	29

coordinator 	30

31

32

33

34

35

36

37

38

39

Re-association response (with QoS resource allocation) 	40

41

42

Association and data exchange 	43

44

45

46

47

48

Figure 41—Handover initiated by device	49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	79

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.1.16.2 Handover initiated by master coordinator

2

3	After association to a OWPAN, a device may scan the area for available neighboring coordinators and

4	perform received signal strength (RSS) measurement. The measurement is based on beacons or reference

5	signals.

6

7	A device may perform alpha-filtering on the measurements based on

8

9	RSSn =

10

11

 1 –   RS Sn – 1 + Mn

(3)

12	Where Mn is the latest received measurement result from the physical layer; RSSn is the updated filtered

13	measurement result, that is used for evaluation of reporting criteria or for measurement reporting; RSSn-1 is

14	the old filtered measurement result; α is a filtering-coefficient that can be configured.

15

16	The device may report the measured RSS of neighboring OWPANs to the coordinator using the procedure

17	specified in 5.1.4.

18

19	The  coordinator can  send  the  measurement report  to  the  master  coordinator together  with  the  QoS

20	requirement of the device. If the master coordinator decides to handover the device to the target coordinator,

21	it sends its decision to the current coordinator. It also notify the target coordinator about the upcoming

22	handover  together  with  QoS  requirement.  The  procedures  for  the  communications  between  master

23	coordinator and the coordinator are out the scope of this specification.

24

25	Current coordinator sends handover command frame to the device.

26

27	Then the device sends re-association request (see TBD) to the target device.

28

29	Editor note: reassociation request has not been specified. Please John provide text for reassociation request

30	comment then refer to it. see comment 102 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-

31	comment-against-7m-d1.xlsx

32

33	In the re-association response command, the target coordinator confirms the handover. Besides, the target

34	coordinator also informs the QoS resources allocated to the device, or suggests alternate level of QoS the

35	target coordinator can support.

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

80	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Device 		Old coordinator

Association and data exchange

Search for Neighbor VPAN and RSS measurement

RSS measurement report

New coordinator

RSS measurement report and QoS requirement

Handover decision

1

Master 	2

coordinator 	3

4

5

6

7

8

9

10

11

12

13

Handover 	14

decision

15

16

17

Handover command 	Handover decision 	18

19

Re-association request 	20

21

22

Re-association response with QoS resource allocation 	23

24

25

26

Figure 42—Handover initiated by master coordinator	27

28

29

30

5.2 MAC frame formats	31

32

This subclause specifies the format of the MAC frame (MPDU). Each MAC frame consists of the following	33

basic components:	34

35 a)	A MHR, which comprises frame control, sequence number, address information, and security-		36 related information.	37

b)	A  MSDU,   of   variable   length,   which   contains   information  specific   to   the   frame   type.	38

Acknowledgment frames do not contain a payload.	39

c)	A MFR, which contains a FCS.	40

41

The frames in the MAC sublayer are described as a sequence of fields in a specific order. All frame formats	42

in this subclause are depicted in the order in which they are transmitted by the PHY, from left to right, where	43

the left most bit is transmitted first in time. Bits within each field are numbered from 0 (left most and least	44

significant) to k – 1 (right most and most significant), where the length of the field is k bits. Fields that are	45

longer than a single octet are sent to the PHY in the order from the octet containing the lowest numbered bits	46

to the octet containing the highest numbered bits.	47

48

For every MAC frame, all reserved bits shall be ignored upon receipt.	49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	81

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.2.1 General MAC frame format

2

3	The MAC frame format is composed of a MHR, a MSDU, and a MFR. The fields of the MHR appear in a

4	fixed order; however, the addressing fields may not be included in all frames. The general MAC frame shall

5	be formatted as illustrated in Figure 43.

6

| Octets : 2 | 1 | 0/2 | 0/2/8 | 0/2 | 0/2/8 | 0/3/9 | 0/2/8 | 0/5/6/ 10/14 | variab le | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Con- trol | Sequen ce Number | Destina- tion OWPAN Identifier | Destina- tion Address | Source OWPA N Identi- fier | Sourc e Addre ss | Ackno wledg e Field | Poll- ing Field | Auxil- iary Security Header | Frame Pay- load | FCS |
|  |  | Addressing fields |  |  |  |  |  |  |  |  |
| MHR |  |  |  |  |  |  |  |  | MSDU | MFR |

7

8

9

10

11

12

13

14

15

16

17

18

19

20	Figure 43—General MAC frame format

21

22

23	5.2.1.1 Frame control field

24

25	The frame control field is 2 octets in length and contains information defining the frame type, addressing

26	fields, and other control flags. The frame control field shall be formatted as illustrated in Figure 44.

27	Reserved bits are set to zero on transmission and ignored on reception.

28

29

| Bits: 0–1 | 2–5 | 6–8 | 9 | 10 | 11 | 12–13 | 14–15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Version | Reserved | Frame Type | Security Enabled | Frame Pending | Ack Request | Dest Addressing Mode | Source Addressing Mode |

30

31

32

33

34

35

36

37	Figure 44—Format of the frame control field

38

39

40	5.2.1.1.1 Frame Ve rs io n s ubfie ld

41

42	The Frame Version subfield specifies the version number corresponding to the frame. This subfield shall be

43	set to 0b00 to indicate a frame compatible with IEEE Std 802.15.13. All other subfield values shall be

44	reserved for future use.

45

46	5.2.1.1.2 Frame type s ubfie ld

47

48	The Frame Type subfield shall be set to one of the nonreserved values listed in Table 6.

49

50

51

52

53

54

82	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 6—Values o f the Frame Type s ubfie ld	1

| Frame type value b2 b1 b0 | Description |
| --- | --- |
| 000 | Beacon |
| 001 | Data |
| 010 | Acknowledgment |
| 011 | Command |
| 100 | CVD |
| 101 | Control |
| 110–111 | Reserved |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

5.2.1.1.3 S e c urity Enable d s ubfie ld	19

20

The Security Enabled subfield is 1 bit in length, and it shall be set to one if the frame is protected by the	21

MAC sublayer and shall be set to zero otherwise. The Auxiliary Security Header field of the MHR shall be	22

present only if the Security Enabled subfield is set to one.	23

24

5.2.1.1.4 Frame Pe nding s ubfie ld	25

26

The Frame Pending subfield is 1 bit in length and shall be set to one if the device sending the frame has more	27

data for the recipient. This subfield shall be set to zero otherwise (see 5.1.9.3).	28

29

The Frame Pending subfield shall be used only in beacon frames or frames transmitted either during the	30

CAP by  devices operating on  a  beacon-enabled OWPAN or  at  any  time  by  devices operating on  a	31

nonbeacon-enabled OWPAN.	32

33

At all other times, it shall be set to zero on transmission and ignored on reception.	34

35

5.2.1.1.5 Ac kno wle dg me nt Re que s t s ubfie ld	36

37

The Acknowledgment Request subfield is 1 bit in length and specifies whether an acknowledgment is	38

required from the recipient device on receipt of a data or MAC command frame. If this subfield is set to one,	39

the recipient device shall send an acknowledgment frame only if, upon reception, the frame passes the third	40

level of filtering as shown in 5.1.9.2. If this subfield is set to zero, the recipient device shall not send an	41

acknowledgment frame.	42

43

5.2.1.1.6 De s tinatio n Addre s s ing Mo de s ubfie ld	44

45

The Destination Addressing Mode subfield shall be set to one of the nonreserved values listed in Table 7.	46

47

If this subfield is equal to zero and the Frame Type subfield does not specify that this frame is an	48

acknowledgment or beacon frame, the Source Addressing Mode subfield shall be nonzero, implying that the	49

frame is directed to the OWC coordinator with the OWPAN identifier as specified in the Source OWPAN	50

Identifier field. If this subfield is equal to 01, the Source Addressing Mode subfield shall be equal to 01,	51

implying that the frame is a broadcast frame, and no source or destination address fields are present in the	52

frame.	53

54

Copyright © 20xx IEEE. All rights reserved.	83

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table7—Po ss iblevalue so ftheDe s tinatio nAddre s s ingMo deandS o urc e Addre s s ing Mo des ubfie lds

| Addressing mode value b1 b0 | Description |
| --- | --- |
| 00 | OWPAN identifier and address fields are not present. |
| 01 | No address field (broadcast only mode with no address fields present). Addresses with all ones of 16 bits or 64 bits are defined as broadcast. |
| 10 | Address field contains a 16-bit short address. |
| 11 | Address field contains a 64-bit extended address. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15	5.2.1.1.7 S o urc e Addre s s ing Mo de s ubfie ld

16

17	The Source Addressing Mode subfield shall be set to one of the nonreserved values listed in Table 7.

18

19	If this subfield is equal to zero and the Frame Type subfield does not specify that this frame is an

20	acknowledgment frame, the Destination Addressing Mode subfield shall be nonzero, implying that the

21	frame has originated from the coordinator with the OWPAN identifier as specified in the Destination

22	OWPAN Identifier field.

23

24	If this subfield is equal to 01, the Source Addressing Mode subfield shall be equal to 01, implying that the

25	frame is a broadcast frame, and no source or destination address fields are present in the frame.

26

27	5.2.1.2 Sequence Number field

28

29	The Sequence Number field is 1 octet in length and specifies the sequence identifier for the frame.

30

31	For a beacon frame, the Sequence Number field shall specify a BSN. For a data, acknowledgment, or MAC

32	command frame, the Sequence Number field shall specify a DSN that is used to match an acknowledgment

33	frame to the data or MAC command frame.

34

35	5.2.1.3 Destination OWPAN Identifier field

36

37	The Destination OWPAN Identifier field, when present, is 2 octets in length and specifies the unique

38	OWPAN identifier of the intended recipient of the frame. A value of 0xffff in this field shall represent the

39	broadcast OWPAN identifier, which shall be accepted as a valid OWPAN identifier by all devices currently

40	listening to the channel.

41

42	This field shall be included in the MAC frame only if the Destination Addressing Mode subfield of the

43	frame control field is 10 or 11.

44

45	5.2.1.4 Destination Address field

46

47	The Destination Address field, when present, is either 2 octets or 8 octets in length, according to the value

48	specified in the Destination Addressing Mode subfield of the frame control field, see 5.2.1.1.6, and specifies

49	the address of the intended recipient of the frame. A 16-bit value of 0xffff in this field shall represent the

50	broadcast short address, which shall be accepted as a valid 16-bit short address by all devices currently

51	listening to the channel.

52

53	This field shall be included in the MAC frame only if the Destination Addressing Mode subfield of the

54	frame control field is nonzero.

84	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.2.1.5 Source OWPAN Identifier field	1

2

The Source OWPAN Identifier field, when present, is 2 octets in length and specifies the unique OWPAN	3 identifier of the originator of the frame. This field shall be included in the MAC frame only if the Source	4

Addressing Mode is nonzero.	5

6

The OWPAN identifier of a device is initially determined during association on a OWPAN, but may change	7 following a OWPAN identifier conflict resolution as discussed in 5.1.5.	8

9

5.2.1.6 Source Address field	10

11

The Source Address field, when present, is either 2 octets or 8 octets in length, according to the value              12 specified in the Source Addressing Mode subfield of the frame control field, as shown in 5.2.1.1.7, and              13 specifies the address of the originator of the frame. This field shall be included in the MAC frame only if the              14

Source Addressing Mode subfield of the frame control field is 10 or 11.	15

16

5.2.1.7 Auxiliary Security Header field	17

18

The Auxiliary Security Header field has a variable length and specifies information required for security              19 processing, including how the frame is actually protected (security level) and which keying material from              20 the MAC security PIB is used (see 7.5.1). This field shall be present only if the Security Enabled subfield is              21 set to one. For details on formatting, see 7.4.                                                                                                                   22

23

5.2.1.8 Frame Payload field	24

25

The Frame Payload field has a variable length and contains information specific to individual frame types. If              26 the Security Enabled subfield is set to one in the frame control field, the frame payload is protected as              27 defined by the security suite selected for that frame.                                                                                                        28

29

5.2.1.9 FCS field	30

31

The FCS field is 2 octets in length and is explained in Annex C. The FCS is calculated over the MHR and	32

MSDU parts of the frame. The FCS shall be only generated for payloads greater than zero bytes.	33

34

5.2.2 Format of individual frame types	35

36

Six frame types are defined: beacon, data, acknowledgment, command, control and CVD. These frame types	37 are discussed in 5.2.2.1 through 5.2.2.4.3.	38

39

5.2.2.1 Beacon frame format	40

41

The beacon frame shall be formatted as illustrated in Figure 45.	42

43

The GTS fields shall be formatted as illustrated in Figure 46, and the pending address fields shall be	44 formatted as illustrated in Figure 47.	45

46

The order of the fields of the beacon frame shall conform to the order of the general MAC frame as	47 illustrated in Figure 43.	48

49

5.2.2.1.1 Be ac o n frame MHR fie lds	50

51

The MHR for a beacon frame shall contain the frame control field, the Sequence Number field, the Source	52

OWPAN Identifier field, and the Source Address field.	53

54

Copyright © 20xx IEEE. All rights reserved.	85

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

| Octet s: 2 | 1 | 4/10 | 0/5/6/ 10/14 | 3 | 0/1 | variab le | variab le | variab le | 0/1 | varia ble | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fram e Con- trol | Seque nce Num- ber | Addres sing fields | Auxil- iary Secu- rity Header | Super- frame Spec | BP Descri ptor | CAP Descri ptor | GTS fields (Figure 46) | Pend- ing addres s fields (Figure 47) | cell- Search Lengt h | Bea- con Pay- load | FC S |
| MHR |  |  |  | MSDU |  |  |  |  |  |  | M FR |

1

2

3

4

5

6

7

8

9

10

11

12

13

14	Figure 45—Beacon frame format

15

16

| Octets: 1 | 0/1 | variable |
| --- | --- | --- |
| GTS Specification | GTS Directions | GTS List |

17

18

19

20

21

22	Figure 46—Format of the GTS information fields

23

| Octets: 1 | variable |
| --- | --- |
| Pending Address Specification | Address List |

24

25

26

27

28

29	Figure 47—Format of the pending address information fields

30

31	In the frame control field, the Frame Type subfield shall contain the value that indicates a beacon frame, as

32	shown in Table 6, and the Source Addressing Mode subfield shall be set as appropriate for the address of the

33	coordinator transmitting the beacon frame. If protection is used for the beacon, the Security Enabled subfield

34	shall be set to one. If a broadcast data or command frame is pending, the frame pending subfield shall be set

35	to one. All other subfields shall be set to zero by the sender and ignored on reception.

36

37	The Sequence Number field shall contain the current value of macBSN.

38

39	The addressing fields shall comprise only the source address fields. The Source OWPAN Identifier and

40	Source Address  fields  shall  contain  the  OWPAN  identifier  and  address,  respectively,  of  the  device

41	transmitting the beacon.

42

43	The Auxiliary  Security  Header  field,  if  present,  shall  contain  the  information  required  for  security

44	processing of the beacon frame, as specified in 5.2.1.7.

45

46	5.2.2.1.2 S upe rfram e S pe c ific atio n fie ld

47

48	The Superframe Specification field shall be formatted as illustrated in Figure 48.

49

50	The Beacon Order subfield shall specify the transmission interval of the beacon. Refer to 5.1.1 for an

51	explanation of the relationship between the beacon order and the beacon interval.

52

53

54

86	Copyright © 20xx IEEE. All rights reserved.

| Bits: 0–3 | 4-7 | 8-10 | 11-14 | 15 | 16 | 17 | 18 | 19 | 20-23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beacon Order | Super- frame Order | OWPAN mode | Final CAP Slot / Broadcast bandwidth | RTS/ CTS | Bea- con type | OWPAN Coordina- tor | Associ- ation Permit | cell- Search En | Count down |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  | 1 |
| --- | --- |
|  | 2 |
|  | 3 |
|  | 4 |
|  | 5 |
|  | 6 |
|  | 7 |
|  | 8 |
| Figure 48—Format of the Superframe Specification field | 9 |
|  | 10 |
|  | 11 |
| The Superframe Order subfield shall specify the length of time during which the superframe is active (i.e., | 12 |
| receiver enabled), including the beacon frame transmission time. Refer to 5.1.1 for an explanation of the | 13 |

relationship between the superframe order and the superframe duration.	14

15

The OWPAN Mode subfield specifies the topology mode of the OWPAN, and shall be contain one of the	16

nonreserved values listed in Table 8.	17

18

19

Table 8—Valid value s o f the OWPAN Mo de s ubfie ld	20

| OWPAN Mode | Description |
| --- | --- |
| 0b000 | Peer-to-Peer Mode |
| 0b001 | Star Mode |
| 0b010 | Broadcast Mode |
| 0b011 | Relay Mode |
| 0b100 | Coordinated Mode |
| 0b101 - 0b111 | Reserved |

21

22

23

24

25

26

27

28

29

30

31

32

33

The Final CAP Slot / Broadcast bandwidth subfield defines the modulation bandwidth for broadcast when	34

OWPAN mode is set to 0b010. (Editor note: Detailed signaling mapping for broadcast bandwidth is FFS)	35

The Final CAP Slot / Broadcast bandwidth subfield defines the final superframe slot utilized by the CAP if	36

OWPAN mode is set to values other than 0b010. The duration of the CAP, as implied by this subfield, shall              37 be greater than or equal to the value specified by aMinCAPLength. However, an exception is allowed for the              38 accommodation of the temporary increase in the beacon frame length needed to perform GTS maintenance,              39 as in 5.2.2.1.4.                                                                                                                                                                   40

41

The RTS/CTS subfield shall be set to one if RTS/CTS protocol specified in 5.1.2.2 is enabled in the	42

OWPAN, and set to zero otherwise.	43

44

The Beacon Type subfield shall be set to one if the beacon frame is a normal beacon which is transmitted in              45 the beacon period regularly, and set to zero if the beacon frame is an additional beacon which is transmitted              46 in a GTS in the CFP period.                                                                                                                                              47

48

The OWPAN Coordinator subfield shall be set to one if the beacon frame is being transmitted by the	49 coordinator. Otherwise, the OWPAN Coordinator subfield shall be set to zero.	50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	87

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The Association Permit subfield shall be set to one if macAssociationPermit is set to TRUE (i.e., the

2	coordinator is accepting association to the OWPAN). The association permit bit shall be set to zero if the

3	coordinator is currently not accepting association requests on its network.

4

5	If the cellSearchEn bit is set, the cellSearchLength is transmitted as an additional field in the beacon frame,

6	as shown in Figure 45.

7

8	The Countdown field indicates the number of superframes after which the previously announced new

9	configuration parameters of the OWPAN shall take effect. The value of this field shall decrement by one in

10	each superframe until the Countdown is complete.

11

12	5.2.2.1.3 BP De s c ripto r fie ld

13

14	The BP Descriptor field only exists when the OWPAN Mode sub-field in Superframe Specification field is

15	set to 10, i.e., the coordinated mode.

16

17	The BP Descriptor field shall be formatted as illustrated in Figure 49.

18

| Bits: 0 - 3 | 4 - 7 |
| --- | --- |
| Beacon Slot Number | Beacon Slot Used |

19

20

21

22

23

24	Figure 49—BP descriptor field

25

26

27	The Beacon Slot Number subfield shall contain the number of the beacon slots in the BP.

28

29	The Beacon Slot Used subfield shall indicates which beacon slot is used by this OWPAN.

30

31	5.2.2.1.4 GTS S pe c ific atio n fie ld

32

33	The GTS Specification field shall be formatted as illustrated in Figure 50.

34

35

| Bits: 0–2 | 3–6 | 7 |
| --- | --- | --- |
| GTS Descriptor Count | Reserved | GTS Permit |

36

37

38

39

40	Figure 50—Format of the GTS Specification field

41

42

43	The GTS Descriptor Count subfield specifies the number of 3-octet GTS descriptors contained in the GTS

44	List field of the beacon frame. If the value of this subfield is greater than zero, the size of the CAP shall be

45	allowed to dip below aMinCAPLength to accommodate the temporary increase in the beacon frame length

46	caused by the inclusion of the subfield. If the value of this subfield is zero, the GTS Directions field and

47	GTS List field of the beacon frame are not present.

48

49	The GTS Permit subfield shall be set to one if macGTSPermit is equal to TRUE (i.e., the coordinator is

50	accepting GTS requests). Otherwise, the GTS Permit field shall be set to zero.

51

52

53

54

88	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.2.2.1.5 GTS Dire c tio ns fie ld	1

2

The GTS Directions field shall be formatted as illustrated in Figure 51.	3

4

| Bits: 0–6 | 7 |
| --- | --- |
| GTS Directions Mask | Reserved |

5

6

7

8

9

Figure 51—Format of the GTS Directions field	10

11

12

The GTS Directions Mask subfield contains a mask identifying the directions of the GTSs in the superframe.	13

The lowest bit in the mask corresponds to the direction of the first GTS contained in the GTS List field of the              14 beacon frame, with the remainder appearing in the order that they appear in the list. Each bit shall be set to              15 one if the GTS is a receive-only GTS or to zero if the GTS is a transmit-only GTS. GTS direction is defined              16 relative to the direction of the data frame transmission by the device.                                                                             17

18

5.2.2.1.6 GTS Lis t fie ld	19

20

The size of the GTS List field is defined by the values specified in the GTS Specification field of the beacon              21 frame and contains the list of GTS descriptors that represents the GTSs that are being maintained. The              22 maximum number of GTS descriptors shall be limited to seven.                                                                                     23

24

Each GTS descriptor shall be formatted as illustrated in Figure 52.	25

26

27

| Bits: 0–15 | 16–19 | 20–23 |
| --- | --- | --- |
| Device Short Address | GTS Starting Slot | GTS Length |

28

29

30

31

Figure 52—Format of the GTS descriptor	32

33

34

The Device Short Address subfield shall contain the short address of the device for which the GTS	35 descriptor is intended.	36

37

The GTS Starting Slot subfield contains the superframe slot at which the GTS is to begin.	38

39

The GTS Length subfield contains the number of contiguous superframe slots over which the GTS is active.	40

41

5.2.2.1.7 P e nding Addre s s S pe c ific atio n fie ld	42

43

The Pending Address Specification field shall be formatted as illustrated in Figure 53.	44

45

46

| Bits: 0–2 | 3 | 4–6 | 7 |
| --- | --- | --- | --- |
| Number of Short Addresses Pending | Reserved | Number of Extended Addresses Pending | Reserved |

47

48

49

50

51

Figure 53—Format of the Pending Address Specification field	52

53

54

Copyright © 20xx IEEE. All rights reserved.	89

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The Number of Short Addresses Pending subfield indicates the number of 16-bit short addresses contained

2	in the Address List field of the beacon frame.

3

4	The Number of Extended Addresses Pending subfield indicates the number of 64-bit extended addresses

5	contained in the Address List field of the beacon frame.

6

7	5.2.2.1.8 Addre s s Lis t fie ld

8

9	The size of the Address List field is determined by the values specified in the Pending Address Specification

10	field of the beacon frame and contains the list of addresses of the devices that currently have messages

11	pending with the coordinator. The address list shall not contain the broadcast short address 0xffff.

12

13	The maximum number of addresses pending shall be limited to seven and may comprise both short and

14	extended addresses. All pending short addresses shall appear first in the list followed by any extended

15	addresses. If the coordinator is able to store more than seven transactions, it shall indicate them in its beacon

16	on a first-come-first-served basis, ensuring that the beacon frame contains at most seven addresses.

17

18	5.2.2.1.9 Be ac o n P aylo ad fie ld

19

20	The Beacon Payload field is an optional sequence of up to aMaxBeaconPayloadLength octets specified to be

21	transmitted in the beacon frame by the next higher layer. The set of octets contained in macBeaconPayload

22	shall be copied into this field.

23

24	5.2.2.2 Data frame format

25

26	The data frame shall be formatted as illustrated in Figure 54.

27

| Octets: 2 | 1 | (As defined in 5.2.2.2.1) | 0/5/6/10/14 | variable | 2 |
| --- | --- | --- | --- | --- | --- |
| frame control | Sequence Number | Addressing fields | Auxiliary Security Header | Data Payload | FCS |
| MHR |  |  |  | MSDU | MFR |

28

29

30

31

32

33

34

35

36

37	Figure 54—Data frame format

38

39

40	The order of the fields of the data frame shall conform to the order of the general MAC frame as illustrated

41	in Figure 43.

42

43	5.2.2.2.1 Data frame MHR fie lds

44

45	The MHR for a data frame shall contain the frame control field, the Sequence Number field, the destination

46	OWPAN identifier/address fields, and/or the source OWPAN identifier/address fields.

47

48	In the frame control field, the Frame Type subfield shall contain the value that indicates a data frame, as

49	shown in Table 6. If protection is used for the data, the Security Enabled subfield shall be set to one. All

50	other subfields shall be set appropriately according to the intended use of the data frame. All reserved

51	subfields shall be set to zero by the sender and ignored on reception.

52

53	The Sequence Number field shall contain the current value of macDSN.

54

90	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The  addressing fields  shall  comprise the  destination address  fields  and/or  the  source  address fields,	1 dependent on the settings in the frame control field.	2

3

The Auxiliary  Security  Header  field,  if  present,  shall  contain  the  information  required  for  security	4 processing of the data frame, as specified in 5.2.1.7.	5

6

5.2.2.2.2 Data Paylo ad fie ld	7

8

The payload of a data frame shall contain the sequence of octets that the next higher layer has requested the	9

MAC sublayer to transmit. The data type field is 1 byte and is explained in Table 9.	10

11

12

Table 9—Data Paylo ad fie ld	13

| Bits 0–1 | Bits 2–7 | variable |
| --- | --- | --- |
| 00—Single 01—Packed 10—Burst 11—Reserved | Number of PPDUs per data frame | Data payload |

14

15

16

17

18

19

20

21

22

The data type field mentions the format used for sending the data—single, packed, or burst. It also mentions	23 the number of PPDUs that are associated for this data frame.	24

25

The payload of a data frame shall contain the sequence of octets that the next higher layer has requested the	26

MAC sub layer to transmit.	27

28

5.2.2.3 Acknowledgment frame format	29

30

The acknowledgment frame shall be formatted as illustrated in Figure 55.	31

32

| Octets: 2 | 1 | variable | 2 |
| --- | --- | --- | --- |
| frame control | sequence number | B-ACK frame payload (optional) | FCS |
| MHR |  | MSDU | MFR |

33

34

35

36

37

38

39

40

Figure 55—Acknowledgment frame format	41

42

43

The order of the fields of the acknowledgment frame shall conform to the order of the general MAC frame	44 as illustrated in Figure 43. The sequence number is defined in 5.2.2.1.1.	45

46

In B-ACK frames, the DestAddr field is set to the SrcAddr of the frame that requested the B-ACK. The B-	47

ACK frame acknowledges correct or incorrect receipt of the previous sequence of frames and provides              48 information for the transmission of the next sequence of frames as described in 5.2.2.3. The B-ACK frame              49 payload is defined in Figure 56.                                                                                                                                        50

51

The Buffer Size field specifies the maximum number of octets in the sum of the frame payloads of all frames	52 in the next B-ACK sequence. The Frame Count field specifies the maximum number of frames in the next	53

B-ACK sequence. The Sequence Control and frame bitmap fields together specify an acknowledgment	54

Copyright © 20xx IEEE. All rights reserved.	91

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

| Octets: 2 | 1 | 1 | 2 | 0 – n |
| --- | --- | --- | --- | --- |
| buffer size | frame count | reserved | sequence control | frame bitmap |

1

2

3

4

5	Figure 56—B-ACK frame payload

6

7

8	window of MSDU fragments and their reception status. The Sequence Control field specifies the Sequence

9	Number and Fragment Number that start the acknowledgment window.

10

11

| Bits: b15–b14 | b13–b3 | b2–b0 |
| --- | --- | --- |
| reserved | sequence number | fragment number |

12

13

14

15

16	Figure 57—B-ACK frame bitmap

17

18

19	The frame bitmap field varies in length. A zero-length frame bitmap field indicates an acknowledgment

20	window of length zero. Otherwise, the least-significant octet of the frame bitmap field corresponds to the

21	MSDU indicated by the Sequence Control field, and each bit of the octet corresponds to a fragment of that

22	MSDU. The  least-significant bit  in  each  octet  corresponds to  the  first  fragment and  successive bits

23	correspond to successive fragments. Successive octets present in the frame bitmap field correspond to

24	successive MSDUs, and each bit corresponds to a fragment of the MSDU. The acknowledgment window

25	ends at fragment seven of the MSDU that corresponds to the most-significant octet in the frame bitmap. For

26	all bits within the frame bitmap, a value of ONE indicates that the corresponding fragment was received in

27	either the current sequence or an earlier one. A value of ZERO indicates that the corresponding fragment

28	was not received in the current sequence (although it may have been received in an earlier one). Bits of the

29	least-significant octet of  the frame bitmap field corresponding to  fragments prior to  the start of  the

30	acknowledgment window are undefined. Frames with a  Sequence Number earlier than the Sequence

31	Number indicated in the Sequence Control field were not received in the last B-ACK sequence. Such frames

32	were previously received or are no longer expected.

33

34	The block ACK is applicable to the packed data type. The bitmap and sequence number is repeated for every

35	frame in the burst mode (multiple frames)

36

37	The order of the fields of the acknowledgment frame shall conform to the order of the general MAC frame

38	as illustrated.

39

40	The MHR for an acknowledgment frame shall contain only the frame control field and the Sequence

41	Number field.

42

43	In the frame control field, the Frame Type subfield shall contain the value that indicates an acknowledgment

44	frame, as shown in Table 6. If the acknowledgment frame is being sent in response to a received data request

45	command, the device sending the acknowledgment frame shall determine whether it has data pending for the

46	recipient. If the device can determine this before sending the acknowledgment frame (see 5.1.9.4.2), it shall

47	set the frame pending subfield according to whether there is pending data. Otherwise, the frame pending

48	subfield shall be set to one. If the acknowledgment frame is being sent in response to either a data frame or

49	another type of MAC command frame, the device shall set the frame pending subfield to zero. All other

50	subfields, except the security enabled subfield, shall be set to zero by the sender and ignored on reception.

51

52	The Sequence Number field shall contain the value of the sequence number received in the frame for which

53	the acknowledgment is to be sent.

54

92	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.2.2.4 Command frame format	1

2

The command frame shall be formatted as illustrated in Figure 58.	3

4

| Octets: 2 | 1 | (As defined in 5.2.2.4.1) | 0/5/6/10/14 | 1 | variable | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Sequence Number | Addressing fields | Auxiliary Security Header | Command Frame Identifier | Command Payload | FCS |
| MHR |  |  |  | MSDU |  | MFR |

5

6

7

8

9

10

11

12

13

14

Figure 58—Command frame format	15

16

17

The order of the fields of the MAC command frame shall conform to the order of the general MAC frame as	18

illustrated in Figure 43.	19

20

5.2.2.4.1 MAC c o m mand frame MHR fie lds	21

22

The MHR for a MAC command frame shall contain the frame control field, the Sequence Number field, the	23

destination OWPAN identifier/address fields and/or the source OWPAN identifier/address fields.	24

25

In the frame control field, the Frame Type subfield shall contain the value that indicates a MAC command	26

frame, as shown in Table 6. If the frame is to be secured, the Security Enabled subfield of the frame control	27

field shall be set to one and the frame secured according to the process described in 7.5.4. Otherwise the	28

Security Enabled subfield of the frame control field shall be set to zero. All other subfields shall be set	29

appropriately according to the intended use of the MAC command frame. All reserved subfields shall be set	30

to zero by the sender and ignored on reception.	31

32

The Sequence Number field shall contain the current value of macDSN.	33

34

The  addressing fields  shall  comprise the  destination address  fields  and/or  the  source  address fields,	35

dependent on the settings in the frame control field.	36

37

The Auxiliary  Security  Header  field,  if  present,  shall  contain  the  information  required  for  security	38

processing of the MAC command frame, as specified in 5.2.1.7.	39

40

5.2.2.4.2 Co mmand Frame Ide ntifie r fie ld	41

42

The Command Frame Identifier field identifies the MAC command being used. This field shall be set to one	43

of the nonreserved values listed in Table 28.	44

45

5.2.2.4.3 Co mmand Paylo ad fie ld	46

47

The Command Payload field contains the MAC command itself. The formats of the individual commands	48

are described in 5.4.	49

50

5.2.2.5 Control frame format	51

52

The Control frame shall be formatted as illustrated in Figure 59.	53

54

Copyright © 20xx IEEE. All rights reserved.	93

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

| Octets: 2 | 1 | (As defined in 5.2.2.4. 1) | 0/5/6/ 10/14 | 1 | 2 |  |  | 1 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame control | Sequen- ceNum- ber | Address ing fields | Auxil- iary Secu- rity Header | Control frame type | Dura- tion | RA | TA | Band- width Info | FCS |
| MHR |  |  |  | MSDU |  |  |  |  | MFR |

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17	Figure 59—Control frame format

18

19	The order of the fields of the control frame shall conform to the order of the general MAC frame as illus-

20	trated in Figure 43.

21

22	5.2.2.5.1 Co ntro l frame MHR fie lds

23

24	The MHR for a control frame shall contain the frame control field, the SequenceNumber field.

25

26	In the frame control field, the Frame Type subfield shall contain the value that indicates a control frame, as

27	shown in Table 6. The Source Addressing Mode and Destination Addressing Mode shall be set to 01, to indi-

28	cate that the control frames are broadcast frames.

29

30	5.2.2.5.2 Co ntro l frame type fie ld

31

32	The control frame type field identifies the type of the MAC control frame being used. This field shall be set

33	to one of the nonreserved values in Table 10.

34

35	Table 10—Valid values o f the contro l frame type fie ld

36

37

38

| Control frame type hex value | control frame |
| --- | --- |
| 0x01 | RTS |
| 0x02 | CTS |
| 0x03-0xff | Reserved |

39

40

41

42

43

44

45

46

47

48

49	5.2.2.5.3 Duratio n fie ld

50

51	If the Control frame type subfield is set to 0x01, i.e., for all RTS frames, the duration time value is the time,

52	in microseconds, required to transmit the pending PPDU, plus one CTS frame, plus one ACK frame (if

53	acknowledge is required), plus the IFSs: two SIFS and a tack (if acknowledge is required). If the calculated

54	duration includes a fractional microsecond, that value is rounded up to the next higher integer.

94	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

If the Control frame type subfield is set to 0x02, i.e., for CTS frames, the duration value is the value obtain              1 from the Duration subfield of the immediately previous RTS frame, minus the time, in microseconds,              2 required to transmit the CTS frame and its SIFS interval. If the calculated duration includes a fractional              3 microsecond, that value is rounded up to the next higher integer.                                                                                    4

5

5.2.2.5.4 RA fie ld	6

7

The RA field of a RTS frame is the 64-bit extended address of the device or coordinator that is the intended	8 immediate recipient of the pending individually addressed data, command frame.	9

10

The RA field of a CTS frame is set to the 64-bit extended address of the device or coordinator that sent the	11 immediately previous RTS frame.	12

13

5.2.2.5.5 TA fie ld	14

15

The TA field of a RTS frame is the 64-bit extended address of the device or coordinator that sends this con-	16 trol frame.	17

18

5.2.2.5.6 Bandwidth Info fie ld	19

20

In a RTS frame, this subfield is used to indicate the recommended bandwidth that will be used to modulate	21 the immediately pending PPDU and shall be set to the one of the nonreserved values in Table zz.	22

23

In a CTS frame, this subfield is used to indicate the bandwidth that is assigned to the receiver of the CTS	24 frame by the coordinator for the modulation of immediately pending PPDU.	25

26

Table 11—Valid value s o f the Bandwidth Info fie ld	27

28

| Bandwidth Info Values hex value | Bandwidth |
| --- | --- |
| 0x01 | 5 MHz |
| 0x02 | 10 MHz |
| 0x03 | 15 MHz |
| 0x04 | 20 MHz |
| 0x05 | 40 MHz |
| 0x06 | 50 MHz |
| 0x07 | 100 MHz |
| 0x08 | 200 MHz |
| 0x09 | 600 MHz |
| 0x0A | 1000 MHz |
| 0x0B-0xFF | Reserved |

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	95

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

| Octets: 2 | Octet: 2 | variable |
| --- | --- | --- |
| Frame control | FCS | Visibility pattern |
| MHR | MFR |  |

26

27	5.2.2.7 Data Null Frame

28

29	Editor note: whether Data Null Frame is a new MAC frame type, a new command frame or a new type of

30	acknowledgment frame should be further decided. See comment 123 of https://mentor.ieee.org/802.15/dcn/

31	17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

32

33	The data null frame is used for acknowledgment and polling when no data or control / management

34	information should be exchanged between two communication nodes. The frame header contains the MAC

35	header fields defined in subclause 6.4.1 and the frame body is left empty.

36

37	5.3 MAC frame formats

38

39	Editor note: Nikola need to instruct how to merge clause 5.2 and clause 5.3. See comment 124 of https://

40	mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

41

42	The format of the MAC frames is specified in this clause. A STA shall be able to properly construct a subset

43	of the frames specified in this clause for transmission and to decode a subset of the frames specified in this

44	clause upon validation following reception. All STAs shall be able to validate every received frame using the

45	frame check sequence (FCS) and to interpret certain fields from the MAC headers of all frames.

46

47	Each frame consists of the following basic components:

48

49	a) A MAC header, which comprises frame control, acknowledgment information, address,  \and sequence

50	control information;

51

52	b) A variable length frame body, which contains information specific to the frame type and subtype;

53

54

96	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

c) A FCS, which contains an IEEE 32-bit CRC.	1

2

5.3.1 Conventions	3

4

The MPDUs or frames in the MAC sublayer are described as a sequence of fields in specific order. Each fig-              5 ure in this clause depicts the fields/subfields as they appear in the MAC frame and in the order in which they              6 are passed to the physical layer convergence procedure (PLCP), from left to right.                                                        7

8

In figures, all bits within fields are numbered, from 0 to k, where the length of the field is k + 1 bits. The	9 octet boundaries within a field can be obtained by taking the bit numbers of the field modulo 8. Octets	10 within numeric fields that are longer than a single octet are depicted in increasing order of significance, from	11 lowest numbered bit to highest numbered bit. The octets in fields longer than a single octet are sent to the	12

PLCP in order from the octet containing the lowest numbered bits to the octet containing the highest num-	13 bered bits.	14

15

Any field containing a CRC is an exception to this convention and is transmitted commencing with the coef-	16 ficient of the highest-order term.	17

18

MAC addresses are assigned as ordered sequences of bits. The Individual/Group bit is always transferred	19 first and is bit 0 of the first octet.	20

21

Values specified in decimal are coded in natural binary unless otherwise stated. The values in Table 1 are in	22 binary, with the bit assignments shown in the table. Values in other tables may be shown in decimal notation.	23

24

Reception, in references to frames or fields within frames (e.g., received Beacon frames or a received Dura-	25 tion/ID field), applies to MPDUs or MAC management protocol data units (MMPDUs) indicated from the	26

PHY layer without error and validated by FCS within the MAC sublayer. Without further qualification,              27 reception by the MAC sublayer implies that the frame contents are valid, and that the protocol version is              28 supported, with no implication regarding frame addressing or regarding whether the frame type or other              29 fields in the MAC header are meaningful to the MAC entity that has received the frame.                                              30

31

5.3.2 General frame format	32

33

The MAC frame format comprises a set of fields that occur in a fixed order in all frames. Figure 5.8.2.1              34 depicts the general MAC frame format. All fields, except for the Frame Body (used for delivering payload              35 information and data), are present in all types of MAC frames. Each field is defined in 5.8.3. The format of              36 each of the individual subtypes of each frame type is defined in 5.8.4.                                                                           37

38

The Frame Body field is of variable size. The maximum frame body size is determined by the maximum	39

MSDU size (2304 octets) plus any overhead from security encapsulation.	40

41

42

43

44

45

46

Figure 61—General MAC frame format.	47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	97

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.3.3 Frame fields

2

3	5.3.3.1 Frame Control field

4

5	The Frame Control field consists of the following subfields: Protocol Version, Type, Subtype, and a

6	Reserved subfield. The format of the Frame Control field is illustrated in Figure 62.

7

8

9

10

11

12

13	Figure 62—Frame control field format.

14

15

16	5.3.3.1.1 P ro to c o l Ve rs io n

17

18	Two-bit field indicating the MAC protocol frame version. The 802.15.7 protocol version is indicated with

19	the value of '00' for this field, while the current 802.15.7r1 protocol version is indicated with the value '10'.

20

21	5.3.3.1.2 Type and S ubtype fie lds

22

23	The Type field is 2 bits in length, and the Subtype field is 4 bits in length. The Type and Subtype fields

24	together identify the function of the frame. There are two frame types in the current MAC layer protocol

25	description: data, and management. Each of the frame types has several defined subtypes. Table 12 defines

26	the valid combinations of type and subtype. (The numeric values in Table 1 are shown in binary.)

27

28

29

30

31	Table 12—Type and S ubtype fie ld value s fo r diffe re nt frame type s

32

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 0000 | Association request |
| 00 | Management | 0001 | Association response |
| 00 | Management | 0010 | Reassociation request |
| 00 | Management | 0011 | Reassociation response |
| 00 | Management | 0100 | Probe request |
| 00 | Management | 0101 | Probe response |
| 00 | Management | 0110 - 0111 | Reserved |
| 00 | Management | 1000 | Beacon |
| 00 | Management | 1001 | ATIM |
| 00 | Management | 1010 | Disassociation |
| 00 | Management | 1011 | Authentication |
| 00 | Management | 1100 | Deauthentication |
| 00 | Management | 1101 | Action |

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

98	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 12—Type and S ubtype fie ld value s fo r diffe re nt frame type s	1

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 1101 - 1111 | Reserved |
| 10 | Data | 0000 | Data |
| 10 | Data | 0001 - 0011 | Reserved |
| 10 | Data | 0100 | Null (no data) |
| 10 | Data | 0101 - 1111 | Reserved |
| 01 | Reserved | 0000 - 1111 | Reserved |
| 11 | Control | 0000 | Waveform Control |
| 11 | Control | 0001 | Advanced Modulation Control |
| 11 | Control | 0010 | CSI Control |
| 11 | Reserved | 0011 - 1111 | Reserved |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

5.3.3.1.3 Re s e rve d fie ld	22

23

The Reserved field constitutes the rest of the Frame Control field and has been introduced for possible	24 extension of the frame control field.	25

26

5.3.3.2 Reserved field	27

28

Two octets reserved for possible protocol frame extension.	29

30

5.3.3.3 Acknowledgment Information (Ack Info) field	31

32

In all existing frame types and subtypes (except for the Beacon frame), the Ack Info field contains the infor-              33 mation necessary to identify the MSDU or MMPDU sequence number and the station which transmitted the              34 acknowledged packet. The information in this field has the format depicted in Figure 63.                                             35

36

37

38

39

40

41

42

Figure 63—Ack Info field format.	43

44

45

5.3.3.3.1 S e que nc e #	46

47

Bits 0 to 7 contain the address of the STA which transmitted the packet which is being acknowledged. In the              48 uplink transmission, these bits identify the STA transmitting the current packet as the acknowledgment can              49 be only for packets transmitted by the AP.                                                                                                                        50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	99

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.3.3.3.2 Re s e rve d

2

3	Bits 8 to 11 are reserved for possible frame extension or functionality extension for packet fragmentation

4	support.

5

6	5.3.3.3.3 S e que nc e Numbe r

7

8	Bits 12 to 23 identify the number of the packet which is being acknowledged.

9

10	5.3.3.3.4 ACK

11

12	Bit 24 is set to '1' when a packet is being acknowledged with the current frame, and set to '0' otherwise.

13

14	Bit 25 is set to '1' when the last Beacon frame reception is being acknowledged, and set to '0' otherwise.

15

16	5.3.3.3.5 Re s e rve d

17

18	Bits 26 to 31 are reserved for possible future extension of the functionality specification.

19

20	5.3.3.4 Receiver Address field

21

22	The Receiver Address field is a 48-bit field indicating the network address of the STA for which the trans-

23	mitted packet is intended.

24

25	5.3.3.5 Transmitter Address field

26

27	The Transmitter Address field is a 48-bit field indicating the network address of the STA transmitting the

28	packet.

29

30	5.3.3.6 Reserved field

31

32	This field contains 48 bits which are reserved for future references in an extension of the MAC protocol.

33

34	5.3.3.7 Sequence Control field

35

36	In data frames, the Sequence Control field contains the information necessary to identify the MSDU or

37	MMPDU sequence number. The information in this field has the format depicted in Figure 64. Bits 4 to 15

38	identify the packet sequence number, while bits 0 to 3 are reserved for possible future extensions of the pro-

39	tocol functionality such as packet fragmentation support. This same information is used by the receiving sta-

40	tion and placed in the Ack Info field of a subsequent frame used for package acknowledgment.

41

42

43

44

45

46

47	Figure 64—Sequence Control field format.

48

49

50

51	5.3.3.8 Reserved field

52

53	This field contains 16 bits which are reserved for future references in an extension of the MAC protocol.

54

100	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.3.3.9 5.8.3.9 Frame Body field	1

2

The Frame Body is a variable length field that contains information specific to individual frame types and              3 subtypes as defined in subclause 5.8.4.  The minimum frame body is 0 octets.  The maximum length frame              4 body is defined by the maximum length (MSDU + ICV + higher layer overhead), where ICV stands for              5 integrity check value.                                                                                                                                                        6

7

5.3.3.10 Frame check sequence (FCS) field	8

9

The FCS field is a 32-bit field containing a 32-bit CRC. The FCS is calculated over all the fields of the MAC	10 header and the Frame Body field. These are referred to as the calculation fields.	11

12

The FCS is calculated using the following standard generator polynomial of degree 32:	13

14

G(x) = x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1	15

16

The FCS is the ones complement of the sum (modulo 2) of the following:	17

18 a) The remainder of xk × (x31 + x30 + x29 + …+ x2 + x + 1) divided (modulo 2) by G(x), where k is the num-              19 ber of bits in the calculation fields, and                                                                                                                            20

21 b) The remainder after multiplication of the contents (treated as a polynomial) of the calculation fields by              22 x32 and then division by G(x).                                                                                                                                           23

24

The FCS field is transmitted commencing with the coefficient of the highest-order term.	25

26

As a typical implementation, at the transmitter, the initial remainder of the division is preset to all ones and              27 is then modified by division of the calculation fields by the generator polynomial G(x). The ones comple-              28 ment of this remainder is transmitted, with the highest-order bit first, as the FCS field.                                                 29

30

At the receiver, the initial remainder is preset to all ones and the serial incoming bits of the calculation fields              31 and FCS, when divided by G(x), results in the absence of transmission errors, in a unique nonzero remainder              32 value. The unique remainder value is the polynomial:                                                                                                     33

34

x31 + x30 + x26 + x25 + x24 + x18 + x15 + x14 + x12 + x11 + x10 + x8 + x6 + x5 + x4 + x3 + x + 1	35

36

5.3.4 Specific Frame Formats	37

38

5.3.4.1 Management Frames Format	39

40

5.3.4.1.1 Manag e me nt Frame s Info rmatio n Co mpo ne nts	41

42

5.8.4.1.1.1 Beacon Frame Format	43

44

The Beacon frame body contains the following information in the presented order:	45

46

5.8.4.1.1.2 IBSS ATIM Frame Format	47

48

The body of an IBSS ATIM management frame is null.	49

50

5.8.4.1.1.3 Disassociation Frame Format	51

52

The Disassociation frame body contains the following information in the presented order:	53

54

Copyright © 20xx IEEE. All rights reserved.	101

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 13—Beac o n Frame Bo dy

| Order | Information |
| --- | --- |
| 1 | Timestamp |
| 2 | Beacon Interval |
| 3 | Capability |
| 4 | Service Set Identifier (SSID) |
| 5 | Supported Rates |
| 6 | DS Parameter Set |
| 7 | Traffic Indication Map |
| 8 | Country |
| 9 | Extended Support Rates |
| 10 | Vendor Specific Information |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24	Table 14—Dis as s o c iatio n Frame Bo dy

25

| Order | Information |
| --- | --- |
| 1 | Reason Code |
| 2 | Vendor Specific Information |

26

27

28

29

30

31

32

33	5.8.4.1.1.4 Association Request Frame Format

34

35	The Association Request frame body contains the following information in the presented order:

36

37

38

39

40	Table 15—As s o c iatio n Re que s t Frame Bo dy

41

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Listen Interval |
| 3 | SSID |
| 4 | Supported Rates |
| 5 | Extended Supported Rates |
| 6 | Vendor Specific Information |

42

43

44

45

46

47

48

49

50

51

52

53

54

102	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.1.1.5 Association Response Frame Format	1

2

The Association Response frame body contains the following information in the presented order:	3

4

5

6

7

Table 16—As s oc iation Re s pons e Frame Bo dy	8

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Status Code |
| 3 | AID |
| 4 | Supported Rates |
| 5 | Extended Supported Rates |
| 6 | Vendor Specific Informatino |

9

10

11

12

13

14

15

16

17

18

19

20

21

22

5.8.4.1.1.6 Reassociation Request Frame Format	23

24

The Reassociation Request frame body contains the following information in the presented order:	25

26

27

28

Table 17—Re as s o c iatio n Re que s t Frame Bo dy	29

30

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Listen Interval |
| 3 | Current AP address |
| 4 | SSID |
| 5 | Supported Rates |
| 6 | Extended Supported Rates |
| 7 | Vendor Specific Information |

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

5.8.4.1.1.7 Reassociation Response Frame Format	46

47

The Reassociation Response frame body contains the following information in the presented order:	48

49

5.8.4.1.1.8 Probe Request Frame Format	50

51

The Probe Request frame body contains the following information in the presented order:	52

53

54

Copyright © 20xx IEEE. All rights reserved.	103

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 18—Reas s oc iation Re s pons e Frame Bo dy

| Order | Information |
| --- | --- |
| 1 | Capability |
| 2 | Status Code |
| 3 | AID |
| 4 | Supported Rates |
| 5 | Extended Supported Rates |
| 6 | Vendor Specific Information |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16	Table 19—Pro be Re que s t Frame Bo dy

| Order | Information |
| --- | --- |
| 1 | SSID |
| 2 | Supported Rates |
| 3 | Request Information |
| 4 | Extended Supported Rates |
| 5 | Vendor Specific Information |

17

18

19

20

21

22

23

24

25

26

27

28

29	5.8.4.1.1.9 Probe Response Frame Format

30

31	The Probe Response frame body contains the following information in the presented order:

32

33

34

35

36	Table 20—Pro be Re s po ns e Frame Bo dy

| Order | Information |
| --- | --- |
| 1 | Timestamp |
| 2 | Beacon Interval |
| 3 | Capability |
| 4 | Service Set Identifier (SSID) |
| 5 | Supported Rates |
| 6 | Country |
| 7 | Extended Supported Rates |
| 8 | Vendor Specific Information |
| 9 | Requested information elements |

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

104	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.1.1.10 Authentication Frame Format	1

2

The Probe Request frame body contains the following information in the presented order:	3

4

5

6

7

Table 21—Authe ntic atio n Fram e Bo dy	8

| Order | Information |
| --- | --- |
| 1 | Authentication Algorithm Number |
| 2 | Authentication Transaction Sequence Number |
| 3 | Status Code |
| 4 | Challenge Text |
| 5 | Vendor Specific |

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

Table 22—Challe ng e te xt info rmatio n	24

| Authentication Algorithm | Authentication Transaction Sequence No. | Status Code | Challenge Text |
| --- | --- | --- | --- |
| Open System | 1 | Reserved | Not present |
| Open System | 2 | Status | Not present |
| Shared Key | 3 | Reserved | Not present |
| Shared Key | 4 | Status | Present |
| Shared Key | 5 | Reserved | Present |
| Shared Key | 6 | Status | Not present |

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

5.8.4.1.1.11 Deauthentication Frame Format	40

41

The Deauthentication frame body contains the following information in the presented order:	42

43

44

45

Table 23—De authe ntic atio n Frame Bo dy	46

47

| Order | Information |
| --- | --- |
| 1 | Reason Code |
| 2 | Vendor Specific Information |

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	105

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.8.4.1.1.12 Action Frame Format

2

3	The Action frame body contains the following information in the presented order:

4

5

6

7

8	Table 24—Ac tio n Frame Bo dy

| Order | Information |
| --- | --- |
| 1 | Action |
| 2 | Vendor Specific Information |

9

10

11

12

13

14

15

16

17	5.3.4.1.2 Manag e me nt Frame Bo dy Fie lds

18

19	5.8.4.1.2.1 Authentication Algorithm Number Field

20

21	The Authentication algorithm number is a 16-bit field, which indicates a single algorithm used for authenti-

22	cation (see Figure 65). Two values have been defined:

23

24	'0' Open System

25

26	'1'  Shared Key

27

28	'2-65536'  Reserved for future use

29

30

31

32

33

34

35	Figure 65—Authentication algorithm number field.

36

37

38

39	5.8.4.1.2.2 Authentication Transaction Sequence Number Field

40

41	The authentication transaction sequence number is a 16-bit field, which indicates the current step in a multi-

42	step authentication procedure (see Figure 66).

43

44

45

46

47

48

49	Figure 66—Authentication transaction sequence number field.

50

51

52	5.8.4.1.2.3 Beacon Interval Field

53

54

106	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

Figure 67—Beacon interval field.	7

8

9

The beacon interval field is a 16-bit field, which indicates the interval in time units between two consecutive	10 beacon frames (see Figure 67).	11

12

5.8.4.1.2.4 Capability Information Field	13

14

The beacon interval field is a 16-bit field, which indicates the AP capabilities with regards to optional ser-	15 vices (see Figure 68). The field is left unspecified and reserved for future use.	16

17

18

19

20

21                                                                        22

23

Figure 68—Capability Information field.	24

25

26

5.8.4.1.2.5 Current AP Address Field	27

28

The current AP address field is a 48-bit field, which indicates the address of the AP with which the STA is	29 currently associated. The field is illustrated in Figure 69.	30

31

32

33

34

35

36

37

Figure 69—Current AP address field.	38

39

40

5.8.4.1.2.6 Listen Interval Field	41

42

The listen interval field is a 16-bit field, which indicates to the AP how often the STA wakes up to listen for	43

Beacon management frames. The field is illustrated in Figure 70. The value of the field is expressed in units	44 of Beacon Intervals.	45

46

47

48

49                                                                                         50

51

Figure 70—Listen interval field.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	107

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.8.4.1.2.7 Reason Code Field

2

3	The Reason Code is a 16-bit field, which indicates the reason a notification management frame of type Dis-

4	association or Deauthentication has been generated. The field is illustrated in Figure 71. The values of the

5	field are specified in Table 25.

6

7

8

9

10

11

12	Figure 71—Reason Code field.

13

14

15

16

17

18	Table 25—Reas o n c o de s

19

| Reason Code | Meaning |
| --- | --- |
| 0 | Reserved |
| 1 | Unspecified reason |
| 2 | Previous authentication no longer valid |
| 3 | Deauthentication because sending STA is leaving (or has left) IBSS or ESS |
| 4 | Disassociated due to inactivity |
| 5 | Disassociated because AP is unable to handle all currently associated STAs |
| 6 | Class 2 frame received from nonauthenticated STA |
| 7 | Class 3 frame received from nonassociated STA |
| 8 | Disassociated because sending STA is leaving (or has left) BSS |
| 9 | STA requesting (re)association is not authenticated with responding STA |
| 10 - 12 | Reserved |
| 13 | Invalid information element |
| 14 | Message integrity code (MIC) failure |
| 15 | 4-Way Handshake timeout |
| 16 | Group Key Handshake timeout |
| 17 | Information element in 4-Way Handshake different from (Re)Association Request / Probe Response / Beacon frame |
| 18 | Invalid group cipher |
| 19 | Invalid pairwise cipher |
| 20 | Invalid AKMP |
| 21 | Unsupported RSN information element version |
| 22 | Invalid RSN information element capabilities |

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

108	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.1.2.8 Association ID (AID) Field

Table 25—Reas o n c o de s	1

| Reason Code | Meaning |
| --- | --- |
| 23 | IEEE 802.1X authentication failed |
| 24 | Cipher suite rejected because of the security policy |
| 25 - 35 | Reserved |
| 36 | Requested from peer STA as the STA is leaving the BSS (or resetting) |
| 37 | Requested from peer STA as it does not want to use the mechanism |
| 38 | Requested from peer STA as the STA using wrong mechanism |
| 39 | Requested from peer STA due to timeou |
| 40 - 44 | Reserved |
| 45 | Peer STA does not support the requested cipher suite |
| 46 - 65535 | Reserved |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

The AID field is a 16-bit field, which is a station ID assigned by an AP during association. The field is illus-              24 trated in Figure 72. The value of the field is in the range 1 - 2007 and is placed in the 14 LSBs of the AID              25 field, with the two MSBs of the field set to '11'.                                                                                                               26

27

28

29

30                                                                                     31

32

Figure 72—Association ID field.	33

34

35

5.8.4.1.2.9 Status Code Field	36

37

The Status Code field is a 16-bit field used in response to a management frame in order to indicate the suc-              38 cess or the failure of the operation. The field is illustrated in Figure 73. The possible values of this field are              39 presented in Table 26.                                                                                                                                                       40

41

42

43

44

45

46

Figure 73—Status Code field.	47

48

49

50

51

5.8.4.1.2.10 Timestamp Field	52

53

54

Copyright © 20xx IEEE. All rights reserved.	109

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 26—S tatus c o de s

| Reason Code | Meaning |
| --- | --- |
| 0 | Successful |
| 1 | Unspecified failure |
| 2 - 9 | Reserved |
| 10 | Cannot support all requested capabilities in the Capability Information field |
| 11 | Reassociation denied due to inability to confirm that association exists |
| 12 | Association denied due to reasons outside the scope of this standard |
| 13 | Responding STA does not support the specified authentication algorithm |
| 14 | Received an unexpected authentication transaction sequence number |
| 15 | Authentication rejected because of challenge failure |
| 16 | Authentication rejected due to timeout waiting for next frame in sequence |
| 17 | Association denied because AP is unable to handle additional associated STAs |
| 18 | Association denied due to requesting STA not supporting all basics data rates |
| 19 - 33 | Reserved |
| 34 | Association denied due to excessive frame loss rate / poor channel conditions |
| 35 - 36 | Reserved |
| 37 | The request has been declined |
| 38 | The request has not been successful due to invalid parameter values |
| 39 | Reserved |
| 40 | Invalid information element (content is invalid) |
| 41 | Invalid group cipher |
| 42 | Invalid pairwise cipher |
| 43 | Invalid AKMP |
| 44 - 45 | Reserved |
| 46 | Cipher suite rejected because of security policy |
| 47 - 48 | Reserved |
| 49 | The destination STA is not within this BSS |
| 50 | Reserved |
| 51 | Association denied because the Listen Interval is too large |
| 52 - 65535 | Reserved |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50	The Timestamp field is a 64-bit field representing the value of a timing synchronization function. The field

51	is illustrated in Figure 74.

52

53

54

110	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3                                                                                                 4

5

Figure 74—Timestamp field.	6

7

8

5.8.4.1.3. Management Frame Information Elements	9

10

The information elements are defined to have a common general format which consists of 1 octet specifying	11

the element ID, 1 octet specifying the length of the element field in octets, and a variable-length element	12

field with  the  information contained in  the  respective element. The  overall structure is  presented in	13

Figure 75. The set of valid information elements is defined in Table 27.	14

15

16

17

18

19

20

21

Figure 75—Information element format.	22

23

24

25

26

27

Table 27—Valid s e t of info rmatio n e lements	28

29

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| SSID | 0 | 2 to 34 |
| Supported rates | 1 | 3 to 10 |
| Reserved | 2 - 6 |  |
| Country | 7 | 8 to 256 |
| Reserved | 8 - 9 |  |
| Request | 10 | 2 - 256 |
| BSS Load | 11 | 7 |
| Reserved | 12 - 15 | Reserved |
| Challenge Text | 16 | 3 to 255 |
| Reserved | 17 - 43 |  |
| TCLAS Processing | 44 | 3 |
| Reserved | 45 - 49 |  |
| Extended Supported Rates | 50 | 3 to 257 |
| Reserved | 51 - 126 |  |

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	111

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 27—Valid s e t of info rmatio n e lements

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Extended Capabilities | 127 | 2 to 257 |
| Reserved | 128 - 220 |  |
| Vendor Specific | 221 | 3 to 257 |
| Reserved | 222 - 255 |  |

2

3

4

5

6

7

8

9

10

11

12

13	5.3.4.2 Data Frames Format

14

15	5.3.4.3 Control Frames Format

16

17	5.3.4.3.1 Advanc e d Mo dulatio n Co ntro l Frame

18

19	A control frame indicating the advanced modulation capabilities of a communication node. The frame is

20	structured as described in Figure 76.

21

22

B0 	B 1      B4 B5 	B6 	B9  B10 	B11 	B14  B15 	B15+IDFT_SIZE

| Adaptive Loading | eU | RPO | Relaying | MIMO | # of MIMO Channels | SC-FDMA |
| --- | --- | --- | --- | --- | --- | --- |

23

24

25

26

27	Figure 76—Format of the advanced modulation control frame.

28

29

30	5.8.4.3.1.1 Adaptive Loading

31

32	A single bit indicating whether the communication node transmitting the advanced modulation control

33	frame supports adaptive bit and energy loading:

34

35	1  Adaptive bit and energy loading is supported.

36

37	0  Adaptive bit and energy loading is not supported.

38

39	5.8.4.3.1.2 eU

40

41	Four bits indicating if the node supports eU-OFDM. The bit value at a given position out of the four posi-

42	tions indicates whether eU-OFDM implementation with the same number of streams as the bit position is

43	supported. Positions are counted from left to right. For example:

44

45	1000  eU-OFDM with one stream only is supported.

46

47	1100  eU-OFDM with one and two streams only is supported

48

49	1010  eU-OFDM with one and three streams only is supported

50

51	1111  eU-OFDM with all possible streams is supported

52

53	0000  eU-OFDM is not supported

54

112	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.8.4.3.1.3 RPO	1

2

A single bit indicating whether the communication node transmitting the advanced modulation control rame	3 supports RPO-OFDM:	4

5

'1'  RPO-OFDM is supported.	6

7

'0'  RPO-OFDM is not supported.	8

9

5.8.4.3.1.5 Relaying	10

11

Four bits indicating types of relaying operations the communication node transmitting the advanced modula-	12 tion control frame supports. The first bit (B7) indicates whether relaying in FD is supported:	13

14

'1'  Relaying in FD is supported.	15

16

'0'  Relaying in FD is not supported.	17

18

The second bit (B8) indicates whether relaying in HD is supported:	19

20

'1'  Relaying in HD is supported.	21

22

'0'  Relaying in HD is not supported.	23

24

The third bit (B9) indicates whether AF relaying is supported:	25

26

'1'  AF relaying is supported.	27

28

'0'  AF relaying is not supported.	29

30

The fourth bit (B10) indicates whether DF relaying is supported:	31

32

'1'  DF relaying is supported.	33

34

'0'  DF relaying is not supported.	35

36

5.8.4.3.1.6 MIMO	37

38

A single bit indicating whether the communication node transmitting the advanced modulation control	39 frame supports MIMO communication:	40

41

'1'  MIMO is supported.	42

43

'0'  MIMO is not supported.	44

45

5.8.4.3.1.7 # of MIMO Channels	46

47

Four bits indicating the maximum number of MIMO communication channels which the communication              48 node transmitting the advanced modulation control frame supports. A value of '0000' corresponds to 1 chan-              49 nel, and a value of '1111' corresponds to 16 channels.                                                                                                      50

51

5.8.4.3.1.7 SC-FDMA	52

53

54

Copyright © 20xx IEEE. All rights reserved.	113

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	An IDFT_SIZE of bits, where IDFT_SIZE is the size of the IDFT operation used for OFDM time-domain

2	signal generation. The bit at position k indicates whether a DFT transform of size k is supported for SC-

3	FDMA pre-coding. For example, if IDFT_SIZE = 16, then:

4

5	'10000000000000000' - A DFT of size 1 is supported.

6

7	'00100000000000000' - A DFT of size 3 is supported.

8

9	'10100100000000000' - A DFT of size 1, 3 and 6 is supported.

10

11	'00000000000000000' - No DFT size is supported, so SC-FDMA is not supported.

12

13	5.8.4.3.2 CSI Control Frame

14

15	The CSI control frame contains the standard MAC header information defined in subclause 5.8.2. The frame

16	body described in this clause is the only specific field. It is structured as described in Fig. 5.8.4.3.2.1.

17

18

19

20

21

22

23	Figure 77—Format of the CSI control frame.

24

25

26	5.8.4.3.2.1 MIMO Mode

27

28	A one bit value indicating whether the current CSI frame contains information relevant for the SISO or

29	MIMO mode of operation:

30

31	1  MIMO mode of operation.

32

33	0  SISO mode of operation.

34

35	5.8.4.3.2.2 Adaptive Loading

36

37	A one-bit field which indicates whether the CSI in the current frame refers to the adaptive bit loading mode.

38

39	1  The CSI is relevant for the adaptive bit loading scheme

40

41	0  The CSI is relevant for the fixed bit loading scheme

42

43	5.8.4.3.2.3 Frame Info

44

45	A one-bit field indicating the type of information carried by the current frame:

46

47	'0'  The frame carries CQIs

48

49	'1'  The frame carrier bit loading (RATE) information

50

51	5.8.4.3.2.4 Number of MIMO Channels TX

52

53

54

114	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The number of MIMO channels of the transmitting node in the MIMO configuration. Values range between              1

0 and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this              2 field contains the number of the MIMO channels for the node receiving the frame. If the frame carriers bit              3 loading (RATE) information, this field contains the number of the MIMO channels for the node transmitting              4 the frame. This information is expected to have been exchanged via the advanced modulation control frame              5 prior to any node issuing a CSI control frame.                                                                                                                 6

7

5.8.4.3.2.5 Number of MIMO Channels RX                                                                                                                    8

9

The number of MIMO channels of the receiving node in the MIMO configuration. Values range between 0              10 and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this              11 field contains the number of the MIMO channels for the node transmitting the frame. If the frame carriers bit              12 loading (RATE) information, the this field contains the number of the MIMO channels for the node receiv-              13 ing the frame. This information is expected to have been exchanged via the advanced modulation control              14 frame prior to any node issuing a CSI control frame.                                                                                                       15

16

5.8.4.3.2.6 Channel Number                                                                                                                                             17

18

The MIMO channel to which the subsequent RATE/CSI information refers. In a SISO mode, this field is set              19 to '0000'. If the Adaptive Loading field is set to '1', then this field occurs only once and is followed by the              20

CQI / RATE information for every subcarrier of the respective MIMO channel. If the Adaptive Loading              21 field is set to '0', then this field occurs as many times as there are MIMO channels for which relevant infor-              22 mation needs to be exchanged followed by the relevant information as CQI / RATE values. If the Frame Info              23

is set to '0', then this field occurs Number of MIMO Channels TX . Number of MIMO Channels RX times              24

where the 6-bit CQI following each occurrence corresponds to the average SNR achieved for the respective              25 combination between the corresponding MIMO transmitter and MIMO receiver. The order in which the              26 information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2  RX1 | TX2               27

 RX2 | … |TX2  RXN_MIMO| … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | … |TXM_MIMO               28

RXN_MIMO |. If the Frame Info is set to '1', then this field occurs #_of_MIMO_Channels_TX times where              29

the information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO|. In each case, the occurrence of              30

the Channel # is followed by the respective 4-bit RATE value for that TX channel.                                                       31

32

5.8.4.3.2.6 RATE / CSI                                                                                                                                                     33

34

The RATE / CSI values for the MIMO channel specified in Channel #. If the Adaptive Loading field is set to              35

'1', then this field occurs once for every subcarrier modulated with information on the respective MIMO              36 channel. If the Adaptive Loading field is set to '0', then this field occurs as many times as there are MIMO              37 channels for which relevant information needs to be exchanged and is preceded by the relevant MIMO              38

Channel # field. If the Frame Info is set to '0', then this field occurs #_of_MIMO_Channels_TX ? x_of_MI-              39

MO_Channels_RX times where a 6-bit CQI value is specified corresponding to the average SNR achieved              40 for the respective combination between the corresponding MIMO transmitter and MIMO receiver. The order              41 in which the information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2               42

RX1 | TX2  RX2 | … |TX2  RXN_MIMO | … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | …              43

|TXM_MIMO  RXN_MIMO |. If the Frame Info is set to '1', then this field occurs # of MIMO Channels TX              44

times where the 4-bit RATE information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO |. In              45

each case, the occurrence of the RATE field is preceded by the respective Channel # value for that TX chan-              46

nel.                                                                                                                                                                                     47

48

5.8.4.3.2 Dimming Control Frame                                                                                                                                    49

50

The frame contains control information which needs to be exchanged for the purposes of realizing dimming              51 techniques such as RPO-OFDM and/or analog dimming.                                                                                                52

53

54

Copyright © 20xx IEEE. All rights reserved.	115

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.4 MAC command frames

2

3	The command frames defined by the MAC sublayer are listed in Table 28. A coordinator shall be capable of

4	transmitting and receiving all command frame types, with the exception of the GTS request command, while

5	the requirements for a device are indicated by an “X” in Table 28. A P2P device functioning as a coordinator

6	shall be capable of transmitting and receiving all supported command frames in a device. MAC commands

7	shall only be transmitted in the CAP for beacon-enabled OWPANs or at any time for nonbeacon-enabled

8	OWPANs.

9

10	How the MLME shall construct the individual commands for transmission is detailed in 5.4.1 through

11	5.4.21. MAC command reception shall abide by the procedure described in 5.1.9.2.

12

13

14	Table 28—Co mmand frame s

| Command frame identifier | Command name | Device |  | P2P coordinator |  | Subclause |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Tx | Rx | Tx | Rx |  |
| 0x01 | Association request | X |  | X | X | 5.4.1 |
| 0x02 | Association response |  | X | X | X | 5.4.2 |
| 0x03 | Re-association request | X |  | X | X | 5.4.3 |
| 0x04 | Disassociation notification | X | X | X | X | 5.4.5 |
| 0x05 | Data request | X |  | X | X | 5.4.8 |
| 0x06 | OWPAN ID conflict notification | X |  | X | X | 5.4.11 |
| 0x07 | Beacon request |  |  |  |  | 5.4.13 |
| 0x08 | Coordinator realignment |  | X | X | X | 5.4.14 |
| 0x09 | GTS request |  |  |  |  | 5.4.15 |
| 0x0a |  |  |  |  |  |  |
| 0x0b | Fast link recovery |  |  |  |  | 5.4.16 |
| 0x0c |  |  |  |  |  |  |
| 0x0d | GTS Response |  |  |  |  | 5.4.18 |
| 0x0e | Clock rate change notification |  | X | X | X | 5.4.19 |
| 0x0f |  |  |  |  |  |  |
| 0x10 |  |  |  |  |  |  |
| 0x11 |  |  |  |  |  |  |
| 0x12 |  |  |  |  |  |  |
| 0x13 | Information element | X | X | X | X | 5.4.21 |
| 0x14-0xff | Reserved |  |  |  |  | — |

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

116	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.1 Association request command	1

2

The association request command allows a device to request association with a OWPAN through the	3 coordinator. This command shall only be sent by an unassociated device that wishes to associate with a	4

OWPAN. A device shall only associate with a OWPAN through the coordinator as determined through the	5 scan procedure.	6

7

All devices shall be capable of transmitting this command, although a device is not required to be capable of	8 receiving it.	9

10

The association request command shall be formatted as illustrated in Figure 78.	11

12

| Octets: (see 5.2.2.4) | 1 | 1 (see 5.4.23.1) |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | Capability Information |

13

14

15

16

17

18

19

20

Figure 78—Association request command format	21

22

23

5.4.1.1 MHR fields	24

25

The Source Addressing Mode subfield of the frame control field shall be set to three (64-bit extended              26 addressing). The Destination Addressing Mode subfield shall be set to the same mode as indicated in the              27 beacon frame to which the association request command refers.                                                                                     28

29

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and	30 the Acknowledgment Request subfield shall be set to one.	31

32

The Destination OWPAN Identifier field shall contain the identifier of the OWPAN to which to associate.	33

The Destination Address field shall contain the address from the beacon frame that was transmitted by the              34 coordinator to which the association request command is being sent. The Source OWPAN Identifier field              35 shall contain the broadcast OWPAN identifier (i.e., 0xffff). The Source Address field shall contain the value              36 of aExtendedAddress.                                                                                                                                                       37

38

5.4.2 Association response command	39

40

The association response command allows the coordinator or a coordinator to communicate the results of an	41 association attempt back to the device requesting association.	42

43

This command shall only be sent by the coordinator or a coordinator to a device that is currently trying to	44 associate.	45

46

All devices shall be capable of receiving this command, although a device is not required to be capable of	47 transmitting it.	48

49

The association response command shall be formatted as illustrated in Figure 79.	50

51

The capability-negotiation response is the same as that of the color-stabilization scheme in Table 39.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	117

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

| Octets: (see 5.2.2.4) | 1 | 2 | 1 | 1 |
| --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | Short Address | Association Status | Capability- negotiation response |

1

2

3

4

5

6

7

8

9

10	5.4.2.1 MHR fields

11

Figure 79—Association response command format

12	The Destination Addressing Mode and Source Addressing Mode subfields of the frame control field shall

13	each be set to three (i.e., 64-bit extended addressing).

14

15	The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and

16	the Acknowledgment Request subfield shall be set to one.

17

18	The Destination OWPAN Identifier field shall contain the value of macOWPANId, while the Source

19	OWPAN Identifier field shall be omitted. The Destination Address field shall contain the extended address

20	of the device requesting association. The Source Address field shall contain the value of aExtendedAddress.

21

22	5.4.2.2 Short Address field

23

24	If the coordinator was not able to associate this device to its OWPAN, the Short Address field shall be set to

25	0xffff, and the Association Status field shall contain the reason for the failure. If the coordinator was able to

26	associate the device to its OWPAN, this field shall contain the short address that the device may use in its

27	communications on the OWPAN until it is disassociated.

28

29	A Short Address field value equal to 0xfffe shall indicate that the device has been successfully associated

30	with a OWPAN, but has not been allocated a short address. In this case, the device shall communicate on the

31	OWPAN using only its 64-bit extended address.

32

33	5.4.2.3 Association Status field

34

35	The Association Status field shall contain one of the nonreserved values listed in Table 29.

36

37

38	Table 29—Valid values o f the As s o c iatio n S tatus fie ld

| Association status | Description |
| --- | --- |
| 0x00 | Association successful. |
| 0x01 | OWPAN at capacity. |
| 0x02 | OWPAN access denied. |
| 0x03–0x7f | Reserved. |
| 0x80–0xff | Reserved for MAC primitive enumeration values. |

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

118	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.2.4 Capability negotiation response field	1

2

The  capability  negotiation  response  field  describes  if  and  where  (device  and/or  coordinator)  color	3 stabilization is performed. All allowed settings are shown in Table 30.	4

5

6

Table 30—Capability ne g o tiatio n re s po ns e fie ld	7

| Bits: 0–1 | Bits: 2–7 |
| --- | --- |
| 00: No color stabilization 01: Color-stabilization information to be sent from device to coordinator upon reception of CVD frames 10: Color-stabilization information to be sent from coordinator to device upon reception of CVD frames 11: Color-stabilization information to be sent from device to coordinator and from coordinator to device when either receives CVD frames | Reserved |

8

9

10

11

12

13

14

15

16

17

18

19

20

21

5.4.3 Re-association request command	22

23

The re-association request command allows a device to request re-association with a target OWPAN through              24 the coordinator. This command shall only be sent by a device that is already associate with an original coor-              25 dinator and that wishes to associate with another OWPAN. A device shall only re-associate with an OWPAN              26 through the coordinator as determined through neighboring OWPAN measurements. All devices shall be              27 capable of transmitting this command, although a device is not required to be capable of receiving it.                         28

29

The re-association request command shall be formatted as illustrated in Figure 80.	30

31

32

33

34

| Octets: (see 5.2.2.4) | 1 | 1 (see 5.4.23.1) | Variable |
| --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | Capability Infor- mation | Originating OWPAN Information |

35

36

37

38

39

40

41

Figure 80—Re-association request command format	42

5.4.3.1 MHR field	43

44

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and              45 the Acknowledgment Request subfield shall be set to one. The Destination OWPAN Identifier field shall              46 contain the identifier of the OWPAN to which to associate.                                                                                            47

48

The Destination Address field shall contain the address from the beacon frame that was transmitted by the              49 coordinator to which the association request command is being sent. The Source OWPAN Identifier field              50 shall contain the value of macOWPANId. The Source Address field shall contain the value of aExtendedAd-              51 dress.                                                                                                                                                                                 52

53

54

Copyright © 20xx IEEE. All rights reserved.	119

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.4.3.2 Originating OWPAN information

2

3	The Originating OWPAN information contains the information of the OWPAN with which the device is cur-

4	rently associate, see Table 62.

5

6	5.4.4 Disassociation notification command

7

8	The OWC coordinator or an associated device may send the disassociate notification command.

9

10	All devices shall implement this command.

11

12	The disassociation notification command shall be formatted as illustrated in Figure 81.

13

| Octets: (see 5.2.2.4) | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | Disassociation reason |

14

15

16

17

18

19

20	Figure 81—Disassociation notification command format

21

22

23	5.4.4.1 MHR fields

24

25	The Destination Addressing Mode subfield of the frame control field shall be set according to the addressing

26	mode specified by the corresponding primitive. The Source Addressing Mode subfield shall be set to three

27	(i.e., 64-bit extended addressing).

28

29	The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and

30	the Acknowledgment Request subfield shall be set to one.

31

32	The Destination OWPAN Identifier field shall contain the value of macOWPANId, while the Source

33	OWPAN Identifier field shall be omitted. If the coordinator wants an associated device to leave the

34	OWPAN, then the Destination Address field shall contain the address of the device being removed from the

35	OWPAN. If an associated device wants to leave the OWPAN, then the Destination Address field shall

36	contain the value of either macCoordShortAddress, if the Destination Addressing Mode subfield is equal to

37	two, or macCoordExtendedAddress, if the Destination Addressing Mode subfield is equal to three. The

38	Source Address field shall contain the value of aExtendedAddress.

39

40	5.4.4.2 Disassociation Reason field

41

42	The Disassociation Reason field shall contain one of the nonreserved values listed in Table 31.

43

44	5.4.5 Data request command

45

46	The data request command is sent by a device to request data from the coordinator.

47

48	There are three cases for which this command is sent. On a beacon-enabled OWPAN, this command shall be

49	sent by a device when macAutoRequest is equal to TRUE and a beacon frame indicating that data are

50	pending for that device is received from its coordinator. The coordinator indicates pending data in its beacon

51	frame by adding the address of the recipient of the data to the Address List field. This command shall also be

52	sent when instructed to do so by the next higher layer on reception of the MLME-POLL.request primitive. In

53	addition, a device may send this command to the coordinator macResponseWaitTime optical clocks after the

54	acknowledgment to an association request command.

120	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 31—Valid dis as s o c iatio n re as o n c o de s	1

| Disassociate reason | Description |
| --- | --- |
| 0x00 | Reserved. |
| 0x01 | The coordinator wishes the device to leave the OWPAN. |
| 0x02 | The device wishes to leave the OWPAN. |
| 0x03 | Device cannot support communications for the requested dimming value. |
| 0x04f–0x7f | Reserved. |
| 0x80–0xff | Reserved for MAC primitive enumeration values. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

All devices shall be capable of transmitting this command, although a device is not required to be capable of	16 receiving it.	17

18

The data request command shall be formatted as illustrated in Figure 82.	19

20

21

| Octets: (see 5.2.2.4) | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) |

22

23

24

25

26

Figure 82—Data request command format	27

28

29

If the data request command is being sent in response to the receipt of a beacon frame indicating that data are              30 pending for that device, the Destination Addressing Mode subfield of the frame control field may be set to              31 zero (i.e., destination addressing information not present) if the beacon frame indicated in its Superframe              32

Specification field (see 5.2.2.1.2) that it originated from the coordinator (see 5.2.1.1.6) or set otherwise              33 according to the coordinator to which the data request command is directed. If the destination addressing              34 information is to be included, the Destination Addressing Mode subfield shall be set according to the value              35 of macCoordShortAddress. If macCoordShortAddress is equal to 0xfffe, extended addressing shall be used:              36 the Destination Addressing Mode subfield shall be set to three, and the Destination Address field shall              37 contain the value of macCoordExtendedAddress. Otherwise, short addressing shall be used: the Destination              38

Addressing Mode subfield shall be set to two, and the Destination Address field shall contain the value of	39

macCoordShortAddress.	40

41

If the data request command is being sent in response to the receipt of a beacon frame indicating that data are              42 pending for that device, the Source Addressing Mode subfield shall be set according to the addressing mode              43 used for the pending address. If the Source Addressing Mode subfield is set to two, short addressing shall be              44 used: the Source Address field shall contain the value of macShortAddress. Otherwise, extended addressing              45 shall be used: the Source Addressing Mode subfield shall be set to three, and the Source Address field shall              46 contain the value of aExtendedAddress.                                                                                                                           47

48

If the data request command is triggered by the reception of an MLME-POLL.request primitive from the              49 next higher layer, then the destination addressing information shall be the same as that contained in the              50 primitive. The Source Addressing Mode subfield shall be set according to the value of macShortAddress. If              51 macShortAddress is less than 0xfffe, short addressing shall be used. Extended addressing shall be used              52 otherwise.                                                                                                                                                                          53

54

Copyright © 20xx IEEE. All rights reserved.	121

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	If the data request command is being sent following the acknowledgment to an association request command

2	frame, the Destination Addressing Mode subfield of the frame control field shall be set according to the

3	coordinator to which the data request command is directed. If macCoordShortAddress is equal to 0xfffe,

4	extended addressing shall be used. Short addressing shall be used otherwise. The Source Addressing Mode

5	subfield shall be set to use extended addressing.

6

7	If the Destination Addressing Mode subfield is set to zero (i.e., destination addressing information not

8	present), the source OWPAN identifier shall contain the value of macOWPANId. Otherwise, the Destination

9	OWPAN Identifier field shall contain the value of macOWPANId, while the Source OWPAN Identifier field

10	shall be omitted.

11

12	The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and

13	the Acknowledgment Request subfield shall be set to one.

14

15	5.4.6 Scan-over-backhaul request command

16

17	The scan-over-backhaul request command shall be send by a prospective coordinator to other coordinators.

18	The command frame requests the OWPAN descriptor from the recipient through the backhaul link when

19	inter-coordinator communication over backhaul is feasible and scan over backhaul is performed (see

20	5.1.3.1.3).

21

22	The scan-over-backhaul request command shall be formatted as illustrated in Figure 83.

23

| Octets: | 1 |
| --- | --- |
| MHR fields | Command Frame identifier |

24

25

26

27

28

29	Figure 83—Scan-over-backhaul request command

30

31

32	5.4.7 Scan-over-backhaul confirmation command

33

34	The scan-over-backhaul confirmation command shall be sent by a coordinator as a response to a previous

35	received scan-over-backhaul request command through the backhaul.

36

37	The scan-over-backhaul confirmation command shall be formatted as illustrated in Figure 84.

38

39

| Octets: | 1 | Variable |
| --- | --- | --- |
| MHR fields | Command Frame identifier | OWPANDescriptor |

40

41

42

43

44	Figure 84—Scan-over-backhaul confirmation command

45

46

47

48	5.4.7.1 OWPAN Descriptor

49

50	The OWPANDescriptor contains the information of the OWPAN whose coordinator generates the scan-

51	over-backhaul confirmation command, see Table 62. The LinkQuality field is set to 0xff, the TimeStamp

52	field is set to 0xffffff.

53

54

122	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.8 OWPAN ID conflict notification command	1

2

The OWPAN ID conflict notification command is sent by a device to the coordinator to report an OWPAN	3 identifier conflict when the conflict is detected by the device.	4

5

All devices shall be capable of transmitting this command, although a device is not required to be capable of	6 receiving it.	7

8

The OWPAN ID conflict notification command shall be formatted as illustrated in Figure 85.	9

10

| Octets: (see 5.2.2.4) | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) |

11

12

13

14

15

16

Figure 85—OWPAN ID conflict notification command format	17

18

19

The Destination Addressing Mode and Source Addressing Mode subfields of the frame control field shall	20 both be set to three (i.e., 64-bit extended addressing).	21

22

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and	23 the Acknowledgment Request subfield shall be set to one.	24

25

The Destination OWPAN Identifier field shall contain the value of macOWPANId, while the Source	26

OWPAN  Identifier  field  shall  be  omitted. The  Destination Address  field  shall  contain  the  value  of	27

macCoordExtendedAddress. The Source Address field shall contain the value of aExtendedAddress.	28

29

5.4.9 Neighboring OWPAN report request command	30

31

The neighboring OWPAN report request command is sent by the coordinator to devices to request a report	32 of neighboring OWPAN information detected by the devices.	33

34

The neighboring OWPAN report request command shall be formatted as illustrated in Figure 86.	35

36

| Octets | 1 |
| --- | --- |
| MHR fields | Command Frame identifier |

37

38

39

40

41

Figure 86—Neighboring OWPAN report request command	42

43

44

5.4.10 Neighboring OWPAN report indication command	45

46

The neighboring OWPAN report indication command is sent by the device to the coordinator to report	47 neighboring OWPAN information. The neighboring OWPAN information is recorded in a local neighboring	48

OWPAN descriptor list, each record in the list corresponding to the information of one neighboring OWPAN	49 that can be detected by the device.	50

51

The neighboring OWPAN report indication command shall be formatted as illustrated in Figure 87.	52

53

The local neighboring OWPANs descriptor list is shown in Table 32.	54

Copyright © 20xx IEEE. All rights reserved.	123

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

| Octets | 1 | Variable |
| --- | --- | --- |
| MHR fields | Command Frame identifier | Local neighboring OWPAN descriptor list |

1

2

3

4

5

6	Figure 87—Neighboring OWPAN report indication command

7

8

9	Table 32—Lo c al ne ig hbo ring OWPANs de s c riptio n lis t

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Number NeighboringOW- PANRecords | Integer | 0x00 - 0xff | The number of records of neighboring OWPANs that are maintained by the device. |
| OWPANDescriptor[0] |  | Refer to Table 33 | The record of the first neighboring OWPAN that can be detected by the device. |
| LinkQuality[0] | Integer | 0x00 - 0xff | The measured RSS of the first neighboring OWPAN. |
| LastTimeDetected | Integer | 0x00 - 0xff | The sequence number of the superframe during which the device received a frame from the first neighbor- ing OWPAN for the last time. |
| OWPANDescriptor[N-1] |  | Refer to Table 33 | The record of the N-th neighboring OWPAN that can be detected by the device. |
| LinkQuality[N-1] | Integer | 0x00 - 0xff | The measured RSS of the Nth neighboring OWPAN. |
| LastTimeDetected | Integer | 0x00 - 0xff | The sequence number of the superframe during which the device received a frame from the first neighbor- ing OWPAN for the last time. |

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49	5.4.11 Beacon request command

50

51	The beacon request command is used by a device to locate all coordinators within its operating space during

52	an active scan.

53

54	This command is optional for a device.

124	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 33—The fo rmat o f the OWPAN de s c ripto r	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| OWPANid | integer | 0x0000 - 0xffff | The OWPAN ID of the neighboring OWPAN as specified in the received beacon frames. |
| CoordShortAddress | Integer |  | The 16 bits short address of the coordi- nator as specified in the received beacon frames. |
| CoordExtendedAddress | Integer |  | The 64 bits extended address of the coordi- nator as specified in the received beacon frames. |
| AssociationPermit | Boolean | TRUE or FALSE | Indicates whether a coordinator is cur- rently allowing associ- ation. A value of TRUE indicates that association is permit- ted. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

The beacon request command shall be formatted as illustrated in Figure 88.	29

30

31

| Octets: 7 | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) |

32

33

34

35

36

Figure 88—Beacon request command format	37

38

39

5.4.12 Additional beacon request command	40

41

The additional beacon request command is sent by a device to request a coordinator to send additional	42 beacon frames.	43

44

The command is optional for a device.	45

46

The additional beacon request shall be formatted as illustrated in Figure 89.	47

48

5.4.12.1 MHR fields	49

50

The Destination Addressing Mode subfield of the frame control field shall be set to two (i.e., 16-bit short              51 addressing),  and  the  Source Addressing  Mode  subfield  shall  be  set  to  zero  (i.e.,  source  addressing              52 information not present).                                                                                                                                                   53

54

Copyright © 20xx IEEE. All rights reserved.	125

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

| Octets (see 5.2.2.4) | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame identi- fier | Reason |

1

2

3

4

5

6	Figure 89—Additional beacon request command

7

8

9	The frame pending subfield of the frame control field shall be set to zero and ignored upon reception. The

10	Acknowledgment Request subfield and Security Enabled subfield shall also be set to zero.

11

12	The Destination OWPAN Identifier field shall contain the broadcast OWPAN identifier (i.e., 0xffff). The

13	Destination Address field shall contain the broadcast short address (i.e., 0xffff).

14

15	5.4.12.2 Reason field

16

17	The Reason field shall contain one of the non-reserved values listed in Table 34.

18

19

20	Table 34—Re as o n fie ld

| Value | Function |
| --- | --- |
| 0x00 | A new device that may be located at the interferefering area of multiple OWPANs fails to detect the beacon frame due to interference. |
| 0x01 | An associated device that is located at the interferefering area of multi- ple OWPANs fails to detect the beacon frame due to interference. |
| 0x02 - 0xFF | reserved |

21

22

23

24

25

26

27

28

29

30

31

32	5.4.13 Coordinator realignment command

33

34	The coordinator realignment command is sent by a coordinator when any of its OWPAN configuration

35	attributes change (i.e. OWPAN identifier, short address) due to the receipt of an MLME-START.request

36	primitive. It is broadcast to the OWPAN.

37

38	All devices shall be capable of receiving this command, although a device is not required to be capable of

39	transmitting it.

40

41	The coordinator realignment command shall be formatted as illustrated in Figure 90.

42

43

| Octets: 17/18/23/24 | 1 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | OWPAN Identifier | Coordinator Short Address | Short Address | Effec- tive Time |

44

45

46

47

48

49

50

51	Figure 90—Coordinator realignment command format

52

53

54

126	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.13.1 MHR fields	1

2

The Destination Addressing Mode subfield of the frame control field shall be set to two (e.g., 16-bit short              3 addressing) if it is to be broadcast to the OWPAN. The Source Addressing Mode subfield of the frame              4 control field shall be set to three (e.g., 64-bit extended addressing).                                                                               5

6

The frame pending subfield of the frame control field shall be set to zero and ignored upon reception.	7

8

The Acknowledgment Request subfield of the frame control field shall be set to zero if the command is to be              9 broadcast to the OWPAN.                                                                                                                                                 10

11

The Destination OWPAN Identifier field shall contain the broadcast OWPAN identifier (e.g., 0xffff). The	12

Destination Address field shall contain the broadcast short address (e.g., 0xffff). The Source OWPAN	13

Identifier field shall contain the value of macOWPANId, and the Source Address field shall contain the value	14 of aExtendedAddress.	15

16

5.4.13.2 OWPAN Identifier field	17

18

The OWPAN Identifier field shall contain the OWPAN identifier that the coordinator intends to use for all	19 future communications.	20

21

5.4.13.3 Coordinator Short Address field	22

23

The Coordinator Short Address field shall contain the value of macShortAddress.	24

25

5.4.13.4 Short Address field	26

27

If the coordinator realignment command is broadcast to the OWPAN, the Short Address field shall be set to	28

0xffff and ignored on reception.	29

30

5.4.13.5 Effective Time field	31

32

The effective time field shall contain the BSN of the corresponding superframe in which the new parameters	33 (OWPAN IDs, short address, e.g.) shall take effect.	34

35

5.4.14 GTS request command	36

37

The GTS request command is used by an associated device that is requesting the allocation of a new GTS or              38 the deallocation of an existing GTS from the coordinator. Only devices that have a 16-bit short address less              39 than 0xfffe shall send this command.                                                                                                                                40

41

This command is optional.	42

43

The GTS request command shall be formatted as illustrated in Figure 91.	44

45

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | GTS Characteristics |

46

47

48

49

50

51

Figure 91—GTS request command format	52

53

54

Copyright © 20xx IEEE. All rights reserved.	127

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.4.14.1 MHR fields

2

3	The Destination Addressing Mode subfield of the frame control field shall be set to zero (e.g., destination

4	addressing information not present), and the Source Addressing Mode subfield shall be set to two (e.g., 16-

5	bit short addressing).

6

7	The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and

8	the Acknowledgment Request subfield shall be set to one.

9

10	The Source OWPAN Identifier field shall contain the value of macOWPANId, and the Source Address field

11	shall contain the value of macShortAddress.

12

13	5.4.14.2 GTS Characteristics field

14

15	The GTS Characteristics field shall be formatted as illustrated in Figure 92.

16

| Bits: 0–3 | 4 | 5 | 6–7 |
| --- | --- | --- | --- |
| GTS Length | GTS Direction | Characteristics Type | Reserved |

17

18

19

20

21

22	Figure 92—GTS Characteristics field format

23

24

25	The GTS Length subfield shall contain the number of superframe slots being requested for the GTS.

26

27	The GTS Direction subfield shall be set to one if the GTS is to be a receive-only GTS. Conversely, this

28	subfield shall be set to zero if the GTS is to be a transmit-only GTS. GTS direction is defined relative to the

29	direction of data frame transmissions by the device.

30

31	The Characteristics Type subfield shall be set to one if the characteristics refers to a GTS allocation or zero

32	if the characteristics refers to a GTS deallocation.

33

34

35

36

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (see Table 28) | Blinking frequency |

43

44

45

46

47

48

49

50

51

52

54

128	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

3

4

5

6

9

10

| Bits: 0–3 | 4–7 |
| --- | --- |
| Frequency | Reserved |

11

12

13

14

15

16

17

18

5.4.16 Fast link recovery command	19

20

Fast link recovery command is used for the device or coordinator to send the fast link recovery (FLR) signal	21 and the fast link recovery response (FLR RSP), to help the link recovery.	22

23

Fast link recovery signal and response use the fast link recovery command format. The fast link recovery	24 command shall be formatted as illustrated in Figure 95.	25

26

27

| Octets: (as defined in 5.2.2.4) | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | FLR field |
|  |  | FLR field explanation Bit 0: ‘0’ indicating it is FLR signal, ‘1’ indicat- ing it is FLR RSP Bits 1–3: index of FLR signal direction, if bit 0 is ‘0’. received FLR signal direction index if bit 0 is ‘1’. Bits 4–7: reserved |

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

Figure 95—Fast link recovery command	44

45

46

The FLR signal and the FLR response (RSP) are differentiated by the first bit (bit 0) of the FLR field in the              47 fast link recovery command frame. The device can indicate the index of FLR signal direction by using bits 1              48 to 3 of the FLR field in the command frame. If the device receives the FLR signal and needs to send FLR              49

RSP, it repeats the received FLR signal direction index by using bits 1 to 3 of the FLR field in the command	50 frame. If the device is uni-direction, it uses ‘000’ as the index of the direction.	51

52

The usage of the FLR is presented in 5.1.11.	53

54

Copyright © 20xx IEEE. All rights reserved.	129

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

5

| Octets: 7 | 1 | variable |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (Table 28) | cellSearchQuality (see 5.1.12.3) |

6

7

8

9

10

11

12	Figure 96—Mobility notification command

13

14

19

20	5.4.18 GTS response command

21

22	The optional GTS.response primitive is generated in response to a GTS.request primitive. When used, the

23	GTS response command shall be formatted as illustrated in Figure 97.

24

| Octets: 7 | 1 | 1 | 1 |
| --- | --- | --- | --- |
| MHR fields | Command Frame Identifier as defined in Table 28) | GTS characteristics | GTS Starting Slot (see 5.2.2.1.6) |

25

26

27

28

29

30

31	Figure 97—GTS response command format

32

33

34	5.4.18.1 MHR fields

35

36	The Destination Addressing Mode subfield of the frame control field shall be set to zero (e.g., destination

37	addressing information not present), and the Source Addressing Mode subfield shall be set to two (e.g., 16-

38	bit short addressing).

39

40	The frame pending subfield of the frame control field shall be set to zero and ignored upon reception, and

41	the Acknowledgment Request subfield shall be set to one.

42

43	The Source OWPAN Identifier field shall contain the value of macOWPANId, and the Source Address field

44	shall contain the value of macShortAddress.

45

46	5.4.18.2 GTS Characteristics field

47

48	The GTS Characteristics field shall be formatted as illustrated in Figure 98.

49

50	The GTS Length subfield shall contain the number of superframe slots being requested for the GTS.

51

52	The GTS Direction subfield shall be set to one if the GTS is to be a receive-only GTS. Conversely, this

53	subfield shall be set to zero if the GTS is to be a transmit-only GTS. GTS direction is defined relative to the

54	direction of data frame transmissions by the device.

130	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Bits: 0–3 | 4 | 5 | 6–7 |
| --- | --- | --- | --- |
| GTS Length | GTS Direction | Characteristics Type | Reserved |

1

2

3

4

Figure 98—GTS Characteristics field format	5

6

7

The Characteristics Type subfield shall be set to one if the characteristics refers to a GTS allocation or zero	8 if the characteristics refers to a GTS deallocation.	9

10

5.4.19 Clock rate change notification command	11

12

The command format for the clock rate change notification is as shown in Figure 99. This clock rate change              13 notification is sent at the current clock rate negotiated between the devices. All future transmissions from the              14 current device to the receiving device will occur at this new clock rate.                                                                         15

16

17

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | New clock rate for future TX |

18

19

20

21

22

Figure 99—Clock rate change notification format	23

24

25

The modulation and coding scheme (MCS) ID from Table 115 shall be used to indicate the optical clock	26 rate. Any MCS ID corresponding to the chosen future clock rate can be used. The 6 LSBs shall be set to the	27

MCS ID corresponding to the future clock rate. The other bits are set to 0 and reserved for future use.	28

29

30

31

32

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (as defined in Table 28) | Multiple Channels |

33

34

35

36

37

38

39

40

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	131

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command frame identifier (see Table 28) | CVD disable |

3

4

5

6

7

8

9

10

11

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40	5.4.22 CSI Control Frame

41

42	Editor’s note: need to register the new command frame in Table 28, the format Figure 102 need to be

43	adjusted to be compatible with other command frames

44

45	The CSI control frame contains the standard MAC header information defined in subclause 5.8.2. The frame

46	body described in this clause is the only specific field. It is structured as described in Fig. 5.8.4.3.2.1.

47

48	5.4.22.1 MIMO Mode

49

50	A one bit value indicating whether the current CSI frame contains information relevant for the SISO or

51	MIMO mode of operation:

52

53	1  MIMO mode of operation.

54

132	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Bits: 1 | 1 | 1 | 4 | 4 | 4/8 | 4/6 |
| --- | --- | --- | --- | --- | --- | --- |
| MIMO Mode | Adaptive Loading | Frame Info | Number of MIMO Channels TX | Number of MIMO Channels RX | Channel Number | RATE/CSI |

1

2

3

4

5

Figure 102—Format of the CSI control frame body	6

7

8

0  SISO mode of operation.	9

10

5.4.22.2 Adaptive Loading	11

12

A one-bit field which indicates whether the CSI in the current frame refers to the adaptive bit loading mode.	13

14

1  The CSI is relevant for the adaptive bit loading scheme	15

16

0  The CSI is relevant for the fixed bit loading scheme	17

18

5.4.22.3 Frame Info	19

20

A one-bit field indicating the type of information carried by the current frame:	21

22

'0'  The frame carries CQIs	23

24

'1'  The frame carrier bit loading (RATE) information	25

26

5.4.22.4 Number of MIMO Channels TX	27

28

The number of MIMO channels of the transmitting node in the MIMO configuration. Values range between	29

0 and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this              30 field contains the number of the MIMO channels for the node receiving the frame. If the frame carriers bit              31 loading (RATE) information, this field contains the number of the MIMO channels for the node transmitting              32 the frame. This information is expected to have been exchanged via the advanced modulation control frame              33 prior to any node issuing a CSI control frame.                                                                                                                 34

35

5.4.22.5 Number of MIMO Channels RX	36

37

The number of MIMO channels of the receiving node in the MIMO configuration. Values range between 0              38 and 15 corresponding to 1 and 16 channels, respectively. If the current CSI frame carries CQIs, then this              39 field contains the number of the MIMO channels for the node transmitting the frame. If the frame carriers bit              40 loading (RATE) information, the this field contains the number of the MIMO channels for the node receiv-              41 ing the frame. This information is expected to have been exchanged via the advanced modulation control              42 frame prior to any node issuing a CSI control frame.                                                                                                       43

44

5.4.22.6 Channel Number	45

46

The MIMO channel to which the subsequent RATE/CSI information refers. In a SISO mode, this field is set	47 to '0000'. If the Adaptive Loading field is set to '1', then this field occurs only once and is followed by the	48

CQI / RATE information for every subcarrier of the respective MIMO channel. If the Adaptive Loading              49 field is set to '0', then this field occurs as many times as there are MIMO channels for which relevant infor-              50 mation needs to be exchanged followed by the relevant information as CQI / RATE values. If the Frame Info              51

is set to '0', then this field occurs Number of MIMO Channels TX . Number of MIMO Channels RX times	52

where the 6-bit CQI following each occurrence corresponds to the average SNR achieved for the respective	53 combination between the corresponding MIMO transmitter and MIMO receiver. The order in which the	54

Copyright © 20xx IEEE. All rights reserved.	133

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2  RX1 | TX2 

2	 RX2 | … |TX2  RXN_MIMO| … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | … |TXM_MIMO 

3	RXN_MIMO |. If the Frame Info is set to '1', then this field occurs #_of_MIMO_Channels_TX times where

4	the information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO|. In each case, the occurrence of

5	the Channel # is followed by the respective 4-bit RATE value for that TX channel.

6

7	5.4.22.7 RATE / CSI

8

9	The RATE / CSI values for the MIMO channel specified in Channel #. If the Adaptive Loading field is set to

10	'1', then this field occurs once for every subcarrier modulated with information on the respective MIMO

11	channel. If the Adaptive Loading field is set to '0', then this field occurs as many times as there are MIMO

12	channels for which relevant information needs to be exchanged and is preceded by the relevant MIMO

13	Channel # field. If the Frame Info is set to '0', then this field occurs #_of_MIMO_Channels_TX ? x_of_MI-

14	MO_Channels_RX times where a 6-bit CQI value is specified corresponding to the average SNR achieved

15	for the respective combination between the corresponding MIMO transmitter and MIMO receiver. The order

16	in which the information is arranged is | TX1  RX | TX1  RX2 |…| TX1  RXN_MIMO| TX2 

17	RX1 | TX2  RX2 | … |TX2  RXN_MIMO | … | TXM_MIMO  RX1 | TXM_MIMO  RX2 | …

18	|TXM_MIMO  RXN_MIMO |. If the Frame Info is set to '1', then this field occurs # of MIMO Channels TX

19	times where the 4-bit RATE information is arranged in the order | TX1 | TX2 | TX3 | … | TXM_MIMO |. In

20	each case, the occurrence of the RATE field is preceded by the respective Channel # value for that TX chan-

21	nel.

22

23	5.4.23 Information element command

24

25	The format of an individual information element (IE) is shown in Figure 103. The first octet is the Element

26	ID and the second octet is the Length (Ln) of the payload of the IE in octets. The following Ln octets are the

27	payload for the IE. Unless otherwise specified, these elements may appear in any order in the frames that are

28	allowed to include more than one of these elements.

29

| Octets: 1 | 1 | Ln |
| --- | --- | --- |
| Element ID | Length (=Ln) | IE payload |

30

31

32

33

34

35	Figure 103—Information element format

36

37

38	The information elements defined in this standard are listed in Table 36.

39

40

41	Table 36—Info rmatio n e le me nts

| Element ID hex value | Element | Subclause |
| --- | --- | --- |
| 0x01 | Capabilities | 5.4.23.1 |
| 0x02 | Wavelength quality indication | 5.4.23.2 |

42

43

44

45

46

47

48

49

50

51	When the information elements are used, they shall be added at the end of command frame format. Multiple

52	information elements can be part of a single command frame. IEs can be added to any command frame.

53

54

134	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.23.1 Capabilities IE	1

2

The capabilities IE is used to convey device MAC and PHY capabilities to peer devices. The capabilities IE,              3 as shown in Figure 104, consists of two fields: the capability information field, refer to Table 37, which              4 indicates general  capabilities of  the  device;  and  the  aggregation bitmap  field,  which  is  specified in              5

Figure 104.	6

7

| Octets: 8 | variable: 8n | variable: 8n |
| --- | --- | --- |
| Capability Information Field | Aggregation bitmap field | Guard bitmap field |

8

9

10

11

12

Figure 104—Capabilities IE	13

14

15

5.4.23.1.1 Capability info rmatio n fie ld	16

17

The capability information field is illustrated in Table 37.	18

19

20

Table 37—Capability info rm atio n fie ld	21

|  | Bit position | Function |
| --- | --- | --- |
| MAC layer capabilities | 0 | Power source |
|  | 1–2 | Battery information |
|  | 3 | Security capability |
|  | 4 | Coordinator capability |
|  | 5 | Traffic support |
|  | 6–8 | Topology support |
|  | 9–10 | Device type |
|  | 11 | Beacon support |
|  | 12 | Continuous visibility transmission (for infrastructure) |
|  | 13 | CVD support |
|  | 14-23 | Reserved |
| PHY layer capabilities | 24 | PM-PHY PHY support |
|  | 25 | LB-PHY support |
|  | 26 | HB-PHY support |
|  | 27–29 | Max supported TX clock |
|  | 30–32 | Max supported RX clock |
|  | 33 | Explicit clock notification request |
|  | 34 | CCA support |
|  | 35-39 | Reserved |

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	135

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 37—Capability info rmatio n fie ld  (continued)

|  | Bit position | Function |
| --- | --- | --- |
| Physical device capabilities | 40–42 | Number of optical sources |
|  | 43–45 | Multiple direction support |
|  | 46–55 | Number of cells supported (n) |
|  | 56–63 | Reserved |

2

3

4

5

6

7

8

9

10

11

12

13	The power source subfield is 1 bit in length and shall be set to one if the device is receiving power from the

14	alternating current mains. Otherwise, the power source subfield shall be set to zero.

15

16	The battery information subfield, shown in Table 38, is set to reserved (11) if the power source is set to 1.

17

18

19	Table 38—Batte ry Indic atio n

20

| Bits (b2 b1) | Battery indication |
| --- | --- |
| 00 | Unknown |
| 11 | < 50% (low battery) |
| 10 | 50% (sufficient battery) |
| 11 | Reserved |

21

22

23

24

25

26

27

28

29

30

31	The security capability subfield is 1 bit in length and shall be set to one if the device is capable of sending

32	and receiving cryptographically protected MAC frames; otherwise, it shall be set to zero.

33

34	The coordinator capability subfield is 1 bit in length and shall be set to 1 if the device is capable of

35	functioning as a coordinator; otherwise, it shall be set to zero.

36

37	The traffic support capability subfield is 1 bit in length. It shall be set to 0 if the device is only capable of

38	broadcasting (unidirectional) communication. Otherwise, it shall be set to 1.

39

40	The topology support capability subfield can support multiple topologies via the bit maps of Table 38.

41

42

43	Table 39—To po lo g y s uppo rt c apability

| Bits (b8 b7 b6) | Topology indication |
| --- | --- |
| b8 | P2MP |
| b7 | P2P |
| b6 | Broadcast |

44

45

46

47

48

49

50

51

52

53

54

136	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The device-type capability subfield is set according to Table 40. This information is provided to assist upper	1 layers.	2

3

4

Table 40—Devic e -type capability	5

| Bits (b10 b9) | Device capability |
| --- | --- |
| 00 | Infrastructure |
| 01 | Mobile |
| 10 | Vehicle |
| 11 | Unknown/reserved |

6

7

8

9

10

11

12

13

14

15

16

The beacon support capability subfield is 1 bit in length. It shall be set to 1 if the device is capable of sending	17

beacons. Otherwise, it shall be set to 0.	18

19

The dimming support in MAC capability subfield is 1 bit in length. It shall be set to 1 if the device is capable	20

of supporting dimming in the MAC using duty cycling and idle patterns. Otherwise, it shall be set to 0. A	21

device shall honor all dimming requests. If the dimming support bit is not set then the device shall not	22

attempt to communicate when a dimming request is received and shall comply with the dimming request	23

even if the device must disassociate from the network as discussed in 5.1.6.2. Even if the device supports	24

dimming but is unable to communicate during dimming, it shall set the macDimDataFailureIndication	25

MAC PIB attribute as mentioned in Table 91, but shall still comply with the dimming request at the expense	26

of loss of communication.	27

28

The continuous visibility transmission subfield is one bit in length. It shall be set to 1 if the device will be	29

continuously transmitting to maintain illumination. Otherwise, it shall be to 0.	30

31

The CVD support subfield is 1 bit in length. It shall be set to 1 if the device is capable of transmitting various	32

colors; otherwise, it shall be set to 0.	33

34

The PHY I support subfield is 1 bit in length. It shall be set to 1 if the device supports PHY I.	35

36

The PHY II support subfield is 1 bit in length. It shall be set to 1 if the device supports PHY II.	37

38

The PHY III support subfield is 1 bit in length. It shall be set to 1 if the device supports PHY III.	39

40

The max supported TX clock subfield and max RX clock subfields follow the usage as indicated in Table 41.	41

Support for 200 kHz is mandatory for PHY I and support of 3.75 MHz is mandatory for PHY II. Support for	42

12 MHz is mandatory for PHY III and shall be indicated using bits ‘100’ as in Table 41.	43

44

The explicit clock notification subfield is 1 bit in length. The subfield shall be set to 1 if the receiving device	45

needs an explicit clock change notification from the transmitter before any change of clock frequency.	46

47

If CCA is supported, then the CCA Support bit is set to 1; otherwise, the bit is set to 0.	48

49

The number of optical sources subfield indicates the number of optical sources in the transmitter of the	50

device that have distinct frequency responses.	51

52

The multiple direction support subfield indicates the number of distinct directions supported by the device	53

transmitter supported by the multiple optical sources. This is used for fast link recovery as defined in 5.4.16.	54

Copyright © 20xx IEEE. All rights reserved.	137

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 41—Maxim um s uppo rte d o ptic al c lo c k fre que nc y

| Bits (b31 b30 b29) | Description |
| --- | --- |
| 000 | 200 kHz |
| 001 |  400 kHz |
| 010 | 3.75 MHz |
| 011 | 7.5 MHz |
| 100 | 15 MHz |
| 101 | 30 MHz |
| 110 | 60 MHz |
| 111 | 120 MHz |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19	Editor note: the table need to be revised when clock rates have been concluded.

20

21	The number of cells n indicates the maximum number of cells supported in the device. The number of cells

22	supported shall not be more than 1023.

23

24	In regards to the bands used for PHY III, bit 7 is reserved and bits 0–6 map to the bits corresponding to the

25	bandplan. Only 3 bits shall be set to indicate PHY III usage. If the device supports more colors and wants to

26	change the PHY III usage, it needs to send the capabilities information again with the new bitmap.

27

28	editor note: the following advanced modulation capabilities should be merged into capabilities IE, see com-

29	ment 119 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

30

31	Advanced Modulation Control Frame

32

33	This control frame shall indicate the advanced modulation capabilities of a communication node. The frame

34	header contains the MAC header fields defined in subclause 6.4.1 and the frame body shall be structured as

35	described in Fig. 1.

36

| Bits: 1 | 4 | 1 | 4 | 1 | 4 | IDFT_SIZE |
| --- | --- | --- | --- | --- | --- | --- |
| Adative Loading | eU | RPO | Relaying | MIMO | # of MIMO Channel | SC-FDMA |

37

38

39

40

41

42

43	Figure 105—Format of the advanced modulation control frame body

44

45

46	5.4.23.1.2 Adaptive Lo ading

47

48	A single bit indicating whether the communication node transmitting the advanced modulation control

49	frame supports adaptive bit and energy loading:

50

51	1  Adaptive bit and energy loading is supported.

52

53	0  Adaptive bit and energy loading is not supported.

54

138	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

5.4.23.1.3 e U	1

2

Four bits indicating if the node supports eU-OFDM. The bit value at a given position out of the four posi-              3 tions indicates whether eU-OFDM implementation with the same number of streams as the bit position is              4 supported. Positions are counted from left to right. For example:                                                                                    5

6

1000  eU-OFDM with one stream only is supported.	7

8

1100  eU-OFDM with one and two streams only is supported	9

10

1010  eU-OFDM with one and three streams only is supported	11

12

1111  eU-OFDM with all possible streams is supported	13

14

0000  eU-OFDM is not supported	15

16

5.4.23.1.4 RPO	17

18

A single bit indicating whether the communication node transmitting the advanced modulation control rame	19 supports RPO-OFDM:	20

21

'1'  RPO-OFDM is supported.	22

23

'0'  RPO-OFDM is not supported.	24

25

5.4.23.1.5 Re laying	26

27

Four bits indicating types of relaying operations the communication node transmitting the advanced modula-	28 tion control frame supports. The first bit (B7) indicates whether relaying in FD is supported:	29

30

'1'  Relaying in FD is supported.	31

32

'0'  Relaying in FD is not supported.	33

34

The second bit (B8) indicates whether relaying in HD is supported:	35

36

'1'  Relaying in HD is supported.	37

38

'0'  Relaying in HD is not supported.	39

40

The third bit (B9) indicates whether AF relaying is supported:	41

42

'1'  AF relaying is supported.	43

44

'0'  AF relaying is not supported.	45

46

The fourth bit (B10) indicates whether DF relaying is supported:	47

48

'1'  DF relaying is supported.	49

50

'0'  DF relaying is not supported.	51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	139

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	5.4.23.1.6 MIMO

2

3	A single bit indicating whether the communication node transmitting the advanced modulation control

4	frame supports MIMO communication:

5

6	'1'  MIMO is supported.

7

8	'0'  MIMO is not supported.

9

10	5.4.23.1.7 Numbe r o f MIMO Channe ls

11

12	Four bits indicating the maximum number of MIMO communication channels which the communication

13	node transmitting the advanced modulation control frame supports. A value of '0000' corresponds to 1 chan-

14	nel, and a value of '1111' corresponds to 16 channels.

15

16	5.4.23.1.8 S C-FDMA

17

18	An IDFT_SIZE of bits, where IDFT_SIZE is the size of the IDFT operation used for OFDM time-domain

19	signal generation. The bit at position k indicates whether a DFT transform of size k is supported for SC-

20	FDMA pre-coding. For example, if IDFT_SIZE = 16, then:

21

22	'10000000000000000' - A DFT of size 1 is supported.

23

24	'00100000000000000' - A DFT of size 3 is supported.

25

26	'10100100000000000' - A DFT of size 1, 3 and 6 is supported.

27

28	'00000000000000000' - No DFT size is supported, so SC-FDMA is not supported.

29

30

31

32	5.4.23.1.9 Ag g re g atio n and g uard c hanne l

33

34	The aggregation and guard channels are used to support any optical light optical source for OWC that may

35	have variable spectral widths and center frequencies. The aggregation and guard bitmap for a single optical

36	source type is as shown in Figure 106. The bit map is variable in length. The length of the aggregation and

37	guard bit maps are ‘n’ octets each, where ‘n’ is the number of optical source types. The aggregation and

38	guard channel bit usage are defined by an 8-bit bitmap for every optical source type supported by the

39	transmitter of the device. The 8-bit bitmap is indexed by the bandplan identification number. The bit position

40	‘m’ is set to a ‘1’ for band 'm' if that band is used by the optical source. The reserved bit in Figure 106 shall

41	be set to 0.

42

43

44

45

46

47

48

49

50

51

52

53

54

140	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

For example,

1

2

3

4

5

6

7

8

9

10

11

12

Figure 106—Aggregation and guard bitmap per optical source type	13

14

15

16

17

if band 1 and band 2 need to be aggregated [assuming a blue light-emitting diode (LED)], the	18 aggregation bit-map is indicated as 0110000 and the guard bit-map is indicated as 0000000.	19

20 if band 1 is being used but there is leakage in bands 3, 4, 5 (assuming a white LED, which is	21

realized via a blue LED with yellow phosphor), the aggregation bit map is indicated as 0100000 and the	22 guard bit-map is indicated as 0001110.	23

24

5.4.23.2 Wavelength quality indication (WQI) IE	25

26

Editor note: WQI needed to be redefined.	27

28

WQI is communicated to another device using the WQI Information Element. The WQI value to be sent in	29

the Information Element may be an average value across a number of packets, and WQI value sets for a	30

number of band plan ID's can be reported using the WQI information element as shown in Table 42. The	31

wavelength quality indication IE is 7 octets in length and the WQI information is provided for all band plan	32

IDs. If a band plan ID is not supported, WQI of 0 shall be reported.	33

34

Table 42—Wave le ng th quality indic atio n IE	35

36

| Band plan code | WQI value |
| --- | --- |
| 0x00 | 0x00 to 0xff |
| 0x01 | 0x00 to 0xff |
| 0x02 | 0x00 to 0xff |
| 0x03 | 0x00 to 0xff |
| 0x04 | 0x00 to 0xff |
| 0x05 | 0x00 to 0xff |
| 0x06 | 0x00 to 0xff |

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	141

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6. MAC sublayer service specification

2

3

4	6.1 Overview

5

6	The MAC sublayer provides an interface between the SSCS, DME and the PHY. The MAC sublayer

7	conceptually includes a management entity called the MLME. This entity provides the service interfaces

8	through which layer management functions may be invoked. The MLME is also responsible for maintaining

9	a database of managed objects pertaining to the MAC sublayer. This database is referred to as the MAC

10	sublayer PIB.

11

12	Figure 107 depicts the components and interfaces of the MAC sublayer.

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27	Figure 107—MAC sublayer reference model

28

29	The MAC sublayer provides the following two services, accessed through two SAPs:

30

31	a)	The MAC data service, accessed through the MAC common part sublayer (MCPS) data SAP

32	(MCPS-SAP), and

33	b)	The MAC management service, accessed through the MLME-SAP.

34

35	These two services provide the interface between the SSCS and the PHY, via the PD-SAP and PLME-SAP

36	interfaces (see Clause 9). In addition to these external interfaces, an implicit interface also exists between the

37	MLME and the MCPS that allows the MLME to use the MAC data service.

38

39

40	6.2 MAC data service

41

42	The MCPS-SAP supports the transport of SSCS protocol data units (SPDUs) between peer SSCS entities.

43	Table 43 lists the primitives supported by the MCPS-SAP. These primitives are discussed in the subclauses

44	referenced in this table.

45

46

47	Table 43—MCPS -S AP primitive s

| MCPS-SAP primitive | Request | Confirm | Indication |
| --- | --- | --- | --- |
| MCPS-DATA | 6.2.1 | 6.2.2 | 6.2.3 |
| MCPS-PURGE | 6.2.4 | 6.2.5 | — |

48

49

50

51

52

53

54

142	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.2.1 MCPS-DATA.request	1

2

The MCPS-DATA.request primitive requests the transfer of a data SPDU (i.e., MSDU) from a local SSCS              3 entity to a single peer SSCS entity. In the packed mode, multiple MSDU are passed via a local SSCS entity              4 to a single peer SSCS entity.                                                                                                                                             5

6

The semantics of the MCPS-DATA.request primitive are as follows:	7

MCPS -DATA.reque st	(	8

S rcAddrMode,	9

Ds tAddrMode ,	10

Ds tOWPANId,	11

Ds tAddr,	12

Ms duLe ngth,	13

Ms du,	14

Ms duHa ndle ,	15

TxOptions ,	16

SecurityLe ve l,	17

Ke yIdMode ,	18

Ke yS ource ,	19

Ke yInde x,	20

Da ta Ra te ,	21

Burs tMode ,	22

ColorRece ive d,	23

ColorNotRe ce ive d	24

)	25

26

27

28

Table 44 specifies the parameters for the MCPS-DATA.request primitive.	29

30

31

Table 44—MCPS -DATA.re que s t parame te rs	32

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Integer | 0x00–0x03 | The source addressing mode for this primitive and subsequent MPDU. This value can take one of the following values: 0x00 = No address (addressing fields omitted, as defined in 5.2.1.1.7). 0x01 = Reserved. 0x02 = 16-bit short address. 0x03 = 64-bit extended address. |
| DstAddrMode | Integer | 0x00–0x03 | The destination addressing mode for this primitive and subsequent MPDU. This value can take one of the following values: 0x00 = No address (addressing fields omitted, as defined in 5.2.1.1.6). 0x01 = No address field (broadcast only mode with no address fields present). 0x02 = 16-bit short address. 0x03 = 64-bit extended address. |
| DstOWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the entity to which the MSDU is being transferred. |

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	143

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 44—MCPS -DATA.re que s t param e te rs  (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddr | Device address | As specified by the DstAddrMode parameter | The individual device address of the entity to which the MSDU is being transferred. |
| MsduLength | Integer |  aMaxMACPayloadSize | The number of octets contained in the MSDU to be transmitted by the MAC sublayer entity. |
| Msdu | Set of octets | — | The set of octets forming the MSDU to be transmitted by the MAC sublayer entity. |
| MsduHandle | Integer | 0x00–0xff | The handle associated with the MSDU to be transmitted by the MAC sublayer entity. |
| TxOptions | Bitmap | 3-bit field | The 3 bits (b0, b1, b2) indicate the transmission options for this MSDU. For b0, 1 = acknowledged transmission, 0 = unacknowledged transmission. For b1, 1 = GTS transmission, 0 = CAP transmission for a beacon-enabled OWPAN. For b2, 1 = indirect transmission, 0 = direct transmission. For a non-beacon-enabled OWPAN, bit b1 should always be set to 0. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| DataRate | Enumeration | 6-bit field | The data rate of the PHY frame to be transmitted by the PHY entity as shown in Table 115. |
| BurstMode | Boolean | TRUE or FALSE | The BurstMode bit shall be set TRUE if the burst mode is being used (as discussed in 8.6.1); otherwise, the BurstMode bit shall be set FALSE. |
| ColorReceived | Boolean | TRUE or FALSE | ColorReceived shall be set as TRUE, when the ACK frame is sent and the color function for the ACK state indication is used by the CVD frame. ColorReceived shall be set as FALSE when the ACK frame is sent but the color function for the ACK state indication is not used by the CVD frame. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

144	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| (continued) | 1 |
| --- | --- |
|  | 2 |
|  | 3 |
|  | 4 |
|  | 5 |
|  | 6 |
|  | 7 |
|  | 8 |
|  | 9 |
|  | 10 |
|  | 11 |
|  | 12 |
|  | 13 |
|  | 14 |
|  | 15 |

Table 44—MCPS -DATA.re que s t param e te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ColorNotReceived | Boolean | TRUE or FALSE | ColorNotReceived shall be set as TRUE, when the ACK frame is not sent but the color function for the non-ACK state indication is used by the CVD frame. ColorNotReceived shall be set as FALSE when the ACK frame is not sent and the color function for the non-ACK state indication is not used by the CVD frame. |

6.2.1.1 Appropriate usage

16

The MCPS-DATA.request primitive is generated by a local SSCS entity when a data SPDU (i.e., MSDU) is	17

to be transferred to a peer SSCS entity.	18

19

6.2.1.2 Effect on receipt	20

21

On receipt of the MCPS-DATA.request primitive, the MAC sublayer entity begins the transmission of the	22

supplied MSDU.	23

24

The  MAC  sublayer  builds  an  MPDU  to  transmit  from  the  supplied  arguments.  The  flags  in  the	25

SrcAddrMode and DstAddrMode parameters correspond to the addressing subfields in the frame control	26

field, as shown in 5.2.1.1, and are used to construct both the frame control and addressing fields of the MHR.	27

If both the SrcAddrMode and the DstAddrMode parameters are set to 0x00 (i.e., addressing fields omitted),	28

the MAC sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_ADDRESS.	29

30

The TxOptions parameter indicates how the MAC sublayer data service transmits the supplied MSDU. If the	31

TxOptions  parameter  specifies  that  an  acknowledged  transmission  is  required,  the Acknowledgment	32

Request subfield of the frame control field will be set to one (see 5.1.9.4).	33

34

If the TxOptions parameter specifies that a GTS transmission is required, the MAC sublayer will determine	35

whether it has a valid GTS (for GTS usage rules, as defined in 5.1.10.3). If a valid GTS could not be found,	36

the MAC sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_GTS. If a valid	37

GTS was found, the MAC sublayer will defer, if necessary, until the GTS. If the TxOptions parameter	38

specifies that a GTS transmission is not required, the MAC sublayer will transmit the MSDU using either	39

slotted random access in  the  CAP for  a  beacon-enabled OWPAN or  unslotted random access  for  a	40

nonbeacon-enabled OWPAN. Specifying a GTS transmission in the TxOptions parameter overrides an	41

indirect transmission request.	42

43

If the TxOptions parameter specifies that an indirect transmission is required and this primitive is received	44

by the MAC sublayer of a coordinator, the data frame is sent using indirect transmission, i.e., the data frame	45

is added to the list of pending transactions stored on the coordinator and extracted at the discretion of the	46

device concerned using the method described in 5.1.9.3. Transactions with a broadcast destination address	47

will be transmitted using the mechanism described in 5.2.1.1.4. Transactions with a unicast destination	48

address can then be extracted at the discretion of each device concerned using the method described in	49

5.1.9.3. If there is no capacity to store the transaction, the MAC sublayer will discard the MSDU and issue	50

the MCPS-DATA.confirm primitive with a status of TRANSACTION_OVERFLOW. If there is capacity to	51

store the transaction, the coordinator will add the information to the list. If the transaction is not handled	52

within  macTransactionPersistenceTime, the  transaction  information  will  be  discarded  and  the  MAC	53

sublayer will issue the MCPS-DATA.confirm primitive with a status of TRANSACTION_EXPIRED. The	54

Copyright © 20xx IEEE. All rights reserved.	145

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	transaction handling procedure is described in 5.1.8. If the TxOptions parameter specifies that an indirect

2	transmission is required and if the device receiving this primitive is not a coordinator, the destination address

3	is not present, or the TxOptions parameter also specifies a GTS transmission, the indirect transmission

4	option will be ignored.

5

6	If the TxOptions parameter specifies that an indirect transmission is not required, the MAC sublayer will

7	transmit the MSDU using slotted random access either in the CAP for a beacon-enabled OWPAN or

8	immediately  for  a  nonbeacon-enabled  OWPAN.  If  the  TxOptions  parameter  specifies  that  a  direct

9	transmission is required and the MAC sublayer does not receive an acknowledgment from the recipient after

10	macMaxFrameRetries retransmissions (see  5.1.9.4), it  will  discard  the  MSDU  and  issue  the  MCPS-

11	DATA.confirm primitive with a status of NO_ACK.

12

13	If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

14	this frame, the MAC sublayer will set the Security Enabled subfield of the frame control field to one. The

15	MAC sublayer will perform outgoing processing on the frame based on the DstAddr, SecurityLevel,

16	KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during

17	outgoing frame processing, the MAC sublayer will discard the frame and issue the MCPS-DATA.confirm

18	primitive with the error status returned by outgoing frame processing.

19

20	If the requested transaction is too large to fit in the CAP or GTS, as appropriate, the MAC sublayer shall

21	discard the frame and issue the MCPS-DATA.confirm primitive with a status of FRAME_TOO_LONG.

22

23	If the transmission attempts a random access (either slotted or unslotted) and the random access algorithm

24	failed due to adverse conditions on the channel, and the TxOptions parameter specifies that a direct

25	transmission is required, the MAC sublayer will discard the MSDU and issue the MCPS-DATA.confirm

26	primitive with a status of CHANNEL_ACCESS_FAILURE.

27

28	If the MAC sublayer receives the request while transmission is prohibited, it shall delay transmission until

29	transmission is permitted.

30

31	If the MPDU was successfully transmitted and, if requested, an acknowledgment was received, the MAC

32	sublayer will issue the MCPS-DATA.confirm primitive with a status of SUCCESS.

33

34	If any parameter in the MCPS-DATA.request primitive is not supported or is out of range, the MAC sublayer

35	will issue the MCPS-DATA.confirm primitive with a status of INVALID_PARAMETER.

36

37	6.2.2 MCPS-DATA.confirm

38

39	The MCPS-DATA.confirm primitive reports the results of a request to transfer a data SPDU (MSDU) from a

40	local SSCS entity to a single peer SSCS entity.

41

42	The semantics of the MCPS-DATA.confirm primitive are as follows:

43

44	MCP S -DATA.confirm	(

45	Ms duHa ndle ,

46	s ta tus ,

47	Time s ta mp

48	)

49

50	Table 45 specifies the parameters for the MCPS-DATA.confirm primitive.

51

52

53

54

146	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.2.2.1 When generated

Table 45—MCPS -DATA.c o nfirm param e te rs	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MsduHandle | Integer | 0x00–0xff | The handle associated with the MSDU being confirmed. |
| status | Enumeration | SUCCESS, TRANSACTION_OVERFLOW, TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, INVALID_ADDRESS, INVALID_GTS, NO_ACK, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY or INVALID_PARAMETER | The status of the last MSDU transmission. |
| Timestamp | Integer | 0x000000–0xffffff | Optional. The time, in optical clocks, at which the data were transmitted (see 5.1.7.1). The value of this parameter will be considered valid only if the value of the status parameter is SUCCESS; if the status parameter is not equal to SUCCESS, the value of the Timestamp parameter shall not be used for any other purpose. The boundary is described by macTimeStampOffset (as defined in Table 91). The time stamp is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

The MCPS-DATA.confirm primitive is generated by the MAC sublayer entity in response to an MCPS-	36

DATA.request  primitive.  The  MCPS-DATA.confirm  primitive  returns  a  status  of  either  SUCCESS,              37 indicating that the request to transmit was successful, or the appropriate error code. The status values are              38 fully described in 6.2.1.2 and subclauses referenced by 6.2.1.2.                                                                                      39

40

6.2.2.2 Appropriate usage	41

42

On receipt of the MCPS-DATA.confirm primitive, the SSCS of the initiating device is notified of the result	43 of its request to transmit. If the transmission attempt was successful, the status parameter will be set to	44

SUCCESS. Otherwise, the status parameter will indicate the error.	45

46

6.2.3 MCPS-DATA.indication	47

48

The MCPS-DATA.indication primitive indicates the transfer of a data SPDU (i.e., MSDU) from the MAC              49 sublayer to the local SSCS entity. In the packed mode, multiple MSDU are passed via a local SSCS entity to              50 a single peer SSCS entity.                                                                                                                                                 51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	147

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The semantics of the MCPS-DATA.indication primitive are as follows:

2

3	MCP S -DATA.indica tion	(

4	S rcAddrMode ,

5	S rcOWPANId,

6	S rcAddr,

7	Ds tAddrMode ,

8	Ds tOWPANId,

9	Ds tAddr,

10	Ms duLe ngth,

11	Ms du,

12	MpduLinkQua lity,

13	DS N,

14	Time s ta mp,

15	SecurityLe ve l,

16	Ke yIdMode ,

17	Ke yS ource ,

18	Ke yInde x,

19	Da ta Ra te ,

20	Burs tMode ,

21	ColorRe ce ive d,

22	ColorNotRe ce ive d

23	)

24

25

26	Table 46 specifies the parameters for the MCPS-DATA.indication primitive.

27

28

29	Table 46—MCPS -DATA.indic atio n param e te rs

30

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Integer | 0x00–0x03 | The source addressing mode for this primitive corresponding to the received MPDU. This value can take one of the following values: 0x00 = no address (addressing fields omitted). 0x01 = reserved. 0x02 = 16-bit short address. 0x03 = 64-bit extended address. |
| SrcOWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the entity from which the MSDU was received. |
| SrcAddr | Device address | As specified by the SrcAddrMode parameter | The individual device address of the entity from which the MSDU was received. |
| DstAddrMode | Integer | 0x00–0x03 | The destination addressing mode for this primitive corresponding to the received MPDU. This value can take one of the following values: 0x00 = no address (addressing fields omitted). 0x01 = no address field (broadcast only mode with no address fields present). 0x02 = 16-bit short device address. 0x03 = 64-bit extended device address. |

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

148	Copyright © 20xx IEEE. All rights reserved.

| Table 46—MCPS -DATA.indic atio n parame te rs | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstOWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the entity to which the MSDU is being transferred. |
| DstAddr | Device address | As specified by the DstAddrMode parameter | The individual device address of the entity to which the MSDU is being transferred. |
| MsduLength | Integer | aMaxMacPayloadSize | The number of octets contained in the MSDU being indicated by the MAC sublayer entity. |
| Msdu | Set of octets | — | The set of octets forming the MSDU being indicated by the MAC sublayer entity. |
| MpduLinkQuality | Integer | 0x00–0xff | WQI value measured during reception of the MPDU. Lower values represent lower WQI (see 5.4.23.2) |
| DSN | Integer | 0x00–0xff | The DSN of the received data frame. |
| Timestamp | Integer | 0x000000–0xffffff | Optional. The time, in optical clocks, at which the data were received (see 5.1.7.1). The boundary is described by macTimeStampOffset (as defined in Table 91). The time stamp is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the received data frame (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| DataRate | Enumeration | 6-bit field | The data rate of the PHY frame to be transmitted by the PHY entity as shown in Table 115. |
| BurstMode | Boolean | TRUE or FALSE | The BurstMode bit shall be set TRUE if the burst mode is being used (as discussed in 8.6.1); otherwise, the BurstMode bit shall be set FALSE. |
| ColorReceived | Boolean | TRUE or FALSE | ColorReceived shall be set as TRUE, if CVD frame is sent when data frame is successfully received. |
| ColorNotReceived | Boolean | TRUE or FALSE | ColorNotReceived shall be set as TRUE, if CVD frame is sent when data frame is not received. |

Copyright © 20xx IEEE. All rights reserved.	149

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.2.3.1 When generated

2

3	The MCPS-DATA.indication primitive is generated by the MAC sublayer and issued to the SSCS on receipt

4	of a data frame at the local MAC sublayer entity that passes the appropriate message filtering operations as

5	described in 5.1.9.2.

6

7	6.2.3.2 Appropriate usage

8

9	On receipt of the MCPS-DATA.indication primitive, the SSCS is notified of the arrival of data at the device.

10

11	6.2.4 MCPS-PURGE.request

12

13	The MCPS-PURGE.request primitive allows the next higher layer to purge an MSDU from the transaction

14	queue.

15

16	This primitive is optional for a device.

17

18	The semantics of the MCPS-PURGE.request primitive are as follows:

19

20	MCPS -P URGE.re quest	(

21	Ms duHa ndle

22	)

23

24	Table 47 specifies the parameters for the MCPS-PURGE.request primitive.

25

26

27	Table 47—MCPS -P URGE.re que s t parame te rs

28

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MsduHandle | Integer | 0x00–0xff | The handle of the MSDU to be purged from the transaction queue. |

29

30

31

32

33

34

35	6.2.4.1 Appropriate usage

36

37	The MCPS-PURGE.request primitive is generated by the next higher layer whenever a MSDU is to be

38	purged from the transaction queue.

39

40	6.2.4.2 Effect on receipt

41

42	On receipt of the MCPS-PURGE.request primitive, the MAC sublayer attempts to find in its transaction

43	queue the MSDU indicated by the MsduHandle parameter. If an MSDU has left the transaction queue, the

44	handle will not be found, and the MSDU can no longer be purged. If an MSDU matching the given handle is

45	found, the MSDU is discarded from the transaction queue, and the MAC sublayer issues the MCPS-

46	PURGE.confirm primitive with a status of SUCCESS. If an MSDU matching the given handle is not found,

47	the MAC sublayer issues the MCPS-PURGE.confirm primitive with a status of INVALID_HANDLE.

48

49	6.2.5 MCPS-PURGE.confirm

50

51	The MCPS-PURGE.confirm primitive allows the MAC sublayer to notify the next higher layer of the

52	success of its request to purge an MSDU from the transaction queue.

53

54

150	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

This primitive is optional for a device.	1

2

The semantics of the MCPS-PURGE.confirm primitive are as follows:	3

4

MCPS -P URGE.confirm	(	5

Ms duHa ndle ,	6

s ta tus	7

)	8

9

Table 48 specifies the parameters for the MCPS-PURGE.confirm primitive.	10

11

12

Table 48—MCPS -PURGE.c o nfirm param e te rs	13

14

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MsduHandle | Integer | 0x00–0xff | The handle of the MSDU requested to be purge from the transaction queue. |
| status | Enumeration | SUCCESS or INVALID_HANDLE | The status of the request to be purged an MSDU from the transaction queue. |

15

16

17

18

19

20

21

22

23

6.2.5.1 When generated	24

25

The MCPS-PURGE.confirm primitive is generated by the MAC sublayer entity in response to an MCPS-	26

PURGE.request primitive. The MCPS-PURGE.confirm primitive returns a status of either SUCCESS,	27

indicating that the purge request was successful, or INVALID_HANDLE, indicating an error. The status	28

values are fully described in 6.2.5.2.	29

30

6.2.5.2 Appropriate usage	31

32

On receipt of the MCPS-PURGE.confirm primitive, the next higher layer is notified of the result of its	33

request to purge an MSDU from the transaction queue. If the purge request was successful, the status	34

parameter will be set to SUCCESS. Otherwise, the status parameter will indicate the error.	35

36

6.2.6 Data service message sequence chart	37

38

Figure 108 illustrates a sequence of messages necessary for a successful data transfer between two devices.	39

Figure 129 and Figure 130 also illustrate this, including the steps taken by the PHY.	40

41

6.2.7 Primitives for heterogeneous network	42

43

As described in section xx, ACK frames from type 2 devices to a coordinator shall be send trough RF link	44

first and then relayed to the coordinator through a wired backhaul link (power line, Ethernet, cable, etc.) in a	45

heterogeneous network, which is a different way than it is sent through the VLC link. Therefore, particular	46

primitives are needed so as to send the ACK frame successfully through the RF link and backhaul link.	47

48

MCPS-SAP ACK primitives defines how an ACK frame shall be transmitted in a heterogeneous network	49

when a type 2 device has an ACK frame that needs to be sent to the coordinator by the RF link and the	50

backhaul link. For ACK frames sent by type 1 devices and type 3 devices, MCPS-SAP DATA primitives	51

shall be used.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	151

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20	Figure 108—Message sequence chart describing the MAC data service

21

22	6.2.7.1 MCPS-ACK.indication

23

24	The MCPS-ACK.indication primitive allows the MAC sublayer of the device to provide an ACK frame to

25	its next higher layer.

26

27	The semantics of the MCPS-ACK.indication primitive are as follows:

28

29	MCPS-ACK.indication	(

30

31	DstAddr

32

33	SrcAddr

34

35	ACKFrame

36

37	)

38

39	Table 49 specifies the parameters for the MCPS-ACK.indication primitive.

40

41	6.2.7.1.1 Whe n g e ne rate d

42

43	When the MAC sublayer of the device generates an ACK frame that needs to be sent to the coordinator, it

44	shall generate a MCPS-ACK.indication primitive and issue the MCPS-ACK.indication primitive to its next

45	higher layer.

46

47	6.2.7.1.2 Effe c t o n re c e ipt

48

49	On receipt of the MCPS-ACK.indication primitive, the next higher layer of the device may generate a next

50	higher layer message based on the provided arguments and send it to the next higher layer of the coordinator

51	through the RF link and backhaul link. The ACK frame generated by the MAC sublayer shall be embedded

52	in the next higher layer message.

53

54

152	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 49—MCPS -ACK.indic atio n primitive	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddr | Integer | From 0x0000-0xffff, 0x0000 rep- resents without a short address assigned, 0xffff represents a broad- cast transmission. | The short address of the coordinator that the ACK frame is designated to. |
| SrcAddr | Integer | From 0x0000-0xffff, 0x0000 rep- resents without a short address assigned, 0xffff represents a broad- cast transmission. | The short address of the device that has generated the ACK frame. |
| ACKFrame | Set of octets |  | The set of octets forming the ACK frames to be transmitted. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

6.2.7.2 MCPS-ACK.response	17

18

The MCPS-ACK.response primitive is used to initiate a response to an MCPS-ACK.indication primitive.	19

The semantics of the MCPS-ACK.response primitive are as follows:	20

21

MCPS-ACK.response	(	22

23

Status,	24

25

)	26

27

28

Table 50—MCPS -ACK.re s po ns e	29

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, CHANNEL ACCESS FAILURE, NO ACKNOWLEDGEMENT, TRANSACTION TIME EXPIRE | The status of the uplink transmission request. |

30

31

32

33

34

35

36

37

38

6.2.7.2.1 Whe n g e ne rate d	39

40

The next higher layer of the device shall issue a MCPS-ACK.response primitive when it has finished the	41

uplink transmission requested by the MCPS of the device, or when the transaction time of the uplink	42

transmission has expired.	43

44

6.2.7.2.2 Effe c t o n re c e ipt	45

46

The MCPS of the device shall be notified of the result of the requested uplink transmission through the RF	47

link and backhaul link.	48

49

6.2.7.3 MCPS-ACK.request	50

51

The MCPS-ACK.request primitive allows the next higher layer of the coordinator to request its MAC	52

sublayer to process an ACK frame received through the backhaul link.	53

54

Copyright © 20xx IEEE. All rights reserved.	153

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The semantics of the MCPS-ACK.request primitive are as follows:

2

3	MCPS-ACK.request	(

4

5	DstAddr

6

7	SrcAddr

8

9	ACKFrame

10

11	)

12

13	Table 51 specifies the parameters for the MCPS-ACK. request primitive.

14

15

16	Table 51—MCPS -ACK. re que s t primitive

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddr | Integer | From 0x0000-0xffff, 0x0000 rep- resents without a short address assigned, 0xffff represents a broad- cast transmission. | The short address of the coordinator that the ACK frame is designated to. |
| SrcAddr | Integer | From 0x0000-0xffff, 0x0000 rep- resents without a short address assigned, 0xffff represents a broad- cast transmission. | The short address of the device that has generated the ACK frame. |
| ACKFrame | Set of octets |  | The set of octets forming the ACK frames to be transmitted. |

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32	6.2.7.3.1 Whe n g e ne rate d

33

34	The next higher layer of the coordinator shall generate a MCPS-ACK.request primitive when it has received

35	the next higher layer message through the backhaul link and issue the MCPS-ACK.request primitive to its

36	MAC sublayer.

37

38	6.2.7.3.2 Effe c t o n re c e ipt

39

40	On receipt of the MCPS-ACK.request primitive, the MAC sublayer of the coordinator shall process the

41	ACK frame provided in this primitive the same way as it was received through the VLC link.

42

43	6.2.7.4 MCPS-ACK.confirm

44

45	The MCPS-ACK.confirm primitive reports the process result of the received ACK frame.

46

47	The semantics of the MCPS-ACK.confirm primitive are as follows:

48

49	MCPS-ACK.confirm	(

50

51	Result,

52

53	)

54

154	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 52 specifies the parameters for the MCPS-ACK.confirm primitive.	1

2

3

Table 52—parame te rs fo r the MCPS -ACK.c o nfirm primitive	4

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Result | Enumer- ation | SUCCESS, INVALID FRAME | The process result of the received ACK frame. |

5

6

7

8

9

10

11

6.2.7.4.1 Whe n g e ne rate d	12

13

After the FCS re-calculation and the MAC frame header identification, the MCPS of the coordinator shall              14 provide the process result of the received frame to its next higher layer by issuing an MCPS-ACK.response              15 primitive.                                                                                                                                                                           16

17

6.2.7.4.2 Effe c t o n re c e ipt	18

19

The next higher layer of the coordinator is notified of the process result of the received ACK frame via the	20 backhaul link.	21

22

6.3 MAC management service	23

24

25

The MLME-SAP allows the transport of management commands between the next higher layer and the	26

MLME. Table 53 summarizes the primitives supported by the MLME through the MLME-SAP interface.	27

Primitives marked with a diamond () are optional for an RFD. Primitives marked with an asterisk (*) are	28

optional for  both  device types (i.e., RFD and  FFD). The  primitives are  discussed in  the  subclauses	29

referenced in this table.	30

31

Table 53—S ummary o f the primitive s ac c e s s e d thro ug h the MLME-S AP	32

33

| Name | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| MLME-ASSOCIATE | 6.3.1.1 | 6.3.1.2 | 6.3.1.3 | 6.3.1.4 |
| MLME-DISASSOCIATE | 6.3.2.1 | 6.3.2.2 |  | 6.3.2.3 |
| MLME-BEACON-NOTIFY |  | 6.3.3.1 |  |  |
| MLME-GET | 6.3.5.1 |  |  | 6.3.5.2 |
| MLME-GTS | 6.3.6.1 | 6.3.6.2 |  | 6.3.6.3 |
| MLME-RESET | 6.3.7.1 |  |  | 6.3.7.2 |
| MLME-RX-ENABLE | 6.3.8.1 |  |  | 6.3.8.2 |
| MLME-SCAN | 6.3.9.1 |  |  | 6.3.9.2 |
| MLME-COMM-STATUS |  | 6.3.11.1 |  |  |
| MLME-SET | 6.3.13.1 |  |  | 6.3.13.2 |
| MLME-START | 6.3.14.1 |  |  | 6.3.14.2 |
| MLME-SYNC | 6.3.15.1* |  |  |  |

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	155

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 53—Summary o f the primitives ac c e s s ed thro ug h the MLME-S AP  (continued)

| Name | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| MLME-SYNC-LOSS |  | 6.3.16.1 |  |  |
| MLME-POLL | 6.3.17.1 |  |  | 6.3.17.2 |

2

3

4

5

6

7

8

9

10	6.3.1 Association primitives

11

12	MLME-SAP association primitives define how a device becomes associated with a OWPAN.

13

14	All devices shall provide an interface for the request and confirm association primitives. The indication and

15	response association primitives are optional for a device.

16

17	6.3.1.1 MLME-ASSOCIATE.request

18

19	The MLME-ASSOCIATE.request primitive allows a device to request an association with a coordinator.

20

21	The semantics of the MLME-ASSOCIATE.request primitive are as follows:

22

23	MLME-AS S OCIATE.re que s t	(

24	CoordAddrMode ,

25	CoordOWPANId,

26	CoordAddre s s ,

27	Ca pa bilityInforma tion,

28	SecurityLe ve l,

29	Ke yIdMode ,

30	Ke yS ource ,

31	Ke yInde x,

32	ColorAs s oc

33	)

34

35

36	Table 54 specifies the parameters for the MLME-ASSOCIATE.request primitive.

37

38

39	Table 54—MLME-AS S OCIATE.re que s t parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddrMode | Integer | 0x02–0x03 | The coordinator addressing mode for this primitive and subsequent MPDU. This value can take one of the following values: 2 = 16-bit short address. 3 = 64-bit extended address. |
| CoordOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the coordinator as specified in the received beacon frame. |
| CoordAddress | Device address | As specified by the CoordAddrMode parameter | The address of the coordinator with which to associate. |

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

156	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 54—MLME-AS S OCIATE.re que s t parame te rs  (continued)	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CapabilityInformation | Bitmap | As defined in 5.4.23.1 | Specifies the operational capabilities of the associating device. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| ColorAssoc | Boolean | TRUE or FALSE | ColorAssoc shall be set as TRUE if the color CVD frame is to be transmitted after the association request command is sent. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

6.3.1.1.1 Appro priate us ag e	26

27

The MLME-ASSOCIATE.request primitive is generated by the next higher layer of an unassociated device              28 and issued to its MLME to request an association with a OWPAN through a coordinator. If the device wishes              29 to associate through a coordinator on a beacon-enabled OWPAN, the MLME may optionally track the              30 beacon of that coordinator prior to issuing this primitive.                                                                                                31

32

6.3.1.1.2 Effe c t o n re c e ipt	33

34

On receipt of the MLME-ASSOCIATE.request primitive, the MLME of an unassociated device first updates              35 the appropriate PHY and MAC PIB attributes and then generates an association request command as shown              36 in 5.4.1, as dictated by the association procedure described in 5.1.6.1.                                                                           37

38

The SecurityLevel parameter specifies the level of security to be applied to the association request command              39 frame. Typically, the association request command should not be implemented using security. However, if              40 the device requesting association shares a key with the coordinator, then security may be specified.                            41

42

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for              43 this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC              44 sublayer will  perform outgoing processing on  the  frame  based  on  the  CoordAddress, SecurityLevel,              45

KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during              46 outgoing frame processing, the MLME will discard the frame and issue the MLME-ASSOCIATE.confirm             47 primitive with the error status returned by outgoing frame processing.                                                                           48

49

If the association request command cannot be sent to the coordinator due to the unslotted random access              50 algorithm indicating a busy channel, the MLME will issue the MLME-ASSOCIATE.confirm primitive with              51 a status of CHANNEL_ACCESS_FAILURE.                                                                                                                 52

53

54

Copyright © 20xx IEEE. All rights reserved.	157

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	If  the  MLME  successfully  transmits  an  association  request  command,  the  MLME  will  expect  an

2	acknowledgment in return. If an acknowledgment is not received, the MLME will issue the MLME-

3	ASSOCIATE.confirm primitive with a status of NO_ACK (see 5.1.9.4).

4

5	If the MLME of an unassociated device successfully receives an acknowledgment to its association request

6	command, the MLME will wait for a response to the request (see 5.1.6.1). If the MLME of the device does

7	not receive a response, it will issue the MLME-ASSOCIATE.confirm primitive with a status of NO_DATA.

8

9	If the MLME of the device extracts an association response command frame from the coordinator, it will

10	then issue the MLME-ASSOCIATE.confirm primitive with a status equal to the contents of the Association

11	Status field in the association response command as shown in 5.4.2.3.

12

13	On  receipt  of  the  association  request  command,  the  MLME  of  the  coordinator  issues  the  MLME-

14	ASSOCIATE.indication primitive.

15

16	If any parameter in the MLME-ASSOCIATE.request primitive is either not supported or out of range, the

17	MLME will issue the MLME-ASSOCIATE.confirm primitive with a status of INVALID_PARAMETER.

18

19	6.3.1.2 MLME-ASSOCIATE.indication

20

21	The MLME-ASSOCIATE.indication primitive is used to indicate the reception of an association request

22	command.

23

24	The semantics of the MLME-ASSOCIATE.indication primitive are as follows:

25

26	MLME-AS S OCIATE.indica tion	(

27	De vice Addre s s ,

28	Ca pa bilityInforma tion,

29	SecurityLe ve l,

30	Ke yIdMode ,

31	Ke yS ource ,

32	Ke yInde x

33	)

34

35	Table 55 specifies the parameters for the MLME-ASSOCIATE.indication primitive.

36

37	Table 55—MLME-AS S OCIATE.indic atio n parame te rs

38

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64-bit IEEE address | The address of the device requesting association. |
| CapabilityInformation | Bitmap | Refer to 5.4.23.1 | The operational capabilities of the device requesting association. |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the received MAC command frame (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

158	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 55—MLME-AS S OCIATE.indic atio n parame te rs | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |

6.3.1.2.1 Whe n g e ne rate d	14

15

The MLME-ASSOCIATE.indication primitive is generated by the MLME of the coordinator and issued to	16 its next higher layer to indicate the reception of an association request command (as defined in 5.4.1).	17

18

6.3.1.2.2 Appro priate us ag e	19

20

When the next higher layer of a coordinator receives the MLME-ASSOCIATE.indication primitive, the              21 coordinator determines whether to accept or reject the unassociated device using an algorithm outside the              22 scope   of   this   standard.   The   next   higher   layer   of   the   coordinator   then   issues   the   MLME-              23

ASSOCIATE.response primitive to its MLME.	24

25

The association decision and the response should become available at the coordinator within a time of              26 macResponseWaitTime (see 5.1.6.1). After this time, the device requesting association attempts to extract the              27 association response command frame from the coordinator, using the method described in 5.1.9.3, in order              28 to determine whether the association was successful.                                                                                                      29

30

6.3.1.3 MLME-ASSOCIATE.response	31

32

The   MLME-ASSOCIATE.response   primitive   is   used   to   initiate   a   response   to   an   MLME-	33

ASSOCIATE.indication primitive.	34

35

The semantics of the MLME-ASSOCIATE.response primitive are as follows:	36

37

MLME-AS S OCIATE.re s pons e	(	38

De vice Addre s s ,	39

As s ocS hortAddre s s ,	40 s ta tus ,	41

Ca pa bilityNe gotia tionRe s pons e ,	42

SecurityLe ve l,	43

Ke yIdMode ,	44

Ke yS ource ,	45

Ke yInde x	46

)	47

48

49

Table 56 specifies the parameters for the MLME-ASSOCIATE.response primitive.	50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	159

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 56—MLME-AS S OCIATE.re s po ns e parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64-bit IEEE address | The address of the device requesting association. |
| AssocShortAddress | Integer | 0x0000–0xffff | The 16-bit short device address allocated by the coordinator on successful association. This parameter is set to 0xffff if the association was unsuccessful. |
| status | Enumeration | Refer to 5.4.2.3 | The status of the association attempt. |
| CapabilityNegotiatio nResponse | Integer | 00–11 | The coordinator indicates who will send color compensation information (same definitions and usage as the color stabilization scheme subfield in Table 39). |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33	6.3.1.3.1 Appro priate us ag e

34

35	The MLME-ASSOCIATE.response primitive is generated by the next higher layer of a coordinator and

36	issued to its MLME in order to respond to the MLME-ASSOCIATE.indication primitive.

37

38	6.3.1.3.2 Effe c t o n re c e ipt

39

40	When the MLME of a coordinator receives the MLME-ASSOCIATE.response primitive, it generates an

41	association response command as shown in 5.4.2. The command frame is sent to the device requesting

42	association using indirect transmission, i.e., the command frame is added to the list of pending transactions

43	stored on the coordinator and extracted at the discretion of the device concerned using the method described

44	in 5.1.9.3.

45

46	If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

47	this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

48	sublayer  will  perform  outgoing  processing  on  the  frame  based  the  DeviceAddress,  SecurityLevel,

49	KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during

50	outgoing  frame  processing,  the  MLME  will  discard  the  frame  and  issue  the  MLME-COMM-

51	STATUS.indication primitive with the error status returned by outgoing frame processing.

52

53	Upon  receipt  of  the  MLME-ASSOCIATE.response  primitive,  the  coordinator  attempts  to  add  the

54	information contained in the primitive to its list of pending transactions. If there is no capacity to store the

160	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

transaction, the MAC sublayer will discard the frame and issue the MLME-COMM-STATUS.indication             1 primitive with a status of TRANSACTION_OVERFLOW. If there is capacity to store the transaction, the              2 coordinator will add the information to the list. If the transaction is not handled within           3 macTransactionPersistenceTime, the transaction information will be discarded and the MAC sublayer will              4 issue the MLME-COMM-STATUS.indication primitive with a status of TRANSACTION_EXPIRED. The              5 transaction handling procedure is described in 5.1.8.                                                                                                       6

7

If the frame was successfully transmitted and an acknowledgment was received, if requested, the MAC	8 sublayer will issue the MLME-COMM-STATUS.indication primitive with a status of SUCCESS.	9

10

If any parameter in the MLME-ASSOCIATE.response primitive is not supported or is out of range, the	11

MAC   sublayer   will   issue   the   MLME-COMM-STATUS.indication   primitive   with   a   status   of	12

INVALID_PARAMETER.	13

14

6.3.1.4 MLME-ASSOCIATE.confirm	15

16

The MLME-ASSOCIATE.confirm primitive is used to inform the next higher layer of the initiating device	17 whether its request to associate was successful or unsuccessful.	18

19

The semantics of the MLME-ASSOCIATE.confirm primitive are as follows:	20

21

MLME-AS S OCIATE.confirm	(	22

As s ocS hortAddre s s ,	23 s ta tus ,	24

Ca pa bilityNe gotia tionRe s pons e ,	25

SecurityLe ve l,	26

Ke yIdMode ,	27

Ke yS ource ,	28

Ke yInde x	29

)	30

31

32

Table 57 specifies the parameters for the MLME-ASSOCIATE.confirm primitive.	33

34

35

Table 57—MLME-AS S OCIATE.c o nfirm parame te rs	36

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| AssocShortAddress | Integer | 0x0000–0xffff | The short device address allocated by the coordinator on successful association. This parameter will be equal to 0xffff if the association attempt was unsuccessful. |

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	161

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 57—MLME-AS S OCIATE.c o nfirm parame te rs  (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | The value of the Status field of the association response command (as defined in 5.4.2.3), SUCCESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, IMPROPER_KEY_TYPE, IMPROPER_SECURITY_LEVEL, SECURITY_ERROR, UNAVAILABLE_KEY, UNSUPPORTED_LEGACY, UNSUPPORTED_SECURITY INVALID_PARAMETER | The status of the association attempt. |
| CapabilityNegotiati onResponse | Integer | 00–11 | Coordinator indicates who will send (see Table 39). |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was generated following failed outgoing processing of an association request command: The security level to be used (as defined in Table 96 in 7.4.2.1). If the primitive was generated following receipt of an association response command: The security level purportedly used by the received frame (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was generated following failed outgoing processing of an association request command: The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. If the primitive was generated following receipt of an association response command: The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

162	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 57—MLME-AS S OCIATE.c o nfirm parame te rs | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | If the primitive was generated following failed outgoing processing of an association request command: The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. If the primitive was generated following receipt of an association response command: The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was generated following failed outgoing processing of an association request command: The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. If the primitive was generated following receipt of an association response command: The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |

6.3.1.4.1 Whe n g e ne rate d                                                                                                                                           38

39

The MLME-ASSOCIATE.confirm primitive is generated by the initiating MLME and issued to its next              40 higher layer in response to an MLME-ASSOCIATE.request primitive. If the request was successful, the              41 status parameter will indicate a successful association, as contained in the Status field of the association              42 response command. Otherwise, the status parameter indicates either an error code from the received              43 association response command or the appropriate error code from Table 57. The status values are fully              44 described in 6.3.1.1.2 and subclauses referenced by 6.3.1.1.2.                                                                                        45

46

6.3.1.4.2 Appro priate us ag e                                                                                                                                       47

48

On receipt of the MLME-ASSOCIATE.confirm primitive, the next higher layer of the initiating device is              49 notified of the result of its request to associate with a coordinator. If the association attempt was successful,              50 the status parameter will indicate a successful association, as contained in the Status field of the association              51 response command, and the device will be provided with a 16-bit short address as specified in Table 4 in              52

5.1.6.1. If the association attempt was unsuccessful, the address will be equal to 0xffff, and the status              53 parameter will indicate the error.                                                                                                                                      54

Copyright © 20xx IEEE. All rights reserved.	163

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.3.1.5 Association-message sequence charts

2

3	Figure 109 illustrates a sequence of messages that may be used by a device that is not tracking the beacon of

4	the coordinator, specified in 5.1.9.3, to successfully associate with a OWPAN. Figure 125 and Figure 126,

5	and described in 6.6, illustrate this same scenario, including steps taken by the PHY, for a device associating

6	with a coordinator and for a coordinator allowing association by a device, respectively.

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28	Figure 109—Message sequence chart for association

29

30

31	6.3.2 Disassociation primitives

32

33	The MLME-SAP disassociation primitives define how a device can disassociate from a OWPAN.

34

35	All devices shall provide an interface for these disassociation primitives.

36

37	6.3.2.1 MLME-DISASSOCIATE.request

38

39	The MLME-DISASSOCIATE.request primitive is used by an associated device to notify the coordinator of

40	its intent to leave the OWPAN. It is also used by the coordinator to instruct an associated device to leave the

41	OWPAN.

42

43

44

45

46

47

48

49

50

51

52

53

54

164	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The semantics of the MLME-DISASSOCIATE.request primitive are as follows:	1

2

MLME-DIS AS S OCIATE.re que s t	(	3

De vice AddrMode ,	4

De vice OWPANId,	5

De vice Addre s s ,	6

Disa ss ocia te Re a s on,	7

TxIndire ct,	8

SecurityLe ve l,	9

Ke yIdMode ,	10

Ke yS ource ,	11

Ke yInde x,	12

ColorDis As s oc	13

)	14

15

Table 58 specifies the parameters for the MLME-DISASSOCIATE.request primitive.	16

17

18

Table 58—MLME-DIS ASS OCIATE.re ques t parame te rs	19

20

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddrMode | Integer | 0x02–0x03 | The addressing mode of the device to which to send the disassociation notification command. |
| DeviceOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the device to which to send the disassociation notification command. |
| DeviceAddress | Device address | As specified by the DeviceAddrMode parameter. | The address of the device to which to send the disassociation notification command. |
| DisassociateReason | Integer | 0x00–0xff | The reason for the disassociation (as defined in 5.4.4.2). |
| TxIndirect | Boolean | TRUE or FALSE | TRUE if the disassociation notification com- mand is to be sent indirectly. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| ColorDisAssoc | Boolean | TRUE or FALSE | ColorDisAssoc shall be set as TRUE if the color CVD frame is to be transmitted after the disassociation notification command is sent. |

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	165

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.3.2.1.1 Appro priate us ag e

2

3	The MLME-DISASSOCIATE.request primitive is generated by the next higher layer of an associated device

4	and issued to its MLME to request disassociation from the OWPAN. It is also generated by the next higher

5	layer of the coordinator and issued to its MLME to instruct an associated device to leave the OWPAN.

6

7	6.3.2.1.2 Effe c t o n re c e ipt

8

9	On receipt of the MLME-DISASSOCIATE.request primitive, the MLME compares the DeviceOWPANId

10	parameter with macOWPANId. If the DeviceOWPANId parameter is not equal to macOWPANId, the MLME

11	issues the MLME-DISASSOCIATE.confirm primitive with a status of INVALID_PARAMETER. If the

12	DeviceOWPANId parameter is equal to macOWPANId, the MLME evaluates the primitive address fields.

13

14	If  the  DeviceAddrMode  parameter  is  equal  to  0x02  and  the  DeviceAddress  parameter  is  equal  to

15	macCoordShortAddress or if the DeviceAddrMode parameter is equal to 0x03 and the DeviceAddress

16	parameter is equal to macCoordExtendedAddress, the TxIndirect parameter is ignored, and the MLME sends

17	a disassociation notification command (see 5.4.3) to its coordinator in the CAP for a beacon-enabled

18	OWPAN or immediately for a nonbeacon-enabled OWPAN.

19

20	If the DeviceAddrMode parameter is equal to 0x02 and the DeviceAddress parameter is not equal to

21	macCoordShortAddress or if the DeviceAddrMode parameter is equal to 0x03 and the DeviceAddress

22	parameter is not equal to macCoordExtendedAddress, and if this primitive was received by the MLME of a

23	coordinator with the TxIndirect parameter set to TRUE, the disassociation notification command will be

24	sent using indirect transmission, i.e., the command frame is added to the list of pending transactions stored

25	on the coordinator and extracted at the discretion of the device concerned using the method described in

26	5.1.9.3.

27

28	If the DeviceAddrMode parameter is equal to 0x02 and the DeviceAddress parameter is not equal to

29	macCoordShortAddress or if the DeviceAddrMode parameter is equal to 0x03 and the DeviceAddress

30	parameter is not equal to macCoordExtendedAddress, and if this primitive was received by the MLME of a

31	coordinator with the TxIndirect parameter set to FALSE, the MLME sends a disassociation notification

32	command to the device in the CAP for a beacon-enabled OWPAN or immediately for a nonbeacon-enabled

33	OWPAN.

34

35	Otherwise,   the   MLME   issues   the   MLME-DISASSOCIATE.confirm  primitive   with   a   status   of

36	INVALID_PARAMETER and does not generate a disassociation notification command.

37

38	If the disassociation notification command is to be sent using indirect transmission and there is no capacity

39	to store the transaction, the MLME will discard the frame and issue the MLME-DISASSOCIATE.confirm

40	primitive with a status of TRANSACTION_OVERFLOW. If there is capacity to store the transaction, the

41	coordinator will add the information to the list. If the transaction is not handled within macTransaction-

42	PersistenceTime,  the  transaction  information  will  be  discarded,  and  the  MLME  will  issue  the

43	MLME-DISASSOCIATE.confirm with a status of TRANSACTION_EXPIRED. The transaction handling

44	procedure is described in 5.1.8.

45

46	If the disassociation notification command cannot be sent due to an unslotted random access algorithm

47	failure and this primitive was received either by the MLME of a coordinator with the TxIndirect parameter

48	set to FALSE or by the MLME of a device, the MLME will issue the MLME-DISASSOCIATE.confirm

49	primitive with a status of CHANNEL_ACCESS_FAILURE.

50

51	If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

52	this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

53	sublayer will perform outgoing processing on the frame based on the DeviceAddress, SecurityLevel,

54	KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during

166	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

outgoing	frame	processing,	the	MLME	will	discard	the	frame	and	issue	the	MLME-	1

DISASSOCIATE.confirm primitive with the error status returned by outgoing frame processing.	2

3

If the MLME successfully transmits a disassociation notification command, the MLME will expect an              4 acknowledgment in return. If an acknowledgment is not received and this primitive was received either by              5 the MLME of a coordinator with the TxIndirect parameter set to FALSE or by the MLME of a device, the              6

MLME will issue the MLME-DISASSOCIATE.confirm primitive with a status of NO_ACK (see 5.1.9.4).	7

8

If   the   MLME   successfully   transmits   a   disassociation   notification   command   and   receives   an	9 acknowledgment in return, the MLME will issue the MLME-DISASSOCIATE.confirm primitive with a	10 status of SUCCESS.	11

12

On receipt of the disassociation notification command, the MLME of the recipient issues the MLME-	13

DISASSOCIATE.indication primitive.	14

15

If any parameter in the MLME-DISASSOCIATE.request primitive is not supported or is out of range, the	16

MLME	will	issue	the	MLME-DISASSOCIATE.confirm	primitive	with	a	status	of	17

INVALID_PARAMETER.	18

19

6.3.2.2 MLME-DISASSOCIATE.indication	20

21

The MLME-DISASSOCIATE.indication primitive is used to indicate the reception of a disassociation	22 notification command.	23

24

The semantics of the MLME-DISASSOCIATE.indication primitive are as follows:	25

26

MLME-DIS AS S OCIATE.indica tion	(	27

De vice Addre s s ,	28

Disa ss ocia te Re a s on,	29

SecurityLe ve l,	30

Ke yIdMode ,	31

Ke yS ource ,	32

Ke yInde x	33

)	34

35

Table 59 specifies the parameters for the MLME-DISASSOCIATE.indication primitive.	36

37

6.3.2.2.1 Whe n g e ne rate d	38

39

The MLME-DISASSOCIATE.indication primitive is generated by the MLME and issued to its next higher	40 layer on receipt of a disassociation notification command.	41

42

6.3.2.2.2 Appro priate us ag e	43

44

The next higher layer is notified of the reason for the disassociation.	45

46

6.3.2.3 MLME-DISASSOCIATE.confirm	47

48

The MLME-DISASSOCIATE.confirm primitive reports the results of an MLME-DISASSOCIATE.request	49 primitive.	50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	167

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 59—MLME-DIS AS S OCIATE.indic atio n parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64-bit IEEE address | The address of the device requesting disassociation. |
| DisassociateReason | Integer | 0x00–0xff | The reason for the disassociation (as defined in 5.4.4.2). |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the received MAC command frame (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28	The semantics of the MLME-DISASSOCIATE.confirm primitive are as follows:

29

30	MLME-DIS AS S OCIATE.confirm	(

31	s ta tus ,

32	De vice AddrMode ,

33	De vice OWPANId,

34	De vice Addre s s

35	)

36

37	Table 60 specifies the parameters for the MLME-DISASSOCIATE.confirm primitive.

38

39	6.3.2.3.1 Whe n g e ne rate d

40

41	The MLME-DISASSOCIATE.confirm primitive is generated by the initiating MLME and issued to its next

42	higher layer in response to an MLME-DISASSOCIATE.request primitive. This primitive returns a status of

43	either SUCCESS, indicating that the disassociation request was successful, or the appropriate error code.

44	The status values are fully described in 6.3.2.1.2 and subclauses referenced by 6.3.2.1.2.

45

46	6.3.2.3.2 Appro priate us ag e

47

48	On receipt of the MLME-DISASSOCIATE.confirm primitive, the next higher layer of the initiating device

49	is notified of the result of the disassociation attempt. If the disassociation attempt was successful, the status

50	parameter will be set to SUCCESS. Otherwise, the status parameter indicates the error.

51

52

53

54

168	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 60—MLME-DIS AS S OCIATE.c o nfirm param e te rs	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, TRANSACTION_OVERFLOW, TRANSACTION_EXPIRED, NO_ACK, CHANNEL_ACCESS_FAILURE, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY, INVALID_PARAMETER | The status of the disassociation attempt. |
| DeviceAddrMode | Integer | 0x02–0x03 | The addressing mode of the device that has either requested disassociation or been instructed to disassociate by its coordinator. |
| DeviceOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the device that has either requested disassociation or been instructed to disassociate by its coordinator. |
| DeviceAddress | Device address | As specified by the DeviceAddrMode parameter. | The address of the device that has either requested disassociation or been instructed to disassociate by its coordinator. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

6.3.2.4 Disassociation-message sequence charts	28

29

The request to disassociate may originate either from a device or from the coordinator through which the	30

device has associated. Figure 110 illustrates the sequence of messages necessary for a device to successfully	31

disassociate itself from the OWPAN.	32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

Figure 110—Message sequence chart for disassociation initiated by a device	49

50

51

Figure 111 illustrates the sequence necessary for a coordinator in a beacon-enabled OWPAN to successfully	52

disassociate a device from its OWPAN using indirect transmission.	53

54

Copyright © 20xx IEEE. All rights reserved.	169

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29	Figure 111—Message sequence chart for disassociation initiated by a coordinator,

30	using indirect transmission, in a beacon-enabled OWPAN

31

32

33	6.3.3 Beacon notification primitive

34

35	The MLME-SAP beacon notification primitive defines how a device may be notified when a beacon is

36	received during normal operating conditions.

37

38	All devices shall provide an interface for the beacon notification primitive.

39

40	6.3.3.1 MLME-BEACON-NOTIFY.indication

41

42	The MLME-BEACON-NOTIFY.indication primitive is used to send parameters contained within a beacon

43	frame received by the MAC sublayer to the next higher layer. The primitive also sends a measure of the

44	WQI and the time the beacon frame was received.

45

46

47

48

49

50

51

52

53

54

170	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The semantics of the MLME-BEACON-NOTIFY.indication primitive are as follows:	1

2

MLME-BEACON-NOTIFY.indica tion	(	3

BS N,	4

OWPANDe s criptor,	5

P e ndAddrS pe c,	6

AddrLis t,                                                                                                  7 s duLe ngth,                                                                                              8 sdu                                                                                                           9

)	10

11

Table 61 specifies the parameters for the MLME-BEACON-NOTIFY.indication primitive.	12

13

14

Table 61—MLME-BEACON-NOTIFY.indic atio n parame te rs	15

16

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BSN | Integer | 0x00–0xff | The beacon sequence number. |
| OWPANDescript or | OWPANDescript or value | Refer to Table 62 | The OWPANDescriptor for the received beacon. |
| PendAddrSpec | Bitmap | Refer to 5.2.2.1.7 | The beacon pending address specification. |
| AddrList | List of device addresses | — | The list of addresses of the devices for which the beacon source has data. |
| sduLength | Integer | 0 – aMaxBeaconPayloadLength | The number of octets contained in the beacon payload of the beacon frame received by the MAC sublayer. |
| sdu | Set of octets | — | The set of octets comprising the beacon payload to be transferred from the MAC sublayer entity to the next higher layer. |

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

Table 62 describes the elements of the OWPANDescriptor type.	36

37

38

Table 62—Ele me nts o f OWPAN de s c ripto r	39

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddrMode | Integer | 0x02–0x03 | The coordinator addressing mode corresponding to the received beacon frame. This value can take one of the following values: 2 = 16-bit short address. 3 = 64-bit extended address. |
| CoordOWPANId | Integer | 0x0000–0xffff | The OWPAN identifier of the coordinator as specified in the received beacon frame. |
| CoordAddress | Device address | As specified by the CoordAddrMode parameter | The address of the coordinator as specified in the received beacon frame. |
| SuperframeSpec | Bitmap | Refer to 5.2.2.1.2 | The superframe specification as specified in the received beacon frame. |

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	171

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 62—Ele me nts o f OWPAN de s c ripto r  (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| GTSPermit | Boolean | TRUE or FALSE | TRUE if the beacon is from the coordinator that is accepting GTS requests. |
| LinkQuality | Integer | 0x00–0xff | The WQI at which the network beacon was received. Lower values represent lower WQI (see 5.4.23.2). |
| TimeStamp | Integer | 0x000000–0xffffff | The time at which the beacon frame was received, in symbols. This value is equal to the timestamp taken when the beacon frame was received, as described in 5.1.7.1. This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |
| SecurityFailure | Enumer- ation | SUCCESS, COUNTER_ERROR, IMPROPER_KEY_TYPE, IMPROPER_SECURITY_LE VEL, SECURITY_ERROR, UNAVAILABLE_KEY, UNSUPPORTED_LEGACY, UNSUPPORTED_SECURITY | SUCCESS if there was no error in the security processing of the frame. One of the other status codes indicating an error in the security processing otherwise (see 7.2.3). |
| SecurityLevel | Integer | 0x00–0x07 | The security level purportedly used by the received beacon frame (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42	6.3.3.1.1 Whe n g e ne rate d

43

44	The MLME-BEACON-NOTIFY.indication primitive is generated by the MLME and issued to its next

45	higher layer upon receipt of a beacon frame either when macAutoRequest is set to FALSE or when the

46	beacon frame contains one or more octets of payload.

47

48	6.3.3.1.2 Appro priate us ag e

49

50	On receipt of the MLME-BEACON-NOTIFY.indication primitive, the next higher layer is notified of the

51	arrival of a beacon frame at the MAC sublayer.

52

53

54

172	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.4 Additional beacon primitives	1

2

MLME-SAP additional beacon primitive defines how an unassociated device request coordinators to send	3 additional beacons before initiating an association with a OWPAN.	4

5

6.3.4.1 MLME-AdditionalBeacon.requet	6

7

The MLME-AdditionalBeacon.request primitive allows an unassociated device to generate and transmit an	8 additional beacon frame request command.	9

10

The semantics of the MLME-AdditionalBeacon.request primitive are as follows:	11

12

MLME-AdditionalBeacon.request (	13

14

Reason	15

16

)	17

18

19

Table 63—MLME-Additio nalBe ac o n.re que s t primitive	20

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Reason | Integer | 0x00 - 0xff | The reason for the additional beacon request (as defined in Table 33). |

21

22

23

24

25

26

27

6.3.4.1.1 Whe n g e ne rate d	28

29

The MLME-AdditionalBeacon.request primitive is generated by the next higher layer of an unassociated              30 device when the next higher layer of the unassociated device has obtained the result of a requested passive              31 scan which indicates no beacon frames has been detected.                                                                                              32

33

6.3.4.1.2 Effe c t o n re c e ipt	34

35

The MLME of the unassociated device shall generate and transmit an additional beacon request command,	36 then it shall set a timer and enable its receiver to receive beacon frames.	37

38

6.3.4.2 MLME-AdditionalBeacon.indication	39

40

The MLME-AdditionalBeacon.indication primitive is used to indicate the reception of an additional beacon	41 request command.	42

43

The semantics of the MLME-AdditionalBeacon.indication primitive are as follows:	44

45

MLME-AdditionalBeacon.indication (	46

47

Reason	48

49

DeviceAddr	50

51

)	52

53

Table 64 specifies the parameters for the MLME-AdditionalBeacon.indication primitive.	54

Copyright © 20xx IEEE. All rights reserved.	173

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 64—MLME-Additio nalBe ac o n.indic atio n primitive

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Reason | Integer | 0x00 - 0xff | The reason for the additional beacon request (as defined in Table 33). |
| DeviceAddr | Device address | 64 bit extended address | The address of the device sending this additional beacon request. |

2

3

4

5

6

7

8

9

10

11

12	6.3.4.2.1 Whe n g e ne rate d

13

14	The MLME-AdditionalBeacon.indication primitive is generated by the MLME of a coordinator and issued

15	to its next higher layer to indicate the reception of an additional beacon request command received by the

16	PHY.

17

18	6.3.4.2.2 Effe c t o n re c e ipt

19

20	On receipt of the MLME-AdditionalBeacon.indication primitive, the next higher layer of the coordinator is

21	notified of the additional beacon request command sent by an unassociated device. The next higher layer

22	may then allocate a GTS for additional beacon transmission.

23

24	6.3.5 Primitives for reading PIB attributes

25

26	The MLME-SAP get primitives define how to read values from the PIB.

27

28	All devices shall provide an interface for these get primitives.

29

30	6.3.5.1 MLME-GET.request

31

32	The MLME-GET.request primitive requests information about a given PIB attribute.

33

34	The semantics of the MLME-GET.request primitive are as follows:

35

36	MLME -GET.re que s t	(

37	P IBAttribute ,

38	P IBAttribute Inde x

39	)

40

41	Table 65 specifies the parameters for the MLME-GET.request primitive.

42

43

44	Table 65—MLME-GET.re que s t parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBAttribute | Integer | Refer to Table 91 | The identifier of the PIB attribute to read. |
| PIBAttributeIndex | Integer | Attribute specific; as defined in Table 91 | The index within the table of the specified PIB attribute to read. This parameter is valid only for MAC PIB attributes that are tables; it is ignored when accessing PHY PIB attributes. |

45

46

47

48

49

50

51

52

53

54

174	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.5.1.1 Appro priate us ag e	1

2

The MLME-GET.request primitive is generated by the next higher layer and issued to its MLME to obtain	3 information from the PIB.	4

5

6.3.5.1.2 Effe c t o n re c e ipt	6

7

On receipt of the MLME-GET.request primitive, the MLME checks to see if the PIB attribute is a MAC PIB	8 attribute or PHY PIB attribute. If the requested attribute is a MAC attribute, the MLME attempts to retrieve	9 the requested MAC PIB attribute from its database. If the identifier of the PIB attribute is not found in the	10 database,	the	MLME	will	issue	the	MLME-GET.confirm	primitive	with	a	status	of	11

UNSUPPORTED_ATTRIBUTE. If the PIBAttributeIndex parameter specifies an index for a table that is out              12 of range, the MLME will issue the MLME-GET.confirm primitive with a status of INVALID_INDEX. If the              13 requested MAC PIB attribute is successfully retrieved, the MLME will issue the MLME-GET.confirm              14 primitive with a status of SUCCESS.                                                                                                                               15

16

If the requested attribute is a PHY PIB attribute, the request is passed to the PHY by issuing the PLME-	17

GET.request primitive. Once the MLME receives the PLME-GET.confirm primitive, it will translate the	18 received status value because the status values used by the PHY are not the same as those used by the	19

MLME (e.g., the status values for SUCCESS are 0x00 and 0x07 in the MAC and PHY enumeration tables,              20 respectively). Following the translation, the MLME will issue the MLME-GET.confirm primitive to the next              21 higher  layer  with  the  status  parameter  resulting  from  the  translation  and  the  PIBAttribute  and              22

PIBAttributeValue parameters equal to those returned by the PLME primitive.	23

24

6.3.5.2 MLME-GET.confirm	25

26

The MLME-GET.confirm primitive reports the results of an information request from the PIB.	27

28

The semantics of the MLME-GET.confirm primitive are as follows:	29

30

MLME-GET.confirm	(	31 s ta tus ,	32

P IBAttribute ,	33

P IBAttribute Inde x,	34

P IBAttribute Va lue	35

)	36

37

Table 66 specifies the parameters for the MLME-GET.confirm primitive.	38

39

6.3.5.2.1 Whe n g e ne rate d	40

41

The MLME-GET.confirm primitive is generated by the MLME and issued to its next higher layer in              42 response to an MLME-GET.request primitive. This primitive returns a status of either SUCCESS, indicating              43 that the request to read a PIB attribute was successful, or an error code of UNSUPPORTED_ATTRIBUTE.             44

When an error code of UNSUPPORTED_ATTRIBUTE is returned, the PIBAttribute value parameter will	45 be set to length zero. The status values are fully described in 6.3.5.1.2.	46

47

6.3.5.2.2 Appro priate us ag e	48

49

On receipt of the MLME-GET.confirm primitive, the next higher layer is notified of the results of its request	50 to read a PIB attribute. If the request to read a PIB attribute was successful, the status parameter will be set to	51

SUCCESS. Otherwise, the status parameter indicates the error.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	175

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 66—MLME-GET.c o nfirm parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, UNSUPPORTED_ATTRIBUTE or INVALID_INDEX | The result of the request for PIB attribute information. |
| PIBAttribute | Integer | Refer to Table 91 | The identifier of the PIB attribute that was read. |
| PIBAttributeIndex | Integer | Attribute specific; as defined in Table 91 | The index within the table or array of the specified PIB attribute to read. This parameter is valid only for MAC PIB attributes that are tables or arrays; it is ignored when accessing PHY PIB attributes. |
| PIBAttributeValue | Various | Attribute specific; as defined in Table 91 | The value of the indicated PIB attribute that was read. This parameter has zero length when the status parameter is set to UNSUPPORTED_ATTRIBUTE. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23	6.3.6 GTS management primitives

24

25	The MLME-SAP GTS management primitives define how GTSs are requested and maintained. A device

26	wishing to use these primitives and GTSs in general will already be tracking the beacons of its coordinator.

27

28	These GTS management primitives are optional.

29

30	6.3.6.1 MLME-GTS.request

31

32	The MLME-GTS.request primitive allows a device to send a request to the coordinator to allocate a new

33	GTS or to deallocate an existing GTS. This primitive is also used by the coordinator to initiate a GTS

34	deallocation.

35

36	The semantics of the MLME-GTS.request primitive are as follows:

37

38	MLME-GTS .re que s t	(

39	GTS Cha ra cte ris tics ,

40	SecurityLe ve l,

41	Ke yIdMode ,

42	Ke yS ource ,

43	Ke yInde x

44	)

45

46	Table 67 specifies the parameters for the MLME-GTS.request primitive.

47

48	6.3.6.1.1 Appro priate us ag e

49

50	The MLME-GTS.request primitive is generated by the next higher layer of a device and issued to its MLME

51	to request the allocation of a new GTS or to request the deallocation of an existing GTS. It is also generated

52	by the next higher layer of the coordinator and issued to its MLME to request the deallocation of an existing

53	GTS.

54

176	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 67—MLME-GTS .re que s t parame te rs	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| GTSCharacteristics | GTS characteristics | Refer to 5.4.18.2 | The characteristics of the GTS request, including whether the request is for the allocation of a new GTS or the deallocation of an existing GTS. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

6.3.6.1.2 Effe c t o n re c e ipt	21

22

On receipt of the MLME-GTS.request primitive by a device, the MLME of a device attempts to generate a	23

GTS  request  command, specified in  5.4.18, with  the  information contained in  this  primitive and,  if	24

successful, sends it to the coordinator.	25

26

If macShortAddress is equal to 0xfffe or 0xffff, the device is not permitted to request a GTS. In this case, the	27

MLME issues the MLME-GTS.confirm primitive containing a status of NO_SHORT_ADDRESS.	28

29

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for	30

this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC	31

sublayer will perform outgoing processing on the frame based on macCoordExtendedAddress and the	32

SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs	33

during outgoing frame processing, the MLME will discard the frame and issue the MLME-GTS.confirm	34

primitive with the error status returned by outgoing frame processing.	35

36

If the GTS request command cannot be sent due to an unslotted random access algorithm failure, the MLME	37

will issue the MLME-GTS.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.	38

39

If the MLME successfully transmits a GTS request command, the MLME will expect an acknowledgment in	40

return. If an acknowledgment is not received, the MLME will issue the MLME-GTS.confirm primitive with	41

a status of NO_ACK (see 5.1.9.4).	42

43

If a GTS is being allocated (see 5.1.10.2) and the request has been acknowledged, the device will wait for a	44

confirmation via a GTS descriptor specified in a beacon frame from its coordinator. If the MLME of the	45

coordinator can allocate the requested GTS, it will issue the MLME-GTS.indication primitive with the	46

characteristics of the allocated GTS and generate a GTS descriptor with the characteristics of the allocated	47

GTS and the 16-bit short address of the requesting device. If the MLME of the coordinator cannot allocate	48

the requested GTS, it will generate a GTS descriptor with a start slot of zero and the short address of the	49

requesting device.	50

51

If the device receives a beacon frame from its coordinator with a GTS descriptor containing a 16-bit short	52

address that matches macShortAddress, the device will process the descriptor. If no descriptor for that device	53

is received, the MLME will issue the MLME-GTS.confirm primitive with a status of NO_DATA.	54

Copyright © 20xx IEEE. All rights reserved.	177

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	If a descriptor is received that matches the characteristics requested (indicating that the coordinator has

2	approved the GTS allocation request), the  MLME of  the device will issue the  MLME-GTS.confirm

3	primitive with a status of SUCCESS and a GTSCharacteristics parameter with a characteristics type equal to

4	one, indicating a GTS allocation.

5

6	If the descriptor is received with a start slot of zero (indicating that the coordinator has denied the GTS

7	allocation request), the device requesting the GTS issues the MLME-GTS.confirm primitive with a status of

8	DENIED, indicating that the GTSCharacteristics parameter is to be ignored.

9

10	If a GTS is being deallocated (see 5.1.10.4) at the request of a device and the request has been acknowledged

11	by the coordinator, the device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and a

12	GTSCharacteristics parameter with a characteristics type equal to zero, indicating a GTS deallocation. On

13	receipt of a GTS request command with a request type indicating a GTS deallocation, the coordinator will

14	acknowledge the frame and deallocates the GTS. The MLME of the coordinator will then issue the MLME-

15	GTS.indication primitive with the appropriate GTS characteristics. If the coordinator does not receive the

16	deallocation request, countermeasures can be applied by the coordinator to ensure consistency is maintained

17	as discussed in 5.1.10.6.

18

19	If the MLME of the coordinator receives an MLME-GTS.request primitive indicating deallocation, the

20	coordinator will deallocate the GTS and issue the MLME-GTS.confirm primitive with a status of SUCCESS

21	and a GTSCharacteristics parameter with a characteristics type equal to zero.

22

23	If the device receives a beacon frame from its coordinator with a GTS descriptor containing a short address

24	that matches macShortAddress and a start slot equal to zero, the device immediately stops using the GTS.

25	The MLME of the device then notifies the next higher layer of the deallocation by issuing the MLME-

26	GTS.indication  primitive  with  a  GTSCharacteristics  parameter  containing  the  characteristics  of  the

27	deallocated GTS.

28

29	If any parameter in the MLME-GTS.request primitive is not supported or is out of range, the MLME will

30	issue the MLME-GTS.confirm primitive with a status of INVALID_PARAMETER.

31

32	6.3.6.2 MLME-GTS.indication

33

34	The MLME-GTS.indication primitive indicates that a GTS has been allocated or that a previously allocated

35	GTS has been deallocated.

36

37	The semantics of the MLME-GTS.indication primitive are as follows:

38

39	MLME-GTS .indica tion	(

40	De vice Addre s s ,

41	GTS Cha ra cte ris tics ,

42	SecurityLe ve l,

43	Ke yIdMode ,

44	Ke yS ource ,

45	Ke yInde x

46	)

47

48	Table 68 specifies the parameters for the MLME-GTS.indication primitive.

49

50	6.3.6.2.1 Whe n g e ne rate d

51

52	The MLME-GTS.indication primitive is generated by the MLME of the coordinator to its next higher layer

53	whenever a GTS is allocated or deallocated following the reception of a GTS request command by the

54	MLME as discussed in 5.4.18. The MLME of the coordinator also generates this primitive when a GTS

178	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 68—MLME-GTS .indic atio n parame te rs	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | 0x0000–0xfffd | The 16-bit short address of the device that has been allocated or deallocated a GTS. |
| GTSCharacteristics | GTS characteristics | Refer to 5.4.18.2 | The characteristics of the GTS. |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was generated when a GTS deallocation is initiated by the coordinator itself, the security level to be used is set to 0x00. If the primitive was generated whenever a GTS is allocated or deallocated following the reception of a GTS request command: The security level purportedly used by the received MAC command frame (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was generated when a GTS deallocation is initiated by the coordinator itself, this parameter is ignored. If the primitive was generated whenever a GTS is allocated or deallocated following the reception of a GTS request command: The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | If the primitive was generated when a GTS deallocation is initiated by the coordinator itself, this parameter is ignored. If the primitive was generated whenever a GTS is allocated or deallocated following the reception of a GTS request command: The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was generated when a GTS deallocation is initiated by the coordinator itself, this parameter is ignored. If the primitive was generated whenever a GTS is allocated or deallocated following the reception of a GTS request command: The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	179

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	deallocation is initiated by the coordinator itself. The Characteristics Type field in the GTSCharacteristics

2	parameter will be equal to one if a GTS has been allocated or zero if a GTS has been deallocated.

3

4	This primitive is generated by the MLME of a device and issued to its next higher layer when the

5	coordinator  has  deallocated  one  of  its  GTSs.  In  this  case,  the  Characteristics  Type  field  of  the

6	GTSCharacteristics parameter is equal to zero.

7

8	6.3.6.2.2 Appro priate us ag e

9

10	On receipt of the MLME-GTS.indication primitive the next higher layer is notified of the allocation or de-

11	allocation of a GTS.

12

13	6.3.6.3 MLME-GTS.confirm

14

15	The MLME-GTS.confirm primitive reports the results of a request to allocate a new GTS or deallocate an

16	existing GTS.

17

18	The semantics of the MLME-GTS.confirm primitive are as follows:

19

20	MLME-GTS .confirm	(

21	GTS Cha ra cte ris tics ,

22	s ta tus

23	)

24

25	Table 69 specifies the parameters for the MLME-GTS.confirm primitive.

26

27

28	Table 69—MLME-GTS .c o nfirm parame te rs

29

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| GTSCharacteristics | GTS characteristics | Refer to 5.4.18.2 | The characteristics of the GTS. |
| status | Enumeration | SUCCESS, DENIED, NO_SHORT_ADDRESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY, INVALID_PARAMETER | The status of the GTS request. |

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46	6.3.6.3.1 Whe n g e ne rate d

47

48	The MLME-GTS.confirm primitive is generated by the MLME and issued to its next higher layer in

49	response to a previously issued MLME-GTS.request primitive.

50

51	If the request to allocate or deallocate a GTS was successful, this primitive will return a status of SUCCESS

52	and the Characteristics Type field of the GTSCharacteristics parameter will have the value of one or zero,

53	respectively. Otherwise, the status parameter will indicate the appropriate error code. The reasons for these

54	status values are fully described in 6.3.6.1.2 and subclauses referenced by 6.3.6.1.2.

180	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.6.3.2 Appro priate us ag e	1

2

On receipt of the MLME-GTS.confirm primitive the next higher layer is notified of the result of its request	3 to allocate or deallocate a GTS. If the request was successful, the status parameter will indicate a successful	4

GTS operation. Otherwise, the status parameter will indicate the error.	5

6

6.3.6.4 GTS management message sequence charts	7

8

Figure 112 and Figure 113 illustrate the sequence of messages necessary for successful GTS management.	9

The first depicts the message flow for the case in which the device initiates the GTS allocation. The second              10 depicts the message flow for the two cases for which a GTS deallocation occurs, first, by a device (a) and,              11 second, by the coordinator (b).                                                                                                                                          12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

Figure 112—Message sequence chart for GTS allocation initiated by a device	34

35

36

37

6.3.7 Primitives for resetting the MAC sublayer	38

39

MLME-SAP reset primitives specify how to reset the MAC sublayer to its default values.	40

41

All devices shall provide an interface for these reset primitives.	42

43

6.3.7.1 MLME-RESET.request	44

45

The MLME-RESET.request primitive allows the next higher layer to request that the MLME performs a	46 reset operation.	47

48

The semantics of the MLME-RESET.request primitive are as follows:	49

50

MLME-RES ET.re que s t	(	51

Se tDe faultP IB	52

)	53

54

Copyright © 20xx IEEE. All rights reserved.	181

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23	Figure 113—Message sequence chart for GTS deallocation initiated by a device (a) and

24	the PAN coordinator (b)

25

26	Table 70 specifies the parameter for the MLME-RESET.request primitive.

27

28

29	Table 70—MLME-RES ET.re que s t parame te r

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SetDefaultPIB | Boolean | TRUE or FALSE | If TRUE, the MAC sublayer is reset, and all MAC PIB attributes are set to their default values. If FALSE, the MAC sublayer is reset, but all MAC PIB attributes retain their values prior to the generation of the MLME-RESET.request primitive. |

30

31

32

33

34

35

36

37

38

39	6.3.7.1.1 Appro priate us ag e

40

41	The MLME-RESET.request primitive is generated by the next higher layer and issued to the MLME to

42	request a reset of the MAC sublayer to its initial conditions. The MLME-RESET.request primitive is issued

43	prior to the use of the MLME-START.request or the MLME-ASSOCIATE.request primitives.

44

45	6.3.7.1.2 Effe c t o n re c e ipt

46

47	On receipt of the MLME-RESET.request primitive, the MLME issues the PLME-SET-TRX-STATE.request

48	primitive with a state of FORCE_TRX_OFF. On receipt of the PLME-SET-TRX-STATE.confirm primitive,

49	the MAC sublayer is then set to its initial conditions, clearing all internal variables to their default values. If

50	the SetDefaultPIB parameter is set to TRUE, the MAC PIB attributes are set to their default values.

51

52	The MLME-RESET.confirm primitive with a status of SUCCESS is issued on completion.

53

54

182	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.7.2 MLME-RESET.confirm	1

2

The MLME-RESET.confirm primitive reports the results of the reset operation.	3

4

The semantics of the MLME-RESET.confirm primitive are as follows:	5

6

MLME-RES ET.confirm	(	7

s ta tus	8

)	9

10

Table 71 specifies the parameter for the MLME-RESET.confirm primitive.	11

12

13

Table 71—MLME-RES ET.c o nfirm parame te r	14

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS | The result of the reset operation. |

15

16

17

18

19

20

6.3.7.2.1 Whe n g e ne rate d	21

22

The MLME-RESET.confirm primitive is generated by the MLME and issued to its next higher layer in	23 response  to  an  MLME-RESET.request primitive  and  following  the  receipt  of  the  PLME-SET-TRX-	24

STATE.confirm primitive.	25

26

6.3.7.2.2 Appro priate us ag e	27

28

On receipt of the MLME-RESET.confirm primitive, the next higher layer is notified of its request to reset              29 the MAC sublayer. This primitive returns a status of SUCCESS indicating that the request to reset the MAC              30 sublayer was successful.                                                                                                                                                   31

32

6.3.8 Primitives for specifying the receiver enable time	33

34

MLME-SAP receiver state primitives define how a device can enable or disable the receiver at a given time.	35

36

These receiver state primitives are optional.	37

38

6.3.8.1 MLME-RX-ENABLE.request	39

40

The MLME-RX-ENABLE.request primitive allows the next higher layer to request that the receiver is either	41 enabled for a finite period of time or disabled.	42

43

The semantics of the MLME-RX-ENABLE.request primitive are as follows:	44

MLME-RX-ENABLE.re que s t	(	45

De fe rP e rmit,	46

RxOnTime ,	47

RxOnDura tion	48

)	49

50

51

52

Table 72 specifies the parameters for the MLME-RX-ENABLE.request primitive.	53

54

Copyright © 20xx IEEE. All rights reserved.	183

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 72—MLME-RX-ENABLE.re que s t parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeferPermit | Boolean | TRUE or FALSE | TRUE if the requested operation can be deferred until the next superframe if the requested time has already passed. FALSE if the requested operation is only to be attempted in the current superframe. This parameter is ignored for nonbeacon-enabled OWPANs. If the issuing device is the OWPAN coordinator, the term superframe refers to its own superframe. Otherwise, the term refers to the superframe of the coordinator through which the issuing device is associated. |
| RxOnTime | Integer | 0x000000–0xffffff | The number of optical clocks measured from the start of the superframe before the receiver is to be enabled or disabled. This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. This parameter is ignored for nonbeacon-enabled OWPANs. If the issuing device is the OWPAN coordinator, the term superframe refers to its own superframe. Otherwise, the term refers to the superframe of the coordinator through which the issuing device is associated. |
| RxOnDuration | Integer | 0x000000–0xffffff | The number of optical clocks for which the receiver is to be enabled. If this parameter is equal to 0x000000, the receiver is to be disabled. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32	6.3.8.1.1 Appro priate us ag e

33

34	The MLME-RX-ENABLE.request primitive is generated by the next higher layer and issued to the MLME

35	to enable the receiver for a fixed duration, at a time relative to the start of the current or next superframe on

36	a beacon-enabled OWPAN or immediately on a nonbeacon-enabled OWPAN. This primitive may also be

37	generated to cancel a previously generated request to enable the receiver. The receiver is enabled or disabled

38	exactly once per primitive request.

39

40	6.3.8.1.2 Effe c t o n re c e ipt

41

42	The MLME will treat the request to enable or disable the receiver as secondary to other responsibilities of

43	the device (e.g., GTSs, coordinator beacon tracking, beacon transmissions). When the primitive is issued to

44	enable the receiver, the device will enable its receiver until either the device has a conflicting responsibility

45	or the time specified by RxOnDuration has expired. In the case of a conflicting responsibility, the device will

46	interrupt the receive operation. After the completion of the interrupting operation, the RxOnDuration will be

47	checked to determine whether the time has expired. If so, the operation is complete. If not, the receiver is re-

48	enabled until either the device has another conflicting responsibility or the time specified by RxOnDuration

49	has expired. When the primitive is issued to disable the receiver, the device will disable its receiver unless

50	the device has a conflicting responsibility.

51

52

53

54

184	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

On a nonbeacon-enabled OWPAN, the MLME ignores the DeferPermit and RxOnTime parameters and              1 requests that the PHY enable or disable the receiver immediately. If the request is to enable the receiver, the              2 receiver will remain enabled until RxOnDuration symbols have elapsed.                                                                       3

4

Before attempting to enable the receiver on a beacon-enabled OWPAN, the MLME first determines whether              5 (RxOnTime  +  RxOnDuration)  is  less  than  the  beacon  interval,  as  defined  by  macBeaconOrder.  If              6 (RxOnTime + RxOnDuration) is not less than the beacon interval, the MLME issues the MLME-RX-              7

ENABLE.confirm primitive with a status of ON_TIME_TOO_LONG.	8

9

The MLME then determines whether the receiver can be enabled in the current superframe. The OWPAN              10 coordinator issuing this primitive makes the determination based on its own superframe. A device that is not              11 the OWPAN coordinator makes the determination based on the superframe of the coordinator through which              12 it is associated. If the current number of optical clocks measured from the start of the superframe is less than              13

RxOnTime, the MLME attempts to enable the receiver in the current superframe. If the current number of	14 optical clocks measured from the start of the superframe is greater than or equal to RxOnTime and	15

DeferPermit is equal to TRUE, the MLME defers until the next superframe and attempts to enable the              16 receiver in that superframe. Otherwise, if the MLME cannot enable the receiver in the current superframe              17 and is not permitted to defer the receive operation until the next superframe, the MLME issues the MLME-              18

RX-ENABLE.confirm primitive with a status of PAST_TIME.	19

20

If the RxOnDuration parameter is equal to zero, the MLME requests that the PHY disable its receiver.	21

22

If any parameter in the MLME-RX-ENABLE.request primitive is not supported or is out of range, the MAC	23 sublayer will issue the MLME-RX-ENABLE.confirm primitive with a status of INVALID_PARAMETER.	24

25

If  the  request  to  enable  or  disable  the  receiver  was  successful, the  MLME  issues  the  MLME-RX-	26

ENABLE.confirm primitive with a status of SUCCESS.	27

28

6.3.8.2 MLME-RX-ENABLE.confirm	29

30

The MLME-RX-ENABLE.confirm primitive reports the results of the attempt to enable or disable the	31 receiver.	32

33

The semantics of the MLME-RX-ENABLE.confirm primitive are as follows:	34

| MLME-RX-ENABLE.confirm | ( | 36 |
| --- | --- | --- |
|  | s ta tus | 37 |
|  | ) | 38 |
|  |  | 39 |

35

Table 73 specifies the parameter for the MLME-RX-ENABLE.confirm primitive.	40

41

42

Table 73—MLME-RX-ENABLE.c o nfirm parame te r	43

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, PAST_TIME, ON_TIME_TOO_LONG, INVALID_PARAMETER | The result of the request to enable or disable the receiver. |

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	185

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.3.8.2.1 Whe n g e ne rate d

2

3	The MLME-RX-ENABLE.confirm primitive is generated by the MLME and issued to its next higher layer

4	in response to an MLME-RX-ENABLE.request primitive.

5

6	6.3.8.2.2 Appro priate us ag e

7

8	On receipt of the MLME-RX-ENABLE.confirm primitive, the next higher layer is notified of its request to

9	enable or disable the receiver. This primitive returns a status of either SUCCESS, if the request to enable or

10	disable the receiver was successful, or the appropriate error code. The status values are fully described in

11	6.3.8.1.2.

12

13	6.3.8.3 Message sequence chart for changing the state of the receiver

14

15	Figure 114 illustrates the sequence of messages necessary for enabling the receiver for a fixed duration when

16	the device does not have any conflicting responsibilities. Figure 114a) illustrates the case for a beacon-

17	enabled OWPAN where it is assumed both that the MLME-RX-ENABLE.request has been received by the

18	MLME without sufficient time available to enable the receiver in the current superframe and that the

19	DeferPermit parameter is TRUE. Figure 114b) illustrates the case for a nonbeacon-enabled OWPAN where

20	the receiver is enabled immediately.

21

22	6.3.9 Primitives for channel scanning

23

24	MLME-SAP scan primitives define how a device can determine the energy usage or the presence or absence

25	of OWPANs in a communications channel.

26

27	All devices shall provide an interface for these scan primitives.

28

29	6.3.9.1 MLME-SCAN.request

30

31	The MLME-SCAN.request primitive is used to initiate a channel scan over a given list of channels. A device

32	can use a channel scan to measure the energy on the channel, search for the coordinator with which it

33	associated, or search for all coordinators transmitting beacon frames within the coverage area of the

34	scanning device.

35

36	The semantics of the MLME-SCAN.request primitive are as follows:

37

38	MLME-S CAN.re que s t	(

39	ScanType ,

40	ScanDura tion,

41	SecurityLe ve l,

42	Ke yIdMode ,

43	Ke yS ource ,

44	Ke yInde x,

45	ColorS ca n

46	)

47

48

49	Table 74 specifies the parameters for the MLME-SCAN.request primitive.

50

51

52

53

54

186	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

Figure 114—Message sequence chart for changing the state of the receiver	36

37

38

39

40

Table 74—MLME-S CAN.re que s t param e te rs	41

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ScanType | Integer | 0x00–0x01 | Indicates the type of scan performed: 0x00 = active scan (optional for a device). 0x01 = passive scan. 0x10 = scan-over-backhaul |
| ScanDuration | Integer | 0–14 | The time spent scanning each channel is [aBaseSuperframeDuration  (2n + 1)] optical clocks, where n is the value of the ScanDuration parameter. |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 96 in 7.4.2.1). |

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	187

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 74—MLME-S CAN.re que s t parame te rs  (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| ColorScan | Boolean | TRUE or FALSE | ColorScan shall be set as TRUE if the color CVD frame is to be transmitted either during passive scan or after the beacon request command is sent (see 5.1.3.1) for an active scan. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22	6.3.9.1.1 Appro priate us ag e

23

24	The MLME-SCAN.request primitive is generated by the next higher layer and issued to its MLME to

25	initiate a channel scan to search for activity within the coverage area of the device. This primitive can be

26	used to perform an active, passive scanor scan-over-backhaul to locate beacon frames containing any

27	OWPAN identifier. Refer to 5.1.5.1 for a description of each type of scan in detail.

28

29	All devices shall be capable of performing passive scans, while active scans are optional for a device.

30	However, a device may support active scanning to participate in a nonbeacon-enabled network.

31

32	6.3.9.1.2 Effe c t o n re c e ipt

33

34	If the MLME receives the MLME-SCAN.request primitive while performing a previously initiated scan

35	operation,  it  issues  the  MLME-SCAN.confirm  primitive  with  a  status  of  SCAN_IN_PROGRESS.

36	Otherwise, the MLME initiates a scan in all channels specified in the ScanChannels parameter.

37

38	The active scan is performed on each channel by the MLME first sending a beacon request command as

39	specified in 5.4.11. The MLME then enables the receiver and records the information contained in each

40	received beacon in a OWPAN descriptor structure as shown in Table 62. The active scan on a particular

41	channel terminates when the number of OWPAN descriptors stored equals an implementation-specified

42	maximum or when [aBaseSuperframeDuration  (2n  + 1)] optical clocks, where n is the value of the

43	ScanDuration  parameter,  have  elapsed,  whichever  comes  first.  Refer  to  5.1.3.1.1  for  more  detailed

44	information on the active channel scan procedure.

45

46	The passive scan is performed on each channel by the MLME enabling its receiver and recording the

47	information contained in each received beacon in a OWPAN descriptor structure as specified in Table 62.

48	The passive scan on a particular channel terminates when the number of OWPAN descriptors stored equals

49	an implementation-specified maximum or when [aBaseSuperframeDuration  (2n  + 1)] optical clocks,

50	where n is the value of the ScanDuration parameter, have elapsed, whichever comes first. Refer to 5.1.3.1.2

51	for more detailed information on the passive channel scan procedure.

52

53	The scan-over-backhaul is performed on each channel by the MLME first sending a scan-over-backhaul

54	request command to other coordinators through the backhaul as specified in 6.7.6. The MLME then enables

188	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

the receiver and records the information contained in each received scan-over-backhaul confirmation              1 command. The  scan-over-backhaul on  a  particular channel terminates when  the  number of  OWPAN              2 descriptors stored equals an implementation-specified maximum or when [aBaseSuperframeDuration ? (2n              3

+ 1)]  optical clocks, where n is the value of the ScanDuration parameter, have elapsed, whichever comes	4 first. Refer to 6.2.2.1.1 for more detailed information on the scan-over-backhaul procedure.	5

6

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for              7 this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC              8 sublayer  will  perform  outgoing  processing  on  the  frame  based  on  macCoordExtendedAddress,  the              9

SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs              10 during outgoing frame processing, the MLME will discard the frame and issue the MLME-SCAN.confirm              11 primitive with the error status returned by outgoing frame processing.                                                                           12

13

The results of an active, or passive scan or scan-over-backhaul are reported to the next higher layer through              14 the MLME-SCAN.confirm primitive. If the scan is successful and macAutoRequest is set to TRUE, the              15 primitive  results  will  include  a  set  of  OWPAN  descriptor  values.  If  the  scan  is  successful  and              16 macAutoRequest is set to FALSE, the primitive results will contain a null set of OWPAN descriptor values;              17 each OWPAN descriptor value will be sent individually to the next higher layer using separate MLME-              18

BEACON-NOTIFY (see  6.3.3.1) primitives. In  both  cases,  the  MLME-SCAN.confirm primitive will	19 contain a list of unscanned channels and a status of SUCCESS.	20

21

If, during an active scan, the MLME is unable to transmit a beacon request command on a channel specified              22 by the ScanChannels parameter due to a channel access failure, the channel will appear in the list of              23 unscanned channels returned by the MLME-SCAN.confirm primitive. If the MLME was able to send a              24 beacon request  command on  at  least  one  of  the  channels but  no  beacons were  found,  the  MLME-              25

SCAN.confirm primitive will contain a null set of OWPAN descriptor values, regardless of the value of	26

macAutoRequest, and a status of NO_BEACON.	27

28

If, during an active or passive scan, the implementation-specified maximum is reached thus terminating the	29 scan  procedure,  the  MAC  sublayer  will  issue  the  MLME-SCAN.confirm primitive  with  a  status  of	30

LIMIT_REACHED.	31

32

If any parameter in the MLME-SCAN.request primitive is not supported or is out of range, the MAC	33 sublayer will issue the MLME-SCAN.confirm primitive with a status of INVALID_PARAMETER.	34

35

6.3.9.2 MLME-SCAN.confirm	36

37

The MLME-SCAN.confirm primitive reports the result of the channel scan request.	38

39

The semantics of the MLME-SCAN.confirm primitive are as follows:	40

41

MLME-S CAN.confirm	(	42

s ta tus ,	43

ScanType ,	44

Re sultLis tS ize ,	45

OWPANDe s criptorLis t	46

)	47

48

49

Table 75 specifies the parameters for the MLME-SCAN.confirm primitive.	50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	189

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 75—MLME-S CAN.c o nfirm parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, LIMIT_REACHED, NO_BEACON, SCAN_IN_PROGRESS, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY or INVALID_PARAMETER | The status of the scan request. |
| ScanType | Integer | 0x00–0x01 | Indicates the type of scan performed: 0x00 = active scan 0x01 = passive scan 0x10 = scan-over-backhaul |
| ResultListSize | Integer | Implementation specific | The number of elements returned in the appropriate result lists. |
| OWPANDescriptorL ist | List of OWPAN descriptor values | Refer to Table 62 | The list of OWPAN descriptors, one for each beacon found during a scan if macAutoRequest is set to TRUE. This parameter is null when macAutoRequest is set to FALSE during a scan. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27	6.3.9.2.1 Whe n g e ne rate d

28

29	The MLME-SCAN.confirm primitive is generated by the MLME and issued to its next higher layer when

30	the  channel  scan  initiated  with  the  MLME-SCAN.request primitive  has  completed.  If  the  MLME-

31	SCAN.request primitive requested a scan with macAutoRequest set to FALSE, the OWPANDescriptorList

32	parameter will be null.

33

34	The MLME-SCAN.confirm primitive returns a status of either SUCCESS, indicating that the requested scan

35	was successful, or the appropriate error code. The status values are fully described in  6.3.9.1.2 and

36	subclauses referenced by 6.3.9.1.2.

37

38	6.3.9.2.2 Appro priate us ag e

39

40	On receipt of the MLME-SCAN.confirm primitive, the next higher layer is notified of the results of the scan

41	procedure. If the requested scan was successful, the status parameter will be set to SUCCESS. Otherwise,

42	the status parameter indicates the error.

43

44	6.3.9.3 Channel scan message sequence charts

45

46	Figure 127 and Figure 128 illustrate the sequence of messages necessary to perform a passive scan and an

47	active scan. These figures include steps taken by the PHY.

48

49	6.3.10 Neighbor report primitives

50

51	The  MLME-SAP  NeighborReport  management  primitives  define  how  the  coordinator  triggers  a

52	neighboring OWPAN report and how the requested devices to perform the report.

53

54

190	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.10.1 MLME-NeighborReport.request	1

2

The MLME-NeighborReport.request primitive allows the coordinator to generate and send a Neighboring	3

OWPAN report request command to devices.	4

5

The semantics of the MLME-NeighborReport.request primitive are as follows:	6

7

MLME-NeighborReport.request (	8

9

ReportDevicesList,	10

11

ReportType,	12

13

)	14

15

16

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ReportDevicesList | List of short address |  | Indicates to which device(s) the coordinator shall send a neighboring OWPAN report command. |
| ReportType | Integer | 0x00 - 0x01 | 0x00 = only update on neighboring OWPAN sta- tus is requested; 0x01= full report on neighboring OWPAN status is requested; |

Table 76—MLME-Ne ig hbo rRe po rt.re que s t	17

|  |  | 18 |
| --- | --- | --- |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
| 6.3.10.1.1 Whe n g e ne rate d |  | 30 |
|  |  | 31 |
| On receipt of the MLME-SYNC-LOSS.indication primitive with the LossReason | parameter set to | 32 |
| OWPAN_ID_CONFLICT by the next higher layer of the coordinator, it shall | issue a MLME- | 33 |
| NeighborReport.request primitive to its MLME. |  | 34 |
|  |  | 35 |
| 6.3.10.1.2 Effe c t o n re c e ipt |  | 36 |
|  |  | 37 |

The MLME of the coordinator shall generate a neighboring OWPAN report request command and send the	38

command to the device specified by the ReportDeviceList. If multiple devices have been requested by the	39

next higher layer, the MLME shall generate multiple neighboring OWPAN report request command and	40

send the command to the multiple devices respectively.	41

42

6.3.10.1.3 MLME-Ne ig hbo rRe po rt.indic atio n	43

44

The MLME-NeighborReport.indication primitive indicates to the next higher layer of the device that the	45

MLME has received a neighboring OWPAN report request command from the coordinator.	46

47

The semantics of the MLME-NeighborReport.indication primitive are as follows:	48

49

MLME-NeighborReport.indication (	50

|  | 51 |
| --- | --- |
| ReportType, | 52 |
|  | 53 |
| ) | 54 |

Copyright © 20xx IEEE. All rights reserved.	191

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.3.10.1.4 Whe n g e ne rate d

2

3	On receipt of a neighboring OWPAN report request command by the MLME of the device, it shall issue the

4	MLME-NeighborReport.indication primitive to its next higher layer.

5

6	6.3.10.1.5 Effe c t o n re c e ipt

7

8	On receipt of the MLME-NeighborReport.indication primitive, the next higher layer of the device is notified

9	of the neighboring OWPAN report request of the coordinator. The next higher layer shall then decide

10	whether to perform an active scan or not and then provide the updated local neighboring OWPAN descriptor

11	list to the MLME by issuing a MLME- NeighborReport.response primitive.

12

13	6.3.10.1.6 MLME-Ne ig hbo rRe po rt.re s po ns e

14

15	The   MLME-NeighborReport.response  primitive   is   used   to   initiate   a   response   to   an   MLME-

16	NeighborReport.indication primitive.

17

18	The semantics of the MLME-NeighborReport.response primitive are as follows:

19

20	MLME-NeighborReport.response (

21

22	ReportType,

23

24	NumberNeighboringOWPAN Descriptor,

25

26	NeighboringOWPANDescriptor[0],

27

28	……

29

30	NeighboringOWPANDescriptor[N-1],

31

32	)

33

34

35	Table 77—MLME-Ne ig hbo rRe po rt.re s po ns e

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ReportType | Integer | 0x00 - 0x001 | 0x00= only update on neighboring OWPAN status is requested; 0x01=full report on neighboring OWPAN sta- tus is requested; |
| NumberNeighborin- gOWPANDescriptor | Integer |  | Indicates the number of Neighboring OWPAN Descriptors. |
| NeighboringOW- PANDescriptor[x] |  |  | Indicates the basic information of the neigh- boring OWPAN. |

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51	6.3.10.1.7 Whe n g e ne rate d

52

53	The MLME-NeighborReport.response primitive is generated by the next higher layer of a device and issued

54	to its MLME in order to respond the MLME-NeighborReport.indication primitive.

192	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.10.1.8 Effe c t o n re c e ipt	1

2

On receipt of the MLME- NeighborReport.response primitive, the MLME of the device shall generate a              3 neighboring OWPAN report indication command based on the provided arguments and send the command              4 to the coordinator.                                                                                                                                                             5

6

6.3.10.2 MLME-NeighborReport.confirm	7

8

The MLME-NeighborReport.confirm primitive provides the result of a requested neighboring OWPAN              9 report to the next higher layer of the coordinator.                                                                                                            10

11

The semantics of the MLME-NeighborReport.confirm primitive are as follows:	12

13

MLME-NeighborReport.confirm (	14

15

Result,	16

17

UnacknowledgedDevices,	18

19

NumberNeighboringOWPAN Descriptor,	20

21

NeighboringOWPANDescriptor[0],	22

23

DetectDeviceList[0],	24

25

……	26

27

NeighboringOWPANDescriptor[N-1],	28

29

DetectDeviceList[N-1],	30

31

)	32

33

34

Table 78—MLME-Ne ighbo rRepo rt.confirm	35

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Result | Numeration | SUCCESS, PARTIAL_AC- KNOWLEDGE | Indicates the result of the requested report. |
| Unacknowledged- DevicesList | List of short addresses |  | Indicates the short address of the devices from which the corresponding neighboring OWPAN report indication commands have not been received by the coordinator within the macNeighborRepor- tResponseTime optical clocks. The field is valid only when the Result equals to "PARTIAL_ACKNOWLEDGED". |
| NumberNeighbor- ingOWPAN Descriptor | Integer | 0x00 - 0xff | Indicates the number of Neighboring OWPAN Descriptors. |
| NeighboringOW- PANDescriptorL- ist | lists of Neigh- boringOWPAN Descriptor | Refer to Table 3 | Indicates the basic information of the neighboring OWPAN. |

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	193

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.3.10.2.1 Whe n g e ne rate d

2

3	When the neighboring OWPAN report indication commands from each requested device have been received

4	by the MLME of the coordinator, or when the macNeighborReportResponseTime optical clocks time

5	expires, the MLME of the coordinator shall issue the MLME-NeighborReport.confirm primitive to its next

6	higher layer to report the result of the requested report.

7

8	6.3.10.2.2 Effe c t o n re c e ipt

9

10	The next higher layer of the coordinator shall be notified of the result of the requested neighboring OWPAN

11	report on receipt of the MLME-NeighborReport.confirm primitive.

12

13	6.3.11 Communication status primitive

14

15	The MLME-SAP communication status primitive defines how the MLME communicates to the next higher

16	layer about transmission status, when the transmission was instigated by a response primitive, and about

17	security errors on incoming packets.

18

19	All devices shall provide an interface for this communication status primitive.

20

21	6.3.11.1 MLME-COMM-STATUS.indication

22

23	The MLME-COMM-STATUS.indication primitive allows the MLME to indicate a communications status.

24

25	The semantics of the MLME-COMM-STATUS.indication primitive are as follows:

26

27	MLME-COMM-S TATUS .indica tion	(

28	OWPANId,

29	S rcAddrMode ,

30	S rcAddr,

31	Ds tAddrMode ,

32	Ds tAddr,

33	s ta tus ,

34	SecurityLe ve l,

35	Ke yIdMode ,

36	Ke yS ource ,

37	Ke yInde x

38	)

39

40	Table 79 specifies the parameters for the MLME-COMM-STATUS.indication primitive.

41

42

43	Table 79—MLME-COMM-S TATUS .indic atio n parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| OWPANId | Integer | 0x0000–0xffff | The 16-bit OWPAN identifier of the device from which the frame was received or to which the frame was being sent. |

44

45

46

47

48

49

50

51

52

53

54

194	Copyright © 20xx IEEE. All rights reserved.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Integer | 0x00–0x03 | The source addressing mode for this primitive. This value can take one of the following values: 0 = no address (addressing fields omitted). 0x01 = no address field (broadcast only mode with no address fields present). 0x02 = 16-bit short address. 0x03 = 64-bit extended address. |
| SrcAddr | Device address | As specified by the SrcAddrMode parameter | The individual device address of the entity from which the frame causing the error originated. |
| DstAddrMode | Integer | 0x00–0x03 | The destination addressing mode for this primitive. This value can take one of the following values: 0x00 = no address (addressing fields omitted). 0x01 = reserved. 0x02 = 16-bit short address. 0x03 = 64-bit extended address. |
| DstAddr | Device address | As specified by the DstAddrMode parameter | The individual device address of the device for which the frame was intended. |
| status | Enumeration | SUCCESS, TRANSACTION_OVERFLOW, TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, NO_ACK, COUNTER_ERROR, FRAME_TOO_LONG, IMPROPER_KEY_TYPE, IMPROPER_SECURITY_LEVEL, SECURITY_ERROR, UNAVAILABLE_KEY, UNSUPPORTED_LEGACY, UNSUPPORTED_SECURITY, INVALID_PARAMETER | The communications status. |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was generated following a transmission instigated through a response primitive: The security level to be used (as defined in Table 96 in 7.4.2.1). If the primitive was generated on receipt of a frame that generates an error in its security processing: The security level purportedly used by the received frame (as defined in Table 96 in 7.4.2.1). |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 79—MLME-COMM-S TATUS .indic atio n parame te rs | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

Copyright © 20xx IEEE. All rights reserved.	195

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 79—MLME-COMM-S TATUS .indic atio n parame te rs  (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was generated following a transmission instigated through a response primitive: The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. If the primitive was generated on receipt of a frame that generates an error in its security processing: The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | If the primitive was generated following a transmission instigated through a response primitive: The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. If the primitive was generated on receipt of a frame that generates an error in its security processing: The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was generated following a transmission instigated through a response primitive: The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. If the primitive was generated on receipt of a frame that generates an error in its security processing: The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

196	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.11.1.1 Whe n g e ne rate d	1

2

The MLME-COMM-STATUS.indication primitive is generated by the MLME and issued to its next higher              3 layer either following a transmission instigated through a response primitive or on receipt of a frame that              4 generates an error in its security processing (see 7.2.3).                                                                                                   5

6

The MLME-COMM-STATUS.indication primitive is generated by the MAC sublayer entity following the	7

MLME-ASSOCIATE.response primitive. This primitive returns a status of either SUCCESS, indicating that	8 the	request	to	transmit	was	successful,	an	error	code	of   TRANSACTION_OVERFLOW,	9

TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, NO_ACK, or INVALID_PARAMETER             10 (these status values are fully described in  6.3.1.3.2), or  an  error code resulting from failed security              11 processing (these status values are fully described in 7.2.1 and 7.2.3).                                                                            12

13

6.3.11.1.2 Appro priate us ag e	14

15

On receipt of the MLME-COMM-STATUS.indication primitive, the next higher layer is notified of the              16 communication status of a transmission or notified of an error that has occurred during the secure processing              17 of incoming frame.                                                                                                                                                            18

19

6.3.12 Primitive for heterogeneous RF & OWC functionality	20

21

As described in 4.2.6, command frames (e.g. association request command.) from type 2 devices with             22 destination address set to the coordinator may be first transmitted through RF link and then forwarded to the              23 coordinator  through  the  backhaul  link,  if  RF-OWC  functionality  is  enabled.  MLME-SAP command              24 primitives are specified to provide interface to the next higher layer so as to route the command frame              25 through RF link.                                                                                                                                                                26

27

6.3.12.1 MLME-COMMAND.indication	28

29

The MLME-COMMAND.indication primitive allows the MLME of the device to provide a command frame	30 to its next higher layer.	31

32

The semantics of the MLME-COMMAND.indication primitive are as follows:	33

34

MLME-COMMAND.indica tion	(	35

Ds tAddr,	36

S rcAddr,	37

Comma ndFra me	38

)	39

40

Table 80 specifies the parameters for the MLME-COMMAND.indication primitive.	41

42

Table 80—MLME-COMMAND.indic atio n parame te rs	43

44

45

46

47

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the coordinator that the command frame is designated to. |

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	197

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DSrcAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the device that has generated the command frame. |
| CommandFra me | Set of Octets |  | The set of octets forming the command frames to be transmitted. |

1

2

3

4

5

6

7

8

9

10

11	6.3.12.1.1 Whe n g e ne rate d

12

13	When the MLME of the device generates a command frame that needs to be sent to the coordinator, it shall

14	generate a MLME-COMMAND.indication primitive and issue the MLME-COMMAND.indication primi-

15	tive to its next higher layer.

16

17	6.3.12.1.2 Effe c t o n re c e ipt

18

19	On receipt of the MLME-COMMAND.indication primitive, the next higher layer of the device may gener-

20	ate a next higher layer message based on the provided arguments and shall send the message to the the next

21	higher layer of the coordinator through the RF link and backhaul link. The command frame generated by the

22	MLME shall be embedded in the next higher layer message.

23

24	6.3.12.2 MLME-COMMAND.request

25

26	The MLME-COMMAND.request primitive allows the next higher layer of the coordinator to request its

27	MLME to identify and process a command frame received through the RF link and backhaul link.

28

29	The semantics of the MLME-COMMAND.request primitive are as follows:

30

31	MLME-COMMAND.re que s t	(

32	Ds tAddr,

33	S rcAddr,

34	Comma ndFra me

35	)

36

37	Table z specifies the parameters for the MLME-COMMAND. request primitive.

38

39	Table 81—MLME-COMMAND.re que s t param e te rs

40

41

42

43

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the coordinator that the command frame is designated to. |
| DSrcAddr | Integer | From 0x0000-0xffff, 0x0000 represents without a short address assigned, 0xffff represents a broadcast transmission. | The short address of the device that has generated the command frame. |

44

45

46

47

48

49

50

51

52

53

54

198	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CommandFra me | Set of Octets |  | The set of octets forming the command frames to be transmitted. |

1

2

3

4

5

6

6.3.12.2.1 Whe n g e ne rate d	7

8

The next higher layer of the coordinator shall generate a MLME-COMMAND.request primitive when it has	9 received the next higher layer message and issue the MLME-COMMAND.request primitive to its MLME.	10

11

6.3.12.2.2 Effe c t o n re c e ipt	12

13

On receipt of the MLME-COMMAND.request primitive, the MLME of the coordinator shall process the	14 command frame provided in this primitive the same way as it was received through the OWC link.	15

16

6.3.13 Primitives for writing PIB attributes	17

18

MLME-SAP set primitives define how PIB attributes may be written.	19

20

All devices shall provide an interface for these set primitives.	21

22

6.3.13.1 MLME-SET.request	23

24

The MLME-SET.request primitive attempts to write the given value to the indicated PIB attribute.	25

26

6.3.13.1.1 S e mantic s o f the primitive	27

28

The semantics of the MLME-SET.request primitive are as follows:	29

30

MLME -S ET.re que s t	(	31

P IBAttribute ,	32

P IBAttribute Inde x,	33

P IBAttribute Va lue	34

)	35

36

Table 82 specifies the parameters for the MLME-SET.request primitive.	37

38

39

Table 82—MLME-S ET.re que s t parame te rs	40

41

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBAttribute | Integer | Refer to Table 91 and Table 98 | The identifier of the PIB attribute to write. |
| PIBAttributeIndex | Integer | Attribute specific; as defined in Table 91 and Table 98 | The index within the table of the specified PIB attribute to write. This parameter is valid only for MAC PIB attributes that are tables; it is ignored when accessing PHY PIB attributes. |
| PIBAttributeValue | Various | Attribute specific; as defined in Table 91 and Table 98 | The value to write to the indicated PIB attribute. |

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	199

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.3.13.1.2 Appro priate us ag e

2

3	The MLME-SET.request primitive is generated by the next higher layer and issued to its MLME to write the

4	indicated PIB attribute.

5

6	6.3.13.1.3 Effe c t o n re c e ipt

7

8	On receipt of the MLME-SET.request primitive, the MLME checks to see if the PIB attribute is a MAC PIB

9	attribute or PHY PIB attribute. If the requested attribute is a MAC attribute, the MLME attempts to write the

10	given value to the indicated MAC PIB attribute in its database. If the PIBAttribute parameter specifies an

11	attribute that is a read-only attribute, shown in Table 91, the MLME will issue the MLME-SET.confirm

12	primitive with a status of READ_ONLY. If the PIBAttribute parameter specifies an attribute that is not

13	found  in  the  database,  the  MLME  will  issue  the  MLME-SET.confirm  primitive  with  a  status  of

14	UNSUPPORTED_ATTRIBUTE. If the PIBAttributeIndex parameter specifies an index for a table that is out

15	of range, the MLME will issue the MLME-SET.confirm primitive with a status of INVALID_INDEX. If the

16	PIBAttributeValue parameter specifies a value that is out of the valid range for the given attribute, the

17	MLME will issue the MLME-SET.confirm primitive with a status of INVALID_PARAMETER. If the

18	requested MAC PIB attribute is  successfully written, the MLME will issue the MLME-SET.confirm

19	primitive with a status of SUCCESS.

20

21	If the PIBAttribute parameter indicates that macBeaconPayloadLength is to be set and the length of the

22	resulting beacon frame exceeds aMaxPHYFrameSize (e.g., due to the additional overhead required for

23	security processing), the MAC sublayer shall not update macBeaconPayloadLength and will issue the

24	MLME-GET.confirm primitive with a status of INVALID_PARAMETER.

25

26	If the requested attribute is a PHY PIB attribute, the request is passed to the PHY by issuing the PLME-

27	SET.request primitive. Once the MLME receives the PLME-SET.confirm primitive, it will translate the

28	received status value because the status values used by the PHY are not the same as those used by the

29	MLME (e.g., the status values for SUCCESS are 0x00 and 0x07 in the MAC and PHY enumeration tables,

30	respectively). Following the translation, the MLME will issue the MLME-SET.confirm primitive to the next

31	higher layer with the status parameter resulting from the translation and the PIBAttribute parameter equal to

32	that returned by the PLME primitive.

33

34	6.3.13.2 MLME-SET.confirm

35

36	The MLME-SET.confirm primitive reports the results of an attempt to write a value to a PIB attribute.

37

38	The semantics of the MLME-SET.confirm primitive are as follows:

39

40	MLME -S ET.confirm	(

41	s ta tus ,

42	P IBAttribute ,

43	P IBAttribute Inde x

44	)

45

46	Table 83 specifies the parameters for the MLME-SET.confirm primitive.

47

48	6.3.13.2.1 Whe n g e ne rate d

49

50	The MLME-SET.confirm primitive is generated by the MLME and issued to its next higher layer in

51	response to an MLME-SET.request primitive. The MLME-SET.confirm primitive returns a status of either

52	SUCCESS, indicating that the requested value was written to the indicated PIB attribute, or the appropriate

53	error code. The status values are fully described in 6.3.13.1.3.

54

200	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 83—MLME-S ET.c o nfirm parame te rs	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, READ_ONLY, UNSUPPORTED_ATTRIBUTE, INVALID_INDEX, INVALID_PARAMETER | The result of the request to write the PIB attribute. |
| PIBAttribute | Integer | Refer to Table 91 and Table 98 | The identifier of the PIB attribute that was written. |
| PIBAttributeIndex | Integer | Attribute specific; as defined in Table 91 and Table 98 | The index within the table of the specified PIB attribute to write. This parameter is valid only for MAC PIB attributes that are tables; it is ignored when accessing PHY PIB attributes. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

6.3.13.2.2 Appro priate us ag e	18

19

On receipt of the MLME-SET.confirm primitive, the next higher layer is notified of the result of its request	20

to set the value of a PIB attribute. If the requested value was written to the indicated PIB attribute, the status	21

parameter will be set to SUCCESS. Otherwise, the status parameter indicates the error.	22

23

6.3.14 Primitives for updating the superframe configuration	24

25

MLME-SAP start  primitives define  how  a  coordinator can  request  to  start  using  a  new  superframe	26

configuration in order to initiate a OWPAN, begin transmitting beacons on an already existing OWPAN,	27

thus facilitating device discovery, or to stop transmitting beacons.	28

29

These start primitives are optional for a device.	30

31

6.3.14.1 MLME-START.request	32

33

The MLME-START.request primitive allows the OWC coordinator to initiate a new OWPAN or to begin	34

using a new superframe configuration. This primitive may also be used by a device already associated with	35

an existing OWPAN to begin using a new superframe configuration.	36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	201

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The semantics of the MLME-START.request primitive are as follows:

2

3	MLME-S TART.re que s t	(

4	OWPANId,

5	Logica lCha nne l,

6	S ta rtTime,

7	Be a conOrde r,

8	S upe rfra me Orde r,

9	OWPANCoordina tor,

10	CoordRe a lignme nt,

11	CoordRe a lignS e curityLe ve l,

12	CoordRe a lignKe yIdMode ,

13	CoordRe a lignKe yS ource ,

14	CoordRe a lignKe yInde x,

15	Be a conS e curityLe ve l,

16	Be a conKe yIdMode ,

17	Be a conKe yS ource ,

18	Be a conKe yInde x

19	)

20

21

22

23	Table 84 specifies the parameters for the MLME-START.request primitive.

24

25

26	Table 84—MLME-S TART.re que s t parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| OWPANId | Integer | 0x0000–0xffff | The OWPAN identifier to be used by the device. |
| LogicalChannel | Integer | Selected from the available logical channels | The logical channel on which to start using the new superframe configuration. |
| StartTime | Integer | 0x000000–0xffffff | The time at which to begin transmitting beacons. If this parameter is equal to 0x000000, beacon transmissions will begin immediately. Otherwise, the specified time is relative to the received beacon of the coordinator with which the device synchronizes. This parameter is ignored if either the BeaconOrder parameter has a value of 15 or the OWPANCoordinator parameter is TRUE. The time is specified in optical clocks and is rounded to a backoff slot boundary. This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

202	Copyright © 20xx IEEE. All rights reserved.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BeaconOrder | Integer | 0–15 | How often the beacon is to be transmitted. A value of 15 indicates that the coordinator will not transmit periodic beacons. Refer to 5.1.1 for an explanation of the relationship between the beacon order and the beacon interval. |
| SuperframeOrder | Integer | 0–BO or 15 | The length of the active portion of the superframe, including the beacon frame. If the BeaconOrder parameter (BO) has a value of 15, this parameter is ignored. Refer to 5.1.1 for an explanation of the relationship between the superframe order and the superframe duration. |
| OWPANCoordinator | Boolean | TRUE or FALSE | If this value is TRUE, the device will become the coordinator of a new OWPAN. If this value is FALSE, the device will begin using a new superframe configuration on the OWPAN with which it is associated. |
| CoordRealignment | Boolean | TRUE or FALSE | TRUE if a coordinator realignment command is to be transmitted prior to changing the superframe configuration or FALSE otherwise. |
| CoordRealignSecurity- Level | Integer | 0x00–0x07 | The security level to be used for coordinator realignment command frames (as defined in Table 96 in 7.4.2.1). |
| CoordRealignKeyId- Mode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the CoordRealignSecurityLevel parameter is set to 0x00. |
| CoordRealignKey- Source | Set of 0, 4, or 8 octets | As specified by the CoordRealignKeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the CoordRealignKeyId- Mode parameter is ignored or set to 0x00. |
| CoordRealignKeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the CoordRealignKeyIdMode parameter is ignored or set to 0x00. |
| BeaconSecurityLevel | Integer | 0x00–0x07 | The security level to be used for beacon frames (as defined in Table 96 in 7.4.2.1). |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 84—MLME-S TART.re que s t parame te rs | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

Copyright © 20xx IEEE. All rights reserved.	203

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 84—MLME-S TART.re que s t parame te rs  (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BeaconKeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the BeaconSecurityLevel parameter is set to 0x00. |
| BeaconKeySource | Set of 0, 4, or 8 octets | As specified by the BeaconKeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the BeaconKeyIdMode parameter is ignored or set to 0x00. |
| BeaconKeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the BeaconKeyIdMode parameter is ignored or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19	6.3.14.1.1 Appro priate us ag e

20

21	The MLME-START.request primitive is generated by the next higher layer and issued to its MLME to

22	request that a device start using a new superframe configuration.

23

24	6.3.14.1.2 Effe c t o n re c e ipt

25

26	If the MLME-START.request primitive is received when macShortAddress is set to 0xffff, the MLME will

27	issue the MLME-START.confirm primitive with a status of NO_SHORT_ADDRESS.

28

29	When the CoordRealignment parameter is set to TRUE, the coordinator attempts to transmit a coordinator

30	realignment command frame as described in 5.1.5.3. If the transmission of the coordinator realignment

31	command fails due to a channel access failure, the MLME will not make any changes to the superframe

32	configuration (i.e., no PIB attributes will be changed) and will issue an MLME-START.confirm with a status

33	of CHANNEL_ACCESS_FAILURE. If the coordinator realignment command is successfully transmitted,

34	the MLME updates the appropriate PIB parameters with the values of the BeaconOrder, SuperframeOrder,

35	OWPANId,  and  LogicalChannel  parameters,  as  described  in  5.1.5.5,  and  will  issue  an  MLME-

36	START.confirm with a status of SUCCESS.

37

38	When the CoordRealignment parameter is set to FALSE, the MLME updates the appropriate PIB parameters

39	with the values of the BeaconOrder, SuperframeOrder, OWPANId, and LogicalChannel parameters, as

40	described in 5.1.5.5.

41

42	The  address  used  by  the  coordinator  in  its  beacon  frames  is  determined  by  the  current  value  of

43	macShortAddress, which is set by the next higher layer before issuing this primitive.

44

45	If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

46	this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

47	sublayer will perform outgoing processing on the frame, as described in 7.2.1. If the CoordRealignment

48	parameter is set to TRUE, the CoordRealignSecurityLevel, CoordRealignKeyIdMode, CoordRealignKey-

49	Source, and CoordRealignKeyIndex parameters will be used to process the MAC command frame. If the

50	BeaconOrder parameter indicates a beacon-enabled network, the BeaconSecurityLevel, BeaconKeyIdMode,

51	BeaconKeySource, and BeaconKeyIndex parameters will be used to process the beacon frame. If any error

52	occurs during  outgoing frame  processing, the  MLME  will  discard the  frame  and  issue  the  MLME-

53	START.confirm primitive with the error status returned by outgoing frame processing.

54

204	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

If the length of the beacon frame exceeds aMaxPHYFrameSize (e.g., due to the additional overhead required	1 for  security  processing),  the  MAC  sublayer  shall  discard  the  beacon  frame  and  issue  the  MLME-	2

START.confirm primitive with a status of FRAME_TOO_LONG.	3

4

The MLME shall ignore the StartTime parameter if the BeaconOrder parameter is equal to 15 because this              5 indicates a nonbeacon-enabled OWPAN. If the BeaconOrder parameter is less than 15, the MLME examines              6 the StartTime parameter to determine the time to begin transmitting beacons; the time is defined in optical              7 clocks and is rounded to a backoff slot boundary. If the OWC coordinator parameter is set to TRUE, the              8

MLME  ignores  the  StartTime  parameter  and  begins  beacon  transmissions  immediately.  Setting  the	9

StartTime parameter to 0x000000 also causes the MLME to begin beacon transmissions immediately. If the	10

OWPANCoordinator parameter is set to FALSE and the StartTime parameter is nonzero, the MLME              11 calculates the beacon transmission time by adding StartTime optical clocks to the time, obtained from the              12 local clock, when the MLME receives the beacon of the coordinator through which it is associated. If the              13 time calculated causes the outgoing superframe to overlap the incoming superframe, the MLME shall not              14 begin beacon transmissions. In this case, the MLME issues the MLME-START.confirm primitive with a              15 status of SUPERFRAME_OVERLAP. Otherwise, the MLME then begins beacon transmissions when the              16 current time, obtained from the local clock, equals the number of calculated optical clocks.                                         17

18

If the StartTime parameter is nonzero and the MLME is not currently tracking the beacon of the coordinator	19 through which it is associated, the MLME will issue the MLME-START.confirm primitive with a status of	20

TRACKING_OFF.	21

22

On completion of this procedure, the MLME responds with the MLME-START.confirm primitive. If the	23 attempt to start using a new superframe configuration was successful, the status parameter will be set to	24

SUCCESS. If any parameter is not supported or is out of range, the status parameter will be set to	25

INVALID_PARAMETER.	26

27

6.3.14.2 MLME-START.confirm	28

29

The MLME-START.confirm primitive reports the results of the attempt to start using a new superframe	30 configuration.	31

32

The semantics of the MLME-START.confirm primitive are as follows:	33

34

MLME-S TART.confirm	(	35

s ta tus	36

)	37

38

Table 85 specifies the parameters for the MLME-START.confirm primitive.	39

40

41

Table 85—MLME-S TART.c o nfirm parame te rs	42

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, NO_SHORT_ADDRESS, SUPERFRAME_OVERLAP, TRACKING_OFF, INVALID_PARAMETER, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY, CHANNEL_ACCESS_FAILURE | The result of the attempt to start using an updated superframe configuration. |

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	205

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.3.14.2.1 Whe n g e ne rate d

2

3	The MLME-START.confirm primitive is generated by the MLME and issued to its next higher layer in

4	response to an MLME-START.request primitive. The MLME-START.confirm primitive returns a status of

5	either SUCCESS, indicating that the MAC sublayer has started using the new superframe configuration, or

6	the appropriate error code. The status values are fully described in 6.3.14.1.2 and subclauses referenced by

7	6.3.14.1.2.

8

9	6.3.14.2.2 Appro priate us ag e

10

11

12	On receipt of the MLME-START.confirm primitive, the next higher layer is notified of the result of its

13	request to start using a new superframe configuration. If the MAC sublayer has been successful, the status

14	parameter will be set to SUCCESS. Otherwise, the status parameter indicates the error.

15

16	6.3.14.3 Message sequence chart for updating the superframe configuration

17

18	Figure 115  illustrates  the  sequence  of  messages  necessary  for  initiating  beacon  transmissions  as  a

19	coordinator. Figure 124 illustrates the sequence of messages necessary for the OWC coordinator to start

20	beaconing on a new OWPAN; this figure includes steps taken by the PHY.

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43	Figure 115—Message sequence chart for updating the superframe configuration

44

45

46	6.3.15 Primitive for synchronizing with a coordinator

47

48	MLME-SAP synchronization primitives define how synchronization with a coordinator may be achieved

49	and how a loss of synchronization is communicated to the next higher layer.

50

51	All devices shall provide an interface for the indication primitive. The request primitive is optional.

52

53

54

206	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

6.3.15.1 MLME-SYNC.request	1

2

The MLME-SYNC.request primitive requests to synchronize with the coordinator by acquiring and, if	3 specified, tracking its beacons.	4

5

The semantics of the MLME-SYNC.request primitive are as follows:	6

7

MLME-S YNC.re que s t	(	8

Logica lCha nne l,	9

TrackBeacon	10

)	11

12

13

14

Table 86 specifies the parameters for the MLME-SYNC.request primitive.	15

16

Table 86—MLME-S YNC.re que s t parame te rs	17

18

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| LogicalChannel | Integer | Selected from the available logical channels supported by the PHY | The logical channel on which to attempt coordinator synchronization. |
| TrackBeacon | Boolean | TRUE or FALSE | TRUE if the MLME is to synchronize with the next beacon and attempt to track all future beacons. FALSE if the MLME is to synchronize with only the next beacon. |

19

20

21

22

23

24

25

26

27

28

6.3.15.1.1 Appro priate us ag e	29

30

The MLME-SYNC.request primitive is generated by the next higher layer of a device on a beacon-enabled	31

OWPAN and issued to its MLME to synchronize with the coordinator.	32

33

6.3.15.1.2 Effe c t o n re c e ipt	34

35

If the MLME-SYNC.request primitive is received by the MLME on a beacon-enabled OWPAN, it will first              36 set phyCurrentChannel equal to the values of the LogicalChannel parameters, respectively; both attributes              37 are updated by issuing the PLME-SET.request primitive. If the TrackBeacon parameter is equal to TRUE,              38 the MLME will track the beacon, i.e., enable its receiver just before the expected time of each beacon so that              39 the beacon frame can be processed. If the TrackBeacon parameter is equal to FALSE, the MLME will locate              40 the beacon, but not continue to track it.                                                                                                                            41

42

If this primitive is received by the MLME while it is currently tracking the beacon, the MLME will not	43 discard the primitive, but rather treat it as a new synchronization request.	44

45

If the beacon could not be located either on its initial search or during tracking, the MLME will issue the	46

MLME-SYNC-LOSS.indication primitive with a loss reason of BEACON_LOST.	47

48

6.3.16 Primitive for synchronization loss with a coordinator	49

50

6.3.16.1 MLME-SYNC-LOSS.indication	51

52

The MLME-SYNC-LOSS.indication primitive indicates the loss of synchronization with a coordinator.	53

54

Copyright © 20xx IEEE. All rights reserved.	207

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The semantics of the MLME-SYNC-LOSS.indication primitive are as follows:

2

3	MLME-S YNC-LOS S .indica tion	(

4	Los s Re a s on,

5	OWPANId,

6	Logica lCha nne l,

7	SecurityLe ve l,

8	Ke yIdMode ,

9	Ke yS ource ,

10	Ke yInde x

11	)

12

13

14	Table 87 specifies the parameters for the MLME-SYNC-LOSS.indication primitive.

15

16

17	Table 87—MLME-S YNC-LOS S .indic atio n parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| LossReason | Enumeration | OWPAN_ID_CONFLICT, REALIGNMENT, or BEACON_LOST | The reason that synchronization was lost. |
| OWPANId | Integer | 0x0000–0xffff | The OWPAN identifier with which the device lost synchronization or to which it was realigned. |
| LogicalChannel | Integer | Selected from the available logical channels supported by the PHY (see Table 108). | The logical channel on which the device lost synchronization or to which it was realigned. |
| SecurityLevel | Integer | 0x00–0x07 | If the primitive was either generated by the device itself following loss of synchronization or generated by the coordinator upon detection of a OWPAN ID conflict, the security level is set to 0x00. If the primitive was generated following the reception of either a coordinator realignment command or a OWPAN ID conflict notification command: The security level purportedly used by the received MAC frame (as defined in Table 96 in 7.4.2.1). |

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

208	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 87—MLME-S YNC-LOSS.indication param e te rs | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| KeyIdMode | Integer | 0x00–0x03 | If the primitive was either generated by the device itself following loss of synchronization or generated by the coordinator upon detection of a OWPAN ID conflict, this parameter is ignored. If the primitive was generated following the reception of either a coordinator realignment command or a OWPAN ID conflict notification command: The mode used to identify the key purportedly used by the originator of the received frame (as defined in Table 97 in 7.4.2.2). This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | If the primitive was either generated by the device itself following loss of synchronization or generated by the coordinator upon detection of a OWPAN ID conflict, this parameter is ignored. If the primitive was generated following the reception of either a coordinator realignment command or a OWPAN ID conflict notification command: The originator of the key purportedly used by the originator of the received frame (see 7.4.4.1). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | If the primitive was either generated by the device itself following loss of synchronization or generated by the coordinator upon detection of a OWPAN ID conflict, this parameter is ignored. If the primitive was generated following the reception of either a coordinator realignment command or a OWPAN ID conflict notification command: The index of the key purportedly used by the originator of the received frame (see 7.4.4.2). This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |

48

6.3.16.2 Message sequence chart for synchronizing with a coordinator	49

50

Figure 116 illustrates the sequence of messages necessary for a device to synchronize with a coordinator. In	51

Figure 116a), a single synchronization request is issued. The MLME then searches for a beacon and, if	52

found, determines whether the coordinator has any data pending for the device. If so, the data are requested	53

as described in 5.1.9.3. In Figure 116b), a track synchronization request is issued. The MLME then searches	54

Copyright © 20xx IEEE. All rights reserved.	209

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	for a beacon and, if found, attempts to keep track of it using a timer that expires just before the expected time

2	of the next beacon.

3

4	For both examples Figure 116a) and Figure 116b), the received beacon frames do not contain payload, and

5	macAutoRequest is set to TRUE. The MLME also checks for any data pending in the coordinator for the

6	device when a beacon frame is received.

7	device next

8	higher layer

9

10

11

13	a)

14

15

16

17

18

19

20

21	b)

22

23

24

26

27

28

29

30

31

32

33

34

35

MLME-SYNC.request(FALSE )

MLME-SYNC.request(TRUE)

timer to expire before the next beacon

device

MLME

beacon

beacon

coordinator

MLME

data request

single

synchronization request

track request

36	Figure 116—Message sequence chart for synchronizing to a coordinator in a beacon-

37

38

39	6.3.16.2.1 Whe n g e ne rate d

40

41	The MLME-SYNC-LOSS.indication primitive is generated by the MLME of a device and issued to its next

42	higher layer in the event of a loss of synchronization with the coordinator. It is also generated by the MLME

43	of the OWC coordinator and issued to its next higher layer in the event of a OWPAN ID conflict.

44

45	If a device that is associated through the OWC coordinator has detected a OWPAN identifier conflict and

46	communicated it to the OWC coordinator, the MLME will issue this primitive with the LossReason

47	parameter set to OWPAN_ID_CONFLICT. Similarly, if the OWC coordinator receives a OWPAN ID

48	conflict  notification  command,  as  specified  in  5.4.8,  the  MLME  will  issue  this  primitive  with  the

49	LossReason parameter set to OWPAN_ID_CONFLICT.

50

51	If a device has received the coordinator realignment command, specified in 5.4.13, from the coordinator

52	through which it is associated, the MLME will issue this primitive with the LossReason parameter set to

53	REALIGNMENT and the OWPANId, LogicalChannel, and security-related parameters set as described in

54	5.1.5.4.

210	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

If a device has not heard the beacon for aMaxLostBeacons consecutive superframes following an MLME-	1

SYNC.request primitive, either initially or during tracking, the MLME will issue this primitive with the	2

LossReason parameter set to BEACON_LOST. The OWPANId, LogicalChannel parameters shall be set              3 according to the coordinator with which synchronization was lost. The SecurityLevel parameter shall be set              4 to zero and the KeyIdMode, KeySource, and KeyIndex parameters shall be ignored. If the beacon was being              5 tracked, the MLME will not attempt to track the beacon any further.                                                                              6

7

6.3.16.2.2 Appro priate us ag e	8

9

On receipt of the MLME-SYNC-LOSS.indication primitive, the next higher layer is notified of a loss of	10 synchronization.	11

12

6.3.17 Primitives for requesting data from a coordinator	13

14

MLME-SAP polling primitives define how to request data from a coordinator.	15

16

All devices shall provide an interface for these polling primitives.	17

18

6.3.17.1 MLME-POLL.request	19

20

The MLME-POLL.request primitive prompts the device to request data from the coordinator.	21

22

The semantics of the MLME-POLL.request primitive are as follows:	23

24

MLME-P OLL.re que s t	(	25

CoordAddrMode ,	26

CoordOWPANId,	27

CoordAddre s s ,	28

SecurityLe ve l,	29

Ke yIdMode ,	30

Ke yS ource ,	31

Ke yInde x	32

)	33

34

Table 88 specifies the parameter for the MLME-POLL.request primitive.	35

36

37

Table 88—MLME-POLL.re que s t parame te rs	38

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddrMode | Integer | 0x02–0x03 | The addressing mode of the coordinator to which the poll is intended. This parameter can take one of the following values: 2 = 16-bit short address, 3 = 64-bit extended address. |
| CoordOWPANId | Integer | 0x0000–0xfffe | The OWPAN identifier of the coordinator to which the poll is intended. |
| CoordAddress | Device- Address | As specified by the CoordAddrMode parameter | The address of the coordinator to which the poll is intended. |

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	211

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 88—MLME-POLL.re que s t parame te rs  (continued)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SecurityLevel | Integer | 0x00–0x07 | The security level to be used (as defined in Table 96 in 7.4.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (as defined in Table 97 in 7.4.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.4.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.4.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20	6.3.17.1.1 Appro priate us ag e

21

22	The MLME-POLL.request primitive is generated by the next higher layer and issued to its MLME when

23	data are to be requested from a coordinator.

24

25	6.3.17.1.2 Effe c t o n re c e ipt

26

27	On receipt of the MLME-POLL.request primitive, the MLME generates and sends a data request command,

28	as specified in 5.4.5. If the poll is directed to the coordinator, the data request command may be generated

29	without any destination address information present. Otherwise, the data request command is always

30	generated with the destination address information in the CoordOWPANId and CoordAddress parameters.

31

32	If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

33	this frame, the MLME will set the Security Enabled subfield of the frame control field to one. The MAC

34	sublayer will  perform outgoing processing on  the  frame  based  on  the  CoordAddress, SecurityLevel,

35	KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.2.1. If any error occurs during

36	outgoing frame processing, the MLME will discard the frame and issue the MLME-POLL.confirm primitive

37	with the error status returned by outgoing frame processing.

38

39	If the data request command cannot be sent due to an unslotted random access algorithm failure, the MLME

40	will issue the MLME-POLL.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.

41

42	If the MLME successfully transmits a data request command, the MLME will expect an acknowledgment in

43	return. If an acknowledgment is not received, the MLME will issue the MLME-POLL.confirm primitive

44	with a status of NO_ACK (see 5.1.9.4).

45

46	If an acknowledgment is received, the MLME will request that the PHY enable its receiver if the frame

47	pending  subfield  of  the  acknowledgment frame  is  set  to  one.  If  the  frame  pending  subfield  of  the

48	acknowledgment frame is set to zero, the MLME will issue the MLME-POLL.confirm primitive with a

49	status of NO_DATA.

50

51	If a frame is received from the coordinator with a zero length payload or if the frame is a MAC command

52	frame, the MLME will issue the MLME-POLL.confirm primitive with a status of NO_DATA. If a frame is

53	received  from  the  coordinator  with  nonzero  length  payload,  the  MLME  will  issue  the  MLME-

54

212	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

POLL.confirm primitive with a status of SUCCESS. In this case, the actual data are indicated to the next	1 higher layer using the MCPS-DATA.indication primitive as specified in 6.2.3.	2

3

If a frame is not received within macMaxFrameTotalWaitTime CAP optical clocks in a beacon-enabled	4

OWPAN, or optical clocks in a nonbeacon-enabled OWPAN, even though the acknowledgment to the data              5 request command has its frame pending subfield set to one, the MLME will issue the MLME-POLL.confirm              6 primitive with a status of NO_DATA.                                                                                                                              7

8

If any parameter in the MLME-POLL.request primitive is not supported or is out of range, the MLME will              9 issue the MLME-POLL.confirm primitive with a status of INVALID_PARAMETER.                                                 10

11

6.3.17.2 MLME-POLL.confirm	12

13

The MLME-POLL.confirm primitive reports the results of a request to poll the coordinator for data.	14

15

The semantics of the MLME-POLL.confirm primitive are as follows:	16

MLME-P OLL.confirm	(	17 s ta tus	18

)	19

20

Table 89 specifies the parameters for the MLME-POLL.confirm primitive.	21

22

23

Table 89—MLME-P OLL.c o nfirm parame te rs	24

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Integer | SUCCESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY or INVALID_PARAMETER | The status of the data request. |

25

26

27

28

29

30

31

32

33

34

35

36

6.3.17.2.1 Whe n g e ne rate d	37

38

The MLME-POLL.confirm primitive is generated by the MLME and issued to its next higher layer in              39 response to an MLME-POLL.request primitive. If the request was successful, the status parameter will be              40 equal to SUCCESS, indicating a successful poll for data. Otherwise, the status parameter indicates the              41 appropriate error code. The status values are fully described in 6.3.17.1.2 and the subclauses referenced by              42

6.3.17.1.2.	43

44

6.3.17.2.2 Appro priate us ag e	45

46

On receipt of the MLME-POLL.confirm primitive, the next higher layer is notified of the status of the	47 procedure to request data from the coordinator.	48

49

6.3.17.3 Message sequence chart for requesting data from a coordinator	50

51

Figure 117 illustrates the sequence of messages necessary, including the layer behavior of the device and the	52 over-the-air interface, for a device to request data from a coordinator.	53

54

Copyright © 20xx IEEE. All rights reserved.	213

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	In both scenarios Figure 117a) and Figure 117b), a poll request is issued to the MLME, which then sends a

2	data request command to the coordinator. In Figure 117a), the corresponding acknowledgment has the frame

3	pending (FP) subfield set to zero and the MLME issues the poll request confirmation immediately. In

4	Figure 117b), the corresponding acknowledgment has the frame pending subfield set to one and the MLME

5	enables the receiver in anticipation of the data frame from the coordinator. On receipt of this data frame, the

6	MLME issues a poll request confirmation followed by a data indication containing the data of the received

7	frame.

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31	Figure 117—Message sequence chart for requesting data from a coordinator

32

33

34	6.4 MAC constants and PIB attributes

35

36	This subclause specifies the constants and attributes required by the MAC sublayer.

37

38	6.4.1 MAC constants

39

40	The constants that define the characteristics of the MAC sublayer are presented in Table 90.

41

42	6.4.2 MAC PIB attributes

43

44	The MAC PIB comprises the attributes required to manage the MAC sublayer of a device. The attributes

45	contained in the MAC PIB are presented in Table 91 and Table 98. Attributes marked with a dagger (†) are

46	read-only attributes (i.e., attribute can only be set by the MAC sublayer), which can be read by the next

47	higher layer using the MLME-GET.request primitive. All other attributes can be read or written by the next

48	higher layer using the MLME-GET.request or MLME-SET.request primitives, respectively. Higher layers

49	may  impose  additional  constraints  on  read/write  operations,  without  making  devices  non-compliant.

50	Attributes marked with a diamond () are optional for a device (i.e., not operating as a coordinator).

51

52	The read-only attribute macAckWaitDuration is dependent on a combination of constants and PHY PIB

53	attributes. The formula for relating the constants and attributes is shown in Equation (4).

54

214	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 90—MAC s ublaye r c o ns tants	1

| Constant | Description | Value |
| --- | --- | --- |
| aBaseSlotDuration | The number of optical clocks forming a superframe slot when the superframe order is equal to 0 (see 5.1.1). | 60 |
| aBaseSuperframeDuration | The number of optical clocks forming a superframe when the superframe order is equal to 0. | aBaseSlotDuration  aNumSuperframeSlots |
| aExtendedAddress | The 64-bit (IEEE) address assigned to the device. | Device specific |
| aGTSDescPersistenceTime | The number of superframes in which a GTS descriptor exists in the beacon frame of the coordinator. | 4 |
| aMaxBeaconOverhead | The maximum number of octets added by the MAC sublayer to the MSDU of a beacon frame. | 75 |
| aMaxBeaconPayloadLength | The maximum size, in octets, of a beacon payload. | aMaxPHYFrameSize – aMaxBeaconOverhead |
| aMaxLostBeacons | The number of consecutive lost beacons that will cause the MAC sublayer of a receiving device to declare a loss of synchronization. | 4 |
| aMaxMACSafePayloadSize | The maximum number of octets that can be transmitted in the MSDU field of an unsecured MAC frame that will be guaranteed not to exceed aMaxPHYFrameSize. | aMaxPHYFrameSize – aMaxMPDUUnsecure dOverhead |
| aMaxMACPayloadSize | The maximum number of octets that can be transmitted in the MSDU field. | aMaxPHYFrameSize – aMinMPDUOverhead |
| aMaxMPDUUnsecuredOverhead | The maximum number of octets added by the MAC sublayer to the PSDU without security. | 25 |
| aMaxSIFSFrameSize | The maximum size of an MPDU, in octets, that can be followed by a SIFS period. | 18 |
| aMinCAPLength | The minimum number of optical clocks forming the CAP. This ensures that MAC commands can still be transferred to devices when GTSs are being used. An exception to this minimum shall be allowed for the accommodation of the temporary increase in the beacon frame length needed to perform GTS maintenance (as defined in 5.2.2.1.4). | 440 |
| aMinMPDUOverhead | The minimum number of octets added by the MAC sublayer to the PSDU. | 9 |
| aNumSuperframeSlots | The number of slots contained in any superframe. | 16 |
| aUnitBackoffPeriod | The number of optical clocks forming the basic time period used by the unslotted random access algorithm. | 20 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

AckWaitTime =backoff period + aTurnaroundTime-RX-TX + clock period  numSymAckFrame	(4)	47

48

where numSymAckFrame is the number of bits in the acknowledgment frame and is equal to 103 for PHY I              49 and II and 111 for PHY III. For B-ACK mode, the AckWaitTime would be larger, depending on the number              50 of acknowledgments in the B-ACK mode as explained in 5.2.2.2. The clock period is obtained via the              51 optical rates specified in Table 105, Table 106, and Table 107.                                                                                       52

53

54

Copyright © 20xx IEEE. All rights reserved.	215

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The attribute macMaxFrameTotalWaitTime may be set by the next higher layer and is dependent upon a

2	combination of PHY and MAC PIB attributes and constants. The formula relating the attributes and

3	constants is shown in Equation (5).

4

5	macMaxFrameTotalWaitTime =	(5)

6



7	 m – 1

8	  2

9	 k = 0

10	where

11



+

macMinBE + k     2





macMaxBE

– 1   macMaxCSMABackoffs – m

 aUnitBackoffPeriod + phyMaxFrameDuration

12	m is min(macMaxBE-macMinBE, macMaxCSMABackoffs)

13

14

15	Table 91—MAC PIB attribute s

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macAckWaitDuration† | 0x40 | Integer | Refer to Equation (4) | The maximum number of optical clocks to wait for an acknowledgment frame to arrive following a transmitted data frame. This value is dependent on the supported PHY, which determines both the selected logical channel. The calculated value is the time to commence transmitting the ACK plus the length of the ACK frame. The commencement time is described in 5.1.9.4.2. | Dependent on currently selected PHY. |
| macAssociatedOWPAN- Coord | 0x41 | Boolean | TRUE or FALSE | Indication of whether the device is associated to the OWPAN through the coordinator. A value of TRUE indicates the device has associated through the coordinator. Otherwise, the value is set to FALSE. | FALSE |
| macAssociation- Permit | 0x42 | Boolean | TRUE or FALSE | Indication of whether a coordinator is currently allowing association. A value of TRUE indicates that association is permitted. | FALSE |

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

216	Copyright © 20xx IEEE. All rights reserved.

| Table 91—MAC P IB attribute s | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macAutoRequest | 0x43 | Boolean | TRUE or FALSE | Indication of whether a device automatically sends a data request command if its address is listed in the beacon frame. A value of TRUE indicates that the data request command is automatically sent. This attribute also affects the generation of the MLME- BEACON- NOTIFY.indication primitive (see 6.3.3.1.1). | TRUE |
| macBeaconPayload | 0x44 | Set of octets | — | The contents of the beacon payload. | NULL |
| macBeaconPayload- Length | 0x45 | Integer | 0 – aMax- Beacon- PayloadLength | The length, in octets, of the beacon payload. | 0 |
| macBeaconOrder | 0x46 | Integer | 0–15 | Specification of how often the coordinator transmits its beacon. If BO = 15, the coordinator will not transmit a periodic beacon. Refer to 5.1.1 for an explanation of the relationship between the beacon order and the beacon interval. | 15 |
| macBeaconTxTime† | 0x47 | Integer | 0x000000– 0xffffff | The time that the device transmitted its last beacon frame, in symbol periods. The measurement shall be taken at the same symbol boundary within every transmitted beacon frame, the location of which is implementation specific. This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest four bits being the least significant. | 0x000000 |
| macBSN | 0x48 | Integer | 0x00–0xff | The sequence number added to the transmitted beacon frame. | Random value from within the range |
| macCoordExtended- Address | 0x49 | IEEE address | An extended 64-bit IEEE address | The 64-bit address of the coordinator through which the device is associated. | — |

Copyright © 20xx IEEE. All rights reserved.	217

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 91—MAC P IB attribute s  (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macCoordShort-Address | 0x4a | Integer | 0x0000–0xffff | The 16-bit short address assigned to the coordinator through which the device is associated. A value of 0xfffe indicates that the coordinator is only using its 64-bit extended address. A value of 0xffff indicates that this value is unknown. | 0xffff |
| macDSN | 0x4b | Integer | 0x00–0xff | The sequence number added to the transmitted data or MAC command frame. | Random value from within the range |
| macGTSPermit | 0x4c | Boolean | TRUE or FALSE | TRUE if the coordinator is to accept GTS requests. FALSE otherwise. | TRUE |
| macMaxBE | 0x4d | Integer | 3–15 | The maximum value of the backoff exponent, BE, in the unslotted random access algorithm. Refer to 5.1.2.1 for a detailed explanation of the backoff exponent. | 5 |
| macMaxCSMABackoffs | 0x4e | Integer | 0–5 | The maximum number of backoffs the unslotted random access algorithm will attempt before declaring a channel access failure. | 4 |
| macMaxFrameTotal- WaitTime | 0x4f | Integer | Refer to Equation (5) | The maximum number of optical clocks in a beacon- enabled OWPAN, or in a nonbeacon-enabled OWPAN, to wait either for a frame intended as a response to a data request frame or for a broadcast frame following a beacon with the frame pending subfield set to one. This attribute, which shall only be set by the next higher layer, is dependent upon macMinBE, macMaxBE, macMaxCSMABackoffs and the number of optical clocks per octet. Refer to 6.4.2 for the formula relating the attributes. | Dependent on currently selected PHY |
| macMaxFrameRetries | 0x50 | Integer | 0–7 | The maximum number of retries allowed after a transmission failure. | 3 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

218	Copyright © 20xx IEEE. All rights reserved.

| Table 91—MAC P IB attribute s | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macMaxRABackoffs | TBD | Integer | 0-5 | The maximum number of backoffs the unslotted random access algorithm will attempt before declaring a channel access failure. | 4 |
| macMinBE | 0x51 | Integer | 0–macMaxBE | The minimum value of the backoff exponent (BE) in the unslotted random access algorithm. Refer to 5.1.2.1 for a detailed explanation of the backoff exponent. | 3 |
| macMinLIFSPeriod† | 0x52 | Integer | As defined in Table 110 in | The minimum number of optical clocks forming a LIFS period. | Dependent on currently selected PHY. |
| macMinSIFSPeriod† | 0x53 | Integer | As defined in Table 110 in | The minimum number of optical clocks forming a SIFS period. | Dependent on currently selected PHY. |
| macOWPANId | 0x54 | Integer | 0x0000–0xffff | The 16-bit identifier of the OWPAN on which the device is operating. If this value is 0xffff, the device is not associated. | 0xffff |
| macResponseWaitTime | 0x55 | Integer | 2–64 | The maximum time, in multiples of aBaseSuperframeDuration, a device shall wait for a response command frame to be available following a request command frame. | 32 |
| macRxOnWhenIdle | 0x56 | Boolean | TRUE or FALSE | Indication of whether the MAC sublayer is to enable its receiver during idle periods. For a beacon-enabled OWPAN, this attribute is relevant only during the CAP of the incoming superframe. For a nonbeacon-enabled OWPAN, this attribute is relevant at all times. | FALSE |
| macSecurityEnabled | 0x57 | Boolean | TRUE or FALSE | Indication of whether the MAC sublayer has security enabled. A value of TRUE indicates that security is enabled, while a value of FALSE indicates that security is disabled. | TRUE |

Copyright © 20xx IEEE. All rights reserved.	219

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 91—MAC P IB attribute s  (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macShortAddress | 0x58 | Integer | 0x0000–0xffff | The 16-bit address that the device uses to communicate in the OWPAN. If the device is the coordinator, this value shall be chosen before a OWPAN is started. Otherwise, the address is allocated by a coordinator during association. A value of 0xfffe indicates that the device has associated but has not been allocated an address. A value of 0xffff indicates that the device does not have a short address. | 0xffff |
| macSuperframe-Order† | 0x59 | Integer | 0–15 | The length of the active portion of the outgoing superframe, including the beacon frame. If superframe order, SO, = 15, the superframe will not be active following the beacon. Refer to 5.1.1 for an explanation of the relationship between the superframe order and the superframe duration. | 15 |
| macTimestamp- Supported† | 0x5a | Boolean | TRUE or FALSE | Indication of whether the MAC sublayer supports the optional time stamping feature for incoming and outgoing data frames. | Imple- mentation specific |
| macTransaction- PersistenceTime | 0x5b | Integer | 0x0000–0xffff | The maximum time (in unit periods) that a transaction is stored by a coordinator and indicated in its beacon. The unit period is governed by macBeaconOrder, BO, as follows: For 0  BO  14, the unit period will be aBase- SuperframeDuration * 2BO. For BO = 15, the unit period will be aBaseSuperframe- Duration. | 0x01f4 |
| macNumAcks | 0x5c | Integer | 0–15 | Maximum number of times not receiving ACKs to trigger fast link recovery procedure. | 3 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

220	Copyright © 20xx IEEE. All rights reserved.

| Table 91—MAC P IB attribute s | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macLinkTimeOut | 0x5d | Integer | 0–255 | A timer initiated when the link recovery procedure is triggered. If the timer expires while the device has not received any fast link recovery response (FLR RSP) signal since the fast link recovery procedure is triggered, the device assumes that the link is broken and cannot be recovered. The range for macLinkTimeOut is defined in terms of the number of superframes. | 63 |
| macDimDataFailureIndic ation | 0x5e | Boolean | TRUE or FALSE | shall be set to ‘1’ when the device is unable to perform data communication under dimming | 0 |
| macDuringASSOCColor | 0x5f | Unsigned | 0–255 | Use macDuringASSOCColor for the color assignment of the CVD frame when the color function for the association MAC state indication between MLME- ASSOCIATE.request and MLME- ASSOCIATE.confirm is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |
| macDuringDISASSOCCo lor | 0x60 | Unsigned | 0–255 | Use macDuringDISASSOCColor for the color assignment of the CVD frame when the color function for the disassociation MAC state indication between MLME- DISASSOCIATE.request and MLME- DISASSOCIATE.confirm is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |

Copyright © 20xx IEEE. All rights reserved.	221

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macDuringSCANColor | 0x61 | Unsigned | 0–255 | Use macDuringSCANColor for the color assignment of the CVD frame when the color function for the scan MAC state indication between MLME- SCAN.request and MLME- SCAN.confirm is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |
| macColorReceived | 0x62 | Unsigned | 0–255 | Use macColorReceived for the color assignment oft he CVD Frame when the ACK frame is sent and the color function for the ACK state indication is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |
| macColorNotReceived | 0x63 | Unsigned | 0–255 | Use macColorNotReceived for the color assignment of the CVD Frame when the ACK frame is not sent but the color function for the non- ACK state indication is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |
| macCQIColorLFER | 0x64 | Unsigned | 0–255 | Use macCQIColorLFER for the color assignment of the CVD frame when the color function for the channel quality indication showing the low FER is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 91—MAC P IB attribute s  (continued)

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

222	Copyright © 20xx IEEE. All rights reserved.

| Table 91—MAC P IB attribute s | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macCQIColorMFER | 0x65 | Unsigned | 0–255 | Use macCQIColorMFER for the color assignment of the CVD frame when the color function for the channel quality indication showing the medium FER is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |
| macCQIColorHFER | 0x66 | Unsigned | 0–255 | Use macCQIColorHFER for the color assignment of the CVD frame when the color function for the channel quality indication showing the high FER is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |
| macCFAppColor | 0x67 | Unsigned | 0–255 | Use macCFAppColor for the color assignment of the CVD frame when the color function for the indication of application-dependent information is used by the CVD frame. The unsigned integer is the index for the look-up table for the color function table, phyColorFunction, as shown in Table 140, PHY PIB attributes. | 0 |
| macColorStabilizationTi mer | 0x68 | Integer | 0x0–0xffffffff | Minimum time between two stabilization measurements (see 8.6) that are send back to the corresponding CSK Tx. The time is measured in multiples of aMaxPHYFrameSize frames for color stabilization. | 0x004000 00 |
| macTimeStampOffset | 0x69 | Octet | 0x00–0xff | The location of the time stamp after the end of the preamble in optical clocks. | 0 |
|  | 0x6a |  |  |  |  |

Copyright © 20xx IEEE. All rights reserved.	223

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 91—MAC P IB attribute s  (continued)

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
|  | 0x6b |  |  |  |  |
| mac_symbol_rate | 0x6c |  | 5-20 | To adapt TX symbol rate versus RX frame rate variation when RX has the minimum frame rate below the symbol rate. | 10 |
| mac_DS_rate | 0x6d | int | 60 - 120 | To control the length of a data subframe. Shorter data frame can support for longer distance transmission. | 100 |
| macTxMode | 0x6e | Unsigned | 0-255 | This attribute indicates the MAC transmission mode is visible or Invisible. 0 : Visible VTASC Mode 1 : Invisible Mode - Blending Method 2 : Invisible Mode - Watermarking Method | 0 |
| macTxDataType | 0x6f | Unsigned | 0-255 | This attribute indicates the type of data to be transmitted. 0 : Normal Data (Media Content, Information Content based on the Application used for) 1 : ID Data 2 : Authentication Data | 0 |
| maxDataLength | 0x70 | Integer | 0-65535 | This attribute specify the length of the data to be transmitted | 0 |
| macCoordinationTime | 0x71 | Integer | 0-127 | This attribute indicates the number of superframes that consist of a coordination period | 10 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

224	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 91—MAC P IB attribute s  (continued)	1

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macNeighborReportResp onseTime | 0x72 | Integer | 2-128 | The maximum time, in multi- ples of aBaseSuperframeDur- ation, a coordinator shall wait for the neighboring VPAN report indication command frames to be available follow- ing the neighboring VPAN report request command frame. | 64 |
| macAgeingTime | 0x73 | Integer | 0-31 | Ageing time for neighboring OWPAN monitoring records | 10 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

Table 92—Ambig uity Re s o lutio n Me tho d	20

| PIB Attribute Value | Method Name | Method |
| --- | --- | --- |
| 0 | ID without payload | Preappend ID to SSID |
| 1 | ID and SSID hash | Preappend ID to hash resolved SSID |
| 2 | w/ or w/o ID and IP address | Preappend ID to provided IP address |

21

22

23

24

25

26

27

28

29

6.5 Optical-clock-rate selection	30

31

The standard supports multiple optical clock rates in order to accommodate a wide variety of optical sources              32 and receivers. The standard also supports the use of asymmetric clock rates between transmitter and receiver              33 since they constitute independent chains and may support different clock-rate ranges. The multiple clocks              34 associated with each PHY type are respectively shown in Table 105, Table 106, and Table 107.                                   35

36

Support for the minimum clock rate for a given PHY type shall be mandatory for all TX and RX devices. All              37 specified clock rates less than the maximum supported clock rate in a given device shall also be supported in              38 that device. If a clock rate is supported, all data rates associated with that clock rate shall be supported. The              39 preamble, headers, and payload in the PHY shall have the same clock rate. The header shall be sent at lowest              40 data rate for the chosen clock rate. The payload can choose any data rate belonging to the chosen clock rate.              41

42

The clock-rate negotiation can be supported with or without explicit clock-rate negotiation, as indicated in              43 the capabilities information field in Table 37. Explicit clock-rate negotiation implies that the devices shall              44 transmit a clock rate change notification command as in 5.4.19 before a new clock rate is used. If explicit              45 clock-rate negotiation is not used, the device shall have the capability to perform synchronization at all              46 supported optical clock rates without any prior knowledge of the clock rate chosen at the transmitter for              47 communication.                                                                                                                                                                 48

49

6.5.1 Optical-clock-rate selection for P2P topology	50

51

54

Let us assume that Device 1 supports clock rates at the transmitter (C1, C2, … Ct1), where Ct1  is the	52 maximum clock rate supported at the transmitter at of the coordinator. Also, C1 < C2 < ... < Ct1. Within a	53

Copyright © 20xx IEEE. All rights reserved.	225

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	PHY type, the clock rates are integral multiples of each other to make the clock generation and selection

2	simple at the transmitter (i.e., Ci+1/Ci = m, which is an integer). The receiver may support more or less clock

3	rates than the transmitter since the receiver optronics is physically independent of the transmitter clock. Let

4	the clocks supported by the receiver of device 1 be C1, C2, … Cr1, where Cr1 is the maximum clock rate

5	supported at the receiver of device 1. Similarly, let Ct2 and Cr2 be the maximum clock rates supported by the

6	device 2. Support for the lowest clock rate C1 is mandatory at both the transmitter and receiver for all

7	devices i.e., t1, t2, r1, r2  1. For every clock rate, there is an associated set of data rates at the physical layer.

8	This data rate is dependent on the modulation, RLL coding, and FEC used at the physical layer for a given

9	clock rate. Let the data rate be represented by rate{Ci,p}, where Ci is the chosen clock rate and 1  p  N(Ci),

10	where N(Ci) is the number of physical-layer data rates associated with clock rate Ci.

11

12	6.5.1.1 Explicit notification

13

14	In Figure 118, a device sends the association request at the lowest clock C1 at a physical layer data rate

15	rate{C1,k}. The data rate index k is typically chosen to be the lowest data rate to guarantee maximum range

16	and reliability for the given clock rate. In this association request, the device also informs the coordinator of

17	the maximum clock rate supported by its transmitter and receiver (Ct2, Cr2). The maximum-clock-rate

18	information is provided by the capabilities IE as shown in Table 37, which shall also be transmitted during

19	this  association request. The  coordinator receives the  association request  and  compares the  received

20	information about the supported clocks at the device and compares it with its supported clocks. In order for

21	it to communicate, it shall select a clock rate C_SEL1 that is equal to or lower than MAX_C_SEL1, which is

22	the  minimum of  its  maximum transmitter  clock  and  the  maximum receiver clock  supported  by  the

23	coordinator. The decision to use clock rates lower than MAX_C_SEL1 and MAX_C_SEL2 at the coordinator

24	and the device for, respectively, the transmission depends on the performance and throughput needs of the

25	coordinator and the devices. The coordinator also sends an association grant back to the device at the same

26	lowest clock rate C1 supported by all devices. The devices then exchange the selected clock frequencies by

27	using the clock-rate-change notification command for future communication before they switch to the

28	selected clock frequencies. The devices may also decide to  change the clock rate anytime in  future

29	communication, as long as it is below MAX_C_SEL1 and MAX_C_SEL2 for transmission at the coordinator

30	and device, respectively.

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

226	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

coordinator (supports clocks TX: C1,C2,...Ct1

RX: C1,C2,...Cr 1)

MAX _C_SEL1  = min (Cr2, Ct1)

C_SEL1  ≤ MAX _C_SEL1

clock-rate selection for P2P topology

association request (clock C1, rate{C1,k})

inform max clock of device (Ct2, Cr2)

association grant (clock C1, rate{C1,j})

inform max clock of coordinator(Ct1,Cr1)

Dev1 to Dev2 (clock Csel1)

inform selected clock C_SEL1  (clock C1) inform selected clock C_SEL2  (clock C1) device to coordinator(clock C_SEL2) coordinator to device (clock C_SEL1)

1

2

3

device 	4

(supports clocks 	5

TX: C1,C2, ... Ct2 	6

RX: C1,C2,...Cr2) 	7

8

9

10

11

12

13

14

15

MAX_C_SEL 2 = min (Cr1, Ct2) 	16

C_SEL 2 ≤ MAX _C_SEL2 	17

18

19

20

21

22

23

24

25

26

27

28

There is a data rate table rate{Ci,j } associated with each clock rate 	29

Ci where j is the data rate index associated with clock rate Ci 	30

Figure 118—Clock-rate selection for P2P topology (explicit notification)	31

32

33

6.5.1.2 Without explicit notification	34

35

It is also possible for the coordinator to send the association grant at the new clock rate C_SEL2 and not have              36 to explicitly exchange notification information, as shown in Figure 119, if the coordinator has the capability              37 to detect all clock rates less than its maximum receive clock rate. In this case, communication can occur              38 without overhead for explicit notification.                                                                                                                        39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	227

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3	coordinator

4	(supports clocks

5	TX: C1,C2,... Ct1

6	RX: C1,C2,...Cr1)

7

8

9

10

clock-rate selection for P2P topology

association request (clock C1, rate{C1,k})

device

(supports clocks

TX: C1,C2,... Ct2

RX: C1,C2,...Cr2)

11	MAX _C_SEL

= min (Cr , Ct )

inform max clock of device (Ct2,Cr2)

1 	2 	1

12	C_SEL1  ≤ MAX_C_SEL1

13

14

15

16

17

18

19

20

21

22

23

24

25

26

association grant(clock C_SEL1, rate{C_SEL2,j})

inform max clock of coordinator (Ct1,Cr1)

MAX _C_SEL2  = min (Cr1, Ct2)

C_SEL2  ≤ MAX_C_SEL2

device to coordinator(clock C_SEL2)

coordinator to device(clock C_SEL1)

27	There is a data rate table rate{Ci,j} associated with each clock rate

28	Ci where j is the data rate index associated with clock rate Ci

29

30	Figure 119—Clock-rate selection for P2P topology (without explicit notification)

31

32	6.5.2 Optical-clock-rate selection for star topology

33

34	6.5.2.1 Explicit notification

35

36	Figure 120 shows the optical clock-rate selection for a star topology. In this case, let us assume device 1 to

37	be a coordinator. The coordinator will send a broadcast message via a beacon to all nodes, such as devices 2

38	and 3, and inform them of its supported clock rates. The CAP always uses the lowest clock rate C1 for uplink

39	contention. The coordinator and the devices communicate the selected clock frequencies during the CFP

40	using  clock  rate  C1  before  switching  to  the  selected  clock  frequencies. The  information  about  the

41	coordinator capabilities is broadcast using the capabilities IE. The current clock in use, and any change of

42	clock, is communicated via the clock rate change notification.

43

44

45

46

47

48

49

50

51

52

53

54

228	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

coordinator (supports clocks TX: C1,C2,...Ct1

RX: C1,C2,...Cr1)

clock-rate selection for star topology device 2

(supports clocks

TX: C1,C2,... Ct2

RX: C1 ,C2,... Cr2)

1

2

3

4

5

device 3

(supports clocks

TX: C1,C2,...Ct3 	6

RX: C1,C2,... Cr3) 	7

8

beacon (clock C1,rate{C

1,k}) 	9

10

inform max clock of coordinator(Ct1,Cr1) 	11

12

13 contention access period (clock C1,rate{C1,j } ) 		14 inform max clock of device 2 (Ct2,Cr2) 	15

16 contentionaccess  period (clock C1,rate{C1,m}) 		17 inform max clock of device 3 (Ct3,Cr3) 	18

19

C_SEL12  ≤ min (Cr2, Ct1)

C_SEL13  ≤ min (Cr3, Ct1)

C_SEL2  ≤ min (Cr1, Ct2) 	C_SEL3  ≤ min (Cr1, Ct3) 	20

21

22

CFP: inform clock C_SEL12  (clock C1) CFP: inform clock C_SEL2  (clock C1)

CFP: device 2 to coordinator (clock C_SEL2) CFP: coordinator to device 2 (clock C_SEL12) CFP: device 3 to coordinator (clock C_SEL3)

23

CFP: inform clock C_SEL13  (clock C1) 	24

25

CFP: inform clock C_SEL3  (clock C1) 	26

27

28

29

30

CFP: coordinator to device 3 (clock C_SEL13) 	31

32

33

34

35

Figure 120—Clock-rate selection for star topology (explicit notification)	36

37

6.5.2.2 Without explicit notification	38

39

Similar to the P2P topology, the clock-rate selection for the star topology can also occur without explicit	40 notification, as shown in Figure 121.	41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	229

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

coordinator

6	(supports clocks

7	TX: C1,C2,... Ct1

8	RX: C1,C2,...Cr1)

9

clock-rate selection for star topology device 2

(supports clocks

TX: C1,C2,... Ct2

RX: C1,C2,...Cr2)

beacon (clock C1, rate{C1,k})

device 3

(supports clocks

TX: C1,C2 ,...Ct3

RX: C1,C2,... Cr 3)

10	inform max clock of coordinator (Ct1,Cr1)

11

12

C_SEL2 ≤ min (Cr1, Ct2)

C_SEL3   ≤ min (Cr1, Ct3)

13	contention access period (clock C1, rate{C1,j})

14	inform max clock of device 2 (Ct2 ,Cr2)

15

16	contention access period(clock C1,rate{C1,m})

17	inform max clock of device 3 (Ct3 ,Cr3)

18

19	C_SEL12  ≤ min (Cr 2, Ct1)

20	C_SEL13  ≤ min (Cr3, Ct1 )

21

22	CFP : device 2 to coordinator (clock C_SEL2)

23	CFP : coordinator to device 2 (clock C_SEL12)

24	CFP : device 3 to coordinator (clock C_SEL3)

25

26	CFP : coordinator to device3 (clock C_SEL13)

27

28

29

30

31	Figure 121—Clock-rate selection for star topology (without explicit notification)

32

33

34	6.5.3 Clock-rate selection for multicast topology

35

36	6.5.3.1 Explicit notification

37

38	Figure 122 and Figure 123 show the clock-rate selection for multicast topologies assuming bi-directional

39	communication.

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

230	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

coordinator (supports clocks TX: C1,C2, ...Ct1

RX: C1,C2,...Cr1)

clock-rate selection for multicast topology device 2

(supports clocks

TX: C1,C2,...Ct2

RX: C1 ,C2,... Cr2)

beacon (clock C1, rate{C1,j})

1

2

3

4

5

device 3

(supports clocks

TX: C1,C2,... Ct3 	6

RX: C1,C2,...Cr3) 	7

8

9

inform max clock of coordinator (Ct1,Cr1) 	10

11

12

C_SEL2  ≤ min (Cr1, Ct2)	C_SEL3   ≤ min (Cr1, Ct3)	13

14

contention access period(clock C1, rate{C1,j}) 	15

inform max clock of device 2 (Ct2, Cr2) 	16

17

contention access period(clock C1, rate{C1,m}) 	18

inform max clock of device 3 (Ct3,Cr3)

19

C_SEL1  ≤ min (Cr2, Cr3, Ct1) 	20

C_SEL12  ≤ min (Cr2, Ct1) 	21

C_SEL13  ≤ min (Cr3, Ct1) 	22

23

CFP: inform clock change to C_SEL1  (clock C_SEL12) 	24

CFP: inform clock change to C_SEL1  (clock C_SE L13)	25

26

multicast message (clock C_SEL1) 	27

28

29

30

31

Figure 122—Clock-rate selection for multicast (assuming bi-directional communication)	32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	231

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.5.3.2 Without explicit notification

2

3

4

5

6	coordinator

7	(supports clocks

TX: C1,C2,...Ct1

8	RX: C1,C2,...Cr1)

9

clock-rate selection for multicast topology device 2

(supports clocks

TX: C1,C2,...Ct2

RX: C1,C2,...Cr2)

device 3

(supports clocks

TX: C1,C2,...Ct3

RX: C1,C2,...Cr3)

10	beacon (clock C1, rate{C

1,j})

11	inform max clock of coordinator (Ct1, Cr1)

12

13

14	C_SEL2  ≤ min (Cr1, Ct2)

15

16	contention access period(clock C1, rate{C1,j})

17	inform max clock of device 2 (Ct2,Cr2)

18

19	contention access period(clock C1, rate{C1,m})

20	inform max clock of device 3 (Ct3,Cr3)

21

22

23	C_SEL1 ≤ min (Cr2, Cr3, Ct1)

24

25	multicast message (clock C_SEL1)

26

27

28

29

30

C_SEL3   ≤ min (Cr1, Ct3)

31	Figure 123—Clock-rate selection for multicast (bi-directional communication and no

32	explicit notification)

33

34

35	6.6 Message sequence charts illustrating MAC-PHY interaction

36

37	This subclause illustrates the main tasks specified in this standard. Each task is described by use of a

38	message sequence chart to illustrate the chronological order, rather than the exact timing, of the primitives

39	required for each task.

40

41	The primitives necessary for the coordinator to start a new OWPAN are shown in Figure 124. The first

42	action the next higher layer takes after resetting the MAC sublayer is to initiate a scan to search for other

43	OWPANs in the area. An active scan is required. The steps for performing an active scan are shown in

44	Figure 128.

45

46	Once a new OWPAN is established, the coordinator is ready to accept requests from other devices to join the

47	OWPAN. Figure 125 shows the primitives issued by a device requesting association, while Figure 126

48	illustrates the steps taken by a coordinator allowing association. In the process of joining a OWPAN, the

49	device requesting association will perform either a passive or an active scan to determine which OWPANs in

50	the area are allowing association; Figure 127 and Figure 128 detail the primitives necessary to complete a

51	passive scan and an active scan, respectively.

52

53

54

232	Copyright © 20xx IEEE. All rights reserved.

IEEE	RELESS MAC AND PHY SPECIFICATIONS FOR 0IC 	IEEE Draft P802.15.131

The primitives  necessary  for    ansrnitting  and receiving  a single  data frarne are shown  next. The actions

taken by the originator  of the frame are shown in Figure 129while  the actions taken by the recipient are 	2 shown in Figure 130. 	3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright I(J 20xx IEEE.All rights reserved

This is an unapproved IEEE Standards Draftsubject  to change

233

4nM4KmKUUMU1w23456789muMAMmHMMUMmumum

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR 0IC

IEEE Draft P802.15.131D1

device next hìgher layer	device MAC	device I'I I Y

MlME-RëSET.reqllest

(SetDefoll hPI U)	PLMιSET-TRX-STATE.reqllet' (RX ofl)

MI Mt-;. R ESET.confinn

M t.M?St-;l.'ruest

(PAN ld)

Mt.ME.SET.confirl1l

(SUCCtJSS)

Mt.ME.SCAN.ruest

(Active)

t'LME.SET.TRX. STAT.continn

(SUCCliSS)

MLME.SCAN.confirm (SUCCESS)

r nctlve scan

.I.ct WPANld ShortAddre" LogicnlChunnel

MLME.SCAN.rlest

(energj'deleClion)l	X

MLME.SCAN.confirm (SUCCESS)

MlME SET.requ.'t

(Short ddre  )

MlME-SET.confirm

(SUCCESS)

ml el

detecríon sco.n

MLME.START.r\Icsr	PLME.SE.T.rcqucst

(phyClIrrcntCh.oncl)

PLME.SET.col1firm

(SUCCESS)

PLME.SET-TRX-STATE.rcqucst

(TX 011)

PlMιSET.TRX.STAl'E.confirm

(SUCCt;SS)

PD.DATA request

MlME.STARTconfirrn	PD.DATAconform (SUCC ESS) (SUCCESS)'

h '{/ω11

PLME.SET-TRX-$TATE.request

(Rxol1)

I'I.Mf.SET'.rRX.STAn;.conrm

(SUCCESS)

•	•	•

I The energy detection $cnn is optionnl

'The M IM tJ.START  confirm and t'LM tJ.S6T.R' X.STA1Yrequest pri ll1itives ll1ay not need to be in the Ol'dcr show"

Figure 124-0WPAN start message sequence chart-coordinator

234

Copyright @ 20xx IEEE. All rights reserved

This is an unapproved IEEE Standards Draftsuect to change.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC 	IEEE Draft P802.15.13/DO

ðe"ice I'IIY	de\'ice MAC 	de\.ìce next higher layer

ML .IE.SCAN.conrm (SUCCESS)

sclcct a PAN bascd on list of PAN dcscriptors and sc  MACPIB attributcS accordíngly

PD-DATA.confiml (SUCCESS)

PL 1E-SET-TR.X-STATE.T<uesc

(RX on)

PLMSET-TRX-STATE.confinn

adr.now/ed}!menl

clula .e (Juej'l

PD-OATAÎndica\ion

macRespon   WailTÎme

PD-DATA.confinn (SUCCESS)

PLME-SET-TRX-SrATE.ruesc

ii

]'LME-$fT-TRX-STAn:.conm

S CbSS)

ackll Clwledgmenr

(lSSOClalHHI resI.lse

f1[)-DA  A.indicatiOIl

PD-OA "A.indìcatiotl

PL!l.;-SE-TKX-STATIuest

j

I'LME-SET-TRX-STATE.confirm

AiA1i1i3if.O

l

PJ)-I)AγA quest

'

I)[)-I)ATA.oonfiml (SVCChSS)

PL .1E-S .T-TIX-STAT!;    'Ques

σJ{X 0(1)

J AF33A.3131OQJ11AJd3i1i1AAAi-----f1i1Fi1i1i1OOV/QOQJ//Uo"U"U"1QOQJAUUAv11UO1F-QAF-O

I'LM-SE1TX-$TA1'Ii.confirm

(SUCCESS)

M L\1E.ASSOCIATE.confirm

Figure 125-Association message sequence chart-device

Cop  ight i )20xx IEEE. All rights rese   ed

This is an unapproved IEEE Standards Draftsubject to change.

235

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48	Figure 126—Association message sequence chart—coordinator

49

50

51

52

53

54

236	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

Figure 127—Passive scan message sequence chart	47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	237

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46	Figure 128—Active scan message sequence chart

47

48

49

50

51

52

53

54

238	Copyright © 20xx IEEE. All rights reserved.

IE EE WI RELESS MAC AN D PHY SPE CI FI CATI ONS FOR OWC	1 EEE Dra1t P802.15.131DO

OrlS"lator

l1exr hisher layer

ongl nmor

MAC

onglllAlor

PIIY

| MCPS.DATA quest | PLM E.SETT. RX.STATE.rcqucst (RX on) |  | A perform CCA datll |
| --- | --- | --- | --- |
| MCI'S-I)ATA.confíl'll1 |  |  |  |
|  | PLME.SETT. RX.STATE.confi l'ln (SUCCιSS) |  |  |
|  | 1t' _M E.CCA.requesl |  |  |
|  | PLM E.CCA.col1fol'll1 (SUCCESS) |  |  |
|  | PLME.CCA .request |  |  |
|  | PLMli CCA.conrorm (SUCCliSS) |  |  |
|  | PL:'vIE.SET.TRX-STATE.rcqucst (TX 011) |  |  |
|  | PLM E-SET-TRX-STATE.conrorm (SUCCESS) |  |  |
|  | PD-DATA.rcqucsl |  |  |
|  | PD. DATA.col1fim (SUCCESS) |  |  |
|  | PlME-SET.STATE.rcqucsl (RX on) |  | rnοwlcdgfllClI1 (ïf req uested) |
|  | PLME-SET-TRX-STATE.conûrm (SUCCESS) |  |  |
|  | I'I)-I)AγA.indicatiOIl I'I. M 1'.-51'.1-'1'1{ X-S1ATI'..quest ('1'KX 0 |  |  |
|  | PLM E.SET.TR X.STATE.confirm (SUCCESS) |  |  |

'-'-'-'-'-'-'-'-'-'-93939393939393939393AAAAAAAvAAA'JJJJ-J93AnyJ4U

AUv'''--ni99-'33y-AA93A9n3J4UuJ4UA

•	•

Figure 129-Data-transmission  message sequence chart-originator

Copyright 10 2    x IEEE. All rights reseIVed

This is an unapproved IEEE Standards Draftsubject to change

239

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40	Figure 130—Data-transmission message sequence chart—recipient

41

42

43

44

45	7. Security suite specifications

46

47

48	7.1 Overview

49

50	The MAC sublayer is responsible for providing security services on specified incoming and outgoing frames

51	when requested to do so by the higher layers. This standard supports the following security services (as

52	defined in 4.6 for definitions):

53

54	—	Data confidentiality

240	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

—	Data authenticity	1

—	Replay protection	2

3

The information determining how to provide the security is found in the security-related PIB (as defined in	4

Table 98 in 7.5.1).	5

6

7

7.2 Functional description	8

9

A device may optionally implement security. A device that does not implement security shall not provide a	10

mechanism for the MAC sublayer to perform any cryptographic transformation on incoming and outgoing	11

frames nor require any PIB attributes associated with security. A device that implements security shall	12

provide a mechanism for the MAC sublayer to provide cryptographic transformations on incoming and	13

outgoing   frames   using   information   in   the   PIB   attributes   associated   with   security   when   the	14

macSecurityEnabled attribute is set to TRUE.	15

16

If the MAC sublayer is required to transmit a frame or receives an incoming frame, the MAC sublayer shall	17

process the frame as specified in 7.2.1 and 7.2.3, respectively.	18

19

7.2.1 Outgoing frame security procedure	20

21

The inputs to this procedure are the frame to be secured and the SecurityLevel, KeyIdMode, KeySource, and	22

KeyIndex parameters from the originating primitive or automatic request PIB attributes. The outputs from	23

this procedure are the status of the procedure and, if this status is SUCCESS, the secured frame. If outgoing	24

frame security procedure is not successful, the frame is discarded	25

26

The outgoing frame security procedure involves the following steps as applicable:	27

28 a)	If the Security Enabled subfield of the frame control field of the frame to be secured is set to zero,		29 the procedure shall set the security level to zero.	30

b)     If the Security Enabled subfield of the frame control field of the frame to be secured is set to one, the              31 procedure shall set the security level to the SecurityLevel parameter. If the resulting security level is              32 zero, the procedure shall return with a status of UNSUPPORTED_SECURITY.                                               33

c)	If the macSecurityEnabled attribute is set to FALSE and the security level is not equal to zero, the	34 procedure shall return with a status of UNSUPPORTED_SECURITY.	35

d)	The procedure shall determine whether the frame to be secured satisfies the constraint on the	36 maximum length of MAC frames, as follows:	37

1)	The procedure shall set the length M, in octets, of the Authentication field to zero if the security              38 level is equal to zero and shall determine this value from the security level and Table 96              39 otherwise.	40

2)	The procedure shall determine the length AuxLen, in octets, of the auxiliary security header	41 (see 7.4) using KeyIdMode and the security level.	42

3)	The procedure shall determine the data expansion as AuxLen + M.	43

4)	The procedure shall check whether the length of the frame to be secured, including data	44

expansion and FCS, is less than or equal to aMaxPHYFrameSize. If this check fails, the	45

procedure shall return with a status of FRAME_TOO_LONG.	46

e)	If the security level is zero, the procedure shall set the secured frame to be the frame to be secured	47

and return with the secured frame and a status of SUCCESS.	48

f)	The procedure shall set the frame counter to the macFrameCounter attribute. If the frame counter	49

has the value 0xffffffff, the procedure shall return with a status of COUNTER_ERROR and all keys	50

associated with the device shall be reinitialized and updated as discussed in 7.5.5.	51

g)	The procedure shall obtain the key using the outgoing frame key retrieval procedure as described in	52

7.2.2. If that procedure fails, the procedure shall return with a status of UNAVAILABLE_KEY.	53

h)	The procedure shall insert the auxiliary security header into the frame, with fields set as follows:	54

Copyright © 20xx IEEE. All rights reserved.	241

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	1)	The Security Level subfield of the Security Control field shall be set to the security level.

2	2)	The Key Identifier Mode subfield of the Security Control field shall be set to the KeyIdMode

3	parameter.

4	3)	The Frame Counter field shall be set to the frame counter.

5	4)	If the KeyIdMode parameter is set to a value not equal to zero, the Key Source and Key Index

6	subfields of the Key Identifier field shall be set to the KeySource and KeyIndex parameters,

7	respectively.

8	i)	The procedure shall then use aExtendedAddress, the frame counter, the security level, and the key to

9	produce the  secured frame according to  the  transformation process known as  CCM* [or  the

10	extension of CCM, which is the combined counter with CBC-MAC (i.e., cipher block chaining

11	message authentication code) mode of operation] that is described in the security operations (see

12	7.3.4).

13	1)	If the SecurityLevel parameter specifies the use of encryption (as defined in Table 96 in

14	7.4.2.1), the encryption operation shall be applied only to the actual payload field within the

15	MSDU, i.e., the Beacon Payload field (see 5.2.2.1.9), Command Payload field (see 5.2.2.4.3),

16	or Data Payload field (see 5.2.2.2.2), depending on the frame type. The corresponding payload

17	field is passed to the CCM* transformation process described in 7.3.4 as the unsecured payload

18	(as defined in Table 93 in 7.3.4.2). The resulting encrypted payload shall substitute the original

19	payload.

20	2)	The  remaining  fields  in  the  MSDU  part  of  the  frame  shall  be  passed  to  the  CCM*

21	transformation process described in 7.3.4 as the nonpayload fields (see Table 93).

22	3)	The ordering and exact manner of performing the encryption and integrity operations and the

23	placement of the resulting encrypted data or integrity code within the MSDU field shall be as

24	defined in 7.3.4.

25	j)	The procedure shall increment the frame counter by one and set the macFrameCounter attribute to

26	the resulting value.

27	k)	The procedure shall return with the secured frame and a status of SUCCESS.

28

29	7.2.2 Outgoing frame key retrieval procedure

30

31	The inputs to this procedure are the frame to be secured and the KeyIdMode, KeySource, and KeyIndex

32	parameters from the originating primitive. The outputs from this procedure are a passed or failed status and,

33	if passed, a key.

34

35	The outgoing frame key retrieval procedure involves the following steps as applicable:

36

37	a)	If  the  KeyIdMode parameter is  set  to  0x00  (implicit key  identification), the  procedure shall

38	determine the key lookup data and key lookup size as follows:

39	1)	If the Destination Addressing Mode subfield of the frame control field of the frame is set to

40	0x00 and the macOWPANCoordShortAddress attribute is set to a value in the range 0x0000–

41	0xfffd (i.e., the short address is used), the key lookup data shall be set to the 2-octet Source

42	OWPAN Identifier field of the frame right-concatenated (see B.2.1. of IEEE Std 802.15.4-

43	2006) with the 2-octet macOWPANCoordShortAddress attribute right-concatenated with the

44	single octet 0x00. The key lookup size shall be set to five.

45	2)	If the Destination Addressing Mode subfield of the frame control field of the frame is set to

46	0x00 and the macOWPANCoordShortAddress attribute is set to 0xfffe (i.e., the extended

47	address	is	used),	the	key	lookup	data	shall	be	set	to	the	8-octet

48	macOWPANCoordExtendedAddress  attribute  right-concatenated  (see  B.2.1  of  IEEE  Std

49	802.15.4-2006) with the single octet 0x00. The key lookup size shall be set to nine.

50	3)	If the Destination Addressing Mode subfield of the frame control field of the frame is set to

51	0x02, the key lookup data shall be set to the 2-octet Destination OWPAN Identifier field of the

52	frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet Destination

53	Address field of the frame right-concatenated with the single octet 0x00. The key lookup size

54	shall be set to five.

242	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| 4) If the Destination Addressing Mode subfield of the frame control field of the frame is set to | 1 |
| --- | --- |
| 0x03, the key lookup data shall be set to the 8-octet Destination Address field of the frame | 2 |
| right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet 0x00. The key | 3 |

lookup size shall be set to nine.	4

b)	If the KeyIdMode parameter is set to a value not equal to 0x00 (explicit key identification), the	5 procedure shall determine the key lookup data and key lookup size as follows:	6

1)	If the KeyIdMode parameter is set to 0x01, the key lookup data shall be set to the 8-octet              7 macDefaultKeySource attribute right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with              8 the single octet KeyIndex parameter. The key lookup size shall be set to nine.                                          9

2)	If the KeyIdMode parameter is set to 0x02, the key lookup data shall be set to the 4-octet	10

KeySource parameter right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single	11 octet KeyIndex parameter. The key lookup size shall be set to five.	12

3)	If the KeyIdMode parameter is set to 0x03, the key lookup data shall be set to the 8-octet	13

KeySource parameter right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single	14 octet KeyIndex parameter. The key lookup size shall be set to nine.	15

16

c)	The procedure shall obtain the KeyDescriptor by passing the key lookup data and the key lookup	17

size to the KeyDescriptor lookup procedure as described in 7.2.5. If that procedure returns with a	18

failed status, this procedure shall also return with a failed status.	19

d)	The MAC sublayer shall set the key to the Key element of the KeyDescriptor.	20 e)	The procedure shall return with a passed status, having obtained the key identifier and the key.	21

22

NOTE—For broadcast frames, the outgoing frame key retrieval procedure will result in a failed status if implicit key	23

identification is used. Hence, explicit key identification should be used for broadcast frames.	24

25

7.2.3 Incoming frame security procedure	26

27

The input to this procedure is the frame to be unsecured. The outputs from this procedure are the unsecured	28

frame, the security level, the key identifier mode, the key source, the key index, and the status of the	29

procedure. All outputs of this procedure are assumed to be invalid unless and until explicitly set in this	30

procedure. It is assumed that the PIB attributes associating KeyDescriptors in macKeyTable with a single,	31

unique device or a number of devices will have been established by the next higher layer. The incoming	32

frame security procedure involves the following steps:	33

34 a)	If the Security Enabled field of the frame control field of the frame to be unsecured is set to zero, the		35 procedure shall set the security level to zero.	36

b)	If the Security Enabled field of the Frame Control field of the frame to be unsecured is set to one, the	37 procedure shall set the unsecured frame to be the frame to be unsecured and return with a status of	38

UNSUPPORTED_LEGACY.		39 c)	If the Security Enabled field of the Frame Control field of the frame to be unsecured is set to one, the		40 procedure shall set the security level and the key identifier mode to the corresponding fields of the	41

Security Control field of the auxiliary security header of the frame to be unsecured, and the key              42 source and key index to the corresponding fields of the Key Identifier field of the auxiliary security              43 header of the frame to be unsecured, if present. If the resulting security level is zero, the procedure              44 shall  set  the  unsecured frame  to  be  the  frame  to  be  unsecured and  return  with  a  status  of              45

UNSUPPORTED_SECURITY.		46 d)	If the macSecurityEnabled attribute is set to FALSE, the procedure shall set the unsecured frame to		47 be the frame to be unsecured and return with a status of SUCCESS if the security level is equal to	48

zero and with a status of UNSUPPORTED_SECURITY otherwise.		49 e)	The procedure shall determine whether the frame to be unsecured meets the minimum security level		50 by passing the security level, the frame type, and, depending on whether the frame is a MAC	51 command frame, the first octet of the MSDU (i.e., command frame identifier for a MAC command	52 frame) to the incoming security level checking procedure as described in 7.2.8. If that procedure	53

54

Copyright © 20xx IEEE. All rights reserved.	243

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	fails, the procedure shall set the unsecured frame to be the frame to be unsecured and return with a

2	status of IMPROPER_SECURITY_LEVEL.

3	f)	If the security level is set to zero, the procedure shall set the unsecured frame to be the frame to be

4	unsecured and return with a status of SUCCESS.

5	g)	The procedure shall obtain the KeyDescriptor, DeviceDescriptor, and KeyDeviceDescriptor using

6	the incoming frame security material retrieval procedure described in 7.2.4. If that procedure fails,

7	the procedure shall set the unsecured frame to be the frame to be unsecured and return with a status

8	of UNAVAILABLE_KEY.

9	h)	The procedure shall determine whether the frame to be unsecured conforms to the key usage policy

10	by passing the KeyDescriptor, the frame type, and, depending on whether the frame is a MAC

11	command frame, the first octet of the MSDU (i.e., command frame identifier for a MAC command

12	frame) to the incoming key usage policy checking procedure as described in 7.2.9. If that procedure

13	fails, the procedure shall set the unsecured frame to be the frame to be unsecured and return with a

14	status of IMPROPER_KEY_TYPE.

15	i)	If the Exempt element of the DeviceDescriptor is set to FALSE and if the incoming security level

16	checking procedure of step e) above had as output the “conditionally passed” status, the procedure

17	shall  set  the  unsecured frame  to  be  the  frame  to  be  unsecured and  return  with  a  status  of

18	IMPROPER_SECURITY_LEVEL.

19	j)	The procedure shall set the frame counter to the Frame Counter field of the auxiliary security header

20	of the frame to be unsecured. If the frame counter has the value 0xffffffff, the procedure shall set the

21	unsecured frame to be the frame to be unsecured and return with a status of COUNTER_ERROR.

22	k)	The  procedure  shall  determine  whether  the  frame  counter  is  greater  than  or  equal  to  the

23	FrameCounter element of the DeviceDescriptor. If this check fails, the procedure shall set the

24	unsecured frame to be the frame to be unsecured and return with a status of COUNTER_ERROR.

25	l)	The procedure shall then use the ExtAddress element of the DeviceDescriptor, the frame counter, the

26	security level, and the Key element of the KeyDescriptor to produce the unsecured frame according

27	to the CCM* inverse transformation process described in the security operations (see 7.3.5).

28	1)	If the security level specifies the use of encryption (as defined in Table 96 in 7.4.2.1), the

29	decryption operation shall be applied only to the actual payload field within the MSDU, i.e., the

30	Beacon Payload field (see 5.2.2.1.9), Command Payload field (see 5.2.2.4.3), or Data Payload

31	field (see 5.2.2.2.2), depending on the frame type. The corresponding payload field shall be

32	passed to the CCM* inverse transformation process described in 7.3.5 as the secure payload.

33	2)	The remaining fields in the MSDU part of the frame shall be passed to the CCM* inverse

34	transformation process described in 7.3.5 as the nonpayload fields.

35	m)	If the CCM* inverse transformation process fails, the procedure shall set the unsecured frame to be

36	the frame to be unsecured and return with a status of SECURITY_ERROR.

37	n)	The procedure shall increment the frame counter by one and set the FrameCounter element of the

38	DeviceDescriptor to the resulting value.

39	o)	If the FrameCounter element is equal to 0xffffffff, the procedure shall set the Blacklisted element of

40	the KeyDeviceDescriptor.

41	p)	The procedure shall return with the unsecured frame and a status of SUCCESS.

42

43	7.2.4 Incoming frame security material retrieval procedure

44

45	The input to this procedure is the frame to be unsecured. The outputs from this procedure are a passed or

46	failed status and, if passed, a KeyDescriptor, a DeviceDescriptor, and a KeyDeviceDescriptor.

47

48	The incoming frame security material retrieval procedure involves the following steps as applicable:

49

50	a)	If the Key Identifier Mode subfield of the Security Control field of the auxiliary security header of

51	the frame is set to 0x00 (implicit key identification), the procedure shall determine the key lookup

52	data and the key lookup size as follows:

53	1)	If the source address mode of the frame control field of the frame is set to 0x00 and the

54	macOWPANCoordShortAddress attribute is set to a value in the range 0x0000–0xfffd (i.e., the

244	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

short address is used), the key lookup data shall be set to the 2-octet Destination OWPAN	1

Identifier field of the frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the	2

2-octet macOWPANCoordShortAddress attribute right-concatenated with the single octet 0x00.	3

The key lookup size shall be set to five.	4

2)	If the source address mode of the frame control field of the frame is set to 0x00 and the              5 macOWPANCoordShortAddress attribute is set to 0xfffe (i.e., the extended address is used), the              6 key lookup data shall be set to the 8-octet macOWPANCoordExtendedAddress attribute right-              7 concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the single octet 0x00. The key lookup              8 size shall be set to nine.                                                                                                                                9

3)	If the source address mode of the frame control field of the frame is set to 0x02, the key lookup              10 data shall be set to the 2-octet Source OWPAN Identifier field of the frame, right-concatenated              11 (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet Source Address field of the frame right-              12 concatenated with the single octet 0x00. The key lookup size shall be set to five.                                     13

4)	If the source address mode of the frame control field of the frame is set to 0x03, the key lookup              14 data shall be set to the 8-octet Source Address field of the frame right-concatenated (see B.2.1              15 of IEEE Std 802.15.4-2006) with the single octet 0x00. The key lookup size shall be set to nine.              16

b)	If the Key Identifier Mode subfield of the Security Control field of the auxiliary security header of              17 the frame is set to a value not equal to 0x00 (explicit key identification), the procedure shall              18 determine the key lookup data and key lookup size as follows:                                                                          19

1)	If the key identifier mode is set to 0x01, the key lookup data shall be set to the 8-octet              20 macDefaultKeySource attribute right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with              21 the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. The              22 key lookup size shall be set to nine.                                                                                                             23

2)	If the key identifier mode is set to 0x02, the key lookup data shall be set to the right-              24 concatenation (see B.2.1 of IEEE Std 802.15.4-2006) of the 4-octet Key Source subfield and              25 the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. The              26 key lookup size shall be set to five.                                                                                                              27

3)	If the key identifier mode is set to 0x03, the key lookup data shall be set to the right-              28 concatenation (see B.2.1 of IEEE Std 802.15.4-2006) of the 8-octet Key Source subfield and              29 the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. The              30 key lookup size shall be set to nine.                                                                                                             31

c)	The procedure shall obtain the KeyDescriptor by passing the key lookup data and the key lookup              32 size to the KeyDescriptor lookup procedure as described in 7.2.5. If that procedure returns with a              33 failed status, the procedure shall also return with a failed status.                                                                        34

d)	The procedure shall determine the device lookup data and the device lookup size as follows:	35

|  | 1) | If the source address mode of the frame control field of the frame is set to 0x00 and the |
| --- | --- | --- |
|  |  | macOWPANCoordShortAddress attribute is set to a value in the range 0x0000–0xfffd (i.e., the |
|  |  | short address is used), the device lookup data shall be set to the 2-octet Destination OWPAN |
|  |  | Identifier field of the frame right-concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the |
|  |  | 2-octet macOWPANCoordShortAddress attribute. The device lookup size shall be set to four. |
|  | 2) | If the source address mode of the frame control field of the frame is set to 0x00 and the |
|  |  | macOWPANCoordShortAddress attribute is set to 0xfffe (i.e., the extended address is used), the |
|  |  | device lookup data shall be set to the 8-octet macOWPANCoordExtendedAddress attribute. The |
|  |  | device lookup size shall be set to eight. |
|  | 3) | If the source address mode of the frame control field of the frame is set to 0x02, the device |
|  |  | lookup data shall be set to the 2-octet Source OWPAN Identifier field of the frame, right- |
|  |  | concatenated (see B.2.1 of IEEE Std 802.15.4-2006) with the 2-octet Source Address field of |
|  |  | the frame. The device lookup size shall be set to four. |
|  | 4) | If the source address mode of the frame control field of the frame is set to 0x03, the device |
|  |  | lookup data shall be set to the 8-octet Source Address field of the frame. The device lookup size |
|  |  | shall be set to eight. |
| e) | Th | e procedure shall obtain the DeviceDescriptor and the KeyDeviceDescriptor by passing the |
|  | Ke | yDescriptor, the device lookup data, and the device lookup size to the blacklist checking |

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	245

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	procedure as described in 7.2.6. If that procedure returns with a failed status, the procedure shall also

2	return with a failed status.

3	f)	The  procedure  shall  return  with  a  passed  status  having  obtained  the  KeyDescriptor,  the

4	DeviceDescriptor, and the KeyDeviceDescriptor.

5

6	7.2.5 Key descriptor lookup procedure

7

8	The inputs to this procedure are the key lookup data and the key lookup size. The outputs from this

9	procedure are a passed or failed status and, if passed, a KeyDescriptor.

10

11	The KeyDescriptor lookup procedure involves the following steps as applicable:

12

13	a)	For each KeyDescriptor in the macKeyTable attribute and for each KeyIdLookupDescriptor in the

14	KeyIdLookupList of the KeyDescriptor, the procedure shall check whether the LookupDataSize

15	element of the KeyIdLookupDescriptor indicates the same integer value, shown in Table 104, as the

16	key lookup size and whether the LookupData element of the KeyIdLookupDescriptor is equal to the

17	key lookup data. If both checks pass (i.e., there is a match), the procedure shall return with this

18	(matching) KeyDescriptor and a passed status.

19	b)	The procedure shall return with a failed status.

20

21	7.2.6 Blacklist checking procedure

22

23	The inputs to this procedure are the KeyDescriptor, the device lookup data, and the device lookup size. The

24	outputs  from  this  procedure are  a  passed  or  failed  status  and,  if  passed,  a  DeviceDescriptor and  a

25	KeyDeviceDescriptor.

26

27	The blacklist checking procedure involves the following steps as applicable:

28

29	a)	For each KeyDeviceDescriptor in the KeyDeviceList of the KeyDescriptor:

30	1)	The procedure shall obtain the DeviceDescriptor using the DeviceDescriptorHandle element of

31	the KeyDeviceDescriptor.

32	2)	If the UniqueDevice element of the KeyDeviceDescriptor is set to TRUE, the procedure shall

33	return  with  the  DeviceDescriptor,  the  KeyDeviceDescriptor,  and  a  passed  status  if  the

34	BlackListed element of the KeyDeviceDescriptor is set to FALSE, or the procedure shall return

35	with a failed status if this Blacklisted element is set to TRUE.

36	3)	If the UniqueDevice element of the KeyDeviceDescriptor is set to FALSE, the procedure shall

37	execute the DeviceDescriptor lookup procedure as described in 7.2.7, with the device lookup

38	data and the device lookup size as inputs. If the corresponding output of that procedure is a

39	passed status, the procedure shall return with the DeviceDescriptor, the KeyDevice-Descriptor,

40	and a passed status if the Blacklisted element of the KeyDeviceDescriptor is set to FALSE, or the

41	procedure shall return with a failed status if this Blacklisted element is set to TRUE.

42	b)	The procedure shall return with a failed status.

43

44	7.2.7 Device descriptor lookup procedure

45

46	The inputs to this procedure are the DeviceDescriptor, the device lookup data, and the device lookup size.

47	The output from this procedure is a passed or failed status.

48

49	The DeviceDescriptor lookup procedure involves the following steps as applicable:

50

51	a)	If the device lookup size is four and the device lookup data is equal to the OWPAN ID element of

52	the  DeviceDescriptor  right-concatenated  (see  B.2.1  of  IEEE  Std  802.15.4-2006)  with  the

53	ShortAddress element of the Device-Descriptor, this procedure shall return with a passed status.

54

246	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

b)	If the device lookup size is eight and the device lookup data is equal to the ExtAddress element of	1 the DeviceDescriptor, this procedure shall return with a passed status.	2

c)	The procedure shall return with a failed status.	3

4

7.2.8 Incoming security level checking procedure	5

6

The inputs to this procedure are the incoming security level, the frame type and the command frame	7 identifier. The output from this procedure is a passed, failed, or “conditionally passed” status.	8

9

The incoming security level checking procedure involves the following steps as applicable:	10

11 a)	It is recommended to use MIC for all secure messages as defined in Table 96. For each	12

SecurityLevelDescriptor in the macSecurityLevelTable attribute:	13

1)	If the frame type is not equal to 0x03 and the frame type is equal to the FrameType element of	14 the SecurityLevelDescriptor, the procedure shall compare the incoming security level (as	15

SEC1)  with  the  SecurityMinimum  element  of  the  SecurityLevelDescriptor  (as  SEC2)	16 according to the algorithm described in 7.4.2.1. If this comparison fails (i.e., evaluates to	17

FALSE),   the   procedure   shall   return   with   a   “conditionally   passed”   status   if   the	18

DeviceOverrideSecurityMinimum element of the SecurityLevelDescriptor is set to TRUE and	19 the security level is set to zero and with a failed status otherwise.	20

2)	If the frame type is equal to 0x03, the frame type is equal to the FrameType element of the	21

SecurityLevelDescriptor, and the command frame identifier is equal to the CommandFrame-	22

Identifier element of the SecurityLevelDescriptor, the procedure shall compare the incoming	23 security level (as SEC1) with the SecurityMinimum element of the SecurityLevelDescriptor (as	24

SEC2) according to the algorithm described in 7.4.2.1. If this comparison fails (i.e., evaluates	25 to  FALSE),  the  procedure  shall  return  with  a  “conditionally  passed”  status  if  the	26

DeviceOverrideSecurityMinimum element of the SecurityLevelDescriptor is set to TRUE and	27 the security level is set to zero and with a failed status otherwise.	28

b)	The procedure shall return with a passed status.	29

30

7.2.9 Incoming key usage policy checking procedure	31

32

The inputs to this procedure are the KeyDescriptor, the frame type, and the command frame identifier. The	33 output from this procedure is a passed or failed status.	34

35

The incoming key usage policy checking procedure involves the following steps as applicable:	36

37

a)	For each KeyUsageDescriptor in the KeyUsageList of the KeyDescriptor:	38

1)	If the frame type is not equal to 0x03 and the frame type is equal to the FrameType element of	39 the KeyUsageDescriptor, the procedure shall return with a passed status.	40

2)	If the frame type is equal to 0x03, the frame type is equal to the FrameType element of the	41

KeyUsageDescriptor, and the command frame identifier is equal to the CommandFrame-	42

Identifier element of the KeyUsageDescriptor, the procedure shall return with a passed status.	43 b)	The procedure shall return with a failed status.	44

45

46

7.3 Security operations	47

48

This subclause describes the parameters for the CCM* security operations, as specified in Annex A of IEEE	49

Std 802.15.4-2006.	50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	247

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	7.3.1 Integer and octet representation

2

3	The integer and octet representation conventions specified in Annex A of IEEE Std 802.15.4-2006 are used

4	throughout 7.3.

5

6	7.3.2 CCM* nonce

7

8	The CCM* nonce is a 13-octet string and is used for the advanced encryption standard (AES)-CCM* mode

9	of operation (see B.2.2 of IEEE Std 802.15.4-2006). The nonce shall be formatted as shown in Figure 131,

10	with the left most field in the figure defining the first (and left most) octets and the right most field defining

11	the last (and right most) octet of the nonce.

12

| Octets: 8 | 4 | 1 |
| --- | --- | --- |
| Source address | Frame counter | Security level |

13

14

15

16

17

18	Figure 131—CCM* nonce

19

20

21	The source address shall be set to the extended address aExtendedAddress of the device originating the

22	frame, the frame counter to the value of the respective field in the auxiliary security header (see 7.4), and the

23	security level to the security level identifier corresponding to the Security Level subfield of the Security

24	Control field of the auxiliary security header as defined in Table 96.

25

26	The source address, frame counter, and security level shall be represented as specified in 7.3.1.

27

28	7.3.3 CCM* prerequisites

29

30	Securing a frame involves the use of the CCM* mode encryption and authentication transformation, as

31	described in  B.4.1. of  IEEE Std 802.15.4-2006. Unsecuring a  frame involves the  use of  the  CCM*

32	decryption and authentication checking process, as described in B.4.2 of IEEE Std 802.15.4-2006. The

33	prerequisites for the CCM* forward and inverse transformations are as follows:

34

35	—	The underlying block cipher shall be the AES encryption algorithm as specified in B.3.1 of IEEE Std

36	802.15.4-2006.

37	—	The bit ordering shall be as defined in 7.3.1.

38

39	—	The length in octets of the Length field L shall be 2 octets.

40	—	The length of the Authentication field M shall be 0 octets, 4 octets, 8 octets, or 16 octets, as required.

41

42	The length of the Authentication field M for the CCM* forward transformation and the CCM* inverse

43	transformation is determined from Table 96, using the Security Level subfield of the Security Control field

44	of the auxiliary security header of the frame.

45

46	7.3.4 CCM* transformation data representation

47

48	This subclause describes how the inputs and output of the CCM* forward transformation, as described in

49	B.4.1 of IEEE Std 802.15.4-2006, are formed.

50

51	The inputs are as follows:

52

53	—	Key

54	—	Nonce

248	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

—	a data	1

—	m data	2

3

The output is c data.	4

5

7.3.4.1 Key and nonce data inputs	6

7

The Key data for the CCM* forward transformation is passed by the outgoing frame security procedure	8 described in 7.2.1. The nonce data for the CCM* transformation is constructed as described in 7.3.2.	9

10

7.3.4.2 a data and m data	11

12

In the CCM* transformation process, the data fields shall be applied as in Table 93.	13

14

15

Table 93—a data and m data fo r all s e c urity le ve ls	16

17

| Security level identifier | a data | m data |
| --- | --- | --- |
| 0x00 | None | None |
| 0x01 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ Unsecured payload fields | None |
| 0x02 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ Unsecured payload fields | None |
| 0x03 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ Unsecured payload fields | None |
| 0x04 | None | Unsecured payload fields |
| 0x05 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields | Unsecured payload fields |
| 0x06 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields | Unsecured payload fields |
| 0x07 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields | Unsecured payload fields |

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

7.3.4.3 c data output	36

37

In the CCM* transformation process, the data fields that are applied, or right-concatenated and applied,	38 represent octet strings.	39

40

The secured payload fields right-concatenated with the authentication tag shall substitute the unsecured	41 payload field in the original unsecured frame to form the secured frame (see Table 94).	42

43

44

Table 94—c data fo r all s e c urity le ve ls	45

| Security level identifier | c data |
| --- | --- |
| 0x00 | None |
| 0x01 | MIC-32 |
| 0x02 | MIC-64 |
| 0x03 | MIC-128 |

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	249

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 94—c data fo r all s e c urity le ve ls

| Security level identifier | c data |
| --- | --- |
| 0x04 | Secured payload fields |
| 0x05 | Secured payload fields ¦¦ MIC-32 |
| 0x06 | Secured payload fields ¦¦ MIC-64 |
| 0x07 | Secured payload fields ¦¦ MIC-128 |

2

3

4

5

6

7

8

9

10

11

12	7.3.5 CCM* inverse transformation data representation

13

14	This subclause describes how the inputs and output of the CCM* inverse transformation, as described in

15	C.4.2 of IEEE Std 802.15.4-2006, are formed.

16

17	The inputs are as follows:

18

19	—	Key

20

21	—	Nonce

22	—	c data

23	—	a data

24

25	The output is m data.

26

27	7.3.5.1 Key and nonce data inputs

28

29	The key data for the CCM* inverse transformation is passed by the incoming frame security procedure

30	described in 7.2.3. The nonce data for the CCM* transformation is constructed as described in 7.3.2.

31

32	7.3.5.2 c data and a data

33

34	In the CCM* inverse transformation process, the data fields shall be applied as in Table 95.

35

36

37	Table 95—c data and a data fo r all s e c urity le ve ls

| Security level identifier | c data | a data |
| --- | --- | --- |
| 0x00 | None | None |
| 0x01 | MIC-32 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦Secured payload fields |
| 0x02 | MIC-64 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ Secured payload fields |
| 0x03 | MIC-128 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields ¦¦ Secured payload fields |

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

250	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 95—c data and a data fo r all s e c urity le ve ls | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |

| Security level identifier | c data | a data |
| --- | --- | --- |
| 0x04 | Secured payload fields | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields |
| 0x05 | Secured payload fields ¦¦ MIC-32 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields |
| 0x06 | Secured payload fields ¦¦ MIC-64 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields |
| 0x07 | Secured payload fields ¦¦ MIC-128 | MHR ¦¦ Auxiliary security header ¦¦ Nonpayload fields |

7.3.5.3 m data output	16

17

The m data shall then substitute secured payload fields and authentication tag in the original secured frame	18 to form the unsecured frame.	19

20

7.4 Auxiliary Security header	21

22

23

The Auxiliary Security Header field has a variable length and contains information required for security	24

processing, including a Security Control field, a Frame Counter field, and a Key Identifier field. The	25

Auxiliary Security Header field shall be present only if the Security Enabled subfield of the frame control	26

field is set to one. The Auxiliary Security Header field shall be formatted as illustrated in Figure 132.	27

28

| Octets: 1 | 4 | 0/1/5/9 |
| --- | --- | --- |
| Security Control | Frame Counter | Key Identifier |

29

30

31

32

33

Figure 132—Format of the auxiliary security header	34

35

36

7.4.1 Integer and octet representation	37

38

The auxiliary security header is a MAC frame field (see 5.2.1.7) and, therefore, uses the representation	39 conventions specified in 5.2.	40

41

7.4.2 Security Control field	42

43

The Security Control field is 1 octet in length and is used to provide information about what protection is	44 applied to the frame. The Security Control field shall be formatted as shown in Figure 133.	45

46

47

| Bit: 0–2 | 3–4 | 5–7 |
| --- | --- | --- |
| Security Level | Key Identifier Mode | Reserved |

48

49

50

51

52

Figure 133—Security Control field format	53

54

Copyright © 20xx IEEE. All rights reserved.	251

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	7.4.2.1 Security Level subfield

2

3	The Security Level subfield indicates the actual frame protection that is provided. This value can be adapted

4	on a frame-by-frame basis and allows for varying levels of data authenticity (to allow minimization of

5	security  overhead  in  transmitted  frames  where  required)  and  for  optional  data  confidentiality. The

6	cryptographic protection offered by the various security levels is shown in Table 96. When nontrivial

7	protection is required, replay protection is always provided.

8

9

10	Table 96—S e c urity le ve ls available to the MAC s ublaye r

| Security level identifier | Security Control field (Figure 133) b2 b1 b0 | Security attributes | Data confidentiality | Data authenticity (including length M of authentication tag, in octets) |
| --- | --- | --- | --- | --- |
| 0x00 | '000' | None | OFF | NO (M = 0) |
| 0x01 | '001' | MIC-32 | OFF | YES (M = 4) |
| 0x02 | '010' | MIC-64 | OFF | YES (M = 8) |
| 0x03 | '011' | MIC-128 | OFF | YES (M = 16) |
| 0x04 | '100' | ENC | ON | NO (M = 0) |
| 0x05 | '101' | ENC-MIC-32 | ON | YES (M = 4) |
| 0x06 | '110' | ENC-MIC-64 | ON | YES (M = 8) |
| 0x07 | '111' | ENC-MIC-128 | ON | YES (M = 16) |

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30	Security levels can be ordered according to the corresponding cryptographic protection offered. Here, a first

31	security level SEC1 is greater than or equal to a second security level SEC2 if and only if SEC1 offers at

32	least the protection offered by SEC2, both with respect to data confidentiality and with respect to data

33	authenticity. The statement “SEC1 is greater than or equal to SEC2” shall be evaluated as TRUE if both of

34	the following conditions apply:

35

36	a)	Bit position b2 in SEC1 is greater than or equal to bit position b2 in SEC2 (where Encryption OFF <

37	Encryption ON).

38	b)	The integer value of bit positions b1 b0 in SEC1 is greater than or equal to the integer value of bit

39	positions  b1  b0  in  SEC2  [where  increasing  integer  values  indicate  increasing  levels  of  data

40	authenticity provided, i.e., message integrity code (MIC)-0 < MIC-32 < MIC-64 < MIC-128].

41

42	Otherwise, the statement shall be evaluated as FALSE.

43

44	For example, ENC-MIC-64  MIC-64 is TRUE because ENC-MIC-64 offers the same data authenticity

45	protection as MIC-64, plus confidentiality. On the other hand, MIC-128  ENC-MIC-64 is FALSE because

46	even though MIC-128 offers stronger data authenticity than ENC-MIC-64, it offers no confidentiality.

47

48	7.4.2.2 Key Identifier Mode subfield

49

50	The Key Identifier Mode subfield indicates whether the key that is used to protect the frame can be derived

51	implicitly or explicitly; furthermore, it is used to indicate the particular representations of the Key Identifier

52	field (see 7.4.4) if derived explicitly. The Key Identifier Mode subfield shall be set to one of the values listed

53

54

252	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

in Table 97. The Key Identifier field of the auxiliary security header (see 7.4.4) shall be present only if this	1 subfield has a value that is not equal to 0x00.	2

3

4

Table 97—Value s of the Ke y Ide ntifie r mo de	5

| Key Identifier mode | Key Identifier Mode subfield b1 b0 | Description | Key Identifier field length (octets) |
| --- | --- | --- | --- |
| 0x00 | '00' | Key is determined implicitly from the originator and recipient(s) of the frame, as indicated in the frame header. | 0 |
| 0x01 | '01' | Key is determined from the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header in conjunction with macDefaultKeySource. | 1 |
| 0x02 | '10' | Key is determined explicitly from the 4-octet Key Source subfield and the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. | 5 |
| 0x03 | '11' | Key is determined explicitly from the 8-octet Key Source subfield and the 1-octet Key Index subfield of the Key Identifier field of the auxiliary security header. | 9 |

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

7.4.3 Frame Counter field	29

30

The Frame Counter field is 4 octets in length and represents the macFrameCounter attribute of the originator              31 of a protected frame. It is used to provide semantic security of the cryptographic mechanism used to protect              32 a frame and to offer replay protection.                                                                                                                              33

34

7.4.4 Key Identifier field	35

36

The Key Identifier field has a variable length and identifies the key that is used for cryptographic protection	37 of outgoing frames, either explicitly or in conjunction with implicitly defined side information. The Key	38

Identifier field shall be present only if the Key Identifier Mode subfield of the Security Control field of the              39 auxiliary security header (see 7.4.2.2) is set to a value different from 0x00. The Key Identifier field shall be              40 formatted as illustrated in Figure 134.                                                                                                                              41

42

43

| Octets: 0/4/8 | 1 |
| --- | --- |
| Key Source | Key Index |

44

45

46

47

Figure 134—Format for the Key Identifier field, if present	48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	253

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	7.4.4.1 Key Source subfield

2

3	The Key Source subfield, when present, is either 4 octets or 8 octets in length, according to the value

4	specified by the Key Identifier Mode subfield of the Security Control field (see 7.4.2.2), and indicates the

5	originator of a group key.

6

7	7.4.4.2 Key Index subfield

8

9	The Key Index subfield is 1 octet in length and allows unique identification of different keys with the same

10	originator.

11

12	It is the responsibility of each key originator to make sure that actively used keys that it issues have distinct

13	key indices and that the key indices are all different from 0x00.

14

15	7.5 Security-related MAC PIB attributes

16

17

18	The security-related MAC PIB attributes contain the following:

19

20	—	Key table (macKeyTable, macKeyTableEntries)

21	—	Device table (macDeviceTable, macDeviceTableEntries)

22	—	Minimum security level table (macSecurityLevelTable, macSecurityLevelTableEntries)

23	—	Frame counter (macFrameCounter)

24

25	—	Automatic request attributes (macAutoRequestSecurityLevel, macAutoRequestKeyIdMode,

26	macAutoRequestKeySource, macAutoRequestKeyIndex)

27	—	Default key source (macDefaultKeySource)

28	—	coordinator address (macOWPANCoordExtendedAddress, macOWPANCoordShortAddress)

29

30	7.5.1 PIB security material

31

32	The PIB security-related attributes are presented in Table 98, Table 99, Table 100, Table 101, Table 102,

33	Table 103, and Table 104.

34

35

36	Table 98—S e c urity-re late d MAC PIB attribute s

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macKeyTable | 0x71 | List of Key- Descriptor entries (see Table 99) | — | A table of KeyDescriptor entries, each containing keys and related information required for secured communications. | (empty) |
| macKeyTableEntries | 0x72 | Integer | Implementati on specific | The number of entries in macKeyTable. | 0 |
| macDeviceTable | 0x73 | List of Device- Descriptor entries (see Table 103) | — | A table of Device- Descriptor entries, each indicating a remote device with which this device securely communicates. | (empty) |
| macDeviceTable- Entries | 0x74 | Integer | Implementa- tion specific | The number of entries in macDeviceTable. | 0 |

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

254	Copyright © 20xx IEEE. All rights reserved.

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macSecurity- LevelTable | 0x75 | Table of SecurityLevel Descriptor entries (see Table 102) | — | A table of SecurityLevel- Descriptor entries, each with information about the minimum security level expected depending on incoming frame type and subtype. | (empty) |
| macSecurity- LevelTableEntries | 0x76 | Integer | Implementa- tion specific | The number of entries in macSecurityLevelTable. | 0 |
| macFrameCounter | 0x77 | Integer | 0x00000000– 0xffffffff | The outgoing frame counter for this device. | 0x00000000 |
| macAutoRequest- SecurityLevel | 0x78 | Integer | 0x00–0x07 | The security level used for automatic data requests. | 0x06 |
| macAutoRequest- KeyIdMode | 0x79 | Integer | 0x00–0x03 | The key identifier mode used for automatic data requests. This attribute is invalid if the macAuto- RequestSecurityLevel attribute is set to 0x00. | 0x00 |
| macAutoRequest- KeySource | 0x7a | As specified by the mac- AutoRequest- KeyIdMode parameter | — | The originator of the key used for automatic data requests. This attribute is invalid if the macAuto- RequestKeyIdMode element is invalid or set to 0x00. | All octets 0xff |
| macAutoRequest- KeyIndex | 0x7b | Integer | 0x01–0xff | The index of the key used for automatic data requests. This attribute is invalid if the macAuto- RequestKeyIdMode attribute is invalid or set to 0x00. | All octets 0xff |
| macDefaultKey- Source | 0x7c | Set of 8 octets | — | The originator of the default key used for key identifier mode 0x01. | All octets 0xff |
| macOWPANCoord- ExtendedAddress | 0x7d | IEEE address | An extended 64-bit IEEE address | The 64-bit address of the coordinator. | — |
| macOWPANCoordS hort-Address | 0x7e | Integer | 0x0000–0xffff | The 16-bit short address assigned to the coordinator. A value of 0xfffe indicates that the coordinator is only using its 64-bit extended address. A value of 0xffff indicates that this value is unknown. | 0x0000 |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 98—Se c urity-re lated MAC PIB attributes | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

Copyright © 20xx IEEE. All rights reserved.	255

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 99—Ele me nts o f Ke yDe s c ripto r

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| KeyIdLookupList | List of KeyId- LookupDescriptor entries (see Table 104) | — | A list of KeyIdLookupDescriptor entries used to identify this KeyDescriptor. |
| KeyIdLookupListEntries | Integer | Implementation specific | The number of entries in KeyIdLookupList. |
| KeyDeviceList | List of KeyDevice- Descriptor entries (see Table 101) | — | A list of KeyDeviceDescriptor entries indicating which devices are currently using this key, including their blacklist status. |
| KeyDeviceListEntries | Integer | Implementation specific | The number of entries in KeyDeviceList. |
| KeyUsageList | List of KeyUsage- Descriptor entries (see Table 100) | — | A list of KeyUsageDescriptor entries indicating the frame types with which this key may be used. |
| KeyUsageListEntries | Integer |  | The number of entries in KeyUsageList. |
| Key | Set of 16 octets | — | The actual value of the key. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25	Table  100—Eleme nts  of Ke yUs age De s c riptor

26

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| FrameType | Integer | 0x00–0x03 | As defined in 5.2.1.1.2. |
| CommandFrameIdentifier | Integer | 0x00–0x09 | As defined in Table 28. |

27

28

29

30

31

32

33

34	Table 101—Eleme nts o f Ke yDevice Des c ripto r

35

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| DeviceDescriptorHandle | Integer | Implementa- tion specific | Handle to the DeviceDescriptor corresponding to the device (see Table 103). |
| UniqueDevice | Boolean | TRUE or FALSE | Indication of whether the device indicated by DeviceDescriptorHandle is uniquely associated with the KeyDescriptor, i.e., it is a link key as opposed to a group key. |
| Blacklisted | Boolean | TRUE or FALSE | Indication of whether the device indicated by DeviceDescriptorHandle previously communicated with this key prior to the exhaustion of the frame counter. If TRUE, this indicates that the device shall not use this key further because it exhausted its use of the frame counter used with this key. |

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

256	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

7.5.2 Key table

Table 102—Ele ments of Sec urityLe ve lDes c ripto r	1

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| FrameType | Integer | 0x00–0x03 | As defined in 5.2.1.1.2. |
| CommandFrameIdentifier | Integer | 0x00–0x09 | As defined in Table 28. |
| SecurityMinimum | Integer | 0x00–0x07 | The minimal required/expected security level for incoming MAC frames with the indicated frame type and, if present, command frame type (as defined in Table 96 in 7.4.2.1). |
| DeviceOverrideSecurity- Minimum | Boolean | TRUE or FALSE | Indication of whether originating devices for which the Exempt flag is set may override the minimum security level indicated by the SecurityMinimum element. If TRUE, this indicates that for originating devices with Exempt status, the incoming security level zero is acceptable, in addition to the incoming security levels meeting the minimum expected security level indicated by the SecurityMinimum element. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

Table 103—Elem ents of De vice De s c riptor	24

25

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| OWPANId | Device OWPAN ID | 0x0000–0xffff | The 16-bit OWPAN identifier of the device in this DeviceDescriptor. |
| ShortAddress | Device short address | 0x0000–0xffff | The 16-bit short address of the device in this DeviceDescriptor. A value of 0xfffe indicates that this device is using only its extended address. A value of 0xffff indicates that this value is unknown. |
| ExtAddress | IEEE address | Any valid 64-bit device address | The 64-bit IEEE extended address of the device in this DeviceDescriptor. This element is also used in unsecuring operations on incoming frames. |
| FrameCounter | Integer | 0x00000000–0xffffffff | The incoming frame counter of the device in this DeviceDescriptor. This value is used to ensure sequential freshness of frames. |
| Exempt | Boolean | TRUE or FALSE | Indication of whether the device may override the minimum security level settings defined in Table 102. |

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

The key table holds key descriptors (keys with related key-specific information) that are required for              49 security processing of outgoing and incoming frames. Key-specific information in the key table is identified              50 based on information explicitly contained in the requesting primitive or in the received frame, as described              51 in the outgoing frame key retrieval procedure (see 7.2.2) and the incoming frame security material retrieval              52 procedure (see 7.2.4), as well as in the KeyDescriptor lookup procedure (see 7.2.5).                                                     53

54

Copyright © 20xx IEEE. All rights reserved.	257

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| LookupData | Set of 5 or 9 octets | — | Data used to identify the key. |
| LookupDataSize | Integer | 0x00–0x01 | A value of 0x00 indicates a set of 5 octets; a value of 0x01 indicates a set of 9 octets. |

2

3

4

5

6

7

8

9

10

11	7.5.3 Device table

12

Table 104—Elem ents of Ke yIdLoo kupDe s c ripto r

13	The device table holds device descriptors (device-specific addressing information and security-related

14	information) that, when combined with key-specific information from the key table, provide all the keying

15	material needed to secure outgoing (see 7.2.1) and unsecure incoming frames (see 7.2.3). Device-specific

16	information in the device table is identified based on the originator of the frame, as described in the

17	DeviceDescriptor lookup procedure (see  7.2.7), and  on  key-specific information, as  described in  the

18	blacklist checking procedure (see 7.2.6).

19

20	7.5.4 Minimum security level table

21

22	The minimum security level table holds information regarding the minimum security level the device

23	expects to have been applied by the originator of a frame, depending on frame type and, if it concerns a

24	MAC command frame, the command frame identifier. Security processing of an incoming frame will fail if

25	the frame is not adequately protected, as described in the incoming frame security procedure (see 7.2.3) and

26	in the incoming security level checking procedure (see 7.2.8).

27

28	7.5.5 Frame counter

29

30	The 4-octet frame counter is used to provide replay protection and semantic security of the cryptographic

31	building block used for securing outgoing frames. The frame counter is included in each secured frame and

32	is one of the elements required for the unsecuring operation at the recipient(s). The frame counter is

33	incremented each time an outgoing frame is secured, as described in the outgoing frame security procedure

34	(see 7.2.1). When the frame counter reaches its maximum value of 0xffffffff, the associated keying material

35	can no longer be used, thus requiring all keys associated with the device to be updated. This provides a

36	mechanism for ensuring that the keying material for every frame is unique and, thereby, provides for

37	sequential freshness.

38

39	7.5.6 Automatic request attributes

40

41	Automatic  request  attributes  hold  all  the  information  needed  to  secure  outgoing  frames  generated

42	automatically and not as a result of a higher layer primitive, as is the case with automatic data requests.

43

44	7.5.7 Default key source

45

46	The default key source is information commonly shared between originator and recipient(s) of a secured

47	frame, which, when combined with additional information explicitly contained in the requesting primitive or

48	in the received frame, allows an originator or a recipient to determine the key required for securing or

49	unsecuring this frame, respectively. This provides a mechanism for significantly reducing the overhead of

50	security information contained in secured frames in particular use cases as shown in 7.2.2 and 7.2.4.

51

52

53

54

258	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

7.5.8 Coordinator address	1

2

The address of the coordinator is information commonly shared between all devices in a OWPAN, which,              3 when combined with additional information explicitly contained in the requesting primitive or in the              4 received frame, allows an originator of a frame directed to the coordinator or a recipient of a frame              5 originating from the coordinator to determine the key and security-related information required for securing              6 or unsecuring, respectively, this frame as shown in 7.2.2 and 7.2.4.                                                                                7

8

9

8. PHY layer specification	10

11

12

8.1 Overview	13

14

This clause specifies seven(ED NOTE: or is this going to be eight?) PHY options for IEEE Std 802.15.13.	15

16

The PHY is responsible for the following tasks:	17

18

—	Activation and deactivation of the OWC transceiver	19

—	WQI for received frames	20

—	Channel selection	21

—	Data transmission and reception	22

23

—	Error correction	24

—	Synchronization	25

26

Constants and attributes that are specified and maintained by the PHY are written in the text of this clause in	27 italics. Constants have a general prefix of “a”, e.g., aMaxPHYFrameSize, and are listed in Table 139.	28

Attributes have a general prefix of “phy”, e.g., phyCurrentChannel, and are listed in Table 140.	29

30

This subclause specifies requirements that are common to all of the IEEE 802.15.13 PHYs.	31

32

8.2 Operating modes	33

34

35

A compliant IEEE 802.15.7-20xx PHY shall implement at least one of the PHY I or PHY II mandatory	36

modes (as defined in Clause 10 and Clause 10) given in Table 105 and Table 106. A device implementing	37

the PHY III mode in Table 107 shall also implement PHY II mode for coexistence as summarized in 4.4.1.2.	38

The PHY modulation modes may operate in the presence of dimming. Modulation using OOK under	39

dimming provides constant range and variable data rate by inserting compensation time as defined in	40

4.4.3.1.4 while modulation using VPPM under dimming provides constant data rate and variable range by	41

adjusting the pulse width as summarized in 4.4.3.1.6.	42

43

As  shown  in  Table 105  through  Table 109,  the  standard  provides  channel  coding  support  for  error	44

correction. PHY I supports concatenated coding with Reed-Solomon (RS) and convolutional coding (CC)	45

since it has been designed for outdoor use with short frames. PHY II and PHY III support only RS coding.	46

PHY I and PHY II also support a run length limited (RLL) code to provide DC balance, clock recovery, and	47

flicker mitigation.	48

49

In addition to modulation and coding, multiple optical rates are provided for all PHY types in order to	50

support a broad class of optical transmitters (LEDs) for various applications. The choice of optical rate used	51

for communication is decided by the MAC during device discovery. The MAC shall select the optical clock	52

rate for communication during the optical clock-rate selection process as defined in 6.5. The preamble shall	53

be sent at clock rate chosen by the TX and supported by the RX. The preamble is a time domain sequence	54

Copyright © 20xx IEEE. All rights reserved.	259

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	and does not have any modulation, channel coding, or line coding. The PHY header shall be sent at the

2	lowest data rate for the chosen clock rate. The clock rate does not change through the frame between the

3	preamble, header, and payload.

4

5

6

7

8	Table 105—PHY I o pe rating mo de s

9

| Modulation | RLL code | Optical clock rate | FEC |  | Data rate |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Outer code (RS) | Inner code (CC) |  |
| OOK | Manchester | 200 kHz | (15,7) | 1/4 | 11.67 kb/s |
|  |  |  | (15,11) | 1/3 | 24.44 kb/s |
|  |  |  | (15,11) | 2/3 | 48.89 kb/s |
|  |  |  | (15,11) | none | 73.3 kb/s |
|  |  |  | none | none | 100 kb/s |
| VPPM | 4B6B | 400 kHz | (15,2) | none | 35.56 kb/s |
|  |  |  | (15,4) | none | 71.11 kb/s |
|  |  |  | (15,7) | none | 124.4 kb/s |
|  |  |  | none | none | 266.6 kb/s |

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32	Table 106—PHY II o pe rating mo de s

33

| Modulation | RLL code | Optical clock rate | FEC | Data rate |
| --- | --- | --- | --- | --- |
| VPPM | 4B6B | 3.75 MHz | RS(64,32) | 1.25 Mb/s |
|  |  |  | RS(160,128) | 2 Mb/s |
|  |  | 7.5 MHz | RS(64,32) | 2.5 Mb/s |
|  |  |  | RS(160,128) | 4 Mb/s |
|  |  |  | none | 5 Mb/s |

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

260	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 106—PHY II o pe rating mo de s	1

| Modulation | RLL code | Optical clock rate | FEC | Data rate |
| --- | --- | --- | --- | --- |
| OOK | 8B10B | 15 MHz | RS(64,32) | 6 Mb/s |
|  |  |  | RS(160,128) | 9.6 Mb/s |
|  |  | 30 MHz | RS(64,32) | 12 Mb/s |
|  |  |  | RS(160,128) | 19.2 Mb/s |
|  |  | 60 MHz | RS(64,32) | 24 Mb/s |
|  |  |  | RS(160,128) | 38.4 Mb/s |
|  |  | 120 MHz | RS(64,32) | 48 Mb/s |
|  |  |  | RS(160,128) | 76.8 Mb/s |
|  |  |  | none | 96 Mb/s |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

Table 107—PHY III o pe rating mo de s	24

| Modulation | Optical clock rate | FEC | Data rate |
| --- | --- | --- | --- |
| 4-CSK | 12 MHz | RS(64,32) | 12 Mb/s |
| 8-CSK |  | RS(64,32) | 18 Mb/s |
| 4-CSK | 24 MHz | RS(64,32) | 24 Mb/s |
| 8-CSK |  | RS(64,32) | 36 Mb/s |
| 16-CSK |  | RS(64,32) | 48 Mb/s |
| 8-CSK |  | none | 72 Mb/s |
| 16-CSK |  | none | 96 Mb/s |

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

Table 108—PHY VII o pe rating mo de s	43

44

| Operating Modes (16/310r0) |  |
| --- | --- |
| Modulation | DCO-OFDM, eU-OFDM, RPO-OFDM |
| FEC | Convolutional Coding (131, 171) |
| Code Rates | 1/2, 2/3, 3/4 |
| MIMO | Up to 16x16 |

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	261

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 108—PHY VII o pe rating mo de s

2

| Operating Modes (16/310r0) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | Bandwidth | Optical Clock Rate | FEC | Data Rate (SISO) |
| BPSK | 5 - 40 MHz | 10 - 80 MHz | 1/2 | 0.75 - 12 Mb/s |
| BPSK |  |  | 3/4 | 1.375 - 18 Mb/s |
| 4-QAM |  |  | 1/2 | 1.5 - 24 Mb/s |
| 4-QAM |  |  | 3/4 | 2.75 - 36 Mb/s |
| 16-QAM |  |  | 1/2 | 3 - 48 Mb/s |
| 16-QAM |  |  | 3/4 | 4.5 - 72 Mb/s |
| 64-QAM |  |  | 2/3 | 6-96 Mb/s |
| 64-QAM |  |  | 3/4 | 6.75 - 108 Mb/s |

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24	Table 109—PHY VIII Ope rating Mo de s

| PHY Operating Modesa |  |  |  |
| --- | --- | --- | --- |
| modulation |  | DC-biased DMT |  |
| subcarrier spacing |  | 195.3125 KHz |  |
| cyclic prefix (CP) |  | 160, 320 ns |  |
| symbol duration (w/o CP) |  | 5.12 us |  |
| FEC |  | Low-density parity-check code (LDPC) |  |
| information block size |  | 120, 540 bytes |  |
| code rates |  | 1/2, 2/3, 5/6, 16/18, 20/21 |  |
| repetitions |  | 1, 2, 3, 4, 6, 8 |  |
| Bandwidth | Optical Clock Rate (Factor) | Used Carriers IFFT size using DMT | Min. to Max. Data Rateb |
| 10 MHz | 25 MS/s (1/8) | 45 (128) | 1.5 to 103 Mb/s |
| 25 MHz | 50 MS/s (1/4) | 107 (256) | 3 to 257 Mb/s |
| 50 MHz | 100 MS/s (1/2) | 215 (512) | 7 to 512 Mb/s |
| 100 MHz | 200 MS/s (1/1) | 450 (1024) | 14 to 1028 Mb/s |
| 200 MHz | 400 MS/s (2/1) | 950 (2048) | 35 to 2056 Mb/s |
| 500 MHz | 1 GS/s (5/1) | 2375 (2x2048) | 87 to 5140 Mb/s |
| 1 GHz | 2 GS/s (100/1) | 4750 (3x2048) | 145 to 10281 Mb/s |

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

262	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

a The PHY parameters are derived from the ITU-T recommendation G.9960 (12/2015). They were generically                        1 scaled towards lower and higher bandwidths, for achieving improved energy efficiency and higher data rate,                        2 respectively.	3

4 b Max./Min. data rates assume that all used subcarriers are occupied. Max. rates assume 12 bits/subcarrier and		5 code rate 20/21. Data rate is variable by means of adaptive bit loading, different FEC rates and repetitions.	6

Min. rates assume 1 bit/subcarrier, rate ½ and 3 repetitions. Even lower data rates are possible by partial usage

of subcarriers though adaptive bit loading.	7

8

9

10

8.3 General requirements	11

12

8.3.1 Optical mapping	13

14

A high switching level from the PHY, applied to the light source, shall result in a high radiated intensity. A              15 low switching level from the PHY, applied to the light source, shall result in a reduced radiated intensity. The              16 extinction ratio, defined as the ratio of the high radiated intensity to the low radiated intensity, is at the              17 discretion of the implementer.                                                                                                                                           18

19

8.3.2 Maximum error tolerance for multiple optical sources	20

21

If multiple optical sources are used for communication, it is recommended the optical sources have similar	22 frequency responses in order to assist communication. The digital input to all the optical sources from the	23

PHY shall be synchronized. Figure 135 shows the allowable spread at the output of the optical sources,              24 assuming a synchronized digital input. The maximum spread at the average signal intensity level during the              25 rise and fall time at the output of the optical sources shall not vary by more than 12.5% of the clock             26 period.Minimum LIFS, SIFS, and RIFS periods                                                                                                              27

28

An interframe spacing (IFS) is used to provide spacing between adjacent frames. The minimum spacing              29 between frames is dependent on the MAC mode of  operation. The standard provides three types of              30 interframe spacing: long (LIFS), short (SIFS), and reduced (RIFS). For peer-to-peer and star topologies, the              31

SIFS, LIFS, and RIFS period is based on the currently negotiated optical clock rate by the MAC before              32 starting data communication. Once the optical clock rate is selected, the SIFS, LIFS, and RIFS period is              33 fixed to the values shown in Table 110. The clock-rate negotiation for a peer-to-peer and star topology is              34 provided in 6.5. For a star topology, the beacon and CAP periods are defined at the lowest optical clock rate              35 to ensure fair access to the medium. For a broadcast topology, the IFS is defined based on the optical clock              36 rate chosen for broadcasting data to other devices. The minimum LIFS, SIFS, and RIFS periods for each of              37 the PHYs are shown in Table 110. A detailed description, use, and illustration of LIFS, SIFS, and RIFS is              38 shown in Figure 15.                                                                                                                                                           39

40

41

Table 110—Minimum LIFS , S IFS , and RIFS pe rio ds	42

| PHY | macMinLIFSPeriod | macMinSIFSPeriod | macMinRIFSPeriod | Units |
| --- | --- | --- | --- | --- |
| PHY I | 400 | 120 | 40 | optical clocks |
| PHY II | 400 | 120 | 40 | optical clocks |
| PHY III | 400 | 120 | 40 | optical clocks |

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	263

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30	Figure 135—Maximum allowable spread at the output of optical sources

31

32	8.3.3 TX-to-RX turnaround time

33

34	The TX-to-RX turnaround time shall be as shown in Table 139 and shall be measured at the air interface

35	from the trailing edge of the last clock of the last transmission until the receiver is ready to begin the

36	reception of the next PHY frame.

37

38	8.3.4 RX-to-TX turnaround time

39

40	The RX-to-TX turnaround time shall be as shown in Table 139 and shall be measured at the air interface

41	from the trailing edge of the last clock of the received frame until the transmitter is ready to begin

42	transmission of the resulting acknowledgment. Actual transmission start times are specified by the MAC

43	sublayer.

44

45	8.3.5 Transmit data clock frequency tolerance

46

47	The transmitted data clock frequency tolerance shall be ±20 ppm maximum.

48

49	8.3.6 Clear channel assessment (CCA)

50

51	The IEEE 802.15.13 PHY may provide the capability to perform CCA according to at least one of the

52	following three methods:

53

54

264	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

a)	CCA Mode 1: Energy above threshold. CCA may report a busy medium upon detecting any energy	1 above the energy detect threshold.	2

b)     CCA Mode 2: Carrier sense only. CCA may report a busy medium only upon the detection of a              3 signal with the modulation characteristics of IEEE Std 802.15.13. This signal may be above or              4 below the energy detect threshold.                                                                                                                       5

c)	CCA Mode 3: Carrier sense with energy above threshold. CCA may report a busy medium only              6 upon the detection of a signal with the modulation characteristics of IEEE Std 802.15.13 with energy              7 above the energy detect threshold. See 4.3 for conceptual guidance.                                                                  8

9

For any of the CCA modes, if the PLME-CCA.request primitive, specified in 9.2.1, is received by the PHY              10 during reception of a PPDU, CCA may report a busy medium. PPDU reception is considered to be in              11 progress following detection of the preamble, and it remains in progress until the number of octets specified              12 by the decoded PHR has been received.                                                                                                                           13

14

A busy channel may be indicated by the PLME-CCA.confirm primitive with a status of BUSY as specified	15 in 9.2.2.	16

17

A clear channel may be indicated by the PLME-CCA.confirm primitive with a status of IDLE as specified in	18

9.2.2.	19

20

The  PHY PIB  attribute  phyCCAMode  may  indicate  the  appropriate  operation  mode  as  specified  in	21

Table 139.	22

23

8.4 Data modes	24

25

26

The PHY shall support the following normal data transmission modes as shown in Figure 136:	27

28 a)     Single mode                                                                                                                                                          29 b)     Packed mode                                                                                                                                                        30 c)     Burst mode                                                                                                                                                           31

32

In addition, there is a fourth mode for data transfer called “dimmed OOK” mode, which is used for data	33

transfer while dimming in conjunction with OOK.	34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	265

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20	Figure 136—Data modes supported by the MAC (single, packed, burst)

21

22

23	The single mode transfers one PPDU per frame. This may be used for very short data communication such

24	as acknowledgments, association, beaconing, or for information broadcast mode, for example.

25

26	The packed mode contains multiple PPDUs per frame and is used to send multiple consecutive PPDUs to the

27	same destination within the frame for high throughput. Thus, the overhead of sending multiple MAC and

28	PHY headers to the same destination is eliminated in this mode, providing higher MAC efficiency. This can

29	be used in most modes as the preferred means of data communication.

30

31	The burst data mode uses a reduced length PHY preamble, as defined in 8.6.1, after the first frame in the

32	burst. In addition, the RIFS is used between frames instead of the SIFS. The shorter preamble increases the

33	efficiency and throughput in this mode.

34

35	The dimmed OOK mode is used to support data transfer under dimming requirements, as summarized in

36	4.4.3.1.4.

37

38

39	8.5 Flicker mitigation

40

41	Flicker mitigation can be divided into intra-frame mitigation and interframe flicker mitigation as described

42	in 4.4.4.

43

44	Intra-frame flicker mitigation refers to mitigation flicker within the transmission of a data frame. Intra-frame

45	flicker in OOK is avoided by the use of the dimmed OOK mode as described in 8.5.2.2, and RLL coding as

46	described in 8.2. VPPM inherently does not cause any interframe flicker and also uses a RLL code.

47	Interframe flicker is avoided in CSK by ensuring constant average power across multiple light sources along

48	with scrambling and the high optical clock rates (MHz) at which this modulation is used.

49

50	Interframe flicker mitigation applies to both data transmission (RX mode) and idle periods. While idling,

51	visibility patterns or idle patterns as described in 8.5.1 may be used to ensure light emission by the OWC

52	transmitters have the same average brightness over adjacent MFTPs as during data transmission. These

53	patterns can be modulated in-band or out-of-band as in 8.5.1.1.

54

266	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

When the dimmer setting is changed above the MAC sublayer, the MAC and PHY layers adjust the data              1 transmission and idle time transmission to adjust to the new dimmer settings. A summary of the different              2 mitigation techniques for interframe and intra-frame flicker is provided in Table 111.                                                   3

4

5

Table 111—Flic ke r mitig atio n fo r vario us mo dulatio n mo de s	6

| Flicker mitigation | Data transmission (Intra-frame flicker) | Idle or RX periods (Interframe flicker) |
| --- | --- | --- |
| OOK modulation | Dimmed OOK mode, RLL code | Idle/visibility patterns |
| VPPM modulation | VPPM guarantees no intra-frame flicker, RLL code |  |
| CSK modulation | Constant average power across multiple light sources, scrambler, high optical clock rates (MHz) |  |

7

8

9

10

11

12

13

14

15

16

17

18

19

8.6 PPDU format	20

21

For convenience, the PPDU frame structure is presented so that the left most field as written in this standard              22 shall  be  transmitted or  received first. All  multiple octet  fields shall be  transmitted or  received least              23 significant octet first and each octet shall be transmitted or received least significant bit (LSB) first. The              24 same transmission order should apply to data fields transferred between the PHY layer and MAC sublayer.              25

The PPDU frame structure shall be formatted as illustrated in Figure 137.	26

27

28

| Preamble (see 8.6.1) | PHY header (see 8.6.2) | HCS (see 8.6.3) | Optional fields (see 8.6.4) | PSDU (see 8.6.5) |
| --- | --- | --- | --- | --- |
| SHR | PHR |  |  | PHY payload |

29

30

31

32

33

34

Figure 137—Format of the PPDU	35

36

37

8.6.1 Preamble field	38

39

The preamble field is used by the transceiver to obtain optical clock synchronization with an incoming              40 message. The  standard  defines  one  fast  locking  pattern  (FLP)  followed  by  choice  of  four  topology              41 dependent patterns (TDPs) for the purposes of distinguishing different PHY topologies. The MAC shall              42 select the optical clock rate for communication during the clock-rate selection process as defined in 6.5. The              43 preamble shall be sent at a clock rate chosen by the TX and supported by the RX. The preamble is a time              44 domain sequence and does not have any channel coding or line coding.                                                                         45

46

The preamble first starts with a FLP of at least 64 alternate ones and zeros. The FLP is fixed to start as a	47 “1010…” pattern i.e., it ends with a ‘0’. This maximum transition sequence is used to lock the CDR circuit.	48

The fast locking pattern length shall not exceed the maximum as shown in Figure 138. After the fast locking	49

pattern, four repetitions of one of four TDPs (defined in Figure 139) shall be sent. The TDP shall be 15 bits	50 in length and the TDP shall be inverted every other repetition to provide DC balance.	51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	267

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3	64 to 16,384 bits

| FLP (101010 …) | TDP | ~TDP | TDP | ~TDP |
| --- | --- | --- | --- | --- |

4

5

6

60 bits

7	Figure 138—Preamble transmission

8

9

10

11

12	P1 : 1 1 1 1 0 1 0 1 1 0 0 1 0 0 0

13	P2 : 0 0 1 0 1 1 1 0 1 1 1 1 1 1 0

14

15	P3 : 1 0 0 1 1 0 0 0 0 0 1 0 0 1 1

16	P4 : 0 1 0 0 0 0 1 1 0 1 0 0 1 0 1

17	Figure 139—TDPs for various topologies

18

19

20

21

22	The preambles shall be transmitted using an OOK modulation. The preamble field for single data mode and

23	packed data mode shall be formatted as illustrated in Figure 138. For PHY III, all the three light sources

24	shall transmit the same preamble pattern simultaneously in the supported frequency bands within the error

25	tolerance specified in 8.3.2.

26

27	For the burst mode transmission, the FLP shall be included only for the first frame. Subsequent frames shall

28	not include the FLP in the burst mode since the receiver is already synchronized to the transmitter. This

29	reduces the preamble length by at least half and provides higher throughput at the MAC layer. The preamble

30	field for burst data mode shall be formatted as illustrated in Figure 140.

31

32

33

34

35

36

37

38	Figure 140—Burst preamble transmission

39

40

41	The TDP used for a specific a topology is defined in Table 112. The topologies are given in 4.2.

42

43	Table 112—TDP as s ig nme nts fo r vario us to po lo g ie s

44

| TDP | Topology |
| --- | --- |
| P1 | Topology independent (visibility) |
| P2 | Peer-to-peer |
| P3 | Star |
| P4 | Broadcast |

45

46

47

48

49

50

51

52

53

54

268	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The same preamble sequences shall be used for all PHY types. The number of repetitions of the FLP can be              1 extended by the MAC during idle time or for different operating modes for better synchronization or to              2 provide visibility or image array receiver based device discovery.                                                                                  3

4

8.6.2 PHY header	5

6

The PHY header, as shown in Table 113, shall be transmitted with an OOK modulation. For PHY III, all	7 light sources shall transmit the same header contents simultaneously within the error tolerance specified in	8

8.3.2 and the band plan ID field shall be set to be the band plan ID of the lowest wavelength. The MAC shall              9 select the optical clock rate for communication during the clock-rate selection process, as defined in 6.5. The              10

PHY header shall be sent at the lowest data rate for the chosen optical clock rate. The clock rate does not              11 change throughout the frame between the preamble, header, and payload. If the dimmed OOK extension bit              12 is set in the PHY header for dimming support, additional fields are transmitted after the PHY header as              13 shown in Table 114.                                                                                                                                                          14

15

16

Table 113—PHY he ade r	17

| PHY header fields | Bit-width | Explanation on usage |
| --- | --- | --- |
| Burst mode | 1 | Reduce preamble and IFS |
| Channel number | 3 | Band plan ID |
| MCS ID | 6 | Provide information about PHY type and data rate |
| PSDU length | 16 | Length up to aMaxPHYFrameSize |
| Reserved fields | 5 | Future use |

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

Table 114—Dimme d OOK e xte ns io n	35

| Extension fields | Bit-width | Explanation on usage |
| --- | --- | --- |
| Compensation length | 10 | Compensation length in optical clocks |
| Resync length | 4 | Number of resync optical clocks |
| Subframe length | 10 | Length of subframe in optical clocks |
| OFCS | 8 | Optional field check sequence |

36

37

38

39

40

41

42

43

44

45

46

47

8.6.2.1 Burst mode	48

49

The burst mode bit indicates that the next frame following the current frame is part of the burst mode. Refer	50 to 5.2.2.2 for more detailed information.	51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	269

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	8.6.2.2 Channel number

2

3	The channel number indicates the code used from Table 108. The channel number field for PHY III shall be

4	the band plan ID of the lowest wavelength. Refer to 8.3.1 for more detailed information.

5

6	8.6.2.3 MCS ID

7

8	The modulation and coding scheme (MCS) ID shall be indicated in the PHY header based on Table 115.

9

10

11	Table 115—MCS ID

|  | MCS indication | PHY | Data rate | Unit |
| --- | --- | --- | --- | --- |
| 0 | 000000 | I | 11.67 | kb/s |
| 1 | 000001 |  | 24.44 |  |
| 2 | 000010 |  | 48.89 |  |
| 3 | 000011 |  | 73.3 |  |
| 4 | 000100 |  | 100 |  |
| 5 | 000101 |  | 35.56 |  |
| 6 | 000110 |  | 71.11 |  |
| 7 | 000111 |  | 124.4 |  |
| 8 | 001000 |  | 266.6 |  |
| 16 | 010000 | II | 1.25 | Mb/s |
| 17 | 010001 |  | 2 |  |
| 18 | 010010 |  | 2.5 |  |
| 19 | 010011 |  | 4 |  |
| 20 | 010100 |  | 5 |  |
| 21 | 010101 |  | 6 |  |
| 22 | 010110 |  | 9.6 |  |
| 23 | 010111 |  | 12 |  |
| 24 | 011000 |  | 19.2 |  |
| 25 | 011001 |  | 24 |  |
| 26 | 011010 |  | 38.4 |  |
| 27 | 011011 |  | 48 |  |
| 28 | 011100 |  | 76.8 |  |
| 29 | 011101 |  | 96 |  |

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

270	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 115—MCS ID  (continued)	1

|  | MCS indication | PHY | Data rate | Unit |
| --- | --- | --- | --- | --- |
| 32 | 100000 | III | 12 | Mb/s |
| 33 | 100001 |  | 18 |  |
| 34 | 100010 |  | 24 |  |
| 35 | 100011 |  | 36 |  |
| 36 | 100100 |  | 48 |  |
| 37 | 100101 |  | 72 |  |
| 38 | 100110 |  | 96 |  |
| others |  | reserved |  |  |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

editor note: need to further discuss on how to unify PHY header	19

20

8.6.2.4 Length of PSDU field	21

22

The PSDU length field specifies the total number of octets contained in the PSDU. It is a value between 0	23

and aMaxPHYFrameSize as shown in 9.5.1.	24

25

8.6.3 Header check sequence (HCS)	26

27

The PHY header shall be protected with a 2 octet CRC-16 HCS. A schematic of the CRC processing used	28

for HCS calculation is shown in Annex C. The HCS bits shall be processed in the transmit order. The	29

registers shall be initialized to all ones.	30

31

8.6.4 Optional fields	32

33

| Compensation length (see 8.6.4.1) | Resync length (see 8.6.4.2) | Subframe length (see 8.6.4.3) | OFCS (see 8.6.4.4) |
| --- | --- | --- | --- |

34

Tail bits

(see 8.6.4.1)

Channel	  35 estimation	 36 sequence	37 (see 8.6.4.5)	  38

39

(a)	(b)	(c)	40

41

Figure 141—PPDU option fields	42

43

44

The optional fields shall be formatted as shown in Figure 141. The optional fields in Figure 141 (a) shall be	45 transmitted only when PHY I is used with an optical clock of 200 kHz based on the MCS ID chosen in the	46

PHR. The optional fields in Figure 141 (b) shall be transmitted after the tail bits only if the dimmed OOK bit              47 is set in the PHR. The optional field in Figure 141 (c) shall be transmitted only if PHY III is selected based              48 on the MCS ID chosen in the PHR. The dimmed OOK mode shall not be used with PHY III. i.e., the              49 optional fields (b) and (c) shall never be used simultaneously. Optional fields (a) and (c) shall also never be              50 transmitted simultaneously since they correspond to different PHY types.                                                                     51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	271

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	8.6.4.1 Compensation length

2

3	The compensation length has a 10-bit value, which indicates the number of compensation symbols at the

4	optical clock rate. The values of these compensation symbols are user defined. When used, this field shall be

5	set to a value between 0 to 1023.

6

7	8.6.4.2 Resync length

8

9	The resync length has a 4-bit value, which indicates the number of resync symbols at the optical clock rate.

10	The resync pattern used is the same as the FLP. When used, this field shall be set to a value from 0 to 15,

11	with a default value of 15.

12

13	8.6.4.3 Subframe length and generation

14

15	The subframe length has a 10-bit value, which indicates the number of uncoded data bits in the subframe.

16	When used, this field shall be set to a value of 0 to 1023. The subframes shall be generated at the transmitter

17	after the FCS has be determined and the FEC has been applied. The FEC and FCS shall not include the

18	compensation symbols and the resync symbols. All subframes shall have the same length except for the last

19	subframe, which may be truncated to meet the frame length.

20

21	8.6.4.4 Optional field check sequence generation

22

23	The PPDU optional field check sequence (OFCS) value is calculated across the compensation length, resync

24	length and subframe length fields (as shown in Figure 141a) and inserted into the OFCS field.

25

26	The OFCS field shall be an 8-bit sequence (ITU-T I.432.1). It shall be the remainder of the division (modulo

27	2) by the generator polynomial x8 + x2 + x + 1 of the product x8 multiplied by the content of the header

28	excluding the OFCS field.

29

30	The initial content of the register of the device computing the remainder of the division is preset to all ones

31	and is then modified by division of the header, excluding the OFCS field, by the generator polynomial. The

32	resulting remainder is the 8-bit OFCS.

33

34	8.6.4.5 Channel estimation sequence

35

36	The channel estimation sequences are three optional 8-bit sequences and are used only for PHY III

37	operation. The  information about  PHY III  is  obtained  after  decoding the  PHY header. The  channel

38	estimation sequence details are discussed in 12.9.

39

40	8.6.5 PSDU field

41

42	The PSDU field has a variable length and carries the data of the PHY frame. The FCS is appended if the

43	PSDU has a non-zero byte payload. Six tail bits of zeros are attached to end of the PSDU, if PHY I is used

44	with data rates of 11.67 kb/s, 24.44 kb/s, or 48.89 kb/s. The structure of the PSDU field is as shown in

45	Figure 142.

46

47

48

49

50

51

52

53

54

272	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

Figure 142—PSDU field structure	8

9

10

The complete PPDU format for the PHY is shown in Figure 143.	11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

Figure 143—PPDU structure	41

42

43

8.7 PPDU Format	44

45

editor note: Clause 8.7 is proposed by for OFDM based PHYs. Clause 8.6 is original 15.7-2011. These two	46

clauses should be merged	47

48

8.7.1 PPDU format for low bandwidth PHY	49

50

The fields of the presented PHY frame are specified in terms of the functionality they are expected to pro-	51 vide. The different parts of the PHY frame structure in Figure 144. are envisioned to occupy different	52

53

54

Copyright © 20xx IEEE. All rights reserved.	273

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	OFDM frames in order to achieve a modular PHY frame design, which can be augmented easily based on

2	the functionality which the specific PHY implementation supports.

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20	Figure 144—PHY frame structure

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38	Figure 145—Timing parameters for the PHY control information (specification for 20 MHz)

39

40

41	8.7.1.1 Preamble

42

43	The function of the preamble is to enable the PHY to identify the existence of a packet and its beginning.

44	The preamble sequence can also be used for signal detection and automatic gain control (AGC). It is speci-

45	fied as the following 160-sample time domain sequence (having the duration equivalent of two OFDM

46	frames). The sequence has been normalized by the factor 1/sqrt(66) in order to ensure an average energy

47	level of 1 in the information signal. Four possible preamble sequences have been defined, where different

48	nodes communicating to each other can adopt different sequences. The sequences have very low cross-cor-

49	relation to each other, which eliminates the possibility for false packet detection between interfering nodes

50	and enabling interference estimation, which in turn, enables interference management techniques at the

51	higher MAC layer.

S1

52

53	160 = {0 0 2 -7 -8 4 9 15 9 -4 -1 -9 -9 4 9 15 9 -4 -2 -10 -8 4 10 14 8 -4 -6 5 6 -4 -10 -15 -8 4 6 -5 -6 4 9 15

54	9 -4 -1 -9 -9 4 8 15 9 -4 -1 -9 -9 4 9 14 8 -5 -6 4 6 -4 -9 -15 -9 3 1 9 8 -4 -9 -15 -9 3 1 9 8 -4 -9 -15 -9 4 2 9 8

274	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

-5 -8 -15 -9 4 1 9 8 -4 -10 -14 -8 4 6 -5 -6 4 8 15 9 -4 -2 -10 -9 4 10 14 8 -4 -7 5 6 -4 -10 -15 -8 4 6 -5 -6 4 10              1

14 8 -4 -6 4 6 -5 -9 -15 -9 3 1 9 8 -5 -9 -14 -9 4 6 -5 -6 4 10 14 8 -5 -6 5 6 -4 -10 -15}/sqrt(66)                                    2

3

S2160 = {0 0 -2 7 7 -4 -10 -15 -9 4 6 -5 -6 4 10 14 8 -4 -6 4 6 -4 -9 -15 -9 4 1 9 8 -5 -10 -15 -8 5 6 -5 -6 4 10              4

14 8 -4 -7 4 6 -4 -10 -15 -8 4 6 -5 -6 4 8 15 9 -3 -2 -9 -9 4 10 14 8 -5 -6 5 6 -4 -9 -15 -9 4 1 9 8 -5 -9 -15 -9 4              5

1 9 8 -5 -8 -15 -9 3 1 9 8 -4 -9 -15 -9 3 2 9 9 -4 -10 -14 -8 4 6 -5 -6 4 9 15 9 -4 -2 -9 -8 4 9 15 9 -4 -2 -10 -8 5              6

9 15 8 -4 -7 5 6 -4 -10 -15 -8 4 6 -5 -6 4 9 15 9 -3 -2 -9 -9 4 9 15 9 -4 -2 -10 -8 4 9 15}/sqrt(66)                                   7

8

S3160 = {0 0 -2 7 7 -5 -9 -15 -10 4 1 9 8 -5 -9 -15 -9 4 1 8 8 -4 -8 -15 -9 4 1 9 8 -4 -10 -14 -8 4 6 -5 -6 4 9 14              9

8 -5 -6 5 6 -4 -10 -15 -9 4 6 -5 -7 4 10 15 8 -5 -7 5 6 -4 -10 -15 -8 4 6 -5 -7 4 8 15 10 -4 -1 -9 -8 4 8 15 9 -3 -              10

1 -9 -9 4 9 15 9 -3 -2 -9 -8 4 9 14 8 -4 -6 4 6 -4 -9 -15 -9 3 1 9 8 -5 -9 -15 -8 4 6 -4 -6 4 9 15 10 -4 -1 -10 -8 5              11

10 14 8 -4 -6 4 6 -5 -10 -15 -8 4 6 -5 -7 4 9 15 9 -4 -1 -9 -8 4 10 14 8 -5 -6 5 6 -4 -10 -15}/sqrt(66)                             12

13

S4160 = {0 0 -2 7 7 -4 -9 -15 -8 4 6 -5 -6 4 9 15 9 -4 -2 -10 -9 4 10 15 8 -4 -6 4 6 -4 -10 -14 -8 4 6 -5 -6 4 9 15              14

9 -4 -2 -10 -9 4 9 14 8 -4 -6 4 6 -5 -9 -15 -9 4 1 9 8 -4 -9 -14 -8 4 6 -5 -6 4 9 15 9 -3 -2 -9 -8 4 9 15 9 -4 -2 -9              15

-9 4 9 15 9 -4 -2 -9 -9 4 9 14 8 -5 -6 4 6 -4 -10 -14 -8 4 6 -5 -6 4 10 14 8 -4 -7 5 6 -4 -10 -15 -9 4 6 -5 -6 4 9              16

14 7 -4 -6 5 6 -4 -9 -15 -9 3 1 9 8 -5 -8 -15 -9 4 1 9 8 -5 -9 -15 -9 3 1 9 8 -5 -10 -15}/sqrt(66)                                      17

18

The time-domain cross-correlation of the specified preamble delivers an easily-identifiable sharp peak value              19 which is expected to be used for packet detection and synchronization.                                                                          20

21

8.7.1.2 Channel Estimation                                                                                                                          22

23

A sequence of two identical OFDM training symbols is used to estimate the channel impulse response. The              24 sequence can also be used for additional fine-timing synchronization. The channel estimation sequence con-              25 tains the following values modulated on the subcarriers of two identical OFDM frames (index 0 corresponds              26 to the DC subcarrier modulation value):                                                                                                                           27

28

E0 to 63 = {0 0 0 -1 -1 1 -1 1 -1 -1 1 1 1 -1 -1 -1 1 -1 1 1 1 1 1 -1 -1 1 1 -1 1 0 0 0 0 0 0 0 1 -1 1 1 -1 -1 1 1 1              29

1 1 -1 1 -1 -1 -1 1 1 1 -1 -1 1 -1 1 -1 -1 0 0}                                                                                                                     30

31

The sequence is Hermitian symmetric in order to satisfy the requirement for a real time-domain signal after              32 the IDFT and has very good auto-correlation properties. In Figure 145, the channel estimation OFDM sym-              33 bol is transmitted twice in two identical copies of the time-equivalent signal of the frequency-domain modu-              34 lation sequence E0 to 63. The time-domain signal is obtained after an IDFT operation on E0 to 63. The guard              35 interval GI2 is a cyclic extension of this same time-domain signal and has a duration of 32 samples (twice              36 the length of the typical cyclic extension for this PHY mode specification).                                                                   37

38

8.7.1.3 Header                                                                                                                                                39

40

The Header contains all information necessary for demodulating the subsequent frame payload. It is always              41 encoded in 1/2 FEC rate BPSK modulation using DCO-OFDM. The header is logically split into three sec-              42 tions, which are encoded in separate OFDM frames (each part may be encoded in more than one OFDM              43 frame) in order to achieve a modular PHY frame design.                                                                                                44

45

The two components of the header in the current PHY take the following form.                                                            46

47

8.7.1.3.1 Bas ic He ade r                                                                                                                                                48

49

The basic header contains the minimum information required for demodulating the subsequent payload. In              50 the low-bandwidth PHY mode, this header includes information such as the constellation size, the FEC rate              51 and the payload size. The Basic Header also indicates whether an Advanced Modulation Header (containing              52 further information regarding the modulation format) is used. If no advanced modulation header is used,              53 then the DATA portion of the PHY frame is expected to appear after the basic header. The basic header con-              54

Copyright © 20xx IEEE. All rights reserved.	275

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	tains 24 bits and fits within 2 OFDM frames of the current PHY mode. The field contains the information

2	and is structured as depicted in Figure 146.

3

4

5

6

7

8

9

10

11

12

13	Figure 146—Structure of the Basic Header

14

15	The individual fields of the basic header are described as follows.

16

17	8.7.1.3.2 RATE

18

19	The RATE field consists of three bits and indicates the QAM constellation size and the FEC rate (achieved

20	with the use of a convolutional encoder and puncturing) used for the subsequent payload. The values speci-

21	fied in Table 116 are valid for the RATE field.

22

23

24

25

26	Table 116— Valid RATE value s

27

| R0-R2 | Modulation | FEC rate | Data rate (Mb/s) (40 MHz) | Data rate (Mb/s) (20 MHz) | Data rate (Mb/s) (15 MHz) | Data rate (Mb/s) (10 MHz) | Data rate (Mb/s) (5 MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 110 | BPSK | 1/2 | 12 | 6 | 4.5 | 3 | 1.5 |
| 111 |  | 3/4 | 18 | 9 | 6.75 | 4.5 | 2.25 |
| 010 | 4-QAM | 1/2 | 24 | 12 | 9 | 6 | 3 |
| 011 |  | 3/4 | 36 | 18 | 13.5 | 9 | 4.5 |
| 100 | 16-QAM | 1/2 | 48 | 24 | 18 | 12 | 6 |
| 111 |  | 3/4 | 72 | 36 | 27 | 18 | 9 |
| 010 | 64-QAM | 2/3 | 96 | 48 | 36 | 24 | 12 |
| 011 |  | 3/4 | 108 | 54 | 40.5 | 27 | 13.5 |

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46	8.7.1.3.3 Re s e rve d bits (R)

47

48	One bit is reserved for introducing additional transmission rates in future modifications of the standard.

49

50	8.7.1.3.4 LENGTH

51

52	An 11-bit field which indicates the size of the payload in octets. Hence, the size of the payload is between 0

53	and 2048 octets.

54

276	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

8.7.1.3.5 Advanc e d Mo dulatio n He ade r Bit (A)	1

2

One bit indicating whether an Advanced Modulation Header is included in the next OFDM frame:	3

4

1  The next OFDM frame is an Advanced Modulation Header.	5

6

0  The next OFDM frame is part of the DATA portion (payload) of the PHY frame.	7

8

8.7.1.3.6 Hig h-re liability MAC He ade r Bit (M)	9

10

One bit indicating whether a high-reliability MAC header encoding is used. If the advanced modulation              11 header is not enabled, then the next OFDM frame encodes the high-reliability MAC header described in sub-              12 clause 6.2.5.1.                                                                                                                                                                    13

14

1  The OFDM frame following the advanced modulation header is part of the high-reliability MAC	15 header encoding.	16

17

0  The OFDM frame following the advanced modulation header is part of the conventional DATA por-	18 tion (payload) of the PHY frame encoding using the RATE specified in subclause	19

20

8.7.1.3.7 P arity Che c k Bit (P)	21

22

An even parity check bit for the information in bits 0 - 16.	23

24

8.7.1.3.8 S IGNAL TAIL	25

26

Six bits set to zero complete the Basic Header. These bits re-set the state machine of the convolutional	27 encoder used in the current PHY mode.	28

29

8.7.1.3.9 Advanc e d Mo dulatio n He ade r	30

31

The advanced modulation header is encoded in separate OFDM frames from the Basic Header, so that the              32 overall structure is modular, i.e., when the advanced modulation header is not required, it can be easily              33 excluded. Like the basic header, the advanced modulation header is encoded using 1/2 FEC rate BPSK. The              34 advanced modulation header is an optional field which contains the information necessary for demodulating              35 the subsequent waveform. It contains information necessary to identify if adaptive bit and energy allocation              36 is used. It also contains commands for the PHY necessary for estimating the channel quality indicators              37 (CQIs), necessary for enabling adaptive bit and energy loading during the real-time operation of the system.              38

The format is presented in Figure 147.	39

40

41

42

43

44

45

46

47

48

49

50

51

52

Figure 147—Advanced Modulation Header format	53

54

Copyright © 20xx IEEE. All rights reserved.	277

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	8.7.1.3.10 Adaptive Bit Lo ading Us e (B)

2

3	One bit indicating whether adaptive bit loading is used to encode the subsequent payload:

4

5	1  Adaptive bit loading is used. In this case, the subsequent payload is encoded with the loading scheme

6	which the receiving station has estimated and suggested to the transmitter using the feedback channels and

7	negotiation procedures in the MAC layer.

8

9	0  Adaptive bit loading is not used. In this case, the subsequent payload is encoded with the default load-

10	ing scheme and the rate specified in the Basic Header.

11

12	The low-bandwidth PHY mode is not required to support adaptive bit loading. If a PHY which does not sup-

13	port bit loading encounters a packet which has this bit set to 1, it will ignore the packet, as it will no be able

14	to demodulate it.

15

16	8.7.1.3.11 Es tim ate CQIs (C)

17

18	One bit indicating whether the CQIs should be calculated in the PHY for the current transmission frame. The

19	channel estimation symbols preceding the PHY header are used for the estimation of the CQIs if the MIMO

20	mode is not enabled. If the MIMO mode is enabled, the MIMO reference symbols described in 6.2.4.

21

22	1  the CQIs should be estimated

23

24	0  the CQIs should not be estimated

25

26	Upon estimation of the CQIs, the PHY conveys the results to the MAC using a predefined PHY service

27	primitive. The calculation of the bit and power allocation scheme as well as the necessary exchange for

28	updating the bit and power allocation scheme at the transmitter and receiver are handled at the MAC layer.

29

30	8.7.1.3.12 Us e o f e U-OFDM (e U)

31

32	Indicates whether the PHY DATA (payload) field is encoded using eU-OFDM.

33

34	1  the DATA (payload) is encoded using eU-OFDM.

35

36	0  the DATA (payload) is not encoded using eU-OFDM.

37

38	Note that the use of this alternative waveform can be negotiated in advance using control/management

39	frames. Furthermore, the use of eU-OFDM encoding does not prohibit the use of any additional advanced

40	waveforms such as SC-FDMA or RPO-OFDM.

41

42	8.7.1.3.13 Numbe r o f S tre ams in e U-OFDM (S TR)

43

44	Two bits indicating the number of U-OFDM streams superimposed in the signal encoding procedure. The

45	valid values for this field are:

46

47	8.7.1.3.14 Us e o f S ing le -c arrie r FDMA

48

49	Indicates whether the PHY DATA (payload) field is encoded using SC-FDMA.

50

51	1  the DATA (payload) is encoded using SC-FDMA.

52

53	0  the DATA (payload) is not encoded using SC-FDMA.

54

278	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 117— Valid S TR value s	1

| STR | Number of Streams |
| --- | --- |
| 00 | 1 |
| 10 | 2 |
| 01 | 3 |
| 11 | 4 |

2

3

4

5

6

7

8

9

10

11

12

Note that the use of this alternative waveform can be negotiated in advance using control/management	13

frames. Furthermore, the use of SC-FDMA encoding does not prohibit the use of any additional advanced	14

waveforms such as eU-OFDM or RPO-OFDM.	15

16

8.7.1.3.15 S C-FDMA pre -c o de r DFT S ize (DFT)	17

18

A number specifiying the DFT size used in the SC-FDMA pre-coding procedure. As only 24 subcarriers are	19

modulated with unique data, the DFT size is a number between 1 and 24.	20

21

8.7.1.3.16 Numbe r o f Re ve rs e Po larity Optic al OFDM frame s (NRP O)	22

23

Indicates the number of OFDM data frames which are encoded using RPO-OFDM as described in subclause	24

6.3.8. By convention, the LSB of this number is transmitted first. A value of NRPO='000000' indicates that	25

RPO-OFDM is not used.	26

27

Note that the use of this alternative waveform can be negotiated in advance using control/management	28

frames. Furthermore, the use of RPO-OFDM encoding does not prohibit the use of any additional advanced	29

waveforms such as eU-OFDM or SC-FDMA.	30

31

8.7.1.3.17 S ig nal s c aling fac to r in Re ve rs e Po larity Optic al OFDM frame s (GRPO)	32

33

Indicates  the  factor  by  which  the  OFDM  data  signal  is  scaled  as  described  in  subclause  6.3.8.  If	34

NRPO='000000', this field is not relevant for the signal transmission.	35

36

8.7.1.3.18 Re laying Ope ratio n Enable d (RE)	37

38

1  Relaying should be performed for the current PHY frame.	39

40

0  Relaying should not be performed for the current PHY frame.	41

42

8.7.1.3.19 Re laying Ope ratio n Type (RT)	43

44

Two bits which specify the type of relaying operation that should be performed. The valid values for this	45

field are specified as:	46

47

8.7.1.3.20 MIMO Enable Bit (M)	48

49

One bit indicating whether a MIMO mode is enabled for the current PHY frame:	50

51

1  MIMO mode is enabled and the subsequent payload is encoded using the MIMO scheme already	52

negotiated by the transmitter and the receiver.	53

54

Copyright © 20xx IEEE. All rights reserved.	279

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 118— Valid RT value s

| RT | Relaying and Duplexing Mode |
| --- | --- |
| 00 | FD-AF |
| 01 | FD-DF |
| 10 | HD-AF |
| 11 | HD-DF |

2

3

4

5

6

7

8

9

10

11

12

13	0  MIMO mode is not enabled and the subsequent payload is encoded using the SISO scheme specified

14	by the parameters in the basic header and the advanced modulation header.

15

16	8.7.1.3.21 MIMO Re fe re nc e S ymbo ls Fo rmat (RS )

17

18	One bit which indicates the format of the reference symbols used for CQI estimation.

19

20	1  MIMO Reference Symbols Format I is used.

21

22	0  MIMO Reference Symbols Format II is used.

23

24	The two MIMO reference symbols formats are specified in 6.2.4.

25

26	8.7.1.3.22 6.2.3.2.13. Parity Che c k Bit (P)

27

28	An even parity check bit for the information in bits 0 - 16.

29

30	8.7.1.3.23 S IGNAL TAIL

31

32	Six bits set to zero complete the advanced modulation header. These bits re-set the state machine of the con-

33	volutional encoder used in the current PHY mode.

34

35	8.7.1.3.24 MIMO Re fe re nc e S ymbo ls (RS s )

36

37

38

39

40

41

42

43

44

45

46

47	Figure 148—High-reliability MAC header structure

48

49	The MIMO reference symbols constitute NMIMO OFDM frames (or the time-frame equivalent of NMIMO

50	OFDM frames), where NMIMO = number of MIMO channels. Depending on the reference symbol format in

51	use, the bit RS is set to '0' for reference symbols format I and set to '1' for reference symbols format II. The

52	symbol formats are described as follows.

53

54

280	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

8.7.1.4 Data	1

2

The Data field contains the MPDU transferred from the higher layer. The size of this field is specified in the	3 basic PHY header. The MPDU is encoded according to the modulation format specified in the PHY header.	4

The Data field contains a Service field, a MAC header portion of the MPDU encoded in the most robust              5 modulation format, as well as the remaining portion of the MPDU complemented with a TAIL field and pad-              6 ding bits, which extend the length of the field to an integer number of OFDM frames.                                                  7

8

8.7.1.4.1 Hig h-re liability MAC He ade r	9

10

Robust transmission of the polling and acknowledgement information ensures avoiding a lot of unnecessary              11 retransmissions. Furthermore, when this information is encoded separately from the rest of the payload,              12 errors in the payload (especially for long payloads) which cause the packet to be discarded (and retransmit-              13 ted) do not influence the polling and acknowledgement mechanism. As a result, the MAC header (the part of              14 the header required for polling and acknowledgments) is encoded in a special high-reliability header using              15 the lowest data-rate (most robust) modulation format 1/2 FEC rate BPSK separately from the data payload.              16

The high-reliability MAC header has the structure described in Fig. 6.2.5.2.1.	17

18

8.7.1.4.2 P ro to c o l Ve rs io n	19

20

The first two bits of the MAC frame indicating the MAC protocol version. If an incompatible MAC frame	21 version is encountered in this frame, the PHY will drop the rest of the packet and signal an incompatible	22

MAC protocol version to the MAC layer management entity.	23

24

8.7.1.4.3 P o lle d s tatio n numbe r	25

26

In the downlink transmission, this field specified the number of the station which is being polled by the	27 access point. On the uplink, these bits are left unspecified.	28

29

8.7.1.4.4 P ac ke t s e que nc e numbe r	30

31

A twelve-bit number which specifies the sequence number of the packet that is being acknowledged.	32

33

8.7.1.4.5 Ac kno wle dg e me nt (ACK)	34

35

A two-bit field which enables the AP and the stations to acknowledge packet reception as well as beacon              36 reception. Bit 30 is set to '1', when a transmitting node (station or access point) is acknowledging the recep-              37 tion of the packet with the sequence number specified by the field described in 6.2.5.1.3. Bit 31 is set to '1',              38 when a station is acknowledging the reception of a beacon frame, indicating it is still active and connected to              39 the AP.                                                                                                                                                                               40

41

8.7.1.4.6 Re s e rve d bits (R)	42

43

Nine bits (at positions 32-40) are reserved for future use. Hence, they have not been specified.	44

45

8.7.1.4.7 P arity c he c k bit (P)	46

47

An even parity check bit for the information in bits 0 - 40.	48

49

8.7.1.4.8 S IGNAL TAIL	50

51

Six bits set to zero complete the high-reliability MAC header. These bits re-set the state machine of the con-	52 volutional encoder used in the current PHY mode.	53

54

Copyright © 20xx IEEE. All rights reserved.	281

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	8.7.1.4.9 S e rvic e Fie ld

2

3	The Service field has 16 bits,  which shall be denoted as bits 0-15.  The bit 0 shall be transmitted first in

4	time. The bits from 0-6 of the SERVICE field, which are transmitted first, are set to zeros and are used esti-

5	mate the initial state of the transmitter scrambler and to synchronize the descrambler in the receiver. The

6	remaining 9 bits (7-15) of the SERVICE field shall be reserved for future use. All reserved bits shall be set to

7	0. Refer to Figure 149.

8

9

10

11

12

13

14

15

16

17	Figure 149—SERVICE field bit assignment

18

19

20	8.7.1.4.10 Ge ne ral Paylo ad Data

21

22	The general payload is encoded according to the modulation format specified in the PHY header. This field

23	contains the MPDU transferred from the higher layer except for the parts of the MAC Header already

24	encoded in the high-reliability MAC header as described in 6.2.5.2. followed by a tail field of six zero bits

25	necessary to reset the convolutional encoder and a four-byte frame check sequence. In case the DATA field

26	does not fit within an exact number of OFDM frames, padding bits are introduced.

27

28	8.7.1.4.11 Tail Fie ld

29

30	The PPDU TAIL field shall be six zero bits, which are required to return the convolutional encoder to the

31	zero state. This procedure improves the error probability of the convolutional decoder, which relies on future

32	bits when decoding and which may not be available past the end of the message. The PLCP tail bit field shall

33	be produced by replacing six scrambled zero bits following the message end with six nonscrambled zero

34	bits.

35

36	8.7.1.4.12 Padding Bits

37

38	The number of bits in the DATA field shall be a multiple of NCBPS, the number of coded bits in an OFDM

39	symbol (24, 48, 96, or 144 bits). To achieve this, the length of the message is extended so that it becomes a

40	multiple of NDBPS, the number of data bits per OFDM symbol. At least 6 bits are appended to the message,

41	in order to accommodate the TAIL bits, as described in 3.5.2. The number of OFDM symbols, NSYM; the

42	number of bits in the DATA field, NDATA; and the number of pad bits, NPAD, are computed from the length

43	of the PSDU (LENGTH) as follows:

44

45	N SYM

46

47

 Ceiling 16  8  LENGTH  6

N DBPS 

(6)

48	N DATA   N SYM   N DBPS

49

50

51	N PAD   N DATA  16  8  LENGTH  6

52

53

54

(7) (8)

282	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

Figure 150—Data Scrambler / Descrambler	12

13

The function Ceiling (.) is a function that returns the smallest integer value greater than or equal to its argu-              14 ment value. The appended bits ("pad bits") are set to zeros and are subsequently scrambled with the rest of              15 the bits in the DATA field.                                                                                                                                                 16

17

8.7.1.4.13 Data s c ramble r and de s c ram ble r	18

19

The DATA field, composed of SERVICE, PSDU, tail, and pad parts, shall be scrambled with a length-127              20 frame-synchronous scrambler. The octets of the PSDU are placed in the transmit serial bit stream, bit 0 first              21 and bit 7 last. The frame synchronous scrambler uses the generator polynomial S(x) as follows, and is illus-              22 trated in Figure 150:                                                                                                                                                          23

24

25

S x  x7

 x 4   1

(9)	26

27

The 127-bit sequence generated repeatedly by the scrambler shall be (leftmost used first), 00001110	28

11110010 11001001 00000010 00100110 00101110 10110110 00001100 11010100 11100111 10110100	29

00101010 11111010 01010001 10111000 1111111, when the all ones initial state is used. The same scram-              30 bler is used to scramble transmit data and to descramble receive data. When transmitting, the initial state of              31 the scrambler will be set to a pseudo-random nonzero state. The seven LSBs of the SERVICE field will be              32 set to all zeros prior to scrambling to enable estimation of the initial state of the scrambler in the receiver.?                33

34

8.7.2 PPDU Format for high bandwidth PHY	35

36

8.7.2.1 Frame structure	37

38

The data signal, denoted as payload in Figure 151, is transmitted together with additional signals in a com-	39 pound frame, so that the payload can be decoded frame-wise. The preamble is followed by a PHY header.	40

The preamble allows coarse time synchronization in its first two parts, followed by a reference sequence              41 used for fine timing as well as channel estimation, what enables the receiver to decode the header informa-              42 tion. After the header, eventually there are additional channel estimation (ACE) symbols, basically needed              43 for the support of multiple-input multiple-output (MIMO), before the payload is then transmitted. In case of              44 using MIMO in 802.15.13, all transmitters transmit the same preamble and header information, while only              45 the payload makes explicit use of MIMO. The ACE symbols contain an orthogonal sequence of symbols              46 (same symbols but with alternate signs) in order to estimate the MIMO channel.                                                           47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	283

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

| Preamble Channel | Header 1, 2 or 4 symbols | Additional channel estimation symbols | Payload |
| --- | --- | --- | --- |

4

5	estimation symbol

6

7

8

9

10	Figure 151—P2P PHY frame, taken over from G.hn specification

11

12	8.7.2.2 Frame types

13

14	By using the same frame structure, the PHY can transport different frame types. An overview is provided in

15	Table 119.

16

17	editor’s note: frame types in 802.15.7 and 802.15.4 are on MAC level. Different types of MAC frame types

18	are specified in 5.4. Please consider how to harmonize.

19

20

21

22

23	Table 119—PHY fram e type s

24

| Frame type | Frame type (FT) | Description | Header | Payload | Duration indicator (DRI( | Ext. Header indicator |
| --- | --- | --- | --- | --- | --- | --- |
| MAP/ RMAP | 0000 | A frame carrying the medium access plan (MAP) or relayed MAP (RMAP) where the payload contains a MAC Pro- tocol Data Unit (MPDU) | Yes | Yes | 1 | 0 |
| MSG | 0001 | Data and management frame, carrying user data or manage- ment data or both; the pay- load contains an MPDU | Yes | Yes | 1 | 0 |
| ACK | 0010 | Acknowledge (ACK) control frame, the relevant ARQ data is communicated in the header | Yes | No | 1 | 0 or 1 |
| RTS | 0011 | A request-to-send control frame; the relevant data is communicated in the header | Yes | No | 1 | 0 |
| CTS | 0100 | A clear-to-send control frame; the relevant data is communicated in the header | Yes | No | 1 | 0 |
| CTMG | 0101 | Sort control frame, carrying a short control message | Yes | No | 1 | 0 or 1 |
| PROBE | 0110 | A frame carrying probe sym- bols in its payload | Yes | Yes | 1 | 0 |

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

284	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

8.7.2.3 Preamble

Table 119—PHY fram e type s	1

| Frame type | Frame type (FT) | Description | Header | Payload | Duration indicator (DRI( | Ext. Header indicator |
| --- | --- | --- | --- | --- | --- | --- |
| ACKRQ | 0111 | An ACK retransmission request frame; the relevant data is communicated in the header | Yes | No | 0 | 0 |
| BMSG | 1000 | A bidirectional MSG frame, it contains data and manage- ment frames in the payload and ACK | Yes | Yes | 1 | 0 or 1 |
| BACK | 1001 | A bidirectional ACK frame, contains ACK and data and management frames in the payload | Yes | Yes | 1 | 0 or 1 |
| ACTMG | 1010 | An acknowledge frame for a control message (CTMG) frame | Yes | No | 0 | 0 |
| Reserved | 1011 to 1110 | Reserved frame types |  |  |  |  |
| FTE | 1111 | Frame Type Extension, pointer to a set of additional frame types | Yes | No | 0 or 1 | 0 or 1 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

The preamble is prepended to a PHY frame and is intended for frame detection/synchronization, initial              31 channel estimation and OFDM symbol alignment. It may be used for recovery of the following header seg-              32 ment of the PHY frame.                                                                                                                                                    33

34

The preamble is divided into three sections; each consists of a repetition of Nn OFDM symbols Sn, where the              35 subscript n=1,2,3 denotes the section. Each Sn is output by the IFFT. The inputs of the IFFT, i.e. complex              36 valued symbols, are generated by feeding into the constellation scrambler described in 2.2.2.7.2 a constant              37 series of 1s. The three sections are windowed and combined additively as outlines in Figure 152.                                38

39

40

41

42

43

44

Figure 152—Preamble structure	45

46

47

In the default case, every kn-th subcarrier is used, starting from the subcarrier index i=0. For n=3, the used	48 subcarrier spacing kn and the LSFR seed differs from the cases in which n=1,2 as described in Table 120.	49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	285

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 120— De fault pro pe rtie s o f pre amble s ub-s e que nc e

| property | Section 1 | Section 2 | Section 3 |
| --- | --- | --- | --- |
| OFDM symbol | S1 | S1=-S1 | S3 |
| Nn | 10 | 4 | 2.5 |
| kn | 4 | 4 | 1 |
| LFSR seed | 16E616 | - | 1105 |

2

3

4

5

6

7

8

9

10

11

12

13	8.7.2.4 Channel estimation

14

15	The final OFDM symbol in the preamble is used for channel estimation. It is useful to detect the header

16	information and when using single-input single-output (SISO) transmission, it can also be used for the

17	detection of data. In case MIMO is used additional channel estimation (ACE) symbols are used for channel

18	estimation. These additional symbols are sent after the header, as part of the data block in the frame.

19

20	8.7.2.5 PHY frame header

21

22	The PHY-frame header always fits into an integer number of OFDM symbols and is transmitted using a sin-

23	gle, predefined set of modulation and coding parameters. i.e. 1 bit/symbol with code rate ½. The core part of

24	the PHY-frame header is 168 bits long. It is composed of a common part and a variable part. The fields of the

25	PHY-frame header are defined in the following table.

26

27

28

29	Table 121—PHY-frame he ade r de finitio n

30

| Field | Octet | Bits | Description |
| --- | --- | --- | --- |
| Common Port |  |  |  |
| FT | 0 | [3:0] | Frame type |
| DOD |  | [7:4] | Domain ID |
| SID | 1 | [7:0] | DEVICE_ID of the source node |
| DID | 2 | [7:0] | DEVICE_ID, MULTICAST_ID or BROADCAST_ID of the destination node(s) |
| MI | 3 | [0] | Multicast indication identifying whether the DID is a unicast or multicast destination |
| DRI |  | [1] | Duration indication identifying whether FTSF starts with a 16-bit duration field |
| EHI |  | [2] | Extended header indication |
| HSI |  | [3] | Header segmentation indication |
| Reserved |  | [7:4] | Reserved (zero if not used) |

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

286	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 121—PHY-frame he ade r de finitio n	1

| Field | Octet | Bits | Description |
| --- | --- | --- | --- |
| Variable Port |  |  |  |
| FTSF | 4 to 18 | [119:0] | Frame-type specific field |
| Common Port |  |  |  |
| HCS | 19+20 | [15:0] | Header check sequence |

2

3

4

5

6

7

8

9

10

11

12

The common part contains fields that are common for all PHY-frame types. The variable frame-type specific	13

field (FTSF) part contains fields according to the PHY-frame type. The content of the core part is protected	14

by the 16-bit header check sequence (HCS).	15

16

8.7.2.5.1 Co mmo n part fie lds o f the PHY-fram e he ade r	17

18

The frame type (FT) field is a 4-bit field that indicates the type of PHY frame.	19

20

The domain ID (DOD) field contains the domain ID to which the source and destination devices of the PHY	21

frame belong. It is represented as a 4-bit unsigned integer with valid values in the range from 0 to 15. Value	22

0 is a special value reserved for inter-domain communication.	23

24

The destination ID (DID) field identifies the destination node(s) of the PHY frame. It is represented as an 8-	25

bit unsigned integer with valid values in the range from 0 to 250.	26

27

If the multicast indication (MI) bit is set to zero, the DID field shall contain the DEVICE_ID of the destina-	28

tion node (for unicast transmission). If the MI bit is set to one, the DID field shall contain a MULTICAST-	29

_ID or BROADCAST_ID of the destination nodes.	30

31

If the duration indication (DRI) bit is set to zero, the PHY frame shall not contain any payload (i.e., contains	32

only preamble and PHY-frame header). If the DRI bit is set to one, the FTSF shall start with a duration field.	33

The duration field contains the duration of a single PHY frame or PHY frame sequence. It shall be repre-	34

sented as a 16-bit unsigned integer with valid values in steps of 0.25 us. It shall be the smallest integer larger	35

than or equal to the actual duration.	36

37

If the extended header indication (EHI) field is set to zero, the PHY-frame header shall contain 168 informa-	38

tion bits. If it is set to one, the PHY frame header shall contain 2×168 information bits. The EHI field shall	39

be set according to the frame type as shown in Table 121.	40

41

The header check sequence (HCS) field is intended for PHY-frame header verification. It is a 16-bit cyclic	42

redundancy check (CRC) and shall be computed over all the fields of the PHY-frame header in the order	43

they are transmitted, starting with the LSB of the first field of the PHY frame header (FT) and ending with	44

the MSB of the last field of the FTSF. The HCS is computed using the following generator polynomial of	45

degree 16	46

48

G(x) = x16 + x12 + x5 + 1	47

49

The value of the HCS is the remainder after the contents (treated as a polynomial where the first input bit is              50 associated with the highest degree, X168-17, where 168 is the header length in bits, and the last input bit is              51 associated with X0 of the calculation field is multiplied by x16 and then divided by G(x). The HCS field is              52 transmitted starting with the coefficient of the highest order term.                                                                                  53

54

Copyright © 20xx IEEE. All rights reserved.	287

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	8.7.2.5.2 Variable part fie lds o f the PHY-frame he ade r

2

3	The ITU-T recommendation G.9960-2015 contains a very detailed description of the variable frame-type

4	specific field (FTSF) in the PHY-frame header. Here, only the main parameters in each frame type are

5	explained.

6

7	The medium access plan (MAP) and relayed MAP (RMAP) PHY frames are used to schedule transmission

8	opportunities among the devices in a direct or relayed transmission. The FTSF of MAP and RMAP frames

9	contains the duration of the MAP frame, the network time reference, i.e. the start of the first OFDM symbol

10	of the preamble with 10 ns resolution. The start time of the next MAC cycle, the size of repetition blocks,

11	scrambler initialization value, the information block size of the FEC code word used for the payload, the

12	number of repetitions used for encoding the data, the concatenation factor in the FEC, the band plan which

13	contains the used bandwidth for this frame. The MAP type indicates what kind of bit allocation table is used

14	for the bitloading of the payload. Finally, the type of MAP or RMAP transmission is indicated and the num-

15	ber of hops from the master domain.

16

17	The data and management frame (MSG) is used to carry user data or management data or both. The parame-

18	ters in the MSG frame are the duration of the MSG frame, the information block size of the payload, code

19	rate, number of repetitions, concatenation factor, scrambler initialization, and if a master is detected or not.

20	Next there are the identifier of the bit allocation table, either the band plan (for uniform number of bits

21	loaded) or the subcarrier grouping used for the bit loading, what cyclic prefix is used, a connection identifier,

22	whether or not an acknowledgement reply is required, the number of the PHY frame in a burst frame and the

23	burst end flag. An important number for the use of MIMO is the number of additional channel estimation

24	symbols (ranging from 0 to 7). Notice also the important connection management field, by which connec-

25	tions can be initialized or released, with or without acknowledgement but always without transporting a pay-

26	load. Moreover, the MSG frame contains information about the bandwidth reservation, by which the devices

27	get informed about the length of the connection queue, requests for bidirectional transmissions and further

28	information to control the transmission of the payload.

29

30	The acknowledgement frame (ACK) is used to communicate the relevant ARQ data in the header. It con-

31	tains detailed information for the flow control, requests for bidirectional transmissions, a bad burst indicator,

32	means for channel estimation control, the identifier for the runtime (i.e. time-variant) bit allocation table and

33	a complex field for acknowledgement data depending on whether unicast or multicast is being used. Essen-

34	tially, this field indicates to the transmitter what segment of a frame was lost. Obviously, that list is being

35	used to control the selective repeat mechanism, which is one of the key features in G.hn to reduce the latency

36	and improve the robustness likewise.

37

38	The request-to-send (RTS) control frame contains specific means to establish the exchange of data. It con-

39	tains the transmission times of the RTS, CTS and MSG frames, as well as of eventually needed ACK frames

40	and their duration, the device ID of the node that should respond to the request.

41

42	The PROBE FTSF is composed of a common part and a variable part. The common part contains fields for

43	the duration of the probe frame and its type, containing either silent signals or channel estimation probe

44	symbols, the number of probe symbols, the used probe guard interval and an additional probe frame type

45	specific field.

46

47	The ACK retransmission request (ACKRQ) frame shall allow the receiver to request retransmission of spe-

48	cific receiver windows for either data or management connections or both. Moreover, there are bidirectional

49	variants of the bidirectional MSG (BMSG) and ACK (BACK) frames and an opportunity to add additional

50	frame types by using the frame type extension (FTE) and an opportunity to add an extended header which is

51	similar to the original frame types while adding further functionalities that the original frames can no longer

52	host. This information is sent in further OFDM symbols immediately after the original header.

53

54

288	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

9. PHY service specifications	1

2

3

9.1 Overview	4

5

The PHY provides an interface between the MAC sublayer and the physical optical channel. The PHY	6

conceptually includes a management entity called the PLME. This entity provides the layer management	7

service  interfaces  through  which  layer  management  functions  may  be  invoked.  The  PLME  is  also	8

responsible for maintaining a database of managed objects pertaining to the PHY. This database is referred	9

to as the PHY PAN information base (PIB).	10

11

Figure 153 depicts the components and interfaces of the PHY.	12

13

14

15

16

17

18

19

20

21

22

Figure 153—PHY layer service access points	23

24

25

The PHY provides two services, accessed through two SAPs: the PHY data service, accessed through the	26

PHY  data  SAP  (PD-SAP),  and  the  PHY  management  service,  accessed  through  the  PLME’s  SAP              27 (PLMESAP). The optical SAP provides an interface between the PHY layer and the optical channel and is              28 not specified in this standard. Any required light source drivers are considered to be part of the optical              29 channel.                                                                                                                                                                             30

31

32

9.2 PHY management service	33

34

The PLME-SAP allows the transport of management commands between the MLME or the DME and the	35

PLME. Table 122 lists the primitives supported by the PLME-SAP. These primitives are discussed in the	36

subclauses referenced in Table 122.	37

38

39

Table 122—PLME-S AP primitive s	40

| PLME-SAP primitive | Request | Confirm |
| --- | --- | --- |
| PLME-CCA | 9.2.1 | 9.2.2 |
| PLME-GET | 9.2.3 | 9.2.4 |
| PLME-SET | 9.2.5 | 9.2.6 |
| PLME-SET-TRX-STATE | 9.2.7 | 9.2.8 |
| PLME-SWITCH | 9.2.9 | 9.2.10 |

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	289

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	9.2.1 PLME-CCA.request

2

3	The PLME-CCA.request primitive requests that the PLME perform a CCA as defined in 8.3.6.

4

5	The semantics of the PLME-CCA.request primitive are as follows:

6

7	P LME-CCA.re que s t	()

8

9	There are no parameters associated with the PLME-CCA.request primitive.

10

11	9.2.1.1 When generated

12

13	The PLME-CCA.request primitive is generated by the MLME and issued to its PLME whenever the access

14	algorithm requires an assessment of the channel.

15

16	9.2.1.2 Effect on receipt

17

18	If the receiver is enabled on receipt of the PLME-CCA.request primitive, the PLME will cause the PHY to

19	perform a CCA.

20

21	9.2.2 PLME-CCA.confirm

22

23	The PLME-CCA.confirm primitive reports the results of a CCA.

24

25	The semantics of the PLME-CCA.confirm primitive are as follows:

26

27	P LME-CCA.confirm	(

28	s ta tus

29	)

30

31	Table 123 specifies the parameters for the PLME-CCA.confirm primitive.

32

33

34	Table 123—P LME-CCA.c o nfirm parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | TRX_OFF, TX_ON, BUSY, IDLE | The result of the request to perform a CCA. |

35

36

37

38

39

40

41

42

43	9.2.2.1 When generated

44

45	The PLME-CCA.confirm primitive is generated by the PLME and issued to its MLME in response to a

46	PLME-CCA.request primitive. When the PHY has completed the CCA, the PLME will issue the PLME-

47	CCA.confirm primitive with a status of either BUSY or IDLE, depending on the result of the CCA.

48

49	If the PLME-CCA.request primitive is received while the transceiver is disabled (TRX_OFF state) or if the

50	transmitter is enabled (TX_ON state), the PLME will issue the PLME-CCA.confirm primitive with a status

51	of TRX_OFF or TX_ON, respectively.

52

53

54

290	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

9.2.2.2 Effect on receipt	1

2

On receipt of the PLME-CCA.confirm primitive, the MLME is notified of the results of the CCA. If the	3

CCA attempt was successful, the status parameter is set to either BUSY or IDLE. Otherwise, the status	4 parameter will indicate the error.	5

6

9.2.3 PLME-GET.request	7

8

The PLME-GET.request primitive requests information about a given PHY PIB attribute.	9

10

The semantics of the PLME-GET.request primitive are as follows:	11

12

P LME-GET.re que s t	(	13

P IBAttribute	14

)	15

16

Table 124 specifies the parameters for the PLME-GET.request primitive.	17

18

19

Table 124—P LME-GET.re que s t parame te rs	20

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBAttribute | Enumeration | As defined in Table 140 | The identifier of the PHY PIB attribute to get. |

21

22

23

24

25

26

27

9.2.3.1 Appropriate usage	28

29

The PLME-GET.request primitive is generated by the MLME and issued to its PLME to obtain information	30 from the PHY PIB.	31

32

9.2.3.2 Effect on receipt	33

34

On receipt of the PLME-GET.request primitive, the PLME will attempt to retrieve the requested PHY PIB	35 attribute from its database.	36

37

9.2.4 PLME-GET.confirm	38

39

The PLME-GET.confirm primitive reports the results of an information request from the PHY PIB.	40

41

The semantics of the PLME-GET.confirm primitive are as follows:	42

43

P LME-GET.confirm	(	44 s ta tus ,	45

P IBAttribute ,	46

P IBAttribute Va lue	47

)	48

49

Table 125 specifies the parameters for the PLME-GET.confirm primitive.	50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	291

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, UNSUPPORTED_ATTRIBUTE | The result of the request for PHY PIB attribute information. |
| PIBAttribute | Enumeration | As defined in Table 140 | The identifier of the PHY PIB attribute to get. |
| PIBAttributeValue | Various | Attribute specific | The value of the indicated PHY PIB attribute to get. |

2

3

4

5

6

7

8

9

10

11

12

13

14	9.2.4.1 When generated

15

Table 125—PLME-GET.confirm parame te rs

16	The PLME-GET.confirm primitive is generated by the PLME and issued to its MLME in response to a

17	PLME-GET.request primitive. If the identifier of the PIB attribute is not found in the database, the PLME

18	will issue the PLME-GET.confirm primitive with a status of UNSUPPORTED_ATTRIBUTE.

19

20	If the requested PHY PIB attribute is successfully retrieved, the PLME will issue the PLME-GET.confirm

21	primitive with a status of SUCCESS.

22

23	9.2.4.2 Effect on receipt

24

25	On receipt of the PLME-GET.confirm primitive, the MLME is notified of the results of its request to read a

26	PHY PIB attribute. If the request to read a PHY PIB attribute was successful, the status parameter is set to

27	SUCCESS. Otherwise, the status parameter will indicate the error.

28

29	9.2.5 PLME-SET.request

30

31	The PLME-SET.request primitive attempts to set the indicated PHY PIB attribute to the given value.

32

33	The semantics of the PLME-SET.request primitive are as follows:

34

35	P LME-S ET.reque st	(

36	P IBAttribute ,

37	P IBAttribute Va lue

38	)

39

40	Table 126 specifies the parameters for the PLME-SET.request primitive.

41

42

43	Table 126—PLME-S ET.re que s t parame te rs

44

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PIBAttribute | Enumeration | As defined in Table 140 | The identifier of the PIB attribute to set. |
| PIBAttributeValue | Various | Attribute spe- cific, as defined in Table 140 | The value of the indicated PIB attribute to set. |

45

46

47

48

49

50

51

52

53

54

292	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

9.2.5.1 When generated	1

2

The PLME-SET.request primitive is generated by the MLME and issued to its PLME to write the indicated	3

PHY PIB attribute.	4

5

9.2.5.2 Effect on receipt	6

7

On receipt of the PLME-SET.request primitive, the PLME will attempt to write the given value to the	8 indicated PHY PIB attribute in its database.	9

10

9.2.6 PLME-SET.confirm	11

12

The PLME-SET.confirm primitive reports the results of the attempt to set a PIB attribute.	13

14

The semantics of the PLME-SET.confirm primitive are as follows:	15

16

P LME-S ET.confirm	(	17 s ta tus ,	18

P IBAttribute	19

)	20

21

Table 127 specifies the parameters for the PLME-SET.confirm primitive.	22

23

24

Table 127—PLME-S ET.c o nfirm parame te rs	25

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, UNSUPPORTED_ATTRIBUTE, INVALID_PARAMETER | The status of the attempt to set the requested PIB attribute. |
| PIBAttribute | Enumeration | As defined in Table 140 | The identifier of the PIB attribute being confirmed. |

26

27

28

29

30

31

32

33

34

35

9.2.6.1 When generated	36

37

The PLME-SET.confirm primitive is generated by the PLME and issued to its MLME in response to a	38

PLME-SET.request primitive.	39

40

If the PIBAttribute parameter specifies an attribute that is not found in the database, as shown in Table 140,              41 the PLME will issue the PLME-SET.confirm primitive with a status of UNSUPPORTED_ATTRIBUTE. If              42 the PIBAttibuteValue parameter specifies a value that is out of the valid range for the given attribute, the              43

PLME will issue the PLME-SET.confirm primitive with a status of INVALID_PARAMETER.	44

45

If the requested PHY PIB attribute is successfully written, the PLME will issue the PLME-SET.confirm	46 primitive with a status of SUCCESS.	47

48

9.2.6.2 Effect on receipt	49

50

On receipt of the PLME-SET.confirm primitive, the MLME is notified of the result of its request to set the              51 value of a PHY PIB attribute. If the requested value was written to the indicated PHY PIB attribute, the              52 status parameter is set to SUCCESS. Otherwise, the status parameter will indicate the error.                                        53

54

Copyright © 20xx IEEE. All rights reserved.	293

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	9.2.7 PLME-SET-TRX-STATE.request

2

3	The PLME-SET-TRX-STATE.request primitive requests that the PHY entity change the internal operating

4	state of the transceiver. The transceiver will have three main states as follows:

5

6	a)	Transceiver disabled (TRX_OFF)

7	b)	Transmitter enabled (TX_ON)

8	c)	Receiver enabled (RX_ON)

9

10	The semantics of the PLME-SET-TRX-STATE.request primitive are as follows:

11

12	P LME-S ET-TRX-S TATE.re que s t	(

13	s ta te

14	)

15

16	Table 128 specifies the parameters for the PLME-SET-TRX-STATE.request primitive.

17

18

19	Table 128—PLME-S ET-TRX-S TATE.re que s t parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| state | Enumeration | RX_ON, TRX_OFF, FORCE_TRX_OFF, TX_ON | The new state in which to configure the transceiver. |

20

21

22

23

24

25

26

27

28	9.2.7.1 When generated

29

30	The PLME-SET-TRX-STATE.request primitive is generated by the MLME and issued to its PLME when

31	the current operational state of the receiver needs to be changed.

32

33	9.2.7.2 Effect on receipt

34

35	On receipt of the PLME-SET-TRX-STATE.request primitive, the PLME will cause the PHY to attempt to

36	change to the requested state.

37

38	9.2.8 PLME-SET-TRX-STATE.confirm

39

40	The PLME-SET-TRX-STATE.confirm primitive reports the result of a  request to change the internal

41	operating state of the transceiver.

42

43	The semantics of the PLME-SET-TRX-STATE.confirm primitive are as follows:

44

45	P LME-S ET-TRX-S TATE.confirm	(

46	s ta tus

47	)

48

49	Table 129 specifies the parameters for the PLME-SET-TRX-STATE.confirm primitive.

50

51	9.2.8.1 When generated

52

53	The PLME-SET-TRX-STATE.confirm primitive is generated by the PLME and issued to its MLME after

54	attempting to change the internal operating state of the transceiver.

294	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 129—PLME-S ET-TRX-S TATE.c o nfirm parame te rs	1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, RX_ON, TRX_OFF, TX_ON, BUSY_RX, BUSY_TX | The result of the request to change the state of the transceiver. |

2

3

4

5

6

7

8

9

9.2.8.2 Effect on receipt	10

11

On receipt of the PLME-SET-TRX-STATE.confirm primitive, the MLME is notified of the result of its	12

request to change the internal operating state of the transceiver.	13

14

If the state change is accepted, the PHY will issue the PLME-SET-TRX-STATE.confirm primitive with a	15

status of SUCCESS. If this primitive requests a state that the transceiver is already configured, the PHY will	16

issue the PLME-SET-TRX-STATE.confirm primitive with a status indicating the current state, i.e., RX_ON,	17

TRX_OFF, or TX_ON. If this primitive is issued with RX_ON or TRX_OFF argument and the PHY is busy	18

transmitting a PPDU, the PHY will issue the PLME-SET-TRXSTATE.confirm primitive with a status	19

BUSY_TX and defer the state change until the end of transmission. If this primitive is issued with TX_ON	20

or TRX_OFF argument and the PHY is in RX_ON state and has already received a valid preamble, the PHY	21

will issue the PLME-SET-TRX-STATE.confirm primitive with a status BUSY_RX and defer the state	22

change until the end of reception of the PPDU. If this primitive is issued with FORCE_TRX_OFF, the PHY	23

will cause the PHY to go the TRX_OFF state irrespective of the state the PHY is in.	24

25

9.2.9 PLME-SWITCH.request	26

27

The PLME-SWITCH.request primitive request is used by the DME to request that the PHY entity select the	28

switch to enable the appropriate cells in the SW-BIT-MAP. The semantics of the PLME-SWITCH.request	29

primitive are as follows:	30

31

P LME-S WITCH.re que s t	(	32

S W-BIT-MAP,	33

DIR	34

)	35

36

Table 130 specifies the parameters for the PLME-SET-TRX-STATE.request primitive.	37

38

39

Table 130—PLME-S WITCH.re que s t parame te rs	40

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SW-BIT-MAP | Vector of ‘n’  ‘m’entries | Boolean | One bit for each optical source or photode- tector and is dependent on the direction. Setting the kth bit to a “1” brings the corre- sponding optical source or photodetector into the cell group. ‘n’ is the number of cells and ‘m’ is the number of distinct data streams from the PHY. The value of ‘m’ is three for PHY III. |
| DIR |  | Boolean | ‘0’ is for TX and ‘1’ is for ‘RX’ |

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	295

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	9.2.9.1 When generated

2

3	The PLME-SWITCH.request primitive is generated by the DME and issued to its PLME when the current

4	cell selection needs to be changed.

5

6	9.2.9.2 Effect on receipt

7

8	On receipt of the PLME-SWITCH.request primitive, the PLME will cause the PHY to attempt to change to

9	the cell.

10

11	9.2.10 PLME-SWITCH.confirm

12

13	The PLME-SWITCH.confirm primitive reports the result of a request to change the currently operating cell.

14

15	The semantics of the PLME-SWITCH.confirm primitive are as follows:

16

17	P LME-S WITCH.confirm	(

18	s ta tus

19	)

20

21	Table 131 specifies the parameters for the PLME-SWITCH.confirm primitive.

22

23

24	Table 131—PLME-S WITCH.c o nfirm parame te rs

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS | The result of the request to change the cell. |

25

26

27

28

29

30

31	9.2.10.1 When generated

32

33	The PLME-SWITCH.confirm primitive is generated by the PLME and issued to its DME after attempting to

34	change the cell.

35

36	9.2.10.2 Effect on receipt

37

38	On receipt of the PLME-SWITCH.confirm primitive, the DME is notified of the result of its request to

39	change the currently operating cell.

40

41	If the PHY switch is able to select the new cell, the PHY will issue the PLME-SWITCH.confirm primitive

42	with a status of SUCCESS.

43

44

45	9.3 PHY data service

46

47	The PD-SAP supports the transport of MPDUs between a local MAC sublayer and a local PHY layer entity.

48	Table 132 lists the primitives supported by the PD-SAP.

49

50	9.3.1 PD-DATA.request

51

52	The PD-DATA.request primitive requests the transfer of data from the MAC sublayer to form a PSDU at the

53	local PHY entity.

54

296	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 132—PD-S AP prim itive s	1

| PD-SAP primitive | Request | Confirm | Indication |
| --- | --- | --- | --- |
| PD-DATA | 9.3.1 | 9.3.2 | 9.3.3 |

2

3

4

5

6

7

The semantics of the PD-DATA.request primitive are as follows:	8

9

P D-DATA.re quest                                   (                                                                                                                 10 ps duLe ngth,                                                                                             11 ps du,                                                                                                         12macheaderLength

13

)	14

15

Table 133 specifies the parameters for the PD-DATA.request primitive.	16

17

18

Table 133—PD-DATA.re que s t parame te rs	19

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| psduLength | Unsigned Integer | 0–aMaxPHYFrameSize | The number of octets in the PSDU to be transmitted by the PHY entity. |
| psdu | Set of octets |  | The set of octets forming the PSDU to be transmitted by the PHY entity. |
| bandplanID | Unsigned Integer | 0–6 | Color band channel of PSDU. |

20

21

22

23

24

25

26

2

28

29

MHRLength        Unsigned Integer       0- aMaxPHYFrameSize   The number of octets in the MHR

30

31

9.3.1.1 When generated	32

33

The PD-DATA.request primitive is generated by a local MAC sublayer entity and issued to its PHY entity to	34

request the transmission of an PPDU.	35

36

9.3.1.2 Effect on receipt	37

38

The receipt of the PD-DATA.request primitive by the PHY entity will cause the transmission of the supplied	39

PPDU.	40

41

9.3.2 PD-DATA.confirm	42

43

The PD-DATA.confirm primitive confirms the end of the transmission of data from a local MAC sublayer	44

entity.	45

46

The semantics of the PD-DATA.confirm primitive are as follows:	47

48

P D-DATA.confirm	(	49

s ta tus	50

)	51

52

Table 134 specifies the parameters for the PD-DATA.confirm primitive.	53

54

Copyright © 20xx IEEE. All rights reserved.	297

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, RX_ON, TRX- _OFF | The result of the request to transmit a frame. |

2

3

4

5

6

7

8

9

10	9.3.2.1 When generated

11

Table 134—PD-DATA.c o nfirm parame te rs

12	The PD-DATA.confirm primitive is generated by the PHY entity and issued to its MAC sublayer entity in

13	response to a PD-DATA.request primitive. Provided the transmitter is enabled (TX_ON state), the PHY will

14	first construct a PPDU containing the supplied PSDU, and then transmit the PPDU. When the PHY entity

15	has completed the transmission, it will issue the PD-DATA.confirm primitive with a status of SUCCESS.

16

17	If the PD-DATA.request primitive is received while the receiver is enabled (RX_ON state) or if the

18	transceiver is disabled (TRX_OFF state), the PHY entity will issue the PD-DATA.confirm primitive with a

19	status of RX_ON or TRX_OFF, respectively.

20

21	9.3.2.2 Effect on receipt

22

23	On receipt of the PD-DATA.confirm primitive, the MAC sublayer entity is notified of the result of its

24	request to transmit. If the transmission attempt was successful, the status parameter is set to SUCCESS.

25	Otherwise, the status parameter will indicate the error.

26

27	9.3.3 PD-DATA.indication

28

29	The PD-DATA.indication primitive indicates the transfer of data from the PHY to the local MAC sublayer

30	entity.

31

32	The semantics of the PD-DATA.indication primitive are as follows:

33

34	P D-DATA.indica tion	(

35	ps duLe ngth,

36	ps du,

37	ppduLinkQua lity

38	)

39

40	Table 135 specifies the parameters for the PD-DATA.indication primitive.

41

42	9.3.3.1 When generated

43

44	The PD-DATA.indication primitive is generated by the PHY entity and issued to its MAC sublayer entity to

45	transfer a received PSDU. This primitive will not be generated if the received psduLength field is zero or

46	greater than aMaxPHYFrameSize.

47

48	9.3.3.2 Effect on receipt

49

50	On receipt of the PD-DATA.indication primitive, the MAC sublayer is notified of the arrival of data across

51	the PHY data service.

52

53

54

298	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 135—PD-DATA.indic atio n parame te rs	1

| Name | Type | Valid | Description |
| --- | --- | --- | --- |
| psduLength | Unsigned Integer | 0–aMaxPHYFrameSize | The number of octets contained in the PSDU received by the PHY entity. |
| psdu | Set of octets | — | The set of octets forming the PSDU received by the PHY entity. |
| ppduLinkQuality | Unsigned Integer | 0x00–0xff | Wavelength quality indication (WQI) value measured during reception of the PPDU as defined in 5.4.23.2. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

9.3.3.3 PHY Data Service	16

17

editor note: contents here should be merged into Clause 9.3. Consider merging first. Please PureLIFI pro-	18

vide instructions on how to merge. See comment 223 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-	19

02-0013-comment-against-7m-d1.xlsx	20

21

9.3.3.3.1 P HY Data S e rvic e Primitive s	22

23

Table 136 presents the PHY data service primitives.	24

25

26

27

28

Table 136—PHY data s e rvic e primitive s .	29

| Primitive | Request | Indicate | Confirm |
| --- | --- | --- | --- |
| PHY-DATA | X | X | X |
| PHY-MAC-HEADER | X | X | X |

30

31

32

33

34

35

36

37

Table 137 presents the PHY data service primitive parameters.	38

39

40

41

42

Table 137—PHY data s e rvic e primitive parame te rs .	43

| Parameter | Associated Primitive | Value |
| --- | --- | --- |
| DATA | PHY-DATA.request PHY-DATA.indicate | One Octet taking values in the interval X'00' – X'FF' |
| MAC_HEADER | PHY-MAC-HEADER.request PHY-MAC-HEADER.indicate | Six octets containing the information for the high-reliability MAC header described in subclause 6.2.5.1. |

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	299

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	9.3.3.3.2 De taile d PHY Data S e rvic e S pe c ific atio n

2

3

28

29

51

52

300	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

21

48

49

52

53

54

Copyright © 20xx IEEE. All rights reserved.	301

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

20

21

45

46	9.4 PHY enumeration description

47

48

49	Table 138 shows a description of the PHY enumeration values defined in the PHY specification.

50

51	9.5 PHY constants and PIB attributes

52

53	This subclause specifies the constants and attributes required by the PHY.

54

302	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

9.5.1 PHY constants

Table 138—PHY e nume ratio n de s c riptio n	1

| Enumeration | Value | Description |
| --- | --- | --- |
| BUSY | 0x00 | The CCA attempt has detected a busy channel. |
| BUSY_RX | 0x01 | The transceiver is asked to change its state while receiving. |
| BUSY_TX | 0x02 | The transceiver is asked to change its state while transmitting. |
| FORCE_TRX_OFF | 0x03 | The transceiver is to be switched off. |
| IDLE | 0x04 | The CCA attempt has detected an idle channel. |
| INVALID_PARAMETER | 0x05 | A SET/GET request was issued with a parameter in the primitive that is out of the valid range. |
| RX_ON | 0x06 | The transceiver is in, or is to be configured into, the receiver enabled state. |
| SUCCESS | 0x07 | The request completed successfully. |
| TRX_OFF | 0x08 | The transceiver is in, or is to be configured into, the transceiver disabled state. |
| TX_ON | 0x09 | The transceiver is in, or is to be configured into, the transmitter enabled state. |
| UNSUPPORTED_ATTRIBUTE | 0x0a | A SET/GET request was issued with the identifier of an attribute that is not supported. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

The constants that define the characteristics of the PHY are presented in Table 139. These constants are	30 hardware dependent and shall not be changed during operation.	31

32

45

46

9.5.2 PHY PIB attributes	47

48

The PHY PIB comprises the attributes required to manage the PHY of a device. Each of these attributes can              49 be read or written using the PLME-GET.request and PLME-SET.request primitives, respectively. The              50 attributes contained in the PHY PIB are presented in Table 140.                                                                                     51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	303

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 139—PHY c o ns tants

| Constant | Description | Value |
| --- | --- | --- |
| aMaxPHYFrameSize | The maximum PSDU size (in octets) the PHY shall be able to receive. | 1023 for PHY I, 65535 for PHY II, III |
| aTurnaroundTime-TX-RX | TX-to-RX maximum turnaround time (as defined in 8.3.3) | zero optical clock cycles |
| aTurnaroundTime-RX-TX | RX-to-TX maximum turnaround time (as defined in 8.3.4) | PHY I: ≤ 240 optical clock cycles, PHY II, III: ≤ 5120 opti- cal clock cycles |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

38

39

40	Table 140—PHY PIB attribute s

| Attribute | Identifier | Type | Range | Description |
| --- | --- | --- | --- | --- |
| phyCurrentChannel | 0x00 | Integer | 0–6 | The wavelength used for all following transmissions and receptions (as defined in 8.3.1). |
| phyCCAMode | 0x01 | Octet | enumerated | b0=CCA mode 1 b1=CCA mode 2 b2=CCA mode 3 b3–b7=reserved The CCA modes are defined in 8.3.6. |

41

42

43

44

45

46

47

48

49

50

51

52

53

54

304	Copyright © 20xx IEEE. All rights reserved.

| Attribute | Identifier | Type | Range | Description |
| --- | --- | --- | --- | --- |
| phyFrequencyLabeling* | 0x02 |  | Frequency ration = 1.5 | Indicates the PSDU carries the frequencies that will be used for the data frame later on. * This PIB attribute is explic- itly defined for future extensi- bility. For now, only aFrequencyLabelingRatio can control the frequency ratio of the Frequency Labeling field. |
| phyUseSplitterSymbol | 0x03 |  | Frequency ratio = 0.83 | Indicates the the PSDU car- ries the flag to toggle whether the device is going to use SSs or not. |
| phySplitterFrequency | 0x04 |  | Frequency ratio = 0.71 | Indicates the PSDU carries the splitter frequency. If the SS is already in used, it will use the original phySplitter- Frequency until next cycle. |
| phySplitterDuration | 0x05 |  | Frequency ratio = 0.62 | Indicates the PSDU carries the duration of the SS. This is represented as a ratio of sym- bol duration to splitter dura- tion in integer. |
| phySymbolDuration | 0x06 |  | Frequency ratio = 0.55 | Indicates the PSDU carries the duration of a data symbol in the PSDU. This is repre- sented as a ratio of the symbol duration to 1/30 second in the base 2 log scale. For example, if the symbol duration is 1/ 120 second, then the PSDU would contain an integer -2. If the symbol duration is 1/15 second, then the PSDU would contain an integer 2. Note that this does not affect the dura- tion of the preamble field and the optional field. |
| phyUseExtendedMode | 0x07 | Integer | 0–1 | This attribute is set to a one to indicate that an extended pre- amble or visibility pattern is to be used. Otherwise, it is set to zero. |

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

| Table 140—PHY PIB attribute s | (continued) | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |
|  |  | 16 |
|  |  | 17 |
|  |  | 18 |
|  |  | 19 |
|  |  | 20 |
|  |  | 21 |
|  |  | 22 |
|  |  | 23 |
|  |  | 24 |
|  |  | 25 |
|  |  | 26 |
|  |  | 27 |
|  |  | 28 |
|  |  | 29 |
|  |  | 30 |
|  |  | 31 |
|  |  | 32 |
|  |  | 33 |
|  |  | 34 |
|  |  | 35 |
|  |  | 36 |
|  |  | 37 |
|  |  | 38 |
|  |  | 39 |
|  |  | 40 |
|  |  | 41 |
|  |  | 42 |
|  |  | 43 |
|  |  | 44 |
|  |  | 45 |
|  |  | 46 |
|  |  | 47 |
|  |  | 48 |
|  |  | 49 |
|  |  | 50 |
|  |  | 51 |
|  |  | 52 |
|  |  | 53 |
|  |  | 54 |

Copyright © 20xx IEEE. All rights reserved.	305

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 140—PHY PIB attribute s  (continued)

| Attribute | Identifier | Type | Range | Description |
| --- | --- | --- | --- | --- |
|  | 0x09 |  |  |  |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23	10. Pulsed Modulation PHY specifications

10.1 PPDU format

24	       TBD

10.2 Transmission

10.2.1 Preamble field

TBD

10.2.2 PHY Header

TBD

10.2.3 PSDU

TBD

25	PHY II is targeted towards applications requiring high data rates, as shown in Table 106. For PHY II, the

26	PHY header shall be sent at one of the following data rates: 1.25 Mb/s, 2.5 Mb/s, 6 Mb/s, 12 Mb/s, 24 Mb/s,

27	or 48 Mb/s, depending on the selected optical clock rate. Support for 1.25 Mb/s at an optical clock of

28	3.75 MHz is mandatory.

29

30

31	10.1 Reference modulator diagram

32

33	A reference implementation is in Figure 154. The PHR and PSDU parts of the frame are subject to the FEC

34	for error protection. The PHR is encoded using parameters corresponding to the lowest data rate for the

35	currently negotiated clock rate.

36

37

38

39

40

41

42

43

44	Figure 154—Reference modulator diagram for PHY II

45

46

47	10.2 Forward error correction encoder

48

49	A systematic Reed-Solomon code operating on GF(256) shall be used for PHY II to correct errors and

50	increase the system reliability. The Reed-Solomon code may be shortened for the last block if it does not

51	meet the block size requirements, as specified for PHY I in 10.2. No zero padding is required for the RS

52	code.

53

54

306	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The Reed-Solomon code  is  defined over GF(256) with  a  primitive polynomial x8+x4+x3+x2+1. The	1 generator for the RS(160, 128) code and the RS(64, 32) code is given by	2

3 g(x) =	4 x32+α11x31+α8x30+α109x29+α194x28+α254x27+α173x26+α11x25+α75x24+α218x23+α148x23+α14 		5

9x21+α44x20+α0x19+α137x18+α104x17+α43x16+α137x15+α203x14+α99x13+α176x12+a59x11+α91 	6

10 	194  9

84  8

53  7

248  6

107  5

80  4

28  3

215  2

251 	18 	7

x   +α

x +α

x +α

x +α

x +α

x +α

x +α

x +α

x +α

x+α

(10)	8

where α is a primitive element in GF(256).	9

10

For the VPPM modes using 4B6B encoding, the RS code word (d1,..., d8) from the GF(256) RS code is              11 broken into 2 nibbles (d1,.., d4) and (d5,.., d8). These nibbles are sent LSB first to the 4B6B encoder as              12 shown in Figure 155.                                                                                                                                                         13

14

15

16

17

18

19

20

21

22

23

24

Figure 155—GF(256) RS encoder usage with 4B6B encoder	25

26

27

28

29

10.3 Run-length limiting encoder	30

31

All PHY II VPPM modes shall use 4B6B encoding as defined in 10.5.1. All OOK PHY II modes shall use	32

8B10B encoding as specified in ANSI/INCITS 373.	33

34

10.4 Data mapping for VPPM	35

36

All PHY II VPPM modes shall use data mapping as defined in 10.6.	37

38

39

11. Low-bandwidth OFDM PHY specifications	40

41

42

11.1 PPDU format

TBD

11.2 Transmission

11.2.1 Preamble field

TBD

11.2.2 PHY Header

TBD

11.2.3 PSDU

TBD

11.1 Adaptive OFDM Concept	43

44

OFDM in the current specification enables a highly adaptive modular implementation, which supports both	45 a high-efficiency PHY mode designed to enable optimal utilization of the low-bandwidth resources (up to 40	46

MHz of single-sided bandwidth) as well as a low-complexity PHY mode designed to enable high energy              47 efficiency in mobile applications. Furthermore, the specification enables the adaptive and modular introduc-              48 tion of energy-efficient waveforms such as enhanced unipolar OFDM (eU-OFDM), performance enhancing              49 waveforms such as single-carrier FDMA (SC-FDMA) and waveforms designed for improved illumination              50 control such as reverse polarity optical OFDM (RPO-OFDM). In addition, the specification supports the              51 application of adaptive bit and energy loading techniques as well as multiple-input multiple-output (MIMO)              52 techniques which can leverage the additional communication capacity of multiple light sources as well as              53 the additional communication capacity introduced by the utilization of different optical wavelengths and              54

Copyright © 20xx IEEE. All rights reserved.	307

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	light polarization for communication. The PHY specification supports relaying mechanisms for the cases

2	when dedicated relay terminals are available. A DC-biased optical OFDM (DCO-OFDM) waveform is the

3	basis and the foundation of all concepts, which are specified as optional features and can be applied in a non-

4	conflicting manner.

5

6	11.2 Waveform

7

8

9	11.2.1 Adaptive OFDM signal generation

10

11	The adaptive OFDM concept adopted in the current PHY specification allows for adaptive adjustment of the

12	waveform in terms of the carrier mapping as well as in terms of the application of sophisticated signal pro-

13	cessing and dimming techniques such as single-carrier pre-coding, unipolar modulation for improved trans-

14	mission rates and efficiency and reverse polarity OFDM for dimming applications. The specification

15	supports the application of adaptive bit and energy loading techniques as well as adaptive multiple-input

16	multiple-output (MIMO) techniques which can leverage the additional communication capacity of multiple

17	light sources as well as the additional communication capacity introduced by the utilization of different opti-

18	cal wavelengths and light polarization for communication. The individual features of the adaptive signal

19	generation approach are described as follows.

20

21	11.2.2 Carrier mapping

22

23	The Low-bandwidth PHY mode is required to support fixed carrier mapping. All subcarriers are designated

24	for information transfer by a single communication node in an allocated time interval. No special carrier

25	designation for multiple access techniques exists, and no special carriers are designated for control informa-

26	tion. Depending on whether the current PHY implementation supports adaptive bit allocation for the carrier

27	mapping procedure (an optional feature), the mapping can be described as follows.

28

29	11.2.2.1 PHY does not support adaptive bit and power allocation

30

31	The QAM modulation symbols to which the binary information has been mapped are divided into groups of

32	24 symbols, where the original order at the output of the QAM modulation encoder is preserved (the bit-

33	stream mapping to QAM symbols is described in subclause 6.3.5). Each ordered group of 24 symbols is

34	mapped to the ordered set of subcarrier indices [3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 22,

35	23, 24, 25, 26, 27, 28], where indices 7 and 21 are assigned to fixed pilot values of 1 and -1, respectively.

36	Zeros are assigned to subcarrier indices [0,1,2,29,30,31,32,33,34,62,63]. The Hermitian symmetric sequence

37	of the 26 symbols assigned to indices [3-28] is assigned to subcarrier indices [35-61].

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

308	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

11.2.2.2 PHY supports adaptive bit and power allocation	1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

Figure 156—IDFT realization by means of an IFFT algorithm.	18

19

20

If the PHY mode supports adaptive bit loading, the carrier mapping is determined by means of a bit alloca-              21 tion table negotiated and specified at the MAC layer protocol through an exchange of control and manage-              22 ment frames.                                                                                                                                                                      23

24

11.2.3 IDFT	25

26

The IDFT size is fixed in the Low-bandwidth PHY mode to enable lower implementation complexity. The              27 modulation of the different frequency-domain subcarriers is achieved through an IDFT operation, described              28 as follows:                                                                                                                                                                         29

30

63



k = 0

S k  e

j2 k

-----------

64

31

32 (11)	33

34

35

where S[k] is the symbol mapped to subcarrier index k, as described in 6.3.2. Conventionally, the inverse              36 discrete Fourier transform, as shown in Equation (6.3.3.1), is implemented with an IFFT algorithm. The              37

DFT/IDFT size in the current PHY mode is fixed to 64. The DC subcarrier (subcarrier index 0 in the IFFT              38 operation) and the 180-degree subcarrier (subcarrier index 32 in the IFFT operation) are set to 0. The infor-              39 mation and pilot symbols with coefficients 1 to 26 are mapped to IFFT inputs 3 to 28, while the Hermitian              40 symmetry symbols are mapped onto IFFT inputs 35 to 61. The high-frequency subcarriers, at inputs 29 to              41

34, form a guard interval and are set to zero. The low-frequency subcarriers, at inputs 0-2 and 62-63, are set              42 to zero in order to avoid possible low-frequency distortion in the system due to baseline wandering and              43 background light interference. The mapping is illustrated in Figure 156. After performing an IFFT, the out-              44 put is cyclically extended to the desired length as described in 17.2.4.                                                                            45

46

11.2.4 Cyclic prefix                                                                                                                                        47

48

The cyclic prefix size is fixed in the Low-bandwidth PHY mode to enable lower implementation complexity.              49

The cyclic prefix/guard interval (GI) is a cyclic extension of the IDFT result in each OFDM frame, where              50 the cyclic prefix is formed by the last 16 samples of the IDFT result, appended in front of the 64-sample              51

IDFT result. In the current PHY mode, the cyclic prefix size is not variable and cannot be changed.                            52

53

54

Copyright © 20xx IEEE. All rights reserved.	309

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	11.2.5 Subcarrier modulation mapping

2

3	The OFDM subcarriers shall be modulated by using BPSK, QPSK, 16-QAM, or 64-QAM, depending on the

4	RATE requested. The encoded and interleaved binary serial input data shall be divided into groups of NBPSC

5	(1, 2, 4, or 6) bits and converted into complex numbers representing BPSK, QPSK, 16-QAM, or 64-QAM

6	constellation points. The conversion shall be performed according to Gray-coded constellation mappings,

7	illustrated in Figure 10, with the input bit, b0, being the earliest in the stream. The output values, d, are

8	formed by multiplying the resulting   (I+jQ)   value by a normalization factor KMOD, as described in

9	Equation (12).

10

11	d

12

=  I + jQ   KMOD

(12)

13	The normalization factor, KMOD, depends on the base modulation mode, as prescribed in Table 141. Note

14	that the modulation type can be different from the start to the end of the transmission, as the signal changes

15	from SIGNAL to DATA, as shown in Figure 512. The purpose of the normalization factor is to achieve the

16	same average power for all mappings. In practical implementations, an approximate value of the normaliza-

17	tion factor can be used, as long as the device conforms with the modulation accuracy requirements.

18

19

20

21

22	Table 141—Mo dulatio n-de pe nde nt no rmalizatio n fac to r KMOD

23

| Modulation | KMOD |
| --- | --- |
| BPSK | 1 |
| QPSK | --1- ---- 2 |
| 16-QAM | 1 --------- 10 |
| 64-QAM | 1 ----- ----42 |

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39	For BPSK, b0 determines the I value, as illustrated in Table 142. For QPSK, b0 determines the I value and b1

40	determines the Q value, as illustrated in Table 143. For 16-QAM, b0b1 determines the I value and b2b3 deter-

41	mines the Q value, as illustrated in Table 144. For 64-QAM, b0b1b2 determines the I value and b3b4b5 deter-

42	mines the Q value, as illustrated in Table 145.

43

44

45

46

47	Table 142—BPS K e nc o ding table

48

| Input bit (b0) | I-out | Q-out |
| --- | --- | --- |
| 0 | -1 | 0 |
| 1 | 1 | 0 |

49

50

51

52

53

54

310	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

Table 143—QPS K e nc o ding table	4

| Input bit (b0) | I-out |  | Input bit (b1) | Q-out |
| --- | --- | --- | --- | --- |
| 0 | -1 |  | 0 | -1 |
| 1 | 1 |  | 1 | 1 |

5

6

7

8

9

10

11

12

13

14

Table 144—16-QAM e nc o ding table	15

16

| Input bit (b0b1) | I-out |  | Input bit (b0b1) | Q-out |
| --- | --- | --- | --- | --- |
| 00 | -3 |  | 00 | -3 |
| 01 | -1 |  | 01 | -1 |
| 11 | 1 |  | 11 | 1 |
| 10 | 3 |  | 10 | 3 |

17

18

19

20

21

22

23

24

25

26

27

28

29

Table 145—64-QAM e nc o ding table	30

31

| Input bit (b0b1b2) | I-out |  | Input bit (b3b4b5) | Q-out |
| --- | --- | --- | --- | --- |
| 000 | -7 |  | 000 | -7 |
| 001 | -5 |  | 001 | -5 |
| 011 | -3 |  | 011 | -3 |
| 010 | -1 |  | 010 | -1 |
| 110 | 1 |  | 110 | 1 |
| 111 | 3 |  | 111 | 3 |
| 101 | 5 |  | 101 | 5 |
| 100 | 7 |  | 100 | 7 |

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

11.2.6 Single-carrier modulation	48

49

Optional pre-coding before the OFDM modulator can be used to reduce the probability of clipping and              50 enhance power efficiency. As explained in subclause 6.3.2, the current specification supports loading of 24              51 subcarriers with unique information. Hence, N<24 M-QAM symbols are passed through an N-point DFT              52 before mapped to subcarriers [3, 4, 5, 6, 8, …, 20, 22, …, 28] as illustrated in Fig. 6.3.3.1. The DFT is com-              53 puted as:                                                                                                                                                                            54

Copyright © 20xx IEEE. All rights reserved.	311

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2	N – 1

3	

4	k = 0

5

S k e

j 2k

–

-----------

N

(13)

6	and the output result is mapped onto the different subcarriers in the order [12, 13, 14, …, 23, 0, 1, 2, …, 11]

7	→ [3,..,6,8,…,20,22,…,28]. If N<24, then the result of the N-point DFT is mapped to the middle of the 24

8	data subcarriers using the order ['0', '0', …, '0', floor(N/2)+1, floor(N/2)+2, …, N-1 0, 1, 2, …, floor(N/2), '0',

9	…, '0', '0'] →[3,..,6,8,…,20,22,…,28], where '0' corresponds to a zero value placed on that respective sub-

10	carrier since there are less values than subcarriers. When N<24, the spectral efficiency is reduced. If N<24,

11	the output of the N-point DFT is replicated floor(24/N) times before mapped to the 24 data-carrying subcar-

12	riers. Any subcarriers not loaded with information are set to '0'.  For example, if N=9, then the output of the

13	10-point DFT is organized in the order ['0', '0', '0', 0, 1, 2, 3, 4, 5, 6, 7, 8, 0, 1, 2, 3, 4, 5, 6, 7, 8, '0', '0', '0']. If

14	N=6, then the output of the 6-point DFT is organized in the order [0, 1, 2, 3, 4, 5, 0, 1, 2, 3, 4, 5, 0, 1, 2, 3, 4,

15	5, 0, 1, 2, 3, 4, 5]. If N=5, then the output of the 5-point DFT is organized in the order ['0', '0', 0, 1, 2, 3, 4, 0,

16	1, 2, 3, 4,  0, 1, 2, 3, 4, 0, 1, 2, 3, 4, '0', '0']. In all implementations, the DC value (value with index 0) of the

17	DFT-precoding output should be mapped to the 13th data subcarrier (subcarrier 16 of the IDFT presented in

18	Figure 156). Figure 157 illustrates the concept, where for the presented examples in this subclause, A=[0, 1,

19	2, …, 11] B=[12, 13, 14, …, 23], A=[0, 1, 2, 3, 4] B=[5, 6, 7, 8], A=[0, 1, 2] B=[3, 4, 5], and A=[0, 1, 2]

20	B=[3, 4], respectively. If more than one copy of the N-DFT output is mapped onto the OFDM subcarriers, a

21	pulse shaping filter such as a root-raised cosine filter can be used to shape the signal spectrum centred

22	around subcarrier 16. The filter should not be applied to the pilot subcarriers (at positions 7 and 21 of the 64-

23	point IDFT operation described in 6.3.2.1 and 6.3.3) as this might lead to significant attenuation of the pilot

24	values, which are necessary for phase correction at the receiver. The use of the RRC filter is indicated with

25	the value SC bits specified in subclause 6.2.3.2.5.

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49	Figure 157—SC-OFDMA subcarrier mapping

50

51

52

53

54

312	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

11.2.7 Unipolar modulation	1

2

The real time-domain OFDM signal, generated at the PHY layer, is used to modulate the light emitting              3 device (a light emitting diode (LED) or a laser diode (LD)), which serves as the transmitter front-end. The              4 modulation is conducted only within the active operational range of the device. In this range, the electrical              5 signal and the light output signal can only be positive at all times. The conventional approach for modulating              6 the LED active range with an OFDM signal shall be to set a positive operating point, around which the bipo-              7 lar OFDM signal can be realized. Figure 158(a) illustrates this principle. The positive bias can be introduced              8 as part of the analog front-end (in the case of AC-coupled LED drivers) or as part of the information signal              9 (in case of DC-coupled drivers). This approach is known as DC-biased optical OFDM (DCO-OFDM).                       10

11

12

13

14

15

16

17

18

19

20

21

22

23

24 (a) DC-biased optical OFDM (DCO-OFDM).	25

26

27

28

29

30

31

32

33

34

35

36

37

38 (b) Enhanced Unipolar OFDM (eU-OFDM).	39

40

Figure 158—LED active range modulation	41

42

An optional alternative modulation approach termed enhanced unipolar OFDM (eU-OFDM) is introduced in              43 this subclause. It constitutes a digital processing algorithm, which can turn the bipolar OFDM signal into a              44 strictly unipolar information signal without the addition of an energy intensive DC component that carries              45 no additional information.                                                                                                                                                 46

47

The transmitter signals to the receiver the new transmission PHY mode using the eU and STR bits in the              48 advanced modulation PHY header. For compliance purposes, the PLCP preamble and the PHY headers are              49 encoded in a DCO-OFDM fashion as described in 3.3 - 3.5. Following the four BPSK OFDM symbols con-              50 taining the PHY header, as well as the NMIMO reference symbols when applicable, the data field is encoded              51

in an eU-OFDM fashion (see Fig. 6.3.7.1(b)). The eU-OFDM algorithm works as follows.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	313

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	For a single eU-OFDM stream, specified by STR = '00', two consecutive copies of every OFDM symbol are

2	generated. The polarity of the samples in the second copy is inverted, and finally, all negative samples in the

3	resulting time-domain signal are set to zero. The resulting positive signal can be used to modulate the trans-

4	mitter. The concept is illustrated in Figure 159.

5

6	For two eU-OFDM streams, indicated by STR = '10', every three OFDM symbols are grouped into an eU-

7	OFDM block, where the first two symbols are assigned to data stream 1 (St1) and the remaining one symbol

8	is assigned to data stream 2 (St2). The two symbols in St1 are modulated using the algorithm described for

9	STR='00' and presented in Figure 159. The symbol in St2 is modulated in a similar manner, but instead of

10	two copies, four consecutive copies are created for the OFDM symbol in St2, where the first two copies are

11	kept unchanged, while the polarity of the samples in the next two copies is inverted. Following this proce-

12	dure, all negative samples in both St1 and St2 are removed, and the two signals are summed. Any time-

13	domain oversampling and pulse shaping should be done after the removal of the negative samples. If done

14	before the negative samples are removed, the oversampling and pulse shaping should also be performed at

15	the receiver side during the signal re-modulation process required for the data recovery as explained in the

16	RX algorithm. The resulting positive signal can be used to modulate the transmitter.  The concept is pre-

17	sented in Figure 160.

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44	Figure 159—Unipolar OFDM generation (one stream)

45

46

47	For three eU-OFDM streams, indicated by STR = '01'', every seven OFDM symbols are grouped into an eU-

48	OFDM block, where the first four symbols are assigned to data stream 1 (St1), the next 2 symbols are

49	assigned to data stream 2 (St2) and the last symbol is assigned to data stream 3 (St3). The four symbols in

50	St1 and the two streams in St2 are modulated using the algorithm described for STR='10' and presented in

51	Figure 160. The symbol in St3 is modulated in a similar manner, however, eight consecutive copies of that

52	symbol are generated, where the first four copies are left unchanged, while the polarity of the samples in the

53	following four copies is reversed. Following this procedure, all negative samples in St1, St2 and St3 are

54	removed. Any time-domain oversampling and pulse shaping should be done after the removal of the nega-

314	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

tive samples. If done before the negative samples are removed, the oversampling and pulse shaping should              1 also be performed at the receiver side during the signal re-modulation process required for the data recovery              2 as explained in the RX algorithm. The signals in the three streams are summed and the resulting positive sig-              3 nal can be used to modulate the transmitter. The concept is presented in Figure 161.                                                     4

5

6

7

8

9                                                                                              10

11                                                                                                          12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32                                                                                          33

34                                35

36

37

38

39

40

41

42

43

44

45

46

47

Figure 160—Unipolar OFDM generation (two streams)	48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	315

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45	Figure 161—Unipolar OFDM generation (three streams)

46

47

48	For four eU-OFDM streams, indicated by STR = '11', every fifteen OFDM symbols are grouped into an eU-

49	OFDM block, where the first eight symbols are assigned to data stream 1 (St1), the next four symbols are

50	assigned to data stream 2 (St2), the following two symbols are assigned to data stream 3 (St3) and the last

51	symbol is assigned to data stream 4 (St4). In the first stream, two consecutive copies of every OFDM symbol

52	are transmitted, where the second copy is multiplied by -1 (the signs of all samples are inverted in the time-

53	domain) as described in the cases for STR = '00', STR = '10' and STR = '01'. In the second stream, four con-

54	secutive copies of every OFDM symbol are transmitted, where the first two copies of the symbol are trans-

316	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

mitted in their original format, while the signs of the time-domain samples  of the third and the fourth copy              1 are inverted, i.e., the samples are multiplied by -1 as described in the cases for STR='10' and STR = '01'. In              2 the third stream, eight consecutive copies of every OFDM symbol are transmitted, where the first four cop-              3 ies are conveyed in their original format, while the signs of the time-domain samples of the fifth, sixth, sev-              4 enth and eighth copy are inverted, i.e., the samples are multiplied by -1 as described in the case for STR =              5

'01'. In the fourth stream, sixteen consecutive copies of every OFDM symbol are transmitted, where the first	6 eight copies are conveyed in their original format, while the signs of the time-domain samples of the ninth,	7 tenth, eleventh and twelfth, thirteenth, fourteenth, fifteenth and sixteenth copy are inverted, i.e., the samples	8 are multiplied by -1 At this point, all negative samples in the four streams are removed and the signals from	9 the three streams are added together. Any oversampling and pulse shaping should be done after the removal	10 of the negative samples. If done before the negative samples are removed, the oversampling and pulse shap-	11 ing should also be performed at the receiver side during the signal re-modulation process required for the	12 data recovery as explained in the RX algorithm.  The resulting positive signal can be used to modulate the	13 transmitter.	14

15

An algorithm for demodulating the received eU-OFDM data is recommended as follows. In order to demod-              16 ulate the data encoded in the first stream (St1), the samples in every even OFDM symbol interval are sub-              17 tracted from the samples in every odd OFDM symbol interval. For example, the samples in the second              18 interval are subtracted from the samples in the first interval, the samples in the fourth interval are subtracted              19 from the samples in the third interval, etc. Note that due to the structure of the other data streams (St2 up to              20

St4), the subtraction operation completely removes interference from these streams. After the subtraction              21 operation, the resulting data at St1 can be demodulated using the conventional OFDM demodulator algo-              22 rithm. After the data at St1 is demodulated, the stream is re-modulated again and subtracted from the overall              23 received signal. If time-domain oversampling and pulse shaping has been conducted in the time-domain              24 before the negative samples have been removed in the signal generation procedure, then the same procedure              25 (including the subsequent matched filtering at the receiver) should be applied to the re-modulated signal              26 before it is subtracted from the overall received stream. Following the subtraction of the re-modulated St1              27 signal, all symbol copies at St2 which were originally identical (the symbols in question are the symbols that              28 have identical polarity) should be summed. For example, in Fig. 6.3.7.3, the first and the second symbols              29 should be summed, the third and the fourth symbols should be summed, the fifth and the sixth symbols              30 should be summed, etc. The resulting signal at St2, then, has the same format as the signal at St1. Hence, it              31 can be demodulated after each even symbol is subtracted from each odd symbol. Upon demodulation, the              32 resulting bits can be re-modulated and the resulting waveform can be subtracted from the overall received              33 waveform as in the case for St1. The procedure is re-iterated for the remaining streams up to St4 (depending              34 on the configuration) until all data bits are recovered.                                                                                                     35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	317

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27	11.3 MIMO

28

Figure 162—MIMO reference symbols format I

29	11.3.1 Modified frame structure

30

31	In MIMO mode, the PHY layer frame structure is modified with the addition of the MIMO reference sym-

32	bols (see Figure 162) described in the following subclause 6.4.2.

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

318	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

11.3.2 Reference symbols for MIMO channel estimation	1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

Figure 163—MIMO reference symbols format II	26

27

28

11.3.2.1 MIMO Reference Symbols Format I	29

30

The MIMO reference symbols consist of NMIMO OFDM frame intervals. For each MIMO transmitter, only              31 one OFDM frame interval is set to the desired channel estimation sequence (CES). All other intervals are set              32 to zero. At the same time, the CES transmission intervals never coincide for any two transmitters. Hence, the              33

MIMO reference symbols for the different transmitters are orthogonal to each other. The format is presented	34 in Fig. 6.4.2.1.1.	35

36

11.3.2.2 MIMO Reference Symbols Format II	37

38

The MIMO reference symbols consist of NMIMO OFDM frame intervals. Every frame interval is set to the              39 desired channel estimation sequence (CES). In addition, the CESs for each transmitter are modified by              40 adjusting  the  polarity  of  the  individual CES  sequences according  to  a  pre-determined set  of Walsh              41 sequences, where a value of '1' in the Walsh sequence corresponds to an unmodified CES sequence while a              42 value of '-1' corresponds to a CES sequence with reverse polarity. The format is presented in Fig. 6.4.2.2.1.              43

The CES sequences in white are left unmodified, while the CES sequences in gray are multiplied by -1.	44

45

The Walsh sequences for a MIMO configuration with two transmitters (NMIMO = 2) correspond to the rows	46 of the matrix W2MIMO:	47

48

49

1  1

1 –1

(14)	50

51

52

The Walsh sequences for four transmitters (NMIMO = 4) correspond to the rows of the matrix W4MIMO:	53

54

Copyright © 20xx IEEE. All rights reserved.	319

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2	1  1   1   1

3	1 –1

1  –1

(15)

4	1  1

–1 –1

5	1 –1 –1  1

6

7	The Walsh sequences for four transmitters (NMIMO = 8) correspond to the rows of the matrix W8MIMO:

8

9

10	1  1   1   1   1   1   1   1

11	1 –1

1  –1

1  –1

1  –1

12	1  1

–1 –1

1   1  –1 –1

13	1 –1 –1

14

1   1  –1 –1  1

(16)

15	1  1   1   1

–1 –1 –1 –1

16	1 –1

1  –1 –1  1

–1  1

17	1  1

–1 –1 –1 –1  1   1

18	1 –1 –1  1

19

–1  1   1  –1

20	As a general rule,

21

22

23	2k – 1

24	2k	WMIMO

25	WMIMO =

26	2k – 1

27	WMIMO

28

29

30

2k – 1

WMIMO

2k – 1

–WMIMO

(17)

x

31

32	4

33

34

35

36

37

x 	x 	x

3 	2 	1

38	x 	x 	x 	x

39	4 	3 	2 	1

40	Figure 164—Spatial repetition coding

41

42

43

44

45

46

47

48

49

50

51

52

53

54

320	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

x 	x 	x 	x 	3

7 	5 	3 	1 	4

5

6

7

8

9

x 	x 	x 	x 	10

8 	6 	4 	2 	11

Figure 165—Spatial multiplexing	12

13

14

11.3.3 MIMO transmission modes	15

16

The system supports up to MMIMO=8 transmitters and NMIMO=8 receivers. MIMO operation mode refers to              17 the combination of selected transmission type and configuration. MIMO configuration refers to the active              18 number of transmitters and receivers. Based on the communication channel conditions, a MIMO system              19 with MMIMO transmitters and NMIMO receivers (denoted as MMIMO × NMIMO MIMO) may operate using              20 only Mactive < MMIMO transmitters and Mactive < NMIMO receivers. For example, an 8×8 MIMO system may              21 operate as a 4×4 MIMO system keeping only a subset of their transmitters and receivers active. Supported              22 transmission types shall be repetition coding and spatial multiplexing. The transmission type for the individ-              23 ual transmitter elements and the subcarrier loading shall be negotiated at the MAC layer. The receiver shall              24 monitor the MIMO operation modes using the MIMO reference symbols and shall signal the mode as well              25 as the subcarrier loading using control frames in the MAC layer.                                                                                    26

27

11.3.3.1 Spatial Repetition Coding:	28

29

In the repetition coding (RC), the same information is transmitted from more than one transmit elements.	30

The concept is illustrated in Figure 165 for a 2×2 MIMO system.	31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	321

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27	Figure 166—MIMO mode selection algorithm

28

29	11.3.3.2 Spatial Multiplexing:

30

31	In the spatial multiplexing (SMUX) more than one transmit element sends independent information. The

32	concept is illustrated in Figure 165 for a 2×2 MIMO system.

33

34	The MIMO mode selection and adaptive bit loading routine is expected to follow the algorithm described in

35	Figure 166. In the figure, the constraint metric denotes quality of service requirements that must be satisfied

36	in a given application, for example the level of maximum bit error rate. The performance metric denotes a

37	quality of service metric which the system attempts to optimize, such as the Phy_SAP data rate. Valid data

38	rates for spatial multiplexing MIMO communication with different number of transmitted streams are

39	shown in Table 146 to Table 150.

40

41

42	11.4 Channel coding

43

44	The PPDU shall be encoded with a convolutional encoder of coding rate R = 1/2, 2/3, or 3/4, corresponding

45	to the desired data rate. The convolutional encoder shall use the industry-standard generator polynomials, g0

46	= 1338 and g1 = 1718, of rate R = 1/2, as shown in Figure 167. The bit denoted as "A" shall be output from

47	the encoder before the bit denoted as "B". The summation operation presented in Figure 167 is a modulo-2

48	summation, i.e., an XOR operation. Higher rates shall be derived from this encoding mechanism by employ-

49	ing "puncturing." Puncturing is a procedure for omitting some of the encoded bits in the transmitter (thus

50	reducing the number of transmitted bits and increasing the coding rate) and inserting a dummy "zero" metric

51	into the convolutional decoder on the receive side in place of the omitted bits. The puncturing patterns are

52	illustrated in Figure 168. Decoding by the Viterbi algorithm is recommended.

53

54

322	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 146—Valid rate value s fo r MIMO S patial Multiple xing fo r 5 MHz Bandwidth	1

| Modulation | FEC rate | Data rate (Mb/s) 1 Tx | Data rate (Mb/s) 2 Tx | Data rate (Mb/s) 3 Tx | Data rate (Mb/s) 4 Tx | Data rate (Mb/s) 5 Tx | Data rate (Mb/s) 6 Tx | Data rate (Mb/s) 7 Tx | Data rate (Mb/s) 8 Tx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 1.5 | 3 | 4.5 | 6 | 7.5 | 9 | 10.5 | 12 |
|  | 3/4 | 2.25 | 4.5 | 6.75 | 9 | 11.25 | 13.5 | 15.75 | 18 |
| 4-QAM | 1/2 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
|  | 3/4 | 4.5 | 9 | 13.5 | 18 | 22.5 | 27 | 31.5 | 36 |
| 16-QAM | 1/2 | 6 | 12 | 18 | 24 | 30 | 36 | 42 | 48 |
|  | 3/4 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |
| 64-QAM | 2/3 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |
|  | 3/4 | 13.5 | 27 | 40.5 | 54 | 67.5 | 81 | 94.5 | 108 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

Table 147—Valid rate value s fo r MIMO S patial Multiple xing fo r 10 MHz Bandwidth	22

| Modulation | FEC rate | Data rate (Mb/s) 1 Tx | Data rate (Mb/s) 2 Tx | Data rate (Mb/s) 3 Tx | Data rate (Mb/s) 4 Tx | Data rate (Mb/s) 5 Tx | Data rate (Mb/s) 6 Tx | Data rate (Mb/s) 7 Tx | Data rate (Mb/s) 8 Tx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
|  | 3/4 | 4.5 | 9 | 13.5 | 18 | 22.5 | 27 | 31.5 | 36 |
| 4-QAM | 1/2 | 6 | 12 | 18 | 24 | 30 | 36 | 42 | 48 |
|  | 3/4 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |
| 16-QAM | 1/2 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |
|  | 3/4 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |
| 64-QAM | 2/3 | 24 | 48 | 72 | 96 | 120 | 144 | 168 | 192 |
|  | 3/4 | 27 | 54 | 81 | 108 | 135 | 162 | 189 | 216 |

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

11.4.1 Channel coding for the header	43

44

The PHY headers (both the basic header and the advanced modulation header) shallbe encoded using 1/2              45 rate FEC coding (no puncturing is used). The encoded bits shall also be mapped to a BPSK symbol constel-              46 lation.                                                                                                                                                                                47

48

11.4.2 Channel coding for the data	49

50

The DATA field, composed of SERVICE, PSDU, tail, and pad parts, shallbe encoded using the rate specified	51 by the MAC layer protocol.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	323

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Table 148—Valid rate value s fo r MIMO S patial Multiple xing fo r 15 MHz Bandwidth

| Modulation | FEC rate | Data rate (Mb/s) 1 Tx | Data rate (Mb/s) 2 Tx | Data rate (Mb/s) 3 Tx | Data rate (Mb/s) 4 Tx | Data rate (Mb/s) 5 Tx | Data rate (Mb/s) 6 Tx | Data rate (Mb/s) 7 Tx | Data rate (Mb/s) 8 Tx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 4.5 | 9 | 13.5 | 18 | 22.5 | 27 | 31.5 | 36 |
|  | 3/4 | 6.75 | 13.5 | 20.25 | 27 | 33.75 | 40.5 | 47.25 | 54 |
| 4-QAM | 1/2 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |
|  | 3/4 | 13.5 | 27 | 40.5 | 54 | 67.5 | 81 | 94.5 | 108 |
| 16-QAM | 1/2 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |
|  | 3/4 | 27 | 54 | 81 | 108 | 135 | 162 | 189 | 216 |
| 64-QAM | 2/3 | 36 | 72 | 96 | 144 | 180 | 216 | 252 | 288 |
|  | 3/4 | 40.5 | 81 | 121.5 | 162 | 202.5 | 243 | 283.5 | 324 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22	Table 149—Valid rate value s fo r MIMO S patial Multiple xing fo r 20 MHz Bandwidth

| Modulation | FEC rate | Data rate (Mb/s) 1 Tx | Data rate (Mb/s) 2 Tx | Data rate (Mb/s) 3 Tx | Data rate (Mb/s) 4 Tx | Data rate (Mb/s) 5 Tx | Data rate (Mb/s) 6 Tx | Data rate (Mb/s) 7 Tx | Data rate (Mb/s) 8 Tx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 6 | 12 | 18 | 24 | 30 | 36 | 42 | 48 |
|  | 3/4 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 |
| 4-QAM | 1/2 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |
|  | 3/4 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |
| 16-QAM | 1/2 | 24 | 48 | 72 | 96 | 120 | 144 | 168 | 192 |
|  | 3/4 | 36 | 72 | 96 | 144 | 180 | 216 | 252 | 288 |
| 64-QAM | 2/3 | 48 | 96 | 144 | 192 | 240 | 288 | 336 | 384 |
|  | 3/4 | 54 | 108 | 162 | 216 | 270 | 324 | 378 | 432 |

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

324	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 150—Valid rate value s fo r MIMO S patial Multiple xing fo r 40 MHz Bandwidth	1

| Modulation | FEC rate | Data rate (Mb/s) 1 Tx | Data rate (Mb/s) 2 Tx | Data rate (Mb/s) 3 Tx | Data rate (Mb/s) 4 Tx | Data rate (Mb/s) 5 Tx | Data rate (Mb/s) 6 Tx | Data rate (Mb/s) 7 Tx | Data rate (Mb/s) 8 Tx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 12 | 24 | 36 | 48 | 60 | 72 | 84 | 96 |
|  | 3/4 | 18 | 36 | 54 | 72 | 90 | 108 | 126 | 144 |
| 4-QAM | 1/2 | 24 | 48 | 72 | 96 | 120 | 144 | 168 | 192 |
|  | 3/4 | 36 | 72 | 108 | 144 | 180 | 216 | 252 | 288 |
| 16-QAM | 1/2 | 48 | 96 | 144 | 192 | 240 | 288 | 336 | 384 |
|  | 3/4 | 72 | 144 | 216 | 288 | 360 | 432 | 504 | 576 |
| 64-QAM | 2/3 | 96 | 192 | 288 | 384 | 480 | 576 | 672 | 768 |
|  | 3/4 | 108 | 216 | 324 | 432 | 540 | 648 | 756 | 864 |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

11.4.3 Data interleaving	22

23

24

25

26

27

28

29

30

31                                                                                                                                                        32

33

34

35

36

37

38

39

Figure 167—Convolution encoder (133,171)	40

41

42

All encoded data bits shall be interleaved by a block interleaver with a block size corresponding to the num-	43

ber of bits in two OFDM symbols, 2NCBPS. The interleaver is defined by a two-step permutation. The first	44

permutation ensures that adjacent coded bits are mapped onto nonadjacent subcarriers. The second ensures	45

that adjacent coded bits are mapped alternately onto less and more significant bits of the constellation and,	46

thereby, long runs of low reliability (LSB) bits are avoided. The index of the coded bit before the first per-	47

mutation shall be denoted by k; i shall be the index after the first and before the second permutation; and j	48

shall be the index after the second permutation, just prior to modulation mapping.	49

50

The first permutation is defined by the rule:	51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	325

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2	i  =

3

 2NCBPS

-----------

 --- 16 ---  k mod 16  + floor

-----

 k 

 16

,	k =

0, 1, , 2NCBPS – 1

(18)

4	The function floor (.) denotes the largest integer not exceeding the parameter.

5

6	The second permutation is defined by the rule

7

8

9	j  =

s  floor i +  i + 2N

– floor 16  -------- i---------  mod s,	i

= 0, 1, , 2N

– 1	(19)

 -	

CBPS	

----	 

CBPS

10	s

11

2NCBPS

12	The value of s is determined by the number of coded bits per subcarrier, NBPSC, according to

13

N

14

   BPSC   

2

15	s =

16

17

max ------------------, 1

(20)

18	The deinterleaver, which performs the inverse relation, is also defined by two permutations.

19

20	Here the index of the original received bit before the first permutation shall be denoted by j; i shall be the

21	index after the first and before the second permutation; and k shall be the index after the second permutation,

22	just prior to delivering the coded bits to the convolutional (Viterbi) decoder. The first permutation is defined

23	by the rule

24

25	i

= s  floor j +  j + floor  16  ----------j -------  mod s,	j

= 0, 1, , 2N	– 1

(21)

 - 	 	

----	 

CBPS

26	s

27

28	where s is defined in Equation (20).

29

2NCBPS

30	This permutation is the inverse of the permutation described in Equation (19). The second permutation is

31	defined by the rule:

32

33	 	j	

34	k =

16  i –  2NCBPS – 1 floor 16  2N

---------- ,	i

= 0, 1, , 2NCBPS – 1

(22)

-----------

35	CBPS

36

37	This permutation is the inverse of the permutation described in Equation (5).

38

39	11.5 Relaying

40

41	11.5.1 Amplify and forward

42

43	The PPDU shallbe buffered at the receiving node and upon successful demodulation of the advanced modu-

44	lation header and identification of the amplify and forward specification described in subclauses 6.2.3.2.8

45	and 6.2.3.2.9, the PPDU shallbe retransmitted without any further processing at the PHY layer. In FD relay-

46	ing mode, the relay STA shall simultaneously receive the information on the downlink and shall transmit it

47	on the uplink. In HD relaying mode, the relay STA shall store the data packets in order to retransmit them to

48	the STA during its designated transmission period. The relay STA can erase the stored packet after it

49	receives the ACK. The method is proprietary. In both configurations, the PHY header shallbe demodulated

50	before a decision for relaying can be made.

51

52

53

54

326	Copyright © 20xx IEEE. All rights reserved.

IEEE	RELESS MAC AND PHY SPECIFICATIONS  FOR 0IC 	IEEE  Draft P802.15.131

11.5.2 Decode and forward

2

The PPDU shallbe completely demodulated  and upon successful identification  of the decoded and forward               3 specification  described  in subclauses  6.2.3.2.8 and 6.2.3.2.9e PPDU shallbe re-modulated  and re-trans-                     4 mitted. The relay STA can erase the stored packet after it receives the ACK.The method is proprietary.                        5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright I(J 20xx IEEE.All rights reserved

This is an unapproved IEEE Standards Draftsubject  to change

327

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC 	IEEE Draft P802.15.13/D1

2 	R	eCoding(r=3/4)

3

5

4 	SourceData	I Xo I X1 I     | [;] | | GJ

6

7

8

9	EncodedData

10

11

12

13

14 	BitStolenData

15 	(senνreceived)

16

17

18

|?|;1| |:3|?|?|:6|C|?|

I I I I I I I I I I I I I

Al	A3 A4 As   A6

;3lm

19 	BitInstedData	Bo B1 B2    B3 B4 B B6 B7 B8 	Bit

20

21

22

23 	DecodedData	|YO|Y1| | | | | |Y1|Y8|

24

25

26

27 	PunctureCoding(r=2/3)

28

29 	SOUIceData

30

31

32

I Xo I X1 I X2  I I	X4 I         |

33 	EncodedData 	Aa A1	A3 A4 A 	sylm

34 	R B. B B. B. B.

35

36

37 	BitStolenData

38 	(sentlreceived)

39

40

41

Dummeyd

42 	Al	A3 A4 As

43 	Bitlns tedData

44 	Bo B1 B2    B3 B4 Bs 	Bit

45

46

48

47 	DecodedData 	I yo I Yl I Y2 1 Y31 Y41	|

49 	Figure 168-Puncturing(bitstealing)algorithm

50

51

52

53

54

328 	Copyrightð 20xx IEEE. All rightsreserved

Thisisanunapproved IEEE StandardsDraft subjectωchange

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

11.6 PHY Service Specifications	1

2

editor note: Clause 11.6 should be merged into Clause 9.2. Suggest Nikola give instructions on how to	3

merge. See comment 232 of https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-	4

7m-d1.xlsx	5

6

11.6.1 Overview	7

8

The PHY services provided to the IEEE 802.15.13 WLAN MAC are described in this clause. These services	9 are described in an abstract way and do not imply any particular implementation or exposed interface.	10

11

11.6.2 PHY Management Service	12

13

11.6.2.1 PHY Management Service Primitives	14

15

Table 151 presents the PHY management service primitives.	16

17

18

19

20

Table 151—PHY manag e me nt s e rvic e	21

| Primitive | Request | Indicate | Confirm |
| --- | --- | --- | --- |
| PHY-TXSTART | X |  | X |
| PHY-TXEND | X |  | X |
| PHY-RXSTART |  | X |  |
| PHY-RXEND |  | X |  |
| PHY-SET-LOADING | X |  | X |
| PHY-PROVIDE-CSI |  | X |  |

22

23

24

25

26

27

28

29

30

31

32

33

34

35

Table 152 presents the parameters used by the management service primitives.	36

37

38

39

40

Table 152—PHY manag e me nt s e rvic e primitive parame te rs	41

| Parameter | Associated Primitive | Value |
| --- | --- | --- |
| TXVECTOR | PHY-TXSTART.request | A set of parameters |
| RXVECTOR | PHY-RXSTART.indicate | A set of parameters |
| RXERROR | PHY-RXEND.indicate | NoError, FormatViolation, Unsupported Rate |
| BIT_LOADING_VECTOR | PHY-SET-LOADING.request | A set of parameters |
| CSI_VECTOR | PHY-PROVIDE-CSI.request | A set of parameters |

42

43

44

45

46

47

48

49

50

51

52

53

Table 153 presents the scalar parameters associated with the vector parameters in Table 152.	54

Copyright © 20xx IEEE. All rights reserved.	329

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4	Table 153—PHY manag e m e nt s e rvic e primitive parame te r ve c to r de s c riptio ns

| Parameter | Associated Vector | Value |
| --- | --- | --- |
| DATARATE | TXVECTOR RXVECTOR | A 3-bit value setting the data rate according to the specification in Table 6.2.3.1.1.1 |
| LENGTH | TXVECTOR RXVECTOR | The length of the data to be transmitted in octets as indicated by the LENGTH parameter described in subclause 6.2.3.1.3 |
| BIT_LOADING | TXVECTOR | Indicates whether adaptive bit and energy loading should be used at the PHY to encode the data. The possible values are ‘YES’ or ‘NO’. |
| CHANNEL_ESTIMATION | TXVECTOR | Indicates whether the channel state informa- tion should be estimated along with the current transmission. |
| EU_OFDM | TXVECTOR | Indicates whether eU-OFDM encoding should be used with the current transmission. The pos- sible values of this parameter are ‘YES’ or ‘NO’ |
| EU_OFDM_STREAMS | TXVECTOR | Indicates the number of eU-OFDM streams that should be used for the encoding in the cur- rent transmission. |
| SC_FDMA | TXVECTOR | Indicates whether SC-FDMA precoding should be used at the PHY layer for the current transmission. |
| DFT_SIZE | TXVECTOR | The size of the DFT that should be used in the SC-FDMA precoding. |
| RPO_FRAMES | TXVECTOR | The number of OFDM frames transmitted with reversed polarity. Possible values are integers between 0 and 127. |
| RPO_GAIN | TXVECTOR | A gain factor for the RPO-OFDM data trans- mission. Possible values are between 0 and 63. |
| RELAYING | TXVECTOR | Indicates whether relaying should be used for the current transmission. Possible values are 'YES' or 'NO'. |
| RELAYING_TYPE | TXVECTOR | The type of relaying operation that should be used. Possible values are 'FD-AF', 'HD-AF', 'FD-DF' or 'HD-DF' as specified in Table 6.2.3.2.10. |
| MIMO_ENABLED | TXVECTOR BIT_LOADING_VECTOR CSI_VECTOR | Indicates whether MIMO is used in the current transmission. Possible values are 'YES' or 'NO'. |
| REFERENCE_SYMBOLS | TXVECTOR | The type of reference symbols used for chan- nel estimation in the MIMO transmission. Pos- sible values are 'Type I' or 'Type II'. |

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

330	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Table 153—PHY manag e m e nt s e rvic e primitive parame te r ve c to r de s c riptio ns	1

| Parameter | Associated Vector | Value |
| --- | --- | --- |
| MIMO_CHANNELS | BIT_LOADING_VECTOR | The number of MIMO Channels. Possible val- ues are between 0 and 16. |
| BIT_LOADING | BIT_LOADING_VECTOR | A set of 24 values, corresponding to the bit loading on each of the 24 information-bearing subcarriers. Possible values are any one of the constellation sizes presented in subclause 6.3.5. If MIMO_ENABLED is equal to 'YES', then this parameter is a vector containting MIMO_CHANNELS vectors each with 24 values corresponding to the bit loading on each of the 24 subcarriers for the respective MIMO channel out of MIMO_CHANNELS channels. |
| ENERGY_LOADING | BIT_LOADING_VECTOR | A set of 24 values, corresponding to the scal- ing of the energy on each of the 24 informa- tion-bearing subcarriers. A value of 1 corresponds to the energy being scaled by 1 or, i.e., unchnaged. If MIMO_ENABLED is equal to 'YES', then this parameter is a vector con- tainting MIMO_CHANNELS vectors each with 24 values corresponding to the energy scaling on each of the 24 subcarriers for the respective MIMO channel out of MIMO_CHANNELS channels. |
| CSI | CSI_VECTOR | A set of 24 values, corresponding to the chan- nel state information on each of the 24 infor- mation-bearing subcarriers. If MIMO_ENABLED is equal to 'YES', then this parameter is a vector containting MIMO_CHANNELS vectors each with 24 values corresponding to the energy scaling on each of the 24 subcarriers for the respective MIMO channel out of MIMO_CHANNELS channels. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

11.6.2.2 Detailed PHY Management Service Specification	37

38

11.6.2.2.1 PHY-TXS TART.re que s t	39

40

6.7.2.2.1.1 Function	41

42

This primitive is a request by the MAC sublayer to the local PHY entity to start the transmission of an	43

MPDU.	44

45

6.7.2.2.1.2 Semantics of the service primitive	46

47

The primitive provides the following parameters:	48

49

PHY-TXSTART.request(TXVECTOR)	50

51

The TXVECTOR represents a list of parameters that the MAC sublayer provides to the local PHY entity in	52

order to transmit an MPDU. This vector contains PHY management parameters. The required PHY param-	53

eters are listed in Table 6.7.2.1.3.	54

Copyright © 20xx IEEE. All rights reserved.	331

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.7.2.2.1.3 When generated

2

3	This primitive will be issued by the MAC sublayer to the PHY entity when the MAC sublayer needs to begin

4	the transmission of an MPDU.

5

6	6.7.2.2.1.4 Effect of receipt

7

8	The effect of receipt of this primitive by the PHY entity will be to start the transmission process.

9

10	11.6.2.2.2 PHY-TXS TART.c o nfirm

11

12	6.7.2.2.2.1 Function

13

14	This primitive is issued by the PHY to the local MAC entity to confirm the start of a transmission. The PHY

15	will issue this primitive in response to every PHY-TXSTART.request primitive issued by the MAC sublayer.

16

17	6.7.2.2.2.2 Semantics of the service primitive

18

19	The semantics of the primitive are as follows:

20

21	PHY-TXSTART.confirm

22

23	This primitive has no parameters.

24

25	6.7.2.2.2.3 When generated

26

27	This  primitive will  be  issued by  the  PHY to  the  MAC entity when the  PHY has  received a  PHY-

28	TXSTART.request from the MAC entity and is ready to begin accepting outgoing data octets from the MAC.

29

30	6.7.2.2.2.4 Effect of receipt

31

32	The receipt of this primitive by the MAC entity will cause the MAC to start the transfer of data octets.

33

34	11.6.2.2.3 PHY-TXEND.re que s t

35

36	6.7.2.2.3.1 Function

37

38	This primitive is a request by the MAC sublayer to the local PHY entity that the current transmission of the

39	MPDU be completed.

40

41	6.7.2.2.3.2 Semantics of the service primitive

42

43	The semantics of the primitive are as follows:

44

45	PHY-TXEND.request

46

47	This primitive has no parameters.

48

49	6.7.2.2.3.3 When generated

50

51	This primitive will be generated when the MAC sublayer has received the last PHY-DATA.confirm from the

52	local PHY entity for the MPDU currently being transferred.

53

54	6.7.2.2.3.4 Effect of receipt

332	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The effect of receipt of this primitive by the local PHY entity will be to stop the transmission process upon	1 transmission of the last octet transferred from the MAC layer.	2

3

11.6.2.2.4 PHY-TXEND.c o nfirm	4

5

6.7.2.2.4.1 Function	6

7

This primitive is issued by the PHY to the local MAC entity to confirm the completion of a transmission.	8

The PHY issues this primitive in response to every PHY-TXEND.request primitive issued by the MAC sub-              9 layer.                                                                                                                                                                                  10

11

6.7.2.2.4.2 Semantics of the service primitive	12

13

The semantics of the primitive are as follows:	14

15

PHY-TXEND.confirm	16

17

This primitive has no parameters.	18

19

6.7.2.2.4.3 When generated	20

21

This  primitive will  be  issued by  the  PHY to  the  MAC entity when the  PHY has  received a  PHY-	22

TXEND.request immediately after transmitting the end of the last bit of the last data octet indicating that the	23 symbol containing the last data octet has been transferred.	24

25

6.7.2.2.4.4 Effect of receipt	26

27

The receipt of this primitive by the MAC entity provides a possible time reference for a contention backoff	28 protocol.	29

30

11.6.2.2.5 PHY-RXS TART.indic ate	31

32

6.7.2.2.5.1 Function	33

34

This primitive is an indication by the PHY to the local MAC entity that the PHY layer has received a valid	35 packet synchronization sequence and PHY header.	36

37

6.7.2.2.5.2 Semantics of the service primitive	38

39

The primitive provides the following parameter:	40

41

PHY-RXSTART.indicate (RXVECTOR)	42

43

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of              44 a valid PHY header. This vector may contain both MAC and MAC management parameters. The required              45 parameters are listed in Table 6.7.2.1.3.                                                                                                                            46

47

6.7.2.2.5.3 When generated	48

49

This primitive is generated by the local PHY entity to the MAC sublayer when the PHY has successfully	50 validated the PHY header at the start of a new PPDU.	51

52

6.7.2.2.5.4 Effect of receipt	53

54

Copyright © 20xx IEEE. All rights reserved.	333

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The effect of receipt of this primitive by the MAC is unspecified.

2

3	11.6.2.2.6 PHY-RXEND.indic ate

4

5	6.7.2.2.6.1 Function

6

7	This primitive is an indication by the PHY to the local MAC entity that the MPDU currently being received

8	is complete.

9

10	6.7.2.2.6.2 Semantics of the service primitive

11

12	The primitive provides the following parameter:

13

14	PHY-RXEND.indicate (RXERROR)

15

16	The RXERROR parameter can convey one or more of the following values:  NoError,  FormatViolation, or

17	UnsupportedRate. A number of error conditions may occur after the PHY layer protocol has detected what

18	appears to be a valid preamble and header. The following describes the parameter returned for each of those

19	error conditions.

20

21	- NoError.  This value is used to indicate that no error occurred during the receive process at the

22	PHY layer.

23

24	- FormatViolation. This value is used to indicate that the format of the received PPDU was in error.

25

26	- UnsupportedRate. This value is used to indicate that during the reception of the incoming PPDU,

27	a nonsupported date rate was detected.

28

29	6.7.2.2.6.3 When generated

30

31	This primitive is generated by the PHY for the local MAC entity to indicate that the receive state machine

32	has completed a reception with or without errors. In the case of an RXERROR value of NoError, the MAC

33	can use the PHY-RXEND.indicate as reference for channel access timing.

34

35	6.7.2.2.6.4 Effect of receipt

36

37	The effect of receipt of this primitive is for the MAC to begin inter-frame space processing if applicable.

38

39	11.6.2.2.7 PHY-S ET-LOADING.re que s t

40

41	6.7.2.2.7.1 Function

42

43	This primitive is a request by the MAC sublayer to the local PHY entity to update the bit loading scheme

44	used for adaptive bit and energy loading.

45

46	6.7.2.2.7.2 Semantics of the service primitive

47

48	The primitive provides the following parameters:

49

50	PHY-SET-LOADING.request(BIT_LOADING_VECTOR)

51

52	The BIT_LOADING_VECTOR represents a list of parameters that the MAC sublayer provides to the local

53	PHY entity in order to set the energy and the constellation size on each data-bearing subcarrier in a SISO or

54

334	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

MIMO configuration.  This vector contains PHY management parameters.  The required PHY parameters	1 are listed in Table 6.7.2.1.3.	2

3

6.7.2.2.7.3 When generated	4

5

This primitive will be issued by the MAC sublayer to the PHY entity when the MAC sublayer needs to	6 update the bit and energy allocation scheme.	7

8

6.7.2.2.7.4 Effect of receipt	9

10

The effect of receipt of this primitive by the PHY entity will be to update the bit and energy allocation	11 scheme accordingly.	12

13

11.6.2.2.8 PHY-S ET-LOADING.c o nfirm	14

15

6.7.2.2.8.1 Function	16

17

This primitive is issued by the PHY to the local MAC entity to confirm the update of the bit and energy allo-	18 cation scheme.	19

20

6.7.2.2.8.2 Semantics of the service primitive	21

22

The semantics of the primitive are as follows:	23

24

PHY-SET-LOADING.confirm	25

26

This primitive has no parameters.	27

28

6.7.2.2.8.3 When generated	29

30

This primitive will be issued by the PHY to the MAC entity when the PHY has updated the bit and energy	31 allocation scheme following the PHY-SET-LOADING.request by the MAC.	32

33

6.7.2.2.8.4 Effect of receipt	34

35

The receipt of this primitive by the MAC will cause the MAC to start the next MAC entity request.	36

37

11.6.2.2.9 PHY-PROVIDE-CS I.indic ate	38

39

6.7.2.2.9.1 Function	40

41

This primitive is an indication by the PHY to the local MAC entity that the PHY layer has received a request	42 to estimate the channel state information and has successfully completed the task.	43

44

6.7.2.2.9.2 Semantics of the service primitive	45

46

The primitive provides the following parameter:	47

48

PHY-PROVIDE-CSI.indicate (CSI_VECTOR)	49

50

The CSI_VECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt	51 of a valid PHY header and successful estimation of the CSI. The required parameters are listed in Table	52

6.7.2.1.3.	53

54

Copyright © 20xx IEEE. All rights reserved.	335

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	6.7.2.2.9.3 When generated

2

3	This primitive is generated by the local PHY entity to the MAC sublayer when the PHY has successfully

4	validated the PHY header at the start of a new PPDU and estimated the CSI.

5

6	6.7.2.2.9.4 Effect of receipt

7

8	The effect of receipt of this primitive by the MAC is unspecified.

9

10

11	12. High-bandwidth OFDM PHY Specifications

12.1 PPDU format

TBD

12.2 Transmission

12.2.1 Preamble field

TBD

12.2.2 PHY Header

TBD

12.2.3 PSDU

TBD

12

13

14	12.1 Adaptive OFDM

15

16	The idea of the adaptive OFDM physical layer is shown in Figure 169. At the transmitter (Tx), the input data

17	are transported via orthogonal subcarriers. Each data symbol carrying one or more bits is mapped onto a

18	constellation point, according to a variable modulation format for each subcarrier. A Hermetian symmetry

19	operation is then performed to create a real-valued waveform. An OFDM symbol is generated by feeding

20	symbols in the frequency domain into the inverse fast Fourier transform (IFFT) followed by the insertion of

21	a cyclic prefix (CP). The output of the OFDM signal may then be clipped in the digital domain and passed

22	through the digital-to-analog converter (DAC) and low-pass filter (LPF). If any pulse shaping is employed,

23	then it is recommended to perform clipping after the pulse shaping operation. A bias is normally added to

24	ensure a unipolar all positive signal before it is used for intensity modulation of the optical source (i.e., light

25	emitting diode (LED) or a laser diode (LD)).

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52	Figure 169—Overview of the standalone link (one link direction)

53

54

336	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Following conversion from optical to electrical signal and signal detection, the inverse operations are per-              1 formed at the receiver (Rx), where a frequency-domain equalizer (FDE) is used to reconstruct the received              2 constellation points on each sub-carrier, after passing them through the OWC channel. The desired mapping              3 of information bits onto the sub-carriers is sent by the receiver to the transmitter over the reverse link. In the              4 example shown in Figure 169, the metrics reporting is carrying a so-called noise-enhancement vector. A              5 power- and bit-loading algorithm determines the power and modulation formats for the data transport on              6 each used subcarrier. The loading algorithm typically maximizes the throughput assuming a fixed power              7 budget while satisfying a predefined bit error rate (BER) before forward error correction (FEC).                                 8

9

12.1.1 Adaptive OFDM Waveform	10

11

The PHY uses the adaptive OFDM waveform in both link directions with following extensions:	12

13

- The waveform is always non-negative and real-valued.	14

15

- A bias is added and clipping (if needed) is implemented in the digital domain.	16

17

- Optional single-carrier or unipolar modulation to improve power efficiency and support dimming.	18

19

OFDM signal generation is shown in Figure 170. A block of 2N data symbols is transmitted. An optional              20 pre-coding is used for the generation of singlecarrier or unipolar modulation schemes based on OFDM to              21 improve power efficiency and support dimming. The signal is then passed through a carrier mapping unit,              22 used for precoding and Hermitian symmetry. Next, the IFFT is performed, the CP is added and controlled              23 clipping is performed in the digital domain.                                                                                                                     24

25

26

| Precoding (optional) |  | Carrier Mapping |  | IFFT |  | CP insertion |  | controlled clipping |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27

28

29

Figure 170—OFDM signal generation	30

31

32

12.1.1.1 Carrier mapping	33

34

Carrier mapping is performed as illustrated in Figure 171. Note that the subcarrier x0 may be used to add a	35 constant bias signal to the output signal. In order to create a real-valued waveform, only half of the subcarri-	36

ers are used, while conjugate symmetry is enforced as	37

38

x2 N – i

= x*, i

= 1, 2, , N – 1

(23)	39

40

41

i

where the star indicates complex conjugation. The resulting discrete multi-tone (DMT) signal is real-valued,	42

even if symbols xi are complex.	43

44

45

|  | IFFT |  |
| --- | --- | --- |
| 1 |  |  |

|  | conjugate symmetry x2N-i = xi * i=1,2,...N-1 |  |
| --- | --- | --- |

s/p

x

x

0

1

x N-1

x

x

0

1

x

x N-1

46

47

p/s 	48

N	49

x

x2N-	50

2N	51

Figure 171—Carrier mapping for standalone link	52

53

54

Copyright © 20xx IEEE. All rights reserved.	337

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	12.1.1.2 IFFT

2

3	The time-domain signal X(k) is given by

4

5	ik

j2

2N – 1

6	1

------

2N

7	X  k  =

8

9

------

2N



i = 0

xie

(24)

10	where i denotes the sample index, xi the complex-valued baseband signals in the frequency domain and 2N

11	the block size of the IFFT.

12

13	12.1.1.3 Cyclic prefix

14

15	At the output of the IFFT, in the serial block of 2N samples, the last CP samples are copied as a sub-block

16	being repeated and appended at the beginning of the block of samples, see Figure 172. By adding the CP at

17	the transmitter, and removing it at the receiver, the multipath channel matrix can be transformed from

18	Toeplitz-shape into a circulant shape, which allows the use of IFFT at the transmitter and FFT at the receiver

19	to obtain a diagonal channel in the frequency domain, so that a simple single-tap frequency-domain equal-

20	izer (FDE) can be used.

21

22

23

24	last CP

25	samples

26	repeated

27

28

29

30

31

32

2N samples	last CP

samples

Figure 172—Cyclic prefix insertion

33	12.1.2 Single-carrier modulation

34

35	Optional pre-coding before the OFDM modulator can be used to reduce the probability of clipping and

36	enhance power efficiency while sacrificing no or minor spectral efficiency [1, 2].

37

38	For single-carrier (SC) transmission, "outer" pre-coding, together with an "inner" OFDM transmitter is used

39	to emulate SC transmission inside the OFDM concept. The novel schemes require little more advanced sig-

40	nal processing, and the same minor increase of sophistication can be expected at the receiver, i.e. decoding is

41	straightforward. The schemes are shown in principle in Figure 173. Details can be found in [1, 2].

42

43

44

45

46

47

48

49

50

51

52

53

54

338	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

A.

M data symbols

B.

M data symbols

| M data |  | M- |  | Map M | repeat |  | frequency- |  | 2N- |  | MSK | 2N- |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| symbols |  | DFT |  | to 2N | signals |  | domain filter |  | IDFT |  | modulator | DFT |

C.

M- DFT

M- DFT

Map M

to 2N

Map M

to 2N

repeat signals

frequency- domain filter

1

CS	2N-	CP	2

IDFT	3

4

5

6

CS	2N-	CP	7

IDFT	8

9

10

11

CS	2N-	CP	12

IDFT

13

Figure 173—Single-carrier modulation schemes used to improve the power efficiency of		14 the OFDM transmitter. Top: Pure DFT precoding uses a roll-off factor = 0. Center: A root-		15 raised-cosine filter can be added in the frequency domain, to realize > 0. Bottom: A	16

Gaussian filter can be used in the same way and a minimum-shift keying (MSK) modulation	17 can be added in the time domain. In this way, the classical GMSK waveform can be realized	18 inside an OFDM system.	19

20

21

12.1.2.1 DFT-pre-coded OFDM	22

23

The simplest SC transmitter is shown in Figure 173 row A. First, the symbol sequence is passed through the	24

N-DFT and then mapped directly onto the desired frequency sub-band after using a cyclic shift (CS) so that	25

the DC signal is in the center. Finally, the precoded sequence is passed through the M-IDFT and the cyclic	26

prefix (CP) is added.	27

28

As shown in [2], this procedure yields a SC signal having a roll-off factor α=0. The rectangular filtering	29

causes "ringing" in the time domain which increases the peak-to-average power ratio (PAPR). As this is a	30

special case of the RRC-filtered SC transmission, details are described in the next subsection.	31

32

12.1.2.2 RRC-filtered single-carrier modulation	33

34

In the middle row in Figure 173, an additional root-raised-cosine filter is introduced in the frequency domain

≥

where α  0. In order to realize filtering in the frequency domain, oversampling is emulated by repeating the	35

36

DFT output block in the frequency domain. Afterwards, the root-raised cosine (RRC) filter is applied in the	37

frequency domain. The sequence is then mapped directly onto the desired frequency sub-band using a cyclic	38

shift (CS) so that the DC signal is in the center. In the following, these steps are described in detail.	39

40

A data sequence a(n) of length M is used where n = 1, 2, …, M. The sequence is up-sampled by factor F as	41

follows	42

43

 	44

b k 

=   a n , if k=FN

 	0, else

– 0.5

2N

(25)	45

46

47

with F =

------

M

, where k = 1, 2, …, FM and 2N is the number of samples in the final waveform w/o the	48

CP. The notation

z  is used here to indicate that z is rounded to the nearest integer less than or equal to z.	49

Note that F-times up-sampling followed by M-DFT is equivalent to M-DFT and subsequent spectral repeti-	50 tion, provided that the ratio 2N/M is an integer. The proof is given in [B20]. Accordingly, up-sampling and	51

2N-DFT can be replaced by M-DFT and repeating the output signal in the frequency domain.	52

53

54

Copyright © 20xx IEEE. All rights reserved.	339

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Next step is a flexible frequency-domain filter. It is implemented so that bandwidth can be easily changed as

2	a function of the block size M. Therefore, a vector is defined with running index s = [-M, …, M] the bell-

3	shape part of the filter is computed as

4

5	M

6	 	  si – 1 –   2  

-----------

7	Gl =



0.5 1 + cos

---- 

--------------- M---------------- 

(26)

8	 	

9

10	where l = 1, 2, …2M+1. The filter is transparent in the range

11

12

13	a	M	1

 1 –   M

 M	1

 1 –   M

(27)

=	+   –

14

15

---------------------- ,	,

2

+	+   ----------------------

2

16	There are two regions where the filter attenuates totally. They are given by

17

18

19	b =

20

21

22

1, , M + 1 –

 1 +   M

----------------------

2

(28)

23	c =

24

M + 1 +

 1 +   M

----------------------

2

, , 2M + 1

(29)

25	Gl is now set as Ga = 1, Gb = 0 and Gc = 0 in the respective regions indicated by vectors a, b and c. Note

26	that up-conversion is equivalent to performing sequentially 2N-DFT of the time-domain sequence, a cyclic

27	shift by Ncenter and 2N-IDFT of the shifted signal, as shown in [2].

28

29	In row B. Figure 173, the synthesis of filtered QAM is summarized in the frequency domain. First the data

30	symbol sequence is passed through the M-DFT and the output is repeated in the frequency domain. Next, the

31	signal is filtered in the frequency domain and the cyclic shift is applied to up-convert the signal to the

32	desired center sub-carrier Ncenter. Finally, the signal is passed through the 2N-IDFT and the cyclic prefix is

33	added [2].

34

35	Note that in the SC transmitter, carrier mapping has been modified compared to LTE. In this way, wave-

36	forms become comparable to time-domain single-carrier signal, see [2]. The new mapping is sketched in

37	Figure 174.

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

340	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

DC	4

6

A	B	DFT output signal	5

7

1 ...

A

M/2  ... M

8

9

B	10

11

1 ...

M/2	2N- M/2 +1...2N	12

13

A	B

A	B	A

cyclic shift (CS)

B

A	B	14

15

16

17

18

19

Figure 174—Generation of filtered single-carrier signals	20

21

The direct current (DC) sub-carrier of the M-DFT output vector (having index 1) is first mapped onto the	22

DC sub-carrier of the 2N-IDFT. The two blocks	23

24

25

A =   2, ,

M

----

2

and	B =

M

----

2

+ 1, , M

(30)	26

27

28

are then mapped onto the first and last sub-carriers, see Figure 174. Periodic replica are added in the fre-              29 quency domain to emulate up-sampling. Finally, the frequency-domain filter is applied and the cyclic shift is              30 used to modulate the signal onto the center subcarrier.                                                                                                    31

32

12.1.2.3 Gaussian minimum shift keying	33

34

GMSK is known for zero PAPR in radio links. Although this goal is not reached in OWC links, due to the              35 real-valued waveform, GMSK offers ultra- robust signaling in case of very low SINR. First, the classical              36 time-domain GMSK single-carrier transmitter is reviewed. The serial data symbol sequence a(n) is up-sam-              37 pled as in subsection 2.2.2.5.2 yielding b(k). After applying a Gaussian filter in the time domain, the filtered              38 signal c(k) is obtained. The classical Gaussian filter is approximated in the time domain using a finite              39 impulse response (FIR) with some memory. Next, c(k) is passed into a minimum shift keying (MSK) modu-              40 lator where it is first accumulated yielding the phase                                                                                                       41

42

43

c k – 1 

 k  =

 k – 1  +



------

2F

(31)	44

45

and then inserted into the complex amplitude	46

47

x k 

= I + jQ =

cos   k   + j sin   k  

(32)	48

49

Note that in-phase signal I and the quadrature signal Q in are fed by the same phase but at a shift of 90°              50 yielding single side band (SSB) modulation when up-converting the sequence to the desired center fre-              51 quency. This is often performed using an analog IQ modulator. The same SSB up-conversion can be reached              52 by means of digital signal processing. Therefore, the complex-valued GMSK baseband signal is multiplied              53

54

Copyright © 20xx IEEE. All rights reserved.	341

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	sample-by-sample with a digitally synthesized complex-valued oscillation due to single OFDM sub-carrier,

2	being the center frequency of the desired GMSK-modulated signal. Finally, a window of length M is applied

3	in the time domain.

4

5	The equivalent processing for GMSK using OFDM is summarized on row C. in Figure 173. As before, the

6	data sequence a(n) is fed into the M-DFT and up-sampling is emulated by repeating the output signal in the

7	frequency domain. Next, a Gaussian filter is applied in the frequency domain. A vector is created with run-

8	ning index s = [-R, …, R] where R≤M, the filter is computed as

9

10	2 2

11	–  sn

ln 2    1

12	Gn = e

where	 =

-- 2 --- MB T

(33)

-------

13

------------

14	where n = 1, 2, ..., 2R +1 and BT is the bandwidth-time product. BT=0.3 is recommended. GMSK is a non-

15	linear SSB phase modulation. Thus, the two functions of accumulating the signal and generating the in-

16	phase and quadrature signals are better realized in the time domain. The main idea is to insert the GMSK

17	modulator after frequency-domain filtering, but in the time domain. Using M-IDFT of the filtered data

18	sequence, c(k) is obtained. Next, c(k) is normalized to unit peak amplitude and feed it into the time-domain

19	MSK modulator described above. Then up-conversion is applied. It can be equivalently implemented as

20	shown in row C in Figure 173 or in the time-domain as described above. Finally, the CP is added.

21

22	GMSK causes adjacent channel interference since SSB phase modulation is a non-linear process. Even if the

23	GMSK modulator input is confined in the frequency domain, four-wave mixing between in-band sub-carri-

24	ers creates out-of-band interference. Such interference can be cut using an optional post-modulation filter in

25	the frequency domain attenuating totally outside the range s=[-R, ...,R] and correct the power, accordingly.

26

27	12.1.2.4 From complex- to real-valued single-carrier transmission

28

29	The above waveforms yield complex-valued sequences. Same as in the adaptive OFDM approach, the com-

30	plex-valued waveform covers only the first N subcarriers and then Hermetian symmetry is needed to gener-

31	ate a real-valued waveform. Conjugate symmetry is obtained as

32

34

33	x2 N – i

35

= xi ,	i =

1, 2, , N – 1

(34)

36	The resulting discrete multi-tone (DMT) signal is real-valued, even if symbols xn are complex.

37

38	12.1.3 Unipolar modulation

39

40	To modulate the intensity of light, the signal needs to be real-valued and non-negative. For adaptive OFDM,

41	besides the Hermitean symmetry operation described in subsection 3.2.2.1, a DC bias has to be added, which

42	results in a reduced power efficiency.

43

44	12.1.3.1 ACO-OFDM

45

46	One way to overcome this disadvantage is asymmetrically-clipped optical (ACO) OFDM as a first unipolar

47	modulation scheme to improve the power efficiency [F-5].

48

49	For ACO-OFDM, the time domain signal is made unipolar by simply clipping the negative part at the zero

50	level, which does not need a large DC bias. That is

51

52	

53	xclipped k  = 

x k    if

 x k   0 

(35)

54	

0  else

342	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

where x (k) is the bipolar real-valued OFDM signal after the IFFT and xclipped(k) the unipolar signal after	1 clipping.	2

3

If only the odd subcarriers are modulated by signals, the effect of clipping is that inter-carrier interference              4 (ICI) is created that falls only on the even subcarriers. The effect of clipping on the odd subcarriers is simply              5 a multiplication of these components by a constant 0.5. I.e., clipping does not result in inter-carrier interfer-              6 ence (ICI) on the odd subcarriers, if the even subcarriers remain vacant. The diagram of ACO-OFDM mod-              7 ulation is shown in Figure 175.                                                                                                                                         8

9

10

11

12

13

Figure 175—ACO-OFDM modulation	14

15

12.1.3.2 eU-OFDM	16

17

A real-valued OFDM signal, generated at the PHY layer, is used to modulate the light. The modulation is              18 conducted only within the active operational range of the device. In this range, the electrical signal and the              19 light output signal cannot be negative at all times. The conventional approach is to set a positive operating              20 point, around which the bipolar OFDM signal can be realized. Figure 176 (left) illustrates this principle. The              21 positive bias can be introduced as part of the analog front-end (in the case of AC-coupled LED drivers) or as              22 part of the information signal (in case of DC-coupled drivers). This approach is known as DC-biased optical              23

OFDM (DCO-OFDM).	24

25

Enhanced unipolar OFDM (eU-OFDM) is an optional alternative modulation scheme. It turns the bipolar	26

OFDM signal into a strictly unipolar information signal without the addition of an energy intensive DC com-	27 ponent that carries no additional information, see Figure 176 (right).	28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

Figure 176—Left: Frame start using DC-biased optical OFDM (DCO-OFDM). Right: Same	43

using Enhanced Unipolar OFDM (eU-OFDM)	44

45

46

12.1.3.3 Enhanced unipolar OFDM signal generation	47

48

The eU-OFDM signals are generated in layers, indicated by the variable STR.	49

50

For one stream, i.e. STR = '00', two consecutive copies of every OFDM symbol are created. The polarity of	51

the samples in the second copy is inverted, and finally, all negative samples in the resulting time-domain	52

double-symbol are set to zero. Any time-domain oversampling and pulse shaping is done after the removal	53

54

Copyright © 20xx IEEE. All rights reserved.	343

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	of negative samples. The resulting positive signal is used to modulate the transmitter. The concept is illus-

2	trated in Figure 177.

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29	Figure 177—Unipolar OFDM signal generation (one stream).

30

31	For two streams, STR = '10', every three OFDM symbols are grouped into one eU-OFDM block, where the

32	first two symbols are assigned to stream 1 (St1) and the remaining single symbol is assigned to stream 2

33	(St2).

34

35	The first two symbols in St1 are modulated using the algorithm described for STR='00' and shown in

36	Figure 177. The single symbol in St2 is modulated in a similar manner, but instead of two copies, four con-

37	secutive copies are created for the OFDM symbol in St2, where the first two copies are kept unchanged,

38	while the polarity of the samples in the next two copies is inverted.

39

40	Following this procedure, all negative samples in both St1 and St2 are removed, and the two signals are

41	summed up. Any time-domain oversampling and pulse shaping is done after the removal of the negative

42	samples. The resulting positive signal can be used to modulate the transmitter.

43

44	For three streams, i.e. STR = '01', every seven OFDM symbols are grouped into an eU-OFDM block, where

45	the first four symbols are assigned to data stream 1 (St1), the next 2 symbols are assigned to data stream 2

46	(St2) and the last symbol is assigned to data stream 3 (St3).

47

48	The four symbols in St1 and the two symbols in St2 are modulated using the algorithm described for two

49	streams, see 4.2.4.2.1.2. The symbol in St3 is modulated in a similar manner, however, eight consecutive

50	copies of that symbol are generated, where the first four copies are left unchanged, while the polarity of the

51	samples in the following four copies is reversed.

52

53

54

344	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Following this procedure, all negative samples in St1, St2 and St3 are removed. Any time-domain oversam-              1 pling and pulse shaping is done after the removal of the negative samples. The signals in the three streams              2 are summed up and the resulting positive signal can be used to modulate the transmitter.                                              3

4

For four streams, i.e. STR = '11', every fifteen OFDM symbols are grouped into an eU-OFDM block, where	5 the first eight symbols are assigned to data stream 1 (St1), the next four symbols are assigned to data stream	6

2 (St2), the following two symbols are assigned to data stream 3 (St3) and the last symbol to data stream 4	7 (St4).	8

9

In the first stream, two consecutive copies of every OFDM symbol are transmitted, where the second copy is              10 multiplied by -1. In the second stream, four consecutive copies of every OFDM symbol are transmitted,              11 where the first two copies are transmitted in their original format, while the signs of the third and the fourth              12 copy are multiplied by -1. In the third stream, eight consecutive copies of every OFDM symbol are transmit-              13 ted, where the first four copies are conveyed in their original format, while the signs of last four copies are              14 multiplied by -1. In the fourth stream, sixteen consecutive copies of the single OFDM symbol are transmit-              15 ted, where the first eight copies are used in their original format, while the last eight copies are are multiplied              16 by -1.                                                                                                                                                                                 17

18

Finally, all negative samples in the four streams are removed and the signals from the four streams are              19 summed up. Any oversampling and pulse shaping is done after the removal of the negative samples. The              20 resulting positive signal can be used to modulate the transmitter.                                                                                    21

22

12.1.4 Multiple-input multiple-output	23

24

The use of several multiple-input multiple-output (MIMO) schemes is foreseen in order to support diversity	25 and spatial multiplexing, which can both be realized both as non-imaging and imaging MIMO [F-6, F-7].	26

The performance of wavelength-division multiplexing (WDM) and wavelength-shift keying (WSK) trans-	27 mission can also be improved by using MIMO transmission schemes.	28

29

12.1.4.1 Signal model	30

31

For two transmitters and two receivers, the signal model for non-imaging MIMO transmission on each sub-	32 carrier can be expressed as	33

34

35

y =   y1

y2

= Hx + n =

H11 H12

H21 H22

x	+  n1

1

x2	n2

(36)	36

37

38

39

where, in general, bold upper case letters describe matrices and bold lower case letters describe vectors. The              40 received signals are denoted as yi where i=1…nRx and nRx is the number of receivers. The transmitted sig-              41 nals are described as xj where j=1…nTx and nTx is the number of transmitters. The channel matrix elements              42

Hij represent the channel gain from the jth transmitter to the ith receiver.                                                                        43

44

12.1.4.2 Non-imaging MIMO                                                                                                                         45

46

In case of non-imaging MIMO, see Figure 178, light from each of all transmitters (usually having a wide              47

beam) is received by all receivers (usually having wide field-of-views, FOV) , but channels may have differ-              48

ent path loss and impulse response as shown in Figure 178. In case of non-imaging MIMO, the channel              49

matrix H is usually fully occupied and the elements Hij may depend on the subcarrier index n because of the              50

frequency response of optoelectronic components and the superposition of LOS and NLOS propagation              51

effects.                                                                                                                                                                               52

53

54

Copyright © 20xx IEEE. All rights reserved.	345

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13	12.1.4.3 Imaging MIMO

14

Figure 178—Non-imaging MIMO transmission

15	Imaging MIMO uses imaging optics between the transmitters and receivers, which are usually arranged in

16	arrays. Because of the lens, the LOS component is often dominant while the diffuse reflections are reduced

17	due to the reduced FOV at the receiver. Ideally, the transmitter array is imaged 1:1 onto the receiver array. In

18	this case, the channel crosstalk can be neglected, and the channel matrix H has a diagonal shape, i.e. can be

19	simplified and becomes diagonal, i.e.

20

21	

22	

23	Hij = 



24	

25

26

27

28

29

30

31

32

33

34

35

36

37

38

Hii  if	i=1

0   else

Figure 179—Imaging MIMO transmission

(37)

39	12.1.4.4 Polarization-division multiplexing

40

41	Lights emitting from incoherent LEDs include all polarization directions that can be decomposed as two

42	orthogonal bases, e.g. of horizontal (h) and vertical (v) polarization. For LED-based polarization-division

43	multiplexing (PDM), a linear h-polarizer is inserted that can only allow h-polarized components passing

44	through at the first transmitter (TX1); meanwhile, a linear v-polarizer is used at TX2. Note that many laser

45	diodes yield linearly polarized light so that the transmitter-sided polarizers may not be needed when using

46	lasers. After passing through Tx- sides polarizers, the polarized light is mixed up after free-space propaga-

47	tion. At the receiver (RX), two polarizers are then implemented to filter out the unwanted polarized lights,

48	thus obtaining the transmitting signals.

49

50

51

52

53

54

346	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

Figure 180—Polarization-division multiplexing	17

18

19

12.1.4.5 Wavelength-division multiplexing	20

21 t.b.d.	22

23

12.1.4.6 Link setup for MIMO	24

25

Initial link setup and header transmission detection are performed in the single-input single-output (SISO)	26

mode, in order to improve the reliability of transmission. Transmission of the preamble and the header is	27

28

done using all transmitters, while detection can be improved by using maximum ratio combining (MRC)	29

based  on  individual  estimates  of  the  superimposed  channels  from  all  transmitters  at  each  receiver	30

i		ij

31

h  	nTx   H   . The number of used transmitters is included in the header.	32

j 1

33

12.1.4.7 Additional channel estimation symbols for MIMO	34

35

Additional channel estimation (ACE) symbols are sent in the beginning of the data field in the PHY frame.	36

They are defined in the frequency domain and over multiple OFDM symbols. Each ACE symbol contains	37 the same sequence of bits on all subcarriers that is only passed through the constellation scrambler. For	38

MIMO, the sequence {sn} contains all 1s, but is multiplied as a whole with a sign taken out of an orthogonal	39 sequence. For 1 transmitter (Tx), the channel estimation symbol in the preamble is used	40

41

n

1 Tx:  [{s }]	42

43

For 2 Txs, the first symbol and one ACE signal are sent as	44

45

2 Txs:	46

47

n	n

Tx1 [{s } {s }],	48

49

n	n

Tx2 [{s } {-s }].	50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	347

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	where the first symbol is always contained in the header and only one additional symbol is sent for ACE.

2	The scheme in G.9963 obviously makes use of the Hadamard sequences. It can be easily extended to 4 and 8

3	transmitters

4

5	4 Txs:

6

7	Tx1 [{sn} {sn} {sn} {sn}]

8

9	Tx2 [{sn} {-sn} {sn} {-sn}],

10

11	Tx3 [{sn} {sn} {-sn} {-sn}],

12

13	Tx4 [{sn} {-sn} {-sn} {sn}].

14

15	8 Txs:

16

17	Tx1 [{sn} {sn} {sn} {sn}{sn} {sn} {sn} {sn}]

18

19	Tx2 [{sn} {-sn} {sn} {-sn}{sn} {-sn} {sn} {-sn}],

20

21	Tx3 [{sn} {sn} {-sn} {-sn}{sn} {sn} {-sn} {-sn}],

22

23	Tx4 [{sn} {-sn} {-sn} {sn}{sn} {-sn} {-sn} {sn}].

24

25	Tx5 [{sn} {sn} {sn} {sn}{-sn} {-sn} {-sn} {-sn}]

26

27	Tx6 [{sn} {-sn} {sn} {-sn}{-sn} {sn} {-sn} {sn}],

28

29	Tx7 [{sn} {sn} {-sn} {-sn}{-sn} {-sn} {sn} {sn}],

30

31	Tx8 [{sn} {-sn} {-sn} {sn}{-sn} {sn} {sn} {-sn}].

32

33	12.1.4.8 Adaptive MIMO transmission

34

35	In the following, several transmission modes will be described that can be used to operate a MIMO link. The

36	main objective is to enable a dynamic trade-off between spatial diversity and spatial multiplexing, so that the

37	best number of streams is always selected to maximize the throughput and to operate the link reliably. It is

38	assumed that the MIMO link will be operated adaptively in a bidirectional closed-loop manner and that

39	MIMO metrics reports regarding the forward link are provided over the reverse link.

40

41	12.1.4.8.1 Optimal MIMO trans mis s io n

42

43	Ideally, with having full channel state information conveyed from the receiver to the transmitter, the MIMO

44	transmission can be described as follows. The transmission on each subcarrier is most conveniently formu-

45	lated in the frequency domain as

46

47

48	y n   H n x n   ν n

49

(38)

50	where the nTx  1 vector xn contains the signals transmitted from all transmitters at the OFDM sub-carrier

51	with index n. The nRx1 vectors yn and vn contain the received signals and the noise, respectively. The inte-

52	gers nTx and nRx denote the numbers of transmitters and receivers, respectively.

53

54

348	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

The nRx nTx matrix Hn denotes the channel matrix for sub-carrier n with the channel coefficients between	1 each transmitter and each receiver. It is related to the time-domain channel impulse response matrices Hl as	2

3

L1	

nl 	4

H n    H l exp  j 2 	

(39)	5

l 0		N 	6

7

where L denotes the number of resolved multi-paths. In the optimal way, based on full channel state infor-              8 mation (CSI) at the transmitter and at the receiver, the channel capacity is approached asymptotically by per-              9 forming a singular value decomposition (SVD) of Hn on each sub-carrier,                                                                     10

11

n	n	n     n

H    U D V H

(40)	12

13

14

which gives the matrices Vn and Un containing the Eigenvectors of the channel matrix in the transmit and	15 receive spaces, respectively.	16

17



The diagonal matrix Dn which contains i = 1…min(nTx, nRx) singular values

n , referred to as the amplitude	18

i

gains of the spatial Eigenmodes. The superscript H denotes the conjugate transpose of a matrix. In the infor-              19 mation theory, the capacity is asymptotically approached for infinite N by a joint water-filling across all spa-              20 tial Eigenmodes i and all sub-carriers n. Unlike in the information theory, in practise we employ discrete              21 instead of continuous modulation alphabets. A joint bit-loading and power allocation algorithm is therefore              22 used with individual modulation on each Eigenmode and each sub-carrier, according to the current channel              23 state, so that important optimization criteria (throughput, fairness, stability of queues) can be fulfilled.                        24

25

The transmitted signal vector xn=Vndn is obtained from the data vector dn and the spatially multiplexed data	26

27

n

n 	n

signals are reconstructed at the receiver as

dˆ 	 D  1

 U H y

. The noise in each stream is boosted differ-	28

ently, according to the singular value for each stream.	29

30

Depending on the availability of the CSI, there are modifications. When CSI is available only at the receiver,	31

no pre-processing is applied. Assuming additionally linear detection which requires a simple matrix-vector	32

multiplication, the transmitted signals on each sub-carrier may be reconstructed using the minimum mean-	33

square error detector given by the formula	34

35

	H	2

1	H	36

xˆ n  

H n H n      I

H n y n

(41)	37

38

where I and ² are the (nTx×nTx) identity matrix and noise variance at one receiver, respectively.	39

40

41

42

43

44

45

46

47

48

49

50

51

Figure 181—Adaptive MIMO transmission	52

53

54

Copyright © 20xx IEEE. All rights reserved.	349

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	12.1.4.8.2 Prac tic al imple me ntatio n

2

3	Following the optimal MIMO transmission scheme, spatial processing is introduced both at the transmitter

4	and at the receiver, see Figure 181. Moreover, a variable number of streams is used because in some cases,

5	depending on the time-variant wireless channel, a higher capacity is reached with a reduced number of

6	streams. For instance, if an LED is directed away from the receiver, it is hardly useful for transmission and

7	may be switched off.

8

9	User data are de-multiplexed yielding Q parallel data streams, where Q is an integer ranging from 1 to nTx.

10	The data in each stream are transported using an individually selectable modulation scheme on each subcar-

11	rier, in order to maximize the throughput, which is also denoted as per-stream rate control. All active streams

12	are then passed through a spatial scheme processing unit, in which channel knowledge, obtained over the

13	reverse link is used to identify the best spatial pre-processing of all streams transported in parallel. It is clear

14	that the MIMO channel rank can vary over time and also as a function of the subcarrier index n. Accord-

15	ingly, and as a natural extension of the adaptive OFDM approach, the selection of the best MIMO transmis-

16	sion mode is done for each subcarrier or for a group of subcarriers. Several simplifications of the spatial

17	processing are introduced now in order to reduce complexity.

18

19	12.1.4.8.3 MIMO trans mis s io n mo de s

20

21	Spatial repetition code: One important simplification is that only one stream is transmitted and

22	received over all LEDs and PDs, respectively. This mode is useful, e.g., in order to create an omnidirectional

23	transmitter characteristics. This can be reached using the precoding vector vn = (1 1 1 1 …1)T.

24

25	Transmitter selection: In order to save energy, modulation may be switched off for some LEDs,

26	which results in zeros at the respective positions in the precoding vector vn.

27

28	Receiver selection: As only one stream is transmitted using multiple LEDs, maximum ratio com-

29	bining (MRC) is optimal. However, it requires an ADC at each receiver as well as multiple FFTs. Often, few

30	links in the MIMO channel have free LOS and a reduced path loss, accordingly, and for all modulation fre-

31	quencies. Hence, the channel matrix is "sparse". For reduced complexity, it may be sufficient to select the

32	strongest received signals and to combine them using equal gain combining (EGC). This can be realized

33	already in the analog domain so that fewer ADC are sufficient.

34

35	Combined transmitter and receiver selection: There can be a combination of transmitter and

36	receiver selection.

37

38	Transmitter and receiver selection for multiple streams: The above two schemes can even be com-

39	bined with multi-stream transmission as long as the number of streams Q is equal or smaller than the mini-

40	mum of the numbers of active transmitters and receivers. At the receiver side, the residual cross-talk is then

41	reduced by multi-stream processing.

42

43	WDM transmission: For WDM, because different colors are used, normally the number of streams

44	is the same as the number of transmitter ports. In this case, multiple streams are transmitted in parallel and

45	the precoding matrix on all subcarriers is given by Vn = 1n. Because color separation behind the receiver fil-

46	ters may be imperfect, MIMO reference symbols can be transmitted, and MIMO channel estimation and pro-

47	cessing can be performed in order to reduce the residual cross-talk and to increase the spectral efficiency.

48

49	WSK transmission: For WSK transmission, e.g. in case of an RGBY LED, the precoding vector vn

50	= (aR aG aB aY)T is used. If MIMO reference symbols are transmitted, imperfect color calibration at the

51	transmitter, which could also be falsified by reflecting surfaces, can be compensated by MIMO processing at

52	the receiver side.

53

54

350	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

12.1.5 Coordinated wireless network                                                                                                          1

2

The idea of the coordinated wireless network (COW) is to deploy multiple APs so that continuous coverage              3 is reached for mobile optical wireless user devices in a desired service area. There may be overlapping cov-              4 erage areas, where horizontal handover from one AP to another AP is needed and non-overlapping areas,              5 where vertical handover to another wireless technology is needed. In the following, the overlapping case is              6 described.                                                                                                                                                                          7

8

12.1.5.1 Link setup for COW transmission                                                                                                 9

10

Same as in the MIMO mode, initial link setup and header transmission detection are performed in the single-	11

input single-output (SISO) mode, in order to improve the reliability of transmission. Transmission of the	12

13

preamble and the header is done over all transmitters of all APs. Channel estimation symbols used by differ-	14

ent APs are made orthogonal to each other. Header detection can be improved by using maximum ratio com-	15

16

bining (MRC) based on individual estimates of the superimposed channels from all transmitters at each	17

receiver hi

n

  Tx    H

j 1 	ij

. A list of APs in the jointly served area is included in the header, together with the	18

19

assigned comb shift, see 4.2.6.2. As described below, the list can include spatial reuse of comb shifts for	20

multiple APs.	21

22

23

24

LED-specific	25

{

reference sequence

for noise estimation

frequency	26

reuse 1/7	27

28

29

30

31

32

33

34

35

36

37

38

39

carrier index

symbol index	40

Figure 182—Additional channel estimation symbols used in COW.	41

42

43

In Figure 182 the center figure shows additional channel estimation symbols used in the coordinated wire-	44 less network over multiple ACE symbols, each LED at a given access point transmits another sequence.	45

Subcarriers marked with the same color are assigned to the same access point. Subcarrier combs can be              46 reused after a certain distance. The assignment of ACE symbols can be defined and changed dynamically by              47 the network management, e.g. if a new AP is added to the network.                                                                               48

49

12.1.5.2 ACE symbols for COW transmission	50

51

In order to identify different APs in the COW topology and to maintain the possibility to use MIMO at each	52

AP, channel estimation symbols for different APs are made orthogonal in the frequency domain. The general	53

54

Copyright © 20xx IEEE. All rights reserved.	351

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	idea is that, during the entire channel estimation block, each AP is assigned another comb of subcarriers

2	(marked with the same specific color in Figure 182), from a set of orthogonal combs [11].

3

4	Using a comb of subcarriers only, instead of all subcarriers, is possible because only as many subcarriers are

5	needed for channel estimation as there are taps in the CP, in order to identify all multi-paths. Advanced and

6	computationally efficient algorithms are available in the literature in order to accurately interpolate the chan-

7	nel frequency response based on such subcarrier combs [12]. Note that the same comb can be reused by APs

8	after a certain distance, at which the signal is sufficiently attenuated.

9

10	The comb scheme can be extended to MIMO. Based on the comb symbols used for COW transmission, the

11	same rules are applied as for single-cell MIMO, see 4.2.5.7.

12

13	12.1.5.3 Cell- and user-specific ACE

14

15	Note that ACE signals are sent twice in the COW topology. In the first period, also denoted as cell-specific

16	AC (CS-ACE), the ACRE sequence is sent directly from all transmitters, so that the physical channel matrix

17	H is estimated on each subcarrier. This information is useful for joint transmitter optimization, after the AP

18	received the estimated CSI via feedback from its UDs.

19

20	In the second period, also denoted as user-specific ACE (US-ACE), the ACE sequence is passed though the

21	transmitter optimization, before being transmitted. Note that the joint transmitter optimization depends on

22	the channel of other UDs attached to other APs as well. Only by using US-ACE, the UD can estimate the

23	modified effective channel matrix Heffective and adapt its receiver processing, accordingly.

24

25	12.1.6 Relaying

26

27	Relaying operation supports two modes: 1) HD/DF: half duplex (HD) with decode and forward (DF) relay-

28	ing  mode and 2) FD/AF: full duplex (FD) with amplify and forward (AF) relaying mode sand full duplex

29	(DF) duplexing mode.

30

31	12.1.6.1 Full duplex (FD) Mode

32

33	In FD relaying mode, the REL simultaneously receives and transmits both in uplink and downlink support-

34	ing the AF mode.

35

36	12.1.6.2 Half duplex (HD) relaying

37

38	In HD relaying mode, the relay REL shall decode and store the data packets in order to retransmit them to

39	the REL in its transmission period supporting the DF mode. The REL can erase the stored packet after it

40	receives the ACK.

41

42	12.1.7 Payload data

43

44	In front of the OFDM modulator (and an eventual preprocessing to generate single-carrier and unipolar sig-

45	nals), payload data are scrambled and then fed into the forward error correction (FEC). Next they are

46	mapped onto constellation points. Finally there is an additional constellation scrambling operation.

47

48	12.1.8 Data scrambling

49

50	For data scrambling, a linear feedback shift register (LSFR) is used. It is defined by the number of registers

51	NLFSR. The position of feedback taps is commonly described via a generator polynomial and the initializa-

52	tion of the registers s1 to sNLFSR.

53

54

352	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

In Figure 183 the LFSR used for data scrambling is shown. It implements the generator polynomial	1

g(x)=x23+x18+1 and its register range from s1 to s23.	2

3

4

5

Data in	6

7

|  | MSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| s18 |  | s19 | s20 | s21 | s22 | s23 |

8

LSB	9

| s1 | s2 | s3 | s4 | s5 | s6 |
| --- | --- | --- | --- | --- | --- |

Data out	10

...	11

Figure 183—LFSR for data scrambling	12

13

14

All data starting from the first bit of the PHY-frame header (PFH) and ending by the last bit of the payload	15 shall be scrambled with a pseudorandom sequence generated by this LFSR.	16

17

The LFSR generator is initialized at the first bit of the header with the initialization vector equal to	18

0x2AAAAA in hexadecimal notation or 0010 1010 1010 1010 1010 1010 in binary notation where the LSB              19 corresponds to C1. This initialization is used for the scrambling of the header data. The first bit to be scram-              20 bled shall be XOR'ed with the first bit generated by the LFSR after initialization (i.e., C18 XOR C23 of the              21 initialization vector).                                                                                                                                                         22

23

The special value for SI (scrambler initialization) in the PHY-frame header of 016 in hexadecimal notation	24 indicates that the scrambler is not re-initialized between the header and the payload. An initialization of the	25

SI field to values other than the special value is optional, as described in the G.hn specification.	26

27

12.1.8.1 Forward-error correction	28

29

The channel coding for the header uses low-density parity-check codes (LDPC) according to the G.hn stan-              30 dard [B53]. A short LDPC code of K=168 data bits with code rate ½ is used. The resulting block is modu-              31 lated with QPSK with repetitions in frequency. If bandwidth is low and SNR shall be increased, the header              32 symbol can be repeated in time.                                                                                                                                        33

34

35

36

37

38

39

40

41

42

Figure 184—The low-density parity check code from the G.hn standard is used	43

44

The channel coding for the data uses LDPC according to the G.hn standard, see [4]. The LDPC code may be              45 selected between two information block sizes of 4320 or 920 bits bits and 5 code rates of 1/2, 2/3, 5/6, 16/18              46 or 20/21.                                                                                                                                                                            47

48

The channel coding for MIMO stream-interleaved encoding is used, consistent with the G.hn standard [B53].	49

50

12.1.8.2 Bit-to-Symbol Mapper	51

52

t.b.d., see G.9960-2015	53

54

Copyright © 20xx IEEE. All rights reserved.	353

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	editor note: suggest Volker to provide text in the comments resolution against D0. See comment 244 of

2	https://mentor.ieee.org/802.15/dcn/17/15-17-0297-02-0013-comment-against-7m-d1.xlsx

3

4	12.1.8.3 Constellation Scrambling

5

6	In Figure 185 the LFSR used for the constellation scrambler is shown. It implements the generator polyno-

7	mial g(x)=x13+x12+x11+x8+1 and its registers range from s1 to s13 being the least significant bit (LSB) and

8	most significant bits (MSB), respectively.

9

10

11

| LSB |  |  |  |  |  |  |  |  |  |  |  |  |  | MSB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s1 | s2 | s3 | s4 | s5 | s6 | s7 | s8 |  | s9 | s10 | s11 |  | s12 |  | s13 |

12

13

14

15

16

17	Figure 185—LFSR for constellation scrambling

18

19

20	The phase of the complex symbols generated by the bit-to-symbol mapper is shifted before being fed into

21	the IFFT. The phase shifts are determined by the pseudo-random bit-sequence generated by the LSFR

22

23	depicted in Figure 185. The two LSBs s1 and s2 determine the shift as given by Table 154. For a generated

24

25	phase shift φi, the LSFR is shifted by 2i. Due to the constellation scrambling, the phase of an originally gen-

i	i	i

26	erated complex symbol x0 is shifted by φ resulting in x , the actual input of the IFFT:

27

28

0

i

29	x

30

31

= xi exp  j i 

(42)

32	For the header, ACE and payload, the shift of the i-th subcarrier is φi and therefore the LSFR is shifted by 2i

33

34	Table 154—Phas e s hift value s fo r LFS R s tate s s 1 and s 2

35

| Register states of the LFSR |  | Phase shift |
| --- | --- | --- |
| s2 | s1 |  |
| 0 | 0 | 0 |
| 0 | 1 | π/2 |
| 1 | 0 | π |
| 1 | 1 | 3π/4 |

36

37

38

39

40

41

42

43

44

45

46

47

48

49	for each subcarrier. For these cases, the LSFR is initialized to the seed of 1FFFF16 in hexadecimal notation

50	for each OFDM symbol, where the LSB of the seed corresponds to s1.

51

52

53

54

354	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex A                                                                                                                      1

2

(informative)                                                                                                                                      3

4

5

Bibliography                                                                                                               6

7

8

A.1 Ge ne ral                                                                                                                                     9

10

11

Bibliographical references are resources that provide additional or helpful material but do not need to be              12

understood or used to implement this standard. Reference to these resources is made for informational use              13

only.                                                                                                                                                                                   14

15 [B1]  Berman,  S.  M.,  Greenhouse,  D.  S.,  Bailey,  I.  L.,  Clear,  R.  and  Raasch,  T.  W.,  “Human              16 electroretinogram responses to video displays, fluorescent lighting and other high frequency sources,”              17

Optometry and Vision Science 68:645–662, 1991.                                                                                                           18

[B2] CIE (1932) Commission internationale de l'Eclairage proceedings, 1931. Cambridge University Press,              19

Cambridge.                                                                                                                                                                        20

21

[B3] Colman, R. S., Frankel, F., Ritvo, E., and Freeman, B., “The effects of fluorescent and incandescent              22 illumination upon repetitive behaviors in autistic children,” J. Autism Childhood Schz. 6:157–162, 1976.                   23

24

[B4] Harding, G. F. A. and Jeavons, P., “Photosensitive Epilepsy,” Mac Keith Press, 1994.                                           25

26

[B5] IEEE 802.15 document 15-10-0151-01-0007, “High-power high-bandwidth linear driving circuit for              27

OWC  applications,” Baumgartner, Robert, et.  al.,  https://mentor.ieee.org/802.15/dcn/10/15-10-0151-01-             28

0007-high-power-high-bandwidth-linear-driving-circuit-for-OWC-applications.pdf.                                                    29

30 [B6] IEEE Std 802.11TM-2007, Information technology—Telecommunications and information exchange              31 between systems—Local and metropolitan area networks—Specific requirements—Part 11: Wireless LAN              32

Medium Access Control (MAC) and Physical Layer (PHY) Specifications.                                                                   33

34 [B7] IEEE Std 802.15.3TM-2003, IEEE Standard for Information technology—Telecommunications and              35 information exchange between systems—Local and metropolitan area networks—Specific requirements—              36

Part 15.3: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for High Rate              37

Wireless Personal Area Networks (OWPANs).                                                                                                                38

[B8] ISO/IEC 7498-1:1994, Information technology—Open Systems Interconnection—Basic Reference              39

Model: The Basic Model.8                                                                                                                                                                                    40

41

[B9] Kang, T. G., et. al., “White Paper: IEEE 802.15.7 OWC Regulations,” July 2010, IEEE802.15              42 contribution 15-10-0615-01-0007, https://mentor.ieee.org/802.15/dcn/10/15-10-0615-01-0007-white-paper-             43 ieee-802-15-7-OWC-regulations.pdf.                                                                                                                               44

45

[B10] Küller, R. and Laike, T., “The impact of flicker from fluorescent lighting on well-being, performance,              46 and physiological arousal,” Ergonomics 41(4): 433–447, 1998.                                                                                      47

48

[B11] Lindner, H. and Kropf, S., “Asthenopic complaints associated with fluorescent lamp illumination              49 (FLI): The role of individual disposition,” Lighting Res. Technol. Vol. 25:59–69, 1993.                                                50

51

8ISO  publications are  available from the  ISO  Central Secretariat, Case Postale 56, 1  rue  de Varembé, CH-1211, Genève 20,                 52

Switzerland/Suisse (http://www.iso.ch/). ISO publications are also available in the United States from the Sales Department, American                 53

National Standards Institute, 25 West 43rd Street, 4th Floor, New York, NY 10036, USA (http://www.ansi.org/).                                                      54

Copyright © 20xx IEEE. All rights reserved.	355

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	[B12] Schubert, E. Fred, “Light Emitting Diodes,” Cambridge University Press, 2003.

2

3	[B13] Stone, P. T., “Fluorescent lighting and health,” Lighting Res. Technol. 24:55–61, 1990.

4

5	[B14] Walewski, J. W., “Color stabilization for CSK by use of visibility frames,” January 2011, IEEE802.15

6	contribution	15-1-0262-05-0007,	https://mentor.ieee.org/802.15/dcn/11/15-10-0262-05-0007-color-

7	stabilization-for-csk-by-use-of-visibility-frames.pdf.

8	[B15] Wilkins, A. J., I. Nimmo-Smith, A. Slater, and L. Bedocs, “Fluorescent lighting headaches and eye-

9	strain,” Lighting Res. Technol. Vol. 21:11–18, 1989.

10

11	[B16]  Wright, P.  S.,  “An  overview of  harmonic and  flicker emission standards and  their  associated

12	measurements,” Power Eng. J. 15(2): 87–93, 2001.

13

14	[B17] Yokoi, A., et. al., “More description about CSK constellation,” March 2011, IEEE 802.15 contribution

15	15-11-0247-00-0007,	https://mentor.ieee.org/802.15/dcn/11/15-11-0247-00-0007-csk-constellation-in-all-

16	color-band-combinations.pdf.

17

18	[B18] H. Elgala and T.D.C. Little, "Reverse polarity optical OFDM (RPO-OFDM): dimming compatible

19	OFDM for gigabit OWC links", Optics Express, vol. 21, issue 20, pp. 24288 - 24299, 2013.

20

21	[B19] V. Jungnickel, T. Hindelang, T. Haustein, W. Zirwas, SC-FDMA Waveform Design, Performance,

22	Power Dynamics and Evolution to MIMO, Proc. IEEE Portable, March 2007.

23

24	[B20] V. Jungnickel, L. Grobe, Localized SC-FDMA with Constant Envelope, Proc. Int. Symp. Personal,

25	Indoor and Mobile Radio Systems (PIMRC), IEEE, London, UK, Sept. 2013, pp. 24-29.

26	[B21] Xingxing Huang, Siyuan Chen, Zhixin Wang, Jianyang Shi, Yiguang Wang, Jiangnan Xiao and Nan

27	Chi, "2.0-Gb/s visible light link based on adaptive bit allocation OFDM of a single phosphorescent white

28	LED," IEEE Photonics Journal 7(5), October 2015.

29

30	[B22] H. Elgala and T.D.C. Little, "Reverse polarity optical OFDM (RPO-OFDM): dimming compatible

31	OFDM for gigabit OWC links", Optics Express, vol. 21, issue 20, pp. 24288 - 24299, 2013.

32

33	[B23]  Mohammad  Noshad  and  Maite  Brandt-Pearce,  "Hadamard-coded  modulation  for  visible  light

34	communications", IEEE Transactions on Communications, Vol. 64, No.3, March 2016.

35

36	[B24]  Sridhar  Rajagopal,  Richard  D.  Roberts,  and  Sang-Kyu  Lim,  "IEEE  802.15.7  visible  light

37	communication: modulation schemes and dimming support", IEEE Communications Magazine, Vol.50,

38	No.3, March 2012.

39

40

41	A.2 Re g ulato ry do c ume nts

42

43	[B25] ANSI C82.77-2002, American National Standard for Harmonic Emission Limits—Related Power

44	Quality Requirements for Lighting Equipment.9

45

46	[B26] ANSI C82.SSL1 Operational Characteristics and Electrical Safety of SSL Power Supplies and

47	Drivers.

48

49	[B27] ANSI Z136.1, American National Standard for Safe Use of Lasers (2007).

50

51	[B28] ANSI Z136.6, American National Standard for Safe Use of Lasers Outdoors (2005).

52

53	9ANSI C82 publications are available from NEMA publications are available from the National Electrical Manufacturers Association,

54	1300 N. 17th St., Ste. 1847, Rosslyn, VA 22209, USA.

356	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

[B29] ARIB STD-T66, Second Generation Low Power Data Communication System/Wireless LAN System	1

1999.12.14 (H11.12.14) Version 1.0.10	2

3 [B30] BS EN 29241-3:1993, Ergonomic requirements for office work with visual display terminals (VDTs).              4

Visual display requirements.	5

6 [B31] EN61000-3-3: Electronic compatibility (EMC) Part 3: Limits—Section 3: Limitation of voltage             7 fluctuations and flicker in low voltage supply systems for equipment with rated current 16 A and smaller.              8

International Electrotechnical Commission, 1994.	9

10 [B32] ERC Recommendation 70-03, Relating to the Use of Short Range Devices (SRDs), April 2002.11                          11

12 [B33] IEC 1000-3-3 (1994), Part 3: Limits—Section 3: Limitation of voltage fluctuations and flicker in low-              13 voltage supply systems for equipment with rated current 16 A.                                                                                       14

15 [B34] IEC 1000-3-5: Electromagnetic compatibility. Part 3: Limits—Section 5: Limitation of voltage             16 fluctuations and flicker in low-voltage power supply systems for equipment with rated current greater than              17

16 A. 1994.	18

19 [B35] IEC 60050-112 ed1.0 (2010-01) TC/SC 1—International Electrotechnical Vocabulary—Part 112:              20

Quantities and units.	21

22 [B36] IEC 60050-300 ed1.0 (2001-07), International Electrotechnical Vocabulary—Electrical and electronic              23 measurements and measuring instruments—Part 311: General terms relating to measurements—Part 312:              24

General terms relating to electrical measurements—Part 313: Types of electrical measuring instruments—	25

Part 314: Specific terms according to the type of instrument.	26

27 [B37] IEC 60050-845 ed1.0 (1987-12), International Electrotechnical Vocabulary, Lighting.                                        28

29 [B38] IEC 60825-1 ed2.0 (2007-03), Safety of laser products—Part 1: Equipment classification and             30 requirements.                                                                                                                                                                     31

32 [B39] IEC 60825-12 ed1.0 (2004-02), Safety of laser products—Part 12: Safety of free space optical             33 communication systems used for transmission of information.                                                                                        34

35 [B40] IEC 61347-1 ed2.0 (2007-01), Lamp control gear—Part 1: General and safety requirements.                             36

37 [B41] IEC 61347-2-13 ed1.0 (2006-05), Lamp control gear—Part 2-13: Particular requirements for d.c. or              38 a.c. supplied electronic control gear for LED modules                                                                                                     39

40 [B42] IEC 62384-am1 ed1.0 (2009-07) Amendment 1—DC or AC supplied electronic control gear for LED              41 modules—Performance requirements.                                                                                                                              42

43 [B43] IEC 62471 ed1.0 (2006-07), Photobiological safety of lamps and lamp systems.                                                 44

45 [B44] IEC/TR 62471-2 ed1.0 (2009-08), Photobiological safety of lamps and lamp systems—Part 2:              46

Guidance on manufacturing requirements relating to non-laser optical radiation safety.	47

48

49

50

51

10ARIB publications are available from the Association of Radio Industries and Businesses (http://www.arib.or.jp).	52

11ERC publications are available from Global Engineering Documents, 15 Inverness Way East, Englewood, Colorado 80112, USA	53

(http://global.ihs.com/).	54

Copyright © 20xx IEEE. All rights reserved.	357

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	[B45] IEC/TS 61000-3-5 Ed. 2.0 b Cor.2:2010 Corrigendum 2—Electromagnetic compatibility (EMC)—

2	Part 3-5: Limits—Limitation of voltage fluctuations and flicker in low-voltage power supply systems for

3	equipment with rated current greater than 75 A.

4

5	[B46] ANSI/IESNA RP-16-05, Nomenclature and Definitions for Illuminating Engineering.12

6

7	[B47]  ANSI/IESNA  RP-27.1-05,  Photobiological  Safety  for  Lamps  and  Lamp  Systems—General

8	Requirements.

9	[B48] IESNA LM-79-08, IES Approved Method for the Electrical and Photometric Measurements of Solid-

10	State Lighting Products.

11

12	[B49] IESNA LM-80-08, IES Approved Method: Measuring Lumen Maintenance of LED Light Sources.

13

14	[B50] IESNA TM-16-05, Technical Memorandum on Light Emitting Diode (LED) Sources and Systems.

15

16	[B51]  Recommendation  ITU-R  RA.1630,  Technical  and  operational  characteristics  of  ground-based

17	astronomy systems for use in sharing studies with active services between 10 THz and 1000 THz.

18

19	[B52]  Recommendation  ITU-R  RS.1744,  Technical  and  operational  characteristics  of  ground-based

20	meteorological aids systems operating in the frequency range 272–750 THz.13

21

22	[B53]  ITU-T  Recommendation G.9960, Unified  high-speed wireline-based home  networking

23	transceivers – System architecturec and physical layer specification, Nov. 2015.

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51	12IESNA is the Illuminating Engineering Society Standards from ANSI. IESNA publications are available from the Sales Department,

52	American National Standards Institute, 25 West 43rd Street, 4th Floor, New York, NY 10036, USA (http://www.ansi.org/).

53	13This recommendation replaces ITU-R SA.1744. ITU-T publications are available from the International Telecommunications Union,

54	Place des Nations, CH-1211, Geneva 20, Switzerland/Suisse (http://www.itu.int/).

358	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

34

35

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddr | IEEE address | Any valid IEEE address | The individual IEEE address of the entity from which the MSDU is being transferred. |
| DstAddr | IEEE address | Any valid IEEE address | The individual IEEE address of the entity to which the MSDU is being transferred. |
| RoutingInformation | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |
| data | Set of octets | — | The set of octets forming the MSDU to be transmitted by the MAC sublayer entity. |
| priority | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |
| ServiceClass | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	359

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

5

6

19

20

39

40

41

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddr | IEEE address | Any valid IEEE address | The individual IEEE address of the entity from which the MSDU has been received. |
| DstAddr | IEEE address | Any valid IEEE address | The individual IEEE address of the entity to which the MSDU is being transferred. |
| RoutingInformation | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |

42

43

44

45

46

47

48

49

50

51

52

53

54

360	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

|  |  | 1 |
| --- | --- | --- |
|  |  | 2 |
|  |  | 3 |
|  |  | 4 |
|  |  | 5 |
|  |  | 6 |
|  |  | 7 |
|  |  | 8 |
|  |  | 9 |
|  |  | 10 |
|  |  | 11 |
|  |  | 12 |
|  |  | 13 |
|  |  | 14 |
|  |  | 15 |

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| data | Set of octets | — | The set of octets forming the MSDU received by the MAC sublayer entity. |
| ReceptionStatus | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |
| priority | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |
| ServiceClass | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |

45

46

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	361

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SrcAddr | IEEE address | Any valid IEEE address | The individual IEEE address of the entity from which the MSDU has been transferred. |
| DstAddr | IEEE address | Any valid IEEE address | The individual IEEE address of the entity to which the MSDU has been transferred. |
| status | Enumeration | SUCCESS, TRANSMIS- SION_PENDING, NO_BEACON, or CHANNEL_ACCESS_FAIL- URE | The status of the last MSDU transmission. |
| ProvPriority | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |
| ProvServiceClass | — | null | This parameter is not used by the MAC sublayer and shall be specified as a null value. |

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

362	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex C	1

2

(normative)	3

4

5

Cyclic redundancy check	6

7

The CRC field is 2 octets in length. The CRC shall be calculated using the following standard generator	8 polynomial of degree 16:	9

10

G16  x  =

x16

+ x12

+ x5 + 1

11 (C.1)	12

13

The CRC shall be calculated for transmission using the following algorithm:	14

—	Let M x

= b0 x

k – 1

+ b1 x

k – 2

15

+  + bk – 2 x + bk – 1  be the polynomial representing the sequence of	16

bits for which the checksum is to be computed.	17

—	Multiply M(x) by x16, giving the polynomial x16  M x .	18

—	Divide

x16

 M x modulo  2  by  the  generator  polynomial,  G16(x),  to  obtain  the  remainder	19

15

14

polynomial, R x

= r0 x

+ r1 x

+  + r14

x + r

15.	20

21

—	The CRC field is given by the coefficients of the remainder polynomial, R(x).	22

23

Here, binary polynomials are represented as bit strings, in highest polynomial degree first order.	24

25

A typical implementation is depicted in Figure C.1.	26

27

28

CRC-16 Generator Polynomial: G(x) = x16 + x12 + x5 + 1	29

30

31

32

Input	33

Data Field	34

35

36

37

38

1. Initialize the remainder register (r0 through r15) to all ones.	39

2. Shift the data into the divider in the order of transmission (LSB).	40

3. After the last bit of the data field is shifted into the divider,	41

the remainder register contains the CRC.	42

43

4. The CRC is appended to the data so that r0 is transmitted first.	44

45

Fig ure C.1—Typical CRC implementation	46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	363

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Annex D

2

3	(informative)

4

5

6	Considerations for OWC using LED displays

7

8

9	D.1 Intro duc tio n—Dynamic dis plays vs . addre s s e d dis plays

10

11	This annex discusses two types of LED involved displays: dynamic displays and addressed displays. The

12	fundamental different between the two has to do with the amount of time the pixel is illuminated. In dynamic

13	displays the pixels on a line are illuminated once every frame for T_frame/N_line seconds where N_line is

14	the number of lines controlled by a line scan controller and T_frame is the time to sweep N_line once; that

15	is, the pixels on a line are operated dynamically so that they are only on for a fraction of the total frame time

16	(hence the name “dynamic”). In the addressed display, the pixel is illuminated for the duration of the frame

17	and is readdressed once per frame for possible state change.

18

19

21

20	D.2 Dynamic dis plays

22

23	D.2.1 Ope ratio n me c hanis m

24

25	In general, a dynamic display consists of a host controller, a line scan controller, a display data buffer, and

26	LED matrix, as shown in Figure E.1. The line scan controller selects a line for display, and the display data

27	buffer transmits state information, such as the on/off state or the color selection, to each LED pixel on the

28	selected line. The line scan controller determines the active time of each line, where the active time indicates

29	whether the LED pixels on the selected line are switched ON or OFF as per the display data buffer for the

30	active time of the selected line. Therefore, OWC using a dynamic display is tightly coupled with the active

31	time.

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50	Fig ure E.1—Ge ne ral arc hite c ture o f LED s ig nbo ard o pe rate d by the dynamic

51	dis play me c hanis m

52

53

54

364	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

It is well known that most of PC monitors and TV display images use 60 displayed-frames per second. In the	1 case of PC monitors or TVs, the total displayed-frame number (i.e., different display information) is actually	2

30 frames per second because each displayed-frame is transmitted twice. The display mechanism of a              3 dynamic display is similar to a PC monitor or TV. Assuming that a dynamic display consists of 16x16 lines              4 and displays images or text through 60 displayed-frames per second, with 16 lines per displayed-frame, the              5 active time slot period for OWC assigned to each line is 1.042 ms per displayed-frame, as shown in              6

Figure E.2. Therefore, an active time slot can transmit 130 byte at 1 Mbit/s, as shown in Figure E.3.	7

8

9

1 display frame time = 1/60 sec 	10

11

12

line 1 	13

14

15

16

line 2 	17

•

18

•

•

•

line 16

1.042ms

• 	19

• 	20

21

22

23

24

active time per display -frame per line = (1/60)/16 sec 	25

26

Fig ure E.2—Ope ratio n me c hanis m o f dynamic dis play	27

28

29

30

line 1 line 2

•

slot 1

slot 2

slot 1

31

32

33

slot 2	34

35

36

37

•

•

•

•

line 16

header	data	130 bytes	38

39

•	40

•	41

•	42

slot 16	43

44

45

Figure E.3—An example of OWC data trans mis s ion on a dynamic dis play	46

47

48

D.2.2 Re duc e d brig htne s s mitig atio n o n OWC dynamic dis plays	49

50

The OWC dynamic display may be less bright than the non-OWC dynamic display because of the OWC              51 modulation during the active time periods. Therefore, it is important to minimize the reduction of the              52 average brightness of the dynamic display during the shortest time period that the human can distinguish.              53

54

Copyright © 20xx IEEE. All rights reserved.	365

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	This is done to keep the display performing adequately for its intended function. Figure E.4 shows an

2	example of an operational mechanism to mitigate the average brightness reduction per display frame time

3	that can arise for a dynamic display having 16x16 lines operating as shown in Figure E.2 (i.e., the OWC data

4	is carried during the active time of each line). The average brightness per display frame time in Figure E.4 is

5	twice as large as that in Figure E.2 because the active time is twice as long.

6

7

8	1 frame time = 1/60 sec

9

10	line 1

11

12	line 2

13

14

slot 1

slot 2

• • •

slot 1

slot 2

15

16

17

18	line 8

19

header 	data 	260 bytes

•

•

•

slot 8

20	line 9

21

22	line 10

23

24

slot 1

slot 2

• • •

slot 1

slot 2

25

26

27

28	line 16

29

30

31

header 	data 	260 bytes

•

•

•

slot 8

active time per display -frame per line = (1/60)/8 sec

32	Fig ure E.4—An e xample fo r the mitig atio n o f the ave rag e brig htne s s

33	re duc tio n o n the OWC dynamic dis play

34

35

36	D.2.3 OWC applic atio n us ing dynamic dis plays

37

38	A OWC enabled dynamic display can be used in the broadcast topology. The OWC broadcast topology in

39	this standard consists of mainly the beacon and the downlink, as shown in Figure 12. Therefore, the OWC

40	broadcast topology using a dynamic display can be constructed by the assignment of the active time slots

41	and the use of GTS field in the beacon frame. Figure E.5 shows the OWC broadcast topology construction

42	using the dynamic display. The active time slot #1 is assigned to the beacon and the active time slots from #2

43	to #8 are assigned to the downlink in Figure E.5. The GTS fields of the beacon frame can be used to indicate

44	the GTS number, GTS length, and GTS direction for the broadcast topology. Multiple GTS slots can also be

45	used depending upon the desired service level, the subscriber's grade, and the QoS policy.

46

47

48

49

50

51

52

53

54

366	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

CFP

3

line 1 line 2

•

•

•

•

•

line 9 line 10

•

•

•

line 16

beacon

beacon

GTS #1

GTS #1

GTS #2

•

•

•

•

•

•

GTS #7

GTS #7

beacon

beacon

GTS #1

GTS #1

5

6

7

8

9

GTS #2

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

Fig ure E.5—OWC bro adc as t to po lo g y c o ns truc tio n us ing the dynam ic dis play	25

26

27

28

D.3 Addre s s e d dis plays	29

30

In an addressed display a particular pixel is addressed (i.e., refreshed) once every frame; that is, once              31 addressed the pixel maintains its current state until readdressed during the next frame. The role of the LED              32 can be either to provide pixel back lighting, as in the case of an LCD (liquid crystal display), or the pixel can              33 be formed directly from a LED device as in the case of LED signage (which would typically be implemented              34 as a pixel constructed from a compound RGB LED). Generally the addressed pixels in a frame are serially              35 updated and there is no need for a retrace blanking interval as in the old days of cathode ray tubes.                             36

37

38

D.3.1 LCD dis play us ing LED bac klig hting mo dulatio n	39

40

In LED backlighting, there are numerous LEDs that provide illuminance for all LCD pixels, while the	41

intensity of a particular pixel is determined by the transmittance of the pixel LCD. This means the data	42

modulation of the LED backlighting is going to radiate from all the pixels. The radiation is a density (mW/	43

unit area) and the best performance occurs when the sensor views the whole screen (i.e., ingests the most	44

power).  Viewing  the  whole  screen  also  provides  intensity  averaging  over  all  the  pixels,  which  is	45

advantageous since in the some scenes various pixels might go dark. Given enough area averaging, there no	46

particular relationship between the data rate and the frame refresh rate.	47

48

D.3.2 LED pixe l mo dulatio n	49

50

One has the option of either modulating individual pixels or groups of pixels. In other words, one can              51 generate at least two display sections that can transmit different data. One could even go further and create              52 intermediate sections that transmit an aggregate of the data transmitted in the adjacent sections. For this to              53

54

Copyright © 20xx IEEE. All rights reserved.	367

IEEE  IIRELESS  MAC AND PHY SPECIFICATIONS  FOR 0IC 	IEEE  raft P802.15.1311

work  one needs  a detector  capable of spatially resolving such sections. Partitioning of the display  is done by

2 	using  the cell  mechanism and the PHY  SAPwhich were  introduced for mobility support (see 5. 1.12).  For

3 	examplethe cell  partitioning in  Figure  34  can  be  interpreted as  a  2x2  display  with  four  sections if the

4 	transrnltters	e operated in the broadcast mode.  Alsowavelength division multiplexing of RGB  pixels  can

5 	be enabled with this mechanism so at each color  carries  a different data s 	earn. Averaging over an area of

6 	the sign allows  sorne insensitivity to scene-to-scene pixel intensity variation

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

368 	Copyright I(J 20xx IEEE.All rights reserved

This is an unapproved IEEE  Standards Draftsubject to change

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex E	1

2

(informative)	3

4

5

Receiver performance variation on multi-color channels	6

7

Many applications using colored light sources can be considered in OWC. For example, Figure F.1 describes	8 the scenario that a OWC receiver receives some information from a traffic signal light sources with color	9 “A” and color “B”. Figure F.2 describes that an user with a OWC receiver can get the audio information	10 from a color “A” lamp, the video information from a color “B” lamp, or the navigation information from a	11 color  “C”  lamp.  Figure F.3  shows  that  the  multiplexing  technologies  such  as  Wavelength  Division	12

Multiplexing (WDM) can be applied to OWC applications using colorful light sources.	13

14

OWC services, using multiple color channels according to the OWC band plan, should be attained by only              15 one OWC receiver. It is undesirable if a OWC receiver exhibits better receiver performance only on, for              16 example, the color “A” channel but it does not exhibit the same performance on the color “B” or color “C”              17 channel as on the color “A” channel. Therefore, a uniform performance on each color channel may be              18 desired.                                                                                                                                                                              19

20

There are two main factors influencing the performance variation of a multi-color OWC receiver. One is the              21 conversion relations between the radiometric and photometric units. The other is the photo sensitivity             22 characteristics of a photo-detector, such as a Si photo-detector that depends on the wavelength variation,              23 assuming such photo-detectors will be used as a receiver in OWC.                                                                                 24

25

First, suppose that there are two light sources or  OWC transmitters with red color and green color              26 respectively and a OWC receiver to perceive the variation of received powers under the multiple color              27 channels which originates from the conversion relations between the radiometric and photometric units.              28

Also suppose that each color light radiates from two light sources at the same divergence angle, and they              29 radiate with the same luminous flux, (lumens), so that the human eye senses the same brightness when              30 simultaneously viewing the two light sources respectively at the same distance. However, when each color              31 light radiates with the same luminous flux then the radiation of the two light sources in radiometric power              32 (Watts) are each different, which is the origin of the CIE sensitivity curves. The CIE sensitivity curves              33 indicate what the human eye senses, and it turns out that the green light is perceived as being brighter than              34 red light when the radiometric radiation powers of the two light sources are equal. Therefore, the radiometric              35 received powers of a receiver are different on red and green channels, respectively, even though the              36 divergence angles and luminous fluxes of two light sources are equal, the same receiver is used, and the              37 distances between the receiver and the light source are equal.                                                                                         38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	369

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22	Fig ure F.1—OWC applic atio n us ing traffic s ig nal lig ht s o urc e s

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51	Fig ure F.2—OWC applic atio n us ing c o lo re d lig ht s o urc e s

52

53

54

370	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

Figure F.3—OWC applic atio n us ing WDM techno logy	23

24

25

26

Of course, the OWC light source power can be also defined in radiometric unit, Watt. However, it is more	27

reasonable that the light source power is defined in photometric unit because the light sources will be used	28

not only for a OWC transmitter but also for illumination or visual display related to human eye.	29

30

Table F.1 describes the receiver input powers, calculated in Watts, from the assumption of 1 lumen on each	31

of the seven color bands (given in Table 108). The assumption that the lights have only monochromatic	32

component (shown as example wavelength in Table F.1) on each color band is also used for simple	33 calculations. V() is the human eye sensitivity function, which indicates CIE sensitivity curves (Schubert	34

[B12]).	35

36

Table F.1—Calc ulate d c o lo r c hanne l po we r at re c e ive r	37

38

| Wavelength band (nm) |  | Spectral width (nm) | Example wavelength (nm) | V() at example wavelength | Receiver input power (Watts) @ 1m |
| --- | --- | --- | --- | --- | --- |
| 380 | 478 | 98 | 430 | 0.0273 | 0.0536 |
| 478 | 540 | 62 | 510 | 0.5030 | 0.0029 |
| 540 | 588 | 48 | 565 | 0.9788 | 0.0015 |
| 588 | 633 | 45 | 610 | 0.5030 | 0.0029 |
| 633 | 679 | 46 | 655 | 0.0817 | 0.0179 |
| 679 | 726 | 47 | 700 | 0.0041 | 0.3571 |
| 726 | 780 | 54 | 750 | 0.0001 | 14.6413 |

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	371

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	The second factor causing the performance variation of a OWC receiver across multiple color channels is

2	the photo sensitivity characteristics of optical receivers, such as Si photo-detectors, which is wavelength

3	dependent. Figure F.4 shows the photo sensitivity characteristics of a Si photo-detector according to the

4	wavelength variation. It has been known that the photo sensitivity value of Si photo-detector is higher on

5	longer wavelength than on shorter wavelength in the visible band as shown in Figure F.4. Figure F.4 shows

6	that a Si photo-detector produce more electrical current on red color channel than on green or blue color

7	channel even though the radiometric received powers on each color channel are equal.

8

9	Table F.2 shows the photo-detector output current obtained from both the wavelength dependence of photo

10	sensitivity shown in Figure F.4 and the conversion relations between the radiometric and photometric units

11	described in Table F.1. The photo-detector output currents in Table F.2 were calculated only at 430 nm,

12	510 nm, and 655 nm among the example wavelengths in seven color bands, as shown in Table F.1, for

13	convenience.

14

15	Table F.2 indicates that a OWC receiver with Si photo-detector performs differently on multiple color

16	channels even though the radiometric received powers are equal on each color channel. Therefore, two main

17	factors, the unit conversion and the photo sensitivity of a photo-detector depending on wavelength, need to

18	be sufficiently considered in order that the performance of a OWC receiver can be maintained uniformly on

19	multiple color channels.

20

21	0.8

22

23	0.7

24

25	0.6

photo sensitivity (A/W)

26

27	0.5

28	(0.46) 	Red

29	0.4

30

31	0.3

32

33	0.2

34

35	0.1

36

37

0.0

38

(0.3) 	Green

(0.21)

Blue

430 	510 	655

39	300 	400 	500 	600 	700 	800 	900 	1000

40	wavelength (nm )

41	Fig ure F.4—Typic al S i pho to -de te c to r wave le ng th s e ns itivity

42

43

44

45	Table F.2—P ho to -de te c tor current from Figure F.4 with conditio ns of Table F.1

46

| Example wavelength (nm) | Receiver input power (Watt) @ 1 m | Photo sensitivity (A/W) | Photo-detector output current (mA) @ 1 lm |
| --- | --- | --- | --- |
| 430 | 0.0536 | 0.21 | 11.26 |
| 510 | 0.0029 | 0.30 | 0.87 |
| 655 | 0.0179 | 0.46 | 8.23 |

47

48

49

50

51

52

53

54

372	Copyright © 20xx IEEE. All rights reserved.

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D0

Annex F	1

2

(informative)	3

4

5

Notes on the receiver algorithm	6

7

In order to demodulate the data encoded in the first stream (St1), the samples in every even OFDM symbol	8 interval are subtracted from the samples in every odd OFDM symbol interval. For example, the samples in	9 the second symbol are subtracted from the samples in the first symbol, the samples in the fourth symbol are	10 subtracted from the samples in the third symbol, etc. Note that due to the structure of the other data streams	11 (St2 up to St4), the subtraction operation completely removes interference from these streams.	12

13

After the subtraction operation, the resulting data at St1 can be demodulated using the conventional OFDM              14 demodulator algorithm. After the data in St1 are demodulated, the stream is re-modulated again and sub-              15 tracted from the overall received signal.                                                                                                                           16

17

Following the subtraction of the re-modulated St1 signal, all frame copies at St2 which were originally iden-              18 tical (the symbols in question are the symbols that have identical polarity) should be summed up. For exam-              19 ple, the first and the second symbols should be summed, the third and the fourth symbols should be summed,              20 the fifth and the sixth symbols frames should be summed, etc. The resulting signal at St2, then, has the same              21 format as the signal at St1. Hence, it can be demodulated after each even symbol is subtracted from each odd              22 frame. Upon demodulation, the resulting bits can be re-modulated and the resulting waveform can be sub-              23 tracted from the overall received waveform as in the case for St1. The procedure is re-iterated for the              24 remaining streams up to St4 (depending on the configuration) until all data bits are recovered.                                    25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

Copyright © 20xx IEEE. All rights reserved.	373

IEEE WIRELESS MAC AND PHY SPECIFICATIONS FOR OWC	IEEE Draft P802.15.13/D1

1	Annex G

2

3	(informative)

4

5

6	Color function support

7

8	Color function is a function that provides information, such as device status and channel quality, to the

9	human eye via color. In IEEE Std 802.15.7TM-2011, color function is specified. The CVD frame, using

10	various colors, can be used to display various statuses of a device. The colors mapped for each status of the

11	devices are based on the phyColorFunction (see IEEE Std 802.15.7TM-2011). The colors chosen for different

12	statuses are  left to  the  discretion of  the implementer. Multiple statuses may choose the  same color,

13

14	depending on the number of colors supported by the device. The use of color function through the CVD

15	frame has the potential to change the color of the emitted light.

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

374	Copyright © 20xx IEEE. All rights reserved.