IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG 802.15 minutes for January, 2018 Interim meeting of SG LPWA |  |
| Date Submitted | [January 17th, 2018] |  |
| Source | [Charlie Perkins] [Futurewei] | Voice: [+1-408-330-4586] Fax: [ ] E-mail: [charliep@computer.org] |
| Re: | [TG 802.15 SG LPWA meeting minutes] |  |
| Abstract | TG 802.15 SG LPWA minutes from the Irvine IEEE 802 Wireless Interim Meeting |  |
| Purpose | [Record the transactions of the TG 802.15 SG LPWA Interim meeting during January, 2018] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Minutes for IEEE TG 802.15 SG LPWA Irvine Interim Meeting

15-18 January, 2018

Chair – Jörg Robert

Minutes – Charles Perkins

Objectives (all meetings in room Woodbridge):

- Approval of Orlando Minutes

- Contribution

- Discussion of PAR / CSD

- Hardware demonstration

Agenda Review:

- Monday PM1

  - Open

  - IEEE-SA Stds. Board Bylaws on Patents in Std's. & Guidelines

  - Approval of the Agenda

  - Approval of Orlando Minutes

  - Contributions

  - Recess

- Tuesday PM1

  - Open

  - Hardware Demonstration

  - Discussion of PAR/CSD

  - Recess

- Wednesday PM1

  - Open

  - Discussion of PAR/CSD

  - Recess

- Thursday PM1

  - Open

  - Discussion of PAR/CSD

  - Discussion of future schedule

  - AoB

  - Adjourn SG LPWA

Monday PM1 (January 15) – Agenda / Timeline review / Contributions

- Open at 1:35pm

- Attendance: 21

Patent Policy and call for Essential Patents

- Chair presents IEEE slides #1 to #4 of the IEEE patent and meeting conduct slides. Chair provides an opportunity for disclosure.  None was heard.

Agenda Minutes Approved

- 15-18-0025-00-lpwa-sg-lpwa-agenda-january-2018-interim.pptx.

- Move to accept: Bob Heile.  Seconded.  No objection, so approved.

Minutes from Orlando Approved

- 15-17-0653-01-lpwa-tg-802-15-minutes-for-november-2017-plenary-meeting-of-ig-lpwa

- Move to accept: Bob Heile.  Seconded Charlie Perkins.  No objection, so approved.

Proposed Timeline

- January 2017 Interim (Irvine)

  - Discussion on PAR/CSD

- March 2017 Plenary (Chicago)

  - Start working on TG issues if PAR/CSD agreed by the group.

Latest News from ETSI LTN

- Standard document is almost complete (only editorial things)

  - Official publication expected mid. 2018

Contribution: Time-Split Multiple-Access

- https://mentor.ieee.org/802.15/dcn/18/15-18-0016-00-lpwa-time-split-multiple-access.ppt

- Presented by Johannes Wechsler

- Requirements

- Results

- Very narrow band, but occupied for a long time

  - Challenge: Interference

- Time-Split Multiple-Access to reduce frame loss

  - Question about dynamics with hundreds of devices

  - Results depend on channel load.  If channel is heavily loaded, no gains available.

- Data processing functional diagrams for transmitter and receiver

- Device implementation

  - software solution combinable with existing MAC & PHY

  - time gaps allow energy harvesting

  - requires a timer and a TSMA lookup table

  - software not hardware or protocol dependent – works on various controllers

- Additional latency required because of time gaps

- Question about how many nodes are envisioned to use the technology.

  - Answer: 1.0 – 1.5 million messages per day.

- Simulations show that technology scales to about .7 available bandwidth

- Summary: TSMA is simple, effective

Contribution: PAR and CSD document discussion

- https://mentor.ieee.org/802.15/dcn/18/15-18-0024-00-lpwa-par-and-csd-document-discussion.pptx

- Presented by Jörg Robert (FAU Erlangen-Nuernberg)

- Motivation: definition of PAR and CSD

- Goal of New Amendment: use-cases with very low payload bit-rates

- All modifications to be supported by current IEEE 802.15.4 chipsets with updated SW

- Required Amendments for Powerful LPWA Functionality

  - FCC compliant ultra-low bit-rate modes based on 802.15.4k FSK PHY

  - Fragmented transmission of FEC code-words

  - Fragmentation mechanisms and channel hopping patterns

  - Convolutional codes with lower rates?

- Proposed Scope

  - Focus on low bit-rates

  - Definition of mechanism for fragmented transmission of FEC code-words

  - Definition of time / frequency patterns for the transmission of the fragments

  - Minor adjustments of  the MAC required for the efficient transmission of the fragments

  - No definition of new modulation schemes wrt. the 802.15.4k FSK PHY

  - Definition of additional convolutional codes based on the K=7 R=1/2

- Proposed PAR Text for “Need”

- CSD Document: SG LPWA has to agree on “Distinct Identity” and “Broad Market Potential”

- Question: this proposal moves channel hopping to the PHY?  Can we harmonize so that MAC does this?

- Proposing to add a new convolutional code and code rate, not change existing ones, if can show performance gains on the order of several dB <significant Eb/N0 gain>

- Would the preamble change?

- Is there a lower bound on the data rates under consideration?  [Why place a bound?]

- Chipset manufacturers need to know so that they can better support low-low data rates.

- Current spec allows synchronization from 4-64 byte preambles

- Proposed PAR Text for “Identity”

  - Do not need to restrict to license-exempt

- Proposed Text for “Broad sets of applicability”

  - Add low-throughput, etc.

- Proposed Text for “Multiple vendors and numerous users”

  - Allow existing devices to work as well.

Recess at 3:11pm

Tuesday PM1 (January 16) – Hardware Demonstration

- Open at 1:35pm

- Attendance: 15

Patent Policy and call for Essential Patents

- Chair presents IEEE slides #1 to #4 of the IEEE patent and meeting conduct slides. Chair provides an opportunity for disclosure.  None was heard.

. Agenda:

- Open

- Hardware Demonstration

- Discussion of PAR/CSD <delay until Wednesday to focus on demonstration>

- Recess.

Hardware Demonstration

- Presented by Johannes Wechsler (Fraunhofer)

- Exhibit real-time Spectrograph

- Shows message reception with link budget 154dB

- One transmitter (0x4b) on 6th floor but some messages still are received

- One transmitter (0x47) nearby -- most messages are received

- MIOTY M300 Base Station

- MSK with 1100 bits/sec.  DMSK is used to be spectrally efficient.

- 16 byte payload becomes 48 bytes because of coding.

- MSK noise requirement is 10dB

- 1/3 convolutional coding

- New prototype uses higher data rate.

- Use 30 sub-packets now, new data rate uses 24 sub-packets

- Very low throughput  synchronization issues with sub-packet.  Not every sub-packet is detected individually, saving some number of pilot symbols.

- Transmitting more pilot symbols increases length of occupancy of medium, thus increasing chance of interference.

- Have simulations with thousands of nodes, but not actual deployment

- 100 KHz band total bandwidth, 24 channels. 1.5 million messages per day [according to simulation].  With better equipment, could scale higher.

- In U.S. could do more.

- If two nodes arrive at the same time and use the same sequence, will end up with total overlap and both packets will be lost.

- In this example, use 24 frequencies.  Each sub-packet has its own frequency.

- Shut down the radio between transmission of sub-packets to save power.

- USB power bank will not see high-enough power consumption and shut down power to the device.

- Implementations are not limited to the specific chipset, have made various generations.

- Receiver is always-on software-defined radio.

- Beacon-mode is referred to as “synchronous” since the timeframe is then known.

- Lower-power devices transmit at whatever time is convenient, and then open for reception at a particular time afterwards.

Recess at 2:03pm

Wednesday PM1 (January 17) – Discussion and Finalization of PAR/CSD

- Open at 1:38pm

- Attendance: 17

Patent Policy and call for Essential Patents

- Chair presents IEEE slides #1 to #4 of the IEEE patent and meeting conduct slides. Chair provides an opportunity for disclosure.  None was heard.

Agenda:

- Open

- Discussion of PAR/CSD

- SG 802.15.4w Motion #1

- Adjourn

Discussion: PAR document contribution

- https://mentor.ieee.org/802.15/dcn/18/15-18-0050-00-0000-802-15-4w-par-draft.pdf

- Presented by Jörg Robert (FAU Erlangen-Nuernberg)

- Suggestion to leave out convolutional code specifics, and mention that the convolutional codes are as defined in the current standard

- Discussion about other LECIM FSK modifications are to be allowable; still want to restrict to FSK and not use CDMA, etc.

- Will upload modified PAR

Discussion: CSD document contribution

- 15-18-0053-00-lpwa-csd-for-802-15-4w-lpwan-phy.docx

- Minor editorial updates as also suggested for PAR wording

- Will upload modified CSD

SG 802.15.4w Motion #1:

- Moved: Pat Kinney

- Seconded: Henk Ruijter

- No discussion or objection, so motion is approved

Proposed TG4w Draft Schedule:

- See 15-18-0025-01-lpwa-sg-lpwa-agenda-january-2018-interim

- Discussion about whether SG or TG should collect proposals (leave for TG)

- No objection to schedule otherwise

AoB

- None suggested

Since all Study Group business has concluded, the Thursday PM1 session is cancelled.

Adjourned at 2:21pm