IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Correction for an error in Expected Airtime metric definition |  |
| Date Submitted | [July 10th, 2018] |  |
| Source | [Charlie Perkins] [Futurewei] | Voice: [+1-408-330-4586] Fax: [ ] E-mail: [charliep@computer.org] |
| Re: | [TG 802.15.10 Expected Airtime metric needs correction] |  |
| Abstract | Expected Airtime metric needs correction; discussion of approaches |  |
| Purpose | [Prepare Expected Airtime metric section in draft document] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

In the defining equation for the ExpectedAirtime (page 51 of 802.15.10-2017), I believe there is a mistake.

To discuss the problem, I rewrite the equation as

Expected airtime = ( overhead time + payload time ) * (BER enlargement)

Next, reformulate the payload time:

payload time = {(payload bits) / (effective data-rate in bits-per-second)}

And then effective data-rate:

effective data-rate = {PHY data-rate} * {% time allowed for xmit}

On page 51, BO/SO would be the number for (% time allowed for xmit).  This is unlikely, since BO > SO.

We could try SO / BO, but that would mean that the transmitter gets to use the entire active period of the superframe.  And, besides that, the expression should actually be written as follows:

(2 ^ SO) / (2 ^ BO)

which is the same as

{ 1 / 2 ^ (BO - SO) }.

We could try some other factor, something like:

(% time allowed for xmit)  =  (# of slots assigned to transmitter) / (2 ^ SO)

but this could be pretty dynamic, which would probably be O.K.

Or we could make it an implementation-defined parameter.

Or we could delete the ExpectedAirtime metric.

In summary, I could offer the following four corrections for Expected Airtime:

- Replace BO/SO  by { 1 / (2 ^ (BO - SO)) }.

- Replace BO/SO  by  (# of slots assigned to transmitter) / (2 ^ SO)

- Replace BO/SO  by  an implementation-defined parameter.

- Delete the ExpectedAirtime metric.

Given the uncertainty surrounding the definition, perhaps (4) is the best choice.