IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | IEEE 802.15.4z MAC LRP - CRG |
| Date Submitted | 19st September 2019 |
| Source | Peter Sauer (Microchip), David Barras (3db Access), Boris Danev (3db Access), Patrik Leu (ETH Zurich) |
| Re: | Letter Ballot Comments with Figures and Tables – P802.15.4z-D2 |
| Abstract | This contribution proposes updated text for the baseline draft P802.15.4z-D2 |
| Purpose | Provision of the text to facilitate its incorporation into the draft text of the IEEE 802.15.4z standard currently under development in TG4z. |
| Notice | This document does not represent the agreed views of the IEEE 802.15 Working Group. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |

- Overview

  - Scope

  - Purpose

- Normative references

- Definitions, acronyms and abbreviations

- Format conventions

- General description

- MAC functional description

  - Device types and conventions

  - Channel access

  - Starting and maintaining PANs

  - Association and disassociation

  - Synchronization

  - Transaction handling

- MAC frame formats

  - Device extended address

  - General MAC frame format

  - Format of individual frame types

  - IEs

    - IE list termination

  - MAC commands

    - Command ID field

Add the following new rows into Table 7-49:

Table 7-49—MAC commands

| Command ID | Command name | RFD |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | TX | RX |  |
| <ANA> | Ranging Verifier command |  |  | 7.5.27 |
| <ANA> | Ranging Prover command |  |  | 7.5.28 |

Insert the following new subclauses (7.4.5.27 and 7.4.5.28) after 7.4.5.26:

    - Ranging Verifier command

The Ranging Verifier command is sent by the Verifier MAC sublayer as a result of the invocation of an MCPS-RANGING-VERIFIER.request primitive by the Verifier device next higher layer.

The Verifier MAC shall set the Frame Pending field and the AR field to zero and the Frame Version field shall set to 0b10.

The Destination Addressing Mode field and Source Addressing Mode field shall be set to indicate no addressing or short addressing and the Sequence Number Suppression field shall be set to one. This allows a minimal command frame to be sent over the air ..

The Ranging Verifier command Content field shall be formatted as illustrated in Figure 1.

| Octets: 1 | 4/8/16/32/64/128 |
| --- | --- |
| Reserved | Challenge |

Figure 1—Ranging Challenge command Content field format

The Reserved field is reserved for future use and shall be set to zero.

The Challenge field contains challenge data of length defined by the security level parameter of the MCPS-RANGING-VERIFIER.request primitive, as per Error: Reference source not found or Table 9 in case of tolerance of bit errors in the challenge payload is used.

    - Ranging Prover command

The Ranging Prover command issent by the Prover MAC sublayer as a result of the invocation of an MCPS-RANGING-PROVER.request primitive by the Verifier device next higher layer.

The Verifier MAC sublayer shall set the Frame Pending field and the AR field set to zero and the Frame Version field shall set to 0b10.

The Destination Addressing Mode field and Source Addressing Mode field shall be set to indicate no addressing or short addressing and the Sequence Number Suppression field shall be set to one. This allows a minimal command frame to be sent over the air.

The Ranging Prover command Content field shall be formatted as illustrated in Figure 2.

| Octets: 1 | 4/8/16/32/64/128 |
| --- | --- |
| Reserved | Response |

Figure 2—Ranging Response Reply command Content field format

The Reserved field is reserved for future use and shall be set to zero.

The Response field contains response data. The response data is createdby the MAC sublayer according to the authenticated challenge-response ranging mode and schemes described in 6.9.8. Each of the schemes define the content of the response data according to configured ranging mode and security level.

- MAC services

  - Overview

  - MAC management service

  - MAC data service

    - MCPS-DATA.request

    - MCPS-DATA.confirm

    - MCPS-DATA.indication

    - MCPS-PURGE.request

    - MCPS-PURGE.confirm

Insert the following new subclauses 8.3.6 to 8.3.11 after 8.3.5:

    - MCPS-RANGING-VERIFIER.request

The MCPS-RANGING-VERIFIER.request primitive requests the ERDEV to initiate a challenge-response ranging exchange.

The semantics of this primitive are as follows:

MCPS-RANGING-VERIFIER.request	(

TimeOut,

AuthenticatedChallengeResponseRangingMode,

RawMode,

NumMaxAllowedBitErrors,

AddressMask,

SrcAddrMode,

DstAddrMode,

DstPanId,

DstAddr,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

DistanceCommitmentLevel,

UwbPreambleSymbolRepetitions,

DataRate,

LocationEnhancingInformationPostamble,

LocationEnhancingInformationPostambleLength,

PanIdSuppressed,

SeqNumSupressed

)

The primitive parameters are defined in Table 30.

Table 30—MCPS-RANGING-VERIFIER.request

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TimeOut | Integer | 0x000000 – 0xFFFFFF | Max. time period for the activation of the ranging. When the timeout period expires or the MCPS-RANGING-VERIFIER.confirm is received the current ranging transfer will be aborted and ranging disabled. The time out period is defined by TimeOut * phyLrpUwbFixedReplyTime |
| AuthenticatedChallengeResponseRangingMode | Enumeration | ACRRM_SS_TWR_OWA, ACRRM_SS_TWR_MA, ACRRM_DS_TWR_OWA, ACRRM_DS_TWR_MA | Specifies the types of supported authenticated challenge response ranging methods |
| RawMode | Boolean | TRUE, FALSE | If set to TRUE the FCS check is ignored and the received frame is returned to the higher layer. If set to FALSE FCS check is active. |
| NumMaxAllowedBitErrors | Integer | 0-255 | Specifies the number of maximum allowed bit errors in the challenge and response data |
| AddressMask | -- | As specified by the SrcAddrMode parameter. | The address mask bits enable the corresponding address bit check when set to 1. When set to 0 the corresponding address bit is don’t care. |
| SrcAddrMode | Enumeration | NONE, SHORT, EXTENDED | The source addressing mode. |
| DstAddrMode | Enumeration | NONE, SHORT, EXTENDED | The destination addressing mode. |
| DstPanId | Integer | 0x0000–0xffff | The PAN ID of the entity to which the command is being transferred. |
| DstAddr | -- | As specified by the DstAddrMode parameter. | The address of the entity to which the command is being transferred. |
| SecurityLevel | Integer | As defined in Table 9-6 | The security level to be used. |
| KeyIdMode |  | As defined in Table 9-7 | The mode used to identify the key to be used. This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of octets | As specified by the KeyIdMode parameter | The originator of the key to be used, as described in 9.4.3.1. This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00 or 0x01. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used, as described in 9.4.3.2. This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| DistanceCommitmentLevel | Enumeration | DCL_1_4096, DCL_2_2048, DCL_1_1024, DCL_1_512, DCL_1_256, DCL_1_128, DCL_1_64, DCL_DISABLED | Specifies the aperture time Tint,RF in the fraction of one RSTU. For ranging capable PHYs, this is specified in 10.3. |
| UwbPreambleSymbolRepetitions | Integer | 0, 16, 32, 64, 128, 256, 512, 1024, 4096, 8192 | The preamble symbol repetitions of the UWB frame |
| DataRate | Integer | -- | Indicates the data rate. |
| LocationEnhancingInformationPostamble | Enumeration | LEIP_NONE, LEIP_IMMEDIATE, LEIP_DELAYED | For the LRP UWB PHY this parameter specifies whether the Location enhancing information postamble sequence is to be sent or not and, if present, whether it directly follows the CRC or is delayed by the aLeipDelayTime. A value of LEIP_NONE is used for non-LRP UWB PHYs. |
| LocationEnhancingInformationPostambleLength | Enumeration | LEIP_LEN_16, LEIP_LEN_64, LEIP_LEN_128, LEIP_LEN_192, LEIP_LEN_256, LEIP_LEN_512, LEIP_LEN_1024 | LEIP_LEN_16, LEIP_LEN_64, LEIP_LEN_128, LEIP_LEN_192, LEIP_LEN_256, LEIP_LEN_512, LEIP_LEN_1024 For the LRP UWB PHY when the LocationEnhancingInformationPostamble parameter has a value of either LEIP_IMMEDIATE or LEIP_DELAYED, then this parameter specifies the length in pulses of the location enhancing information postamble to send. This parameter is ignored when the LocationEnhancingInformationPostamble parameter has a value of LEIP_NONE. |
| PanIdSuppressed | Boolean | TRUE, FALSE | Set to TRUE if the PAN ID is suppressed in the frame, FALSE otherwise. |
| SeqNumSuppressed | Boolean | TRUE, FALSE | Set to TRUE if the sequence number is suppressed in the frame, FALSE otherwise. |

|  |  |  | Error: Reference source not found |
| --- | --- | --- | --- |

Upon receipt of the MCPS-RANGING-VERIFIER.request primitive, the MAC sublayer is enabled with a set of selected parameters including

a timeout, the authenticated challenge-response ranging mode, enabling or disabling the FCS checking (RawMode), the number of maximum allowed bit errors, distance commitment level and the other  MCPS parameters as defined in Table 8-75.t. Setting these parameters depends on the authenticated challenge-response ranging scheme and it is described in 6.9.8 for each scheme.

After the request, the Verifier MAC sublayer  generates a fresh unguessable cryptographic random number. The generation is typically achieved by a well-established and industry accepted cryptographically secure pseudo-random number generator (CSPRNG). the Ranging Verifiee command (as defined in 7.5.27) containing this number in the challeng data is then transmitted, and the receiver is enabled to await the response.

In multi-node ranging with broadcast address an AddressMask is configured to filter addresses from the Prover responses.

If the TimeOut timer expires, ranging will be aborted and the MCPS-RANGING-VERIFIER.confirm primitive will be issued with Status parameter value of TRANSACTION_EXPIRED.

    - MCPS-RANGING-VERIFIER.indication

The MCPS-RANGING-VERIFIER.indication primitive indicates the reception of the Ranging Prover command from a Prover device as part of a challenge-response ranging exchange.

The semantics of this primitive are as follows:

MCPS-RANGING-VERIFIER.indication (

SrcAddrMode,

SrcPanId,

SrcAddr,

DstAddrMode,

DstPanId,

DstAddr,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

DistanceCommitmentLevel,

RangingStatus,

RxRangingCounter,

Rssi,

RangingChallenge,

RangingResponse

)

The primitive parameters are defined in Table 31.

Table 31—MCPS-RANGING-VERIFIER.indication

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Enumeration | NONE, SHORT, EXTENDED | The source addressing mode of the received command. |
| SrcPanId | Integer | 0x0000–0xffff | The PAN ID of the entity from which the command was received. Valid only when a source PAN ID is included in the received frame. |
| DstAddrMode | Enumeration | NONE, SHORT, EXTENDED | The destination addressing mode of the received command. |
| DstPanId | Integer | 0x0000–0xffff | The PAN ID of the entity to which the command is being transferred. Set to the receiver’s PAN ID if the PAN ID is not carried in the received frame. |
| DstAddr | -- | As specified by the DstAddrMode parameter. | The address of the entity to which the command is being transferred. |
| SecurityLevel | Integer | 0x00-0x07 | The security level purportedly used by the received command, as defined in Table 9-6. |
| KeyIdMode | Integer | 0x00-0x03 | The mode used to identify the key purportedly used by the originator of the received frame, as defined in Table 9-7. This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of octets | As specified by the KeyIdMode parameter | The originator of the key purportedly used by the originator of the received frame, as described in 9.4.3.1. This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00 or 0x01. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the originator of the received frame, as described in 9.4.3.2. This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| DistanceCommitmentLevel | Enumeration | DCL_1_4096, DCL_2_2048, DCL_1_1024, DCL_1_512, DCL_1_256, DCL_1_128, DCL_1_64, DCL_DISABLED | The aperture time Tint,RF in the fraction of one microsecond used to collect earliest path(s) by the receiver for symbol decoding in the PSDU. For LRP UWB PHY refer to clause 19.10, Table 60. |
| RangingStatus | Enumeration | RANGING_ACTIVE, RANGING_ABORTED, NO_RANGING_RECEIVED | A value of NO_RANGING_ RECEIVED indicates that the received frame was not a ranging frame. A value of RANGING_ACTIVE denotes ranging operations is active and enabled. A value of RANGING_ ABORTED denotes that ranging is disabled or timed out. |
| RxRangingCounter | Unsigned Integer | 0x00000000– 0xffffffff | A count of the time units corresponding to an RMARKER at the antenna with respect to the reception of the frame delivered by this MCPS-RANGING-VERIFIER.indication. The value here is meaningless if ranging is not supported or enables. The 36bit ranging counter operates with the time unit defined in clause 6.9.1.4. for the LRP UWB PHY. The RxRangingCounter are the 32bit MSB of this counter. |
| Rssi | Integer | 0x00-0xff | The Received Signal Strength Indicator is a measure of the RF power level at the input of the transceiver measured during the SFD. |
| RangingChallenge | Set of octets | As defined in clause 7.5.27 | Payload send by the Verifier with Ranging Verifier command. |
| RangingResponse | Set of octets | As defined in clause 7.5.28 | Payload send by the Prover with Ranging Prover command. |

    - MCPS-RANGING-VERIFIER.confirm

The MCPS-RANGING-VERIFIER.confirm primitive reports the result of a ranging request to transfer a Ranging Reply command from a Prover device.

The semantics of this primitive are as follows:

MCPS-RANGING-VERIFIER.confirm	(

Status

)

The primitive parameters are defined in Table 3.

Table 3—MCPS-RANGING-VERIFIER.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, TRANSACTION_EXPIRED, INVALID_PARAMETER, RANGING_VERIFIER_NOT_SUPPORTED | The result of the request for the ranging operation. |

    - MCPS-RANGING-PROVER.request

The MCPS-RANGING-PROVER.request primitive prepares the ERDEV to receive a challenge in a challenge-response ranging exchange, and to respond accordingly.

The semantics of this primitive are as follows:

|  |  |  | Error: Reference source not found |
| --- | --- | --- | --- |
|  |  |  | Error: Reference source not found |

MCPS-RANGING-PROVER.request	(

TimeOut,

AuthenticatedChallengeResponseRangingMode

RawMode,

NumMaxAllowedBitErrors,

SrcAddrMode,

DstAddrMode,

DstPanId,

DstAddr,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

DistanceCommitmentLevel,

UwbPreambleSymbolRepetitions,

DataRate,

LocationEnhancingInformationPostamble,

LocationEnhancingInformationPostambleLength,

PanIdSuppressed,

SeqNumSupressed

)

The primitive parameters are defined in Table 33.

Table 33—MCPS-RANGING-PROVER.request

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TimeOut | Integer | 0x000000 – 0xFFFFFF | Max. time period for the activation of the ranging. When the timeout period expires or the MCPS-RANGING-PROVER.confirm is received the current ranging transfer will be aborted and ranging disabled. The time out period is defined by TimeOut * phyLrpUwbFixedReplyTime |
| AuthenticatedChallengeResponseRangingMode | Enumeration | ACRRM_SS_TWR_OWA, ACRRM_SS_TWR_MA, ACRRM_DS_TWR_OWA, ACRRM_DS_TWR_MA | Specifies the types of supported authenticated challenge response ranging methods |
| RawMode | Boolean | TRUE, FALSE | If set to TRUE the FCS check is ignored and the received frame is returned to the higher layer. If set to FALSE FCS check is active. |
| NumMaxAllowedBitErrors | Integer | 0-255 | Specifies the number of maximum allowed bit errors in the challenge and response data |
| SrcAddrMode | Enumeration | NONE, SHORT, EXTENDED | The source addressing mode. |
| DstAddrMode | Enumeration | NONE, SHORT, EXTENDED | The destination addressing mode. |
| DstPanId | Integer | 0x0000–0xffff | The PAN ID of the entity to which the command is being transferred. |
| DstAddr | -- | As specified by the DstAddrMode parameter. | The address of the entity to which the command is being transferred. |
| SecurityLevel | Integer | As defined in Table 9-6 | The security level to be used. |
| KeyIdMode |  | As defined in Table 9-7 | The mode used to identify the key to be used. This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of octets | As specified by the KeyIdMode parameter | The originator of the key to be used, as described in 9.4.3.1. This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00 or 0x01. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used, as described in 9.4.3.2. This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| DistanceCommitmentLevel | Enumeration | DCL_1_4096, DCL_2_2048, DCL_1_1024, DCL_1_512, DCL_1_256, DCL_1_128, DCL_1_64, DCL_DISABLED | Specifies the aperture time Tint,RF in the fraction of one microsecond used to collect earliest path(s) by the receiver for symbol decoding in the PSDU. For LRP UWB PHY refer to clause 19.10, Table 60. |
| UwbPreambleSymbolRepetitions | Integer | 0, 16, 32, 64, 128, 256, 512, 1024, 4096, 8192 | The preamble symbol repetitions of the UWB frame. |
| DataRate | Integer | -- | Indicates the data rate. For LRP UWB PHYs, valid values are defined in Table 19-1. For all other PHYs, the parameter is set to zero. |
| LocationEnhancingInformationPostamble | Enumeration | LEIP_NONE, LEIP_IMMEDIATE, LEIP_DELAYED | For the LRP UWB PHY this parameter specifies whether the Location enhancing information postamble sequence is to be sent or not and, if present, whether it directly follows the CRC or is delayed by the aLeipDelayTime. A value of LEIP_NONE is used for non-LRP UWB PHYs. |
| LocationEnhancingInformationPostambleLength | Enumeration | LEIP_LEN_16, LEIP_LEN_64, LEIP_LEN_128, LEIP_LEN_192, LEIP_LEN_256, LEIP_LEN_512, LEIP_LEN_1024 | LEIP_LEN_16, LEIP_LEN_64, LEIP_LEN_128, LEIP_LEN_192, LEIP_LEN_256, LEIP_LEN_512, LEIP_LEN_1024 For the LRP UWB PHY when the LocationEnhancingInformationPostamble parameter has a value of either LEIP_IMMEDIATE or LEIP_DELAYED, then this parameter specifies the length in pulses of the location enhancing information postamble to send. This parameter is ignored when the LocationEnhancingInformationPostamble parameter has a value of LEIP_NONE. |
| PanIdSuppressed | Boolean | TRUE, FALSE | Set to TRUE if the PAN ID is suppressed in the frame, FALSE otherwise. |
| SeqNumSuppressed | Boolean | TRUE, FALSE | Set to TRUE if the sequence number is suppressed in the frame, FALSE otherwise. |

Upon receipt of the MCPS-RANGING-PROVER.request primitive, the MAC sublayer enables the receiver to receive the challenge and process it according to the set parameters including a timeout, the authenticated challenge-response ranging mode, enabling or disabling the FCS checking (RawMode), the number of maximum allowed bit errors, distance commitment level and the other MCPS parameters as defined in Table 8-75. The choice of parameter values depends on the authenticated challenge-response ranging scheme and it is described in 6.9.8 for each scheme.

.

When a Ranging Verifier command is received , a ti tha response data is created b the Prover MAC sublayer according to the selected authenticated challenge-response scheme and security level and is sent usinger, the Ranging Proply command (as defined in d.

In multi-node ranging the Prover replies to the broadcast address as described in Error: Reference source not found.

If the Timeout timer expires, ranging will be aborted and the MCPS-RANGING-PROVER.confirm primitive will be issued with Status parameter value of TRANSACTION_EXPIRED.

    - MCPS-RANGING-PROVER.indication

The MCPS-RANGING-PROVER.indication primitive indicates the reception of the Ranging command from a Verifier device as part of a challenge-response ranging exchange.

The semantics of this primitive are as follows:

MCPS-RANGING-PROVER.indication	(

SrcAddrMode,

SrcPanId,

SrcAddr,

DstAddrMode,

DstPanId,

DstAddr,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

DistanceCommitmentLevel,

Rssi,

RangingChallenge,

RangingResponse

)

The primitive parameter is defined in Table 34.

Table 34—MCPS-RANGING-RESPONSE.indication

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Enumeration | NONE, SHORT, EXTENDED | The source addressing mode of the received command. |
| SrcPanId | Integer | 0x0000–0xffff | The PAN ID of the entity from which the command was received. Valid only when a source PAN ID is included in the received frame. |
| DstAddrMode | Enumeration | NONE, SHORT, EXTENDED | The destination addressing mode of the received command. |
| DstPanId | Integer | 0x0000–0xffff | The PAN ID of the entity to which the command is being transferred. Set to the receiver’s PAN ID if the PAN ID is not carried in the received frame. |
| DstAddr | -- | As specified by the DstAddrMode parameter. | The address of the entity to which the command is being transferred. |
| SecurityLevel | Integer | 0x00-0x07 | The security level purportedly used by the received command, as defined in Table 9-6. |
| KeyIdMode | Integer | 0x00-0x03 | The mode used to identify the key purportedly used by the originator of the received frame, as defined in Table 9-7. This parameter is invalid if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of octets | As specified by the KeyIdMode parameter | The originator of the key purportedly used by the originator of the received frame, as described in 9.4.3.1. This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00 or 0x01. |
| KeyIndex | Integer | 0x01–0xff | The index of the key purportedly used by the originator of the received frame, as described in 9.4.3.2. This parameter is invalid if the KeyIdMode parameter is invalid or set to 0x00. |
| DistanceCommitmentLevel | Enumeration | DCL_1_4096, DCL_2_2048, DCL_1_1024, DCL_1_512, DCL_1_256, DCL_1_128, DCL_1_64, DCL_DISABLED | The aperture time Tint,RF in the fraction of one microsecond used to collect earliest path(s) by the receiver for symbol decoding in the PSDU. For LRP UWB PHY refer to clause 19.10, Table 60. |
| Rssi | Integer | 0x00-0xff | The Received Signal Strength Indicator is a measure of the RF power level at the input of the transceiver measured during the SFD. |
| RangingChallenge | Set of octets | As defined in clause 7.5.27 | Payload send by the Verifier with Ranging Verifier command. |
| RangingResponse | Set of octets | As defined in clause 7.5.28 | Payload send by the Prover with Ranging Prover command. |

    - MCPS-RANGING-PROVER.confirm

The MCPS-RANGING-PROVER.confirm primitive reports the result of a ranging request to transfer a Ranging Reply command to a Verifier device.

The semantics of this primitive are as follows:

MCPS-RANGING-PROVER.confirm	(

Status

)

The primitive parameters are defined in Table 6.

Table 6—MCPS-RANGING-PROVER.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS,TRANSACTION_EXPIRED , INVALID_PARAMETER, RANGING_PROVER_NOT_SUPPORTED | The result of the request for the ranging operation. |

Addressing comments: r1-0779, r1-0780, r1-0781

11.3 PHY PIB attributes

Table 11-2–PHY PIB attributes

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| phyLrpUwbFixedDelayFactor | Integer | 1 to 32767 | Define the reply delay factor which multiplies the phyLrpUwbFixedReplyTime to be used in multi-node ranging |