IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | D0-PHY VIII specifications |  |
| Date Submitted | January 2022 |  |
| Source | Huy Nguyen, Md. Osman Ali, and Yeong Min Jang (Kookmin University) | Voice: [ ] Fax: [ ] E-mail: [yjang@kookmin.ac.kr] |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [Description of document contents.] |  |
| Purpose | [Description of what the author wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# 17. PHY VIII specifications

## 17.1. HOOK-OFDM

The Hybrid Orthogonal Frequency Division Multiplexing- On-Off Keying (HOOK-OFDM) Modulation for high-speed OCC system uses the PHY VIII – Singular Point Source.

The PHY VIII Operating Modes system specifications are given in Table xx. The additional PHY Operating Modes by HOOK-OFDM for Smart Device is presented the Table xx – PHY VIII Operating Modes.

Table xx – PHY VIII Operating Modes

| PHY Operating Modes |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Mode | Optical Clock rate | Modulation | RLL Code | FEC (Convolution code) | Packet length | Total data rate |
| HOOK-OFDM | Mode 1 | 16 kHz | ACO-OFDM | None | CC (3/4) | 32 | 20.16 kbps |
|  |  |  | OOK | Manchester | CC (3/4) | 20 |  |
|  | Mode 2 | 32 kHz | DCO-OFDM | None | CC (3/4) | 32 | 38.40 kbps |
|  |  |  | OOK | Manchester | CC (3/4) | 40 |  |
|  | Mode 3 | 16 kHz | ACO-OFDM | None | CC (3/4) | 64 | 48.00 kbps |
|  |  |  | OOK | 4B6B | CC (3/4) | 24 |  |
|  | Mode 4 | 32 kHz | DCO-OFDM | None | CC (3/4) | 64 | 94.08 kbps |
|  |  |  | OOK | 4B6B | CC (3/4) | 48 |  |

### 17.1.1. Reference architecture

A reference architecture to implement HOOK-OFDM is shown in Figure xx.

Figure xx. HOOK-OFDM block diagram

### 17.1.2. Encoder configuration

A packet of data is modulated using OFDM modulation. The optical clock rate is at 16 kHz or 32 kHz. The configuration of the mode of HOOK-OFDM scheme shall be implemented via the PHY PIB attribute phyHookOfdmMode

Figure xx. Data frame structure of hybrid OFDM-OOK scheme. (a) OOK packet. (b) OOK data (c) hybrid signal (d) OFDM signal.

In each ‘high’ and ‘low’ period of C-OOK waveform, we can embed the high-frequency OFDM waveform to increase data rate of the system.

In the low data rate stream, we apply the C-OOK frame as the above figure. With high data rate stream, each period of C-OOK waveform will be put one OFDM frame to generate hybrid waveform.

### 17.1.3. Hermitian mapping

Unlike the conventional OFDM in Radio Frequency, instead of feeding the data symbol directly into the IDFT block, each symbol must pass through the Hermitian block. The signal is then fed into the IFFT. The special purpose of the Hermitian block is that it ensures the output of the IDFT is entirely real.

### 17.1.4. Sequence Number inserting

The data sub-packet payload shall consist of two subparts: SN data, payload. The SN Data consists of asynchronous information, which helps the receiver side decode data.

SN shall be implemented over the PHY PIB attribute phyHookOfdmSn

### 17.1.5. Forward error correction (FEC)

The data sub-packet payload may be coded by FEC to protect the payload from error. Convolution code (CC) may be used as an FEC.

The configuration of error correction for HOOK-OFDM, including FEC for OOK scheme and FEC for OFDM scheme, shall be implemented via the PHY PIB attribute phyHookOfdmFec

| Attribute | Type | Range | Description |
| --- | --- | --- | --- |
| phyHookOfdmMode | Integer | 0-7 | The mode applied for HOOK-OFDM. 0: Mode 1 1: Mode 2 2: Mode 3 3: Mode 4 Others: Reserved |
| phyHookOfdmSn | Integer | 0-3 | This attribute specifies the length of Sequence Number per packet of HOOK-OFDM 0: 2 bits 1: 3 bits 2-3: reserved |
| phyHookOfdmFec | Integer | 0-7 | This attribute specifies FEC for HOOK-OFDM modulation. 0: None 1: Hamming (8/4) 2: Hamming (15/11) 3: RS(15,11) Other values: Reserved |

## 17.2. HS2PSK-OFDM

The Hybrid Orthogonal Frequency Division Multiplexing- Spatial-2 Phase Shift Keying (HS2-PSK-OFDM) Modulation for high-speed OCC system uses the PHY VIII – Multiple Point Source. The output waveforms are a hybrid modulation of S2-PSK and OFDM.

The PHY VIII Operating Modes system specifications are given in Table xx. The additional PHY Operating Modes by HS2PSK-OFDM for Smart Device is presented the Table xx – PHY VIII Operating Modes.

| PHY Operating Modes |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Mode | Optical Clock rate | Modulation | RLL Code | FEC (Convolution code) | Packet length | Total data rate |
| HOOK-OFDM | Mode 1 | 16 kHz | OFDM | None | CC (3/4) | 32 | 20.16 kbps |
|  |  |  | S2-PSK | Manchester | CC (3/4) | 20 | 20 bps |
|  | Mode 2 | 32 kHz | OFDM | None | CC (3/4) | 32 | 38.40 kbps |
|  |  |  | S2-PSK | Manchester | CC (3/4) | 30 | 30 bps |
|  | Mode 3 | 16 kHz | OFDM | None | CC (3/4) | 64 | 48.00 kbps |
|  |  |  | S2-PSK | 4B6B | CC (3/4) | 12 | 12 bps |
|  | Mode 4 | 32 kHz | OFDM | None | CC (3/4) | 64 | 94.08 kbps |
|  |  |  | S2-PSK | 4B6B | CC (3/4) | 24 | - ps |

### 17.2.1. Reference architecture

A reference architecture to implement HS2PSK-OFDM is shown in Figure xxx

Figure xx. HS2PSK-OFDM block diagram

A packet of data is modulated using OFDM modulation. The optical clock rate is at 16 kHz or 32 kHz. The configuration of the mode of HS2PSK-OFDM scheme shall be implemented via the PHY PIB attribute phyHs2pskOfdmMode

Figure xx. Hybrid S2PSK OFDM waveform

### 17.2.2. Sequence Number inserting

The data sub-packet payload shall consist of two subparts: SN data, payload. The SN Data consists of asynchronous information, which helps the receiver side decode data.

SN shall be implemented over the PHY PIB attribute phyHs2pskOfdmSn

### 17.2.3. Forward error correction (FEC)

The data sub-packet payload may be coded by FEC to protect the payload from error. Convolution code (CC) may be used as an FEC.

The configuration of error correction for HS2PSK-OFDM, including FEC for S2-PSK scheme and FEC for OFDM scheme, shall be implemented via the PHY PIB attribute phyHs2pskOfdmFec

S2-PSK waveform

In each ‘high’ and ‘low’ period of S2-PSK waveform, we can embed the high-frequency OFDM waveform to increase data rate of the system.

In the low data rate stream, we apply the S2-PSK frame as the above figure. With high data rate stream, each period of S2-PSK waveform will be put one OFDM frame to generate hybrid waveform.

Figure xxx. Data frame structure for Rolling-OFDM system in high speed stream

| Attribute | Type | Range | Description |
| --- | --- | --- | --- |
| phyHs2pskOfdmMode | Integer | 0-7 | The mode applied for HS2PSK-OFDM. 0: Mode 1 1: Mode 2 2: Mode 3 3: Mode 4 Others: Reserved |
| phyHs2pskOfdmSn | Integer | 0-3 | This attribute specifies the length of Sequence Number per packet of HS2PSK -OFDM 0: 2 bits 1: 3 bits 2-3: reserved |
| phyHs2pskOfdmFec | Integer | 0-7 | This attribute specifies FEC for HS2PSK-OFDM modulation. 0: None 1: Hamming (8/4) 2: Hamming (15/11) 3: RS(15,11) Other values: Reserved |

## 17.3. BPPM

The bi-level pulse position modulation (BPPM) Modulation for vehicular OCC system uses the PHY VIII – Multiple Point Source. High-speed data streams are modulated with a PPM scheme and low-speed data streams are transmitted by modulating the pulse amplitudes of the high-speed streams at two different intensity levels.

### 17.3.1. Reference architecture

The reference architecture of bi-level pulse position modulation (BPPM) is shown in Figure xxx. Each of the rear LEDs transmits the same information concurrently. High-speed data streams are modulated with a PPM scheme and low-speed data streams are transmitted by modulating the pulse amplitudes of the high-speed streams at two different intensity levels.

Figure xxx. Block diagram of BPPM transmitter.

### 17.3.2. Encoder configuration

Figure xxx. Proposed BPPM packet

The PSDU field carries the data of the PHY frame and it can contain an arbitrary number of payload bits. The end of the PSDU field is indicated by the presence of another SFD, i.e., the preamble frame. The overall structure of the proposed BPPM data packet format is shown in Figure xxx.

The optical clock rate is at 8 kHz or 10 kHz. The optical clock rate at which BPPM symbols are clocked out is configurable over PHY PIB attribute phyBppmOpticalClockRate.

### 17.3.3. Forward error correction (FEC)

The data sub-packet payload may be coded by FEC to protect the payload from error. Convolution code (CC) may be used as an FEC.

The configuration of error correction for BPPM, shall be implemented via the PHY PIB attribute phyBppmFec

| Attribute | Type | Range | Description |
| --- | --- | --- | --- |
| phyBppmOpticalClockRate | Integer | 0-7 | The optical clock rate (or symbol rate) applied for BPPM. 0: 8 kHz 1: 10 kHz Others: Reserved |
| phyBppmFec | Integer | 0-7 | This attribute specifies FEC for BPPM scheme. 0: None 1: Hamming (8/4) 2: Hamming (15/11) 3: RS(15,11) Other values: Reserved |