IEEE 802.15

Wireless Specialty Networks

| Project | IEEE 802.15 Working Group for Wireless Specialty Networks |
| --- | --- |
| Title | 15.6a functional technical requirements for PHY proposals |
| Date Submitted | May 13th, 2022 |
| Source | Marco Hernandez1, Ryuji Kohno1,2, Takumi Kobayashi1,2, Minsoo Kim2 1YRP-IAI, 2YNU, Japan; Marco.Hernandez@ieee.org Kohno@ynu.ac.jp Kobayashi-Takumi-ch@ynu.ac.jp Minsoo@minsookim.com. |
| Response | To call for contributions |
| Abstract | This document contains a summary of the technical requirements for proposals to form the P802.15.6 revision specification. |
| Purpose | For discussion in the join session 15.6a, 15.4ab, 15.14 |
| Notice | This document has been prepared to assist the IEEE P802.15.6a. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |

- System Performance

  - Throughput measured at MAC SAP

The 802.15.6a amendment (revision) shall provide at least one mode of operation capable of achieving a throughput of at least 40 Mb/s, operating at a maximum mandatory data rate of 50 Mb/s* in the high band of UWB (500 MHz channel) under a Packet Delivery Ratio of 99%.

*The new data rates proposed in 15.4ab of 62.4 Mb/s and 124.8 Mb/s look promising. We need to check reliability in congested environments and transmission ranges.

  - Transmission range

The 802.15.6a amendment (revision) shall provide at least one mode of operation that achieves the same transmission range as the one provided by IEEE Std 802.15.6-2012 of 3m at the highest possible data rate operating in the high band of UWB band.

  - Transmission reliability

The 802.15.6a amendment (revision) shall improve transmission reliability under congested communication environments compared to IEEE Std 802.15.6-2012 operating in the UWB band, including interference from other 15.6 BANs (intra-interference) and other wireless systems (inter-interference) operating in the UWB band.

A 15.6a device may be equipped with multiple antennas configuration to support a higher data rate mode.

  - Latency

The 802.15.6a amendment shall improve latency under congested communication environments compared to IEEE Std 802.15.6-2012 operating in the UWB band.

Proposals should include end-to-end latency in the interval [250 msec, 1 sec]. Association latency bounded to 1 sec.

Proposals for critical use cases should include end-to-end latency bounded to 100 msec.

  - Ranging

A data rate of 500 kb/s is aimed to be compatible with the 15.4ab proposal.

- PHY layer

The PHY layer includes, but not be limited to, mechanisms for synchronization, channel estimation, modulation, FEC, ranging, Clear Channel Assessment (CCA), and interference mitigation that satisfy the performance requirements mentioned above (clause 1).