IEEE 802.15

Wireless Specialty Networks

| Project | IEEE 802.15 Working Group for Wireless Specialty Networks |
| --- | --- |
| Title | UWB Channel Models Document |
| Date Submitted | January 18th, 2024 |
| Source | Takumi Kobayashi1,4, Kamran Sayrafian5 , Daisuke Anzai4, Marco Hernandez1,3, Ryuji Kohno1,2, Minsoo Kim2; 1YRP-IAI, 2YNU, Japan, 3CWC Oulu Univ. Finland, 4Nagoya Institute of Technology, Japan, 5NIST, USA; Marco.Hernandez@ieee.org Kohno@ynu.ac.jp Kobayashi@nitech.ac.jp Minsoo@minsookim.com Anzai@nitech.ac.jp Kamran.Sayrafian@nist.gov. |
| Response | To call for contributions |
| Abstract | This document contains the channel models to evaluate proposals. |
| Purpose | For contributions to P802.15.6ma |
| Notice | This document has been prepared to assist the IEEE P802.15.6ma. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |

Contributors

| Name & affiliation | Email |
| --- | --- |
| Marco Hernandez, CWC, Oulu Univ. Finland, YRP-IAI Japan. | Marco.Hernandez@ieee.org |
| Ryuji Kohno, YRP-IAI, YNU, Japan | Kohno@ynu.ac.jp |
| Takumi Kobayashi, YRP-IAI, Nagoya Institute of Technology, Japan | Kobayashi@nitech.ac.jp |
| Minsoo Kim, YRP-IAI, Japan. | Minsoo@minsookim.com |
| Kamran Sayrafian, NIST, USA. | kamran.sayrafian@nist.gov |
| Daisuke Anzai, Nagoya Institute of Technology, Japan. | Anzai@nitech.ac.jp |

Revision history

| Revision | Date | Notes |
| --- | --- | --- |
| 0 | 7/12/2022 | Document created from 15-0344-03-006a plus adding edits to Section 6.2 and use cases S2.3. Removing S4.1, updates to S6.1 and Fig 3. |
| 1 | 3/15/2023 | S2.1 Option2, S2.2, S2.3, S6.2, S8, S8.1, S8.2, S9, S10, S11, S12, S13 and S14 models have been added and updated. Simulation MATLAB code program has been added. |
| 2 | 5/16/2023 | S3.1, S6.2, S9, S10, S11, S12, S13 and S14 have been updated. |
| 3 | 7/13/2023 | Minor collections of texts and statements. |
| 4 | 9/5/2023 | Clause 5.2.4 and sub-clause 5.2.4.1 have been added. |
| 5 | 1/18/2024 | Minor modifications on numbering of sections, sub-sections, clauses and sub-clauses. |
| 6 | 3/14/2024 | Valid range of channel models have been added. Parameters of CM6.2 has been updated. |

Contents

1. Introduction	1

2. Scenarios for 15.6 revision	2

2.1 Human BAN scenarios and channel models	2

2.2 Scenario 2.1	2

2.3 Scenario 2.2	3

2.4 Scenario 2.3	3

2.5 Scenario 6.1 Body Surface (head) to External (LOS)	4

2.6 Scenario 6.2 HBAN coordinator to HBAN coordinator	4

2.7 Vehicle BAN scenarios and channel models	5

3. Antenna Effect	8

3.1 Electrical antennas, such as dipole	8

3.2 Magnetic antennas, such as loop	8

4. Channel Characterization	9

4.1 Electrical Properties of Body Tissues	9

4.2 Model Types	9

4.3 Fading	9

4.3.1 Small-Scale Fading	9

4.3.1.1 UWB Channel model	10

4.3.1.2 Simulation methodology	10

4.3.1.3 MatLab™ code	11

4.3.2 Large-Scale Fading	13

4.4 Path loss	13

4.5 Shadowing	13

5. Models and Scenarios	14

5.1 Human BAN (HBAN) Channel models	14

5.2 In-body	14

5.2.1 Implant to implant CM1 (Scenario S1) for 402 – 405 MHz	14

5.2.2 Implant to body surface CM2 (Scenario S2) for 402 – 405 MHz	14

5.2.3 Implant (upper body) to Body Surface CM2.1 (Scenario S2.1) for 3.1 – 10.6 GHz	14

5.2.4 Capsule endoscopy use-case low band of UWB  (3.1 GHz – 4.1 GHz)	14

5.2.4.1 MatLab™ code	16

5.2.5 Capsule endoscopy use-case UWB band (3.1 GHz – 10.6 GHz)	17

5.2.6 Implant (head) to Body Surface CM2.2 (Scenario S2.2) for 3.1 – 10.6 GHz	17

5.2.7 Implant (head) to External CM2.3 (Scenario S2.3) for 3.1 – 10.6 GHz	20

5.2.8 Implant (upper body) to External CM3 (Scenario 3) for 402 – 405 MHz	20

5.2.9 Implant (upper body) to External CM3.1 (Scenario 3.1) for 3.1 – 10.6 GHz	20

5.3 Body surface	21

5.3.1 Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 400 MHz	21

5.3.2 Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 600 MHz	21

5.3.3 Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 900 MHz	21

5.3.4 Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 2.4 GHz	22

5.3.5 Body surface to body surface CM3 (Scenario S4 & S5) for 3.1-10.6 GHz	22

5.3.5.1 Option 2	24

5.3.6 Dynamic channel model for body surface to body surface CM3 (Scenario S4 & S5) at 4.5 GHz	24

5.3.7 Body surface to external CM4 (Scenario S6 & S7) for 900MHz	30

5.3.8 Body surface to external CM4 (Scenario S6 & S7) for 2.4 GHz	30

5.3.9 Body surface to external CM4 (Scenario S6 & S7) for 3.1-10.6 GHz	30

5.3.10 Body surface (head) to external CM6.1 (Scenario S6.1) for 3.1 – 10.6 GHz	31

5.3.11 BAN coordinator to BAN coordinator CM6.2 (Scenario S6.2) for 3.1 – 10.6 GHz	32

5.3.12 Body surface to external CM4 (Scenario S7) for 900 MHz	33

5.3.13 Body surface to external CM4 (Scenario S7) for 2.4 GHz	33

5.3.14 Body surface to external CM4 (Scenario S7) for 3.1 – 10.6 GHz	33

5.4 Vehicle BAN (VBAN) channel models	33

5.5 In Vehicle models	34

5.5.1 In-vehicle to In-vehicle (Scenario 8, CM 8) 3.1 - 10.6GHz	34

5.5.2 Cabin room to cabin room in omnibus (Scenario 8.1) 3.1 - 10.6GHz	34

5.5.2.1 OPTION 1	35

5.5.2.2 OPTION 2	36

5.5.3 In-vehicle to In-vehicle (functional compartment) (Scenario 8.2, CM 2)	37

5.6 On vehicle	39

5.6.1 On-vehicle to on-vehicle (Scenario 11, 12, CM11, 12)	39

5.6.2 In-vehicle to on vehicle (Scenario 9, CM9) 3.1 – 10.6 GHz	39

5.7 External	40

5.7.1 In-vehicle to External (Scenario 10, CM 10) 3.1 – 10.6GHz	40

5.7.2 On-vehicle to External (LOS) (Scenario 13, CM 13) 3.1 – 10.6GHz	41

5.7.3 On-vehicle to External (NLOS) (Scenario 14, CM 14) 3.1 – 10.6GHz	41

6. MATLAB code program	43

6.1 Path loss model	43

Annex A Bibliography	48

Annex B (informative) Matlab code for IEEE802.15.4a UWB channel model	50

Annex C (informative) MATLAB script for the IEEE 802.15.4a channel model	62

Introduction

The channel models used by IEEE Std 802.15.6-2012 Wireless Body Area Network (BAN) during the development of the Std cover use cases for medical and non-medical applications inside or on the surface of the human body. The frequency bands of operation are ISM bands used by narrowband and UWB implementations [1]  .

The channel models covered in this document are for the UWB band intended for the human body surfaces, in-body to on-body communication such as BCI, BMI and WCE, and vehicles. The channel models are aimed for the development of the PHY and MAC for the revision of IEEE 802.15.6 Std.

The channel model document of IEEE Std 802.15.6-2012 defined seven scenarios and four-channel models[1]   as shown in Table 1 for illustration.

—Channel models used in IEEE Std 802.15.6-2012

| Scenario | Description | Frequency Band | Channel Model |
| --- | --- | --- | --- |
| S1 | Implant to Implant | 402-405 MHz | CM1 |
| S2 | Implant to Body Surface | 402-405 MHz | CM2 |
| S3 | Implant to External | 402-405 MHz | CM2 |
| S4 | Body Surface to Body Surface (LOS) | 13.5, 50, 400, 600, 900 MHz 2.4, 3.1-10.6 GHZ | CM3 |
| S5 | Body Surface to Body Surface (NLOS) | 13.5, 50, 400, 600, 900 MHz 2.4, 3.1-10.6 GHZ | CM3 |
| S6 | Body Surface to External (LOS) | 900 MHz 2.4, 3.1-10.6 GHZ | CM4 |
| S7 | Body Surface to External (NLOS) | 900 MHz 2.4, 3.1-10.6 GHZ | CM4 |

Figure 1 illustrates such channel models.

—Channel models for IEEE Std 802.15.6-2012

Scenarios for 15.6 revision

Human BAN scenarios and channel models

—15.6ma HBAN scenarios (notation in [1]  ).

| Scenario | Description | Frequency Band | Channel Model |
| --- | --- | --- | --- |
| S1 | Implant to Implant | 402-405 MHz | CM1 |
| S2 | Implant to Body Surface | 402-405 MHz | CM2 |
| S2.1 | Implant (upper body) to Body Surface | 3.1-10.6 GHz | CM2.1 |
| S2.2 | Implant (head) to Body Surface | 3.1-10.6 GHz | CM2.2 |
| S2.3 | Implant (head) to external | 3.1-10.6 GHz | CM2.3 |
| S3 | Implant to External | 402-405MHz | CM3 |
| S3.1 | Implant (upper body) to External | 3.1-10.6GHz | CM3.1 |
| S4 | Body Surface to Body Surface (LOS) | 400, 600, 900 MHz 2.4, 3.1-10.6 GHz | CM3 |
| S5 | Body Surface to Body Surface (NLOS) | 400, 600, 900 MHz 2.4, 3.1-10.6 GHz | CM5 |
| S6 | Body Surface to External (LOS) | 900MHz, 2.4 GHz, 3.1-10.6 GHz | CM4 |
| S6.1 | Body Surface (head) to External (LOS) | 3.1-10.6 GHz | CM6.1 |
| S6.2 | BAN coordinator to BAN coordinator | 3.1-10.6 GHz | CM6.2 |
| S7 | Body Surface to External (NLOS) | 900MHz , 2.4 GHz, 3.1-10.6 GHz | CM7 |

The present document describes the new channel models for scenarios S2.1, S2.2, S2.3, S3.1, S6.1and S6.2.

Scenario 2.1

The new scenario S2.1 is for the ingestible device (upper body) to body surface on the UWB band for applications such as capsule endoscopy.

—Scenario 2.1: Capsule endoscopy application

Scenario 2.2

The new scenario S2.2 is for the implant (head) to the body surface on the UWB band for applications such as the brain-computer interface (BCI). Note the transmitting antenna is integrated into the transmitter device over a titanium plaque that replaces the skull bone. In that way, the embedded transmitter is under the skin to avoid infection. Electrodes are implanted on the brain and join by a small cable to the transmitter device. The receiver may be located around the head (headband or other wearable) or torso.

The receiver could be located on the head by using head gear or something wearable to make easy to implementation of the antennas.

—Scenario 2.2: implant (head) in-body to body surface.

Scenario 2.3

The new scenario S2.3 is for the implant (head) to external on the UWB band for applications such as the brain-computer interface (BCI). Note the transmitting antenna is integrated into the transmitter device over a titanium plaque. In that way, the embedded transmitter is under the skin to avoid infection.

The receiver may be located within a range of 2m in LOS.

—Scenario 2.3: implant (head) to external.

Scenario 6.1 Body Surface (head) to External (LOS)

The new scenario 6.1 is for the body surface (head) to external on the UWB band for applications such as the brain-computer interface (BCI).

—Scenario 6.1: body surface (head) to external.

Scenario 6.2 HBAN coordinator to HBAN coordinator

The new scenario 6.2 covers HBAN coordinator to HBAN coordinator.

—Scenario 6.2: HBAN coordinator to HBAN coordinator.

Vehicle BAN scenarios and channel models

—15.6ma VBAN scenarios

| Scenario | Description | Frequency Band | Channel Model |
| --- | --- | --- | --- |
| S8 | In-vehicle to In-vehicle (sedan) | 2.4, 3.1-10.6 GHZ | CM8 |
| S8.1 | In-vehicle to In-vehicle (passenger bus) | 2.4, 3.1-10.6 GHZ | CM8.1 |
| S8.2 | In-vehicle to In-vehicle (functional compartment) | 2.4, 3.1-10.6 GHZ | CM8.2 |
| S9 | In-vehicle to On-vehicle | 2.4, 3.1-10.6 GHZ | CM9 |
| S10 | In vehicle to External | 2.4, 3.1-10.6 GHZ | CM10 |
| S11 | On-vehicle to on-vehicle (LOS) | 2.4, 3.1-10.6 GHZ | CM11 |
| S12 | On-vehicle to on-vehicle (NLOS) | 2.4, 3.1-10.6 GHZ | CM12 |
| S13 | On-vehicle to external (LOS) | 2.4, 3.1-10.6 GHZ | CM13 |
| S14 | On-vehicle to external (NLOS) | 2.4, 3.1-10.6 GHZ | CM14 |

—Channel models and scenarios for VBAN

Figure 7 shows an overview of models of VBAN applications.

—Scenario S8.1: In-vehicle to In-vehicle (bus)

Figure 8 shows scenario S8.1 which is targeting a use case in passenger bus. There are two scenarios S8.1(A) and S8.1(B). Scenario S8.1(A) illustrates the scenario VBAN to VBAN, while S8.1(B) illustrates the scenario of VBAN to HBAN.

—Scenario S8.2: In-vehicle to In-vehicle (functional compartment)

Figure 9 shows scenario S8.2, which can be used for the analysis of wireless connection between cabin’s room and the other functional compartments such as the engine compartment and cargo space   as shown in Figure 10.

—Scenario S8.2: In-vehicle to In-vehicle (functional compartment) ; Cargo vehicle use case

In order to reduce disconnection between devices in the  cabin’s room and functional space room, a coordinator may be placed in between these two the cabin’s room and cargo space.

Antenna Effect

An antenna placed on the surface or inside a body will be heavily influenced by its surroundings [2]  . The consequent changes in antenna pattern and other characteristics need to be understood and accounted for during any propagation measurement campaign.

The form factor of an antenna will be highly dependent on the requirements of the application. For MICS applications, for example, a circular antenna may be suitable for a pacemaker implant, while a helix antenna may be required for a stent or urinary implant. The form factor will affect the performance of the antenna and, the antenna performance will be very important to the overall system performance. Therefore, an antenna which has been designed with respect to the body tissues (or considered the effect of human body) shall be used for the channel model measurements [3]  .

Electrical antennas, such as dipole

Electrical antenna typically generates large components of E-field normal to the tissue interface, which overheat the fat tissue. This is because boundary conditions require the normal E-field at the interface to be discontinuous by the ratio of the permittivity, and since fat has a lower permittivity than muscle, the E-field in the fat tissue is higher.

Magnetic antennas, such as loop

Magnetic antenna produces an E-field mostly tangential to the tissue interface, which seem not to couple as strongly to the body as electrical antennas. Therefore, it does not overheat the fat.

There are antennas same as helical-coil, which is similar to a magnetic antenna in some respect, but its heating characteristics appear to be more like an electrical antenna. The strong E-field generated between the turns of coil is mainly responsible for tissue heating

It should be noted that SAR in the near field of the transmitting antenna depends mainly on the H-field; however, SAR in the far field of the transmitting antenna depends mainly on the E-field.

Channel Characterization

Electrical Properties of Body Tissues

The human body is not an ideal medium for radio frequency wave transmission. It is partially conductive and consists of materials of different dielectric constants, thickness, and characteristic impedance. Therefore, depending on the frequency of operation, the human body can lead to high losses caused by power absorption, central frequency shift, and radiation pattern destruction. The absorption effects vary in magnitude with both frequency of applied field and the characteristics of the tissue [[4]  [5]  [6]  [7]  ].

Model Types

In all cases, two types of models may be generated:

A theoretical or mathematical model including numerical simulation based modeling.

An empirical model

A theoretical model may be traceable back to the fundamental principles of electromagnetic propagation and will permit precise modeling of a specific situation at radio link level. It is intended for detailed exploration of, for example, the influence of body structures on antenna patterns. It will require a detailed description of the propagation environment and is therefore probably not suitable for modeling of macro environments.

An empirical model may be traceable to an agreed set of propagation measurements and is intended to provide a convenient basis for statistical modeling of the channel. Compared to the theoretical model, the empirical model will use a greatly simplified description of the environment and, although statistically accurate at network level, will not be precise at link level. Appropriate efforts could be made to ensure that the two sets of models are consistent with each other.

Fading

In the body area network communications, propagation paths can experience fading due to different reasons, such as energy absorption, reflection, diffraction, shadowing by body, and body posture. The other possible reason for fading is multipath due to the environment around the body. Fading can be categorized into two categories; small scale and large-scale fading.

Small-Scale Fading

Small scale fading refers to the rapid changes of the amplitude and phase of the received signal within a small local area due to small changes in location of the on-body device or body positions, in a given short period of time. The small-scale fading can be further divided into flat fading and frequency selective fading.

Averaging the attenuation between each antenna position on the body and each antenna location in the room will remove the effect of small scale fading due to small changes in the body position.

UWB Channel model

The UWB channel impulse responses are taken from the channel models document of former IEEE802.15.4a TG. Scenarios, bandwidths and central frequencies are similar to the ones targeted for IEEE802. 15.6ma TG. The MatLab code is attach in Annex A.

Note: Add what specific scenarios of 4a are used for 6ma (Marco)

TBC: with Marco and Kamran

Large-Scale Fading

Large scale fading refers to the fading due to motion over large areas; this is referring to the distance between antenna positions on the body and external node (home, office, or hospital).

Path loss

Unlike traditional wireless communications, the path loss for body area network system (on body applications), is both distance and frequency dependent. The frequency dependence of bod tissues shall be considered. The path loss model in dB between the transmitting and the receiving antennas as a function of the distance d based on the Friis formula in free space is described by [[10]  [11]  ]:

where PL0 is the path loss at a reference distance d0, and n is the path-loss exponent.

The path loss near the antenna depends on the separation between the antenna and the body due

to antenna mismatch. This mismatch indicates that a body-aware antenna design could improve

system performance.

Shadowing

Due to the variation in the environment surrounding of body or even movement of the body parts, path loss will be different from the mean value for a given distance as shown in equation (1). This phenomenon is called shadowing, and it reflects the path loss variation around the mean. The shadowing should be considered for stationary and non-stationary position of body.

When considering shadowing, the total path loss PL can be expressed by:

where PL(d) is expressed by the Equation (1) and S represents the shadowing component.

- Models and Scenarios

Human BAN (HBAN) Channel models

In-body

Implant to implant CM1 (Scenario S1) for 402 – 405 MHz

Scenario S1 is a model for the communication between implanted device to implanted device. Channel model for this scenario, Implant to Implant CM1 (Scenario S1) for 402-405 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Implant to body surface CM2 (Scenario S2) for 402 – 405 MHz

Scenario S2 is a model for the communication between implanted device to on-body device. Channel model for this scenario, Implant to body surface CM2 (Scenario S2) for 402-405 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Implant (upper body) to Body Surface CM2.1 (Scenario S2.1) for 3.1 – 10.6 GHz

The channel model for scenario S2.1 is intended for capsule endoscopy application. This application is a special case as the use of the UWB band varies from country to country for medical applications. Therefore, we provided two possibilities: a channel model for the low band of UWB (3.1 to 4.1 GHz) and a channel model for the entire UWB band (3.1 to 10.6 GHz). It is up to the implementer to choose the most appropriate channel model to test implementations.

—Scenario 2.1: Capsule endoscopy application

Capsule endoscopy use-case low band of UWB  (3.1 GHz – 4.1 GHz)

This model is developed based on doc.# P.802.15-22-0401-00-6a [12]  and reference [29].

Define PLd, f to be the pathloss in dB between the capsule antenna and an on-body receiver at distance d≥d0 and frequency fϵ3.1 4.1 GHz. The following log-distance statistical pathloss model is used to represent the capsule endoscope propagation channel:

PLd, f=PLd0, f+10nflog10dd0+Sf   d≥d0

where d0 is the reference distance (here chosen to be 5 cm), nf is the pathloss exponent at frequency f and Sf is the fading component at frequency f, modeling the deviation from the mean value due to different body tissues in the propagation path of the transmitted signal (e.g., bone, muscle, fat, etc.), receiver and transmitter positions and orientations and their antenna gains in different directions. A normal distribution with zero mean and standard deviation σs(f) (i.e., N(0,σs(f))) can be used to model the Sf component in the pathloss model (3). Variations of these parameters (PLd0, f, nf, σs(f)) versus frequency are shown in Figure 14.

(a)

(b)

(c)

—Parameters of the capsule pathloss model versus frequency: 
                         (a) PL (d0, f), (b) n(f), (c) σs (f)

. The mean value of these parameters (PL d0, n, σs)  over the frequency range 3.1 GHz to 4.1 GHz are provided in Table 4.

Mean values of the parameters of the capsule pathloss model 
over 3.1 GHz to 4.1 GHz

| PL(d0) (dB) | n | σS (dB) |
| --- | --- | --- |
| 51.9 | 8.14 | 18.19 |

MatLab™ code

function Path_Loss = pathloss_model ( d, f, N_point )

%============================================================

%Inputs

% d: distance (mm) 50mm <= d <= 250mm

% f: frequency (GHz) 3.1GHz <= f <=  4.1GHz

% N_point  : number of pathloss realizations

%============================================================

% Output

% Path_Loss : N_point realizations

%============================================================

freq_vector = [ 3.1, 3.2, 3.3, 3.4, 3.5, 3.6, 3.7, 3.8, 3.9, 4.0, 4.1 ];

PL_d0_vector = [ 49.4436041978323, 48.9083224250882, 49.2524918677139, 49.3371560091868, 50.2372056332785, 51.6031119805002, 52.3806016343197, 53.3694304010727, 54.3874602064049, 55.7889674636883, 56.2066985520886 ];

n_vector = [ 7.70453131966832, 7.82567248460083, 7.94410273207653, 8.25059339011392, 8.27965458898027, 8.20233411122469, 8.22248578163399, 8.17971826060753, 8.23993803137704, 8.25279392797003, 8.49732076058004 ];

sigma_vector = [ 16.4840688658356, 16.9010436657147, 17.3800004912188, 18.1486276565873, 18.2947077412370, 18.4339737105141, 18.3643456513965, 18.2687780146179, 18.8258822964394, 19.3414402253254, 19.6704396226310 ] ;

if(f<3.1 | f>4.1)

error('ERROR: Please specify proper frequency');

end

if(d<50 | d>250)

error('ERROR: Please specify proper distance');

end

d0=50; %mm

diff = abs ( freq_vector - f );

id = find(diff==min(diff));

PL_d0 = PL_d0_vector (id);

n = n_vector (id);

sigma_s = sigma_vector (id);

% Upper and lower limit settings for Shadowing as multiples

% of the standard deviation (i.e. uplimit & lowlimit)

% 1*Standard Deviation (68%); 2*Standard Deviation (95%);

% 3*Standard Deviation (99.7%)

% (Default Truncation is within 2 standard deviation)

uplimit=2; lowlimit=-uplimit;

S_normal=randn(1,N_point);

index=find(S_normal>uplimit);

S_normal(index)=uplimit;

index=find(S_normal<lowlimit);

S_normal(index)=lowlimit;

Path_Loss = PL_d0 + 10*n*log10(d/d0) + sigma_s*S_normal;

Capsule endoscopy use-case UWB band (3.1 GHz – 10.6 GHz)

This model is developed based on 15-23-0145-00-6a[14]  . Path Loss (PL) versus distance (d) can be represented by:

PLdB = PL0, dB + 10nlog10dd0+S,  ……….

where S is the fading component modeled with a normal distribution with zero mean and standard deviation σs , and the parameters that calculated by using FDTD simulations [14]  . There are two configurations of direction of the implanted antennas with horizontal arrangement and vertical arrangement (Figure 15). Horizontal arrangement and vertical arrangement are shown in Table 5(a) and Table 5(b) respectively.

(a) Horizontal arrangement                       (b) Vertical arrangement

—Implanted antenna arrangement

Implant to Body Surface CM2.1 for 3.1 – 10.6 GHz parameters (option 2)

(a) Horizontal arrangement                                       (b) Vertical arrangement

Implant (head) to Body Surface CM2.2 (Scenario S2.2) for 3.1 – 10.6 GHz

Scenario S2.2 is for the implant to the body surface on the UWB band for specific applications such as the brain-computer interface (BCI). Note the transmitting antenna is integrated into the transmitter device over a titanium plaque that replaces the skull bone. In that way, the embedded transmitter is under the skin to avoid infection.

– Scenario S2.2 implant (head) to body surface

Path loss model is defined as

PLdB = PL0, dB + 10nlog10dd0+S,

where S is the fading component modeled with a normal distribution with zero mean and standard deviation σs . Path loss parameters are simulated and provided in [13]  ,[16]  ,[17]  ,[18]  ,[19]  ,[20]  as follows.

OPTION 1: Pass loss model of Implanted device in head to head surface (Figure 17) channel has been derived as shown in Table 6.

Z

X

Y

– Scenario S2.2 implant (head) to body surface (head)

Implant (head) to Body Surface CM2.2 for 3.1 – 10.6 GHz parameters (head surface)

| Received components | d0 [m] | PL0 (dB) | n | σs [dB] |
| --- | --- | --- | --- | --- |
| X | 0.01 | 24.84 | 3.84 | 9.52 |
| Y |  | 3.78 | 6.80 | 7.27 |
| Z |  | 3.73 | 6.03 | 9.31 |

OPTION 2: Path loss model parameters of Implanted device in head to body surface (head, arm, and chest) (Figure 18) channel has been derived as shown in Table 7.

Z

X

Y

– Scenario S2.2 implant (head) to body surface (head, arm, and chest)

Implant (head) to Body Surface CM2.2 for 3.1 – 10.6 GHz parameters (body surface)

| Received components | d0 [m] | PL0 (dB) | n | σs [dB] |
| --- | --- | --- | --- | --- |
| X | 0.1 | 23.70 | 9.76 | 0.74 |
| Y |  | 102.62 | 0.57 | 8.15 |
| Z |  | 104.27 | 0.59 | 4.98 |

These parameters are valid in range of distance d < 36cm .

Implant (head) to External CM2.3 (Scenario S2.3) for 3.1 – 10.6 GHz

Scenario S2.3 is for the implant (head) to external on the UWB band for applications such as the brain-computer interface (BCI). Note the transmitting antenna is integrated into the transmitter device over a titanium plaque. In that way, the embedded transmitter is under the skin to avoid infection.

—Scenario 2.3: implant (head) to external.

This model is developed based on 15-23-0145-00-6a[14]  . Path Loss (PL) versus distance (d) can be represented by:

PLdB = PL0, dB + 10nlog10dd0+S,  ……….

where S is the fading component modeled with a normal distribution with zero mean and standard deviation σs , and the parameters that calculated by using FDTD simulations [14]  .

Implant (head) to external CM2.3 for 3.1 – 10.6 GHz parameters

| Received components | d0 [m] | PL0 (dB) | n | σs [dB] |
| --- | --- | --- | --- | --- |
| X | 0.01 | 48.47 | 1.45 | 9.63 |
| Y |  | 19.13 | 2.67 | 2.72 |
| Z |  | 22.81 | 2.69 | 7.55 |

These parameters are valid in range of distance d < 36cm .

Implant (upper body) to External CM3 (Scenario 3) for 402 – 405 MHz

Scenario S3 is a model for the communication between implanted device in upper body to external body device. Channel model for 402 – 405 MHz of this scenario, Implant to external CM2 (Scenario S3) for 402-405 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Implant (upper body) to External CM3.1 (Scenario 3.1) for 3.1 – 10.6 GHz

Scenario S3.1 is a channel model for the implanted device to external on the UWB band for applications such as the capsule endoscopy. Channel models for implant(upper body) to external was provided by simulation result [●●●]  . Simulation was performed by using human 3D model [●●] and pathloss characteristics has been calculated as follows.

Implant (capsule endoscopy) to external CM3.1 for 3.1 – 10.6 GHz parameters

| Received components | d0 [m] | PL0 (dB) | n | σs [dB] |
| --- | --- | --- | --- | --- |
| X | 0.05 | 49.79 | 16.15 | 11.10 |
| Y |  | 46.77 | 17.85 | 11.55 |
| Z |  | 49.97 | 16.82 | 10.74 |

Path Loss (PL) versus distance (d) can be represented by:

PLdB = PL0, dB + 10nlog10dd0+S,  ……….

where S is the fading component modeled with a normal distribution with zero mean and standard deviation σs . This path loss model is valid in range of distance d < 99cm .

Body surface

Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 400 MHz

Scenario S4 is a model for the communication between a device on body surface to body surface device. Channel model for this scenario, Body surface to body surface CM3 (Scenario S4 & S5) for 400 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 600 MHz

Scenario S4 is a model for the communication between a device on body surface to body surface device. Channel model for this scenario, Body surface to body surface CM3 (Scenario S4 & S5) for 600 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 900 MHz

Scenario S4 is a model for the communication between a device on body surface to body surface device. Channel model for this scenario, Body surface to body surface CM3 (Scenario S4 & S5) for 900 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Body surface to body surface (LOS) CM3 (Scenario S4 & S5) for 2.4 GHz

Scenario S4 is a model for the communication between a device on body surface to body surface device. Channel model for this scenario, Body surface to body surface CM3 (Scenario S4 & S5) for 2.4 GHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Body surface to body surface CM3 (Scenario S4 & S5) for 3.1-10.6 GHz

The following path loss model is based on measurements that cover frequencies of 3.1-10.6 GHz. Measurement set up, derivation and data analysis can be found in [21]  . The table below summarizes the model and corresponding parameters.

Parameters of the path loss model for CM3 (S4 & S5) for 3.1 – 10.6 GHz

|  | Hospital Room | Anechoic Chamber |
| --- | --- | --- |
| Path loss model | PL(d)[dB] = a log10(d) + b + N |  |
| a | 19.2 | 34.1 |
| b | 3.38 | -31.4 |
| σN | 4.40 | 4.85 |

- a and b : Coefficients of linear fitting

- d : Tx -Rx distance in mm

- N : Normally distributed variable with zero mean and standard deviation σN

A power delay profile (PDP) model for 3.1 – 10.6 GHz is also given in [21]  . The table below summarizes this model and corresponding parameter.

Parameters of the PDP model for CM3 (S4 & S5) for 3.1 – 10.6 GHz

| PDP Model | φl is modeld by a uinform distribution over [0,2π) |  |
| --- | --- | --- |
| al | γ0 | -4.60 dB |
|  | Γ | 59.7 |
|  | σs | 5.02 dB |
| tl | 1/λ | 1.85 ns |
| L | L | 38.1 |

Option 2

The following path loss model is based on measurements that cover frequencies of 3.1-10.6 GHz. Measurement set up, derivation and data analysis can be found in [22]  . The table below summarizes the corresponding parameters.

Parameters of the path loss model for CM3 (S4 & S5) (Option 2)

| Path loss model |  |  |
| --- | --- | --- |
| Around torso |  |  |
| Antenna separation from body surface | 0 mm | 5 mm |
| P0 [dB] | 56.1 | 48.4 |
| d0 [m] | 0.1 | 0.1 |
| n | 5.8 | 5.9 |
| Along torso |  |  |
| Antenna separation from body surface | 0 mm | 5 mm |
| P0 [dB] | 56.5 | 44.6 |
| d0 [m] | 0.1 | 0.1 |

Dynamic channel model for body surface to body surface CM3 (Scenario S4 & S5) at 4.5 GHz

A real time channel measurements by use of channel sounder has been performed in [24]  . The channel measurements were carried out in an anechoic chamber with center frequency of 4.5 GHz and bandwidth of 120 MHz. Details of the measurement set up, derivation and data analysis can be found in [24]  . The measurements are focusing on the fading effect due to movements of the human body, therefore conducted in an anechoic chamber, where the multipath from the surrounding objects are negligible.

The transmitter antenna was fixed on around navel. The table below summarize the position of receiving antennas and distance between Tx and Rx antennas.

Summary of the position of receiving antennas and distance between Tx and Rx antennas for CM3 (Scenario S4 & S5) at 4.5 GHz

| Position | Distance [mm] |
| --- | --- |
| A Right wrist | 440 ~ 525 |
| B Right upper arm | 360 |
| C Left ear | 650 |
| D Head | 710 |
| E Shoulder | 310 |
| F Chest | 230 |
| G Right rib | 183 |
| H Left waist | 140 |
| I Thigh | 340 |
| J Ankle | 815 ~ 940 |

To provide a statistical model, the probability distribution functions such as normal, log-normal and Weibull distributions have been tried to fill the measurement results. The tables below summarize the normal, log-normal and Weibull distributions.

Summary of the normal distributions parameter for CM3 (Scenario S4 & S5) at 4.5 GHz

Summary of the log-normal distribution parameters for CM3 
(Scenario S4 & S5) at 4.5 GHz

- μ: Mean value

- σ: Variable value

- -LogL: Negative log loklihood

Summary of the Weibull distribution parameters for CM3 
(Scenario S4 & S5) at 4.5 GHz

- a : Scale factor

- b: Shape factor

- -LogL: Negative log loklihood

The below table summarize the best fitting distributions for dynamic channel at 4.5 GHz.

Summary of the best fitting distributions for dynamic channel parameters for CM3 (Scenario S4 & S5) at 4.5 GHz

| Position | Still | Walking | Up-down |
| --- | --- | --- | --- |
| Right wrist | Normal | Weibull | Weibull |
| Right upper arm | Log-normal | Weibull | Weibull |
| Right ear | Normal | Log-normal | Weibull |
| Head | weibull | Log-normal | Log-normal |
| Shoulder | Log-normal | Weibull | Weibull |
| Chest | Log-normal | Log-normal | Weibull |
| Right rib | Log-normal | Log-normal | Weibull |
| Left waist | Normal | Log-normal | Weibull |
| Right thigh | Log-normal | Log-normal | Weibull |
| Right ankle | Log-normal | Weibull | Weibull |

- Normal distribution seems to fit the still posture best, but it can be seen that fittings with any distributions have large from the PDFs.

- Log-normal distribution shows good match in cases of still postures and small movements such as walking posture in case of head , right ear, chest, right rib, left waist, right thigh, and stand up/down posture in case of head.

- Weibull distribution can represent much better large movement behaviors such as walking posture in case of right wrist, right upper arm, shoulder, right ankle, and all stand up/down postures except for head.

Based on statistical analysis of fading duration a 5-state Fritchman model for dynamic on-body channels, as shown in the below Figure, has been used to classifies channel states according to the dwelling time (ϴ) in different channel qualities to quantitatively describe the time-varying property of on-body channels.

Five-state Fritchman model for dynamic on-body channels

The five-state Fritchman model for describing the burst behaviors of on-body channels are:

- ST1: unstable error-free state, good channels which last less than 20 ms;

- ST2: semi-constant error-free state, good channel which are over 20 ms and less than 400

- ms;

- ST3: constant error-free state, good channel which are over 400 ms;

- ST4: unstable error state, bad channel which last less than 20 ms, and

- ST5: semi-constant error state, bad channel which are less than 400 ms.

The statistical parameters for the on-body dynamic channel at ϴ=-10dB for different locations are;

statistical parameters for the on-body dynamic channel

The statistical parameters for the on-body dynamic channel at ϴ=-10dB in different states are

— statistical parameters for the on-body dynamic channel

Parameters of the 5-state Fritchman model for different action scenarios are

Parameters of the five-state Fritchman model for different action scenarios

Body surface to external CM4 (Scenario S6 & S7) for 900MHz

Channel model for Body surface to external CM4 (Scenario S6 & S7) for 900 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]  .

Body surface to external CM4 (Scenario S6 & S7) for 2.4 GHz

Channel model for Body surface to external CM4 (Scenario S6 & S7) for 2.4 GHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]  .

Body surface to external CM4 (Scenario S6 & S7) for 3.1-10.6 GHz

Measurement for the UWB frequency band of 3.1-10.6 GHz has been performed in [23]  . On body antenna characteristics were measured in anechoic chamber, while channel measurements were done in office environment. For this measurement, the Tx antenna is fixed near to wall, while the Rx antenna (placed on body) positions were changed in office area. The effect of ground is considered in measurements. All data were averaged for statically analysis; therefore, the detail data of each measurement is not provided. The Further detail on set-up, derivation and data analysis can be found in [23]  . The following figures summarize the delay profile of front, side, and backside of body while the Tx antenna is in the front of body.

Parameters of the impulse response model for CM4 (Scenario S6 & S7) for 3.1-10.6 GHz

Body surface (head) to external CM6.1 (Scenario S6.1) for 3.1 – 10.6 GHz

Scenario S6.1 is for the implant to the body surface on the UWB band for specific applications of brain-computer interface (BCI).

– Scenario 6.1: body surface (head) to external.

– Scenario 6.1: On-body(head) antenna location.

Path loss model is defined as

⁡PLdB = PL0, dB + 10nlog10dd0+S,

where S is the fading component modeled with a normal distribution with zero mean and standard deviation σs . Path loss parameters are simulated and provided in,[16]  as follows.

Pass loss model parameters of on body (head) device to external (Figure 23) channel has been derived as shown in Table 22. These parameters are valid in range of distance d < 61cm .

– External device locations (Top view of human body)

5.3.10 Body surface (head) to external CM6.1 (Scenario S6.1) for 3.1 – 10.6 GHz parameters

| Received components | d0 [m] | PL0 (dB) | n | σs [dB] |
| --- | --- | --- | --- | --- |
| X | 0.01 | 40.26 | 1.65 | 6.19 |
| Y |  | 23.09 | 2.25 | 3.85 |
| Z |  | 22.12 | 2.66 | 5.81 |

BAN coordinator to BAN coordinator CM6.2 (Scenario S6.2) for 3.1 – 10.6 GHz

— HBAN coordinator to HBAN coordinator (Scenario 6.2, CM 6.2)

Channel models for HBAN coordinator to HBAN coordinator was provided by simulation result [14]  . Simulation was performed by using simplified human 3D model and pathloss characteristics has been calculated as follows.

BAN coordinator to BAN coordinator model parameters in S6.2

|  | Model type | n | PL0 (dB) (d0=1000 (mm)) |
| --- | --- | --- | --- |
| S6.2 | LOS | 1.26 | 60.70 |
|  | NLOS | 0.03 | 68.36 |

This model is valid in range of disc d < 10m.

Body surface to external CM4 (Scenario S7) for 900 MHz

Scenario S7 is a model for the communication between a device on body surface to external device. Channel model for this scenario, Body surface to external CM4 (Scenario S6 & S7) for 900 MHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Body surface to external CM4 (Scenario S7) for 2.4 GHz

Scenario S7 is a model for the communication between a device on body surface to external device. Channel model for this scenario, Body surface to external CM4 (Scenario S6 & S7) for 2.4 GHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Body surface to external CM4 (Scenario S7) for 3.1 – 10.6 GHz

Scenario S7 is a model for the communication between a device on body surface to external device.  Channel model for this scenario, Body surface to external CM4 (Scenario S6 & S7) for 3.1 – 10.6 GHz which is defined in IEEE 802.15.6 – 2012 Channel Model Document [1]   can be applicable.

Vehicle BAN (VBAN) channel models

VBAN channel models are defined as four cases; in-vehicle to in-vehicle, on-vehicle to on-vehicle and on-vehicle to surrounding vehicle cases.

The considered channel models are based on the channel model parameters contribution for VBAN [15]  .

Scenarios 8 to 14 channel model parameters was calculated based on ray-tracing method simulations using simplified 3D model as shown in Figure 25.

—Simplified car 3D model used for simulation of Scenario 8 to 14

In Vehicle models

In-vehicle to In-vehicle (Scenario 8, CM 8) 3.1 - 10.6GHz

—In-vehicle to In-vehicle (Scenario 8, CM 8)

Channel models for In-vehicle to In-vehicle was provided by the ray-tracing simulation result[15]  . Simulation was performed by using simplified car 3D model and pathloss characteristics has been calculated as follows.

In-vehicle to In-vehicle pathloss model parameters in S8

Cabin room to cabin room in omnibus (Scenario 8.1) 3.1 - 10.6GHz

In large vehicle like omnibus type of vehicle, multiple HBAN is in the same cabin room. In such situation, channel model CM8.1 can be applied to analyze communication between VBAN and multiple HBANs.

—Scenario8.1 for Omnibus HBANs to VBAN connection

In multiple passengers on a vehicle such as omnibus, VBAN coordinator is connected with multiple VBAN nodes and also connected with HBAN coordinator as well as HBAN nodes that is used passengers personally. In this situation, channel models are defined based on simulation results [13]  and [15]  .

OPTION 1

This channel model can be used to estimate pathloss in situation that transmitter (or node) is on passenger’s body surface and receiver (or coordinator) is on the other passenger’s body surface. Also, this channel model can be applicable to use to estimate pathloss between transmitter (or node) to receiver (or coordinator) on inner surface of bus body. This channel model has been derived based on a ray-tracing method simulation with simplified bus and human body tissue models[13]  [15]   . This model provides simulated path-loss in omnibus environment.

—simplified omnibus 3D models for ray-tracing simulation

—transmitter location for 2 persons model

—transmitter location for 10 persons model

—transmitter and receiver positions

On-vehicle to on-vehicle pathloss model parameters in S8.1 (Option 1)

OPTION 2

Additionally, following channel models can be used for the use case that node or coordinator supposed to be located at the wall side of bus, or driver’s sheet as shown in Figure 32.

—transmitter positions

This channel model can be represented as follows.

On-vehicle to on-vehicle pathloss model parameters in S8.1 (Option 2)

| d0 = 1000 (mm) |  |  |
| --- | --- | --- |
| Tx | Driver | Getting on |
| n | 1.01 | 1.19 |
| PL0 [dB] | 70.29 | 71.82 |

In-vehicle to In-vehicle (functional compartment) (Scenario 8.2, CM 2)

— 6.5.1.1 Engine compartment to engine compartment (Scenario 8.2, CM 8.2)

Channel models for in functional compartment such as engine compartment of the vehicle is defined separately. The model was provided by the ray-tracing simulation result[15]  . Simulation was performed by using simplified car 3D model and pathloss characteristics has been calculated.

— 6.5.1.1 Scenario 8.2 simulation model

From the results of simulations, pathloss model is defined as follows.

In-vehicle to In-vehicle (functional compartment) (Scenario 8.2, CM 8.2) pathloss parameters

| d0 = 250 (mm) |  |
| --- | --- |
| Tx | Engine |
| n | 8.0×10-5 |
| PL0 [dB] | 54.18 |

On vehicle

On-vehicle to on-vehicle (Scenario 11, 12, CM11, 12)

—On-vehicle to on-vehicle (Scenario 11, 12, CM 11, 12)

Channel models for on-vehicle to on-vehicle was provided by the ray-tracing simulation result[15]  . Simulation was performed by using simplified car 3D model and pathloss characteristics has been calculated as follows.

On-vehicle to on-vehicle pathloss model parameters in S11 & S12

| Scenario | S11 (LOS) | S12 (NLOS) |
| --- | --- | --- |
| n | 2.76 | 0.39 |
| PL0 [dB] | 67.08 | 88.85 |

In-vehicle to on vehicle (Scenario 9, CM9) 3.1 – 10.6 GHz

―In-vehicle to on vehicle (Scenario 9, CM9) 3.1 – 10.6 GHz

Channel models for In-vehicle to on-vehicle was provided by the ray-tracing simulation result [15]  . Simulation was performed by using simplified car 3D model and pathloss characteristics has been calculated as follows.

In-vehicle to On-vehicle pathloss model parameters in S9

| Scenario | S9 |
| --- | --- |
| n | 1.65 |
| PL0 [dB] | 60.537 |

External

In-vehicle to External (Scenario 10, CM 10) 3.1 – 10.6GHz

—In-vehicle to external (Scenario 10, CM10) 3.1 – 10.6 GHz

Channel models for In-vehicle to external was provided by the ray-tracing simulation result [15]  . Simulation was performed by using simplified car 3D model and pathloss characteristics has been calculated as follows.

In-vehicle to external pathloss model parameters in S10

| Scenario | S10 |
| --- | --- |
| n | 2.97 |
| PL0 [dB] | 64.041 |

On-vehicle to External (LOS) (Scenario 13, CM 13) 3.1 – 10.6GHz

— On-vehicle to External (LOS) (Scenario 13, CM 13) 3.1 – 10.6GHz

Channel models for on-vehicle to external (LOS) was provided by the ray-tracing simulation result  [15]  . Simulation was performed by using simplified car 3D model and pathloss characteristics has been calculated as follows.

On-vehicle to external (LOS) pathloss model parameters in S13

| Scenario | S13(LOS) |
| --- | --- |
| n | 1.30 |
| PL0 [dB] | 66.00 |

On-vehicle to External (NLOS) (Scenario 14, CM 14) 3.1 – 10.6GHz

— On-vehicle to External (NLOS) (Scenario 14, CM 14) 3.1 – 10.6GHz

Channel models for on-vehicle to external (NLOS) was provided by the ray-tracing simulation result [15]  . Simulation was performed by using simplified car 3D model and pathloss characteristics has been calculated as follows.

On-vehicle to external (NLOS) pathloss model parameters in S14

| Scenario | S14(NLOS) |
| --- | --- |
| n | 0.83 |
| PL0 [dB] | 87.78 |

MATLAB code program

Path loss model

The The following MATLAB code can be used to generate path loss model for UWB communication in scenarios S2.1, S2.2, S2.3, S3.1, S6.1 and S6.2 for HBAN. The MATLAB code is based on Nagoya Institute of Technology’s simulation results [13]  [14]  [16]   and provided by Nagoya Institute of Technology.

This MATLAB code can be used for the calculation of path loss for UWB band in VBAN scenarios and channel models such as S8, S8.1 and S8.2 as well as the HBAN models by replacing the channel parameters PL_0, n, and sigma_s  in following code.

%% 1.

%%Written by Sho Asano, Daisuke Anzai on 3/15/2023

N_ch = 100; % # of path loss realizations

distance = 5; % unit: mm (the distance should be at least 10 mm.)

Transmitting_direction = 1; % 1: Position A (Front-back direction), 2: Position B (Left-right direction)

Transmitting_antenna = 1; % 1: Inside the head, 2: Outside the head

Received_components = 1; % 1: X-axis, 2: Y-axis, 3: Z-axis

Receiving_antenna = 1; % 1: Whole head, 2: Scalp, 3: Arm, 4: Torso, 5: Off-body

if distance < 10

fprintf("Warning!: The distance should be at least 10 mm.\n")

end

% =================== Channel parameter ================

if Transmitting_direction == 1

if Transmitting_antenna == 1

if Received_components == 1

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 24.84;  n = 3.84;  sigma_s = 9.52;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 27.13;  n = 3.36;  sigma_s = 10.18;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 23.70;  n = 9.76;  sigma_s = 0.74;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 69.31;  n = 3.85;  sigma_s = 3.02;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 48.47;  n = 1.45;  sigma_s = 9.63;

end

elseif Received_components == 2

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 3.78;  n = 6.80;  sigma_s = 7.27;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 11.44;  n = 5.32;  sigma_s = 6.96;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 102.62;  n = 0.57;  sigma_s = 8.15;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 50.24;  n = 2.76;  sigma_s = 0.65;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 19.13;  n = 2.67;  sigma_s = 2.72;

end

elseif Received_components == 3

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 3.73;  n = 6.03;  sigma_s = 9.31;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 17.58;  n = 3.31;  sigma_s = 7.52;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 104.27;  n = 0.59;  sigma_s = 4.98;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 27.52;  n = 8.61;  sigma_s = 1.98;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 22.81;  n = 2.69;  sigma_s = 7.55;

end

end

elseif Transmitting_antenna == 2

if Received_components == 1

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 32.21;  n = 2.55;  sigma_s = 12.94;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 33.07;  n = 2.40;  sigma_s = 14.17;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 60.60;  n = 2.01;  sigma_s = 1.10;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 70.72;  n = 5.04;  sigma_s = 2.69;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 40.26;  n = 1.65;  sigma_s = 6.19;

end

elseif Received_components == 2

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 10.72;  n = 5.63;  sigma_s = 8.02;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 14.08;  n = 4.89;  sigma_s = 7.88;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 90.00;  n = 0.66;  sigma_s = 10.08;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 59.62;  n = 3.02;  sigma_s = 0.61;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 23.09;  n = 2.25;  sigma_s = 3.85;

end

elseif Received_components == 3

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 13.00;  n = 4.62;  sigma_s = 8.86;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 18.04;  n = 3.52;  sigma_s = 8.75;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 87.49;  n = 0.45;  sigma_s = 5.35;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 32.56;  n = 9.25;  sigma_s = 1.61;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 22.12;  n = 2.66;  sigma_s = 5.81;

end

end

end

elseif Transmitting_direction == 2

if Transmitting_antenna == 1

if Received_components == 1

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 20.37;  n = 3.80;  sigma_s = 7.47;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 10.97;  n = 5.63;  sigma_s = 6.89;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 51.42;  n = 3.14;  sigma_s = 1.13;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 16.68;  n = 10.59;  sigma_s = 2.42;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 0.05;  n = 4.95;  sigma_s = 2.66;

end

elseif Received_components == 2

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 19.60;  n = 4.96;  sigma_s = 7.59;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 25.39;  n = 3.86;  sigma_s = 7.31;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 80.65;  n = 1.99;  sigma_s = 11.42;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 67.02;  n = 3.72;  sigma_s = 1.80;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 34.94;  n = 2.74;  sigma_s = 5.65;

end

elseif Received_components == 3

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 13.36;  n = 4.54;  sigma_s = 7.09;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 19.77;  n = 3.34;  sigma_s = 7.03;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 76.35;  n = 1.85;  sigma_s = 5.12;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 62.07;  n = 6.87;  sigma_s = 2.61;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 44.11;  n = 0.56;  sigma_s = 5.84;

end

end

elseif Transmitting_antenna == 2

if Received_components == 1

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 17.71;  n = 4.01;  sigma_s = 7.33;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 14.01;  n = 4.84;  sigma_s = 7.51;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 51.22;  n = 3.28;  sigma_s = 1.09;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 15.68;  n = 11.30;  sigma_s = 2.58;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = -3.03;  n = 5.25;  sigma_s = 4.76;

end

elseif Received_components == 2

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 26.20;  n = 4.19;  sigma_s = 12.94;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 33.17;  n = 2.65;  sigma_s = 12.88;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 78.08;  n = 2.63;  sigma_s = 12.69;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 56.25;  n = 4.89;  sigma_s = 1.52;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 45.33;  n = 1.24;  sigma_s = 6.31;

end

elseif Received_components == 3

if Receiving_antenna == 1

d_0 = 10;

PL_0 = 20.14;  n = 4.09;  sigma_s = 8.89;

elseif Receiving_antenna == 2

d_0 = 10;

PL_0 = 23.42;  n = 3.39;  sigma_s = 9.35;

elseif Receiving_antenna == 3

d_0 = 100;

PL_0 = 77.09;  n = 1.87;  sigma_s = 5.07;

elseif Receiving_antenna == 4

d_0 = 100;

PL_0 = 36.17;  n = 10.07;  sigma_s = 1.61;

elseif Receiving_antenna == 5

d_0 = 10;

PL_0 = 42.50;  n = 0.80;  sigma_s = 4.96;

end

end

end

end

% ======================================================

% =============== Path loss realizations ===============

% -- Path loss in dB

PL=PL_0+10*n*log10(distance/d_0)+sigma_s*randn(1,N_ch);

% ======================================================

Bibliography

- Kamya Yekeh Yazdandoost, Kamran Sayrafian-Pour, “Channel Model for Body Area Network (BAN),” IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs), IEEE P802.15-08-0780-12-0006, Nov., 2012.

- W.-T. Chen; H.-R. Chuang, “Numerical computation of human interaction with arbitrarily oriented superquadric loop antennas in personal communications,” IEEE Trans. on Antenna and Propagation, vol.46, no. 6, pp. 821-828, June 1998.

- Kamya Y. Yazdandoost and Ryuji Kohno, “The Effect of Human Body on UWB BAN Antennas,” IEEE802.15-07-0546-00-0ban.

- C. H. Duney, H. Massoudi, and M. F. Iskander, “Radiofrequency radiation dosimetry handbook,” USAF School of Aerospace Medicine, October 1986.

- C. Gabriel and S. Gabriel, “Compilation of the dielectric properties of body tissues at RF and microwave frequencies,” AL/OE-TR-1996-0037, June 1996, http://www.brooks.af.mil/AFRL/HED/hedr/reports/dielectricReport/Report.html.

- Italian National Research Council, Institute for Applied Physics, “Dielectric properties of body tissues,” http://niremf.ifac.cnr.it

- P. Gandhi, “.Biological effects and medical applications of electromagnetic energy,” Prentice Hall, Englewood Cliffs, N.J., 1990.

- Marco Hernandez, Huan-Bang Li, Igor Dotlić, Ryu Miura, “Channel Models for TG8,” IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs), IEEE P802. 15-12-0459-07-0008, Sept., 2012

- 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access; Base Station radio transmission and reception

- E. Reusens, W. Joseph, G. Vermeeren, and L. Martens, „On-body measurements and characterization of wireless communication channel fro arm and torso of human,“ International Workshop on Wearable and Implantabel Body Sensor Networks, BSN07, Achen, March 2007, pp. 26-28.

- A. Fort, J. Ryckaert, C. Desset, P. De Doncker, P. Wambacq, and L. Van Biesen, “ Ultrawideband channel model for communication around the human body, ” IEEE Journal on Selected Areas in Communications, vol. 24, pp.927-933, April 2006.

- Kamran Sayrafian, “Statistical Pathloss Model for UWB Wireless Capsule Endoscopy,” doc.# P.802.15-22-0401-00-6a, 2022.

- Daisuke Anzai, “Propagation characteristics of UWB communication applications including medical implants, BCI and passengers bus,” doc.# P.802. 15-22-0469-00-006a, 2022.

- Daisuke Anzai, Sho Asano, “Propagation Channel Parameters of UWB Communication Applications for Human BAN (HBAN) Use Cases,” doc.# P.802.15-23-0145-00-006a, 2023.

- Daisuke Anzai, Sho Asano, “Propagation Channel Parameters of UWB Communication Applications for Vehicle BAN (VBAN) Use Cases,” doc.# P.802.15-23-0146-00-006a, 2023.

- Daisuke Anzai, “Propagation Characteristics of UWB Communication Applications for Human BAN (HBAN) Use Cases, doc.# P.802. 15-23-0018-00-006a, 2023.

- D. Anzai, I. Balasingham, G. Fischer, J. Wang, "Reliable and High-Speed Implant Ultra-Wideband Communications with Transmit-Receive Diversity," EAI/Springer Innovations in Communication and Computing, pp. 27-32, March 2020.

- Y. Shimizu, D. Anzai, R. C-Santiago, P. A. Floor, I. Balasingham, and J. Wang, "Performance evaluation of an ultra-wideband transmit diversity in a living animal experiment" IEEE Trans. Microw. Theory Tech., vol. 65, no. 7, pp. 2596-2606, July 2017.

- D. Anzai, K. Katsu, R. Chavez-Santiago, Q. Wang, D. Plettemeier, J. Wang, and I. Balasingham, "Experimental evaluation of implant UWB-IR transmission with living animal for body area networks," IEEE Trans. Microw. Theory Tech., vol. 62, no. 1, pp. 183-192, Jan. 2014.

- J. Shi, D. Anzai, and J. Wang, "Channel modeling and performance analysis of diversity reception for implant UWB wireless link," IEICE Trans. Commun., no. E95-B, vol. 10, pp. 3197-3205, Oct. 2012.

- Takahiro Aoyagi, Jun-ichi Takada, Kenichi Takizawa, Norihiko Katayama, Takehiko Kobayashi, Kamya Yekeh Yazdandoost, Huan-bang Li and Ryuji Kohno, “Channel model for wearable and implantable WBANs,” IEEE 802.15-08-0416-04-0006, November 2008.

- Guido Dolmans and Andrew Fort, “Channel models WBAN-Holst centre/IMEC-NL,” IEEE 802.15-08-0418-01-0006, July 2008.

- Hirokazu Sawada, Takahiro Aoyagi, Jun-ichi Takada, Kamya Yekeh Yazdandoost, Ryuji Kohno, “Channel model between body surface and wireless access point for UWB band,” IEEE 802.15-08-0576-00-0006, August 2008.

- Minseok Kim, Jun-ichi Takada, Bin Zhen, Lawrence Materum, Tomoshige Kan, Yuuki Terao, yohei Konishi, Kenji Nakai, Takahiro Aoyagi, Ryuji Kohno, “Statistical property of dynamic BAN channel gain at 4.5 GHz,” IEEE 802-.15-08-0489-02-0006, June 2010.

- Andreas F. Molisch, Kannan Balakrishnan, Dajana Cassioli, Chia-Chin Chong, Shahriar Emami, Andrew Fort,Johan Karedal, Juergen Kunisch, Hans Schantz, Ulrich Schuster, Kai Siwiak, “IEEE 802.15.4a channel model - final report,”

- J. Karedal, S. Wyne, P. Almers, F. Tufvesson, and A. F. Molisch, “Statistical analysis of the uwb channel in an industrial environment,” in Proc. VTC fall 2004, 2004.

- T. S. Rappaport 1989.

- W. Pietsch 1994.

- K. Ladic, K. Sayrafian, D. Šimunić and K. Y. Yazdandoost, "Wireless Channel Characterization for UWB Communication in Capsule Endoscopy," in IEEE Access, 
doi: 10.1109/ACCESS.2023.3310216.

(informative)
Matlab code for IEEE802.15.4a UWB channel model

% modified S-V channel model evaluation

%%Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2005

clear;

no_output_files = 1; % non-zero: avoids writing output files of continuous-time responses

num_channels = 100; % number of channel impulse responses to generate

randn(’state’,12); % initialize state of function for repeatability

rand(’state’,12); % initialize state of function for repeatability

cm_num = 6; % channel model number from 1 to 8

% get channel model params based on this channel model number

[Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km, ...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs] = uwb_sv_params_15_4a( cm_num );

fprintf(1,[’Model Parameters\n’ ...

’ Lam = %.4f, Lmean = %.4f, lambda_mode(FLAG) = %d\n’ ...

’ lambda_1 = %.4f, lambda_2 = %.4f, beta = %.4f\n’ ...

’ Gam = %.4f, gamma0 = %.4f, Kgamma = %.4f, sigma_cluster = %.4f\n’ ...

’ nlos(FLAG) = %d, gamma_rise = %.4f, gamma_1 = %.4f, chi = %.4f\n’ ...

’ m0 = %.4f, Km = %.4f, sigma_m0 = %.4f, sigma_Km = %.4f\n’ ...

’ sfading_mode(FLAG) = %d, m0_sp = %.4f, std_shdw = %.4f\n’, ...

’ kappa = %.4f, fc = %.4fGHz, fs = %.4fGHz\n’], ...

Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km,...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs);

ts = 1/fs; % sampling frequency

% get a bunch of realizations (impulse responses)

[h_ct,t_ct,t0,np] = uwb_sv_model_ct_15_4a(Lam,Lmean,lambda_mode,lambda_1, ...

lambda_2,beta,Gam,gamma_0,Kgamma,sigma_cluster,nlos,gamma_rise,gamma_1, ...

chi,m0,Km,sigma_m0,sigma_Km,sfading_mode,m0_sp,std_shdw,num_channels,ts);

% change to complex baseband channel

h_ct_len = size(h_ct, 1);

phi = zeros(h_ct_len, 1);

for k = 1:num_channels

phi = rand(h_ct_len, 1).*(2*pi);

h_ct(:,k) = h_ct(:,k) .* exp(phi .* i);

end

% now reduce continuous-time result to a discrete-time result

[hN,N] = uwb_sv_cnvrt_ct_15_4a( h_ct, t_ct, np, num_channels, ts );

if N > 1,

h = resample(hN, 1, N); % decimate the columns of hN by factor N

else

h = hN;

end

% add the frequency dependency

[h]= uwb_sv_freq_depend_ct_15_4a(h,fc,fs,num_channels,kappa);

%********************************************************************

% Testing and ploting

%********************************************************************

% channel energy

channel_energy = sum(abs(h).^2);

h_len = length(h(:,1));

t = [0:(h_len-1)] * ts; % for use in computing excess & RMS delays

excess_delay = zeros(1,num_channels);

RMS_delay = zeros(1,num_channels);

num_sig_paths = zeros(1,num_channels);

num_sig_e_paths = zeros(1,num_channels);

for k=1:num_channels

% determine excess delay and RMS delay

sq_h = abs(h(:,k)).^2 / channel_energy(k);

t_norm = t - t0(k); % remove the randomized arrival time of first cluster

excess_delay(k) = t_norm * sq_h;

RMS_delay(k) = sqrt( ((t_norm-excess_delay(k)).^2) * sq_h );

% determine number of significant paths (paths within 10 dB from peak)

threshold_dB = -10; % dB

temp_h = abs(h(:,k));

temp_thresh = 10^(threshold_dB/20) * max(temp_h);

num_sig_paths(k) = sum(temp_h > temp_thresh);

% determine number of sig. paths (captures x % of energy in channel)

x = 0.85;

temp_sort = sort(temp_h.^2); % sorted in ascending order of energy

cum_energy = cumsum(temp_sort(end:-1:1)); % cumulative energy

index_e = min(find(cum_energy >= x * cum_energy(end)));

num_sig_e_paths(k) = index_e;

end

energy_mean = mean(10*log10(channel_energy));

energy_stddev = std(10*log10(channel_energy));

mean_excess_delay = mean(excess_delay);

mean_RMS_delay = mean(RMS_delay);

mean_sig_paths = mean(num_sig_paths);

mean_sig_e_paths = mean(num_sig_e_paths);

fprintf(1,’Model Characteristics\n’);

fprintf(1,’ Mean delays: excess (tau_m) = %.1f ns, RMS (tau_rms) = %1.f\n’, ...

mean_excess_delay, mean_RMS_delay);

fprintf(1,’ # paths: NP_10dB = %.1f, NP_85%% = %.1f\n’, ...

mean_sig_paths, mean_sig_e_paths);

fprintf(1,’ Channel energy: mean = %.1f dB, std deviation = %.1f dB\n’, ...

energy_mean, energy_stddev);

figure(1); clf; plot(t, abs(h)); grid on

title(’Impulse response realizations’)

xlabel(’Time (nS)’)

figure(2); clf; plot([1:num_channels], excess_delay, ’b-’, ...

[1 num_channels], mean_excess_delay*[1 1], ’r–’ );

grid on

title(’Excess delay (nS)’)

xlabel(’Channel number’)

figure(3); clf; plot([1:num_channels], RMS_delay, ’b-’, ...

[1 num_channels], mean_RMS_delay*[1 1], ’r–’ );

grid on

title(’RMS delay (nS)’)

xlabel(’Channel number’)

figure(4); clf; plot([1:num_channels], num_sig_paths, ’b-’, ...

[1 num_channels], mean_sig_paths*[1 1], ’r–’);

grid on

title(’Number of significant paths within 10 dB of peak’)

xlabel(’Channel number’)

figure(5); clf; plot([1:num_channels], num_sig_e_paths, ’b-’, ...

[1 num_channels], mean_sig_e_paths*[1 1], ’r–’);

grid on

title(’Number of significant paths capturing > 85% energy’)

xlabel(’Channel number’)

temp_average_power = sum((abs(h))’.*(abs(h))’, 1)/num_channels;

temp_average_power = temp_average_power/max(temp_average_power);

average_decay_profile_dB = 10*log10(temp_average_power);

threshold_dB = -40;

above_threshold = find(average_decay_profile_dB > threshold_dB);

ave_t = t(above_threshold);

apdf_dB = average_decay_profile_dB(above_threshold);

figure(6); clf; plot(ave_t, apdf_dB); grid on

title(’Average Power Decay Profile’)

xlabel(’Delay (nsec)’)

ylabel(’Average power (dB)’)

if no_output_files,

return

end

%**************************************************************************

%Savinge the data

%**************************************************************************

%%% save continuous-time (time,value) pairs to files

save_fn = sprintf(’cm%d_imr’, cm_num);

% A complete self-contained file for Matlab users

save([save_fn ’.mat’], ’t’, ’h’,’t_ct’, ’h_ct’, ’t0’, ’np’, ’num_channels’, ’cm_num’);

% Three comma-delimited text files for non-Matlab users:

% File #1: cmX_imr_np.csv lists the number of paths in each realization

dlmwrite([save_fn ’_np.csv’], np, ’,’); % number of paths

% File #2: cmX_imr_ct.csv can open with Excel

% n’th pair of columns contains the (time,value) pairs for the n’th realization

% save continous time data

th_ct = zeros(size(t_ct,1),3*size(t_ct,2));

th_ct(:,1:3:end) = t_ct; % time

th_ct(:,2:3:end) = abs(h_ct); % magnitude

th_ct(:,3:3:end) = angle(h_ct); % phase (radians)

fid = fopen([save_fn ’_ct.csv’], ’w’);

if fid < 0,

error(’unable to write .csv file for impulse response, file may be open in another application’);

end

for k = 1:size(th_ct,1)

fprintf(fid,’%.4f,%.6f,’, th_ct(k,1:end-2));

fprintf(fid,’%.4f,%.6f\r\n’, th_ct(k,end-1:end)); % \r\n for Windoze end-of-line

end

fclose(fid);

% File #3: cmX_imr_dt.csv can open with Excel

% discrete channel impulse response magnitude and phase pair realization.

% the first column is time. phase is in radians

% save discrete time data

th = zeros(size(h,1),2*size(h,2)+1);

th(:,1) = t’; % the first column is time scale

th(:,2:2:end) = abs(h); % even columns are magnitude

th(:,3:2:end) = angle(h); % odd columns are phase

fid = fopen([save_fn ’_dt.csv’], ’w’);

if fid < 0,

error(’unable to write .csv file for impulse response, file may be open in another application’);

end

for k = 1:size(th,1)

fprintf(fid,’%.4f,%.6f,’, th(k,1:end-2));

fprintf(fid,’%.4f,%.6f\r\n’, th(k,end-1:end)); % \r\n for Windoze end-of-line

end

fclose(fid);

return; % end of program

function [Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km, ...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs] = uwb_sv_params_15_4a( cm_num )

% Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2004

% Return modified S-V model parameters for standard UWB channel models

%————————————————————————–

% Lam Cluster arrival rate (clusters per nsec)

% Lmean Mean number of Clusters

% lambda_mode Flag for Mixture of poission processes for ray arrival times

% 1 -> Mixture of poission processes for the ray arrival times

% 2 -> tapped delay line model

% lambda_1 Ray arrival rate for Mixture of poisson processes (rays per nsec)

% lambda_2 Ray arrival rate for Mixture of poisson processes (rays per nsec)

% beta Mixture probability

%————————————————————————–

% Gam Cluster decay factor (time constant, nsec)

% gamma0 Ray decay factor (time constant, nsec)

% Kgamma Time dependence of ray decay factor

% sigma_cluster Standard deviation of normally distributed variable for cluster energy

% nlos Flag for non line of sight channel

% 0 -> LOS

% 1 -> NLOS with first arrival path starting at t ~= 0

% 2 -> NLOS with first arrival path starting at t = 0 and diffused first cluster

% gamma_rise Ray decay factor of diffused first cluster (time constant, nsec)

% gamma_1 Ray decay factor of diffused first cluster (time constant, nsec)

% chi Diffuse weight of diffused first cluster

%————————————————————————–

% m0 Mean of log-normal distributed nakagami-m factor

% Km Time dependence of m0

% sigma_m0 Standard deviation of log-normal distributed nakagami-m factor

% sigma_Km Time dependence of sigma_m0

% sfading_mode Flag for small-scale fading

% 0 -> All paths have same m-factor distribution

% 1 -> LOS first path has a deterministic large m-factor

% 2 -> LOS first path of each cluster has a deterministic

% large m-factor

% m0_sp Deterministic large m-factor

%————————————————————————–

% std_shdw Standard deviation of log-normal shadowing of entire impulse response

%————————————————————————–

% kappa Frequency dependency of the channel

%————————————————————————–

% fc Center Frequency

% fs Frequency Range

%%

modified by I2R

if cm_num == 1, % Residential LOS

% MPC arrival

Lam = 0.047; Lmean = 3;

lambda_mode = 1;

lambda_1 = 1.54; lambda_2 = 0.15; beta = 0.095;

%MPC decay

Gam = 22.61; gamma_0 = 12.53; Kgamma = 0; sigma_cluster = 2.75;

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.67; Km = 0; sigma_m0 = 0.28; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN;

% Large-scale Fading – Shadowing

std_shdw = 2.22;

% Frequency Dependence

kappa = 1.12;

fc = 6; % GHz

fs = 8; % 2 - 10 GHz

elseif cm_num == 2, % Residential NLOS

% MPC arrival

Lam = 0.12; Lmean = 3.5;

lambda_mode = 1;

lambda_1 = 1.77; lambda_2 = 0.15; beta = 0.045;

%MPC decay

Gam = 26.27; gamma_0 = 17.5; Kgamma = 0; sigma_cluster = 2.93;

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.69; Km = 0; sigma_m0 = 0.32; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN;

% Large-scale Fading – Shadowing

std_shdw = 3.51;

% Frequency Dependence

kappa = 1.53;

fc = 6; % GHz

fs = 8; % 2 - 10 GHz

elseif cm_num == 3, % Office LOS

% MPC arrival

Lam = 0.016; Lmean = 5.4;

lambda_mode = 1;

lambda_1 = 0.19; lambda_2 = 2.97; beta = 0.0184;

%MPC decay

Gam = 14.6; gamma_0 = 6.4; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.42; Km = 0; sigma_m0 = 0.31; sigma_Km = 0;

sfading_mode = 2; m0_sp = 3; % assumption

% Large-scale Fading – Shadowing

std_shdw = 0; %1.9;

% Frequency Dependence

kappa = 0.03;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 4, % Office NLOS

% MPC arrival

Lam = 0.19; Lmean = 3.1;

lambda_mode = 1;

lambda_1 = 0.11; lambda_2 = 2.09; beta = 0.0096;

%MPC decay

Gam = 19.8; gamma_0 = 11.2; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 2;

gamma_rise = 15.21; gamma_1 = 11.84; chi = 0.78;

% Small-scale Fading

m0 = 0.5; Km = 0; sigma_m0 = 0.25; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % assumption

% Large-scale Fading – Shadowing

std_shdw = 3.9;

% Frequency Dependence

kappa =0.71;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 5, % Outdoor LOS

% MPC arrival

Lam = 0.0448; Lmean = 13.6;

lambda_mode = 1;

lambda_1 = 0.13; lambda_2 = 2.41; beta = 0.0078;

%MPC decay

Gam = 31.7; gamma_0 = 3.7; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.77; Km = 0; sigma_m0 = 0.78; sigma_Km = 0;

sfading_mode = 2; m0_sp = 3; % assumption

% Large-scale Fading – Shadowing

std_shdw = 0.83;

% Frequency Dependence

kappa = 0.12;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 6, % Outdoor NLOS

% MPC arrival

Lam = 0.0243; Lmean = 10.5;

lambda_mode = 1;

lambda_1 = 0.15; lambda_2 = 1.13; beta = 0.062;

%MPC decay

Gam = 104.7; gamma_0 = 9.3; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.56; Km = 0; sigma_m0 = 0.25; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % assumption

% Large-scale Fading – Shadowing

std_shdw = 2; % assumption

% Frequency Dependence

kappa = 0.13;

fc =6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 7, % Industrial LOS

% MPC arrival

Lam = 0.0709; Lmean = 4.75;

lambda_mode = 2;

lambda_1 = 1; lambda_2 = 1; beta = 1; % dummy in this scenario

%MPC decay

Gam = 13.47; gamma_0 = 0.615; Kgamma = 0.926; sigma_cluster = 4.32;

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.36; Km = 0; sigma_m0 = 1.13; sigma_Km = 0;

sfading_mode = 1; m0_sp = 12.99;

% Large-scale Fading – Shadowing

std_shdw = 6;

% Frequency Dependence

kappa = -1.103;

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

elseif cm_num == 8, % Industrial NLOS

% MPC arrival

Lam = 0.089; Lmean = 1;

lambda_mode = 2;

lambda_1 = 1; lambda_2 = 1; beta = 1; % dummy in this scenario

%MPC decay

Gam = 5.83; gamma_0 = 0.3; Kgamma = 0.44; sigma_cluster = 2.88;

nlos = 2;

gamma_rise = 47.23; gamma_1 = 84.15; chi = 0.99;

% Small-scale Fading

m0 = 0.3; Km = 0; sigma_m0 = 1.15; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % m0_sp is assumption

% Large-scale Fading – Shadowing

std_shdw = 6;

% Frequency Dependence

kappa = -1.427;

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

elseif cm_num == 9, % Open Outdoor Environment NLOS (Fram, Snow-Covered Open Area)

% MPC arrival

Lam = 0.0305; Lmean = 3.31;

lambda_mode = 1;

lambda_1 = 0.0225; lambda_2 = 1; beta = 1;

%MPC decay

Gam = 56; gamma_0 = 0.92; Kgamma = 0; sigma_cluster = 3; % sigma_cluster is assumption

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN;

% Small-scale Fading

m0 = 4.1; Km = 0; sigma_m0 = 2.5; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % m0_sp is assumption

% Large-scale Fading – Shadowing

std_shdw = 3.96;

% Frequency Dependence

kappa = -1; % Kappa is assumption

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

else

error(’cm_num is wrong!!’)

end

return

function [h]= uwb_sv_freq_depend_ct_15_4a(h,fc,fs,num_channels,kappa)

% This function is used to include the frequency dependency

f0 = 5; % GHz

h_len = length(h(:,1));

f = [fc-fs/2 : fs/h_len/2 : fc+fs/2]./f0;

f = f.^(-2*(kappa));

f = [f(h_len : 2*h_len), f(1 : h_len-1)]’;

i = (-1)^(1/2); % complex i

for c = 1:num_channels

% add the frequency dependency

h2 = zeros(2*h_len, 1);

h2(1 : h_len) = h(:,c); % zero padding

fh2 = fft(h2);

fh2 = fh2 .* f;

h2 = ifft(fh2);

h(:,c) = h2(1:h_len);

% Normalize the channel energy to 1

h(:,c) = h(:,c)/sqrt(h(:,c)’ * h(:,c) );

end

return

function [h,t,t0,np] = uwb_sv_model_ct_15_4a(Lam,Lmean,lambda_mode,lambda_1, ...

lambda_2,beta,Gam,gamma_0,Kgamma,sigma_cluster,nlos,gamma_rise,gamma_1, ...

chi,m0,Km,sigma_m0,sigma_Km,sfading_mode,m0_sp,std_shdw,num_channels,ts)

% Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2005

% IEEE 802.15.4a UWB channel model for PHY proposal evaluation

% continuous-time realization of modified S-V channel model

% Input parameters:

% detailed introduction of input parameters is at uwb_sv_params.m

% num_channels number of random realizations to generate

% Outputs

% h is returned as a matrix with num_channels columns, each column

% holding a random realization of the channel model (an impulse response)

% t is organized as h, but holds the time instances (in nsec) of the paths whose

% signed amplitudes are stored in h

% t0 is the arrival time of the first cluster for each realization

% np is the number of paths for each realization.

% Thus, the k’th realization of the channel impulse response is the sequence

% of (time,value) pairs given by (t(1:np(k),k), h(1:np(k),k))

%%

modified by I2R

% initialize and precompute some things

std_L = 1/sqrt(2*Lam); % std dev (nsec) of cluster arrival spacing

std_lam_1 = 1/sqrt(2*lambda_1);

std_lam_2 = 1/sqrt(2*lambda_2);

% std_lam = 1/sqrt(2*lambda); % std dev (nsec) of ray arrival spacing

h_len = 1000; % there must be a better estimate of # of paths than this

ngrow = 1000; % amount to grow data structure if more paths are needed

h = zeros(h_len,num_channels);

t = zeros(h_len,num_channels);

t0 = zeros(1,num_channels);

np = zeros(1,num_channels);

for k = 1:num_channels % loop over number of channels

tmp_h = zeros(size(h,1),1);

tmp_t = zeros(size(h,1),1);

if nlos == 1,

Tc = (std_L*randn)^2 + (std_L*randn)^2; % First cluster random arrival

else

Tc = 0; % First cluster arrival occurs at time 0

end

t0(k) = Tc;

if nlos == 2 & lambda_mode == 2

L = 1; % for industrial NLOS environment

else

L = max(1, poissrnd(Lmean)); % number of clusters

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tcval = []; Tc_cluster= [];

Tc_cluster(1,1)=Tc;

for i_Tc=2:L+1

Tc_cluster(1,i_Tc)= Tc_cluster(1,i_Tc-1)+(std_L*randn)^2 + (std_L*randn)^2;

end

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

cluster_index = zeros(1,L);

path_ix = 0;

nak_m = [];

for ncluster = 1:L

% Determine Ray arrivals for each cluster

Tr = 0; % first ray arrival defined to be time 0 relative to cluster

cluster_index(ncluster) = path_ix+1; % remember the cluster location

gamma = Kgamma*Tc + gamma_0; % delay dependent cluster decay time

if nlos == 2 & ncluster == 1

gamma = gamma_1;

end

Mcluster = sigma_cluster*randn;

Pcluster = 10*log10(exp(-1*Tc/Gam))+Mcluster; % total cluster power

Pcluster = 10^(Pcluster*0.1);

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tr_len=Tc_cluster(1,ncluster+1)-Tc_cluster(1,ncluster);

else

Tr_len = 10*gamma;

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

while (Tr < Tr_len),

t_val = (Tc+Tr); % time of arrival of this ray

if nlos == 2 & ncluster == 1

% equation (22)

h_val = Pcluster*(1-chi*exp(-Tr/gamma_rise))*exp(-Tr/gamma_1) ...

*(gamma+gamma_rise)/gamma/(gamma+gamma_rise*(1-chi));

else

% equation (19)

h_val = Pcluster/gamma*exp(-Tr/gamma)/(beta*lambda_1+(1-beta)*lambda_2+1);

end

path_ix = path_ix + 1; % row index of this ray

if path_ix > h_len,

% grow the output structures to handle more paths as needed

tmp_h = [tmp_h; zeros(ngrow,1)];

tmp_t = [tmp_t; zeros(ngrow,1)];

h = [h; zeros(ngrow,num_channels)];

t = [t; zeros(ngrow,num_channels)];

h_len = h_len + ngrow;

end

tmp_h(path_ix) = h_val;

tmp_t(path_ix) = t_val;

% if lambda_mode == 0

% Tr = Tr + (std_lam*randn)^2 + (std_lam*randn)^2;

if lambda_mode == 1

if rand < beta

Tr = Tr + (std_lam_1*randn)^2 + (std_lam_1*randn)^2;

else

Tr = Tr + (std_lam_2*randn)^2 + (std_lam_2*randn)^2;

end

elseif lambda_mode == 2

Tr = Tr + ts;

else

error(’lambda mode is wrong!’)

end

% generate log-normal distributed nakagami m-factor

m_mu = m0 - Km*t_val;

m_std = sigma_m0 - sigma_Km*t_val;

nak_m = [nak_m, lognrnd(m_mu, m_std)];

end

Tc = Tc + (std_L*randn)^2 + (std_L*randn)^2;

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tc = Tc_cluster(1,ncluster+1);

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

end

% change m value of the first multipath to be the deterministic value

if sfading_mode == 1

nak_ms(cluster_index(1)) = m0_sp;

elseif sfading_mode == 2

nak_ms(cluster_index) = m0_sp;

end

% apply nakagami

for path = 1:path_ix

h_val = (gamrnd(nak_m(path), tmp_h(path)/nak_m(path))).^(1/2);

tmp_h(path) = h_val;

end

np(k) = path_ix; % number of rays (or paths) for this realization

[sort_tmp_t,sort_ix] = sort(tmp_t(1:np(k))); % sort in ascending time order

t(1:np(k),k) = sort_tmp_t;

h(1:np(k),k) = tmp_h(sort_ix(1:np(k)));

% now impose a log-normal shadowing on this realization

% fac = 10^(std_shdw*randn/20) / sqrt( h(1:np(k),k)’ * h(1:np(k),k) );

% h(1:np(k),k) = h(1:np(k),k) * fac;

end

return

function [hN,N] = uwb_sv_cnvrt_ct_15_4a( h_ct, t, np, num_channels, ts )

% convert continuous-time channel model h_ct to N-times oversampled discrete-time samples

% h_ct, t, np, and num_channels are as specified in uwb_sv_model

% ts is the desired time resolution

%%hN will be produced with time resolution ts /

N.

% It is up to the user to then apply any filtering and/or complex downconversion and then

% decimate by N to finally obtain an impulse response at time resolution ts.

min_Nfs = 100; % GHz

N = max( 1, ceil(min_Nfs*ts) ); % N*fs = N/ts is the intermediate sampling frequency before decimation

N = 2^nextpow2(N); % make N a power of 2 to facilitate efficient multi-stage decimation

Nfs = N / ts;

t_max = max(t(:)); % maximum time value across all channels

h_len = 1 + floor(t_max * Nfs); % number of time samples at resolution ts / N

hN = zeros(h_len,num_channels);

for k = 1:num_channels

np_k = np(k); % number of paths in this channel

t_Nfs = 1 + floor(t(1:np_k,k) * Nfs); % vector of quantized time indices for this channel

for n = 1:np_k

hN(t_Nfs(n),k) = hN(t_Nfs(n),k) + h_ct(n,k);

end

end

(informative)
MATLAB script for the IEEE 802.15.4a channel model

% modified S-V channel model evaluation

%%Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2005

clear;

no_output_files = 1; % non-zero: avoids writing output files of continuous-time responses

num_channels = 100; % number of channel impulse responses to generate

randn(’state’,12); % initialize state of function for repeatability

rand(’state’,12); % initialize state of function for repeatability

cm_num = 6; % channel model number from 1 to 8

% get channel model params based on this channel model number

[Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km, ...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs] = uwb_sv_params_15_4a( cm_num );

fprintf(1,[’Model Parameters\n’ ...

’ Lam = %.4f, Lmean = %.4f, lambda_mode(FLAG) = %d\n’ ...

’ lambda_1 = %.4f, lambda_2 = %.4f, beta = %.4f\n’ ...

’ Gam = %.4f, gamma0 = %.4f, Kgamma = %.4f, sigma_cluster = %.4f\n’ ...

’ nlos(FLAG) = %d, gamma_rise = %.4f, gamma_1 = %.4f, chi = %.4f\n’ ...

’ m0 = %.4f, Km = %.4f, sigma_m0 = %.4f, sigma_Km = %.4f\n’ ...

’ sfading_mode(FLAG) = %d, m0_sp = %.4f, std_shdw = %.4f\n’, ...

’ kappa = %.4f, fc = %.4fGHz, fs = %.4fGHz\n’], ...

Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km,...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs);

ts = 1/fs; % sampling frequency

% get a bunch of realizations (impulse responses)

[h_ct,t_ct,t0,np] = uwb_sv_model_ct_15_4a(Lam,Lmean,lambda_mode,lambda_1, ...

lambda_2,beta,Gam,gamma_0,Kgamma,sigma_cluster,nlos,gamma_rise,gamma_1, ...

chi,m0,Km,sigma_m0,sigma_Km,sfading_mode,m0_sp,std_shdw,num_channels,ts);

% change to complex baseband channel

h_ct_len = size(h_ct, 1);

phi = zeros(h_ct_len, 1);

for k = 1:num_channels

phi = rand(h_ct_len, 1).*(2*pi);

h_ct(:,k) = h_ct(:,k) .* exp(phi .* i);

end

% now reduce continuous-time result to a discrete-time result

[hN,N] = uwb_sv_cnvrt_ct_15_4a( h_ct, t_ct, np, num_channels, ts );

if N > 1,

h = resample(hN, 1, N); % decimate the columns of hN by factor N

else

h = hN;

end

% add the frequency dependency

[h]= uwb_sv_freq_depend_ct_15_4a(h,fc,fs,num_channels,kappa);

%********************************************************************

% Testing and ploting

%********************************************************************

% channel energy

channel_energy = sum(abs(h).^2);

h_len = length(h(:,1));

t = [0:(h_len-1)] * ts; % for use in computing excess & RMS delays

excess_delay = zeros(1,num_channels);

RMS_delay = zeros(1,num_channels);

num_sig_paths = zeros(1,num_channels);

num_sig_e_paths = zeros(1,num_channels);

for k=1:num_channels

% determine excess delay and RMS delay

sq_h = abs(h(:,k)).^2 / channel_energy(k);

t_norm = t - t0(k); % remove the randomized arrival time of first cluster

excess_delay(k) = t_norm * sq_h;

RMS_delay(k) = sqrt( ((t_norm-excess_delay(k)).^2) * sq_h );

% determine number of significant paths (paths within 10 dB from peak)

threshold_dB = -10; % dB

temp_h = abs(h(:,k));

temp_thresh = 10^(threshold_dB/20) * max(temp_h);

num_sig_paths(k) = sum(temp_h > temp_thresh);

% determine number of sig. paths (captures x % of energy in channel)

x = 0.85;

temp_sort = sort(temp_h.^2); % sorted in ascending order of energy

cum_energy = cumsum(temp_sort(end:-1:1)); % cumulative energy

index_e = min(find(cum_energy >= x * cum_energy(end)));

num_sig_e_paths(k) = index_e;

end

energy_mean = mean(10*log10(channel_energy));

energy_stddev = std(10*log10(channel_energy));

mean_excess_delay = mean(excess_delay);

mean_RMS_delay = mean(RMS_delay);

mean_sig_paths = mean(num_sig_paths);

mean_sig_e_paths = mean(num_sig_e_paths);

fprintf(1,’Model Characteristics\n’);

fprintf(1,’ Mean delays: excess (tau_m) = %.1f ns, RMS (tau_rms) = %1.f\n’, ...

mean_excess_delay, mean_RMS_delay);

fprintf(1,’ # paths: NP_10dB = %.1f, NP_85%% = %.1f\n’, ...

mean_sig_paths, mean_sig_e_paths);

fprintf(1,’ Channel energy: mean = %.1f dB, std deviation = %.1f dB\n’, ...

energy_mean, energy_stddev);

figure(1); clf; plot(t, abs(h)); grid on

title(’Impulse response realizations’)

xlabel(’Time (nS)’)

figure(2); clf; plot([1:num_channels], excess_delay, ’b-’, ...

[1 num_channels], mean_excess_delay*[1 1], ’r–’ );

grid on

title(’Excess delay (nS)’)

xlabel(’Channel number’)

figure(3); clf; plot([1:num_channels], RMS_delay, ’b-’, ...

[1 num_channels], mean_RMS_delay*[1 1], ’r–’ );

grid on

title(’RMS delay (nS)’)

xlabel(’Channel number’)

figure(4); clf; plot([1:num_channels], num_sig_paths, ’b-’, ...

[1 num_channels], mean_sig_paths*[1 1], ’r–’);

grid on

title(’Number of significant paths within 10 dB of peak’)

xlabel(’Channel number’)

figure(5); clf; plot([1:num_channels], num_sig_e_paths, ’b-’, ...

[1 num_channels], mean_sig_e_paths*[1 1], ’r–’);

grid on

title(’Number of significant paths capturing > 85% energy’)

xlabel(’Channel number’)

temp_average_power = sum((abs(h))’.*(abs(h))’, 1)/num_channels;

temp_average_power = temp_average_power/max(temp_average_power);

average_decay_profile_dB = 10*log10(temp_average_power);

threshold_dB = -40;

above_threshold = find(average_decay_profile_dB > threshold_dB);

ave_t = t(above_threshold);

apdf_dB = average_decay_profile_dB(above_threshold);

figure(6); clf; plot(ave_t, apdf_dB); grid on

title(’Average Power Decay Profile’)

xlabel(’Delay (nsec)’)

ylabel(’Average power (dB)’)

if no_output_files,

return

end

%**************************************************************************

%Savinge the data

%**************************************************************************

%%% save continuous-time (time,value) pairs to files

save_fn = sprintf(’cm%d_imr’, cm_num);

% A complete self-contained file for Matlab users

save([save_fn ’.mat’], ’t’, ’h’,’t_ct’, ’h_ct’, ’t0’, ’np’, ’num_channels’, ’cm_num’);

% Three comma-delimited text files for non-Matlab users:

% File #1: cmX_imr_np.csv lists the number of paths in each realization

dlmwrite([save_fn ’_np.csv’], np, ’,’); % number of paths

% File #2: cmX_imr_ct.csv can open with Excel

% n’th pair of columns contains the (time,value) pairs for the n’th realization

% save continous time data

th_ct = zeros(size(t_ct,1),3*size(t_ct,2));

th_ct(:,1:3:end) = t_ct; % time

th_ct(:,2:3:end) = abs(h_ct); % magnitude

th_ct(:,3:3:end) = angle(h_ct); % phase (radians)

fid = fopen([save_fn ’_ct.csv’], ’w’);

if fid < 0,

error(’unable to write .csv file for impulse response, file may be open in another application’);

end

for k = 1:size(th_ct,1)

fprintf(fid,’%.4f,%.6f,’, th_ct(k,1:end-2));

fprintf(fid,’%.4f,%.6f\r\n’, th_ct(k,end-1:end)); % \r\n for Windoze end-of-line

end

fclose(fid);

% File #3: cmX_imr_dt.csv can open with Excel

% discrete channel impulse response magnitude and phase pair realization.

% the first column is time. phase is in radians

% save discrete time data

th = zeros(size(h,1),2*size(h,2)+1);

th(:,1) = t’; % the first column is time scale

th(:,2:2:end) = abs(h); % even columns are magnitude

th(:,3:2:end) = angle(h); % odd columns are phase

fid = fopen([save_fn ’_dt.csv’], ’w’);

if fid < 0,

error(’unable to write .csv file for impulse response, file may be open in another application’);

end

for k = 1:size(th,1)

fprintf(fid,’%.4f,%.6f,’, th(k,1:end-2));

fprintf(fid,’%.4f,%.6f\r\n’, th(k,end-1:end)); % \r\n for Windoze end-of-line

end

fclose(fid);

return; % end of program

function [Lam,Lmean,lambda_mode,lambda_1,lambda_2,beta,Gam,gamma_0,Kgamma, ...

sigma_cluster,nlos,gamma_rise,gamma_1,chi,m0,Km,sigma_m0,sigma_Km, ...

sfading_mode,m0_sp,std_shdw,kappa,fc,fs] = uwb_sv_params_15_4a( cm_num )

% Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2004

% Return modified S-V model parameters for standard UWB channel models

%————————————————————————–

% Lam Cluster arrival rate (clusters per nsec)

% Lmean Mean number of Clusters

% lambda_mode Flag for Mixture of poission processes for ray arrival times

% 1 -> Mixture of poission processes for the ray arrival times

% 2 -> tapped delay line model

% lambda_1 Ray arrival rate for Mixture of poisson processes (rays per nsec)

% lambda_2 Ray arrival rate for Mixture of poisson processes (rays per nsec)

% beta Mixture probability

%————————————————————————–

% Gam Cluster decay factor (time constant, nsec)

% gamma0 Ray decay factor (time constant, nsec)

% Kgamma Time dependence of ray decay factor

% sigma_cluster Standard deviation of normally distributed variable for cluster energy

% nlos Flag for non line of sight channel

% 0 -> LOS

% 1 -> NLOS with first arrival path starting at t ~= 0

% 2 -> NLOS with first arrival path starting at t = 0 and diffused first cluster

% gamma_rise Ray decay factor of diffused first cluster (time constant, nsec)

% gamma_1 Ray decay factor of diffused first cluster (time constant, nsec)

% chi Diffuse weight of diffused first cluster

%————————————————————————–

% m0 Mean of log-normal distributed nakagami-m factor

% Km Time dependence of m0

% sigma_m0 Standard deviation of log-normal distributed nakagami-m factor

% sigma_Km Time dependence of sigma_m0

% sfading_mode Flag for small-scale fading

% 0 -> All paths have same m-factor distribution

% 1 -> LOS first path has a deterministic large m-factor

% 2 -> LOS first path of each cluster has a deterministic

% large m-factor

% m0_sp Deterministic large m-factor

%————————————————————————–

% std_shdw Standard deviation of log-normal shadowing of entire impulse response

%————————————————————————–

% kappa Frequency dependency of the channel

%————————————————————————–

% fc Center Frequency

% fs Frequency Range

%%

modified by I2R

if cm_num == 1, % Residential LOS

% MPC arrival

Lam = 0.047; Lmean = 3;

lambda_mode = 1;

lambda_1 = 1.54; lambda_2 = 0.15; beta = 0.095;

%MPC decay

Gam = 22.61; gamma_0 = 12.53; Kgamma = 0; sigma_cluster = 2.75;

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.67; Km = 0; sigma_m0 = 0.28; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN;

% Large-scale Fading – Shadowing

std_shdw = 2.22;

% Frequency Dependence

kappa = 1.12;

fc = 6; % GHz

fs = 8; % 2 - 10 GHz

elseif cm_num == 2, % Residential NLOS

% MPC arrival

Lam = 0.12; Lmean = 3.5;

lambda_mode = 1;

lambda_1 = 1.77; lambda_2 = 0.15; beta = 0.045;

%MPC decay

Gam = 26.27; gamma_0 = 17.5; Kgamma = 0; sigma_cluster = 2.93;

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.69; Km = 0; sigma_m0 = 0.32; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN;

% Large-scale Fading – Shadowing

std_shdw = 3.51;

% Frequency Dependence

kappa = 1.53;

fc = 6; % GHz

fs = 8; % 2 - 10 GHz

elseif cm_num == 3, % Office LOS

% MPC arrival

Lam = 0.016; Lmean = 5.4;

lambda_mode = 1;

lambda_1 = 0.19; lambda_2 = 2.97; beta = 0.0184;

%MPC decay

Gam = 14.6; gamma_0 = 6.4; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.42; Km = 0; sigma_m0 = 0.31; sigma_Km = 0;

sfading_mode = 2; m0_sp = 3; % assumption

% Large-scale Fading – Shadowing

std_shdw = 0; %1.9;

% Frequency Dependence

kappa = 0.03;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 4, % Office NLOS

% MPC arrival

Lam = 0.19; Lmean = 3.1;

lambda_mode = 1;

lambda_1 = 0.11; lambda_2 = 2.09; beta = 0.0096;

%MPC decay

Gam = 19.8; gamma_0 = 11.2; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 2;

gamma_rise = 15.21; gamma_1 = 11.84; chi = 0.78;

% Small-scale Fading

m0 = 0.5; Km = 0; sigma_m0 = 0.25; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % assumption

% Large-scale Fading – Shadowing

std_shdw = 3.9;

% Frequency Dependence

kappa =0.71;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 5, % Outdoor LOS

% MPC arrival

Lam = 0.0448; Lmean = 13.6;

lambda_mode = 1;

lambda_1 = 0.13; lambda_2 = 2.41; beta = 0.0078;

%MPC decay

Gam = 31.7; gamma_0 = 3.7; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.77; Km = 0; sigma_m0 = 0.78; sigma_Km = 0;

sfading_mode = 2; m0_sp = 3; % assumption

% Large-scale Fading – Shadowing

std_shdw = 0.83;

% Frequency Dependence

kappa = 0.12;

fc = 6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 6, % Outdoor NLOS

% MPC arrival

Lam = 0.0243; Lmean = 10.5;

lambda_mode = 1;

lambda_1 = 0.15; lambda_2 = 1.13; beta = 0.062;

%MPC decay

Gam = 104.7; gamma_0 = 9.3; Kgamma = 0; sigma_cluster = 3; % assumption

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.56; Km = 0; sigma_m0 = 0.25; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % assumption

% Large-scale Fading – Shadowing

std_shdw = 2; % assumption

% Frequency Dependence

kappa = 0.13;

fc =6; % GHz

fs = 8; % 3 - 6 GHz

elseif cm_num == 7, % Industrial LOS

% MPC arrival

Lam = 0.0709; Lmean = 4.75;

lambda_mode = 2;

lambda_1 = 1; lambda_2 = 1; beta = 1; % dummy in this scenario

%MPC decay

Gam = 13.47; gamma_0 = 0.615; Kgamma = 0.926; sigma_cluster = 4.32;

nlos = 0;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN; % dummy in this scenario

% Small-scale Fading

m0 = 0.36; Km = 0; sigma_m0 = 1.13; sigma_Km = 0;

sfading_mode = 1; m0_sp = 12.99;

% Large-scale Fading – Shadowing

std_shdw = 6;

% Frequency Dependence

kappa = -1.103;

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

elseif cm_num == 8, % Industrial NLOS

% MPC arrival

Lam = 0.089; Lmean = 1;

lambda_mode = 2;

lambda_1 = 1; lambda_2 = 1; beta = 1; % dummy in this scenario

%MPC decay

Gam = 5.83; gamma_0 = 0.3; Kgamma = 0.44; sigma_cluster = 2.88;

nlos = 2;

gamma_rise = 47.23; gamma_1 = 84.15; chi = 0.99;

% Small-scale Fading

m0 = 0.3; Km = 0; sigma_m0 = 1.15; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % m0_sp is assumption

% Large-scale Fading – Shadowing

std_shdw = 6;

% Frequency Dependence

kappa = -1.427;

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

elseif cm_num == 9, % Open Outdoor Environment NLOS (Fram, Snow-Covered Open Area)

% MPC arrival

Lam = 0.0305; Lmean = 3.31;

lambda_mode = 1;

lambda_1 = 0.0225; lambda_2 = 1; beta = 1;

%MPC decay

Gam = 56; gamma_0 = 0.92; Kgamma = 0; sigma_cluster = 3; % sigma_cluster is assumption

nlos = 1;

gamma_rise = NaN; gamma_1 = NaN; chi = NaN;

% Small-scale Fading

m0 = 4.1; Km = 0; sigma_m0 = 2.5; sigma_Km = 0;

sfading_mode = 0; m0_sp = NaN; % m0_sp is assumption

% Large-scale Fading – Shadowing

std_shdw = 3.96;

% Frequency Dependence

kappa = -1; % Kappa is assumption

fc = 6; % GHz

fs = 8; % 2 - 8 GHz

else

error(’cm_num is wrong!!’)

end

return

function [h]= uwb_sv_freq_depend_ct_15_4a(h,fc,fs,num_channels,kappa)

% This function is used to include the frequency dependency

f0 = 5; % GHz

h_len = length(h(:,1));

f = [fc-fs/2 : fs/h_len/2 : fc+fs/2]./f0;

f = f.^(-2*(kappa));

f = [f(h_len : 2*h_len), f(1 : h_len-1)]’;

i = (-1)^(1/2); % complex i

for c = 1:num_channels

% add the frequency dependency

h2 = zeros(2*h_len, 1);

h2(1 : h_len) = h(:,c); % zero padding

fh2 = fft(h2);

fh2 = fh2 .* f;

h2 = ifft(fh2);

h(:,c) = h2(1:h_len);

% Normalize the channel energy to 1

h(:,c) = h(:,c)/sqrt(h(:,c)’ * h(:,c) );

end

return

function [h,t,t0,np] = uwb_sv_model_ct_15_4a(Lam,Lmean,lambda_mode,lambda_1, ...

lambda_2,beta,Gam,gamma_0,Kgamma,sigma_cluster,nlos,gamma_rise,gamma_1, ...

chi,m0,Km,sigma_m0,sigma_Km,sfading_mode,m0_sp,std_shdw,num_channels,ts)

% Written by Sun Xu, Kim Chee Wee, B. Kannan & Francois Chin on 22/02/2005

% IEEE 802.15.4a UWB channel model for PHY proposal evaluation

% continuous-time realization of modified S-V channel model

% Input parameters:

% detailed introduction of input parameters is at uwb_sv_params.m

% num_channels number of random realizations to generate

% Outputs

% h is returned as a matrix with num_channels columns, each column

% holding a random realization of the channel model (an impulse response)

% t is organized as h, but holds the time instances (in nsec) of the paths whose

% signed amplitudes are stored in h

% t0 is the arrival time of the first cluster for each realization

% np is the number of paths for each realization.

% Thus, the k’th realization of the channel impulse response is the sequence

% of (time,value) pairs given by (t(1:np(k),k), h(1:np(k),k))

%%

modified by I2R

% initialize and precompute some things

std_L = 1/sqrt(2*Lam); % std dev (nsec) of cluster arrival spacing

std_lam_1 = 1/sqrt(2*lambda_1);

std_lam_2 = 1/sqrt(2*lambda_2);

% std_lam = 1/sqrt(2*lambda); % std dev (nsec) of ray arrival spacing

h_len = 1000; % there must be a better estimate of # of paths than this

ngrow = 1000; % amount to grow data structure if more paths are needed

h = zeros(h_len,num_channels);

t = zeros(h_len,num_channels);

t0 = zeros(1,num_channels);

np = zeros(1,num_channels);

for k = 1:num_channels % loop over number of channels

tmp_h = zeros(size(h,1),1);

tmp_t = zeros(size(h,1),1);

if nlos == 1,

Tc = (std_L*randn)^2 + (std_L*randn)^2; % First cluster random arrival

else

Tc = 0; % First cluster arrival occurs at time 0

end

t0(k) = Tc;

if nlos == 2 & lambda_mode == 2

L = 1; % for industrial NLOS environment

else

L = max(1, poissrnd(Lmean)); % number of clusters

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tcval = []; Tc_cluster= [];

Tc_cluster(1,1)=Tc;

for i_Tc=2:L+1

Tc_cluster(1,i_Tc)= Tc_cluster(1,i_Tc-1)+(std_L*randn)^2 + (std_L*randn)^2;

end

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

cluster_index = zeros(1,L);

path_ix = 0;

nak_m = [];

for ncluster = 1:L

% Determine Ray arrivals for each cluster

Tr = 0; % first ray arrival defined to be time 0 relative to cluster

cluster_index(ncluster) = path_ix+1; % remember the cluster location

gamma = Kgamma*Tc + gamma_0; % delay dependent cluster decay time

if nlos == 2 & ncluster == 1

gamma = gamma_1;

end

Mcluster = sigma_cluster*randn;

Pcluster = 10*log10(exp(-1*Tc/Gam))+Mcluster; % total cluster power

Pcluster = 10^(Pcluster*0.1);

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tr_len=Tc_cluster(1,ncluster+1)-Tc_cluster(1,ncluster);

else

Tr_len = 10*gamma;

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

while (Tr < Tr_len),

t_val = (Tc+Tr); % time of arrival of this ray

if nlos == 2 & ncluster == 1

% equation (22)

h_val = Pcluster*(1-chi*exp(-Tr/gamma_rise))*exp(-Tr/gamma_1) ...

*(gamma+gamma_rise)/gamma/(gamma+gamma_rise*(1-chi));

else

% equation (19)

h_val = Pcluster/gamma*exp(-Tr/gamma)/(beta*lambda_1+(1-beta)*lambda_2+1);

end

path_ix = path_ix + 1; % row index of this ray

if path_ix > h_len,

% grow the output structures to handle more paths as needed

tmp_h = [tmp_h; zeros(ngrow,1)];

tmp_t = [tmp_t; zeros(ngrow,1)];

h = [h; zeros(ngrow,num_channels)];

t = [t; zeros(ngrow,num_channels)];

h_len = h_len + ngrow;

end

tmp_h(path_ix) = h_val;

tmp_t(path_ix) = t_val;

% if lambda_mode == 0

% Tr = Tr + (std_lam*randn)^2 + (std_lam*randn)^2;

if lambda_mode == 1

if rand < beta

Tr = Tr + (std_lam_1*randn)^2 + (std_lam_1*randn)^2;

else

Tr = Tr + (std_lam_2*randn)^2 + (std_lam_2*randn)^2;

end

elseif lambda_mode == 2

Tr = Tr + ts;

else

error(’lambda mode is wrong!’)

end

% generate log-normal distributed nakagami m-factor

m_mu = m0 - Km*t_val;

m_std = sigma_m0 - sigma_Km*t_val;

nak_m = [nak_m, lognrnd(m_mu, m_std)];

end

Tc = Tc + (std_L*randn)^2 + (std_L*randn)^2;

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

if Kgamma ~= 0 & nlos == 0

Tc = Tc_cluster(1,ncluster+1);

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

end

% change m value of the first multipath to be the deterministic value

if sfading_mode == 1

nak_ms(cluster_index(1)) = m0_sp;

elseif sfading_mode == 2

nak_ms(cluster_index) = m0_sp;

end

% apply nakagami

for path = 1:path_ix

h_val = (gamrnd(nak_m(path), tmp_h(path)/nak_m(path))).^(1/2);

tmp_h(path) = h_val;

end

np(k) = path_ix; % number of rays (or paths) for this realization

[sort_tmp_t,sort_ix] = sort(tmp_t(1:np(k))); % sort in ascending time order

t(1:np(k),k) = sort_tmp_t;

h(1:np(k),k) = tmp_h(sort_ix(1:np(k)));

% now impose a log-normal shadowing on this realization

% fac = 10^(std_shdw*randn/20) / sqrt( h(1:np(k),k)’ * h(1:np(k),k) );

% h(1:np(k),k) = h(1:np(k),k) * fac;

end

return

function [hN,N] = uwb_sv_cnvrt_ct_15_4a( h_ct, t, np, num_channels, ts )

% convert continuous-time channel model h_ct to N-times oversampled discrete-time samples

% h_ct, t, np, and num_channels are as specified in uwb_sv_model

% ts is the desired time resolution

%%hN will be produced with time resolution ts /

N.

% It is up to the user to then apply any filtering and/or complex downconversion and then

% decimate by N to finally obtain an impulse response at time resolution ts.

min_Nfs = 100; % GHz

N = max( 1, ceil(min_Nfs*ts) ); % N*fs = N/ts is the intermediate sampling frequency before decimation

N = 2^nextpow2(N); % make N a power of 2 to facilitate efficient multi-stage decimation

Nfs = N / ts;

t_max = max(t(:)); % maximum time value across all channels

h_len = 1 + floor(t_max * Nfs); % number of time samples at resolution ts / N

hN = zeros(h_len,num_channels);

for k = 1:num_channels

np_k = np(k); % number of paths in this channel

t_Nfs = 1 + floor(t(1:np_k,k) * Nfs); % vector of quantized time indices for this channel

for n = 1:np_k

hN(t_Nfs(n),k) = hN(t_Nfs(n),k) + h_ct(n,k);

end

end