IEEE 802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Proposed Text for Draft 1.0 Comment Resolution – Part 3 |  |
| Date Submitted | January 15, 2025 |  |
| Sources | Youngwan So (SAMSUNG Electronics) youngwan.so@samsung.com |  |
| Re: |  |  |
| Abstract |  |  |
| Purpose | To propose resolution for miscellaneous hyper block related comments for “P802.15.4ab™/D1.0 Draft Standard for Low-Rate Wireless Networks” . |  |
| Notice | This document does not represent the agreed views of the IEEE 802.15 Working Group or IEEE 802.15.4ab Task Group. It represents only the views of the participants listed in the “Sources” field above. It is offered as a basis for discussion and is not binding on the contributing individuals. The material in this document is subject to change in form and content after further study. The contributors reserve the right to add, amend or withdraw material contained herein. |  |

Rev 0: Initial version. (Totally 5 comments)

Rev 1: Added more comments resolved (Totally 31 comments)

51, 58, 67, 125, 153, 154, 170, 554, 622, 626,

534, 875, 933, 943, 944, 1069, 1111, 1162, 1163, 1164,

1165, 1195, 1219,1220, 1235, 1236, 1332, 1341, 1384, 1407,

1434,

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Billy Verso | 1069 | 42 | 10.32.2 | 22 | Here and line 26, are those "other devices" controllers? YES, I believe so since by definition a controller is a device that sends an RCM (base standard definition). So call them that. | Change "other devices" to "other controllers", line 22 and line 27. | Accepted |
| Alex Krebs | 1384 | 42 | 10.32.2 | 22,23 | Do we need a more clear definition of RCMs? It seems here as if anybody, not only the initiator/controller, are allowed to send RCMs not only for sync, but also with ranging config params. This may be in conflict with the initiator=controller paradigm. | Change lines 21-23 to: - Ranging Control Message (RCM): A message transmitted during ranging control phase, to configure or to synchronize an upcoming ranging phase. | Accepted |

Disposition Detail: 																																				CID#1069 & 1384

Accepted.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change 10.32.2 P42L22 as below ;

20 The following nomenclature is used for messages:

21 ⎯ Ranging Control Message (RCM): A message transmitted

during ranging control phase, to configure or to synchronize an upcoming ranging phase.

24 The following nomenclature is used to describe the functionalities of different exchanges in a ranging

25 round:

26 ⎯ Ranging Control Phase (RCP): A phase in which the controller sends an RCM, and optionally other

27 controllers send RCMs.

28

29 Ranging Response Phase (RRP): A phase in which the responder(s) send their response

30 messages(s) to the initiator(s).

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hong Won Lee | 1407 | 47 | 10.32.9.10 | 27 | There is no description of bitmap-based block scheduling for block-based mode, which can be used. Bitmap-based block scheduling should be described for both hyper block mode and block-based mode, respectively | Change from "element represents the pattern of scheduled blocks to a single device" to "element represents the pattern of scheduled blocks in hyper block mode defined in 10.x.x.x.x or represents the current scheduled block in block-based mode to a single device" | Revised |

Disposition Detail:

CID#1407

Revised as suggested

Proposed text changes on P802.15.4ab™

Change 10.32.9.10 P47L27 as below ;

25 When bitmap-based block scheduling is used, one or multiple ranging blocks may be assigned to a device

26 for transmissions using a single Scheduling List field element. A bitmap in each Scheduling List field

27 element represents the pattern of scheduled blocks in hyper block mode defined in 10.32.3.5 or represents the current scheduled block in block-based mode to a single device. For example, a Scheduling IE with

1 Scheduling List Type field value of five can be transmitted in the same ranging round as an HBS IE for

2 block scheduling in hyper block mode, as described in 10.32.3.5. The Block Scheduling Bitmap field

3 represents one or multiple ranging blocks assigned to a device for transmission using a single Scheduling

4 List field element in a hyper block.
Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Billy Verso | 1111 | 53 | 10.32.9.12 | 10 | "It is recommended that the three values giving block, round and slot duration are all included in the HBS IE at the same time." Rather than recommending, could just define the format so that they all have to be included. | Specify a single presence bit to cover the inclusion/exclusion of all three fields. | Revised |
| B. Rolfe | 1332 | 53 | 10.32.9.12 | 10 | Note sure what "The controller may choose the units to suit its needs." means in the context of an optional behavior (without specifying conditions for "needs"). Are we saying this is determined by some process outside the scope of this standard? (seems likely). | The controller chooses the units as needed based on criteria outside the scope of this standard. | Accepted |

Disposition Detail:

CID#1111

Revised. Added one bit presence flag and description about it as suggested.

CID#1332

Accepted. Intended the controller chooses the units as needed based on criteria outside the scope of this standard.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change the chapter 10.32.9.12 P53L10 as follows:

The Content Control field is formatted as per Figure 21 and indicates the presence of duration fields in the

7 Ranging Block Description List, which is structured as per Figure 22.

| Bits: 0-1 | 2 | 3 | 4 | 5-7 |
| --- | --- | --- | --- | --- |
| Ranging Block Duration Unit | Ranging Block Duration Present | Ranging Round Duration Present | Ranging Slot Duration Present | reserved |

Figure 21— Content Control field format

The Ranging Bloc1k Duration Units field indicates the Ranging Block Duration field size as per Table 12.

The controller chooses the units as needed based on criteria outside the scope of this standard. It is recommended that the three values giving block, round and slot duration are all included in the HBS IE at the same time. But, as the hyper block structure is generally repeating, the round duration and slot duration can be omitted by setting the Ranging Round Duration Present field to zero and the Ranging Slot Duration Present field to zero, in which case the values from the most recent previous hyper blocks are carried froward unchanged.

………….

Change the chapter 10.32.9.12 P54L1 as follows:

The Ranging Block Duration Present field indicates the presence or absence of the Ranging Block Duration field in the elements of the Ranging Block Description List which is formatted as per Figure 22. When the Ranging Block Duration Present field value is one, the Ranging Block Duration field is included in the Ranging Block Description List elements. When the Ranging Block Duration Present field value is zero, the Ranging Block Duration field is not included in the Ranging Block Description List elements.

The Ranging Round Duration Present field indicates the presence or absence of the Ranging Round Duration field in the elements of the Ranging Block Description List which is formatted as per Figure 22. When the Ranging Round Duration Present field value is one, the Ranging Round Duration field is included in the Ranging Block Description List elements. When the Ranging Round Duration Present field value is zero, the Ranging Round Duration field is not included in the Ranging Block Description List elements.

The Ranging Slot Duration Present field indicates the presence or absence of the Ranging Slot Duration field in the elements of the Ranging Block Description List which is formatted as per Figure 22. When the Ranging Slot Duration Present field value is one, the Ranging Slot Duration field is included in the Ranging Block Description List elements. When the Ranging Slot Duration Present field value is zero, the Ranging Slot Duration field is not included in the Ranging Block Description List elements.

Ranging Block Description List Length field specifies the number of elements in the Ranging Block Description List field. The number of elements shall be set equal to the number of ranging blocks in the hyper block. The Ranging Block Description List field contains Ranging Block Description List elements each of which is structured as per Figure 22.

| Octets: 1 | 0/1/2/3 | 0/1 | 0/2 |
| --- | --- | --- | --- |
| Relative Block Index | Ranging Block Duration | Ranging Round Duration | Ranging Slot Duration |

Figure 22— Format of Elements in the Ranging Block Description List Field

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Billy Verso | 1162 | 67 | 10.38.3.9 | 7 | The phrase "the default value of the macMmsNbInitChannel attribute" is used here, which is strange, if a single constant channel number is required at all times for this coordination then let's make that clear with a separate constant value for this. | For this use change macMmsNbInitChannel to say using O-QPSK 5800 MHz band channel 2, or better still use the resolution for my comment on line 8. | Revised |
| Billy Verso | 1163 | 67 | 10.38.3.9 | 8 | The phrase "the default value of the macMmsUwbChannel attribute" is used here, which is strange. If a single constant UWB channel number is required for this, it is probably better to define this as a constant, but note that this is not sufficient for proper UWB interworking. For that, it is also needed to specify the UWB packet format, and the preamble code, and the data rate etc, which are all needed for UWB reception to interwork. Since coordination and scanning is largely an upper layer procedure, perhaps it should be left to the upper layer to set the already defined PHY parameters to select the UWB channel, preamble code, data rate before sending the Acquisition Compact frame, or listening for them, i.e., leave it for the next higher layer specification body to decide what configuration to use, along with the duration and time intervals of the scanning, etc.). | Delete these attributes and change the sentence to say "The initiator transmits the NB Acquisition Compact frame using the O-QPSK PHY and transmits the UWB Acquisition Compact frame using the HRP UWB PHY. | Revised |
| Billy Verso | 1164 | 67 | 10.38.3.9 | 14 | The phrase "the default value of the macMmsNbInitChannel attribute" is used here… see my comments on lines 7 and 8, and their proposed resolutions. | See the resolution for my comments on lines 7 & 8. | Revised |
| Billy Verso | 1165 | 67 | 10.38.3.9 | 15 | The phrase "the default value of the macMmsUwbChannel attribute" is used here... see my comments on lines 7 and 8, and proposed resolutions. | See the resolution for my comment on line 8. | Revised |

Disposition Detail:

CID#1162, 1164

Revised. A common single constant channel number is required at times of coordination. Otherwise, the different controllers are likely to scan on different NB Initialization Channels, which will lead to failure of discovery. Or to find out each other, something like full-search on all the NB initialization may be needed which is inefficient.

Comment looks reasonable, and the suggested change is added on top of current text.

CID#1163, 1165

Revised. Yes it’s right that the UWB packet format, and the preamble code, and the data rate etc., are also needed for UWB reception to interwork in addition to default UWB channel.

Comment looks reasonable, and the suggested change is added on top of current text.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change the chapter 10.38.3.9 P67L7~15 as follows:

1 higher layer determines whether the coordination is active or not. If coordination is active, the initiator

2 opportunistically or periodically transmits an Acquisition Compact frame with information about its UWB

3 channel usage after a session is configured. The transmission of these Acquisition Compact frames may

4 start before the start of the first block. The initiator sends these Acquisition Compact frames using either

5 the O-QPSK PHY (NB Acquisition Compact frame) or the HRP UWB PHY (UWB Acquisition Compact

6 frame) or both. The initiator transmits the NB Acquisition Compact frame in the initialization channel

7 specified by the default value of the macMmsNbInitChannel attribute using O-QPSK PHY 5800 MHz band channel 2, and transmits the UWB Acquisition

8 Compact frame in the channel specified by the default value of the macMmsUwbChannel attribute using the HRP UWB PHY. The NB

9 Acquisition Compact frame and UWB Acquisition Compact frame are described in 10.38.9.20. To provide

10 the information of UWB channel usage, both NB Acquisition Compact frame and UWB Acquisition

11 Compact frame include the UWB Per-Session Info Fields. The higher layer determines a suitable interval

12 between Acquisition Compact frames.

13 If coordination is active, before starting a new session, the initiator scans for Acquisition Compact frame on

14 the initialization channel specified by the default value of the macMmsNbInitChannel attribute using O-QPSK PHY 5800 MHz band channel 2, and/or the

15 channel specified by the default value of the macMmsUwbChannel attribute using the HRP UWB PHY. The length of the scanning

16 period is implementation dependent. The initiator thus obtains information of UWB channel usage from

17 other initiators, and with this knowledge, the initiator may select values for configuring its new session to

18 minimize the overlap with active periods of other sessions nearby. The details of this are implementation

19 specific.

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carl Murray | 875 | 76 | 10.38.8.4.2 | 18 | It is not clear what is meant by this text "In the ranging phase, RSF transmissions are scheduled to have the RSF transmission timing of each responder." | Make the intent of the text clearer. | Revised |
| Bin Qian | 153 | 76 | 10.38.8.4.2 | 19 | It is not clear which Poll compact frame is sent by the initiator in the control phase | Specify the Poll compact frame and the Message Control field value used in the control phase | Revised |
| Youngwan So | 943 | 76 | 10.38.8.4.2 | 19 | There's no "Poll Compact frame" defined in current specification. The term should be exact. | Change From "In the control phase, the initiator sends a poll Compact frame to trigger RSF transmission in the ranging phase." To "In the control phase, the initiator sends either a One-to-Many poll Compact frame to schedule RSF transmission in the ranging phase or a data frame carrying Scheduling IE" | Revised |
| Bin Qian | 154 | 76 | 10.38.8.4.2 | 22 | It is not clear how to set the Message Control field value of the Ranging Report Compact frame | Specify the Message Control field value used in the Ranging Report Compact frame | Revised |
| Youngwan So | 944 | 76 | 10.38.8.4.2 | 22 | Ranging report can be delivered both way "from initiator to responder" as well as "from responder to initiator". But current texts just mentions about one way only. | Change From "The measurement report phase delivers ranging results from the responders to the initiator." To "The measurement report phase delivers ranging results from the responders to the initiator and/or from the initiator to the responders." | Revised |

Disposition Detail:

CID#875

Revised. The intended meaning of the text is that if a certain concurrent (=multiple) RSF transmission is going to happen, the RSF transmission timing should be aligned with each other.

CID#153 & 943

Revised. Either an One-to-Many poll Compact frame (pg.102) to schedule RSF transmission in the ranging phase or a data frame carrying Scheduling IE. Message Control field value is 0x80 as it can designate the number of responders and start/end slot index.

CID#154 & 944

Revised. “One-to-many initiator Report Compact frame” and/or “One-to-many Responder Report Compact frame” can be used for report. In any case, Message Control value can be either 0x00 or 0x10, and fundamentally there’s no big difference between them except extra short term information is piggybacked in 0x00 case.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change 10.38.8.4.2 P76L18-19 as below ;

17 The procedure for multiple RSF transmissions in a slot is divided into three phases, the control phase, the

18 ranging phase, and the measurement report phase. In the ranging phase, RSF transmissions are scheduled to

19 have the RSF transmission timing of each responder the same. In the control phase, the initiator sends an One-to-Many poll (message control field = 0x80) to schedule

20 Compact frame to trigger RSF transmission in the ranging phase or a data frame carrying Scheduling IE. After that, in the ranging phase multiple

21 RSF transmissions occur from the responders to the initiator in the slot. The measurement report phase

22 delivers ranging results from the responders to the initiator and/or from the initiator to the responders.  Responders may send Ranging report Compact

23 frames to the initiator, or the initiator can send reports to the responders to conduct this phase.

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Billy Verso | 1195 | 78 | 10.38.8 | 3 | I have an earlier comment (against clause 10.38.2) saying that 4ab should state UWB/NB/TX/RX are mutually exclusive. Generally each PHY has a turnaround time specification, TX-to-RX and RX-to-TX. Now in MMS we have to switch from UWB to NB and NB to UWB in various combinations of TX to TX, TX to RX, RX to RX, and RX to TX. The standard should specify minimum times for these as a worse case target for implementers. The times need to take into account the MAC operations necessary to decode any received frame, and configure / enable the device for next TX/RX operation. | Insert a new subclause (possibly between 10.38.7 and 10.38.8) to define these times and specify these inter-radio-interframe spacing and TX/RX turnaround times that apply between NB and UWB activity. And we should be including allowance for these turnaround times in the appropriate slot time specifications. | Revised |

Disposition Detail:

Basically, agree with the switching time specification is required. In NBA case of 4ab, we can think about two different types of switching cases and its combination ; PHY-to-PHY, TX/RX-to-TX/RX. Based on this, the new subclause to define and specify these minimum spacing is drafted as below.

NOTE : This is quite primitive level of skeletons, so PHY experts’ help is needed to populate blanks.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Insert the following sub-clause as 10.38.7.5 ;

10.38.7.5 Interframe spacing

The NBA UWB controller and controlee will have both of NB interface and UWB interface. In this case, to take into account the MAC operation necessary to decode any received frames and to configure the device for next transmission/reception mode, minimum time spacing for transition between PHY interfaces and TX/RX mode needs to be specified.

Figure A shows minimum time to secure for switching from NB to UWB (MinTime_NBtoUWB) and switching from UWB to NB (MinTime_UWBtoNB). The transmitter side should secure MinTime_NBtoUWB at least when PHY changes from NB to UWB and receiver side should assume PHY changing time takes MinTime_NBtoUWB when PHY changes from NB to UWB. Table X shows a minimum interframe time spacing attribute value in NBA UWB.

Table X – Inter frame time spacing related attributes

| Attribute (RSTU) | Inter Frame Time Spacing |
| --- | --- |
| MinTime_NBtoUWB | TBD |
| MinTime_UWBtoNB | TBD |

Figure A – Inter frame time spacing when PHY mode changes

Figure B and Figure C show minimum turnaround time to secure when switching between transmission mode and receiving mode in NBA UWB. The transmitter side should secure at least each interframe spacing time when TX/RX mode changes and receiver side should assume for each case, respective TX/RX mode turnaround is allowed.. Table Y shows a minimum interframe time spacing attribute value in NBA UWB when TX/RX mode changes.

Table Y – Inter frame time spacing related attributes when TX/R

| Attribute (RSTU) | Inter Frame Time Spacing |
| --- | --- |
| MinTime_NBtoUWB_TXtoTX | TBD |
| MinTime_NBtoUWB_TXtoRX | TBD |
| MinTime_NBtoUWB_RXtoTX | TBD |
| MinTime_NBtoUWB_RXtoRX | TBD |
| MinTime_UWBtoNB_TXtoTX | TBD |
| MinTime_UWBtoNB_TXtoRX | TBD |
| MinTime_UWBtoNB RXtoTX | TBD |
| MinTime UWBtoNB_RXtoRX | TBD |

Figure B – Inter frame time spacing between TX/RX mode changes (NB to UWB)

Figure C – Inter frame time spacing between TX/RX mode changes (UWB to NB)

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mickael Maman | 51 | 78 | 10.38.8.4.4 | 1 | What is Poll frame form initiator in uwb rangng slot 3? | change to SYNC+SFD or rsf? | Accepted |

Disposition Detail:

Accepted.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Replace Figure 46 with updates below ;

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mickael Maman | 58 | 87 | 10.38.9.3.15 | 28 | SOR Time offset is between ADV conf and not ADV Poll | This is a four-octet, 32-bit field that specifies the time offset in 1/499.2 MHz resolution between the start of the Advertising Confirmation Compact frame, or the Public Advertising Confirmation Compact frame, and the start of the Start of Ranging Compact frame. | Accepted |
| Lei HUANG | 1434 | 87 | 10.38.9.3.15 | 28 | The SOR Time Offset field indicates the time offset between the start of the Advertising Confirmation Compact frame and the start of the Start of Ranging Compact frame. | Change "the start of the Advertising Poll Compact frame, or the Public Advertising Poll Compact frame" to "the start of the Advertising Confirmation Compact frame". | Accepted |

Disposition Detail:

CID58&1434

Accepted.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change 10.38.9.3.15 P87L28 as below ;

26 10.38.9.3.15 The SOR Time Offset field

27 This is a four-octet, 32-bit field that specifies the time offset in 1/499.2 MHz resolution between the start of

28 the Advertising Confirmation Compact frame, or the Public Advertising Confirmation Compact frame, and the start of the

29 Start of Ranging Compact frame.

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rojan Chitrakar | 125 | 96 | 10.38.9.6 | 14 | In the previous LB round, much effort was made to consolidate the different message variants; variant 0x20 can be easily combined into variant 0x10 by adding a presence bit: Block Description List Present. | Combine variant 0x20 into variant 0x10 by adding the Block Description List Present in the Presence Bitmap field. | Rejected |
| Billy Verso | 1219 | 96 | 10.38.9.6 | 15 | "In this case, the responders assume" is strange, not sure what it should be (or even if it should be in this clause). can assume, may assume, should assume ? | Change to "should assume" ? | Accepted |

Disposition Detail:

CID#125

Rejected. I also agree with minimizing variant makes the messages neat and not complicated. But, currently the number of variant of corresponding message (SoR) are rather small (i.e. just only three ; 0x00, 0x10, 0x20). And the purpose of 0x10 variant is already described as “sent by the initiator to indicate the status of the initialization and setup phase,” which is quite far from Hyper Block indication purpose of 0x20. So if we merge those two cases, the SoR message complexity will increase. So I’d like to ask whether we can keep it as is to Rojan.

CID#1219

Accepted. I removed “assume” and made the sentence more definitive and transmitter side operation.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change 10.38.9.6 P96L14 as below ;

14 When the Message Control field value is 0x20, the ranging executed in Hyper Block mode. In this case,

the controller should send HBS IE, ARC IE and Time Offset parameter so that controlee can recognize the Hyper Block mode starts from the specified Time Offset and it can configure to receive it. When the Message Control field value is 0x20

17 the Message Content field shall be formatted as shown in Figure 71.

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tero Kivinen | 554 | 96 | 10.38.9.6 | 27 | As this is talking about the hyper blocks, I assume the 8-bit field called Block Index is really a Relative Block Index, not Block Index. | Change Block Index to Relative Block Index. | Accepted |

Disposition Detail:

Accepted.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change Figure 72 as below ;

| Octets: 1 | 1 | 1 | 2 |
| --- | --- | --- | --- |
| Relative Block Index | Block Duration | Round Duration | Slot Duration |

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Billy Verso | 1220 | 97 | 10.38.9.6 | 5 | This be talking about "ranging slots", "ranging blocks" etc. Also somewhere in the text MMS slots were constrained to 300 RSTU multiples, so maybe we want to make that constraint in the slot duration field here (and elsewhere) | Add in "ranging" before block, round, slot as appropriate. Consider if appropriate to constrain slot durations to 300 RSTU multiples and update accordingly. Here and everywhere. | Rejected |

Disposition Detail:

CID#1220

Rejected. Though it’s right that the ranging is the most probable service now, originally, the intention of Hyper Block mode concept was to accommodate various kind of applications doable through UWB. Therefore, I’d like to keep it open. For example, the slot may be used not only for ranging but also for However if it makes the document unclear, then I will change as suggested.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

No change

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mickael | 67 | 102 | 10.38.9.11 | 1 | In advertising confirmation compact frame, the responder address is missing. | add 1 octet for responder address in Figure 85 and the following text "The Responder Address field identifies the address of a responder selected by the initiator to be involved in the following ranging session." | Rejected |

Disposition Detail:

Rejected. The Figure 85 is the case when Message Control value is 0x00. I am not so much sure but it looks like if we accept the suggested changes, it is the special case when “the Number of Responders” is “1”, which makes 0x00 case is in vain. Therefore, I guess suggested changes will probably be not the right answer, so I made it “rejected” inevitably and original author’s opinion seems to be needed as I am not the original author, either.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

No changes

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Youngwan So | 933 | 104 | 10.38.9.12 | P104L27, P105L17, P106L12, P107L14 | Message Control field values in sentence introducing each Message Content fields should be also updated as it is changed in different CID CRs above. | Update is needed as follows; From "When the Message Control field value is 0x50 the Message Content field shall be formatted as shown in Figure 93" To "When the Message Control field value is 0x30 the Message Content field shall be formatted as shown in Figure 93" From "When the Message Control field value is 0x70 the Message Content field shall be formatted as shown in Figure 94." To "When the Message Control field value is 0x40 the Message Content field shall be formatted as shown in Figure 94." From "When the Message Control field value is 0x80 the Message Content field shall be formatted as shown in Figure 96." To "When the Message Control field value is 0x50 the Message Content field shall be formatted as shown n Figure 96." From "When the Message Control field value is 0x90 the Message Content field shall be formatted as shown in Figure 98." To "When the Message Control field value is 0x60 the Message Content field shall be formatted as shown in Figure 98." | Rejected |

Disposition Detail:

Rejected. I checked the text again, and there’s no problem.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

No changes

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tero Kivinen | 622 | 115 | 10.38.9.20.2 | 21 | The Common Info field has length of 2/4, but I think it is always 4 octets long. | Change 2/4 to 4. | Accepted |

Disposition Detail:

Accepted. Now Common Info field length can be 2 octet or 4 octet. But as you can see Figure 119, Common Info field can be only 4 octets, so comment and proposed change is right.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change Figure 118 as below ;

| Octets: 4 | 0/4 | Variable |
| --- | --- | --- |
| Common Info | UWB AP Info | UWB Per-Session Info List |

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tero Kivinen | 626 | 116 | 10.38.9.20.2 | 4 | There is no point of saying that for one bit field other values than zero and one are reserved. | Remove "Other values are reserved". | Accepted |
| Bin Qian | 170 | 116 | 10.38.9.20.2 | 19 | According to the extended channels in 16.4.1.2, 4 bits Channel Number field is not enough | Extend all Channel Number field to 7 bits in this sub-clause | Accepted |
| Rolfe | 1341 | 116 | 10.38.9.20.2 | 15 | "may not" is never correct in an IEEE standard. Here we're trying to say not all fields are included in all frames. | change to: not all fields are included in all frames | Accepted |

Disposition Detail:

CID#626

Accepted. Seems editorial error.

CID#170

Accepted. In sub-clause 16.4.1.2, Channel Number field ranges from 16 to 113, so at least 7 bit is required as comment says.

CID#1341

Accepted. Updated as suggested comments.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change 10.38.9.20.2 P116L4 as below ;

3 The NB AP Type field value when zero specifies periodic coordination, and when one specifies aperiodic

4 coordination.

Change 10.38.9.20.2 P116L15 and L19 as below ;

15 The fields of the UWB AP Info appear in a fixed order; however, not all fields are included in all frames

If UWB AP Info is present, the initiator shall send the UWB Acquisition Compact frame after the

17 NB Acquisition Compact frame. The UWB AP Info field when present shall be formatted as shown in

18 Figure 120.

| Octets: 2 | Bits: 0-6 | 7 | Octets: 1 |
| --- | --- | --- | --- |
| Common Info | UWB AP Info | Reserved | Preamble Code |

Figure 120 – Format of UWB AP Info field

Comment Indices in 15-24-0371-01-04ab-consolidated-comments-draft-1.0:

| Name | Index# | Pg | Sub-Clause | line | Comment | Proposed Change | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tero Kivinen | 634 | 119 | 10.38.9.20.3 | 7 | Next UWB AP field is present only when UWB AP Type field is zero or one, but all other values on the UWB AP Type field are reserved, so the Next UWB AP field is present always for all currently defined frames. | Mark Next UWB AP field to be there always, i.e., mark its length to 2 and mark Common info field length to 4. Remove text about it being present only when non reserved values are used. | Accepted |
| Billy Verso | 1235 | 119 | 10.38.9.20.3 | 7 | "The Next UWB AP field, should state this as a start to start relationship, as is done for the Next NB AP field in the NB Acquisition Compact frame. | Chane definition to say it is the time "from the start of current UWB Acquisition Compact Frame to the start of the next UWB Acquisition Compact frame". | Accepted |
| Billy Verso | 1236 | 119 | 10.38.9.20.3 | 7 | This says "which is present when the UWB AP Type field value is either zero or one", but looking at the definition of "UWB AP Type field" those are the only allowed values, which means this field always present. | Delete "which is present when the UWB AP Type field value is either zero or one". And change "Octets: 0/2" in Figure 125 to "Octets: 2" since the field is always present, | Accepted |

Disposition Detail:

CID#634&1235&1236

Accepted.

Proposed text changes on P802.15.4ab™/Draft 1.0 :

Change 10.38.9.20.3 P119L7 as below ;

7 The Next UWB AP field, gives

8 the time remaining in RSTU until the start of the next UWB Acquisition Compact frame.

Change 10.38.9.20.3 P118L24 as below ;

| Bits: 0-2 | 3-7 | 8-10 | 11-14 | 15 | Octets: 2 |
| --- | --- | --- | --- | --- | --- |
| UWB AP Type | Reserved | Type of UWB Per-Session Info | Number of UWB Per-Session Info | Reserved | Next UWB AP |

Figure 125 – Format of the Common Info field