| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of M2M paging parameter TLV |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Giwon Park LG Electronics | E-mail: giwon.park@lge.com |
| Re: | IEEE 802.16-11/0049r1, IEEE 802.16 Working Group Letter Ballot #33a |  |
| Abstract | This is to clarify the M2M paging parameter TLV |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification of M2M paging parameter TLV

Giwon Park

LG Electronics

# Introduction

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the table in page 35, line 29 as follows:]

11.17.5 M2M device group paging parameter TLV

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| ULMAP start offset for RNG-REQ | 156.7 | 1 | This parameter indicates the offset in units of frames that M2M device starts to monitor the resource (i.e., UL MAP IE offset) for the RNG-REQ message, where the reference point of this offset value is the frame in which the MOB_PAG-ADV is transmitted. Bits 0-7: Num_MAC_Address_Hash, Number of M2M devices Subsequent (Num_MAC_Address_Hash x 8) bits: For (i=0; i<Num_MAC_Address_Hash; i++){ 8-bit Fast Ranging IE offset } |
| Resource monitor timer | 156.8 | 1 | Time duration that M2M device monitors the resource for RNG-REQ message. |

[Remedy 2. Modify the table in page 36, line 6 as follows:]

11.17.6 M2M report code

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| M2M report code | 157 | Variable: (8+Num_MACs) | Bits 0-7: Num_MAC_Address_Hash, the number of paged devices; Subsequent (Num_MAC_Address_Hash + padding) bits: for (i = 0, i < Num_MAC_Address_Hash, i++) { 1 bit M2M report code 0b0: Reserved 0b1: send the uplink report } Padding | MOB_PAG-ADV |

[Remedy 3. Modify the table in page 36, line 46 as follows:]

11.17.7 M2M paging parameter

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| UL-MAP offset for RNG-REQ | 158.2 | Variable | Indicate offset that UL-MAP IE containing Fast Ranging IE for RNG-REQ message is transmitted. Bits 0-7: Num_MAC_Address_Hash, Number of M2M devices Subsequent (Num_MAC_Address_Hash x 8) bits: For (i=0; i<Num_MAC_Address_Hash; i++) { 8bit UL-MAP offset for RNG-REQ offset } |

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Ranging code and ranging slot assignment | 158.3 | Variable | Specify the ranging code and ranging slot assignment. Bits 0-7: the number of MAC address hash values, denoted by n; Subsequent (n x 16) bits: for (i=0; i<n; i++) { 8-bits code index assigned to an paged device 8-bits transmission opportunity offset assigned to an paged device } |

# References

[1] IEEE P802.16p/D2