| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Timers Impacted by the STID-Sharing Scheme, Revision 1 |  |
| Date Submitted | 2012-01-18 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI Honggang Li Intel Corporation Lei Zhou China Samsung Telecom R&D Center Jaesun Cha ETRI Jeongki Kim LG Electronics | Voice: E-mail: mhtao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-11/0051, LB#36 |  |
| Abstract | This contribution proposes changes to the Section 6.2.1.2.1 of IEEE P802.16.1b/D1 |  |
| Purpose | To be discussed and adopted by TGp |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Timers Impacted by the STID-Sharing Scheme

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Honggang Li

Intel Corporation

Lei Zhou

China Samsung Telecom R&D Center

Jaesun Cha

ETRI

Jeongki Kim

LG Electronics

# Introduction

There are several timers impacted by the STID-Sharing scheme, but the current draft didn’t provide the list of the timers. This contribution tries to make the draft more completed by providing the list of all impacted timers. The timers in the list shall be increased by the value indicated by STID_Valid_Periodicity in terms of number of frames.

# Text Proposal

Adopt the following text modifications to IEEE P802.16.1b/D1:

--------------------------  Begin Text Proposal   -------------------------------

[Remedy #1: Adopt the following proposed modification from page 4, line 38]

6.2.1.2.1 Station Identifier (STID)

For all MAC control message procedures involving an M2M device that has a shared STID, the timers in Table TBD shall be increased by the number of frames indicated by STID_Valid_Periodicity.

[Remedy #2: Adopt the following proposed modification from page 63, line 37]

6.11 Global values

Table 327—Parameters and constants

| System | Name | Time Reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| ABS, AMS, M2M device | T7 | Wait for AAI-DSA/DSC/DSD Response timeout | 10 ms* | —* | 1 s* |
| ABS, AMS, M2M device | T8 | Wait for AAI-DSA/DSC Acknowledge timeout | 10 ms* | —* | 300 ms* |
| ABS, AMS, M2M device | T10 | Wait for Transaction End timeout | 600 ms* | —* | 3 s* |
| AMS, M2M device | T14 | Wait for AAI-DSX-RVD Timeout | —* | —* | 100 ms* |
| ABS, AMS, M2M device | T22 | Wait for AAI-ARQ-Reset | —* | —* | 0.5 s* |
| AMS, M2M device | T43 | Time the AMS waits for AAI-SLP-RSP, AAITRF_IND-RSP, or SCH | —* | —* | —* |
| AMS, M2M device | T44 | Time the AMS waits for AAI-SCN-RSP | —* | —* | —* |
| AMS, M2M device | T45 | Time the AMS waits for AAI-DREG-RSP | —* | 250 ms* | 500 ms* |
| ABS | T46 | Time the ABS waits for AAI-DREG-REQ in case of unsolicited idle mode initiation from ABS | 50 ms* | —* | —* |
| ABS, AMS, M2M device | Key agreement MSG#1 Timer | Time prior to resend of PKMv3 Key agreement MSG#1 | 0.5 s* | 1 s* | 2.0 s* |
| ABS, AMS, M2M device | Key agreement 3-way handshakeTimer | Time prior to resend of PKMv3 Key agreement MSG#2 | 0.1 s* | 0.3 s* | 1.0 s* |
| ABS, AMS, M2M device | TEK Re-Auth Timer | Time prior to sending PKMv3 TEK-Request right after key agreement 3-way handshake | 20 ms* | 40 ms* | 100 ms* |
| ABS | T58 | Minimum duration for ABS to wait for AAI-RNG-CFM message | —* | —* | —* |
| ABS, AMS, M2M device | ACK timer | This timer is for AAI-MSGACK or MAEH. The ACK timer operation is described in 6.2.22 | 8 frames* | —* | —* |

* The value shall be increased by (STID_Valid_Periodicity - 1) * 5 ms, if the timer is associated with a M2M device that has a shared STID.

-----------------------------  End Text Proposal   ----------------------------------

# Reference

[1] IEEE 802.16p-10/0004r3: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD).

[2] IEEE P802.16.1b/D1: WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems

Enhancements to Support Machine-to-Machine Applications

[3] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report.