| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of paging message segmentation |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Giwon Park LG Electronics | E-mail: giwon.park@lge.com |
| Re: | IEEE 802.16-11/0051, IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This is to clarify the text related to paging message segmentation |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification of paging message segmentation

Giwon Park and Youngsoo Yuk

LG Electronics

# Introduction

When the paging message is larger than the maximum size supported by the network, it may be segmented. Moreover, idle mode section of the draft [1] describes that paging message may be segmented. However, regarding the paging message segmentation, some text of idle mode section was wrongly described as fragmentation.

Thus, we clarify the description of paging message segmentation.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the table 49 in page 26, line 7 as follows:]

Table 49– AAI-PAG-ADV message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists 0b0: This is the last fragment of the AAIPAG-ADV message 0b1: This is not the last fragment of the AAI-PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames. If there are remaining fragments segments and the remaining fragments segments include only M2M device's paging, this flag is set to 0b0. If this flag is set to 0b0, AMS enters the paging unavailable interval and M2M device checks the M2M extension flag. If this flag is set to 0b1, AMS and M2M device shall remain awake and monitors the subsequent AAI subframe unless its identifier is found in the received segments. |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information 0b0: Reserved 0b1: There is emergency information | Optional Present if there is emergency information |
| M2M extension flag | 1 | Used to indicate exists of the remaining part of the AAI-PAG-ADV message for M2M device. 0b0: This is the last fragment segment of the AAI-PAG-ADV message 0b1: This is not the last fragment segment of the AAI-PAG-ADV message; the remaining fragments segments of the message will be transmitted in the subsequent subframes or frames. | Optional Shall be present if the Extension Flag is set to 0b0 and if there are remaining fragments segments for paging M2M devices only. |

# References

[1] P802.16.1b/D1