| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text clarification of Fixed M2M Ranging Assignment A-MAP IE |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Giwon Park LG Electronics | E-mail: giwon.park@lge.com |
| Re: | IEEE 802.16-11/0051, IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This is to clarify the fixed M2M Ranging Assignment A-MAP IE description in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Text clarification of Fixed M2M Ranging Assignment A-MAP IE

Giwon Park

LG Electronics

# Introduction

This contribution proposes the clean-up of the description of “Fixed M2M Ranging Assignment A-MAP IE”

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 6.2.18.7.2 (page 44, line362) as follows:]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

When the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID) and M2M network access type is set to 0b00 (i.e., Resource allocation for AAI-RNG-REQ), it starts to monitor the Fixed M2M Ranging Assignment A-MAP IE to obtain the resource for AAI-RNG-REQ message at 'Assignment A-MAP start offset for AAI-RNG-REQ' during the 'Resource monitor timer'. If the M2M device does not decode the Fixed M2M Ranging Assignment A-MAP IE until the expiration of the 'Resource monitor timer', it performs contention-based ranging.

[Remedy 2. Modify the text in 6.2.18.7.2 (page 44, line 61) as follows:]

When the M2M device receives the individual paging message (i.e., AAI-PAG-ADV with FMDID) and M2M network access type is 0b00 (i.e., Resource allocation for AAI-RNG-REQ), it decodes the Fixed M2M Ranging Assignment A-MAP IE to obtain the resource of AAI-RNG-REQ message at 'Assignment A-MAP offset for AAI-RNG-REQ'.

[Remedy 3. Modify the table 49 in page 19, line 23 as follows:]

Table 49 – AAI-PAG-ADV message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For (i=0; i<M; i++) { |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1. | Present only for M2M devices |
| … |  |  |  |
| M2M network access type | 2 | Indicate the network re-entry access type for M2M device; 0b00: Resource allocation (i.e., Fixed M2M Ranging Assignment A-MAP offset) for AAI-RNG-REQ 0b01: dedicated ranging channel allocation in AAI-PAG-ADV 0b10: dedicated ranging channel allocation in broadcast assignment A-MAP IE 0b11: No dedicated ranging channel |  |
| if (M2M network access type == 0b00) { |  |  |  |
| Fixed M2M Ranging Assignment A-MAP offset for AAI-RNG-REQ |  | Indicate offset in units of frames of the Fixed M2M Ranging Assignment A-MAP IE for AAI-RNG-REQ message, where the reference point of this offset value is the frame in which the AAI-PAG-ADV is transmitted. |  |
| } |  |  |  |
| … |  |  |  |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| … |  |  |  |
| if(Action code ==0b00 or 0b01) { |  |  |  |
| Initial ranging backoff start | 4 | Indicate the initial backoff window size for M2M devices included in this group |  |
| M2M network access type | 2 | Indicate the network access scheme for M2M device 0b00: Resource allocation (i.e., Fixed M2M Ranging Assignment A-MAP offset) for AAI-RNG-REQ, This type is only applicable to fixed M2M device (i.e., Localized_Idle_Mode_Accept flag was set to 1 at the idle mode initiation.). Except fixed M2M device, Mobile M2M device shall perform the contention-based ranging. 0b01: dedicated ranging channel allocation, S-RCH 0b10: dedicated ranging channel allocation, NS-RCH 0b11: No dedicated ranging channel |  |
| … |  |  |  |
| If (M2M network access type == 0b00) { |  |  |  |
| Assignment A-MAP start offset for AAI-RNG-REQ |  | This parameter indicates the offset in units of frames that M2M device starts to monitor the resource (i.e., Fixed M2M Ranging Assignment A-MAP IE) for the AAI-RNG-REQ message, where the reference point of this offset value is the frame in which the AAI-PAG-ADV is transmitted. |  |
| Resource monitor timer |  | Time duration that M2M device monitors the resource (i.e., Fixed M2M Ranging Assignment A-MAP IE) for AAI-RNG-REQ message. |  |
| } |  |  |  |
| … |  |  |  |
| For (i=0; i<Num_FMDID; i++) { |  | Num_FMDID indicates the number of FMDIDs included in this paging message [1..32] | Shall be included when the ABS pages the fixed M2M devices in localized idle mode. |
| … |  |  |  |
| M2M network access type | 2 | Indicate the network re-entry access type for M2M device; 0b00: Resource allocation (i.e., Fixed M2M Ranging Assignment A-MAP offset) for AAI-RNG-REQ 0b01: dedicated ranging channel allocation in AAI-PAG-ADV 0b10: dedicated ranging channel allocation in broadcast assignment A-MAP IE 0b11: No dedicated ranging channel |  |
| if (M2M network access type == 0b00) { |  |  |  |
| Fixed M2M Ranging Assignment A-MAP offset for AAI-RNG-REQ |  | Indicate offset in units of frames that M2M device starts to monitor the resource (i.e., Fixed M2M Ranging Assignment A-MAP IE for AAI-RNG-REQ message is transmitted, where the reference point of this offset value is the frame in which the AAI-PAG-ADV is transmitted. |  |
| } |  |  |  |
| … |  |  |  |

# References

[1] P802.16.1b/D1