| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M group zone-based location update in 16p |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Giwon Park LG electronics | E-mail: eunjong.lee@lge.com |
| Re: | IEEE 802.16-11/0049r1, IEEE 802.16 Working Group Letter Ballot #33a |  |
| Abstract | This contribution proposes to define an additional location update condition for an M2M device |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

M2M group zone-based location update in 16p

Eunjong Lee, Youngsoo Yuk and Giwon Park

LG electronics

# Introduction

According to the current texts, the domain of the network entity that assigns M2MCID is identified by M2M GROUP ZONE ID transmitted in the DCD message. If an M2M device moves to another M2M group zone, the current M2MCID should be updated because the assigned M2MCID is only available in the M2M group zone. During connected state, the target BS can reassign the new M2MCID for the M2M device, based on the MS mobility information after handover. However, for an idle mode M2M device, the change of M2M group zone can be only detected by the MS because the selected preferred BS only knows the location of an MS based on the paging group. Therefore, in order to update the M2MCID, the M2M device should notify its location to the preferred BS when detecting the zone change. Then, the preferred BS can reassign the new M2MCID and M2M GROUP ZONE ID. So, this contribution proposes to trigger location update when an M2M device detects that the selected preferred BS does not support its currently assigned M2MCID. The M2M device can detect the change of M2M group zone by monitoring the M2M GROUP ZONE ID in the DCD message which is transmitted by the preferred BS.

# References

[1] IEEE P802.16p/D2, “Enhancements to Support Machine-to-Machine Applications” December 2011.

# Text proposal for inclusion in the 802.16p AWD

[Remedy1: Add new sub-clause 6.3.22.11.3 as indicated (page 14, line 20)]

-------------------------------------- Start of the text proposal -------------------------------------------------------------------------------------

6.3.22.11.3 Location update for M2M devices

6.3.22.11.3.1 Location update conditions

An M2M device in idle state may also perform location update when it detects that an M2M GROUP ZONE ID of the selected preferred BS is changed.

6.2.18.7.6.1.1 M2M Group Zone based update

An M2M device shall perform a Location Update process when an M2M device detects that the selected preferred BS does not support its currently assigned M2M GROUP ZONE ID. The M2M device shall detect that by monitoring M2M DEVICE GROUP ZONE ID in the DCD message, which is transmitted by the preferred BS. During location update, M2M device’s M2MCIDs may be updated.

------------------------------------------- End of the text proposal ----------------------------------------------------------------------------------------

[Remedy2: Add the following texts as indicated (page 6, line 22)]

-------------------------------------- Start of the text proposal -------------------------------------------------------------------------------------

6.3.2.3.5 RNG-REQ (ranging request) message

The following TLV parameter may be included in a RNG-REQ message when an M2M device is request M2MCIDs update.

M2MCID update request

This TLV is used to request M2MCIDs update.

------------------------------------------- End of the text proposal ----------------------------------------------------------------------------------------

[Remedy3: Add the following new TLV as indicated (page 29, line 34)]

-------------------------------------- Start of the text proposal -------------------------------------------------------------------------------------

11.5 RNG-REQ management message encodings

Add the following parameter at the end of Table 687 as indicated

Table 687 – RNG-REQ message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| SFID | 44 | 4 |  | OFDMA |
| M2MCID update request | 45 | Variable 3+ 2* Num M2MCID | Bits 0-15: current M2M GROUP ZONE ID Bits 16-19: Num_M2MCID Bits 20-23: reserved for(i=0; i<Num_M2MCID; i++) { current M2MCID value (16 bits) } | OFDMA |

------------------------------------------- End of the text proposal ----------------------------------------------------------------------------------------

[Remedy3: Modify the following text as indicated (page 30, line 15)]

-------------------------------------- Start of the text proposal -------------------------------------------------------------------------------------

11.6 RNG-RSP management message encodings

Add the following parameter at the end of Table 688 as indicated

Table 688 – RNG-RSP message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| MGID M2MCID Update | 49 | 4 12 | Bits 0-15: current MGID M2MCID value Bits 16-31: new MGID M2MCID value Bits 32-95 : new MGSS | OFDMA |

------------------------------------------- End of the text proposal ----------------------------------------------------------------------------------------