| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of M2M Group Zone Index in the RNG-RSP message |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Giwon Park LG electronics | E-mail: eunjong.lee@lge.com |
| Re: | IEEE 802.16-11/0051, IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This contribution proposes to clarify the M2M Group Zone based location update in 16.1 |  |
| Purpose | For discussion in 802.16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clean-up of M2M Group Zone Index in the RNG-RSP message

Eunjong Lee, Youngsoo Yuk and Giwon Park

LG electronics

# Introduction

The RNG-RSP message is for an individual M2M device and the M2M device belongs to only one M2M Group zone. Therefore, the M2M Group Zone Index of the current AAI-RNG-RSP message should be defined outside For loop. In addition, we deleted some texts related to the MGID and FID mapping.

# References

[1] P802.16.1/D1, “Enhancements to Support Machine-to-Machine Applications,” December 2011.

- Text proposal for inclusion in the 802.16p AWD

[Remedy: Add the fields into the Table 27 in line 63 of page 8 as follows]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

6.2.3.2 AAI-RNG-RSP

Modify Table 28 as indicated

Table 28—AAI-RNG-RSP message field description

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID (Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if the mapping between MGID and FID needs to be updated |
| Current MGID | 12 |  |  |
| New MGID | 12 |  |  |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| New MGSS | 64 |  | Shall be included only in encrypted AAI-RNG-REQ message when the MGID is updated. |
| } |  |  |  |
| … | … | … | … |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------