| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of M2M Group Zone based location update in 16.1 |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Giwon Park LG electronics | E-mail: eunjong.lee@lge.com |
| Re: | IEEE 802.16-11/0051, IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This contribution proposes to clarify the M2M Group Zone based location update in 16.1 |  |
| Purpose | For discussion in 802.16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification of M2M Group Zone based location update in 16.1

Eunjong Lee, Youngsoo Yuk and Giwon Park

LG electronics

# Introduction

In the last meeting, we defined an M2M Group Zone based location update process. This process is triggered when an M2M device detects that the selected preferred ABS does not support its currently assigned MGID. However, there is still an unclear point to reassign new MGID because the preferred ABS may not know information for the service flow of the M2M device. In order to clarify the MGID reassignment, we propose that the M2M device transmit the current MGID and M2M Group Zone ID in the AAI-RNG-REQ message transmitted during the M2M Group Zone based location update.

# References

[1] P802.16.1/D1, “Enhancements to Support Machine-to-Machine Applications,” December 2011.

- Text proposal for inclusion in the 802.16p AWD

[Remedy: Add the fields into the Table 27 in line 63 of page 8 as follows]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

6.2.3.1 AAI-RNG-REQ

Modify Table 27 as indicated

Table 27—AAI-RNG-REQ message field description

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| } else if (Ranging Purpose Indication == 0b0011 ¦0b0110¦0b0111¦ 0b1011) { |  | // Idle mode location update (and with other additional purposes) | . |
| … | … | … | … |
| Current M2M GROUP ZONE ID | 12 | M2M Group Zone ID for the current MGID | Present when the M2M device requests the MGID reassignment. |
| For(i=0;i<Num_MGID;i++){ |  |  |  |
| Current MGID | 12 | Current MGID | Present when the M2M device requests the MGID reassignment. |
| } |  |  |  |
| … | … | … | … |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------