| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of M2M specific broadcast messages (MOB_MTE-IND/ MGMC) |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Jeongki Kim, Giwon Park, Youngsoo Yuk, Jinsam Kawk LG Electronics | Email: jeongki.kim@lge.com |
| Re: | IEEE 802.16-11/0049, IEEE 802.16 Working Group Letter Ballot #33a |  |
| Abstract | This contribution proposes the text to clarify M2M specific broadcast message (MOB_MTE-IND/MOB_MGMC) |  |
| Purpose | For discussion in 802.16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification of M2M specific broadcast message (MOB_MTE-IND/MGMC)

Jeongki Kim Giwon Park, Youngsoo Yuk and Jinsam Kwak

LG Electronics

# Introduction

At the last meeting, it was decided that MOB_MTE-IND message is broadcast. However, it will result in the impact of the legacy devices (SSs/MSs) because the legacy devices will try to decode all broadcast bursts sent by broadcast CID. That is, MAC control messages designed newly for M2M applications should not be decoded by the legacy devices.

To minimize the impact of the legacy devices, the new M2M management CID will be used for M2M devices to receive the MOB_MTE-IND message.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 685 as follows, Page 26, line 34:]

Table 658—CIDs

| Transport, Secondary Man- agement, Tunnel or Manage- ment Tunnel, Multicast management CID | 2m+1–– n0xFE9F | For the secondary management connection, the same value is assigned to both the DL and UL connection. Tunnel CID is used for tunnel transport connections. Management Tun- nel CID is used for tunnel management connections. Multi- cast management CID is used for the downlink multicast management services. |
| --- | --- | --- |
| M2MCID | n+1 – 0xFE9FE | M2M multicast connection identifiers |
| M2M management CID | 0xFE9F | Used in DL-MAP to denote bursts for transmission of DL broadcast information to M2M devices. May also be used in MOB_MTE-IND messages. |

---- End of proposed text ----