| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of M2M specific broadcast messages (MOB_MTE-IND/ MGMC) |  |
| Date Submitted | 2012-01-18 |  |
| Source(s) | Jeongki Kim, Giwon Park, Youngsoo Yuk, Jinsam Kawk LG Electronics Anil Agiwal, Hyunjeong Kang, Anshuman Nigam, Rakesh Taori Samsung Electronics | Email: jeongki.kim@lge.com anilag@samsung.com, hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16-11/0049, IEEE 802.16 Working Group Letter Ballot #33a |  |
| Abstract | This contribution proposes the text to clarify M2M specific broadcast message (MOB_MTE-IND/MOB_MGMC) |  |
| Purpose | For discussion in 802.16p TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification of M2M specific broadcast message (MOB_MTE-IND/MGMC)

Jeongki Kim Giwon Park, Youngsoo Yuk and Jinsam Kwak

LG Electronics

Anil Agiwal, Hyunjeong Kang, Anshuman Nigam, Rakesh Taori

Samsung Electronics

# Introduction

At the last meeting, it was decided that MOB_MTE-IND message is broadcast. However, it will result in the impact of the legacy devices (SSs/MSs) because the legacy devices will try to decode all broadcast bursts sent by broadcast CID. That is, MAC control messages designed newly for M2M applications should not be decoded by the legacy devices.

To minimize the impact of the legacy devices, the new M2M management CID will be used for M2M devices to receive the MOB_MTE-IND message.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 685 as follows, Page 26, line 34:]

Table 658—CIDs

| Transport, Secondary Man- agement, Tunnel or Manage- ment Tunnel, Multicast management CID | 2m+1–– n0xFE9F | For the secondary management connection, the same value is assigned to both the DL and UL connection. Tunnel CID is used for tunnel transport connections. Management Tun- nel CID is used for tunnel management connections. Multi- cast management CID is used for the downlink multicast management services. |
| --- | --- | --- |
| M2MCID | n+1 – 0xFE9FE | M2M multicast connection identifiers |
| M2M management CID | 0xFE9F | Used in DL-MAP to denote bursts for transmission of DL broadcast information to M2M devices. May also be used in MOB_MTE-IND/MGMC messages. |

---- End of proposed text ----

[Remedy 2: Modify the text on page 5, lines 1-20 as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 53—MAC management messages

| No. | Message name | Message description | Connection |
| --- | --- | --- | --- |
| 110 | MOB_MTE_IND | Multicast transmission end indicator | Broadcast/Multicast |
| 111 | MGMC | M2M Group MAC Control Message | Broadcast/Multicast |
| 112 | MAMC | M2M ACK MAC Control Message | Basic |

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy3: Modify the text on page 10, lines 42-47, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

6.3.2.3.99 MGMC (M2M group MAC control) message

The MGMC message may be sent to a group of M2M devices that belong to the same M2M device group (defined by an M2MCID) to indicate parameters and/or instructions. The BS may send the MGMC message to M2M devices in normal operation mode by using either broadcast manner or multicast manner. In case of broadcast manner, M2M management CID will be used for sending the MGMC message. If MGMC carries control information for one M2M device group and Multicast SA is established for that M2M Device Group then the MGMC shall be encrypted using the established Multicast SA (see 7.2.2.2.14 for details on encryption method). The multicast PHY burst with the MAC PDU carrying the MGMC message intended for one M2M device group is indicated by the M2MCID in DL MAP. The CID field in the MAC PDU header is set to any M2MCID other than the M2MCIDs assigned to the M2M device and EC equals to zero or one indicates whether the MGMC message is encrypted or not. In case MGMC message is encrypted the multicast SA corresponding to the M2M device group identified by the M2MCID in DL MAP is used to decrypt the MGMC message.

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy4: Modify the text on page 10, lines 12-17, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

6.3.2.3.98 MOB_MTE-IND (Multicast transmission end indication) message

The BS shall send a MOB_MTE-IND message to a group of M2M devices to indicate the end of multicast transmission by using either broadcast manner or multicast manner. In case of broadcast manner, M2M management CID will be used for sending the MOB_MTE-IND message. If the Multicast SA is established for the group of these M2M devices then the MOB_MTE-IND shall be encrypted using the established Multicast SA (see 7.2.2.2.14 for details on encryption method). The PHY burst with the MAC PDU carrying the MOB-MTE-IND message is indicated by the M2MCID in DL MAP. The CID field in the MAC PDU header is set to any M2MCID other than the M2MCIDs assigned to the M2M device and EC equals to zero or one indicates whether the MOB_MTE-IND message is encrypted or not. In case MOB_MTE-IND message is encrypted the multicast SA corresponding to the M2M device group identified by the M2MCID in DL MAP is used to decrypt the MOB_MTE-IND message. When an M2M device in idle mode receives the MOB_MTE-IND message, the M2M device may enter the paging unavailable interval as specified in 6.3.22.4.

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy5: Modify the text on page 19, lines 38-42, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

7.2.2.2.14 M2M Group Traffic Encryption Key (M2MGTEK)

The M2MGTEK, which is shared among all M2M devices that belong to the multicast group, is used to encrypt data packets of the multicast service flow. MGTEK is also used to encrypt MAC management message transmitted to a group of M2M devices.

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy 6: Replace the figure 27b, on page 40, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

Figure 240b-M2M multicast MAC PDU ciphertext payload format

-------------------------------------------------End of Proposed Text -----------------------------------------------------