| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of M2M specific broadcast messages (AAI-MTE-IND/AAI-MGMC) |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Jeongki Kim, Giwon Park, Youngsoo Yuk, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | IEEE 802.16-11/0051, IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This contribution proposes the text to clarify the AAI-MTE-IND/AAI-MGMC message |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of M2M specific broadcast messages (AAI-MTE-IND/AAI-MGMC)

Jeongki Kim, Giwon Park, Youngsoo Yuk and Jinsam Kwak

LG Electronics

# Introduction

At the last meeting, it was decided that AAI-MTE-IND/AAI-MGMC message are broadcast. However, it will result in the impact of the legacy devices (AMSs) because the legacy devices will try to decode all broadcast bursts. That is, MAC control messages designed newly for M2M applications should not be decoded by the legacy devices.

To minimize the impact of the legacy devices, the new broadcast messages designed for M2M application will be transmitted by the Broadcast Assignment A-MAP IE with function index set to 0b11 instead of function index set to 0b00. The FID of MAC header of M2M broadcast message will be set to the same value (i.e., 0b0000) as that of the legacy broadcast message.

In addition, the masking method of broadcast assignment A-MAP IE with function index = 0b11 (for M2M dedicated ranging channel allocation) is not defined clearly. It should be cleaned up. If the masking code for broadcast assignment or ranging assignment (i.e., Type indicator = 0b001 and masking code = 0) is used for M2M dedicated ranging assignment, it will also result in unnecessary A-MAP IE parsing overhead of legacy devices. In this case we need also to minimize the impact of the legacy devices.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 194 as follows, Page 45, line 20:]

Table 194 - Description of the masking code for type indicator 010

| Decimal Value | Description. |
| --- | --- |
| 4094 | Used to mask Broadcast Assignment A-MAP IE for M2M broadcast assignment or M2M dedicated ranging assignment (i.e., Function Index = 0b11).. |
| 4095 | Used to mask Broadcast Assignment A-MAP IE for multicast assignment (i.e., Function Index = 0b10). |
| Others | Reserved |

[Remedy #2: Modify the Table 209 as follows, Page 57, line 28:]

Table 209—Broadcast Assignment A-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() {. |  |  |
| A-MAP IE Type. | 4 | Broadcast Assignment A-MAP IE. |
| Function Index. | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: Reserved This IE carries ranging channel allocation information for M2M devices or broadcast assignment information for M2M devices. |
| …. |  | … |
| else {//Function Index == 0b11. |  |  |
| Subfunction Index | 1 | 0b0: Ranging channel assignment information for M2M devices 0b1: Broadcast assignment information for M2M devices |
| If (subfunction index = 0b0) { |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity inde | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| Dedicated ranging indicator | 1 | 0: this ranging channel is used for purpose of normal ranging 1: this ranging channel is used for the purpose of dedicated ranging indicated in the AAI-PAG-ADV message |
| } |  |  |
| Reserved | 27/22 |  |
| }else { |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 303 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 UL AAI subframes for FDD or all UL subframes for TDD (Long TTI) |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 12 | Reserved bits |
| }else { |  |  |
| Reserved | 14 | Reserved bits |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

---- End of proposed text ----