| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Method of sending the broadcast message for lower power consumption of M2M devices |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Jeongki Kim, Giwon Park, Youngsoo Yuk, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | IEEE 802.16-11/0051, IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This contribution proposes the text to send the broadcast message for lower power consumption of M2M devices |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Method of sending the broadcast message for lower power consumption of M2M devices

Jeongki Kim, Giwon Park, Youngsoo Yuk and Jinsam Kwak

LG Electronics

# Introduction

Unlike 802.16 Rev3, for AMSs to receive broadcast MAC control message in 802.16.1, the AMSs should decode the all broadcast bursts for all broadcast MAC control message.

- Broadcast MAC control messages defined in 802.16.1 : AAI-SCD, AAI-SII-ADV, AAI-ULPC-NI, AAI-RNG-REQ, AAI-RNG-ACK, AAI-TRF-IND, AAI-NBR-ADV, AAI-PAG-ADV, PGID-Info, AAI-MC-ADV, AAI-DL-IM, AAI-SON-ADV, AAI-L2-XFER, AAI-LBS-ADV

However, in some case AMSs does not need to decode the all broadcast MAC control message. For example, AMSs in connected mode does not need to decode the DL broadcast bursts of MAC control messages for sleep mode (e.g., AAI-TRF-IND) or idle mode (e.g., AAI-PAG-ADV, PGID-Info). It will increase unnecessary process overhead of AMSs and power consumptions of AMSs because the AMS often performs the turbo decoding process to receive the unnecessary broadcast MAC control messages.

In M2M scenarios, the lower power consumption of M2M devices is one of the important features because most of M2M devices are not carried by human or users.

In case of idle mode operation in M2M multicast service, M2M devices will try to decode all broadcast bursts with multicast bursts. Some broadcast messages like AAI-RNG-ACK is not intended to the M2M devices. M2M devices in connected mode don’t have to decode the broadcast burst for idle mode broadcast MAC control messages (e.g., PGID-Info, AAI-PAG-ADV) or sleep mode broadcast MAC control messages(e.g., AAI-TRF-IND)

Also, M2M devices do not have to decode the broadcast burst of AAI-SCD message if the information of AAI-SCD does not change.

To minimize the PHY decoding processes and the power consumption of M2M devices, the type information of broadcast messages will be included in the broadcast assignment A-MAP IE. Based on the type information of broadcast messages in the A-MAP IE M2M devices will decide if the devices decode the broadcast burst of broadcast messages or not.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 209 as follows, Page 57, line 28:]

Table 209—Broadcast Assignment A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Broadcast Assignment A-MAP IE () { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: Reserved This IE carries ranging channel allocation information for M2M devices |
| If(Function Index == 0b00){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 954 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Message type bitmap | 8 | Indicates the type bitmap of broadcast MAC control messages included in this burst. This bitmap is used only for M2M devices. Bit #0: AAI-SCD Bit #1: AAI-TRF-IND Bit #2: AAI-PAG-ADV/PGID-Info Bit #3: AAI-RNG-ACK Bit #4: AAI-NBR-ADV Bit #5: AAI-MC-ADV Bit #6: AAI-E-MBS-CFG Bit #7: Other MAC messages |
| Reserved | 13 5 | Reserved bits |
| }else{ |  |  |
| Message type bitmap | 8 | Indicates the type bitmap of broadcast MAC control messages included in this burst. This bitmap is used only for M2M devices. Bit #0: AAI-SCD Bit #1: AAI-TRF-IND Bit #2: AAI-PAG-ADV/PGID-Info Bit #3: AAI-RNG-ACK Bit #4: AAI-NBR-ADV Bit #5: AAI-MC-ADV Bit #6: AAI-E-MBS-CFG Bit #7: Other MAC messages |
| Reserved | 15 7 | Reserved bits |
| } |  |  |
| } else if (Function Index == 0b01){ |  |  |
| …. |  | … |
| else {//Function Index == 0b11. |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity inde | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| Dedicated ranging indicator | 1 | 0: this ranging channel is used for purpose of normal ranging 1: this ranging channel is used for the purpose of dedicated ranging indicated in the AAI-PAG-ADV message |
| } |  |  |
| Reserved | 27/22 |  |
| } |  |  |
| } |  |  |
| } |  |  |

---- End of proposed text ----