| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of long cycle persistent allocation |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Jeongki Kim, Giwon Park, Youngsoo Yuk, Jinsam Kawk LG Electronics Ming-Hung Tao ITRI Honggang Li Intel | Email: jeongki.kim@lge.com |
| Re: | IEEE 802.16-11/0051, IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This contribution proposes the text related to de-allocation method in long cycle persistent scheduling |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of long cycle persistent scheduling

Jeongki Kim, Giwon Park, Youngsoo Yuk and Jinsam Kwak

LG Electronics

Ming-Hung Tao

ITRI

Honggang Li

Intel

# Introduction

In long cycle persistent scheduling, the one time de-allocation method is used when BS wants to use the persistent allocation for sending normal HTC traffic with higher priority temporally. In legacy persistent scheduling, a HFA is assigned to M2M device for error handling of PA A-MAP IE for de-allocation. However, it is not necessary in case of one time de-allocation. It may results in the unnecessary HFA overhead.

Therefore, in case of send the UL M2M Persistent Allocation A-MAP IE for de-allocation, ABS will not allocate the HARQ feedback channel for handling the error of UL M2M Persistent allocation A-MAP IE. And, although an M2M device receives the UL M2M persistent allocation A-MAP IE for one-time de-allocation, the M2M device does not send the ACK to ABS.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 210b as follows:]

6.3.5.5.2.4.16 UL M2M Persistent allocation A-MAP IE

The UL M2M persistent allocation A-MAP IE is specified in Table 210b.

Table 210b—UL M2M Persistent Allocation A-MAP IE

| Syntax | Size (bits) | Value/Description |
| --- | --- | --- |
| UL M2M Persistent Allocation A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL M2M Persistent Allocation A-MAP_IE |
| Allocation Period | 4 | Period of persistent allocation for M2M: 0b0000: deallocation 0b0001: 2 frames 0b0010: 4 frames 0b0011: 6 frames 0b0100: 5 superframes 0b0101: 10 superframes 0b0110: 25 superframes 0b0111: 50 superframes 0b1000~0b1110: reserved 0b1111: One-time re-allocation |
| if (Allocation Period==0b0000){ | - | - |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| De-allocation flag | 1 | 0b0: Permanent de-allocation. The resource indicated by resource index is de-allocated at UL subframe referenced by this A-MAP IE and the related persistent allocation is terminated. 0b1: One-time de-allocation The resource indicated by resource index is de-allocated at UL subframe referenced by this A-MAP IE and the related persistent allocation is retained. |
| If (De-allocation flag ==0b0) { |  |  |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 12 | Reserved bits |
| } else { |  |  |
| De-allocation flag | 1 | 0b0: Permanent de-allocation. The resource indicated by resource index is de-allocated at UL subframe referenced by this A-MAP IE and the related persistent allocation is terminated. 0b1: One-time de-allocation The resource indicated by resource index is de-allocated at UL subframe referenced by this A-MAP IE and the related persistent allocation is retained. |
| Reserved | 138 | Reserved bits |
| } |  |  |
| } else if (Allocation Period != 0b0000){ | - | - |
| IsizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 3 | HARQ Feedback Allocation |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| Initial_ACID | 4 | Initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| Reserved | 5 | Reserved bits |
| } |  |  |
| } |  |  |

[Remedy #3: Add the subclause 6.2.7.4 in P802.16.1b/D1 :]

6.2.7.4 Error handling procedure

In the case of one-time de-allocation (i.e., de-allocation type = 0b1) in UL M2M Persistent allocation A-MAP IE, the ABS shall not transmit a HARQ feedback allocation (HFA) in the UL M2M Persistent Allocation A-MAP IE.

---- End of proposed text ----