| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Service Architecture and its impact on Air Interface |  |
| Date Submitted | 2012-1-6 |  |
| Source(s) | Anshuman Nigam Hyunjeong Kang Anil Agiwal Rakesh Taori Samsung Electronics | E-mail: anshun@samsung.com |
| Re: | RE: WG LB 36 |  |
| Abstract | M2M Service Architecture and its impact on Air Interface |  |
| Purpose | To be discussed and adopted during Session 77 |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

M2M Service Architecture and MGID Update

Anshuman Nigam, Hyunjeong Kang, Anil Agiwal & Rakesh Taori

Samsung Electronics

# Introduction

In Session 76, a need was gauged for evaluating the need for MGID update. The group decided to study the MCBCS Service Architecture to judge the use of MGID from M2M service point of view.

This contribution briefs the MCBCS architecture and then proposes a parallel architecture for M2M Services.

# Basic MCBCS Service Architecture

Enhanced Multicast and Broadcast Service (E-MBS) provides an efficient method for concurrent transport

of DL data common to a group of users, using a common E-MBS ID and a FID.

Each ABS capable of providing E-MBS belongs to a certain E-MBS Zone and one ABS can belong to multiple

E-MBS zones. An E-MBS zone defined as a set of ABSs where the same E-MBS ID and FID is used

for transmitting the content of certain service flow(s).

To ensure proper multicast operation on networks of ABS employing E-MBS, the E-MBS IDs and FIDs

used for common E-MBS content and service shall be the same for all ABSs within the same E-MBS Zone.

To allow seamless transition from one E-MBS Zone to another without any interruption of E-MBS data service

and operation, the AMS should update E-MBS service flow management encodings including E-MBS

IDs and FIDs, Packet Classification Rule parameter(s), E-MBS Zone Identifier Assignment parameters. If the

AMS has no E-MBS IDs and FIDs information regarding the new E-MBS Zone, and then the AMS is required to acquire E-MBS IDs and FIDs context through the other procedures, i.e., location-update if AMS is in the

idle mode and handover if MS is in connected mode.

MCBCS Network Reference Model

MCBCS Content Server:

MCBCS Content Server provides the content of MCBCS services, like multimedia flows, data files, etc. The content provider can belong to the NSP or be a third party outside the WiMAX network. The interface between MCBCS Server function and Content Server is outside the scope of this specification.

MCBCS Server:

MCBCS server is a network entity, which hosts all MCBCS specific functional components in the CSN. It performs the following functions:

- IP multicast group management

- MCBCS program management

- MCBCS service announcement management including MCBCS service guide manipulation and distribution.

- MCBCS session management

- Data encryption support

- Application layer key management

- Security association between MCBCS Server and MBS Client

- Delivery of the mapping information for the MCBCS content’s IP address to the MCID per MBS Zone

MBS Proxy:

MBS Proxy is a control plane function in the NAP to support MCBCS service. It is unique per MBS zone.

- Interact with MCBCS Server to support MCBCS session management

- Relaying bearer establishment request from MCBCS Server to MBS DPF

- Interact with the MBS DPF to support service synchronization for DL macro diversity

- Interface with the MBS DPF to trigger the MCBCS data path establishment and release

MBS DPF:

The MBS DPF function is a data plane entity in the NAP. It is responsible for data plane bearer management and MCBCS data distribution. MBS DPF is unique per MBS zone. MBS DPF distribution between ASNs over R4 is not assumed.

- MCBCS bearer control management including the data path establishment, maintenance, and release

- MCBCS bearer traffic classification & delivery

- GRE key management and distribution

- After receiving session start trigger from MBS Proxy, act as an IGMP client to send IGMP report message for IPv4 (act as a MLD client to send MLD report message for IPv6) to the last MR between ASN and CSN to join IP multicast group tree between ASN and CSN

- Performing time stamping and packetization (fragmentation and packing) over the single frequency and multi-frequency WiMAX networks

- Forwarding IP multicast packet

MBS Agent:

MBS Agent includes the collection of MCBCS specific functions of ASN which are located at the BS. MBS Agent is a functional entity responsible for data plane bearer management and providing the resource allocation information to support the macro diversity.

- Support DPF functions at the BS

- Construct the MBS subframe based on the information configured by NAP and the sync information included in each SDU, and then transmit over the air.

- Broadcast MBS_MAP_IE, MBS_MAP and MBS_DATA_IE including MBS zone ID and MCID

MBS Client:

MBS Client represents functionality required in the MS for MCBCS service reception. MBS client implements the following functions:

- Compliant to IEEE 802.16 standard as much as possible

- IP multicast capable IPv4 and IPv6 stack

- Application layer – Application client at the application layer may be responsible among others for the following functions:

● Service discovery/announcement

● Service subscription/registration

● Application layer security (if required)

● Optional statistic collection support

● Reconstruct the MCBCS program content

# General Operational Procedures

MS acquires the MBS Zone ID and MCID mapped to the desirable contents from the mapping table acquired during 8 Service Announcement/Service Guide Delivery procedures.

MBS Proxy/MBS DPF calculates a frame number at which the packet shall be transferred on the air frame, using contents delivery start time received from MCBCS Server.

The general service guide includes the mapping table between the application layer ID (Program ID or Content ID) and the IP layer ID (Contents IP) basically.

Mapping table between Contents IP and MCID per MBS Zone ID can be delivered during either a Service Guide acquisition or a Subscription procedure.

{Program ID, Content ID, Mapping table} where Mapping table is {MBS Zone ID, MCID}

When the User wants to watch a specific channel after MBS Client receives a Service Guide from MCBCS Server, he selects the specific channel. Then, the MBS Client pushes the Multicast IP address corresponding to the selected channel down to the IP layer and the MCID and MBS zone ID corresponding to the selected channel down to the  MAC layer using the channel mapping information between layers among the Content ID(s), Multicast IP Address(es) and MCID(s). Based on the obtained MCID and MBS zone ID, MS can choose to only decode the MBS burst corresponding to the selected channel.

# Basic M2M Service Architecture

# M2M Zone

An M2M Zone spans a geographically contiguous or non-contiguous area.

M2M Service Architecture possibly can be very similar to MBS Service Architecture

M2M Service ID should ultimately be mapped to M2M Zone ID and MGID.

Is MGID update needed?

- Like in MBS, a mapping of services to MGID in neighboring zones needs to be provided for seamless service continuation.

Is zone update information needed?

- Yes, as it indicates which services are supported in the new zone besides indicating the associated MGID.

# M2M Device Operation

The M2M device subscribes to a service and after successful subscription, it gets the mapping of the M2M Service ID to the M2M Zone ID and the MGID corresponding to the zone in which it has done the subscription. The M2M device then decodes the service data using the MGID. On change of zone, the device gets/fetches the zone update information which contains the list of services in the zone along with the associated MGID. If the service is continued, the M2M device uses the new MGID as indicated in the zone update information.

# Conclusion

Text Proposal:

Insert at the end of section 6.2.1.3.1, page 5, line 36

The information of the neighboring M2M Group Zones may be advertised by ABSs of a given M2M Group Zone in AAI_NBR-ADV message. Neighboring M2M Group Zones implies the M2M Group Zones that neighboring ABSs belong to which are different from the M2M Group Zones that the serving ABS belongs to.

The AAI_NBR-ADV message contains M2M_GROUP_ZONE_ID of the neighboring M2M Group Zones along with the mappings of MGID from a M2M Group Zone of the serving ABS to one or more neighboring M2M Group Zones. When an M2M device changes its preferred or serving ABS to an ABS which belongs to a different M2M Group Zone than the current serving ABS, it may have the MGID mapping information for the M2M Zones of the preferred ABS and can derive M2M Group Zone Index corresponding to M2M_GROUP_ZONE_IDs supported by the preferred ABS, if it has already received the AAI_NBR-ADV. The ordering of M2M_GROUP_ZONE_IDs is same as in AAI-SCD message for each neighboring ABS.

The AAI_NBR-ADV message including M2M Group Zone information may only be transmitted by the ABSs which are situated at the M2M Group Zone boundaries.

Append at the end of Table 39, Section 6.2.3.13, line 50

6.2.3.13 AAI_NBR-ADV

Table – 39

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| }//end of for N-NBR-ABSs |  |  |  |
| For (j=0;j<N-NBR-ABSs; j++) { | 6 | N-NBR-ABSs denotes the number of neighboring ABSs |  |
| For (n=0;n<N-M2M-GROUP-ZONE;n++) { | 2 | N-M2M-GROUP-ZONE denotes the number of M2M Group Zones that the neighbor ABS belongs to. | Present when N-M2M-GROUP-ZONE > 1 |
| M2M_GROUP_ZONE_ID | 16 |  |  |
| } |  |  |  |
| } |  |  |  |
| For (i-0; i<Num_M2M_Zones; i++) { | 2 | Num_M2M_Zones denotes the number of neighboring M2M Zones |  |
| M2M_GROUP_ZONE_ID | 16 | Denotes the neighbor M2M_GROUP_ZONE_ID |  |
| For (m=0; m<Num_MGID_Mapping; i++) { | 12 | Num_MGID_Mapping is the number of mappings of current MGID and new MGID between the serving M2M Zone and the neighbor M2M Zone, where the current MGID is not the same as the new MGID. | Present when the mapping of M2M Service IDs to MGIDs is different in the neighboring M2M Zone |
| Current MGID to New MGID mapping | 24 | The 12 LSBs denote the MGID in current M2M Zone and the 12 MSBs denote the corresponding MGID in the associated neighboring M2M Zone. A value of 0 for 12 MSBs denotes that the associated service is not supported in the corresponding neighboring M2M Zone. |  |
| For (i=0; i<N-NBR-R1-BSs; i++) { |  |  |  |

# References

- DRAFT-T33-113-R015v01-B_MCBCS-Applayer

- IEEE 802.16m/D12

- IEEE 802.16.1b/D1 WirelessMAN-Advanced Enhancements to Support Machine-to-Machine Applications