| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Zone index in control message |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Anshuman Nigam, Rakesh Taori Samsung Electronics | anilag@samsung.com, hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-11/0051, Working Group Letter Ballot # 36 |  |
| Abstract | The contribution proposes to add M2M zone index in control messages in draft IEEE P802.16.1b/D1 |  |
| Purpose | To be discussed and adopted in IEEE P802.16.1b/D2 |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

M2M zone index in control message

Anil Agiwal, Hyunjeong Kang, Anshuman Nigam, Rakesh Taori

Samsung Electronics

- Introduction

MGID is associated with a M2M group zone. In the current draft [1] for several messages M2M_Group Zone Index is not added together with MGID.

- Proposed Text Change

Adopt the following remedies in IEEE P802.16.1b/D2.

[Remedy1: Modify the text on page 30, lines 18-42, and Table 70 as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

Table 70—AAI-PKM-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (PKM v3 message code == 11) { |  |  |  |
| MGID | 15 12 | Multicast group identifier | May be present when an M2M device is registered for M2M multicast service of the M2M device group |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| MGSS | 64 | MGSS of the currently used MGTEK | May be present when an M2M device is registered for M2M multicast service of the M2M device group |
| M2MGTEK_COUNT |  | The index of the currently used MGTEK | May be present when an M2M device is registered for M2M multicast service of the M2M device group |
| } |  |  |  |

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy2: Modify the text on page 29, lines 24-36, and Table 69 as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

Table 69—AAI-PKM-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (PKM v3 message code == 10) {. |  |  |  |
| MGID | 15 12 | Multicast group identifier that the AMS subscribes. | May be present when an M2M device is registered for M2M multicast service of the M2M device group |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| } |  |  |  |

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy3: Modify the text on page 34, lines 50-55, and Table 106a as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

Table 106a—AAI-MTE-IND message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| MGID | 4 12 | MGID related to the multicast traffic | May be present when an M2M device is registered for M2M multicast service of the M2M device group |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| } |  |  |  |

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy4: Modify the text on page 34, lines 50-55, and Table 106b as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

Table 106b—AAI-MGMC message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| for(i=1; i<=Num_MGID; i++) {. |  | Number of MGID to be updated [1..TBD]. | Presented if a MGID needs to be updated |
| Current MGID. | 12 | Current MGID value |  |
| New MGID | 12 | New MGID value to be assigned |  |
| M2M_Group Zone Index | 2 | M2M Group Zone Index of the corresponding M2M Group Zone ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone. |
| } |  |  |  |

-------------------------------------------------End of Proposed Text -----------------------------------------------------

- Reference

[1] IEEE P802.16.1b/D1 December 2011