| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast M2M MAC Control Messages |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Anshuman Nigam, Rakesh Taori Samsung Electronics | anilag@samsung.com, hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-11/0051, Working Group Letter Ballot # 36 |  |
| Abstract | The contribution proposes to clarify transmission of Multicast M2M MAC Control messages in draft IEEE P802.16.1b/D1 |  |
| Purpose | To be discussed and adopted in IEEE P802.16.1b/D2 |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Multicast M2M MAC Control Messages

Anil Agiwal, Hyunjeong Kang, Anshuman Nigam, Rakesh Taori

Samsung Electronics

- Introduction

In the current draft [1] AAI-MGMC and AAI_MTE_IND messages are intended for a group of M2M devices but they are broadcasted. AAI-MTE-IND should be multi-casted using the MGID. AAI-MGMC should be multi-casted when it carries control information for a single M2M device group, otherwise it should be broadcasted.

The multicast security is also defined in current draft [1]. So AAI-MGMC and AAI_MTE_IND messages should be protected using the multicast security.

- Proposed Text Change

Adopt the following remedies in IEEE P802.16.1b/D2.

[Remedy1: Modify the text on page 7, lines 1-15, and section 6.2.3 as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

Table 26—MAC Control messages

| No. | Functional areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| 70 | Relay | AAI-ARS-CONFIG-CMD | ARS Configuration Command | N/A | Unicast |
| 70 | M2M | AAI-MGMC | M2M device group MAC Control | N/A: If it carries control information for multiple M2M device groups or if it carries control information for one MM device group and Multicast SA is not established for that M2M device group. Encrypted: If it carries control information for one M2M device group and Multicast SA is established for the that M2M Device Group | Broadcast/Multicast (using MGID) |
| 71 | M2M | AAI-MTE-IND | M2M Multicast Traffic End indication | N/A: If Multicast SA is not established for the intended M2M Device Group (identified by MGID) Encrypted: If Multicast SA is established for the intended M2M Device Group | Multicast (using MGID) |
| 712-255 |  |  | Reserved |  |  |

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy2: Modify the text on page 34, lines 60-65, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

6.2.3.65 AAI-MGMC (M2M device group MAC control) message

The AAI-MGMC message may be sent to a group of M2M devices that belong to the same M2M device group (defined by a MGID) to indicate parameters and/or instructions. The ABS may send the AAI-MGMC message to M2M devices in connect state. If AAI-MGMC carries control information for one M2M device group and Multicast SA is established for that M2M Device Group then the AAI-MGMC shall be encrypted using the established Multicast SA (see 6.2.5.5 for details on encryption method). The PHY burst with the MAC PDU carrying the AAI-MGMC message intended for one M2M device group is indicated by the M2M group assignment A-MAP IE and the FID in the MAC PDU carrying the unencrypted or encrypted AAI-MGMC message is set to zero or one respectively. In case AAI-MGMC message is encrypted the multicast SA corresponding to the M2M device group identified by the MGID in M2M group assignment A-MAP IE is used to decrypt the AAI-MGMC message.

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy3: Modify the text on page 34, lines 39-45, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

6.2.3.64 AAI-MTE-IND (Multicast transmission end indication) message

The ABS shall multicast (using MGID) AAI-MTE-IND message to M2M devices to indicate the end of multicast transmission. If the Multicast SA is established for the group of these M2M devices then the AAI-MTE-IND shall be encrypted using the established Multicast SA (see 6.2.5.5 for details on encryption method). The PHY burst with the MAC PDU carrying the AAI-MTE-IND message is indicated by the M2M group assignment A-MAP IE and the FID in the MAC PDU carrying the unencrypted or encrypted AAI-MTE-IND message is set to zero or one respectively. In case AAI-MTE-IND message is encrypted the multicast SA corresponding to the M2M device group identified by the MGID in M2M group assignment A-MAP IE is used to decrypt the AAI-MTE-IND message. If an M2M device in idle mode receives the AAI-MTE-IND message, the M2M device may enter the paging unavailable interval as specified in 6.2.18.2.

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy4: Modify the text on page 38, lines 1-6, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

6.2.5.5.1.1 MGTEK Derivation

The MGTEK is the transport encryption key used to encrypt M2M service multicast data. The MGTEK is derived based on the MGSS, M2MGTEK_COUNT and the MAK (M2M service Authorization Key). The generation and transport of the MAK is outside the scope of the IEEE 802.16 standard. MGTEK is also used to encrypt MAC control message transmitted to a group of M2M devices.

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy 5: Modify the text on page 38, lines 59-64, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

6.2.5.5.3 MGTEK Key Usage

The MGTEK is used for encrypting DL multicast data by the ABS, which is also used for decrypting such DL multicast data by the AMS. MGTEK is also used for encrypting DL MAC control message transmitted by the ABS to a group of M2M devices, which is also used for decrypting such DL MAC control message by the M2M devices.

-------------------------------------------------End of Proposed Text -----------------------------------------------------

[Remedy 6: Replace the figure 27b, on page 40, as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------

Figure 27b-M2M multicast MAC PDU ciphertext payload format

-------------------------------------------------End of Proposed Text -----------------------------------------------------

- Reference

[1] IEEE P802.16.1b/D1 December 2011