| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Signaling Header Protection |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Youngkyo Baek, Anshuman Nigam, Rakesh Taori Samsung Electronics | anilag@samsung.com, hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-11/0051, Working Group Letter Ballot # 36 |  |
| Abstract | This contribution proposes method to protect the MAC signaling headers |  |
| Purpose | To be discussed and adopted in IEEE P802.16.1b/D2 |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

MAC Signaling Header Protection

Anil Agiwal, Hyunjeong Kang, Youngkyo Baek, Anshuman Nigam, Rakesh Taori

Samsung Electronics

- Introduction

MAC signaling headers are not protected in draft [1]. The MAC signaling header carries important control information. These signaling headers are processed by BS and the M2M device without validating the source. A rogue M2M device may impersonate an M2M device and send false information in signaling headers which will affect the operation of M2M device.

In this contribution we propose a method to protect the signaling header. The proposed mechanism does not affect the already defined fields of existing MAC signaling headers. Key points of the proposed scheme are as follows:

- An FID is reserved to indicate the protected MAC signaling header.

- ROC & short CMAC is appended to the contents of MAC signaling header if the newly reserved FID is set in the FID field of MAC signaling header

- Proposed Text Change

Adopt the following remedies in IEEE P802.16.1b/D2.

[Remedy 1: Insert the following text at page 5, line 51]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.2.1.3 MAC Signaling Header

The signaling header shall be sent standalone or concatenated with other MAC PDUs in either DL or UL.

One FID is reserved for MAC signaling header. An additional FID is reserved for M2M devices to indicate the protected MAC signaling header. The value of FID for unprotected MAC signaling header is 0010. The value of FID for protected MAC signaling header is 0100.

All MAC signaling header formats follow the layout defined in Table 661.

Table 661 – MAC Signaling Header Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MAC Signaling Header () { |  |  |
| FID | 4 | Flow Identifier. Set to 0b0010 or 0b0100 |
| Type | 5 | MAC Signaling header type |
| Length | 3 | Indicates the length of the signaling header (includes the FID, Type, Length, and contents): 0b000 and 0b001: Reserved 0b010: 2 bytes 0b011: 3 bytes 0b100: 4 bytes 0b101: 5 bytes 0b110: 6 bytes 0b111: Reserved |
| Contents | variable; ≤ 36 | MAC signaling header contents, with the size indicated by the length field. The size in bits is Length × 8 – 12 |
| If (FID == 0b0100) { |  |  |
| ROC | 8 | Roll over counter (see section 6.2.5.7 for details) |
| CMAC | 16 | CMAC generated on FID, Type. Length & Contents (see section 6.2.5.7 for details) |
| } |  |  |
| } |  |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 2: Insert the following text at page 5, line 48]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2.1.2.2 Flow identifier (FID)

Each AMS connection is assigned a 4-bit FID that uniquely identifies the connection within the AMS. FIDs identify control connection and unicast transport connections. FID for E-MBS connection is used along with a 12-bit E-MBS ID to uniquely identify a specific E-MBS flow in the domain of an E-MBS zone (see 6.9.3.2). FID for multicast connection is used along with a 12-bit Multicast Group ID to uniquely identify a specific multicast flow in the domain of an ABS. DL and UL Transport FIDs are allocated from the transport

FID space as defined in Table 658. An FID that has been assigned to one DL transport connection shall not be assigned to another DL transport connection belonging to the same AMS. An FID that has been assigned to one UL transport connection shall not be assigned to another UL transport connection belonging to the same AMS. An FID that has been used for a DL transport connection can be assigned to another UL transport connection belonging to the same AMS, or vice versa. Some specific FIDs are pre-assigned. The values of 0000 and 0001 are used to indicate control FIDs. The values of 0010 and 0011 are used to indicate FID for unprotected signaling header and FID for default service flow, respectively. The value of 0b0100 is used to indicate FID for protected signaling header. See Table 658 for the specific allocation of FIDs. An FID in combination with an STID uniquely identifies any connection in an ABS.

Table 658 – Flow Identifiers

| Value | Description |
| --- | --- |
| 0000 | Control FID for unencrypted control connection payload in the MAC PDU (unicast control FID when PDU is allocated by unicast assignment A-MAP IE; broadcast control FID when PDU is allocated by broadcast assignment A-MAP IE) |
| 0001 | Unicast Control FID for encrypted control connection payload in the MAC PDU |
| 0010 | FID for unprotected Signaling header |
| 0011 | FID for transport connection associated with default service flow (UL and DL) |
| 0100-1111 | FID for transport connection. For M2M devices FID 0b0100 is reserved for indicating the protected signaling header. |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

[Remedy 3: Insert the new section]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.5.7 Signaling header protection using Cipher-based message Authentication Code (CMAC)

An M2M device may support integrity protection of signaling header based on CMAC-together with the AES block cipher. The CMAC construction as specified in NIST Special Publication 800-38B shall be used. The calculation of the keyed hash value contained in the CMAC Digest attribute and the CMAC Tuple shall use the CMAC algorithm with AES. The authentication keys CMAC_SIG_KEY_U and CMAC_SIG_KEY_D shall be used for authenticating signaling header transmitted in the UL and DL direction respectively. The CMAC_SIG_KEY_U and CMAC_SIG_KEY_D are derived as follows:

CMAC_SIG_KEY_U|CMAC_SIG_KEY_D = Dot16KDF (CMAC-TEK prekey,”CMACSIG”, 256).

ROC shall be reset to zero upon obtaining a new CMAC_SIG_KEY_*. The first 5 most significant bits of the ROC is the rollover counter for the frame number, i.e., when the frame number reaches 0x000000 (from 0xFFFFFF) it is incremented by 1 mod 32. The 3 least significant bits of ROC shall be allocated to signaling header in such manner that no two signaling headers in the same frame using the same CMAC_SIG_KEY_* have the same ROC value. Using this method, up to 8 signaling headers per frame using the same CMAC_SIG_KEY_* can be supported. A new encryption keys (CMAC_SIG_KEY_*) are required every 2^24 * 2^5 = 2^29 frames. The reauthorization process should be initiated to establish a new PMK/AK before the 5 bit of ROC concatenated with the frame number reaches 0x1FFFFFFF.

The CMAC value shall be calculated over a field consisting of the AK ID followed by the super frame number and frame index expressed as an unsigned 24-bit number, followed by the 12-bit STID and 4-bit FID (FID of signaling header), followed by 8 bit ROC, followed by 16-bit of zero padding (for the header to be aligned with AES block size) and followed by the contents of signaling header other than CMAC. The CMAC value for signaling header shall be calculated as follows:

CMAC value = Truncate(CMAC (CMAC_SIG_KEY_*, AK ID |22 bit super frame number|2 bit frame index |STID|FID|ROC|16-bit zero padding |Signaling header contents other than CMAC and ROC), 16).

The LSB 16-bit of the outcome of AES-CMAC calculation shall be used for CMAC value.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 4: Insert the new section]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.5.2.1.2 Key hierarchy

Figure 447—CMAC key, CMAC SIG key and TEK derivation from AK

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 5: Modify lines 15-22, section 6.2.28.4 on page 51 as shown below]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

To access the multicast service, the M2M device that is assigned an MGID shall apply the 16-bit CRC mask with masking prefix = 0b0, message type indicator = 0b010, and decimal value = 4094 to decode the assignment A-MAP IE. If the MGID is included in the Broadcast Assignment A-MAP IE, the M2M device shall obtain the multicast burst according to the instruction in the Broadcast Assignment A-MAP IE. The FID field in the MAC header of the MAC PDU (in multicast burst) carrying the MAC SDUs for the multicast service flow is set to 0100 0101.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

- Reference

[1] IEEE P802.16.1b/D1 December 2011