| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID update related corrections |  |
| Date Submitted | 2012-01-13 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI Eunjong Lee, Youngsoo Yuk and Giwon Park LG electronics | Voice: +82-42-860-5928 E-mail: sjjung@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #36 |  |
| Abstract | This contribution proposes a text of MGID update in idle state to be included in the 802.16.1b amendment. |  |
| Purpose | To be discussed and adopted by the 802.16 M2M TG. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

MGID update related corrections

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

Eunjong Lee, Youngsoo Yuk and Giwon Park

LG electronics

# Introduction

The MGID uniquely identifies a downlink multicast service flow within an M2M Group Zone. During the idle state, the MGID of M2M devices may be changed by location update or network reentry. And MGID may be updated within a zone or on M2M Group Zone change. The AAI-RNG-RSP message is for an individual M2M device and M2M device belongs to only one M2M Group zone. Therefore, the M2M Group Zone Index of the current AAI-RNG-RSP message should be defined outside For loop. In addition , M2M_Group_ Zone_ Index (or M2M_GROUP_ ZONE_ ID) is missed in some control messages. Therefore, we propose to correct the related texts.

# References

[1]IEEE P802.16.1b/D1, “DRAFT Amendment to IEEE Standard for WirelessMAN-Advanced Air interface for Broadband Wireless Access Systems: Enhancements to Support Machine-to-Machine Applications,” December 2011

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Text added by this contribution: blue underline texts

- Text deleted by this contribution: red strikeout texts

-------------------------------Start of the text proposal-----------------------------------------------------------------------

6.2.3.2 AAI-RNG-RSP

Modify Table 28 as indicated

Table 28 – AAI-RNG-RSP message field description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| …. |  | …. | … |
| Bandwidth grant indication | 1 | … | … |
| M2M_Group_Zone_Index | 2 | M2M_Group_Zone_Index of the corresponding M2M_Group_Zone_ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID(Num_MGID) to update in the T-ABS[1,..TBD]. Mapping of current MGID and FID and new MGID and FID to be update. | Present if the mapping between MGID and FID needs to be update. |
| Current MGID | 12 |  |  |
| New MGID | 12 |  |  |
| M2M Group Zone Index | 2 | M2M_Group_Zone_Index of the corresponding M2M_Group_Zone_ID that the MGID belongs to | Present if ABS is part of more than one M2M group zone |
| New MGSS | 64 |  | Shall be included only in encrypted AAI-RNG-REQ message when the MGID updated |
| } |  |  |  |
| …. |  |  |  |
| If(Location Update Response ==0x0){ |  |  |  |
| … |  |  |  |
| New Fixed M2M Deregistration ID | 16 | New FMDID that the fixed M2M device shall maintain in idle mode | Only present if the Localized Idle Mode flag is set to 1 in AAI-DREG-REQ/RSP message. |
| … |  |  |  |
| } |  |  |  |
| … |  |  |  |

6.2.3.64 AAI-MTE-IND(Multicast transmission end indication)message

Change Table 106a as indicated

Table 106a AAI-MTE-IND message field description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| MGID | 412 | MGID related to the multicast traffic |  |
| M2M Group Zone Index | 2 | M2M_Group_Zone_Index of the corresponding M2M_Group_Zone_ID that the MGID belongs to |  |

6.2.3.65 AAI-MGMC (M2M device group MAC control) message.

Change Table 106a as indicated

Table 106b AAI-MGMC message field description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Code | 2 | Use to identify the purpose if this message 0b00:re-assignment of MGID value 0b01-0b11:Reserved |  |
| If(Action Code ==0x00){ |  |  |  |
| For(i=1;i<=Num_MGID;i++){ |  | Number of MGID to be updated[1,…TBD] | Present if a MGID needs to be updated |
| Current MGID | 12 | Current MGID value |  |
| New MGID | 12 | New MGID value to be assigned |  |
| M2M Group Zone index | 2 | M2M_Group_Zone_Index of the corresponding M2M_Group_Zone_ID that the MGID belongs to |  |
| } |  |  |  |
| } |  |  |  |

6.2.18.7.2 Network re-entry from idle mode for M2M devices

Modify the texts on page 44, line 36 as follows

When the M2M device receives the group paging message (i.e., AAI-PAAG-ADV with MGID and M2M Group Zone Index) and M2M network access type is set to 0b00

Modify the texts on page 44, line 44as follows

If the M2M device receives the group paging message( i.e., AAI-PAG-ADV with MGID and M2M Group Zone Index) and M2M network access type is set to 0b01

Modify the texts on page 44, line 50 as follows

If the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID and M2M Group Zone Index) and M2M network access type is set to 0b10

Modify the texts on page 44, line 56 as follows

If the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID and M2M Group Zone Index) and M2M network access type is set to 0b11,

6.2.28 Multicast operation for machine to machine (M2M) applications

Modify the texts on page 51, line 5 as follows

Multicast Service for M2M applications provides concurrent transport of DL data common to M2M devices belonging to an M2M device group using an MGID and M2M Group Zone Index in an ABS. Multicast service is associated with an ABS and is offered in the downlink only.

6.2.28.4.2 Multicast connection establishment

Modify the texts on page 51, line 53 as follows

The AAI-DSC procedures are used to change multicast service flows. The AAI-DSD procedure can be used to delete the multicast service flow for an M2M device. In addition, the multicast service flow of an M2M device are deleted when the M2M device exits from a network or enters DCR mdoe. The M2M device shall retain service flow information associated multicast service during idle mode.The ABS AAI-DSA-REQ/RSP to the M2M device with the relevant multicast parameters including MGID and M2M Group Zone Index.

------------------------------End of the text proposal--------------------------------------------------------------------------