| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of talk-around direct communication |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Miyoung Yun, Eunkyung Kim, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16.1a Draft AWD, IEEE C802.16n-11/0033 |  |
| Abstract | This provides AWD text proposals of clarifying PHY controls and MAC operations of talk-around direct communications in IEEE 802.16.1a |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Clarification of talk-around direct communication

Sungcheol Chang, Seokki Kim, Miyoung Yun, Eunkyung Kim, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

This contribution provides clarification of PHY controls and MAC operations of talk-around direct communications between HR-MSs as the followings:

- Modifying the format of Synchronization channel IE

- Updating sequences and mapping of feedback channel

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Modify the following text in Section 6.12.2.3.2.2.3 in the 802.16.1a AWD]

6.12.2.3.2.2.3 Synchronization channel IE

Synchronization channel message is transmitted after channel encoding. The synchronization channel IE is composed of the fields in Table 1203.

Table 1203—Synchronization channel IE

| Field name | Field size Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Transmitter HR-MS ID | TBD |  |  |
| Reference time type | 2 | Indicate type of the source clock for the Reference Synchronization channel. 0b00: HR-BS 0b01: GPS 0b10: HR-MS 0b11: Reserved |  |
| Hop count | 2 | Indicate the hop counter of the reference synchronization channel from source clock. If an HR-MS receives signal from HR-BS, the Hop counter in Synchronization channel IE sent by the HR-MS is 1. |  |
| Reference signal strength | TBD 8 | The Reference Signal Strength indicates the received signal strength of Reference Preamble Sequences for synchronization channel, which measured by the HR-MS from this preamble sequences. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75dBm. An HR-MS shall report values in the range from -103.75 dBm to -40dBm. |  |
| Frame structure information | 4 | Indicate the usage of direct communication resources for direct communication frame. Bit 0: Common Direct Mode Zone Extended (CDMZ-E) Values from Bit 1 to Bit 3 indicates type of Cell Specific Direct Mode Zone (CSDMZ). 0b000~0b111: Reserved |  |
| CRC | 16 |  |  |

## [Remedy2: Modify the following text in Section 6.12.2.3.2.2.3 in the 802.16.1a AWD]

6.12.2.3.2.4.3 Feedback channel

The Sup-CH can be used to transmit the feedback channel, that includes ACK channel, NAK channel, MCS Change Confirm, RCHG Indication, etc. The feedback channel is transmitted using the slots which are not used by the ranging channel and the CQI channel. The feedback channel uses the same codeword set and symbol sequence to subcarrier mapping as the CQI channel. The codeword sequences and mapping of feedback channel are defined in Table 1205.

Table 1205 – Sequences and mapping of feedback channel

| Index | Sequence | Usage |
| --- | --- | --- |
| 0 | 11111111 | ACK |
| 1 | 0010110001 | NAK for frame 0 |
| 2 | 0100100110 | NAK for frame 1 |
| 3 | 1001101000 | NAK for frame 2 |
| 4 | 1011000100 | NAK for frame 3 |
| 5 | 0110001010 | MCS Change Confirm |
| 6 | 0000011101 | RCHG Request Indication |
| 7 | 1101010011 | ReservedSend Indication |
| 8 | 1100011000 | ReservedToken Request Indication |
| 9 | 0001010110 | Reserved |
| 10 | 0111000001 | Reserved |
| 11 | 1010001111 | Reserved |
| 12 | 1000100011 | Reserved |
| 13 | 0101101101 | Reserved |
| 14 | 0011111010 | Reserved |
| 15 | 1110110100 | Reserved |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE 802.16n-11/0033, “WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems; Enhancements to Support Higher Reliability Operations.”

# Title of Contribution

[Authors of Contribution]

[Affiliation of authors, per <http://standards.ieee.org/faqs/affiliationFAQ.html>]

# Template Instructions

This page provides instructions on using the template and submitting a contribution. Before submitting, delete these instructions or replace them with your text to be submitted.

# Document Format

Edit the cover page. Replace the square-bracketed sections with the correct text for your document. Eliminate the brackets. Leave the non-bracketed words on the cover page. Limit the cover page information to one page, shrinking the font if necessary.

Begin the main document with a page break after the cover page. Restate the document title and author name. You should use heading styles that support an outline view of the document.

# Header

The document number <http://ieee802.org/16/submit.html> goes in the upper right of the header. It takes the form “IEEE 802.16-YY-#####-RR-FFFF”, where:

• Every IEEE 802.16 document number begins with “IEEE 802.16-”.

• Following “IEEE 802.”, the remainder of the document number (“16-YY-#####-RR-FFFF”) is the Document Control Number (DCN), used by IEEE Mentor as the basis of the file name.

• “YY” represents the final two digits of the calendar year of original submission, as assigned by the IEEE Mentor documentation system.

• “####” represents the document serial number, as assigned by the IEEE Mentor documentation system. The serial number is uniquely assigned for each calendar year.

• “RR” represents the revision number, as assigned by the IEEE Mentor documentation system. The initial version of the document uses the revision number “00”.

• “FFFF” represents the Mentor “file code” <http://ieee802.org/16/submit.html>. It is selected by the submitter as the most appropriate identifier among the list of  IEEE 802.16 file codes in the IEEE Mentor documentation system.

# Fonts

Use only the fonts “Times” (or “Times New Roman”), “Arial” (or “Helvetica”),  and “Symbol,” since these fonts need not be embedded in PDF files. You may use other fonts only if necessary.

# Document Submission

To contribute a document to the IEEE 802.16 Working Group or a subgroup, create it using this template or another template at <http://ieee802.org/16/submit.html>. (If it cannot be put into one of the available formats, then create at least a cover page using an available template.) Submit the document following the instructions in the Call for Contributions or the IEEE 802.16 Document Submission Instructions <http://ieee802.org/16/submit.html>.