| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Some additional details of flexible feedback for HR-Network multicast |  |
| Date Submitted | 2012-01-17 |  |
| Source(s) | Eldad Zeira Anh Tuan Hoang InterDigital & I2R | E-mail: eldad.zeira@interdigital.com |
| Re: | Call for comments to 802.16n AWD in 802.16n-11/0032 |  |
| Abstract | The text proposal in this contribution provides some additional details for flexible feedback channel for multicast |  |
| Purpose | To be discussed and accepted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Some additional details of flexible feedback for HR-Network multicast

Eldad Zeira, Anh Tuan Hoang

# Summary

In Atlanta session #76 802.16n has approved the details of feedback for multicast for 802.16.1a based systems.

This contribution proposes similar modifications for 802.16n based systems.

# Text Proposal

16.9.2.2 Feedback for multicast information

To ensure robust multicast and provide the network operator with specific or statistical information of its reception a feedback operation is defined between an HR-MS that is an addressee of a multicast transmission and its serving HR-BS or HR-RS.

The conditions for providing feedback are defined by the network per each multicast channel and include positive feedback only (logical ACK), negative feedback only (logical NAK) or both (logical ACK/NAK). It is expected that all intended recipients of a multicast channel obey the same rules but those can be changed by the network. UL resources for the feedback are also provided by the HR-BS. Feedback parameters may be unicast or multicast.

Feedback operation is supported by multicast addressees in connected as well as in idle states. Code-only feedback may be used to provide feedback for multicast. The procedure for providing the feedback is TBD.

Code-only feedback may be used to provide feedback for multicast

To set up feedback operation, the HR-BS shall include the following parameters in or may be sent as part of the multicast setup DSA-REQ, DSA-RSP and DSC-REQ.

- Feedback request indicator FRI: indicates type of feedback, if any

- Logical channel indicator: index of the logical channel for feedback associated with the service

- Probability for sending feedback (NAK feedback only)

- RNG-ACK indicator: indicator if ACK for feedback is to be expected (ACK feedback only)

- Retransmissions and ramp-up of feedback (if RNG-ACK is used only)

If the above message is multicast then the HR-BS sends the HR-MS its logical channel assignment in a unicast message when setting up the multicast service.

The HR-BS may include the following in UCD: an appropriate broadcast or multicast message (TBD):

N_SIZE_LOG: Indicates the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them

For each distinct size of logical channel:

- p, Log base 2 of the number of physical channel per logical channel

- N_of_Size

N_frame: Delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it

Sub-channel offset for ranging: Counted from the beginning of FCZ assigned for UL

Ranging method: 0b0: Initial ranging over two symbols, 0b1: BR/periodic ranging over one symbol

Initial ranging code: Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS.

2 MSbs indicate index of starting code:

- 0b00: starting code index = 0

- 0b01: starting code index = 4

- 0b10: starting code index = 8

- 0b11: starting code index = 16

6LSbs indicate number of codes used.

Code spacing: The spacing between codes that are used for feedback

6.12.9.2.2.1 HR-MS and HR-BS behavior

The HR-MS shall:

If the HR-MS has been instructed to ACK (FRI=01b), then upon a successful reception of multicast transmission associated with the feedback channel the HR-MS shall send a ranging preamble as indicated in the setup parameters. If the HR-MS has been instructed to NAK (FRI=10b), and failed to receive multicast transmission when expecting one, it shall toss a coin with probability p to determine if further action is needed.

When transmitting ranging preamble, the HR-MS shall select at random a physical feedback channel out of all physical feedback channels associated with the logical feedback channel assigned to it. (Note that it is possible for a logical feedback channel to include a single physical channel).

Determine an initial transmit power as for 802.16.Rev3 baseline.

Transmit the ranging preamble using transmit power, code and resource block as indicated from the physical channel parameters.

If the RNG-ACK is expected, the HR-MS attempts to detect RNG-ACK as for 802.16.Rev3 baseline.

If RNG-ACK is indicated and not received, the HR-MS shall retransmit at the next opportunity, stepping up its power as indicated. If there is more than a single physical channel associated with the logical channel, then the HR-MS shall randomly select a different physical channel for retransmissions. The HR-MS should repeat the retransmissions until RNG-ACK is received or until maximum reached.

The HR-BS shall:

If the HR-BS has indicated RNG-ACK then the HR-BS shall respond with RNG-ACK.

>>>>>>>>>  Modify 6.3.2.3.10 DSA-REQ as follows

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.3.2.3.11 DSA-RSP as follows:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.3.2.3.13 DSC-REQ as follows:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.3.2.3.3 UCD as follows:

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| N_SIZE_LOG | TBD | Indicates the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them |
| For( i=0; i< N_SIZE_LOG; i++) { |  |  |
| p, Log base 2 of the number of physical channel per logical channel | 3 | The number is 2p |
| N_of_Size | TBD | Indicates number of logical channels that have size 2p |
| } |  |  |
| N_frame | TBD | Delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it |
| Sub-channel offset for ranging | 3 | Counted from the beginning of FCZ assigned for UL |
| Ranging method | 1 | 0b0: Initial ranging over two symbols, 0b1: BR/periodic ranging over one symbol |
| Subcarrier start | TBD | The starting sub-carrier for the ranging preamble |
| Initial ranging code | 8 | Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS. 2 MSbs indicate index of starting code: 0b00: starting code index = 0 0b01: starting code index = 4 0b10: starting code index = 8 0b11: starting code index = 16 6LSbs indicate number of codes used |
| Code spacing | 4 | The spacing between codes that are used for feedback |