| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Some additional details of flexible feedback for HR-Network multicast |  |
| Date Submitted | 2012-03-06 |  |
| Source(s) | Anh Tuan Hoang Eldad Zeira I2R & InterDigital | E-mail: athoang@i2r.a-star.edu.sg |
| Re: |  |  |
| Abstract |  |  |
| Purpose | To be discussed and accepted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Some additional details of flexible feedback for HR-Network multicast

Eldad Zeira, Anh Tuan Hoang

# Summary

In Jacksonville, text proposal under 16-12-0062-01-000n has been accepted. However, the editors have problem with incorporating the following accepted text

>>>>>>>>>  Modify 6.3.2.3.10 DSA-REQ as follows

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.3.2.3.11 DSA-RSP as follows:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.3.2.3.13 DSC-REQ as follows:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

EDITORS’ COMMENT: THE PROPOSED CHANGES TO DSA-REQ/DSA-RSP/DSC-REQ DO NOT FOLLOW THE TLV ENCODING APPROACH OF 16REV3

EDITORS PROPOSE TO EXPRESS THE ABOVE TWO SETs OF PARAMETERS IN DSA-REQ/DSA-RSP/DSC-REQ AS THE FOLLOWING TLV ENCODINGS:

# Text Proposal

### 11.13.a Feedback request indicator FRI

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].x | 1 | 2 LSBs represent: 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | DSA-REQ/DSA-RSP/DSC-REQ |

### 11.13.a+1 Logical channel indicator FBACK_LCI

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].(x+1) | 1 | Indicates the index of the logical channel assigned to this multicast | DSA-REQ/DSA-RSP/DSC-REQ |

### 11.13.a+2 Probability indicator of sending ranging preamble, pi

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].(x+2) | 2 | 10 LSBs indicate the probability of sending the NAK if NAK is indicated, probability = 2-pi | DSA-REQ/DSA-RSP/DSC-REQ |

### 11.13.a+3 RNG-ACK parameters

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].(x+3) | 2 | First 4 LSBs: Delay in frames to retransmission, =0 indicates no retransmissions Next 4 bits: Indicates number of retransmissions if no RNG-ACK Next 4 bits: Indicates power difference in dB for retransmissions Last 4 bits: reserved. | DSA-REQ/DSA-RSP/DSC-REQ |