| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HR-MS Paging Details |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Eldad Zeira Anh Tuan Hoang InterDigital & I2R | E-mail: eldad.zeira@interdigital.com |
| Re: | Call for comments to 802.16n AWD in 802.16n-11/0032 |  |
| Abstract | The text proposal in this contribution provides some additional details for HR-MS paging for 802.16n |  |
| Purpose | To be discussed and accepted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HR-MS Paging Details

Eldad Zeira, Anh Tuan Hoang

# Introduction

This contribution provides details of paging by HR-MS for 802.16Rev-3 based specifications.

# Proposed Text for the 802.16n Amendment Working Document (AWD) [2]

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[--------------------------------------------Start of Text Proposal--------------------------------------------]

16.3.2.1.7 Paging by HR-MS

The HR-BS may instruct an HR-MS or a group of HR-MS to broadcast a Paging

Indicator (PI) message or IE. The PI contains ID’s of paged HR-MSs or groups of HR-MSs. The resources for the PI are signaled in the S-NCI. PI format is TBD.

An HR-MS that receives its ID or an assigned group ID in the PI shall access the paging HR-MS as is done for HR-MS neighbor discovery.

The HR-BS may instruct an HR-MS or a group of HR-MS to broadcast a Paging Indicator (PI) IE. The PI contains ID’s of paged HR-MSs or groups of HR-MSs.

An HR-MS that receives its ID or an assigned group ID in the PI shall access the paging HR-MS as is done for HR-MS neighbor discovery 16.3.2.1.1, initial ranging mode, except that HR-MS shall use resources indicated in the PI IE.

16.12.3.2.1.7.1 MAC paging operation

To command a forwarding HR-MS or a group of forwarding HR-MS to page, the serving HR-BS sends to a forwarding HR-MS or group of forwarding HR-MS the contents of the PI IE and the resources for its transmission.

To facilitate the paging set up, the HR-BS may establish a group of pagers. To add or remove an HR-MS to/from a group of pagers, the HR-BS sends an MSPG-GRP message. The message is described in section {TBD by editor}.

To control the paging operation of the group, the HR-BS may multicast an MSPG-PG message which sets the paging ID’s and access parameters paged by the group. The message is described in section {TBD by editor}.

16.12.3.2.1.7.2 PHY paging operation

The paging HR-MS shall transmit the PIM in a Forwarding Control Zone (FCZ) as used for the NCI and described in 16.3.2.1.1.1. The location and length of the PIM is signaled in the NCI.

>>>>>>>>>>>> Modify the NCI message by adding the following fields:

(segment of the NCI)

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| PIM length | 4 | In OFDMA slots, 0b0000 indicates no PIM exists |
| Sub-channel offset for PIM | 5 | Indicates the gap in number of sub-channels between the end of F-MAP and the PIM |
| PIM repetition coding indicator | 2 | 0b00: No repetition coding on NCI 0b01: Repetition coding of 2 used on NCI 0b10: Repetition coding of 4 used on NCI 0b11: Repetition coding of 6 used on NCI |

The PI contains the ID of paged HR-MS or groups of HR-MS. How HR-MS or groups of HR-MS receive their paging ID is out of scope for this amendment. The contents and format of the PI IE is given in Table wxyz.

Table wxyz

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| N_PI | TBD | Indicates the number of distinct paging indicators |
| Sub-channel offset for ranging | 5 | Counted from the beginning of FCZ assigned for transmissions by forwarded HR-MS |
| Ranging method | 1 | 0b0: Initial ranging over two symbols 0b1: BR/periodic ranging over one symbol |
| For (i=0; i<N_PI; i++) { |  |  |
| PI PID | TBD | Paging ID |
| Initial ranging code | 5 | Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS. 2 MSBs indicate index of starting code: 0b00: starting code index = 0 0b01: starting code index = 4 0b10: starting code index = 8 0b11: starting code index = 16 3 LSBs indicate number of codes used. The codes are contiguous from the starting code. |
| } |  |  |

>>>>>>>>>>>> Add the following sections:

{section number to be determined by editor} MSPG-GRP

To add or remove an HR-MS to or from a pagers group the HR-BS shall unicast an MSPG-GRP described in Table {TBD}:

Table TBD – MSPG-GRP message field description

| Field | Size [bits] | Value / Description | Condition |
| --- | --- | --- | --- |
| Purpose | 1 | 0b – remove from group, 1b – add to group |  |
| Pager group ID | TBD |  |  |

{section number to be determined by editor} MSPG-PG

MSPG-PG multicast to a group of paging HR-MS lists of paged HR-MS ID and the access resources to be used by them. The format is defined in table TBD.

Table TBD:

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| Pager group ID | TBD |  | May be omitted if can be uniquely mapped multicast stream identifier, TBD |
| N_PI | TBD | Indicates the number of distinct paging indicators |  |
| Sub-channel offset for ranging | 5 | Counted from the beginning of FCZ assigned for transmissions by forwarded HR-MS |  |
| Ranging method | 1 | 0b0: Initial ranging over two symbols 0b1: BR/periodic ranging over one symbol |  |
| For (i=0; i<N_PI; i++) { |  |  |  |
| PI PID |  |  |  |
| Initial ranging code | 5 | CDMA codes used for initial ranging as in table {PIM} |  |
| } |  |  |  |