| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Some additional details of flexible feedback for HR-Network multicast |  |
| Date Submitted | 2012-01-17 |  |
| Source(s) | Eldad Zeira Anh Tuan Hoang InterDigital & I2R | E-mail: eldad.zeira@interdigital.com |
| Re: | Call for comments to 802.16.1a AWD in 802.16n-11/0033 |  |
| Abstract | The text proposal in this contribution provides some additional details for flexible feedback channel for multicast |  |
| Purpose | To be discussed and accepted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Some additional details of flexible feedback for HR-Network multicast

Eldad Zeira, Anh Tuan Hoang

# Summary

In Atlanta session #76, 802.16n has approved the details of feedback for multicast. The required information fields for the establishment and control of feedback operation have been agreed, although the exact message for their sending has not.

This contribution identifies control messages to which these parameters may be added.

# Text Proposal

------------------  Modify 6.12.9.2.2 as follows:

To ensure robust multicast and provide the network operator with specific or statistical

information of its reception a feedback operation is defined between an HR-MS that is an

addressee of a multicast transmission and its serving HR-BS or HR-RS.

The conditions for providing feedback are defined by the network per each multicast channel

and include positive feedback only (logical ACK), negative feedback 1 only (logical NAK) or

both (logical ACK/NAK). It is expected that all intended recipients of a multicast channel

obey the same rules but those can be changed by the network. UL resources for the feedback

are also provided by the HR-BS. Feedback parameters may be unicast or multicast.

Feedback operation is supported by multicast addressees in connected as well as in idle

states.

Code-only feedback may be used to provide feedback for multicast.

To set up feedback operation, the HR-BS shall include the following parameters in a unicast message when setting up a multicast service or may be sent as part of the multicast setup AAI-DSA-REQ, AAI-DSA-RSP and AAI-DSC. service itself. Further work is required to determine what the best way to send this information is.

- Feedback request indicator FRI: indicates type of feedback, if any

- Logical channel indicator: index of the logical channel for feedback associated with the service

- Probability for sending feedback (NAK feedback only)

- RNG-ACK indicator: indicator if ACK for feedback is to be expected (ACK feedback only)

- Retransmissions and ramp-up of feedback (if RNG-ACK is used only)

{ Table moved below and modified}

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | Sent only if E-MBS is supported |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast |  |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

If the above message is multicast then the HR-BS sends the HR-MS its logical channel assignment in a unicast message when setting up the multicast service. The message is TBD.

The HR-BS may include the following in an appropriate broadcast or multicast message (TBD): AAI-SCD:

N_SIZE_LOG: Indicates the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them

For each distinct size of logical channel:

- p, Log base 2 of the number of physical channel per logical channel

- N_of_Size

N_frame: Delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it

K_subframe: subframe indicator

Feedback ranging format: indicates the type of ranging (synchronized or non)

{ Table moved below}

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| N_SIZE_LOG | TBD | Indicates the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them |  |
| For( i=0; i< N_SIZE_LOG; i++) { |  |  |  |
| p, Log base 2 of the number of physical channel per logical channel | 3 | The number is 2p |  |
| N_of_Size | TBD | Indicates number of logical channels that have size 2p |  |
| } |  |  |  |
| N_frame | TBD | Delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it |  |
| K_subframe | 3 | Subframe indicator |  |
| Feedback_ranging_format | 2 | 00b: S-RCH, 01b: NS-RCH format 0, 10b: NS-RCH format 1, 11b: reserved |  |
| Subcarrier start | TBD | The starting sub-carrier for the ranging preamble |  |
| Starting code index | TBD | The first code used |  |
| Code spacing | 4 | The spacing between codes that are used for feedback |  |
| Total number of codes | TBD | The total number of channels used (note that the number of codes may span multiple subbands and sub-frames) which implicitly defines the number of subbands |  |

>>>>>>>>>  Modify 6.2.3.9 AAI-REG-RSP as follows - Add the following fields:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved |  |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast |  |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.2.3.47.1 AAI-DSA-REQ as follows

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.2.3.47.2 AAI-DSA-RSP as follows:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.2.3.47.4 AAI-DSC-REQ as follows:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |

>>>>>>>>>  Modify 6.2.3.31 AAI-SCD as follows:

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| DeltaXL_X | 4 | DeltaXL_X is the fairness and interference control factor of the MS-MS link to MSMS links. It is broadcast by the HR_BS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3). It may be transmitted only for those frequency partitions where HR-MS to HRMS transmissions are scheduled | Sent if the HR-MS has an associated forwarded HR-MS |
| N_SIZE_LOG | TBD | Indicates the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them | Included if multicast feedback is supported |
| For( i=0; i< N_SIZE_LOG; i++) { |  |  |  |
| p, Log base 2 of the number of physical channel per logical channel | 3 | The number is 2p |  |
| N_of_Size | TBD | Indicates number of logical channels that have size 2p |  |
| } |  |  |  |
| N_frame | TBD | Delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it |  |
| K_subframe | 3 | Subframe indicator |  |
| Feedback_ranging_format | 2 | 00b: S-RCH, 01b: NS-RCH format 0, 10b: NS-RCH format 1, 11b: reserved |  |
| Subcarrier start | TBD | The starting sub-carrier for the ranging preamble |  |
| Starting code index | TBD | The first code used |  |
| Code spacing | 4 | The spacing between codes that are used for feedback |  |
| Total number of codes | TBD | The total number of channels used (note that the number of codes may span multiple subbands and sub-frames) which implicitly defines the number of subbands |  |