| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power control for HR-MS – Homogenized text |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Eldad Zeira Anh Tuan Hoang InterDigital & I2R | E-mail: eldad.zeira@interdigital.com |
| Re: | Call for comments to 802.16n AWD in 802.16n-11/0032 |  |
| Abstract | The text proposal in this contribution provides some additional details for HR-MS power control |  |
| Purpose | To be discussed and accepted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power control for HR-MS – Homogenized text

Eldad Zeira, Anh Tuan Hoang

# Introduction

In Atlanta 802.16n has agreed on power control details for 802.16.1 based amendment for case-2 HR-MS transmission. In a separate contribution for this session, the text is expanded in a homogenous manner to apply to both cases 1 and 2.

This contribution presents a very similar power control concept for 802.16Rev-3 based systems.

# Proposed Text

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[--------------------------------------------Start of Text Proposal--------------------------------------------]

----------  Modify text in 6.12.2.2.2. as follows:

16.2.2.2.2 Power control for mobile to mobile communication

16.2.2.2.2.1 Power control for two HR-MS associated with an HR-BS

When two HR-MS that are associated with an HR-BS are transmitting to each other their power control related commands are generated by their serving HR-BS.

The HR-BS may define measurements to be performed by the HR-MS on resources used for MS-MS communications and on the desired MS-MS signal to be reported to the HR-BS.

Definition of power control procedure is TBD.

16.2.2.2.2.2 Power control for at least one HR-MS associated with an HR-BS

The transmission power of a forwarding HR-MS transmitting to forwarded HR-MS is controlled by messages from the forwarded HR-MS that are derived from HR-BS controls.

The transmission power of a forwarded HR-MS is controlled by messages from the forwarding HR-MS that are derived from HR-BS controls.

Power control procedure details TBD.

The transmission power of a forwarding HR-MS transmitting data or control channels to another HR-MS is controlled by messages from the receiving HR-MS that are derived from HR-BS controls.

The HR-BS signals power control parameters to all HR-MS with active links. HR-MS may be instructed by the HR-BS to estimate path loss between HR-MSs. The receiver of data generate offset controls that are based on constraints or parameters, signaled from HR-BS.

Cross link interference is handled by augmenting the PC to include SIR measured on crosslink resources. The same procedure is applied for BS-controlled FTN and BS-controlled direct communication.

Power control parameters that are generated by the HR-BS are sent directly to both HR-MS provided both have a link to the HR-BS. When one of the HR-MS does not have a link to the HR-BS, then its power control parameters are signaled to the one that does. The associated HR-MS signals them to the forwarded HR-MS.

The operation of power control requires that measurements be performed by the forwarding and forwarded HR-MS. The measurements and their associated signaling are TBD.

6.12.2.2.2.2.2.1 Closed loop power control

Power control is performed as in 8.4.10.3 except as detailed in this section.

Equation 131 (repeated here) is used:

Pnew = Plast + (C/Nnew – C/Nlast) – (10log10(Rnew) – 10log10(Rlast)) + Offset					wxy

Where

Pnew = the power of the new UL burst in the current UL frame

C/Nnew = normalized C/N for the new UL burst in the current UL frame

Rnew = repetition factor R for the new UL burst in the current UL frame

Plast = the power of the burst with the maximum value of (C/N .– 10log10(R)) in the most recently

transmitted UL frame

C/Nlast = normalized C/N associated with Plast (thus referring to the burst with the maximum value of

(C/N .– 10log10(R)) in the most recently transmitted UL frame)

Rlast = repetition factor R associated with Plast (thus referring to the burst with the maximum value

of (C/N .– 10log10(R)) in the most recently transmitted UL frame)

Offset = an accumulation of power correction terms sent by the BS since the last transmission

Power control of channels transmitted to the HR-BS

This case is applicable to any HR-MS in DC or FTN states if it transmits any channels to the HR-BS. Operation is as per OFDMA.

{not clear we need the above section}

Power control of channels transmitted between HR-MS

Offset is derived from correction terms sent by the receiving HR-MS based on OffsetMIN_FWD_D and OffsetMAX_FWD_D constraints signaled by the HR-BS such that OffsetMIN_FWD_D ≤ Offset ≤ OffsetMAX_FWD_D.

A receiving HR-BS that is directly associated with the HR-BS receives OffsetMIN_FWD_C and OffsetMAX_FWD_C from it. A receiving HR-BS that is not directly associated with the HR-BS receives it from the one that is which in turn receives it from the HR-BS.

Signaling of offset constraints from the HR-BS is done per 6.3.2.3.3 in UCD (UL channel descriptor) message

Signaling of the Offset constraints between the HR-MS is done through HR-DC-H2H with reason 0011b.

Signaling of Offset between the HR-MS is done through HR-DC-H2H with reason 0001b.

Message HR-DC-H2H

| Field | Size [bits] | Value / description | Conditions |
| --- | --- | --- | --- |
| Reason code | 4 (?) | 0000b – null, used as keep alive 0001b – power control parameters signaled by the receiver of data and control channels 0010b – reserved 0011b – messages to or from HR-BS that are forwarded between HR-MS |  |
| If (Reason code == 0001b) { |  |  |  |
| Offset | 8 | Offset value, defined as in FPC message format |  |
| } |  |  |  |
| If (Reason code == 0010b) { |  |  |  |
| } |  |  |  |
| If (Reason code == 0011b) { |  |  |  |
| Number of octets in message | TBD | Indicates the number of octets in the message |  |
| Embedded message | Var. | The message to or from the HR-BS in its entirety |  |
| Concatenation bit | 1 | 01b Indicates that an additional message is embedded |  |
| If (concatenation bit == 1b) { |  |  |  |
| Read message |  |  |  |
| } |  |  |  |
| } |  |  |  |