| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification on AAI-RNG-REQ message for higher reliability functions in 16.1a Network |  |
| Date Submitted | 2012-01-17 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Seokki Kim, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16.1a Draft AWD, IEEE C802.16n-11/0033 |  |
| Abstract | This provides AWD text proposals of modification on AAI-RNG-REQ message for higher reliability functionalities in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

- Modification on AAI-RNG-REQ message for higher reliability functions in 16.1a Network

Anseok Lee, Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun,

Hyun Lee, Seokki Kim, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

This contribution provides Modification on AAI-RNG-REQ message to support functionalities for higher reliability in 16.1a network.

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Modify the following updates in Table 27 of Section 6.2.3.1 in the 802.16.1a AWD]

Table 27 – AAI-RNG-REQ message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Interference mitigation request to a CSG Femto ABS when experiencing interference from the CSG Femto ABS 0b1101 = NS/EP call setup 0b1110 = HR multicast service location update 0b1111 = Network reentry for FBIS operation 0b1110 = Ranging purpose for HR-Network 0b1111 = reserved |  |
| … |  |  |  |
| }else if (Ranging Purpose Indication == 0b1110) { |  | // HR multicast location update |  |
| }else if(Ranging Purpose Indication == 0b1110) { |  | // Ranging purpose for HR-Network |  |
| Extended Ranging Purpose Indication | 4 | 0b0000 = HR multicast service location update 0b0001 = Network reentry for FBIS operation 0b0010-0b1111 = reserved |  |
| If( Extended Ranging Purpose Indication == 0b0000) { |  |  |  |
| action code | 3 | bit0: multicast service flow update bit1: location update due to multicast zone change bit2: multicast security key update |  |
| } else if (Ranging Purpose Indication == 0b1111) { |  | // Network reentry for FBIS operation |  |
| } else if (Extended Ranging Purpose Indication == 0b0001) { |  | // Network reentry for FBIS operation |  |
| … |  |  |  |
| } //end of Extended Ranging Purpose Indication |  |  |  |
| } //end of Ranging Purpose Indication |  |  |  |
| … |  |  |  |

## [Remedy2: Modify the following updates in Table 28 of Section 6.2.3.2 in the 802.16.1a AWD]

Table 28 – AAI-RNG-RSP message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... |  |  |  |
| New Multicast Group Zone ID | 12 | Indicates a Multicast Group Zone ID to update in target HR-BS. | Shall be included in HR-Network in response to the AAI-RNG-REQ message where Ranging Purpose Indication is set to 0b1110 and Extended Ranging Purpose Indication is set to 0b0000 and action code bit0 is set to 1. |
| … |  |  |  |
| If (it is under network reentry for HO){ |  |  |  |
| New Multicast Group Zone ID | 12 | Indicates a Multicast Group Zone ID to update in target HR-BS. | Shall be included in HR-Network in response to the AAI-RNG-REQ message where Ranging Purpose Indication is set to 0b1110 and Extended Ranging Purpose Indication is set to 0b0000 and action code bit0 is set to 1. |

## [Remedy3: Modify the following updates in Table 763mc1 of Section 6.2.3.65.65 in the 802.16.1a AWD]

Table 763mc1 – AAI-HR-MG-IND message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... |  |  |  |
| Action Code | 3 | if bit0 = 1, perform network entry or exit sleep mode if bit1 = 1, perform ranging procedure with ranging purpose indication set to 00b1110 and Extended Ranging Purpose Indication set to 0b0000 if bit2 = 1, receiving multicast | Shall be present |
| … |  |  |  |

## [Remedy4: Modify the following updates in Table 763mc2 of Section 6.2.3.65.66 in the 802.16.1a AWD]

Table 763mc2 – AAI-HR-MT-IND message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... |  |  |  |
| Action Code | 3 | if bit0 = 1, perform network entry or exit sleep mode if bit1 = 1, perform ranging procedure with ranging purpose indication set to 00b1110 and Extended Ranging Purpose Indication set to 0b0000 if bit2 = 1, receiving multicast | Shall be present |
| … |  |  |  |

## [Remedy5: Modify the following text of Section 6.12.9.1 in the 802.16.1a AWD]

- 6.12.9.1.2 Multicast communication operation in connected state

When an HR-MS moves across Multicast zone boundaries in Active Mode or Sleep Mode, the HR-MS performs the handover procedure as described in 6.2.6.3.

When the HR-MS transits to a new Multicast Zone while in Active Mode or Sleep Mode, the HR-MS shall send AAI-RNG-REQ message described in 6.2.3.1 with Ranging Purpose Indication = 0b1110 and Extended Ranging Purpose Indication = 0b0000 with action code at the target HR-BS. In response to the request for multicast service flow update (Ranging Purpose Indication = 0b1110 and Extended Ranging Purpose Indication = 0b0000 and action code bit0 is set to 1), the HR-BS shall transmit AAI-RNG-RSP message described in 6.2.3.2, which may include Multicast Group Zone Identifier, Multicast Group ID, FID Update, and feedback parameters if used, to provide updated service flow management encodings for any affected multicast flow(s) as part of the handover procedure.

- 6.12.9.1.3 Multicast communication operation in idle state

When an HR-MS in Idle state moves to an HR-BS which does not belong to HR-MS' previous Multicast Group Zone, the HR-MS is expected to update the multicast service flow management encodings at that HR-BS to provide continuous reception of multicast content. The HR-MS may obtain the multicast information in the target Multicast zone through broadcast messages in the Multicast Zone of the service HR-BS. If the idle HR-MS has not received such information from the serving Multicast Zone, the HR-MS shall use location update procedure to acquire updated multicast service flow management encodings.

In order to perform the multicast location update process, the HR-MS shall transmit AAI-RNG-REQ message with Ranging Purpose Indication = 0b1110 and Extended Ranging Purpose Indication = 0b0000 with action code. When the HR-MS detects the current multicast group zone changes and expects to update service flow, the bit0 of action code is set to 1. In addition to changing the multicast group zone, the HR-MS detects current paging zone changes, the bit1 of action code is set to 1. In the case of performing multicast security key update, the bit2 of the action code is set to 1.

…

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE C802.16n-11/0186r3, “Forwarding Between Infrastructure Stations”

[2] IEEE C802.16n-11/0234r1, “Clarifications on FBIS in 16.1a Network”

[3] IEEE C802.16n-11/0235r2, “MAC Control Messages to support FBIS in 16.1a Network”

[4] IEEE 802.16n-11/0033, “WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems; Enhancements to Support Higher Reliability Operations.”