| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Activating the alternative path in Path Discovery and Management within HR-BS |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | E-mail: hvdien@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for contributions for the 802.16n Amendment Working Draft |  |
| Abstract | This provides a proposal of establishment and maintenance of alternative path in 802.16n networks (Original document in Session #72: C80216n-11_0026.doc) |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Alternative path in path management of 802.16.1-GRIDMAN networks

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

# In this proposal, we propose a detail operation of how the alternative path to a forwarding HR-MS is activated and maintain.

# Text Proposal in AWD

The text in BLACK color: the existing text in the Amendment Draft Standard

The text in RED color: the removal of existing Amendment Draft Standard Text

The text in BLUE color: the new text added to the Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the following text into session 6.12.7.1 of 802.16.1 AWD]

- 6.2.3.65.63 AP-NBR-REQ

Table 763pm1 -- AP-NBR-REQ message field and description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Discovery mode Mode requested | 2 | 0b00: Normal neighbor discovery 0b01: Discovery to prepare for BS determine alternative path 0b10: Discovery to prepare for MS determine alternative path 0b11: reserve 0b11: Inform HR-BS of the forwarding HR-MS | In the case of 0b10, HR-BS shall reply HR-MS in the message AP-NBR-REP. |
| Index of the forwarding HR-MS | 4 | The index of HR-MS chosen as the forwarding HR-MS as in the AP-NBR-REP message when Mode is 0b11 |  |
| } |  |  |  |

6.12.7.1.2 Alternative Path Preparing preparation by HR-MS for fast network reentry

HR-MS determines the alternative path by transmitting AP-NBR-REQ message with mode 0b11 to its serving HR-BS to request for neighbor discovery. HR-BS performs the neighbor discovery as described in 6.12.2.2.1.1 and transmit the HR-MS’s neighbour information to HR-MS in the AP-NBR-REP message. The HR-MS determines the forwarding HR-MS as the alternative path by itself based on its received neighbor information in the AP-NBR-REP message, but how to determine is out of scope of this specification.

The HR-MS informs HR-BS of its forwarding HR-MS chosen by using AP-NBR-REQ message with mode 0b11

6.12.7.1.4 Alternative Path information and Switching to forwarding HR-MS

Within HR-BS, an alternative path of a HR-MS is the target forwarding HR-MS determined in 6.12.7.1.1 or 6.12.7.1.2.

When HR-MS is disconnected with HR-BS in its current path, HR-MS shall try to do the network reentry through the target forwarding HR-MS.

When HR-BS realizes that the HR-MS is disconnected, HR-BS will arrange for the target forwarding HR-MS to perform the Coverage Extension action as in session 6.12.3.2.1.1.2

#### 6.12.7.1.5 6.12.7.1.4 Alternative path maintenance

An alternative path to a target HR-infrastructure station may be updated or changed with a new AAI-HO-CMD message. When a mode-change or mode-release occurs as described in 6.12.1, new alternative path may be selected.

An alternative path to a target forwarding HR-MS may be updated or changed by either HR-MS or HR-BS according to network conditions such as the forwarding HR-MS is moving out of reach etc.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.