| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures for Establishing Secured Communications for FBIS in IEEE 802.16.1a |  |
| Date Submitted | 2012-01-0916 |  |
| Source(s) | Joseph Teo Chee Ming, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | in response to the Contribution for Security Procedures in IEEE 802.16n |  |
| Abstract | Detail description of Security procedures for Direct Communications to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Procedures for Establishing Secured Communications for FBIS in IEEE 802.16.1a

Joseph Teo Chee Ming, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang,

Institute for Infocomm Research

# Introduction

This contribution contains the detailed proposed text from Security Procedures for establishing secured communications for FBIS – forwarding between infrastructure stations. FBIS is a mode of operation when a HR-MS forwards data from a degraded infrastructure station which lost its backhaul to another infrastructure station with a backhaul.

It is to be proposed to be placed into the AWD text for the IEEE 802.16.1a GridMAN standards.

# General Description

The 802.16n SRD introduces a higher-reliability requirement where a HR-MS forwards data for a degraded infrastructure station (i.e., lost its backhaul) to another infrastructure station which has a working backhaul connection. This poses a substantial security risk because (i) a HR-station is deliberately introduced as man-in-the-middle, and (ii) the HR-station is able to access all data communicated between the infrastructure stations.

One straightforward way to implement the forwarding functionality is to use the AAI-L2-XFER message as an encapsulation and dedicate the security to be handled at the higher layer. However, this does not address the first issue stated above. Moreover, the AAI-L2-XFER message itself poses a security risk to the terminal, as described in the IEEE 802.16m standard:

AAI-L2-XFER messages are very likely to bypass any firewalls or other security protection that may be implemented on terminal devices incorporating the AMS, firewalls that mostly, if not exclusively, monitor protocols on L3and L4 and above layers. For this reason, use of the AAI-L2-XFER message poses a substantial security and access control risk to the terminal a) unless the AAI-L2-XFER message is strictly used for very-specific, identified purposes (SMS, etc.), by the designated applications only, and b) unless the vendor implementation takes specific precaution to mitigate and minimize these risks. Vendor implementation and mitigation techniques are outside the scope of this specification. Any other use of the AAI-L2-XFER message may have unintended security and access control risks not contemplated in this specification.

In this contribution, we propose to adapt the security procedure used for direct communications in IEEE 802.16.1a for FBIS operation since the scenario in security for two communicating HR-infrastructure stations is similar to that in direct communications in two hops.

# Proposed text

It should be noted that:

- Proposed texts are placed in the Section they address.  If the text in the contribution addresses more than one section, then the text is split and placed under the appropriate Section numbers (or categories).

- All proposed texts from different contributions that address a particular section are together.

- Existing Section numbers are in black color with a bold face.

- Existing text are colored black

- All proposed sections are colored blue with an underline and bold face.

- All proposed text are initially colored blue with an underline.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Modify the following text into Section 17.3.10.2 Security in the 802.16n AWD accordingly]

6.12.10 Support for Security

### 6.12.10.3 Secure forwarding between HR-infrastructure stations using Public Key Infrastructure

This procedure provides strong protection and data integrity when data is forwarded between HR-infrastructure stations as described in Section 6.12.7.3 using Public Key Infrastructure. Each HR-BS has a public/private key pair and digital certificate (e.g. X.509) issued by a certification authority for mutual authentication and key exchange prior to the start of this direct communications.

The key agreement handshake procedure described below shall be used for HR-BSs to mutually authenticate themselves (without access to a security server) using Public Key Infrastructure and to derive data security keys for secure direct communications. A control connection between HR-BSs via a designated forwarding HR-MS shall be used for transmitting the key agreement messages. The flow diagram for this scenario is depicted in Figure XX and the Flow Chart for this scenario is shown in Figure XX+1.

### 6.12.10.3.1 Key Agreement Handshake Procedure

The key agreement handshake procedure using Public Key Infrastructure includes the following steps:

Step 1: Prior to secure connection establishment as described in Section 6.12.7.3.4, the degraded HR-BS (HR-BSd) shall generate nonce NHR-BSd. Next, HR-BSd computes the signature signature HR-BSd = SIGN(T HR-BSd|N HR-BSd|HR-BS MAC Addr|HR-BSd MAC Addr) and sends BStoBS_KeyAgreement_MSG_#1 message to Designated HR-BS, where DirectComms_KeyAgreement_MSG_#1 = T HR-BSd|NHR-BSd|HR-BS MAC Addr|HR-BSd MAC Addr| signature HR-BSd|Cert(HR-BSd).

Step 2: HR-BS first verifies the received timestamp and nonce for freshness and the certificate Cert(HR-BSd) and signature signature HR-BSd. If the verifications fail, then HR-BS ignores the BStoBS_KeyAgreement_MSG_#1 message. If the verifications are correct, then HR-BS generates nonce NHR-BS and security key DMK and computes DAK =Dot16KDF (DMK, HR-BSd MAC Addr|HR-BS MAC Addr| “B2AK”, 160) and the DCMAC key and CMACHR-BS = MACDCMAC(N HR-BS|NHR-BSd|HR-BS MAC Addr|HR-BSd MAC Addr). HR-BS then uses HR-BSd's public key to encrypt and obtain EHR-BSd_PK(B2MK, key_lifetime, HR-BSd MAC Addr, HR-BS MAC Addr). Finally, HR-BS computes signature signature HR-BS = SIGN(THR-BS|NHR-BS|HR-BSd MAC Addr|HR-BS MAC Addr|NHR-BSd|EHR-BSd_PK(B2MK, key_lifetime, HR-BSd MAC Addr, HR-BS MAC Addr)|CMACHR-BS) and sends BStoBS_KeyAgreement_MSG_#2 message to HR-BSd, where BStoBS_KeyAgreement_MSG_#2  = THR-BS|NHR-BS|HR-BSd MAC Addr|HR-BS MAC Addr|NHR-BSd|EHR-BSd_PK(B2MK, key_lifetime, HR-BSd MAC Addr, HR-BS MAC Addr)|CMACHR-BS| signature HR-BS|Cert({HR-BS).

Step 3: HR-BSd first verifies the received timestamp and nonces for freshness and the certificate Cert(HR-BS) and signature signature HR-BS. If the verification is invalid, then HR-BSd ignores the BStoBS_KeyAgreement_MSG_#2 message. If the verifications are correct, then HR-BSd decrypts EHR-BSd_PK(B2MK, key_lifetime, HR-BSd MAC Addr, HR-BS MAC Addr) and obtains security key B2MK and key_lifetime. Next, HR-BSd computes B2AK and B2CMAC keys and verifies CMACHR-BS. If the verification is invalid, then HR-BSd ignores the BStoBS_KeyAgreement_MSG_#2 message. If the verification is correct, then HR-BSd computes CMACHR-BSd = MACDCMAC(N HR-BSd|N HR-BS|HR-BSd MAC Addr|HR-BS MAC Addr) and sends BStoBS_KeyAgreement_MSG_#3 message to HR-BS, where BStoBS_KeyAgreement_MSG_#3 = NHR-BS|HR-BS MAC Addr|HR-BSd MAC Addr|CMACHR-BSd. If HR-BSd does not receive BStoBS_KeyAgreement_MSG_#2 message from HR-BS within BStoBS_KeyAgreement_MSG_#1 Timeout, it shall resend the BStoBS_KeyAgreement_MSG_#1 message up to BStoBS_KeyAgreement_MSG_#1 MaxResends times. If HR-BSd reaches its maximum number of resends, it shall initiate another authentication or drop the request.

Step 4: HR-BS receives the BStoBS_KeyAgreement_MSG_#3 message and verifies received nonce and the CMAC tuple. If the verification fails, then HR-BS ignores BStoBS_KeyAgreement_MSG_#3 message. If the verification is correct, then HR-BS confirms that HR-BSd has computed the correct keys and commence secure direct communications. If HR-BS does not receive BStoBS_KeyAgreement_MSG_#3 message from HR-BSd within BStoBS_KeyAgreement_MSG_#2 Timeout, it shall resend the BStoBS_KeyAgreement_MSG_#2 message up to BStoBS_KeyAgreement_MSG_#2 MaxResends times. If HR-BS reaches its maximum number of resends, it shall initiate another authentication or drop the request. HR-BSd and HR-BS can now derive B2TEK and commence secure connection establishment as described in Section 6.12.7.3.4.

Figure XX—Flow Diagram of PKI-based Authentication and Key Establishment of Forwarding Communication Between HR-Infrastructure Stations without Infrastructure.

HR-BSd sends a DirectComms_KeyAgreement_MSG_#1 message to HR-BS to request for a direct communication authentication and key establishment. The message contains a nonce, signature generated by HR-BSd, HR-BSd and HR-BS addresses and HR-BSd’s digital certificate.

HR-BS verifies the received certificate and signature. If the verification is correct, HR-BS generates its nonce and B2MK. Next, HR-BS computes B2AK, B2CMAC key and the CMAC tuple of the message. HR-BS then uses the public key of HR-BSd to encrypt the DMK. Finally, HR-BS computes the signature and sends DirectComms_KeyAgreement_MSG_#2 message to HR-BSd. The message also contains HR-BS’s digital certificate.

HR-BSd verifies the received certificate and signature. If the verification is correct, then HR-BSd decrypts the received encrypted message and obtains DMK. Next, HR-BSd computes B2AK and B2CMAC key and verifies the received CMAC tuple. If the verification is correct, then HR-BSd computes its CMAC tuple and sends BStoBS_KeyAgreement_MSG_#3 message to HR-BS.

HR-BS receives the above message and verifies the CMAC tuple. If the verification is correct, then HR-BS confirms that HR-BSd has computed the correct keys and both HR-BSd and HR-BS derives B2TEK and commence secure direct communications.

Figure XX+1—Flow Chart of PKI-based Authentication and Key Establishment of Forwarding Communication Between HR-Infrastructure Stations without Infrastructure..

# YY.YY.YY.YY Message Type

Table YY—PKMv3 message attribute

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| AA | BStoBS_KeyAgreement_MSG #1 | AAI-PKM-RSP |
| AA+1 | BStoBS_KeyAgreement_MSG #2 | AAI-PKM-REQ |
| AA | BStoBS_KeyAgreement_MSG #3 | AAI-PKM-RSP |

# 6.12.10.3.2 Message Attribute

Table ZZ—BStoBS_KeyAgreement_MSG_#1 message attribute

| Attribute | Contents |
| --- | --- |
| T HR-BSd | Timestamp generated by HR-BSd |
| NHR-BSd | Freshly generated random number of 64bits by HR-BSd |
| HR-BS MAC Addr | MAC Address of HR-BS |
| HR-BSd MAC Addr | MAC Address of HR-BSd |
| signatureHR-BSd | Signature of message generated by HR-BSd using its RSA private key |
| Cert(HR-BSd) | Digital certificate of HR-BSd |

Table ZZ+1—BStoBS_KeyAgreement_MSG_#2 message attribute

| Attribute | Contents |
| --- | --- |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Freshly generated random number of 64bits by HR-BS |
| HR-BSd MAC Addr | MAC Address of HR-BSd |
| HR-BS MAC Addr | MAC Address of HR-BS |
| NHR-BSd | Nonce generated by HR-BSd in BStoBS_KeyAgreement_MSG_#1 message |
| EHR-BSd_PK(B2MK, key_lifetime, HR-BSd MAC Addr, HR-BS MAC Addr) | Public key encryption using HR-BSd's Public key where B2MK = BStoBS Master Key generated by HR-MS and key_lifetime = lifetime of B2MK |
| CMACHR-BS | Message digest calculated using B2CMAC key by HR-BS |
| signatureHR-BS | Signature of message generated by HR-BS using its RSA private key |
| Cert(HR-BS). | Digital certificate of HR-BS |

Table ZZ+2—BStoBS_KeyAgreement_MSG_#3 message attribute

| Attribute | Contents |
| --- | --- |
| NHR-BS | Nonce generated by HR-BS in BStoBS_KeyAgreement_MSG_#2 message |
| HR-BS MAC Addr | MAC Address of HR-BS |
| HR-BSd MAC Addr | MAC Address of HR-BSd |
| CMACHR-BSd | Message digest calculated using B2CMAC key by HR-BSd |

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

# 17.2.10.1.1.3 Security Context for BS-coordinate Secure Direct Communication

The direct communications security context describes the set of parameters that links the direct communication security keys for BS-coordinate secure direct communications.

### 17.2.10.1.1.3.1 DMK context

The DMK context includes all parameters associate with the DMK. This context is created when the DMK is derived.

The DMK context is described in Table 1.

Table 1—The DMK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DMK | 160 | Multicast Master Key shared by HR-BS and HR-MSs in a multicast group |
| DMK SN | 4 | DMK sequence number |
| DMK Lifetime | 32 | MMK Lifetime |
| DAK_COUNT | 16 | Counter to ensure freshness of computed CMAC key and prevent replay attacks. |

### 17.2.10.1.1.3.2 DAK context

The DAK context includes all parameters associate with the DAK. This context is created whenever a new DAK is derived. This context shall be deleted when the DAK is not in used.

The DAK context is described in Table 2.

Table 2—The DAK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DAK | 160 | Direct Communications Authentication Key derived from DMK. |
| DAK Lifetime | 32 | DAK Lifetime |
| DAKID | 64 | Identifies the DAK key. |
| DCMAC_KEY | 128 | Key which is used for signing Direct Communications MAC control messages. |
| DCMAC_PN | 24 | Used to avoid multicast replay attack on the control connection. The initial value of DCMAC_PN is zero. |
| DAK_COUNT | 16 | Counter to ensure freshness of computed CMAC key and prevent replay attacks. |

### 17.2.10.1.1.3.3 DSA context

The DSA context is the set of parameters managed by each DSA in order to ensure DTEK management and usage in a secure way for BS-coordinated secure direct communications.

The DSA holds the DTEK context and additional information that belongs to the DSA itself.

### 17.2.10.1.1.3.4 DTEK context

The DTEK context includes all parameters of the DTEK and is described in Table 3.

Table 3—The DTEK context

| Parameter | Size (bit) |  | Usage |
| --- | --- | --- | --- |
| DTEK | 128 |  | Key used for encryption or decryption of direct communications messages |
| DMK SN |  | 4 | DMK sequence number |
| COUNTER_DTEK | 16 |  | The counter used to derive this DTEK |
| DTEK lifetime | 32 |  | DTEK lifetime=DMK lifetime |
| DTEK_PN | 22 |  | The PN used for encrypting multicast packets. After each Multicast MAC PDU transmission, the value shall be increased by 1. (0x000000-0x1FFFFF) |

### 17.2.10.1.1.3.5 DSA context

The DSA context is described in Table 4.

Table 4—The DSA context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DSAID | 8 | The identifier of this DSA, which decribes the applied encryption/decryption method and DTEK contexts. |
| DTEK context | Sizeof(DTEK context) | DTEK context for encryption and decryption |

# 17.2.10.1.1.4 Key Derivation for BS-coordinated Secure Direct Communication

The key hierarchy defines what keys are present in the system for BS-coordinated secure direct communication and how the keys are generated.

17.2.10.1.1.4.1 DMK Derivation

The DMK is the security key/pre-established shared key that is randomly generated by HR-BS or HR-MS or a network entity (e.g. an AAA Server etc). The DMK is a 160-bit key.

The DMK may be used as a source for keying materials required by upper layers.

The DMK is used to derive the Direct Communication Authentication Key (DAK).

17.2.10.1.1.4.2 DAK Derivation

DAK is derived from DMK and belongs to a pair of HR-MSs. The DAK is used for BS-coordinated Direct Communications in the event of failure in the backbone.

The DAK derivation is as follows:

DAK =Dot16KDF (DMK, HR-BSd MAC Addr|HR-BS MAC Addr|”DAK”, 160)

where: HR-BSd MAC Addr and HR-BS MAC Addr are the addresses of HR-BSd and HR-BS respectively.

The DCMAC-DTEK prekey is derived from DAK and is used to derive other keys:

- Direct Communication Cipher-based Message Authentication Code (DCMAC) key

- Direct Communication Traffic Encryption (DTEK) Key

The DCMAC-DTEK prekey derivation is done as follows:

DCMAC-DTEK prekey = Dot16KDF (DAK, DAK_COUNT|”DCMAC-DTEK prekey”, 160)

17.2.10.1.1.4.3 DCMAC Key Derivation

DCMAC key is derived from DAK and used for message authentication for the messages sent during BS-coordinated secure direct communications.

DCMAC key is derived as follows:

DCMAC key = Dot16KDF(DCMAC-DTEK prekey, “DCMAC_KEYS”, 128)

17.2.10.1.1.4.4 DTEK Derivation

DTEK is the transport encryption key used to encrypt data in BS-coordinated secure direct communications.

DTEK is derived as follows:

DTEK = Dot16KDF(DCMAC-DTEK prekey, DSAID|COUNTER_DTEK|”DTEK_KEY”, 128)

Where

SAID is the security association to which the TEK belongs.

COUNTER_DTEK is a counter used to derive different TEKs for the same SAID, the value of the counter is changed everytime a new DTEK needs to be derived within the same AK and AK_COUNT pair is valid. Everytime a new DCMAC-DTEK prekey is derived, this counter is reset.

17.2.10.1.2 Security Procedure for Secure talk-around Direct Communication using dedicated resource

In order to support secure direct communication between two or among more HR-MSs, pre-established shared key is used.

PKMv3 provides HR-MSs with strong protection from theft of service by encrypting talk-around direct connections between two or among HR-MSs.

If a talk-around direct communication connection is to be encrypted, each HR-MS participating in the connection shall have an additional security association (SA) (i.e., talk-around direct communication SA), allowing that connection to be encrypted using keys that are independent of those used for other encrypted transmissions between HR-MSs.

Talk-around direct communication traffic can be encrypted using talk-around direct communication specific key management based on PKMv3.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]