| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Key Request Procedure for IEEE 802.16.1a |  |
| Date Submitted | 2012-01-0918 |  |
| Source(s) | Wai-Leong Yeow, Joseph Teo Chee Ming, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research Eunkyung Kim, Sungcheol Chang ETRI | E-mail: wlyeow@i2r.a-star.edu.sg cmteo@i2r.a-star.edu.sg ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | Detail description of Multicast Security to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Key Request Procedure for IEEE 802.16.1a

Wai-Leong Yeow, Joseph Chee Ming Teo,

Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research (I2R)

Eunkyung Kim, Sungcheol Chang

ETRI

# 1. Introduction

# The IEEE 802.16n System Requirements Document (SRD) specifies shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection. The HR-Network shall provide multicast key management for the group of HR-MSs and the master key shared within the group should be distributed securely.

# To ensure that an attacker is not able to masquerade as a multicast member or eavesdrop in the multicast communications, multicast key management (MKM) protocols have to be designed for the 802.16n networks. Although a Multicast and Broadcast Rekeying Algorithm (MBRA) is already supported in IEEE 802.16-2009, however, it is based on PKMv2 and the rekeying is performed through multicast without any ARQ support. The current IEEE 802.16n AWD supports a key hierarchy for security in multicast but does not describe how the root key is shared between stations of the multicast group.

# In this contribution, we propose a procedure for an HR-MS to initiate a request to the HR-BS and obtain the Multicast Authentication key securely.

# 2. Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Change Table 26 as indicated:]

Table 26—MAC Control messages (continued)

| No. | Functional areas | Message Names | Message Description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| 23 | Security | AAI-PKM-REQ | Privacy Key Management Request | Before AK is derived at network entry: NULL after AK is derived at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived at network entry and the other message is enclosed: CMAC | Unicast |
| 24 | Security | AAI-PKM-RSP | Privacy Key Management Response | Before AK is derived at network entry: NULL at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived after AK is derived at network entry and the other message is enclosed: CMAC | Unicast or Multi-cast |

[Replace Table 71 by following:]

Table 71 – PKMv3 message types

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| 1 | PKMv3 Reauth-Request | AAI-PKM-REQ |
| 2 | PKMv3 EAP-Transfer | AAI-PKM-REQ/AAI-PKM-RSP |
| 3 | PKMv3 Key_Agreement-MSG#1 | AAI-PKM-RSP |
| 4 | PKMv3 Key_Agreement-MSG#2 | AAI-PKM-REQ |
| 5 | PKMv3 Key_Agreement-MSG#3 | AAI-PKM-RSP |
| 6 | PKMv3 TEK-Request | AAI-PKM-REQ |
| 7 | PKMv3 TEK-Reply | AAI-PKM-RSP |
| 8 | PKMv3 TEK-Invalid | AAI-PKM-REQ/AAI-PKM-RSP |
| 9 | Peer_KeyAgreement_MSG#1 | AAI-PKM-RSP |
| 10 | Peer_KeyAgreement_MSG#2 | AAI-PKM-REQ |
| 11 | Peer_KeyAgreement_MSG#3 | AAI-PKM-RSP |
| 12 | PKMv3 MulticastKey-Request | AAI-PKM-REQ |
| 13 | PKMv3 MulticastKey-Reply | AAI-PKM-RSP |
| 914—16 | Reserved | — |

[Add the following text into Section 6.2.3.43 as indicated:]

Table 726 – AAI-PKM-REQ message description field

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 Reauth-Request; PKM v3 message code = 1 - PKMv3 EAP-Transfer; PKM v3 message code = 2 -PKMv3 Key_Agreement-MSG#2; PKM v3 message code = 4 - PKMv3 TEK-Request; PKM v3 message code = 6 - PKMv3 TEK-Invalid; PKM v3 message code =8 - PKMv3 MulticastKey-Request; PKM v3 message code = 12 149–16: Reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests |  |
| CMAC indicator | 1 | Indicates whether this message is protected by CMAC tuple 0: Not protected 1: Protected | Shall always be present |
| … | … | … | … |
| If (PKM v3 message code == 12) { |  |  |  |
| MulticastGrpID | 16 | The identifier of the multicast group 12bits of MSB is MGID and 4bit LSB is FID of the multicast group |  |
| } |  |  |  |

Table 727 – AAI-PKM-RSP message description field

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 EAP-Transfer; PKM v3 message code =2 - PKMv3 Key_Agreement-MSG#1; PKM v3 message code =3 - PKMv3 Key_Agreement-MSG#3; PKM v3 message code =5 - PKMv3 TEK-Reply; PKM v3 message code =7 - PKMv3 TEK-Invalid; PKM v3 message code =8 - PKMv3 MulticastKey-Reply; PKM v3 message code = 13 149–16: Reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests |  |
| CMAC indicator | 1 | Indicates whether this message is protected by CMAC tuple 0: Not protected 1: Protected | Shall always be present |
| … | … | … | … |
| If (PKM v3 message code == 13) { |  |  |  |
| MulticastGrpID | 16 | The identifier of the multicast group 12bits of MSB is MGID and 4bit LSB is FID of the multicast group |  |
| MC_Nonce | 128 | The number used to derive the MCMAC-MTEK Preky |  |
| COUNTER_MTEK | 16 | The current COUNTER_MTEK in use |  |
| MEKS | 2 | Multicast Encryption Key Sequence |  |
| } |  |  |  |

### 6.2.3.43.x PKMv3 MulticastKey-Request message

The HR-MS transmits the PKMv3 MulticastKey-Request message to its serving HR-BS as a first step for an HR-MS initiated multicast key request.

The message shall include the multicast group ID and CMAC Digest attribute for verification, which is computed from CMAC_KEY _U.

Code: 12

The message attributes are shown in Table aaa.

Table aaa – PKMv3 MulticastKey-Request message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | MGID and FID of the multicast group |
| CMAC_digest | Message digest calculated by HR-MS |

### 6.2.3.43.x+1 PKMv3 MulticastKey-Reply message

The HR-BS transmits the PKMv3 MulticastKey-Reply message to derive the multicast keys of a multicast group. The message is either transmitted in response to a MulticastKey-Request message initiated by an HR-MS, or transmitted unsolicitedly.

The message shall include the multicast group ID, MC_Nonce, COUNTER_MTEK and MEKS for an HR-MS to derive the multicast keys.

If the message is unicast to a target HR-MS, it shall be encrypted with the current TEK for confidentiality, and contain the CMAC Digest attribute for verification, which is computed from CMAC_KEY _D.

Otherwise the message is multicast to all HR-MSs and it shall be encrypted with the current MTEK for confidentiality, and contain the MCMAC Digest attribute for MCMAC verification, which is computed from MCMAC_KEY.

Code: 13

The message attributes are shown in Table bbb.

Table bbb – PKMv3 MulticastKey-Response message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | MGID and FID of the multicast group |
| MC_Nonce | The number used to derive the MCMAC-MTEK Preky |
| COUNTER_MTEK | The current COUNTER_MTEK in use |
| MEKS | Multicast Encryption Key Sequence |
| (M)CMAC_digest | Message digest calculated using CMAC or MCMAC key by HR-BS |

[Change Section 6.12.10.2.3.2 as indicated:]

6.12.10.2.3.2 MTEK update

6.12.10.2.3.4 MTEK update

The MTEK update is triggered by MTEKDLE running out the relevant PN space. In particular, HR-BS derives new MTEK when the DL PN space of MTEKDLE is exhausted.

The HR-BS shall indicate the new MC_Nonce, MEKS, COUNTER_MTEK, and MTEK lifetime in the AAI-PKM-RSP message when the current MTEK lifetime expires. If the COUNTER_MTEK or MEKS are updated, the HR-MS updates its MTEK accordingly. Unless the COUNTER_MTEK and MEKS are updated, it means the HR-BS did not derive new MTEK yet and the HR-MS shall maintain the current MTEKs but reset the value of MTEK lifetime.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]