| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of Local Forwarding for IEEE 802.16.1a |  |
| Date Submitted | 2012-01-13 |  |
| Source(s) | Shoukang Zheng, Haiguang Wang, Jaya Shankar, Hoang Anh Tuan, Wai Leong Yeow and Joseph Chee Ming Teo Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2252 E-mail: skzheng@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16.1a AWD |  |
| Abstract | In this document, we propose to support local forwarding for IEEE 802.16.1a network for more efficient communication |  |
| Purpose | To discuss and adopt the proposed text in the 802.16.1a draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support of Local Forwarding for IEEE 802.16.1a

Shoukang Zheng, Haiguang Wang, Jaya Shankar, Hoang Anh Tuan, Wai Leong Yeow and Joseph Chee Ming Teo

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

The SRD in 802.16n requires that HR-Network should allow local forwarding, which allows one HR-MS to communicate to one or more HR-MSs via infrastructure station without going through the backhaul. In this document, we propose the protocol method to support local forwarding for unicast flow.

# 2. Proposed protocol

# 3. Text Proposal for IEEE 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Change the text in section 6.12.6 in 802.16n AWD as follows.]

#### 6.12.6.1 Detection of LF opportunity

When local forwarding opportunity (LF) is determined, an HR-MS may communicate with one or more HR-MSs via an HR-infrastructure station without going through the backhaul. HR-infrastructure station may detect the LF opportunity. Otherwise, LF is detected by upper layer, which is outside of this specification. The capability of LF may be negotiated and may be enabled during HR-RS network entry. HR-RS sends control message AAI-SBC-REQ to HR-BS, indicating that whether it can determine LF opportunity and/or perform LF. HR-BS replies with control message AAI-SBC-RSP to indicate which shall detect LF opportunity and perform LF.

#### 6.12.6.2 LF setup and termination

When HR-infrastructure station detects the LF opportunity, it may communicate with ASN entities to get permission for the LF to be done by HR-infrastructure station.

An infrastructure station capable of providing local forwarding shall (re)assign and manage the uplink FID for the source HR-MS and downlink FID for the destination HR-MS during DSA procedure. Any available FID may be used for the local forwarding (i.e., there are no dedicated FIDs for local forwarding connections). An HR-MS may not be aware of local forwarding but shall follow the same procedure defined in 6.2.

After receiving connection setup control message AAI-DSA-REQ from HR-MS, if LF opportunity is detected and LF is enabled at HR-RS, the uplink connection of the source HR-MS and the downlink connection of destination HR-MS are established using DSA procedure described in 16.2.12.6 individually.

Alternatively, if the LF detection is performed after connection establishment/re-establishment, HR infrastructure station can initiate the LF establishment procedure to set up the LF.  HR infrastructure station shall perform LF as described in 6.12.6.3. HR infrastructure station may communicate with ASN entities to remove the data path between the HR infrastructure station and the ASN for the corresponding uplink flow from the source HR-MS and the corresponding downlink flow to the destination HR-MS.

When DSD procedure is performed, LF is terminated.

When only LF is terminated for the HR-MSs due to the request from ASN entities, HR infrastructure station shall establish the data path for the HR-MSs for which LF is terminated. However, how to terminate LF is outside the scope of this specification.

#### 6.12.6.3 Data traffic forwarding

When LF is adopted and the FIDs for uplink flow from the source to its serving infrastructure station and downlink flow from the serving infrastructure station to the destination HR-MS have been assigned, HR-infrastructure station shall forward the received data traffic from the source HR-MS to the destination HR-MS based on STIDs of the source HR-MS and the destination HR-MS, uplink FID of the source HR-MS and downlink FID of the destination HR-MS.

When the LF opportunity is determined during data traffic forwarding after connection establishment/re-establishment and LF can be done through HR-RS, its serving HR-BS shall continue to deliver the data to maintain the connectivity to the destination HR-MS. Once the LF is setup, the HR-RS shall start to forward the data locally to the destination HR-MS.

The data traffic locally forwarded by HR-RS shall follow MAC PDU formats in 16.2.2. Construction and transmission of MPDUS shall follow 16.2.3. The data traffic locally forwarded by HR-BS shall follow MAC PDU formats in 6.2.2. The construction and transmission of MPDUS shall follow 6.2.3.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0033, “Draft P802.16.1a Draft AWD,” Nov 2011.