| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI-HO-CMD message field |  |
| Date Submitted | 2012-01-16 |  |
| Source(s) | Jeongho Park Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16.1/D3 |  |
| Abstract | This contribution proposes text for clarification on AAI-HO-CMD. |  |
| Purpose | To be discussed and adopted for the 802.16.1 D4 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI-HO-CMD message field

Jeongho Park

Samsung Electronics

- References

[1] IEEE P802.16.1/D3_Nov 2011

- Backgrounds

Handover re-entry has three types.

- CDMA-based RNG is not used.

- CDMA-based RNG is used without dedicated CDMA ranging code

- CDMA-based RNG is used with dedicated CDMA ranging code

The 3rd type is not complete in terms of uplink power control (refer to page 809, 6.3.8.4.9 Uplink power control in handover), because current AAI-HO-CMD does not have necessary fields like iotFP, offsetControl in the specification (refer to page 103, AAI-HO-CMD).

It seems that those fields were removed before by mistake because ASN code still DOES have those fields. (refer to page 979, line 55~57)

This contribution suggests three remedies.

One is simple correction of table number which is currently wrong.

The second is to add the necessary fields in AAI-HO-CMD message.

The third is to change those two fields in AAI-HO-CMD as optional fields so that ABS can skip two fields from AAI-HO-CMD while the two fields are assigned in CDMA-Allocation A-MAP IE. Skipping those two fields prevents AMS from getting confused of using iotFP and offsetControl. (Both AAI-HO-CMD and CDMA-Allocation A-MAP IE would have iotFP and offsetControl.)

- Proposed Text

Remedy #1: Modify line 24 of page 809 as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

6.3.8.4.9 Uplink power control in handover

During the handover processing, if CDMA_RNG_FLAG in message AAI-HO-CMD is set to 0, CDMAbased

ranging will be skipped and the power offset values for the T-ABS will be included in the AAI-HOCMD,

as shown in Table 38. Other uplink power control parameters for T-ABS are set by using the S-ABS

value.

If CDMA_RNG_FLAG in message AAI-HO-CMD is set to 1 and dedicated CDMA ranging code is not

present, CDMA based ranging shall be performed, and the power control parameters of the T-ABS will be

determined using the process defined in 6.3.8.4.8.

Before the handover procedure has been finished, two sets of uplink power control parameters are kept in

AMS to support uplink transmission to S-ABS and T-ABS separately.

If CDMA_RNG_FLAG in message AAI-HO-CMD is set to 1 and dedicated CDMA ranging code is present,

CDMA based ranging shall be performed, and iotFP and offsetControl for the T-ABS will be included in the

AAI-HO-CMD, as shown in Table 3238. In this case, AMS sets iotFP and offsetControl by the values from

AAI-HO-CMD. All other power control parameters of the T-ABS are determined according to 6.3.8.4.8.

After handover, whenever AMS successfully receive AAI-SCD, AAI-ULPC-NI and AAI-UL-POWER-ADJ

messages from T-ABS, the signaled uplink power control parameters will be updated accordingly.

In case of zone switch from LZone to MZone, power control parameters (i.e., iotFP, offsetControl) for the

MZone of the S-ABS shall be included in the Zone Switch TLV if TSTID is also included. In such a case,

CDMA-based ranging will be skipped and AMS will perform bandwidth request procedure in MZone using

the power offset values included in the Zone Switch TLV. Other uplink power control parameters for TABS

are set by using the S-ABS value.

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #2: Add two new fields in line 50 of page 103 as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

Table 38—AAI-HO-CMD message field description (continued)

| Fields | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| Pre-assigned STID | 12 | STID assigned to the AMS by the T-ABS. | Optional |
| iotFP | 7 | IoT value of Frequency Partition used for AMS resource assignment, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB., detail reference to 6.2.3.32 AAI-ULPC-NI message | Optional |
| offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among –15.5 dB to 16 dB with 0.5 dB step | Optional |
| } |  |  |  |

-------------------------------------------------Text Change END ---------------------------------------------------------------

Remedy #3: Modify line 53~60 of page 979 as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

DedicatedRngInfo		 ::= 		SEQUENCE {

dedicatedCDMARngCode 		INTEGER (0..31),

iotFP 					INTEGER (0..3127), 		OPTIONAL,

offsetControl 				INTEGER (0..12763), 	OPTIONAL,

rngOptIndex				INTEGER (0..1)		OPTIONAL,

rngOptSubframeIndex			INTEGER (0..7) 		OPTIONAL

}

-------------------------------------------------Text Change END ---------------------------------------------------------------