| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for MAC Management Messages to Support BS-controlled HR-MS DC in 802.16n |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0032 |  |
| Abstract | In this contribution, we propose modifications/clarifications for some of these MAC Management messages used for BS-controlled DC. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for MAC Management Messages to Support BS-controlled HR-MS DC in 802.16n

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar,

Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

In Atlanta meeting (November 2011), 802.16 TGn has accepted text that describes MAC Management messages to support BS-controlled HR-MS direct communication. In this contribution, we propose:

- Modifications/clarifications for some of these MAC Management messages.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0032.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Table 1019 – HR-RNG-CMD message format as indicated}

Table 1 - HR-RNG-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HR-RNG-CMD_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| Rendezvous time | 4 | Indicates the time offset, in number of frames, when the HR-BS shall provide dedicated ranging opportunity for the transmission of this discovery ranging signal |
| Dedicated CDMA code | 8 | CDMA code assign for discovery purpose, shall be taken from the code set for initial ranging |
| Transmission opportunity offset | 4 | The offset is in number of symbol duration |
| Action | 2 | 0b00: ranging for neighbor discovery and receiving node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out receiving reception 0b10: DC/FTN periodic ranging transmission 0b11: Reserved |
| Reserved | 6 | Shall always be set to 0 |
| if (Action == 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| Reversed action offset | 2 | 0b01-0b11: Indicates number of frames that this HR-MS should switch to receive the same ranging code in the same ranging slot 0b00: no such reversed action. |
| Reserved | 1 | Shall always be set to 0 |
| } |  |  |
| elseif(Action == 0b01){ |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |
| if(Reporting mode == 0b0){ |  |  |
| Reversed action offset | 2 | 0b01-0b11: Indicates number of frames that this HR-MS should switch to receive transmit the same ranging code in the same ranging slot 0b00: no such reversed action. |
| If(Reversed action offset > 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| }else{ |  |  |
| Reserved | 5 |  |
| } |  |  |
| else{ |  | Reporting mode == 0b1 |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |
| Reserved | 3 |  |
| }elseif(Action == 0b10){ |  |  |
| Tx/Rx Indicator Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| Periodicity | 2 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames |
| Tx/Rx Offset | 1 | Offset between transmitting and receiving ranging from the other HR-MS: 0b0: Offset = 1 frame 0b1: Offset = 3 frames |
| } |  |  |
| } |  |  |

HR-MS who receives the HR-RNG-CMD message shall interpret the three parameters for rendezvous time, dedicated ranging code, and transmission opportunity offset in the same way as specified in 6.3.10.4.1.

When HR-RNG-CMD message is used to schedule ranging for neighbor discovery, it is transmitted with the basic or primary CIDs of the receiving HR-MS. When HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled direct communication, it is transmitted with the basic CID allocated to the direct communication link, as defined in 16.2.2.1.2. When HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled FTN, it is transmitted using the basic or primary CID of the forwarded HR-MS.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.