| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for MAC-layer Support for BS-controlled HR-MS FTN in 802.16n |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 Eldad Zeira InterDigital | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0032 |  |
| Abstract | In this contribution, we propose: - Minor modifications to the existing FTN text in 16n AWD; - Description of addressing and connection management involving forwarding HR-MS (Section 16.3.2.1.2); - Details of unspecified control messages like FN-RNG-ACK, FN-RNG-REP, FN-RNG-FLU. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for MAC-layer Support for BS-controlled HR-MS FTN in 802.16n

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

In Atlanta meeting (November 2011), 802.16 TGn has accepted text that describes MAC Data/Control plane procedure related to BS-controlled HR-MS forwarding to network. In this contribution, we propose:

- Minor modifications to the existing FTN text in 16n AWD;

- Description of addressing and connection management involving forwarding HR-MS (Section 16.3.2.1.2);

- Details of unspecified control messages.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0032.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK colors: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Section 16.3.1 as indicated}

### 16.3.1 General Description

In HR-MS Forwarding to Network, an HR-MS forwards user data and control signaling between an HR-MS and an HR infrastructure station. The user data and control signaling do not go through higher layer at the forwarding HR-MS. The origination and termination of the user data and control signaling are at the forwarded HR-MS and the HR infrastructure station respectively and vice versa.

HR-MS Forwarding to Network is applicable when 1) the forwarded HR-MS and the forwarding HR-MS are in coverage of and directly associated to an infrastructure station; 2) the forwarding HR-MS is in coverage of and directly associated to an HR infrastructure station, while the forwarded HR-MS is out of coverage of any HR infrastructure stations.

Under BS-controlled direction communication described in16.2.2, HR-MS forwarding to network is described in16.3.2.

Using talk-around direct communication described in16.2.3, HR-MS forwarding to network is described in16.3.3.

{Remedy 2: Modify Section 16.3.2 as indicated}

### 16.3.2 BS-Controlled HR-MS Forwarding to Network

The general operation of BS-controlled HR-MS forwarding to network (FTN) can be described as follows:

- At the beginning of the process, the serving HR-BS/RS instructs one or a group of its associated HR-MS to broadcast discoverable signals so that an out-of-coverage HR-MS can start network entry and get associated to the HR-BS/RS.

- After the out-of-coverage HR-MS has completed network entry and become an attached forwarded HR-MS, the forwarding HR-MS helps forward control and data messages between the forwarded HR-MS and the serving HR-BS/RS.

##### 16.3.2.1.1 Network entry involving forwarding HR-MS

For an HR-MS that is inside-of-coverage of an HR-BS/RS, the network entry process is as described in 6.3.9.

For an HR-MS that is outside-of-coverage of any HR-BS/RS but within reach of a forwarding HR-MS, the process of network entry can be described as follows.

To enable coverage extension, a serving HR-BS/HR-RS shall schedule some of its directly-associated HR-MSs to transmit preambles so that an outside-of-coverage HR-MS can detect and start network entry.

The HR-BS instruct HR-MS that are associated with it to transmit MS-Frame Control Header (M-FCH) and network configuration information (NCI) at pre-defined resources relative to the preamble transmitted by the HR-MS. The M-FCH and NCI, when transmitted, define resources for access by the HR-MS that is not under HR-BS coverage. This corresponds to the coverage extension procedure defined in 16.3.2.1.1.1. If M-FCH and NCI are omitted then access resources are defined by the index and the sub-carrier set index of the transmitted preamble. This corresponds to the coverage extension procedure defined in 16.3.2.1.1.2.

###### 16.3.2.1.1.1 Coverage Extension with M-FCH and NCI Preceding Initial Ranging

The procedure starts when serving HR-BS/RS transmits an FN-CONFIG-CMD message to instruct one or a group of its directly-associated HR-MS to carry out a coverage-extending process. The transmitted FN-CONFIG-CMD message includes , among other fields, the following information:

- Preamble Index: which identify the preamble to be transmitted by the forwarding HR-MS, together with the frequency segment.

- nstart : the frame number in which the new coverage-extending process should be started.

- mrng: the number of ranging opportunities, together with their particular configurations.

- The parameters determining the transmissions of NCI.

- The parameters determining the transmissions of F-MAP.

- The parameters determining the allocation of resources for ranging response

- The parameters determining the allocation of resources for fast-feedback by the forwarded HR-MS.

- Index of the preamble to be transmitted by the forwarding HR-MS.

- The parameters determining the transmissions of -Frame Control Header (M-FCH), Network Configuration Information (NCI), and Forwarding MAP (F-MAP).

- The parameters allowing a new HR-MS to transmit ranging signal to and receive ranging response from the directly-associated HR-MS.

Prior to sending out the FN-CONFIG-CMD message, HR-BS/RS should reserves a Forwarding Control Zone (FCZ) of three OFDMA symbols at the end of uplink sub-frame for the transmission of preamble and NCI. This zone reservation is achieved by defining a UL_Zone_IE() as specified in Table 472471.

Upon receiving the FN-CONFIG-CMD message, the scheduled HR-MS (or group of HR-MS), from now on simply referred to as forwarding HR-MS, shall start transmitting the specified preamble in the first OFDMA symbol of the FCZ, followed by M-FCH and NCI in the remaining two OFDMA symbols. The formats of M-FCH and NCI are described in 16.3.2.2.1.1 and 6.3.2.3.98.9, respectively. The M-FCH carries similar information to the FCH transmitted by HR-BS, and allows the new HR-MS to receive the NCI, which follows M-FCH.

When a new HR-MS detects the preamble, M-FCH, and NCI transmitted by the forwarding HR-MS and decides to start initial ranging through the forwarding HR-MS, the following process shall be carried out:

- The new HR-MS, after acquiring synchronization and uplink transmission parameters shall choose one ranging CDMA code from those specified in NCI using a uniform random process. The new HR-MS shall send the selected ranging CDMA code on the a ranging slot uniformly selected from those specified in NCI. The new HR-MS is only allowed to transmit a ranging code in the frame that immediately follows a frame in which the NCI containing ranging allocation is transmitted.

- Upon successfully receiving the ranging code from the new HR-MS, the forwarding HR-MS is supposed to respond with an FN-RNG-ACK RNG-RSP message within T3 Timer. As specified in NCI, mrng ranging opportunities shall be allocated. If the new HR-MS has previously transmitted a ranging CDMA code in a ranging opportunity which is not the last opportunity, the HR-MS shall expect to receive an FN-RNG-ACK RNG-RSP message from the forwarding HR-MS before the next ranging opportunity. If the new HR-MS does not receive such an FN-RNG-ACK message, it shall pick one ranging CDMA code from those specified in the NCI with equal probability and transmit on the coming ranging opportunity. The new HR-MS may ramp-up its transmit power as specified in NCI. If the new HR-MS transmits a ranging code in the last ranging opportunity specified in NCI, it shall expect to receive an FN-RNG-ACK message within Tx1 Timer. If the new HR-MS does not receive RNG-RSP from the forwarding HR-MS, it shall wait for the next M-FCH/NCI from the forwarding HR-MS and transmit another ranging signal as specified above.

- The FN-RNG-ACK message provides responses to all the successfully received CDMA ranging codes in the ranging opportunity preceding its transmission. There are four possible ranging status responses from the forwarding HR-MS to the new HR-MS in the FN-RNG-ACK RNG-RSP message, i.e.,

  - Continue: The forwarding HR-MS informs the new HR-MS of needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of “continue”.

  - Success: The forwarding HR-MS informs the new HR-MS of status “success”, but may have adjustment suggestions to the new HR-MS if necessary. With status success, the forwarding HR-MS shall inform the new HR-MS of a BW allocation in the uplink for the new HR-MS to send RNG-REQ message.

  - Abort: The forwarding HR-MS informs the new HR-MS to abort the current initial ranging process.

  - Continue with 2nd Second Phase: This status notification is only applicable if the current coverage extension process follows the Two-Phase approach, as specified in FN-CONFIG-CMD message. The forwarding HR-MS informs the new HR-MS to prepare to receive post-access preambles and post-access NCI. This status shall only be sent once, i.e., it shall not be given when the new HR-MS has already been carrying out ranging using post-access preambles and post-access NCI from forwarding HR-MS.

- Upon receiving a Continue status notification and parameter adjustments in FN-RNG-ACK message, the new HR-MS shall adjust its parameters accordingly and continue the ranging process by selecting one ranging CDMA code from those specified in NCI with equal probability and transmitting in the next available ranging opportunity, or as instructed in the FN-RNG-ACK message.

- Upon receiving a Success status notification, the new HR-MS shall wait for the forwarding HR-MS to inform it of a BW allocation in the UL to send RNG-REQ message. The forwarding HR-MS forwards the corresponding request to the serving HR-BS, and subsequently acts on behalf of the HR-BS in sending an RNG-RSP message to the new HR-MS. Initial ranging process for the new HR-MS is over after receiving RNG-RSP message, which includes a valid Basic CID. If the new HR-MS does not receive bandwidth allocation for RNG-REQ or it does not receive RNG-RSP message for its transmitted RNG-REQ message within Tx1 T3 or the RNG-REQ/RSP is not completed in [TBD] frames, it stops the process and try to associate with another HR-MS or a HR-BS/RS.

- Upon receiving an Abort status notification, the new HR-MS shall stop the ranging process. It shall not attempt to restart initial ranging with a forwarding HR-MS transmitting the same preamble in the next nclosed coverage extending process.

- Upon receiving Continue with 2nd Second Phase status notification, the new HR-MS waits for post-access preamble and post-access NCI to be transmitted by forwarding HR-MS. Upon receiving post-access preamble and NCI, the new HR-MS starts a new round of initial ranging as follows following steps as described above. The initial transmission power for the ranging is determined based on the last ranging prior to detection of post-access preamble and NCI.

The behavior of the forwarding HR-MS and serving HR-BS/RS during initial ranging of the new HR-MS shall be as follows.

- After transmitting NCI, the forwarding HR-MS shall monitor the specified ranging opportunities in the next frame for any initial-ranging attempt by new HR-MS. If no such ranging attempt is detected, the coverage extension process terminates.

- When the forwarding HR-MS detects one or more CDMA ranging codes transmitted on the allocated ranging slots, it shall calculate signal strength and necessary adjustments (time, power, frequency corrections). An FN-RNG-ACK RNG-RSP message shall be transmitted to the new HR-MS.

- FN-RNG-ACK RNG-RSP message shall be transmitted in the FCZ, with the location specified inside the NCI. The HR-BS shall also monitor the pre-allocated resource for the FN-RNG-ACK RNG-RSP in order to carry out any further proactive resource allocation for the coverage extending process.

- The forwarding HR-MS can also transmit an FN-RNG-REP HR-RNG-REP report to the HR-BS. The FN-RNG-REP HR-RNG-REP message contains signal strength and possibly necessary adjustments for the new HR-MS. The HR-BS may receive FN-RNG-REP HR-RNG-REP messages from multiple scheduled forwarding HR-MS. The HR-BS is supposed to follow up with an FN-RNG-FLU message within T66 Timer. before the next allocated ranging opportunity of the coverage extending process if there is any. Otherwise, if this is the last ranging opportunity as specified in NCI, the FN-RNG-FLU message is supposed to be transmitted within Tx2 Timer, where Tx2 < Tx1. The FN-RNG-FLU message specifies the forwarding HR-MS that shall transmit an FN-RNG-ACK RNG-RSP message to the new HR-MS (if such a message needs to be transmitted).

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the UL-MAP and inform the new HR-MS accordingly (using a CDMA Allocation IE). The resource allocation for RNG-REQ message shall be preceded by RNG-RCV-IE which allocates uplink resource for the forwarding HR-MS to relay ranging, registration, capability negotiation, security exchanges from the new HR-MS to serving HR-BS.

- The HR-BS may request the forwarding HR-MS to report any ranging attempt, whether they have crossed the response threshold or not, for management purposes.

{Remedy 3: Removing the entire section 16.3.2.1.1.2 Coverage Extension with Initial Ranging Preceding NCI}

{Remedy 4: Modify Table 1020 – FN-CONFIG-CMD as indicated}

Table 1 - FN-CONFIG-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FN-CONFIG-CMD_Message_Format{ |  |  |
| Management Message Type = [TBD] | 8 | — |
| Frame number action (nstart) | 4 | 4 LSBs of frame number in which the coverage-extending process should be started |
| Preamble index | 7 | As specified in Table 437, 438, 439 |
| Number of ranging opportunities (mrng) | 4 | Ranging opportunities are allocated in every frame that immediately follows a frame in which the forwarding HR-MS transmits preamble, M-FCH, and NCI |
| Used subchannel bitmap | 6 | Bit 0: Subchannel group 0 Bit 1: Subchannel group 1 Bit 2: Subchannel group 2 Bit 3: Subchannel group 3 Bit 4: Subchannel group 4 Bit 5: Subchannel group 5 |
| Repetition_Coding_Indicator | 2 | 0b00: No repetition coding on NCI 0b01: Repetition coding of 2 used on NCI 0b10: Repetition coding of 4 used on NCI 0b11: Repetition coding of 6 used on NCI |
| NCI Coding_Indicator | 2 | 0b00: CC encoding used on NCI 0b01: BTC encoding used on NCI 0b10: CTC encoding used on NCI 0b11: ZT CC encoding used on NCI |
| NCI_Length | 6 | Defines the length of NCI length in OFDMA slots |
| Subchannel offset for NCI | 3 | Indicates the gap, in number of subchannels, between the end of M-FCH and the beginning of NCI. |
| Subchannel offset for ranging feedback | 7 | Counted from the beginning of FCZ. Number of OFDMA slots allocated for ranging feedback is fixed at 2. |
| Subchannel offset for ranging | 5 | Counted from the beginning of FCZ assigned for transmissions by forwarded HR-MS |
| Ranging Method | 1 | 0b0: Initial ranging over two symbols 0b1: BR/periodic ranging over one symbol |
| Initial ranging codes | 5 | Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS. 2 MSBs indicate index of starting code: 0b00: starting code index = 0 0b01: starting code index = 4 0b10: starting code index = 8 0b11: starting code index = 16 3 LSBs indicate number of codes used. The codes are contiguous from the starting code. |
| Forwarding HR-MS EIRP | 7 | Signed in units of 1dBm. |
| EIRxPIR,max | 16 | Initial ranging maximum equivalent isotropic received power at forwarding HR-MS. Signed in units of 1 dBm. |
| F-MAP Length | 4 | In units of OFDMA slots |
| Subchannel offset for F-MAP | 5 | Indicates the gap, in number of subchannels, between the end of NCI and the beginning of F-MAP. |
| F-MAP repetition coding indicator | 2 | 0b00: No repetition coding on NCI 0b01: Repetition coding of 2 used on NCI 0b10: Repetition coding of 4 used on NCI 0b11: Repetition coding of 6 used on NCI |
| Two-phase Indicator | 1 | Indicate whether a two-phase coverage extending process is to be carried out. 0b0: Single phase 0b1: Two phase |
| If (Two-phase Indicator == 0b1){ |  |  |
| Threshold | 6 | The mean CINR measured on the CDMA ranging code above which the forwarding HR-MS should report its reception to serving HR-BS. |
| } |  |  |
| Reserved | 23 |  |
| } |  |  |

{Remedy 5: Insert the following new section}

6.3.2.3.98.x FN-RNG-FLU (Forwarding Ranging Follow-up) message

Serving HR-BS/RS transmits FN-RNG-FLU to instruct an HR-MS to follow-up with the ranging process of a new HR-MS, as part of the coverage extension process.

Table xx1 – FN-RNG-FLU message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HR-RNG-REP_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| Number of ranging codes to be followed up | 4 |  |
| Reserved | 4 |  |
| For (i = 0; i < Number of ranging codes to be followed up; i++){ |  |  |
| Frame number | 4 | 4 LSBs of the frame number in which the code was received |
| CDMA code | 8 | Index of the ranging code to be followed up. |
| Transmission opportunity offset | 4 | The offset is in number of symbol duration |
| } |  |  |
| } |  |  |

{Remedy 6: Modify Tables 1023 - 1025 as indicated}

Table 2 - Extended F-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended-F-MAP_IE() { | — | — |
| EFUC | 4 | 0x0…0xF |
| Length | 4 | Length in bytes of Unspecified Data field |
| Unspecified data | variable variable |  |
| } |  |  |

Table 3 - Extended EFUC code assignment for Extended F-MAP IE

| Extended code EFUC (hexadecimal) | Usage |
| --- | --- |
| 0x0 | Fast-feedback for forwarded HR-MS |
| 0x1 … 0xF | Reserved |

Table 4 - Fast Feedback IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Fast-Feedback_IE() { | — | — |
| EFUC | 4 | 0x0 |
| Length | 4 | 0x21 |
| Subchannel offset | 7 | The lowest index subchannel used for carrying the burst, starting from subchannel 0. |
| Reserved | 1 |  |
| } |  |  |

{Remedy 7: Modify Section 16.3.2.1.2 as indicated}

##### 16.3.2.1.2 Connection management and MPDU transmissions involving forwarding HR-MS

[TBD]

Management and transport connections between a forwarded HR-MS and serving HR-BS/RS are defined in the same way as specified in 6.3.1.1. Each forwarding HR-MS keeps track of all basic/primary/secondary and transport CIDs allocated to all of its associated HR-MSs.

Serving HR-BS/RS schedules resource for the transmissions to/from forwarded HR-MS using FTN Assignment IE, which is an extended UL-MAP IE (UIUC = 15) with Extended UIUC = 0xC (Table xx1). The detailed operation depends on whether the forwarded HR-MS is inside or outside of HR-BS/RS coverage and is described in  16. 3.2.1.2.1 and 16. 3.2.1.2.2.

Table xx1 – FTN Assignment IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FTN Allocation IE () { | — | — |
| Extended UIUC | 4 | 0xC |
| Length | 4 | Set to 0x3 |
| UIUC | 4 | Value is from 1 to 10, for different |
| Duration | 10 | In OFDMA slots |
| Repetition coding indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used. |
| Transmission Indicator | 2 | 0b00: For HR-BS to transmit to forwarding HR-MS 0b01: For forwarding HR-MS to transmit to HR-BS 0b10: For forwarding HR-MS to transmit to forwarded HR-MS 0b11: For forwarded HR-MS to transmit to forwarding HR-MS |
| Frame Offset | 2 | Number of frames, counting from the next frame, that this assignment is applied to. |
| Reserved | 4 |  |
| } |  |  |

16.3.2.1.2.1 Forwarding of UL traffic for an inside-of-coverage forwarded HR-MS

The procedure for forwarding of UL traffic for an inside-of-coverage forwarded HR-MS consists of the following steps:

- The serving HR-BS/RS assigns an UL transmission resource for the forwarded HR-MS through an UL-MAP IE. The forwarded HR-MS, upon receiving such an UL-MAP IE, transmits its MPDU as per normal, i.e., just like transmitting directly to the serving HR-BS/RS.

- As the forwarding HR-MS keeps track of the basic CIDs of all of its forwarded HR-MS, it is aware of the UL resource assignment from the serving HR-BS/RS and shall attempt to receive, if it is not scheduled to transmit uplink traffic on the same frame.

- If the serving HR-BS/RS wants the forwarding HR-MS to forward the received MPDU from forwarded HR-MS, it shall assign UL resource to the forwarding HR-MS in one of the subsequent frames, using an FTN Assignment IE. The FTN Assignment IE is transmitted with the basic CID of the forwarded HR-MS and with the field “Transmission Indicator” set to 0b01.

- The forwarding HR-MS shall buffer the MPDU received from the forwarded HR-MS for at most [TBD] frames. If during those frames, the corresponding FTN Assignment IE is not received from serving HR-BS/RS, the forwarding HR-MS shall discard the received MPDU.

16.3.2.1.2.2 Forwarding of traffic for an outside-of-coverage forwarded HR-MS

The procedure for forwarding of DL traffic for an outside-of-coverage forwarded HR-MS consists of the following steps.

- The serving HR-BS/RS sends a DL-MAP IE with the basic CID of the forwarded HR-MS. As the forwarding HR-MS keeps track of the basic CIDs of all of its forwarded HR-MS, it is aware of the DL resource assignment and shall attempt to receive.

- In one of the subsequent frames, the serving HR-BS/RS assigns an UL resource to forwarding HR-MS to forward the received MPDU toward the forwarded HR-MS, using an FTN Assignment IE. The FTN Assignment IE is transmitted with the basic CID of the forwarded HR-MS and with the field “Transmission Indicator” set to 0b10.

- Upon receiving the FTN Assignment IE, the forwarding HR-MS conveys the assigned resource to the forwarded HR-MS through an F-MAP IE in the coming F-MAP.

- The forwarding HR-MS then proceeds to transmit the MPDU received from the serving HR-BS/RS toward the forwarded HR-MS.

The procedure for forwarding of UL traffic for an outside-of-coverage forwarded HR-MS consists of the following steps.

- The serving HR-BS/RS assigns UL resource for the forwarded HR-MS to transmit its MPDU toward the forwarding HR-MS using an FTN Assignment IE, using the basic CID of the forwarded HR-MS and with the field “Transmission Indicator” set to 0b11.

- Upon receiving the FTN Assignment IE, the forwarding HR-MS conveys the assigned UL resource to the forwarded HR-MS through an F-MAP IE in the coming F-MAP. The forwarding HR-MS subsequently attempts to receive the MPDU transmitted by the forwarded HR-MS in the assigned UL resource.

- In one of the subsequent frames, the serving HR-BS/RS sends a UL-MAP IE with the basic CID of the forwarded HR-MS. The forwarding HR-MS is aware of the UL resource assignment and uses that to forward the MPDU received from the forwarded HR-MS toward the serving HR-BS/RS.

- The forwarding HR-MS shall buffer the MPDU received from the forwarded HR-MS for at most [TBD] frames. If during those frames, the corresponding FTN Assignment IE is not received from the serving HR-BS/RS, the forwarding HR-MS shall discard the received MPDU.

{Remedy 8: Remove section 16.3.2.1.3}

##### 16.3.2.1.3 Construction and transmission of MAC PDUs by forwarding HR-MS

Forwarding and forwarded HR-MS shall construct and transmit MAC PDUs in the same way as described in 6.3.3.

{Remedy 9: Modify Table 465 as indicated}

Table 465.—Extended UIUC code assignment for UIUC = 15

| Extended UIUC (hexadecimal) | Usage |
| --- | --- |
| 0x0 | Power Control IE |
| … | … |
| … | … |
| … | … |
| 0xB | UL Burst Receive IE |
| 0xC … 0xF | Reserved |
| 0xC | FTN Assignment IE |
| 0xD … 0xF | Reserved |

{Remedy 9: Modify Table 688 as indicated}

11.6 RNG-RSP management message encodings

Table 688—RNG-RSP message encodings

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| Ranging Status | 4 | 1 | Used to indicate whether UL messages are received within acceptable limits by BS. 1 = continue, 2 = abort, 3 = success, 4 = second phase. | All |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.