| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for PHY-layer Support for BS-controlled HR-MS FTN in 802.16n |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0032 |  |
| Abstract | In this contribution, we propose minor modifications to the existing FTN text in 16n AWD. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for PHY-layer Support for BS-controlled HR-MS FTN in 802.16n

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

In Atlanta meeting (November 2011), 802.16 TGn has accepted text that describes PHY-layer supports for  BS-controlled HR-MS forwarding to network. In this contribution, we propose:

- Minor modifications to the existing FTN text in 16n AWD;

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0032.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK colors: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Section 16.3.2.2.1 as indicated}

##### 16.3.2.2.1 TDD  Frame Structure

….

To support a forwarded HR-MS that is outside-of-coverage of the serving HR-BS/RS, the TDD frame structure in Fig. 803804 shall be applied. The serving HR-BS/RS reserves a zone, called Forwarding Control Zone (FCZ),  at the end of the uplink subframe for carrying control signaling between forwarding and forwarded HR-MS. The FCZ consists of 3 OFDMA symbols and is assigned to forwarding and forwarded HR-MS in an alternating manner, i.e., in one frame, the FCZ is reserved for forwarding HR-MS to transmit control signaling while in the next frame, the FCZ is reserved for forwarded HR-MS to transmit control signaling, and so on.

- For frames in which FCZ is allocated to transmissions by forwarding HR-MS:

In the first OFDMA symbol of the FCZ, forwarding HR-MS transmits a preamble that is assigned from the same set of frame preambles used by HR-BS/RS. In the remaining two OFDMA symbols of the FCZ, the forwarding HR-MS transmits MS Frame Control Header (M-FCH), Network Configuration Information (NCI), Forwarding MAP (F-MAP), and responses to ranging from forwarded HR-MS within a FCZ. The format of M-FCH is described in 16.3.2.2.1.1. By decoding the M-FCH, a new HR-MS can subsequently receive NCI. The information carried inside the NCI allows a new HR-MS to receive F-MAP and ranging responses. The format of NCI and F-MAP are described in 6.3.2.3.98.9 and 6.3.2.3.98.22, respectively. The FCZ zone has PUSC subcarrier allocations and follows the same specifications for the first DL zone of an HR-BS, with the IDCell being that of the preamble transmitted by the forwarding HR-MS.

- For frames in which FCZ is allocated to transmissions by forwarded HR-MS:

Forwarded HR-MS transmits ranging codes and fast feedback channels over the three OFDMA symbols in the allocated FCZ. OFDMA subcarrier allocation for this FCZ follows the same specifications for the UL subframe of HR-BS.

…

###### 16.3.2.2.1.1 Forwarding zone prefix

M-FCH carries Forwarding zone prefix information. The structure of Forwarding zone prefix is similar to the DL frame prefix transmitted by serving HR-BS, with a 1-bit field “Forwarding_HR-MS_Indicator” to let outside-of-coverage HR-MS know that the transmission is by a forwarding HR-MS. The content of Forwarding frame zone prefix is described in Error: Reference source not found for FFT sizes of 2048, 1024, and 512, and in Error: Reference source not found for FFT size of 128.

{Remedy 2: Modify Section 16.3.2.2.2 as indicated}

##### 16.3.2.2.2 Ranging

Ranging channel is used by both forwarding and forwarded HR-MS to achieved transmission synchronization between them. The forwarding HR-MS makes use of the same periodic ranging channel allocated by HR-BS for its periodic synchronization with HR-BS. The forwarded HR-MS transmit ranging signal in the last 2 OFDMA symbols within the FCZ zone.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.