| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HR-RS Operation when Its Serving HR-BS Loses Backhaul |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to 802.16.1a AWD in 802.16n-11/0033 |  |
| Abstract | In this contribution, we propose modifications/clarifications for some of these MAC Management messages used for BS-controlled DC. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16.1a AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HR-RS Operation when Its Serving HR-BS Loses Backhaul

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar,

Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

In the current AWD, Section 6.12.1.1.2, when HR-BS loses backhaul and needs to operate in TTR mode to connect to another HR-BS, it can continue to support sub-ordinate HR-RS. Corresponding requirements need to be specified for HR-RS operation.

# References

- IEEE 802.16.1a Amendment Working Draft, IEEE 802.16n-11/0033.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Insert the following new section 6.12.5.2}

6.12.5.2 HR-RS operation when serving HR-BS performs dual-mode operation

When an HR-BS performs dual-mode operation as specified in 6.12.1.1, the exchange of control and data between itself and a subordinated TTR HR-RS differs from what described in 6.6 in a number of aspects that are specified in this subclause.

6.12.5.2.1 Resource assignment and HARQ operation

When the dual-role HR-BS assumes BS Operation mode, resource assignment through A-MAP and HARQ timing/operation follow what specified in 6.6. All resource assignments made in BS Operation mode become inactive when the dual-role HR-BS switches to RS Operation.

If the dual-role HR-BS does not transmit to/receive from its subordinated HR-RSs during its RS Operation periods, as indicated in the AAI-ARS-CONFIG-CMD message, no resource assignment shall be made to HR-RS during such RS Operation periods. The dual-role HR-BS and subordinated HR-RS may or may not preserve the contexts of the HARQ channels that were on-going during the preceding BS Operation period. This is indicated by the field “Preserving_HARQ_context” in the AAI-ARS-CONFIG-CMD message.

If the dual-role HR-BS continues to transmit to/receive from its subordinated HR-RSs during its RS Operation periods, as indicated in the AAI-ARS-CONFIG-CMD message, resource assignments shall be made using DL RS-RS Assignment A-MAP IE and UL RS-RS Assignment A-MAP IE. These assignment IEs are specified in Table xx1 and xx2.

Both DL/UL RS-RS Assignment A-MAP IEs shall be transmitted by the dual-role HR-BS to its subordinated HR-RS during its BS Operation periods, e.g., near the end of a BS Operation period. DL/UL RS-RS Assignment A-MAP IEs are persistent in nature and last for the entire duration of an RS Operation period.

Table xx1 – DL RS-RS Assignment A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL_RS-RS_ Assignment A-MAP_IE{ |  |  |
| A-MAP IE Type | 4 | Extended Assignment A-MAP IE |
| Extended Assignment A-MAP IE Type | 4 |  |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| If (MEF == 0b01){ |  |  |
| } else if(MEF == 0b10){ |  |  |
| } |  |  |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| BS_HFA | 3 | HARQ Feedback Allocation, to be used by HR-BS |
| RS_HFA | 3 | HARQ Feedback Allocation, to be used by HR-RS |
| ACID | 4 | HARQ channel identifier |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the DL RS-RS Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192, i.e., with Type Indicator set to “0b011”.

Table xx2 – UL RS-RS Assignment A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| UL_RS-RS_ Assignment A-MAP_IE{ |  |  |
| A-MAP IE Type | 4 | Extended Assignment A-MAP IE |
| Extended Assignment A-MAP IE Type | 4 |  |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| If (MEF == 0b01){ |  |  |
| } else if(MEF == 0b10){ |  |  |
| } |  |  |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| BS_HFA | 3 | HARQ Feedback Allocation, to be used by HR-BS |
| RS_HFA | 3 | HARQ Feedback Allocation, to be used by HR-RS |
| ACID | 4 | HARQ channel identifier |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the DL RS-RS Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192, i.e., with Type Indicator set to “0b011”.

{Remedy 2: Modify Table 100, AAI-ARS-CONFIG-CMD as indicated}

| … |  | … | … |
| --- | --- | --- | --- |
| If (Transmitting or receiving station is affected HR-BS performing a TTR relay operation){ |  |  |  |
| Ta | 11 | Instructed value of timing advance Ta , in units of 0.1 μs | Present if receiving station is affected HR-BS performing TTR relay operation |
| Tbs | 5 | Instructed duration of the BS Operation mode, in units of frames | Always present |
| Trs | 5 | Instructed duration of the RS Operation mode, in units of frames | Always present |
| Start of RS Operation | 5 | Number of frames, not including the current one, that the HR-BS shall start the RS Operation mode; can be used to deduce the operation mode in the current frame. | Always present |
| R-RTI Indicator | 1 | Indicates the value of R-RTI when the HR-BS is assuming RS Operation mode 0b0: R-RTI = 0 0b1: R-RTI = Ts | Present if transmitting station is affected HR-BS performing TTR relay operation |
| Supporting_Subordinated_HR-RS in RS Operation_Indicator | 1 | 0b0: the affected HR-BS does not support subordinated HR-RS when performing RS Operation 0b1: the affected HR-BS continue to support subordinated HR-RS when performing RS Operation |  |
| If (Supporting_Subordinate_HR-RS in RS Operation_Indicator == 0b1){ |  |  |  |
| DB_DCASSB0 | 5/4/3 | Indicates the number of subband-based CRUs in FP0 for DL Relay zone when the affected HR-BS assumes Relay Operation mode. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| DB_DCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 for DL Relay zone when the affected HR-BS assumes Relay Operation mode. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| DB_DCASi | 3/2/1 | Indicates the number of total allocated CRUs, in a unit of a subband, for FPi (i > 0) for DL Relay zone when the affected HR-BS assumes Relay Operation mode. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  |
| DB_UCASSB0 | 5/4/3 | Indicates the number of subband-based CRUs in FP0 for UL Relay zone when the affected HR-BS assumes Relay Operation mode. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| DB_UCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 for UL Relay zone when the affected HR-BS assumes Relay Operation mode. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| DB_UCASi | 3/2/1 | Indicates the number of total allocated CRUs, in a unit of a subbands, for FPi (i > 0) for AAI UL Relay zone. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  |
| } |  |  |  |
| Else{ |  |  |  |
| Preserving_HARQ_context | 1 | All on-going HARQ attempts shall be preserved by the dual-mode HR-BS and subordinated HR-RS |  |
| } |  |  |  |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.