| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for MAC Management Messages to Support BS-controlled HR-MS DC in 802.16.1a |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to 802.16.1a AWD in 802.16n-11/0033 |  |
| Abstract | In this contribution, we propose modifications/clarifications for some of these MAC Management messages used for BS-controlled DC. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16.1a AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for MAC Management Messages to Support BS-controlled HR-MS DC in 802.16.1a

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar,

Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

In the current AWD, the operation of BS-controlled HR-MS direct communication (DC) has been specified. In this contribution, we further provide details on MAC-layer support for the DC operation. Particularly, we focus on:

- Assignment A-MAP IE used to allocate resource for BS-controlled DC.

- HARQ operation of BS-controlled DC.

# References

- IEEE 802.16.1a Amendment Working Draft, IEEE 802.16n-11/0033.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Insert the following new section under 6.12.2.2.1}

6.12.2.2.1.x HARQ operation for HR-MS direct communication

HARQ operation for HR-MS direct communication follows a procedure as described bellow.

- HR-BS/RS initiates the process by transmitting a Direct Link Assignment A-MAP IE, with the field “Direct Link Assignment Type” set to “0bxx”. In such an assignment IE, there are two specifications for HARQ feedback: one is for the feedback in the UL, i.e., from the two involved HR-MSs toward HR-BS/RS, and the other is for the feedback in the DL, i.e., from HR-BS/RS toward the transmitting HR-MS.

- The Direct Link Assignment A-MAP IE is CRC masked using the TWDC address of the transmitting HR-MS. Both transmitting and receiving HR-MS will be able to recognize this allocation and prepare to transmit/receive accordingly.

- Upon receiving the Direct Link Assignment A-MAP IE, the HR-MS scheduled for transmitting shall calculate the transmission timing based on the same specification in 6.2.14.2.2.2.2. Together with transmitting a HARQ subpacket, the HR-MS also sends a HARQ feedback to HR-BS/RS. Even though a standard HARQ ACK/NACK is sent, it is not for acknowledging any transmission. Instead, the feedback signals to the HR-BS/RS whether the transmitting HR-MS wants to retransmit the packet in case the current transmission is not successfully decoded at the receiving HR-MS.

- After receiving the Direct Link Assignment A-MAP ie, the HR-MS scheduled for receiving shall attempt to receive subsequent transmission from the transmitting HR-MS. If the decoding is successful, the receiving HR-MS sends an ACK toward HR-BS/RS in the next frame, in the HARQ feedback channel specified in the original Direct Link Assignment A-MAP IE. Otherwise, if the decoding fails, the receiving HR-MS sends an NACK toward HR-BS/RS.

- Based on the feedback from the receiving HR-MS, together with the indication of the transmitting HR-MS whether to continue with retransmission, the HR-BSs shall carry out one of the following actions:

  - If the feedback from the receiving HR-MS is an ACK: HR-BS/RS sends an ACK in the DL HARQ feedback channel specified in the original Direck Link Assignment A-MAP IE.

  - If the feedback from the receiving HR-MS is an NACK: HR-BS/RS send an NACK in the DL HARQ feedback channel specified in the original Direct Link Assignment A-MAP IE. The HR-BS may schedule a retransmission by sending another Direct Link Assignment A-MAP IE with the same ACID and AI-SN.

- If HR-BS/RS wants to schedule a new HARQ attempt on the HR-MS direct communication link, it sends another Direct Link Assignment A-MAP IE with AI-SN toggled.

Table xx1 – Direct Communication Assignment A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Direct Communication Assignment A-MAP_IE{ |  |  |
| Direct Link A-MAP IE Type | 4 | Set to 0b0000 |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical Encoding with only 1 stream |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| Transmitter Indicator | 1 | Indicate which HR-MS over the direct communication link is the transmitter, the other HR-MS is the receiver |
| HFA DL | 3 | HARQ Feedback Allocation in the DL |
| HFA UL | 3 | HARQ Feedback Allocation in the UL |
| ACID | 4 | HARQ channel identifier |
| AI_SN | 1 | HARQ identifier sequence number |
| Reserved | 7 | To make the size equal 40 bits |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the Direct Communication Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192, i.e., with Type Indicator set to “0b011”.

{Remedy 2: Insert the following new table at the end of section 6.12.2.2.1.4}

Table xx2 – Direct Multicast Assignment A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Direct Multicast Assignment A-MAP_IE{ |  |  |
| Direct Link A-MAP IE Type | 4 | Set to 0b0001 |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical Encoding with only 1 stream |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| Transmitter Indicator | 12 |  |
| Number of valid frames, Nvalid | 2 | For persistent resource allocation 0b00: resource deallocation 0b01: 1 frame 0b10: 2 frame 0b11: 4 frames |
| Reserved | 5 | To make the size equal 40 bits |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the Direct Multicast Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192, i.e., with Type Indicator set to “0b011”.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.