| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for PHY-layer Support for BS-controlled HR-MS FTN in 802.16.1a |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to 802.16.1a AWD in 802.16n-11/0033 |  |
| Abstract |  |  |
| Purpose | To discuss and adopt the proposed text in the 802.16.1a AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for PHY-layer Support for BS-controlled HR-MS FTN in 802.16.1a

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

In the current AWD, the operation of BS-controlled HR-MS FTN has been specified. In this contribution, we further provide details on PHY-layer support for the FTN operation. Particularly, we focus on:

- Specifications for F-MAP and F-MAP Assignment IE;

# References

- IEEE 802.16.1a Amendment Working Draft, IEEE 802.16n-11/0033.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify section 6.12.3.2.2.2.4 as indicated}

6.12.3.2.2.2.4 Forwarding MAP

The Forwarding MAP (F-MAP) carries service control information. Service control information consists of assignment information and power control information. For each forwarding HR-MS, its F-MAP region occupies a number of contiguous DLRUs within an uplink subframe in a forwarding frame. Each F-MAP region is further divided into F-MAP blocks, each occupies a [TBD] number of contiguous DLRUs. F-MAP blocks can be received and decoded independently from each other. The structure of F-MAP is specified in 6.12.3.2.2.2.4.1 and 6.12.3.2.2.2.4.2. The index of the first LRU allocated to the F-MAP region for each forwarding HR-MS is specified by the field “LRU starting index for S-NCI” in I-NCI. When transmitted, F-MAP spans the last 5 OFDMA symbols of the subframe.

F-MAP IE is shown in Table xx1.

Table xx1 – F-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| F-MAP_IE{ |  |  |
| F-MAP IE Type | 2 |  |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical Encoding with only 1 stream |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ Feedback Allocation |
| AI-SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Allocation offset | 2 | 0b00: offset equal -1 0b01: offset equal 0 0b10: offset equal 1 0b11: Reserved |
| Number of valid frames, Nvalid | 2 |  |
| Reserved | 1 | To make the size of this IE equal 32 bits |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the F-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192.

6.12.3.2.2.2.4.1 Normal F-MAP block format

Within each F-MAP region, resource is allocated to F-MAP blocks in the following manner:

- Collect all 6x5 tiles of the allocated DLRUs, each tile contains 26 data tones;

- Each F-MAP block occupies two consecutive tiles.

The two 6x5 tiles allocated to each F-MAP block contains 52 data tones, of which the first NF-MAP_IE data tones are used to carry one F-MAP IE.

6.12.3.2.2.2.4.2 F-MAP block format for supporting WirelessMAN-OFDMA FDM-based UL PUSC Zone

Within each F-MAP region, resource is allocated to F-MAP blocks in the following manner:

- Collect all 4x5 tiles of the allocated DLRUs, each tile contains 14 data tones;

- Each F-MAP blocks occupies 4 consecutive tiles.

The three 4x5 tiles allocated to each F-MAP block contains 56 data tones, of which the first NF-MAP_IE data tones are used to carry one F-MAP IE.

6.12.3.2.2.2.x FTN Assignment A-MAP IE

The following table specifies FTN Assignment A-MAP IE that is transmitted by HR-BS toward forwarding HR-MS to specify resource for the forwarding (reversed) channel.

Table xx2 – FTN Assignment A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Forwarding Assignment A-MAP_IE{ |  |  |
| A-MAP IE Type | 4 | Extended Assignment A-MAP IE |
| Extended Assignment A-MAP IE Type | 4 |  |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical Encoding with only 1 stream |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| Transmitter Indicator | 1 | 0b0: Forwarding HR-MS 0b1: Forwarded HR-MS |
| HFA | 3 | HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier |
| Number of valid frames, Nvalid | 2 |  |
| Extra Resource Indicator | 1 | 0b0: No extra resource can be granted 0b1: Extra resource can be requested |
| F-MAP Block Index | 3 | Index of the F-MAP block to be used to communicate the allocation to forwarded HR-MS |
| Reserved | 1 |  |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the Forwarding Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192, i.e., with Type Indicator set to “0b011”.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.