| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for PHY-layer Support for BS-controlled HR-MS FTN in 802.16.1a |  |
| Date Submitted | 2012-01-09 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 Eldad Zeira InterDigital | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to 802.16.1a AWD in 802.16n-11/0033 |  |
| Abstract |  |  |
| Purpose | To discuss and adopt the proposed text in the 802.16.1a AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for PHY-layer Support for BS-controlled HR-MS FTN in 802.16.1a

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

In the current AWD, the operation of BS-controlled HR-MS FTN has been specified. In this contribution, we further provide details on PHY-layer support for the FTN operation.

# References

- IEEE 802.16.1a Amendment Working Draft, IEEE 802.16n-11/0033.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify section 6.12.3.2.2.2 as indicated}

###### 6.12.3.2.2.2  Control structure for HR-MS FTN

Physical control signaling for BS-controlled HR-MS FTN includes PA/SA-Preambles, I-NCI/S-NCI/P-NCI, F-MAP that are transmitted by forwarding HR-MS, HR-MS-to-HR-MS feedback channels and ranging channels that are used by both forwarding and forwarded HR-MS, and Forwarded-link BR channel that is used by forwarded HR-MS.

{Remedy  : Modify Table 1207  as indicated}

Table 1 – I-NCI IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| BS IDcell | 10 |  |
| Frame Configuration Index | 6 | The mapping between value of this index and frame configuration is listed in Table 149, Table 150, and Table 151 |
| If (WirelessMAN-OFDMA with FDM-based UL PUSC Zone Support){ |  | True if Frame configuration index is equal to – 5, 7, 9, 11, 13, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29 or 30 for 5/10 MHz channel bandwidth according to Table 149; – 4, 6, 8 or 10 for 8.75 MHz channel bandwidth according to Table 150; – 3 or 5 (with CP=1/8) for 7 MHz channel bandwidth according to Table 151. False if Frame configuration index is something else |
| UL_Permbase | 7 | Indicates UL_Permbase used in WirelessMAN-OFDMA system with FDM-based UL PUSC Zone. |
| Reserved | [TBD] |  |
| }else{ |  |  |
| USAC | 5/4/3 | Indicates the number of subbands KSB as defined in Table 246 to Table 248 in 6.3.7.2.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| UFPC | 4/3/3 | Indicate the frequency partition configuration as defined in Table 249 to Table 251 in 6.3.7.2.3 For 2048 FFT size, 4 bits For 1024 FFT size, 3 bits For 512 FFT size, 3 bits |
| UFPSC | 3/2/1 | Indicate the number of subbands allocated to FPi (i > 0) in 6.3.7.2.3 For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |
| UCASSB0 | 5/4/3 | Indicates the number of subband-based CRUs in FP0 in 6.3.7.3.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| UCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 in 6.3.7.3.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| } |  |  |
| Subframe index for S-NCI | 3 | The 2 LSB bits indicate the index of the UL subframe in which S-NCI is transmitted. The MSB bit indicates the frame in which S-NCI is transmitted. MSB bit == 0 indicate the 1st frame while MSB bit = 1 indicate the 3rd frame. |
| LRU starting index for S-NCI | 6 | Indicates the starting index of the LRUs allocated for S-NCI |
| Reserved | [TBD] |  |
| } |  |  |
| PI IE is transmitted | 1 | 1b indicates PI IE is transmitted by HR-MS |
| If PI transmitted { |  |  |
| Subframe index for PI SI_PI | 3 | The 2 LSB bits indicate the index of the UL subframe in which PI is transmitted. The MSB bit indicates the frame in which PI is transmitted. MSB bit == 0 indicate the 1st frame while MSB bit = 1 indicate the 3rd frame |
| LRU starting index for LRU_PI | 6 | Indicates the starting index of the LRUs allocated for PI |
| } |  |  |

{Remedy 3: Modify section 6.12.3.2.2.2.3 as indicated}

6.12.3.2.2.2.3 S-NCI

The S-NCI shall be transmitted in NS-NCI contiguous DLRUs of an uplink subframe. The ways in which S-NCI is transmitted can be described as follows:

- When an HR-MS is instructed by its serving HR-BS/RS to help in a network entry process for some out-of-coverage HR-MS (as described in 6.12.3.2.1.1), the HR-MS transmits S-NCI in an uplink subframe within either the 1st frame or the 3rd frame of the superframe in which I-NCI is transmitted.

- A forwarding HR-MS shall periodically transmit S-NCI as long as there is at least one out-of-coverage forwarded HR-MS attached to it. During this operation, S-NCI can be transmitted in either 1st or 3rd frame of a superframe. The frame-location and periodicity of S-NCI transmission shall be made known to the forwarded HR-MS through a control message [TBD].

In both cases described above, tThe index of the uplink subframe and the index of the starting LRU in which S-NCI is transmitted are signaled in the I-NCI fields “Subframe index for S-NCI” and “LRU starting index for S-NCI”, respectively. When transmitted, S-NCI spans the last 5 OFDMA symbols of the selected subframe.

The content of S-NCI is described in Table.

Table 2 – S-NCI IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| If (Support of WirelessMANOFDMA with FDM-based UL PUSC Zone){ |  |  |
| Subframe offset of the RCH | 2 | Indicates the subframe offset (OSF) of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 |
| Start RP code information of the RCH | 4 | Indicates the kns, which is the parameter for start of the RP code group (rns0). rns0(kns) = 16 × kns + 1, kns = 0,1, ...,15 |
| Number of RP codes allocated for coverage extension ranging | 2 | Indicates the number of RP codes and the particular codes that can be used for ranging with the forwarding HR-MS. |
| }else{ |  |  |
| Subframe offset of the S-RCH | 2 | Indicates the subframe offset (OSF) of the S-RCH allocation |
| Start RP code information of the S-RCH |  | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |
| Transmission timing offset of SRCH | 3 | Indicates NRTO, which is the parameter used for the calculation of the sample number, TRTO, which is applied to advance the ranging signal transmission timing relative to the defined uplink transmission timing point based on the frame structure from AMS perspective when AMS conducts initial or handover ranging in a femtocell. TRTO = floor(NRTO × (Tg – 2) × FS)(samples) where NRTO = min( RTD ⁄ (Tg–2) , 7), and RTD is the round trip delay from the femto-ABS to the overlay macro-ABS. The range of values is 0 ≤ NRTO ≤ 7. |
| } |  |  |
| Indicator of ranging opportunities per superframe | 1 | 0b0: 1 ranging opportunity per superframe, and located at 2nd frame 0b1: 2 ranging opportunities per superframe, and located at 2nd and 4th frames |
| Frame containing the ranging opportunity | 1 | 0b0: The ranging opportunity is located at 2nd frame within a superframe 0b1: The ranging opportunity is located at 4th frame within a superframe |
| Number of ranging opportunities | 2 | The number of ranging opportunities that are allocated after each S-NCI. |
| UCASi | 3/2/1 | Indicates the number of total allocated CRUs, in a unit of a subband, for FPi (i ≥ 0) in 6.3.7.3.1 For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |
| Forwarding EIRP | 5 | Unsigned integer from 1 to 31 in units of 1 dBm, where 0b00000=1 dBm and 0b11111=31 dBm. |
| HR-MS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed HR-MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm. |
| EIRxPIR,min | 5 | Unsigned integer from –133 to –102 in units of 1 dBm, where 0b00000 = –133 dBm and 0b11111 = –102 dBm. |
| Pre-access S-NCI Indicator | 1 | Indicates whether this is a pre-access S-NCI that is used for Group Discovery mode 0b0: not a pre-access S-NCI 0b1: pre-access S-NCI |
| Preamble Periodicity | 2 | Indicates the pattern by which the forwarding HR-MS transmits PA/SA-Preambles during HR-MS FTN 0b00: PA-Preamble is transmitted in the 1st symbol of the 2nd frame of every superframe while SA-Preamble is transmitted in the 1st symbol of the 3rd frame of every superframe 0b01: PA/SA-Preambles are transmitted once every two superframes 0b10: PA/SA-Preambles are transmitted alternatively, with PA-Preamble being transmitted in odd-number superframes and SA-Preamble being transmitted in even-number superframes 0b11: Reserved |
| I-NCI Transmission Indicator | 2 | Indicate the next transmission of I-NCI 0b00: No plan for I-NCI transmission 0b01: I-NCI is going to be transmitted in the next superframe 0b10: I-NCI is going to be transmitted in two superframes from the current superframe 0b11: Reserved |

{Remedy 3: Insert the following new section}

6.12.3.2.2.2.4 P-NCI

The P-NCI shall be transmitted in NP-NCI contiguous DLRUs of an uplink subframe. A forwarding HR-MS shall periodically transmit P-NCI as long as there is at least one out-of-coverage forwarded HR-MS attached to it. During this operation, S-NCI can be transmitted in either 1st or 3rd frame of a superframe. The frame-location and periodicity of P-NCI transmission shall be made known to the forwarded HR-MS through a control message [TBD].

The index of the uplink subframe and the index of the starting LRU in which P-NCI is transmitted are the same as that used for the transmission of S-NCI. When transmitted, P-NCI spans the last 5 OFDMA symbols of the selected subframe.

The content of P-NCI is described in Table xyz.

Table xyz – P-NCI IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Superframe number | 12 |  |
| If (Support of WirelessMANOFDMA with FDM-based UL PUSC Zone){ |  |  |
| Subframe offset of the RCH | 2 | Indicates the subframe offset (OSF) of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 |
| Start RP code information of the RCH | 4 | Indicates the kns, which is the parameter for start of the RP code group (rns0). rns0(kns) = 16 × kns + 1, kns = 0,1, ...,15 |
| Number of RP codes allocated for coverage extension ranging | 2 | Indicates the number of RP codes and the particular codes that can be used for ranging with the forwarding HR-MS. |
| }else{ |  |  |
| Subframe offset of the S-RCH | 2 | Indicates the subframe offset (OSF) of the S-RCH allocation |
| Start RP code information of the S-RCH |  | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |
| Transmission timing offset of SRCH | 3 | Indicates NRTO, which is the parameter used for the calculation of the sample number, TRTO, which is applied to advance the ranging signal transmission timing relative to the defined uplink transmission timing point based on the frame structure from AMS perspective when AMS conducts initial or handover ranging in a femtocell. TRTO = floor(NRTO × (Tg – 2) × FS)(samples) where NRTO = min( RTD ⁄ (Tg–2) , 7), and RTD is the round trip delay from the femto-ABS to the overlay macro-ABS. The range of values is 0 ≤ NRTO ≤ 7. |
| } |  |  |
| Preamble Periodicity | 2 | Indicates the pattern by which the forwarding HR-MS transmits PA/SA-Preambles during HR-MS FTN 0b00: PA-Preamble is transmitted in the 1st symbol of the 2nd frame of every superframe while SA-Preamble is transmitted in the 1st symbol of the 3rd frame of every superframe 0b01: PA/SA-Preambles are transmitted once every two superframes 0b10: PA/SA-Preambles are transmitted alternatively, with PA-Preamble being transmitted in odd-number superframes and SA-Preamble being transmitted in even-number superframes 0b11: Reserved |
| Subframe index for F-MAP | 3 | The 2 LSB bits indicate the index of the UL subframe in which F-MAP is transmitted. The MSB bit indicates the frame in which F-MAP is transmitted. MSB bit == 0 indicate the 1st frame while MSB bit = 1 indicate the 3rd frame. |
| LRU starting index for F-MAP | 6 | Indicates the starting index of the LRUs allocated for F-MAP |
| F-MAP Size | 2 | Indicates the number of LRUs allocated to the F-MAP |
| I-NCI/S-NCI Transmission Indicator | 2 | Indicate the next transmission of I-NCI/S-NCI 0b00: No plan for I-NCI/S-NCI transmission 0b01: I-NCI/S-NCI is going to be transmitted in the next superframe 0b10: I-NCI/S-NCI is going to be transmitted in two superframes from the current superframe 0b11: Reserved |
| PI IE is transmitted | 1 | 1b indicates PI IE is transmitted by HR-MS |
| If PI transmitted { |  |  |
| Subframe index for PI SI_PI | 3 | The 2 LSB bits indicate the index of the UL subframe in which PI is transmitted. The MSB bit indicates the frame in which PI is transmitted. MSB bit == 0 indicate the 1st frame while MSB bit = 1 indicate the 3rd frame |
| LRU starting index for LRU_PI | 6 | Indicates the starting index of the LRUs allocated for PI |
| } |  |  |

{Remedy 5: Modify section 6.12.3.2.2.2.4 as indicated}

6.12.3.2.2.2.5 6.12.3.2.2.2.5 Forwarding MAP

The Forwarding MAP (F-MAP) carries service control information. Service control information consists of assignment information and power control information. For each forwarding HR-MS, its F-MAP region occupies a number of contiguous DLRUs within an uplink subframe in a forwarding frame. Each F-MAP region is further divided into F-MAP blocks, each occupies a [TBD] number of contiguous DLRUs. F-MAP blocks can be received and decoded independently from each other. The structure of F-MAP is specified in 6.12.3.2.2.2.5.1 and 6.12.3.2.2.2.5.2. The index of the first LRU allocated to the F-MAP region, together with its size, for each forwarding HR-MS is specified in P-NCI by the field “LRU starting index for S-NCI” in I-NCI. When transmitted, F-MAP spans the last 5 OFDMA symbols of the subframe.

6.12.3.2.2.2.5.1 F-MAP block format when not supporting WirelessMAN-OFDMA FDM-based UL PUSC Zone

Within each F-MAP region, resource is allocated to F-MAP blocks in the following manner:

- Collect all 6x5 tiles of the allocated DLRUs, each tile contains 26 data tones;

- Each F-MAP block occupies two consecutive tiles.

The two 6x5 tiles allocated to each F-MAP block contains 52 data tones, of which the first NF-MAP_IE = 48 data tones are used to carry one F-MAP IE.

6.12.3.2.2.2.5.2 F-MAP block format for supporting WirelessMAN-OFDMA FDM-based UL PUSC Zone

Within each F-MAP region, resource is allocated to F-MAP blocks in the following manner:

- Collect all 4x5 tiles of the allocated DLRUs, each tile contains 14 data tones;

- Each F-MAP blocks occupies 4 consecutive tiles.

The three 4x5 tiles allocated to each F-MAP block contains 56 data tones, of which the first NF-MAP_IE =48 data tones are used to carry one F-MAP IE.

{Remedy 6: Insert the following new section}

6.12.3.2.2.2.x Resource Assignment IE  for FTN

6.12.3.2.2.2.x.1  FTN Assignment A-MAP IE

The following table specifies FTN Assignment A-MAP IE that is transmitted by HR-BS toward forwarding HR-MS to specify resource for the forwarding (reversed) channel. Each FTN Assignment A-MAP IE is CRC-masked by the STID of the forwarded HR-MS, and with “Type Indicator” set to 0b011 as specified in Table 192.

Table xx1 – FTN Assignment A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Forwarding Assignment A-MAP_IE{ |  |  |
| A-MAP IE Type | 4 | Extended Assignment A-MAP IE |
| Extended Assignment A-MAP IE Type | 4 | Value is set to 0xD |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical Encoding with only 1 stream |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| Transmitter Indicator | 1 | 0b0: Forwarding HR-MS 0b1: Forwarded HR-MS |
| HFA | 3 | HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier |
| Number of valid frames | 2 | Number of consecutive forwarding or reversed frames that this resource allocation applies to |
| Extra Resource Indicator | 1 | 0b0: No extra resource can be granted 0b1: Extra resource can be requested |
| F-MAP Block Index | 3 | Index of the F-MAP block within the F-MAP region of the forwarding HR-MS to be used to communicate the allocation to forwarded HR-MS |
| Reserved | 1 |  |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the Forwarding Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192, i.e., with Type Indicator set to “0b011”.

6.12.3.2.2.2.x.2  F-MAP IE

Forwarding HR-MS signal resource allocation to its forwarded HR-MS through an F-MAP IE, which is specified in Table xx2. Each F-MAP IE is CRC-masked by the STID of the forwarded HR-MS, and with “Type Indicator” set to 0b011 as specified in Table 192.

Table xx2 – F-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| F-MAP_IE{ |  |  |
| F-MAP IE Type | 2 |  |
| ISizeOffset | 5 | Offset use to compute burst size index |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical Encoding with only 1 stream |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ Feedback Allocation |
| AI-SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Allocation offset | 2 | 0b00: offset equal -1 0b01: offset equal 0 0b10: offset equal 1 0b11: Reserved |
| Number of valid frames | 2 | Number of consecutive forwarding or reversed frames that this resource allocation applies to |
| Reserved | 1 | To make the size of this IE equal 32 bits |
| } |  |  |

a A 16-bit CRC is generated based on the randomized contents of the F-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 192.

{Remedy 7: Modify section 6.12.3.2.2.2.6 as indicated}

6.12.3.2.2.2.6 6.12.3.2.2.2.7 Ranging channel

Ranging channel is used by both forwarding and forwarded HR-MS to achieved transmission synchronization between them. The forwarding and forwarded HR-MS make use of the same synchronized ranging channel S-RCH defined in 6.3.8.1.4.2 and 6.3.8.1.4.3.

The transmission of initial ranging code by the new HR-MS shall follow:

- For frame configuration without FDM-based UL PUSC zone support:

  - In the time domain, the S-RCH allocation is specified by the subframe offset OSF transmitted in S-NCI, i.e., ranging opportunity is located at mod(OSF + 1, NUL)th uplink AAI subframe of a particular frame. The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell of the superordinate HR-BS (transmitted in I-NCI) and the allocated number of subbands RSB according to the following equation, where IDcell is defined in 6.3.5.1.2 and RSB is LSB-CRU,FPi/4, where LSBCRU, FPi is the number of allocated subband CRUs as defined in 6.3.7.3 for FPi corresponding to reuse 1 partition or power-boosted reuse 3 partition only if there is no reuse 1 partition.

ISB,s = mod(IDcell + 1, RSB),

where ISB, s denotes the subband index (0, …, RSB– 1) for ranging resource allocation among RSB subbands.

- For frame configuration with FDM-based UL PUSC zone support:

  - The un-associated HR-MS shall transmit ranging preamble in a similar way to what specified in 6.3.8.2.4.3.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.