| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the definition of AAI MAC control message using ASN.1 notation |  |
| Date Submitted | 2012-01-12 |  |
| Source(s) | Hyun Lee, Jeasun Cha, Eunkyung Kim, Sungcheol Chang, Seokki Kim, Won-Ik Kim, Miyoung Yun, Sungkyung Kim, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: hyunlee@etri.re.kr |
| Re: | Proposed text changes to P802.16.1/D3 |  |
| Abstract | This contribution proposes text for clarification on the definition of AAI MAC control message using ASN.1 notation |  |
| Purpose | To be discussed and adopted for the 802.16.1 D4 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the definition of AAI MAC control message using ASN.1 notation

Hyun Lee, Jeasun Cha, Eunkyung Kim, Sungcheol Chang, Seokki Kim, Won-Ik Kim, Miyoung Yun, Sungkyung Kim, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

This contribution is intended to incorporate changes for correction on wrong ASN.1 notations of AAI MAC control message.

# References

- IEEE P802.16.1/D3_Nov 2011

# Proposed Text for the P802.16.1/D3

Note:

The text in BLACK color: the existing text in the P802.16.1/D3

The text in RED color: the removal of existing P802.16.1/D3 Text

The text in BLUE color: the new text added to the P802.16.1/D3 Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify line 10~14 and line 29 on page 954 of P802.16.1/D3 as follows.]

packetIpv4 (1),

packetIpv6 (2),

packetEthernet (3),

packetIpv4OrIpv6 (14),

multiProtocol (15)

} (SIZE (16))

MobilityFeaturesSupported ::= BIT STRING {

HhoSupport (0),

SsleepModeSupport (1),

SsCRModeSupport (2),

Rreserved(3)

} (SIZE (4))

AAI-REG-REQ ::= SEQUENCE {

amsMacAddress MACAddress,

amsCapNegotiation AmsCapabilities,

csCapabilities CsCapabilities,

hostCfgCapIndicator FeatureSupport,

-- maximum requested host configuration size is 1023 octets

requestedHostConfig OCTET STRING (SIZE (0..1023)) OPTIONAL,

globalCarrierConfigChangeCount INTEGER (0..7),

amsInitAgpServiceAdaptation FeatureSupport OPTIONAL,

vendorID VendorID OPTIONAL,

mobilityFeaturesSupported MobilityFeaturesSupported OPTIONAL.,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Registration Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-REG-RSP ::= SEQUENCE {

stidAndMAPMaskSeed SEQUENCE {

stid STID,

mapMaskSeed MapMaskSeed

} OPTIONAL,

crid CRID OPTIONAL,

femtoAbsLdm SEQUENCE {

startSuperframeOffset INTEGER (0..511),

availableIntervalLeng INTEGER (0..15),

unavailableIntervalLeng INTEGER (0..255)

} OPTIONAL,

agpsMethod FeatureSupport OPTIONAL,

imCapabilities ImCapabilities OPTIONAL,

antennaConfig SoundingAntennaSw OPTIONAL,

embsCapabilities EmbsCapabilities OPTIONAL,

persistentAllocation FeatureSupport OPTIONAL,

groupResourceAllocation FeatureSupport OPTIONAL,

hoTriggerMetric ReportMetric OPTIONAL,

csCapabilities CsCapabilities OPTIONAL,

ipv4HostAddress IPv4Address OPTIONAL,

ipv6HomeNetworkPrefix IPv6HomeNetworkPrefix OPTIONAL,

-- maximum additional host configuration IE size is 1023 octets

additionalHostConfigIe OCTET STRING (SIZE (0..1023)) OPTIONAL,

redirectionInfoArray SEQUENCE (SIZE (1..8)) OF RedirectionInfo OPTIONAL,

[Remedy2: Modify line 9 on page 955 of P802.16.1/D3 as follows.]

csgIdlength INTEGER (1..24) OPTIONAL,

globalCarrierCfgChangeCount INTEGER (0..7),

multicarrierCapabilities McCapabilities OPTIONAL,

csTypeOfDefaultServiceFlow CsSpecification OPTIONAL,

clcLimits SEQUENCE (SIZE (1..2)) OF ClcLimits OPTIONAL,

amsInitAgpServiceAdaptation FeatureSupport OPTIONAL,

vendorID VendorID OPTIONAL,

mobilityFeaturesSupported MobilityFeaturesSupported OPTIONAL.,

...

}

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Network exit Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

IdleModeRetain ::= BIT STRING {

sbcMessages (0),

-- Retain info associated with SBC messages

pkmMessages (1),

-- Retain info associated with PKM messages

regMessages (2),

-- Retain info associated with REG messages

networkAddr (3),

-- Retain info associated with network addresses

msState (4)

-- Retain MS state information

} (SIZE (5))

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DeRegistration Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DREG-REQ ::= SEQUENCE {

deRegReqCode CHOICE {

deregFromABSAndNetwork NULL, -- 0x00

deregAndInitIdleMode DeregAndInitIdleMode, -- 0x01

unsolicitedDeregRspWithAct05 NULL, -- 0x02

rejectUnsolicitedDeregRsp NULL, -- 0x03

deregToEnterDcrMode DeregToEnterDcrMode, -- 0x04

unsolicitedDeregRspWithAct00-01-02-03 NULL, -- 0x05

...

},

...

}

DeregAndInitIdleMode ::= SEQUENCE {

pgCycleReq PgCycle,

idleModeRetainInfo IdleModeRetain,

-- if a system supports Mobility information

-- Slow (0-10km/h), Medium (10-120km/h), Fast (above 120km/h)

mobilityInfo AMSMobilityLevel OPTIONAL

}

DeregToEnterDcrMode ::= SEQUENCE {

idleModeRetainInfo IdleModeRetain

}

[Remedy3: Modify line 63 on page 978 of P802.16.1/D3 as follows.]

}

CINRMean ::= INTEGER (0..255)

RSSIMean ::= INTEGER (0..255)

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Handover Command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-HO-CMD ::= SEQUENCE {

mode CHOICE {

hoCmd HandoverCommand, -- 0b00

zsCmd ZoneSwitchCommand, -- 0b01

hoReject NULL, -- 0b10

reserved NULL -- 0b11

},

...

}

HandoverCommand ::= SEQUENCE {

hoReentryMode CHOICE {

bbeHO NULL,

ebbHO EBBHOInfo

},

disconnectTimeOffset INTEGER (0..255),

resourceRetainTime INTEGER (0..255) OPTIONAL,

targetBSList SEQUENCE (SIZE (0..maxNewABSIndices)) OF TargetBSInfo

}

EBBHOInfo ::= SEQUENCE {

-- "interleavingMode" is used to avoid using

-- OPTIONAL to those hoReentryXXXX parameters

interleavingMode CHOICE {

-- hoReentryInterleavingInterval > 0

withInterleaving HoReentryInterleavingInfo,

-- hoReentryInterleavingInterval = 0(multicarrier EBB)

noInterleaving NULL

},

servingPhyCarrierID PhyCarrierIndex OPTIONAL

}

HoReentryInterleavingInfo ::= SEQUENCE {

hoReentryInterleavingInterval INTEGER (1..256),

hoReentryInterval INTEGER (1..256),

hoReentryIteration INTEGER (1..8)

}

TargetBSInfo ::= SEQUENCE {

targetBSID BSID,

preambleIndex CHOICE {

saPreambleIndex PreambleIndex,

Rr1PreambleIndex R1PreambleIndex

},

centerFrequency CenterFreq OPTIONAL,

[-------------------------------------------------End of Text Proposal----------------------------------------------------]