| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of paging message segmentation |  |
| Date Submitted | 2012-01-18 |  |
| Source(s) | Giwon Park LG Electronics | E-mail: giwon.park@lge.com |
| Re: | Proposed text changes to P802.16.1/D3 |  |
| Abstract | This is to clarify the text about the paging message segmentation |  |
| Purpose | To be discussed and adopted for the 802.16.1 D4 Draft |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification of paging message segmentation

Giwon Park

LG Electronics

# Introduction

When the paging message is larger than the maximum size supported by the network, it may be segmented. Moreover, idle mode section of the draft [1] describes that paging message may be segmented. However, regarding the paging message segmentation, some text of idle mode section was wrongly described as fragmentation.

Thus, we clarify the description of paging message segmentation.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the table 49 in page 143, line 47 as follows:]

Table 49– AAI-PAG-ADV message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists 0b0: This is the last <delete>fragment</delete> <insert>segment </insert>of the AAI-PAG-ADV message 0b1: This is not the last <delete>fragment</delete> <insert>segment </insert>of the AAI-PAG-ADV message; the remaining <delete>fragments</delete> <insert>segments </insert>of the message will be transmitted in the subsequent subframes or frames. |  |

# References

[1] IEEE P802.16.1/D3_Nov 2011