| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for A-MAP relevance and HARQ timing |  |
| Date Submitted | 2012-01-17 |  |
| Source(s) | Jisoo Park, Yongseouk Choi, Youngjo Bang, Sookjin Lee ETRI | Voice: +82-42-860-5748 jsp@etri.re.kr |
| Re: | Proposed text changes to P802.16.1/D3 by sb01R0 |  |
| Abstract | The contribution proposes the clean-up text related to A-MAP relevance and HARQ timing. |  |
| Purpose | To be discussed and adopted the proposed text in next revision as the 802.16.1/D4 by Maintenance Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for A-MAP relevance and HARQ timing

Jisoo Park, Yongseouk Choi, Youngjo Bang, Sookjin Lee

ETRI

- Introduction

This contribution proposes the cleanup text related to the A-MAP relevance and HARQ timing in the current draft standard [1].

- References

[1] IEEE P802.16.1/D3, “WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems,” 2011-11-28

- Proposed Text Changes

The blue colored tag is used to represent the inserted texts, figures, or tables using the following INSERT TAG: <insert> </insert>

The red colored tag is used to represent the deleted texts, figures, or tables using the following DELETE TAG: <delete> </delete>

------------------------------------------- Start of Proposed Text Changes --------------------------------------------

[Remedy #1: Adopt the following modification text in line 32 on page 433 section 6.2.14.2.2]

To determine A-MAP relevance and HARQ timing, DL HARQ feedback offset z, UL HARQ <insert>Subpacket </insert>Tx offset v, and UL HARQ feedback offset w shall be set. In DL HARQ transmission, the DL Rx processing time TDL_Rx_Processing at AMS shall be considered for the DL HARQ feedback offset(z). In UL HARQ transmission, UL Tx processing time TUL_Tx_Processing at AMS and the UL Rx processing time TUL_Rx_Processing at ABS are considered for the UL HARQ <insert>Subpacket </insert>Tx offset(v) and the UL HARQ feedback offset(w), respectively.

[Remedy#2: Adopt the following modification text on Table 135 of page 433, section 6.2.14.2.2.1.1 ]

| Table 135 - FDD DL HARQ timing |  |  |
| --- | --- | --- |
| Content | AAI Subframe Index | Frame Index |
| DL Assignment A-MAP IE Tx | l | i |
| DL HARQ Subpacket Tx | m = l | i |
| UL HARQ feedback <delete>Tx</delete> <insert>Tx</insert> | n=ceil(m+F2)modF | <delete>=i+floorceil(m+F2)F+zmod</delete><insert>j=i+floorceil(m+F2)F+zmod4</insert> Where z=0, if((ceil(F2)−NTTI)≥TDL_Rx_Processing) |

[Remedy#3: Adopt the following modification text in line 7 on page 436, section 6.2.14.2.2.1.2]

<delete>p k</delete><insert>k</insert> is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the HARQ acknowledgement is sent

<delete>k p</delete><insert>p</insert> is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where a retransmission of the HARQ subpacket begins

NTTI is the number of AAI subframes which a HARQ subpacket spans; i.e., 1 for the default TTI and 4 for long TTI in TDD UL

TUL_Tx_Processing is the data burst Tx processing time required by the AMS and measured in subframes

TUL_Rx_Processing is the data burst Rx processing time required by the ABS and measured in subframes

Figure 76 shows an example of the timing relationship between an UL Assignment A-MAP IE, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in FDD frame structure, for 5 MHz, 10 MHz, and 20 MHz channel bandwidths. In this example, TUL_Tx_Processing and TUL_Rx_Processing <delete>TUL_Rx_Processing</delete> are 3 AAI subframes.

[Remedy #4: Adopt the following modification text in line 12 on table 137 of page 437 section 6.2.14.2.2.2.1]

UL HARQ feedback <delete>Tx</delete><insert>Tx</insert>

[Remedy #5: Adopt the following modification text on table 138 of page 438 section 6.2.14.2.2.2.2]

| Table 138 - TDD UL HARQ timing |  |  |
| --- | --- | --- |
| Content | AAI Subframe Index | Frame Index |
| UL Assignment A-MAP Tx IE in DL |  |  |
| UL HARQ Subpacket Tx | For default TTI and m=0, for 0≤l<Kl−K, for K≤l<U+K Where | Where |
|  | For default TTI and <delete>0,…,or l−K, for l=0l−K, for 0<l</delete> <insert>m=0,…,or l−K, for l=0l−K, for 0<l<D−1</insert> Where |  |
|  | For long TTI m = 0 |  |
| DL HARQ feedback Tx | l | Where |
| UL HARQ Subpacket ReTx | m | Where |

Where:

D is the number of downlink subframes as defined by the frame configuration table

U is the number of uplink subframes as defined by the frame configuration table

l is the reference to the DL subframe, starting from 0 for the first downlink subframe and numbering up to D-1,    where the A-MAP is transmitted or the HARQ acknowledgement is sent

m is the reference to the UL subframe, starting from 0 for the first downlink subframe and numbering up to U-1, where HARQ subpacket begins its transmission

i is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the A-MAP is transmitted

j is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the HARQ subpacket begins

k is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the HARQ acknowledgement is sent

p is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where a retransmission of the HARQ subpacket begins

NTTI is the number of AAI subframes which a HARQ subpacket spans; i.e., 1 for the default TTI and <delete> D</delete><insert> U</insert> for long TTI in TDD <delete>DL</delete> <insert>UL</insert>

TUL_Tx_Processing is the data burst Tx processing time required by the AMS and measured in subframes

TUL_Rx_Processing is the data burst Rx processing time required by the ABS and measured in subframes

Figure 78 shows an example of the timing relationship between an UL Assignment A-MAP IE, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in TDD frame structure, for 5 MHz, 10 MHz, and 20 MHz channel bandwidths. In this example, TUL_Tx_Processing <delete>TUL_Tx_Processing</delete> and TUL_Rx_Processing <delete>TUL_Rx_Processing</delete> are 3 AAI subframes.

----------------------------------------------- Proposed texts end --------------------------------------------------------