| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal for cognitive radio aspects in hierarchical networks |  |
| Date Submitted | 2012-1-17 |  |
| Source(s) | Satoshi Imata KDDI R&D Laboratories Inc. | E-mail: sa-imata@kddilabs.jp |
| Re: | Call for contributions for the 802.16 PPC |  |
| Abstract | It proposes cognitive radio aspects in Hierarchical Network Study Report. |  |
| Purpose | For discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal for Cognitive radio aspects in hierarchical networks

Satoshi Imata

KDDI R&D Laboratories Inc.

# Introduction

A work plan for a contribution toward ITU-R Cognitive Report was discussed in the #76 session of IEEE 802.16 in November 2011. According to the work plan (see page 9 of IEEE C80216ppc-11/0014r2), it is planed to update the hierarchical network study report in IEEE 802.16ppc-11/0009 with cognitive radio aspects and to decide whether we should submit a contribution to ITU-R by this January session. This contribution suggests modifications to the hierarchical network study report, where cognitive radio aspects are introduced. The proposed text is described in the usage model section and new Appendix C is proposed accordingly.

# Proposed text change

Adopt the following remedies in IEEE 802.16ppc-11/0009.

---------------------------------- The start of text -----------------------------------------

[Remedy #1: Modify the texts as indicated.]

# 2  Usage Models

## 2.2  Multiple Radio Access Technology

Future hierarchical network deployments are expected to be heterogeneous and integrate multiple radio access technologies. Synergistic utilization of the several available multiple radio interfaces, offers a rich dimension for enhancing the capabilities of hierarchical network deployments. This section describes some usages exploiting multi-radio access technologies (Multi-RATs). Especially, we emphasize that an integrated IEEE 802.16/802.11 BS can be used to efficiently manage diverse IEEE 802.11 networks to improve network/user performance, quality of service, battery power and so on.

For example, Multi-RAT deployment has advantage in the circumstance of disaster. There is a possibility that single radio access network is disrupted or degraded for a long period, because of the damage of its core network. Multi-RAT is favorable to the continuity of communications. Appendix C implies the details of the scenario.

# Appendix C: Cognitive Radio Systems with Multi Radio Access Technology

## C.1	Disaster applications of Cognitive Radio Systems

Multi-RAT deployment has the usage scenario as shown in the figure xx, where the following cognitive radio functionalities are utilized.

  - Surveillance of radio access network

- A Multi-RAT device is able to recognize the effects experienced in each radio access network.

- The Multi-RAT device uses an element to define network status and that to measure its variation, and those criteria.

- The intelligent access node, which has Multi-RAT radio interface for WAN and LAN, may cooperate to work with network node or LAN device.

  - Cooperative controlling of Multi-RAT

- A Multi-RAT device is able to communicate through appropriate radio access network based on usage of the communication and its destination, without producing unnecessary traffic load.

- In order to provide continuous operation, user’s traffic is routed to appropriate radio access network.

IEEE802.11 network

BS

BS

IEEE 802.16x

Internet

Ad hoc network

IEEE802.11 mesh network

Mesh network

Cellular

Satellite

Intelligent access node

LAN Interface

WAN Interface

IEEE 802.11 terminals

Figure xx System architectures with intelligent access node

## C.2	Key Features and Requirements

- IEEE 802.16 device supporting cognitive radio function shall support key features and requirements for multi-RAT functionality.

- IEEE 802.16 device supporting cognitive radio function should recognize performance of a radio interface.

- IEEE 802.16 device supporting cognitive radio function should be able to transmit the data packets to an appropriate radio interface based on its application and destination.

- IEEE 802.16 device supporting cognitive radio function should be able to provide the required communication quality at least by combining multiple RATs.

---------------------------------- The end of text -----------------------------------------