| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications of M2M short data burst transmission |  |
| Date Submitted | 2012-01-06 |  |
| Source(s) | Honggang Li, Mohanty Shantidev, Rui Huang, Inbar Anson Bratspiess, Lei Wang, HyunJeong Kang, Lei Zhou, Ming-Hung Tao, Erik Colban Intel, Sequans, InterDigital, Samsung, ITRI | Email: honggang.li@intel.com |
| Re: | IEEE 802.16-11/0049, IEEE 802.16 Working Group Letter Ballot #33a |  |
| Abstract | This contribution proposes the text to modify M2M short data burst transmission |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications of M2M short data burst transmission

Honggang Li, Mohanty Shantidev, Rui Huang

Intel

Inbar Anson Bratspiess

Sequans

Lei Wang

Interdigital

Hyunjeong Kang, Lei Zhou

Samsung

Ming-Hung Tao

ITRI

Erik Colban

Drawmetry

# Introduction

At the last meeting, based on Lei Wang’s suggestion to modify the process of short data burst (SDB) transmission for M2M, we have worked out a solution to replace current two-round RNG-REQ/RSP mechanism with the combining of RNG-REQ based SDB transmission and RNG-REQ based bandwidth request mechanisms (we call it “fast network reentry”), the mechanism selection depends on if the packet to send can be accommodated by one-time RNG-REQ message, as shown in figure 1.

Figure 1, SDB transmission

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the texts as follows, Page 17, line 31:]

6.3.36 M2M short data burst transmission

If the an M2M device receives the a bandwidth allocation that is enough sufficient for piggybacking M2M Short Data Burst contents in a RNG-REQ message duringat network reentry, it will may send the RNG-REQ message with a piggybacked M2M Short Data Burst with SFID for Short Data Burst directly.

If the M2M device receives the bandwidth allocation that is not enough for piggybacking M2M Short Data Burst contents but can accommodate the 3-byte M2M Short Data Burst Request TLV in RNG-REQ at network reentry, the M2M device sends the RNG-REQ with the M2M Short Data Burst Request TLV to indicate it has a short data burst to send. If the BS receives the RNG-REQ with M2M Short Data Burst Request TLV successfully, it may accept or reject the request. In this case the BS shall transmit a RNG-RSP with M2M Short Data Burst Response TLV with an action code instructing the M2M device how to proceed. If the BS accepts the M2M Short Data Burst Request, the BS shall transmit a RNG-RSP with M2M Short Data Burst Response TLV, with a Basic CID and a Temp CID Timer to be used for resource allocation for short data burst transmission. For fixed M2M device, the M2M device can send RNG-REQ for short data burst with the purpose indication for location update, but the paging related parameters can be removed from the RNG-REQ message to reduce the overhead because the BS is aware of its mobility information.

If the M2M device receives a RNG-RSP message rejecting its M2M Short Data Burst Request, it shall proceed according to the action code. If the M2M device receives a RNG-RSP message accepting its M2M Short Data Burst Request, it shall wait for bandwidth allocation on its Basic CID and send a RNG-REQ message with an M2M Short Data Burst TLV.

If the data is received successfully, the BS shall send a RNG-RSP message with an M2M Short Data Burst Confirmation TLV. This concludes the second RNG-REQ/RSPSDB transaction.

The Basic CID is released once the M2M device receives the M2M Short Data Burst Confirmation TLV, or when the Temp CID Timer expires.

If an M2M device receives a bandwidth allocation that is not sufficient for piggybacking M2M Short Data Burst contents, it may include an M2M Bandwidth Request combined with the related SFID in a RNG-REQ message during network re-entry. If the allocation allows for it, it may additionally include the Bandwidth Request Size in the RNG-REQ message. If the BS receives the RNG-REQ message with an M2M Bandwidth Request, the BS shall include M2M Bandwidth Request ACK in the RNG-RSP message. If the BS accepts the bandwidth request, the BS shall allocate UL bandwidth to the M2M device after network re-entry completion.

To transmit short data DL bursts, the BS may include an M2M Short Data Burst TLV in a RNG-RSP message when the action code of MOB_PAG-ADV indicates location update.

For DL short data burst transmission, the BS should send a Basic CID and a Temp CID Timer in a RNG-RSP message. When the M2M device successfully receives a RNG-RSP message with an M2M Short Data Burst, a Basic CID and the Temp CID Timer, it shall wait for bandwidth allocation on the Basic CID and transmit a RNG-REQ message containing an M2M Short Data Burst Confirmation TLV.

[Remedy #2: Modify the table 685 as follows, Page 28, line 26:]

Table 685 - RNG-REQ message encodings

| Name | Type (1byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | - |
| M2M Short Data Burst Request | 25 | 1 | Bit0 -7: No.of bytes of Short Data Burst message | OFDMA |
| M2M Short Data Burst. | 26 | Variable | M2M Short Data Burst message content up to 140bytes Padding bits to align boundary of byte.. | OFDMA |
| M2M Short Data Burst Confirmation | 27 | 1 | Bit0: Short Data Burst Confirmation 0 – NACK 1 – ACK Bit1-7: Reserved | OFDMA |

[Remedy #2: Modify the table 688 as follows, Page 29, line 44:]

Table 688 - RNG-RSP message encodings

| Name | Type (1byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| M2M Short Data Burst Response | 45 | 1 | Bit0 - 1: accept or reject Short Data Burst Request 0b00: reject 0b01: accept Bits 2-3: action code, applied to bits 0-1=0b00 only 0b00: network reentry 0b01-0b11:Reserved Bit 4-7: Reserved | OFDMA |
| … | … | … | … | - |

---- End of proposed text ----