| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested remedies for editor’s notes |  |
| Date Submitted | 2012-01-18 |  |
| Source(s) | Hyunjeong Kang Samsung Electronics Yan-Xiu Zheng ITRI | hyunjeong.kang@samsung.com zhengyanxiu@itri.org.tw |
| Re: | Proposed text changes to P802.16.1/D3 |  |
| Abstract | This contribution proposes text to resolve editor’s notes. |  |
| Purpose | To be discussed and adopted for the 802.16.1 D4 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Suggested remedies for editor’s notes

Hyunjeong Kang

Samsung Electronics

Yan-Xiu Zheng

ITRI

- References

[1] IEEE P802.16.1/D3_Nov 2011

- Introduction

Editor’s notes in the following subclauses are addressed:

- 1. Overview

- 2. Normative references

- 5. Service specific CS

- Proposed Text

Remedy #1: Add the following text after subclause 1.2 (page 1, line 65):

<insert>

1.3 Frequency bands

The primary bands of interest are described in 1.3.1.

1.3.1 Frequencies below 11 GHz

Frequencies below 11 GHz provide a physical environment where, due to the longer wavelength, LOS is not necessary and multipath may be significant. The ability to support near-LOS and non-LOS (NLOS) scenarios requires additional PHY functionality, such as the support of advanced power management techniques, interference mitigation/coexistence, and multiple antennas.

</insert>

Remedy #2: Add the following text at the end of subclause 1.4 (page 2, line 14):

<insert>

Figure 1—IEEE 802.16.1 protocol layering, showing SAPs

The MAC CPS provides the core MAC functionality of system access, bandwidth allocation, connection establishment, and connection maintenance. It receives data from the various CSs, through the MAC SAP, classified to particular MAC connections. Quality of service (QoS) is applied to the transmission and scheduling of data over the PHY.

The MAC also contains a separate security sublayer providing authentication, secure key exchange, and encryption.

Data, PHY control, and statistics are transferred between the MAC CPS and the PHY via the PHY SAP (which is implementation specific).

The PHY specifications supported is discussed in Clause 6.3.

The Management/Control Plane may also include the “CX Management part” of WirelessMAN-CX composed of the “Distributed Coexistence Information Database,” “Distributed Radio Resource Management,” and “Coexistence Protocol (CXP).” All these parts are supported at the MAC level.

</insert>

Remedy #3: Add the following text after subclause 1.4 (page 2, line 15):

<insert>

1.4.1 IEEE 802.16.1 entity

An IEEE 802.16.1 entity is defined as the logical entity in an AMS or ABS that comprises the PHY and MAC layers of the Data Plane and the Management/Control Plane.

1.4.2 Network reference model

Figure 2 describes a simplified network reference model. Multiple AMS may be attached to an ABS. An AMS communicate to the ABS over the U interface using a Primary Management Connection, a Basic Connection, or a Secondary Management Connection.

Figure 2 - IEEE 802.16.1 Network reference model

1.4.2.1 AMS and ABS Interface

This standard observes the following correlation:

— MAC control PDUs that are exchanged on the control connection trigger or are triggered by primitives that are exchanged over either the C-SAP or the M-SAP depending on the particular management or control operation.

</insert>

Remedy #4: Change the title and section number of 1.4.4.2 as follows: (page 2, line 15)

<delete>1.4.4.2</delete><insert>1.4.2.2</insert> IEEE 802.16<insert>.1</insert> entity to NCMS Interface

Remedy #5: Change the figure number in 1.4.4.2 as follows: (page 2, line 18)

This interface is a set of SAP between an IEEE 802.16 entity and NCMS and is represented in Figure <delete>3</delete><insert>2</insert>.

Remedy #6: Add the following text at the end of subclause 1.4.4.2: (page 2, line 29)

<insert>

1.4.3 Management SAP (M-SAP)

The Management SAP may include, but is not limited to primitives related to the following:

— System configuration

— Monitoring statistics

— Notifications/Triggers

— Multi-mode interface management

The NCMS interacts with the MIB through the M-SAP in a method not defined in this standard.

1.4.4 Control SAP (C-SAP)

The Control SAP may include, but is not limited to primitives related to the following:

— Handovers (e.g., notification of HO request from AMS)

— Idle mode mobility management (e.g., Mobile entering idle mode)

— Subscriber and session management (e.g., Mobile requesting session setup)

— Radio resource management

— AAA server signaling (e.g., EAP payloads)

</insert>

Remedy #7: Add the following text at the end of subclause 2: (page 3, line 26)

<insert>

FIPS 197, Advanced Encryption Standard (AES).

IEEE Std 802㉿, IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture.

IEEE Std 802.1D™, IEEE Standard for Local and metropolitan Area Networks: Media Access Control (MAC) Bridges.

IEEE Std 802.1Q, IEEE Standards for Local and Metropolitan Area Networks: Virtual Bridged Local Area Networks.

IEEE Std 802.2™ (ISO/IEC 8802-2: 1998), Information technology—Telecommunications and information exchange between systems—Local and metropolitan area networks—Specific requirements—Part 2: Logical Link Control.

IEEE Std 802.3™, IEEE Standard for Information technology—Telecommunications and information exchange between systems—Local and metropolitan area networks—Specific requirements—Part 3: Carrier sense multiple access with collision detection (CSMA/CD) access method and physical layer specifications.

IEEE Std 802.21™-2008, IEEE Standard for Local and Metropolitan Area Networks: Media Independent Handover Services.

IETF RFC 791, “Internet Protocol,” J. Postel, September 1981.

IETF RFC 868, “Time Protocol,” J. Postel, K. Harrenstien, May 1983.

IETF RFC 1042, “A Standard for the Transmission of IP Datagrams over IEEE 802 Networks,” J. Postel, J. Reynolds, February 1988.

IETF RFC 1123, “Requirements for Internet Hosts—Application and Support,” R. Braden, October 1989.

IETF RFC 1157, “A Simple Network Management Protocol (SNMP),” M. Schoffstall, M. Fedor, J. Davin, and J. Case, May 1990.

IETF RFC 1213, “Management Information Base for Network Management of TCP/IP-based internets: MIB-II,” March 1991.

IETF RFC 2131, “Dynamic Host Configuration Protocol,” R. Droms, March 1997.

IETF RFC 2373, “IP Version 6 Addressing Architecture,” R. Hinden, S. Deering, July 1998. (http://www.ietf.org/rfc/rfc2373.txt)

IETF RFC 2459, “Internet X.509 Public Key Infrastructure Certificate and CRL Profile,” R. Housley, W.Ford, W. Polk, D. Solo, January 1999.

IETF RFC 2460, “Internet Protocol, Version 6 (IPv6) Specification,” S. Deering, R. Hinden, December 1998.

IETF RFC 2462, “IPv6 Stateless Address Autoconfiguration,” S. Thomson, T. Narten, December 1998. (http://www.ietf.org/rfc/rfc2462.txt)

IETF RFC 2474, “Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers,” K. Nichols, S. Blake, F. Baker, D. Black, December 1998.

IETF RFC 2578, “Structure of Management Information Version 2 (SMIv2),” April 1999.

IETF RFC 2758, “Definitions of Managed Objects for Service Level Agreements Performance Monitoring,” K. White, February 2000.

IETF RFC 2789, “Mail Monitoring MIB,.” N. Freed, S. Kille, March 2000. (http://www.ietf.org/rfc/rfc2789.txt)

IETF RFC 2863, “The Interfaces Group MIB,” June, 2000.

IETF RFC 3012, “Mobile IPv4 Challenge/Response Extensions,” C. Perkins, P. Calhoun, November 2000. (http://www.ietf.org/rfc/rfc3012.txt)

IETF RFC 3095, “RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed,” C. Bormann, et al, July 2001. (http://www.ietf.org/rfc/rfc3095.txt)

IETF RFC 3280, “Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile,” R. Housley, W. Polk, W. Ford, D. Solo, April 2002. (http://www.ietf.org/rfc/rfc3280.txt)

IETF RFC 3410, “Introduction and Applicability Statements for Internet-Standard Management Framework,” December 2002.

IETF RFC 3411, “An Architecture for Describing Simple Network Management Protocol (SNMP) Management Frameworks,” December 2002.

IETF RFC 3412, “Message Processing and Dispatching for the Simple Network Management Protocol (SNMP),” December 2002.

IETF RFC 3413, “Simple Network Management Protocol (SNMP) Applications,” December 2002.

IETF RFC 3414, “User-based Security Model (USM) for version 3 of the Simple Network Management Protocol (SNMPv3),” December 2002.

IETF RFC 3415, “View-based Access Control Model (VACM) for the Simple Network Management Protocol (SNMP),” December 2002.

IETF RFC 3418, “Management Information Base (MIB) for the Simple Network Management Protocol (SNMP),” December 2002.

IETF RFC 3545, “Enhanced Compressed RTP (CRTP) for Links with High Delay, Packet Loss and Reordering,” T. Koren, S. Casner, J. Geevarghese, B. Thompson, P. Ruddy, July 2003. (http://www.ietf.org/rfc/rfc3545.txt)

IETF RFC 3584, “Coexistence between Version 1, Version 2, and Version 3 of the Internet-standard Network Management Framework,” August 2003.

IETF RFC 3748, “Extensible Authentication Protocol (EAP),” B. Aboba, L. Blunk, J. Vollbrecht, J. Carlson, H. Levkowetz, June 2004. (http://www.ietf.org/rfc/rfc3748.txt)

IETF RFC 3759, “RObust Header Compression (ROHC): Terminology and Channel Mapping Examples,” L-E Jonsson, April 2004. (http://www.ietf.org/rfc/rfc3759.txt)

IETF RFC 3775, “Mobility Support in IPv6,” D. Johnson, C. Perkins, J. Arkko, June 2004. (http://www.ietf.org/rfc/rfc3775.txt)

IETF RFC 3825, “DHCP for Coordinate-based Location Configuration Information,” July 2004.

IETF RFC 4291, “IP Version 6 Addressing Architecture,” R. Hinden, S. Deering, February 2006.

Internet Assigned Numbers Authority (IANA), “Protocol Numbers,” <http://www.iana.org/assignments/protocol-numbers>, June 2001.

ISO/IEC 8824, Information processing systems—Open Systems Interconnection—Specification of Abstract Syntax Notation One (ASN.1), December 1987.

ISO/IEC 8825, Information technology—Open Systems Interconnection—Specification of the Basic Encoding Rules for Abstract Syntax Notation One (ASN.1), May 1999.

ITU Radio Regulations, Volume 1, Article 5, 2008.

ITU-T Recommendation X.25—Interface between Data Terminal Equipment (DTE) and Data Circuit-terminating Equipment (DCE) for terminals operating in the packet mode and connected to public data networks by dedicated circuit, October 1996.

ITU-T Recommendation X.690, Information Technology—ASN.1 Encoding Rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER), and Distinguished Encoding Rules (DER), December 1997.

NIST Special Publication 800-38A—Recommendation for Block Cipher Modes of Operation—Methods and Techniques.

NIST Special Publication 800-38B—Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication.

WiMAX Forum® Mobile System Profile Release 1—IMT-2000 Edition.

WiMAX Forum Mobile System Profile Release 1.5—Common Part.

WiMAX Forum Mobile System Profile Release 1.5—FDD Specific Part.

WiMAX Forum Mobile System Profile Release 1.5—TDD Specific Part.

</insert>

Remedy #8: Modify subclause 5 and its subclauses as follows: (page 13, line 1)

5. Service-specific CS

The service-specific CS resides on top of the MAC CPS and utilizes, via the MAC SAP, the services provided by the MAC CPS. The CS performs the following functions: — Accepting higher layer protocol data units (PDUs) from the higher layer — Performing classification of higher layer PDUs — Processing (if required) the higher layer PDUs based on the classification — Delivering CS PDUs to the appropriate MAC SAP — Receiving CS PDUs from the peer entity Currently,<delete> three CS specifications are provided: the asynchronous transfer mode (ATM) CS, </delete> the packet CS <delete>, and the Generic Packet CS</delete> <insert>specification is provided</insert>. Other CSs may be specified in the future.

5.1 ATM CS

The ATM CS is not supported by the AMS or the ABS.

5.2 Packet CS

The packet CS resides on top of the IEEE 802.16 MAC CPS. The CS performs the following functions, utilizing the services of the MAC:

a) Classification of the higher layer protocol PDU into the appropriate transport connection

b) Suppression of payload header information (optional)

c) Delivery of the resulting CS PDU to the MAC SAP associated with the service flow for transport to the peer MAC SAP

d) Receipt of the CS PDU from the peer MAC SAP

e) Rebuilding of any suppressed payload header information (optional)

The sending CS is responsible for delivering the MAC service data unit (MAC SDU) to the MAC SAP. The MAC is responsible for delivery of the MAC SDU to peer MAC SAP in accordance with the QoS, fragmentation, concatenation, and other transport functions associated with a particular connection’s service flow characteristics. The receiving CS is responsible for accepting the MAC SDU from the peer MAC SAP and delivering it to a higher layer entity.

The packet CS is used for transport for all packet-based protocols as defined in 11.13.18.3 in IEEE802.16- 2009 [Editor's note: Need to update cross reference before sponsor ballot finalization].

5.2.1 MAC SDU format

Once classified and associated with a specific MAC connection, the Convergence Sublayer SDUs (CS SDUs), i.e., higher layer PDUs, shall be encapsulated in the MAC SDU format as illustrated in Figure <delete>1</delete><insert>3</insert>. The 8-bit PHSI (payload header suppression index) field shall be present when a PHS rule has been defined for the associated connection. PHS is described in 5.2.3. The 8-bit Type ID field shall be present when a Multiprotocol flow is defined for the associated connection. This is described in 5.2.6.

Figure <delete>1</delete> <insert>3</insert> – MAC SDU format

5.2.2 Classification

Several classification rules may each refer to the same service flow. The classification rule priority is used for ordering the application of classification rules to packets. Explicit ordering is necessary because the patterns used by classification rules may overlap. The priority need not be unique, but care shall be taken within a classification rule priority to prevent ambiguity in classification. DL classification rules are applied by the <insert>A</insert>BS to packets it is transmitting and UL classification rules are applied at the <delete>SS</delete><insert>AMS</insert>. <delete>Figure 2 and Figure 3</delete><insert>Figure 4 and Figure 5</insert> illustrate the mappings discussed in the previous paragraph. Classification is the process by which a MAC SDU is mapped onto a particular transport connection for transmission between MAC peers. The mapping process associates a MAC SDU with a transport connection, which also creates an association with the service flow characteristics of that connection. This process facilitates the delivery of MAC SDUs with the appropriate QoS constraints. A classification rule is a set of matching criteria applied to each packet entering the IEEE 802.16 network. It consists of some protocol-specific packet matching criteria (destination IP address, for example), a classification rule priority, and a reference to a CID, or for an ABS or AMS reference to a STID+FID combination. If a packet matches the specified packet matching criteria, it is then delivered to the SAP for delivery on the connection defined by the CID or STID+FID. Implementation of each specific classification capability (as, for example, IPv4 based classification) is optional. The service flow characteristics of the connection provide the QoS for that packet.

It is possible for a packet to fail to match the set of defined classification rules. In this case, the CS shall discard the packet.

<delete>

</delete>

<insert>

Figure 4 – Classification and CID (or STID/FID) mapping (ABS to AMS)

</insert>

<delete>

</delete>

<insert>

Figure 5 – Classification and CID (or STID/FID) mapping (AMS to ABS)

</insert>

5.2.3 Payload header suppression (PHS)

In PHS, a repetitive portion of the payload headers of the higher layer is suppressed in the MAC SDU by the sending entity and restored by the receiving entity. Implementation of PHS capability is optional. On the UL, the sending entity is the <delete>SS</delete><insert>AMS</insert> and the receiving entity is the <delete>BS</delete><insert>ABS</insert>. On the DL, the sending entity is the <delete>BS</delete><insert>ABS</insert> and the receiving entity is the <delete>SS</delete><insert>AMS</insert>. If PHS is enabled at MAC connection, each MAC SDU is prefixed with a PHSI, which references the Payload Header Suppression field (PHSF).

The sending entity uses classification rules to map packets into a service flow. The classification rule uniquely maps packets to its associated PHS Rule. The receiving entity uses the CID or STID+FID and the PHSI to restore the PHSF. Once a PHSF has been assigned to a PHSI, it shall not be changed. To change the value of a PHSF on a service flow, a new PHS rule shall be defined, the old rule is removed from the service flow, and the new rule is added. When all classification rules associated with the PHS rule are deleted, then the PHS rule shall also be deleted.

PHS has a payload header suppression valid (PHSV) option to verify or not verify the payload header before suppressing it. PHS has also a payload header suppression mask (PHSM) option to allow select bytes not to be suppressed. The PHSM facilitates suppression of header fields that remain static within a higher layer session (e.g., IP addresses), while enabling transmission of fields that change from packet to packet (e.g., IP Total Length).

The <delete>BS</delete><insert>ABS</insert> shall assign all PHSI values just as it assigns all CID or STID+FID values. Either the sending or the receiving entity shall specify the PHSF and the payload header suppression size (PHSS). This provision allows for preconfigured headers or for higher level signaling protocols outside the scope of this standard to establish cache entries.

It is the responsibility of the higher layer service entity to generate a PHS Rule that uniquely identifies the suppressed header within the service flow. It is also the responsibility of the higher layer service entity to guarantee that the byte strings that are being suppressed are constant from packet to packet for the duration of the active service flow.

5.2.3.1 PHS operation

<delete>SS</delete><insert>AMS</insert> and <delete>BS</delete><insert>ABS</insert> implementations are free to implement PHS in any manner as long as the protocol specified in this subclause is followed. Figure <delete>4</delete><insert>6</insert> illustrates the following procedure.

A packet is submitted to the packet CS. The <delete>SS</delete><insert>AMS</insert> applies its list of classification rules. A match of the rule shall result in an UL service flow and CID or STID+FID and may result in a PHS Rule. The PHS Rule provides PHSF, PHSI, PHSM, PHSS, and PHSV. If PHSV is set to 0 or not present, the <delete>SS</delete><insert>AMS</insert> shall compare the bytes in the packet header with the bytes in the PHSF that are to be suppressed as indicated by the PHSM. If they match, the <delete>SS</delete><insert>AMS</insert> shall suppress all the bytes in the UL PHSF except the bytes masked by PHSM. The <delete>SS</delete><insert>AMS</insert> shall then prefix the PDU with the PHSI and present the entire MAC SDU to the MAC SAP for transport on the UL.

When the MAC protocol data unit (MAC PDU) is received by the <delete>BS</delete><insert>ABS</insert> from the air interface, the <delete>BS</delete><insert>ABS</insert> MAC shall determine the associated CID or STID+FID by examination of the Generic MAC header or Advanced Generic MAC Header. The <delete>BS</delete><insert>ABS</insert> MAC sends the SDU to the MAC SAP associated with that CID or STID+FID. The receiving packet CS uses the CID or STID+FID and the PHSI to look up PHSF, PHSM, and PHSS. The <delete>BS</delete><insert>ABS</insert> reassembles the packet and then proceeds with normal packet processing. The reassembled packet contains bytes from the PHSF. If verification was enabled, then the PHSF bytes equal the original header bytes. If verification was not enabled, then there is no guarantee that the PHSF bytes match the original header bytes.

A similar operation occurs on the DL. The <delete>BS</delete><insert>ABS</insert> applies its list of Classifiers classification rules. A match of the classification shall result in a DL service flow and a PHS rule. The PHS rule provides PHSF, PHSI, PHSM, PHSS, and PHSV. If PHSV is set to 0 or not present, the <delete>BS</delete><insert>ABS</insert> shall verify the Downlink Suppression field in the packet with the PHSF. If they match, the <delete>BS</delete><insert>ABS</insert> shall suppress all the bytes in the Downlink Suppression field except the bytes masked by PHSM. The <delete>BS</delete><insert>ABS</insert> shall then prefix the PDU with the PHSI and present the entire MAC SDU to the MAC SAP for transport on the DL.

The <delete>SS</delete><insert>AMS</insert> shall receive the packet based upon the CID or STID+FID Address filtering within the MAC. The <delete>SS</delete><insert>AMS</insert> receives the PDU and then sends it to the CS. The CS then uses the PHSI and CID or STID+FID to lookup PHSF, PHSM, and PHSS. The <delete>SS</delete><insert>AMS</insert> reassembles the packet and then proceeds with normal packet processing.

Figure <delete>5</delete><insert>7</insert> demonstrates packet header suppression and restoration when using PHS masking. Masking allows only bytes that do not change to be suppressed. Note that the PHSF and PHSS span the entire suppression field, included suppressed and unsuppressed bytes.

Figure <delete>4</delete><insert>6</insert> – PHS operation

Figure <delete>5</delete><insert>7</insert> – PHS with masking

5.2.3.2 PHS signaling

PHS requires the creation of the following three objects:

a) Service flow

b) Classification rule

c) PHS rule

These three objects may be created either simultaneously or in separate message flows.

PHS rules are created with DSA or dynamic service change (DSC) messages. The ABS shall define the PHSI when the PHS rule is created. If the AMS includes more than one classification rule and at least one PHS rule in AAI-DSx-REQ message when initiating a dynamic service addition via AAI-DSA-REQ or change via AAI-DSC-REG, the MS shall include a temporary PHSI with each PHS rule in the AAI-DSx- REQ message. Temporary PHSI(s) shall only be used by the ABS to identify the association of the PHS rule to corresponding classification rule(s) included in the AAI-DSA-REQ or AAI-DSC-REQ from the AMS by inclusion of the Associated PHSI field (see Table 131) and shall have no further meaning beyond this. Tem-porary PHSI(s) included in AAI-DSx-REQ sent from the AMS shall not constrain the PHSI values assigned by the ABS. If the ABS cannot identify the association between PHS rule(s) and classification rule(s), the ABS shall reject the AAI-DSx-REQ message sent from the AMS. PHS rules are deleted with the DSC or dynamic service deletion (DSD) messages. The AMS or ABS may define the PHSS and PHSF. To change the value of a PHSF on a service flow, a new PHS rule shall be defined, the old rule is removed from the ser-vice flow, and the new rule is added.

Figure <delete>6</delete><insert>8</insert> shows the two ways to signal the creation of a PHS rule.

It is possible to partially specify a PHS rule (in particular the size of the rule) at the time a service flow is created. As an example, it is likely that when a service flow is first provisioned, the header fields to be suppressed will be known. The values of some of the fields [for example: IP addresses, User Datagram Protocol (UDP) port numbers, etc.] may be unknown and would be provided in a subsequent AAI-DSC as part of the activation of the service flow (using the “Set PHS Rule” DSC Action). If the PHS rule is being defined in more than one step, each step, whether it is a AAI-DSA or AAI-DSC message, shall contain both the SFID (or reference) and a PHS index to uniquely identify the PHS rule that is being defined.

Figure <delete>6</delete><insert>8</insert> – PHS signaling example

5.2.4 IEEE 802.3/Ethernet-specific part

5.2.4.1 IEEE 802.3/Ethernet CS PDU format

The IEEE 802.3/Ethernet PDUs are mapped to MAC SDUs according to Figure <delete>7</delete><insert>9</insert> (when header suppression is enabled at the connection, but not applied to the CS PDU) or Figure <delete>8</delete><insert>10</insert> (with header suppression). In the case where PHS is not enabled, PHSI field shall be omitted.

The IEEE 802.3/Ethernet PDU shall not include the Ethernet FCS when transmitted over this CS.

Figure <delete>7</delete><insert>9</insert>—IEEE 802.3/Ethernet CS PDU format without header suppression

<insert>

Figure 10—IEEE 802.3/Ethernet CS PDU format with header suppression

</insert>

ROHC (refer to IETF RFC 3095) may be used in addition to PHS to compress the IP header portion of an IP packet over Ethernet frame. <delete>The MS and the BS shall set bit 7 of Request/Transmission Policy (see 11.13.11 in IEEE802.16-2009 [Editor's note: Need to update cross reference before sponsor ballot finalization]) to 0 to enable ROHC.</delete> The AMS and the ABS shall set bit 7 of Request/Transmission Policy (see Table 131) to 0 to enable ROHC. When ROHC is enabled for a service flow, the service flow constitutes what in RFC 3095 is referred to as a ROHC channel. Two service flows cannot share a ROHC channel, and two ROHC channels cannot share the same service flow. On a service flow for which ROHC has been enabled, all of the IP

<delete>

Figure 8—IEEE 802.3/Ethernet CS PDU format with header suppression

</delete>

packet parts of IP over Ethernet frames shall pass through the ROHC compressor on the sender side and the decompressor on the receiver side.

ROHC compression and decompression operation shall be performed in accordance with RFC 3095, RFC 3759, RFC 3243, RFC 4995, RFC 3843, RFC 4996. To enable ROHC, the following two steps are required:

1) Capability negotiation during REG-REQ/RSP message exchange to determine whether ROHC is supported. In AAI system, capability negotiation during AAI-REG-REQ/RSP message exchange to determine whether ROHC is supported.

2) Indication in DSA-REQ/RSP messages to enable ROHC for the service flow. In AAI system, indication in AAI-DSA-REQ/RSP messages to enable ROHC for the service flow.

5.2.4.2 IEEE 802.3/Ethernet CS classification rules

The following parameters are relevant for IEEE 802.3/Ethernet CS classification rules:

— IEEE 802.3/Ethernet header classification parameters—zero or more of the IEEE 802.3/Ethernet, VLAN and IP headers may be included in the classification. In this case, only the IEEE 802.3/IEEE 802.1Q/IP (11.13.18.3.3.2 through 11.13.18.3.3.12 and 11.13.18.3.3.16 in IEEE802.16-2009[Editor's note: Need to update cross reference before sponsor ballot finalization]) classification parameters are allowed.

— For IP over IEEE 802.3/Ethernet, Ethernet, and VLAN, IP headers may be included in classification. In this case, only the IP, IEEE 802.3 and IEEE 802.1Q (11.13.18.3.3.2 through 11.13.18.3.3.12 and 11.13.18.3.3.16 in IEEE802.16-2009[Editor's note: Need to update cross reference before sponsor ballot finalization]) classification parameters are allowed.

— For IP-header compressed IP over IEEE 802.3/Ethernet, Ethernet and VLAN headers may be included in the classification. In this case, only the IEEE 802.3/IEEE 802.1Q (11.13.18.3.3.8 through 11.13.18.3.3.12 in IEEE802.16-2009[Editor's note: Need to update cross reference before sponsor ballot finalization]) classification parameters are allowed.

5.2.5 IP specific part

This subclause applies when IP (IETF RFC 791 and IETF RFC 2460) is carried over the IEEE 802.16 network.

5.2.5.1 IP CS PDU format

The format of the IP CS PDU shall be as shown in Figure <delete>9</delete><insert>11</insert> (when header suppression is enabled at the connection, but not applied to the CS PDU) or Figure <delete>10</delete><insert>12</insert> (with header suppression). In the case where PHS is not enabled, the PHSI field shall be omitted.

Figure <delete>9</delete><insert>11</insert>—IP CS PDU format without header suppression

Figure <delete>10</delete><insert>12</insert>—IP CS PDU format with header suppression

ROHC (refer to RFC 3095) may be used instead of PHS to compress IP headers. <delete>The MS and the BS signal enabling of ROHC by setting bit 7 of Request/Transmission Policy (see 11.13.11 in IEEE802.16-2009 [Editor's note: Need to update cross reference before sponsor ballot finalization]) to 0.</delete> The AMS and the ABS signal enabling of ROHC by setting Bit 6 of Request/Transmission Policy to 0 in the AAI-DSA-REQ message. When ROHC is enabled for a service flow, the service flow constitutes what in RFC 3095 is referred to as a ROHC channel.

Two service flows cannot share a ROHC channel, and two ROHC channels cannot share the same service flow. All IP packets that are classified onto a service flow for which ROHC has been enabled shall pass through the ROHC compressor on the sender side, and the decompressor on the receiver side.

ROHC compression and decompression operation shall be performed in accordance with RFC 3095, RFC 3759, RFC 3243, RFC 4995, RFC 3843, RFC 4996. To enable ROHC, the following two steps are required:

1) Capability negotiation during REG-REQ/RSP message exchange to determine whether ROHC is supported. In AAI system, Capability negotiation during AAI-REG-REQ/RSP message exchange to determine whether ROHC is supported.

2) Indication in DSA-REQ/RSP messages to enable ROHC for the service flow. In AAI system, indication in AAI-DSA-REQ/RSP messages to enable ROHC for the service flow.

<delete>

</delete>

<insert>

Figure 13 – ROHC Function

</insert>

5.2.5.2 IP classification rules

IP classification rules operate on the fields of the IP header and the transport protocol. <delete>For SS and BS, the parameters (11.13.18.3.3.2 through 11.13.18.3.3.7 and 11.13.18.3.3.16 in IEEE802.16-2009 [Editor's note: Need to update cross reference before sponsor ballot finalization]) may be used in IP classification rules. For AAI,</delete> IP classification rules may operate on the fields of the inner IP header and/or outer IP header. For AMS and ABS, the Packet Classification Rule parameters (Table 86) may be used in IP classification.

5.2.6 Support for multiple protocols on the same flow

In order to transport several types of protocols over the same MAC connection, the Multi-protocol flow can be used. The receiver shall identify the protocol to correctly forward the SDU. For instance, if the information carried by the SDU is a RoHC packet, it should be forwarded to the RoHC decompressor. The receiver does this according to the Type ID field, which is the first byte of a Multi-protocol flow connection as depicted in Figure <delete>12</delete><insert>14</insert> and Figure <delete>13</delete><insert>15</insert>. Type ID may vary from SDU to SDU on the same flow. Multi-protocol CS classification rules operate on the fields of the Multi-protocol Type ID, and/or the Packet Classification Rule parameters in Table 131, for example, the header fields of the IEEE 802.3/Ethernet header, VLAN header, IP header, and the transport protocol.

Figure <delete>12</delete><insert>14</insert>—Multi-protocol flow PDU format without PHS

Figure <delete>13</delete><insert>15</insert>—Multi-protocol flow PDU format with PHS

On the transmitter side, once the type of an incoming packet is determined, the appropriate classification rules are applied to the packet and the correct service flow is identified. It is then optionally forwarded to the header suppression mechanism (PHS or RoHC) and then mapped MAC SAP using the format described in this subclause. The Type ID content is set by the transmitter to the protocol identified by the classification and according to the Table 1.